# PDF to Word Output

## Page 1

u-blox 8 / u-blox M8

Receiver description

Including protocol specification

Abstract

The receiver description including protocol specification

describes the firmware features, specifications and

configuration for u-blox 8 / u-blox M8 high performance

positioning modules.

The receiver description provides an overview and

conceptual details of the supported features.

The protocol specification describes the NMEA and

RTCM protocols as well as the UBX protocol (version 15.

00 up to 19.20, version 20.00 to 20.30, version 22.00 to

22.01 and version 23.00 to 23.01) and serves as a

reference manual. It includes the standard precision

GNSS, Time Sync, Time & Frequency Sync, High

precision GNSS, ADR and UDR products.

www.u-blox.com

UBX-13003221 - R28

## Page 2

u-blox 8 / u-blox M8 Receiver description - Manual

Document Information

Title u-blox 8 / u-blox M8 Receiver description

Subtitle Including protocol specification v15-20.30,22-23.01

Document type Manual

Document number UBX-13003221

Revision and date R28 (d6aadc7) 30 January 2023

Document status Early production information

Document status explanation

Objective Specification Document contains target values. Revised and supplementary data will be published later.

Advance Information Document contains data based on early testing. Revised and supplementary data will be published later.

Early Production Information Document contains data from product verification. Revised and supplementary data may be published later.

Production Information Document contains the final product specification.

This document applies to the following products:

Product name Type number Firmware version Product category

CAM-M8C CAM-M8C-0-10 SPG 3.01 Standard Precision GNSS

CAM-M8Q CAM-M8Q-0-10 SPG 3.01 Standard Precision GNSS

EVA-M8M EVA-M8M-0-10 SPG 3.01 Standard Precision GNSS

EVA-M8M EVA-M8M-1-10 SPG 3.01 Standard Precision GNSS

EVA-M8Q EVA-M8Q-0-10 SPG 3.01 Standard Precision GNSS

MAX-M8C MAX-M8C-0-10 SPG 3.01 Standard Precision GNSS

MAX-M8Q MAX-M8Q-0-10 SPG 3.01 Standard Precision GNSS

MAX-M8W MAX-M8W-0-10 SPG 3.01 Standard Precision GNSS

NEO-M8M NEO-M8M-0-11 SPG 3.01 Standard Precision GNSS

NEO-M8N NEO-M8N-0-12 SPG 3.01 Standard Precision GNSS

NEO-M8Q NEO-M8Q-0-12 SPG 3.01 Standard Precision GNSS

NEO-M8Q NEO-M8Q-01A-10 SPG 3.01 Standard Precision GNSS

NEO-M8J NEO-M8J-0-11 SPG 3.05 Standard Precision GNSS

LEA-M8S LEA-M8S-0-10 SPG 3.01 Standard Precision GNSS

SAM-M8Q SAM-M8Q-0-10 SPG 3.01 Standard Precision GNSS

ZOE-M8G ZOE-M8G-0-11 SPG 3.01 Standard Precision GNSS

ZOE-M8Q ZOE-M8Q-0-10 SPG 3.01 Standard Precision GNSS

ZOE-M8B ZOE-M8B-0-11 SPG 3.51 Standard Precision GNSS

EVA-8M EVA-8M-0-10 SPG 3.01 Standard Precision GNSS

MAX-8C MAX-8C-0-10 SPG 3.01 Standard Precision GNSS

MAX-8Q MAX-8Q-0-10 SPG 3.01 Standard Precision GNSS

NEO-8Q NEO-8Q-0-11 SPG 3.01 Standard Precision GNSS

NEO-M8P NEO-M8P-0-11 HPG 1.40 High Precision GNSS

NEO-M8P NEO-M8P-2-11 HPG 1.40 High Precision GNSS

NEO-M8P NEO-M8P-0-12 HPG 1.43 High Precision GNSS

UBX-13003221 - R28 Early production information Page ii

## Page 3

u-blox 8 / u-blox M8 Receiver description - Manual

NEO-M8P NEO-M8P-2-12 HPG 1.43 High Precision GNSS

NEO-M8L NEO-M8L-0-10 ADR 4.00 / 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-0-11 ADR 4.10 / 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-0-12 ADR 4.11 / 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-04B-00 ADR 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-05B-00 ADR 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-06B-00 ADR 4.50 Dead Reckoning

NEO-M8L NEO-M8L-02A-11 ADR 4.10 / 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-03A-12 ADR 4.11 / 4.21 / 4.31 / 4.50 Dead Reckoning

NEO-M8L NEO-M8L-04A-00 ADR 4.21 / 4.31 / 4.50 Dead Reckoning

EVA-M8E EVA-M8E-0-11 UDR 1.00 / 1.21 / 1.31 / 1.50 Dead Reckoning

NEO-M8U NEO-M8U-0-10 UDR 1.00 / 1.21 / 1.31 / 1.50 Dead Reckoning

NEO-M8U NEO-M8U-04B-00 UDR 1.21 / 1.31 / 1.50 Dead Reckoning

NEO-M8U NEO-M8U-05B-00 UDR 1.31 / 1.50 Dead Reckoning

NEO-M8U NEO-M8U-06B-00 UDR 1.50 Dead Reckoning

NEO-M8T NEO-M8T-0-11 TIM 1.10 Timing

LEA-M8T LEA-M8T-0-10 TIM 1.10 Timing

LEA-M8T LEA-M8T-1-00 TIM 1.11 Timing

LEA-M8T LEA-M8T-1-01 TIM 1.12 Timing

LEA-M8F LEA-M8F-0-00 FTS 1.01 Timing

u-blox reserves all rights to this document and the information contained herein. Products, names, logos and designs

described herein may in whole or in part be subject to intellectual property rights. Reproduction, use, modification or

disclosure to third parties of this document or any part thereof without the express permission of u-blox is strictly

prohibited.

The information contained herein is provided "as is" and u-blox assumes no liability for the use of the information. No

warranty, either express or implied, is given, including but not limited, with respect to the accuracy, correctness, reliability

and fitness for a particular purpose of the information. This document may be revised by u-blox at any time. For most

recent documents, please visit www.u-blox.com.

Copyright © 2023, u-blox AG.

u-blox is a registered trademark of u-blox Holding AG in the EU and other countries.

UBX-13003221 - R28 Early production information Page iii

## Page 4

u-blox 8 / u-blox M8 Receiver description - Manual

Table of Contents

Preface........................................................................................................................................................................ 1

1 Document Overview....................................................................................................................................... 1

2 Firmware and Protocol Versions............................................................................................................... 1

2.1 How to Determine the Version and the Location of the Firmware........................................... 1

2.1.1 Decoding the Boot Screen (for Protocol Version 17 and Below)......................................... 1

2.1.2 Decoding the Boot Screen (for Protocol Version from 18 to 23.01).................................. 3

2.1.3 Decoding the output of UBX-MON-VER (for Protocol Version 17 and below)................ 4

2.1.4 Decoding the output of UBX-MON-VER (for Protocol Version from 18 and 23.01)....... 6

2.2 How to Determine the Supported Protocol Version of the u-blox Receiver.......................... 7

2.2.1 u-blox 8 / u-blox M8 Firmware and Supported Protocol Versions.................................... 7

Receiver Description.............................................................................................................................................. 9

3 Receiver Configuration................................................................................................................................ 9

3.1 Configuration Concept..................................................................................................................... 9

3.2 Organization of the Configuration Sections.............................................................................. 10

3.3 Permanent Configuration Storage Media................................................................................... 11

3.4 u-blox Receiver Default Configuration......................................................................................... 11

3.5 Save-on-Shutdown Feature........................................................................................................... 11

4 Concurrent GNSS........................................................................................................................................ 12

4.1 GNSS Types....................................................................................................................................... 12

4.1.1 Major GNSS................................................................................................................................ 12

4.1.2 Augmentation Systems.......................................................................................................... 13

4.2 Configuration.................................................................................................................................... 14

4.2.1 Switching between GNSS....................................................................................................... 14

4.2.2 Configuring QZSS L1SAIF...................................................................................................... 15

5 SBAS Configuration Settings Description.......................................................................................... 15

5.1 SBAS (Satellite Based Augmentation Systems)....................................................................... 15

5.2 SBAS Features................................................................................................................................. 17

5.3 SBAS Configuration........................................................................................................................ 18

6 QZSS L1S SLAS Configuration Settings Description...................................................................... 19

6.1 QZSS L1S SLAS (Sub-meter Level Augmentation Service).................................................... 19

6.2 QZSS L1S SLAS Features.............................................................................................................. 19

6.3 QZSS L1S SLAS Configuration.................................................................................................... 20

7 IMES Description........................................................................................................................................ 20

7.1 IMES Features................................................................................................................................... 21

8 Navigation Configuration Settings Description................................................................................ 21

8.1 Platform settings............................................................................................................................. 21

8.2 Navigation Input Filters................................................................................................................. 22

8.3 Navigation Output Filters............................................................................................................. 23

8.3.1 Speed (3-D) Low-pass Filter.................................................................................................. 23

UBX-13003221 - R28 Early production information Page iv

## Page 5

u-blox 8 / u-blox M8 Receiver description - Manual

8.3.2 Course over Ground Low-pass Filter................................................................................... 23

8.3.3 Low-speed Course Over Ground Filter................................................................................ 24

8.4 Static Hold........................................................................................................................................ 24

8.5 Freezing the Course Over Ground............................................................................................... 24

8.6 Degraded Navigation..................................................................................................................... 24

8.6.1 2D Navigation........................................................................................................................... 24

8.7 Geodetic Coordinate Systems and Ellipsoids........................................................................... 25

9 Clocks and Time.......................................................................................................................................... 26

9.1 Receiver Local Time........................................................................................................................ 26

9.2 Navigation Epochs.......................................................................................................................... 26

9.3 iTOW Timestamps......................................................................................................................... 27

9.4 GNSS Times..................................................................................................................................... 27

9.5 Time Validity.................................................................................................................................... 28

9.6 UTC Representation...................................................................................................................... 28

9.7 Leap Seconds.................................................................................................................................. 28

9.8 Real Time Clock............................................................................................................................... 29

9.9 Date................................................................................................................................................... 29

9.9.1 GPS-only Date Resolution...................................................................................................... 29

10 Broadcast Navigation Data................................................................................................................... 30

10.1 Parsing Navigation Data Subframes......................................................................................... 30

10.2 GPS.................................................................................................................................................. 30

10.2.1 GPS L1C/A................................................................................................................................. 31

10.3 GLONASS........................................................................................................................................ 31

10.4 BeiDou............................................................................................................................................. 32

10.5 Galileo.............................................................................................................................................. 32

10.5.1 Galileo E1OS............................................................................................................................. 32

10.6 SBAS................................................................................................................................................ 33

10.7 QZSS................................................................................................................................................ 34

10.8 IMES................................................................................................................................................. 34

10.9 Summary........................................................................................................................................ 34

11 Serial Communication Ports Description........................................................................................... 35

11.1 TX-ready indication........................................................................................................................ 35

11.2 Extended TX timeout.................................................................................................................... 36

11.3 UART Ports..................................................................................................................................... 36

11.4 USB Port........................................................................................................................................... 37

11.5 DDC Port.......................................................................................................................................... 37

11.5.1 Read Access............................................................................................................................. 38

11.5.2 Write Access........................................................................................................................... 39

11.6 SPI Port............................................................................................................................................ 40

11.6.1 Maximum SPI clock speed.................................................................................................... 40

11.6.2 Read Access............................................................................................................................ 40

11.6.3 Back-To-Back Read and Write Access.............................................................................. 40

11.7 How to change between protocols.............................................................................................. 41

UBX-13003221 - R28 Early production information Page v

## Page 6

u-blox 8 / u-blox M8 Receiver description - Manual

12 Multiple GNSS assistance (MGA)......................................................................................................... 41

12.1 Introduction..................................................................................................................................... 41

12.2 Assistance Data............................................................................................................................ 42

12.3 AssistNow Online.......................................................................................................................... 42

12.3.1 Host Software......................................................................................................................... 43

12.3.2 AssistNow Online Sequence............................................................................................... 44

12.3.3 Flow Control............................................................................................................................ 44

12.3.4 Authorization.......................................................................................................................... 44

12.3.5 Service Parameters............................................................................................................... 44

12.3.6 Multiple Servers..................................................................................................................... 46

12.4 AssistNow Offline.......................................................................................................................... 47

12.4.1 Service Parameters................................................................................................................ 47

12.4.2 Authorization.......................................................................................................................... 48

12.4.3 Multiple Servers..................................................................................................................... 48

12.4.4 Time, Position and Almanac................................................................................................ 48

12.4.5 Flash-based AssistNow Offline.......................................................................................... 49

12.4.6 Host-based AssistNow Offline........................................................................................... 50

12.5 Preserving Information During Power-off................................................................................. 51

12.6 AssistNow Autonomous............................................................................................................... 51

12.6.1 Introduction.............................................................................................................................. 51

12.6.2 Concept.................................................................................................................................... 51

12.6.3 Interface.................................................................................................................................. 53

12.6.4 Benefits and Drawbacks...................................................................................................... 54

13 Power Management................................................................................................................................. 55

13.1 Continuous Mode........................................................................................................................... 56

13.2 Power Save Mode.......................................................................................................................... 56

13.2.1 Operation.................................................................................................................................. 57

13.2.2 Configuration.......................................................................................................................... 61

13.2.3 Features.................................................................................................................................. 64

13.2.4 Examples................................................................................................................................. 65

13.3 Peak current settings................................................................................................................... 66

13.4 Power On/Off command.............................................................................................................. 66

13.5 EXTINT pin control when Power Save Mode is not active..................................................... 66

13.6 Measurement and navigation rate with Power Save Mode.................................................. 66

13.7 Power mode setup......................................................................................................................... 66

14 Forcing a Receiver Reset........................................................................................................................ 67

15 Receiver Status Monitoring.................................................................................................................. 68

15.1 Input/Output system.................................................................................................................... 68

15.2 Jamming/Interference Indicator................................................................................................ 69

15.3 Jamming/Interference Monitor (ITFM).................................................................................... 69

16 Spoofing Detection.................................................................................................................................. 70

16.1 Introduction..................................................................................................................................... 70

16.2 Scope............................................................................................................................................... 70

UBX-13003221 - R28 Early production information Page vi

## Page 7

u-blox 8 / u-blox M8 Receiver description - Manual

17 Signal Attenuation Compensation...................................................................................................... 70

18 Remote Inventory...................................................................................................................................... 71

18.1 Description....................................................................................................................................... 71

18.2 Usage................................................................................................................................................ 71

19 Time pulse.................................................................................................................................................... 71

19.1 Introduction...................................................................................................................................... 71

19.2 Recommendations........................................................................................................................ 72

19.3 GNSS time bases........................................................................................................................... 73

19.4 Time pulse configuration............................................................................................................. 74

19.5 Configuring time pulse with UBX-CFG-TP5............................................................................. 74

19.5.1 Example 1.................................................................................................................................. 75

19.5.2 Example 2................................................................................................................................ 75

20 Timemark.................................................................................................................................................... 76

21 Odometer..................................................................................................................................................... 77

21.1 Introduction..................................................................................................................................... 77

21.2 Odometer Output.......................................................................................................................... 77

21.3 Odometer Configuration.............................................................................................................. 78

21.4 Resetting the Odometer.............................................................................................................. 78

22 Logging........................................................................................................................................................ 78

22.1 Introduction.................................................................................................................................... 78

22.2 Setting the logging system up................................................................................................... 79

22.3 Information about the log........................................................................................................... 79

22.4 Recording....................................................................................................................................... 80

22.5 Retrieval.......................................................................................................................................... 81

22.6 Command message acknowledgement.................................................................................. 82

23 Data Batching............................................................................................................................................ 82

23.1 Introduction.................................................................................................................................... 82

23.2 Setting up the data batching..................................................................................................... 82

23.3 Retrieval......................................................................................................................................... 83

24 Geofencing................................................................................................................................................. 84

24.1 Introduction.................................................................................................................................... 84

24.2 Interface......................................................................................................................................... 84

24.3 Geofence state evaluation.......................................................................................................... 84

24.4 Using a PIO for Geofence State Output................................................................................... 85

25 Time Mode Configuration...................................................................................................................... 85

25.1 Introduction.................................................................................................................................... 85

25.2 Fixed Position................................................................................................................................ 85

25.3 Survey-in........................................................................................................................................ 85

26 Time & Frequency Sync (FTS).............................................................................................................. 86

26.1 Introduction.................................................................................................................................... 86

26.2 Example use cases....................................................................................................................... 87

26.2.1 Stand-alone synchronization system................................................................................ 87

26.2.2 Oscillator control via host................................................................................................... 88

UBX-13003221 - R28 Early production information Page vii

## Page 8

u-blox 8 / u-blox M8 Receiver description - Manual

26.2.3 Oscillator control via directly-connected DAC................................................................ 89

26.2.4 External (coherent) PPS...................................................................................................... 89

26.3 Synchronization Manager Concept.......................................................................................... 90

26.4 Oscillator and source specification.......................................................................................... 92

26.5 Calibration...................................................................................................................................... 93

26.6 FTS device Output and Top Of Second (TOS) message...................................................... 94

26.7 Message transmission time slot reservations on host interfaces.................................... 95

26.7.1 Example setup........................................................................................................................ 95

27 RTK Mode Configuration........................................................................................................................ 96

27.1 Reference Station Mode Configuration.................................................................................... 96

27.2 Rover Mode Configuration.......................................................................................................... 97

27.3 Moving Baseline RTK Configuration......................................................................................... 97

27.3.1 MB Reference Configuration............................................................................................... 97

27.3.2 MB Rover Configuration....................................................................................................... 97

27.3.3 Expected Performance........................................................................................................ 98

28 Automotive Dead Reckoning (ADR)................................................................................................... 98

28.1 Introduction.................................................................................................................................... 98

28.2 Solution Types.............................................................................................................................. 98

28.2.1 GAWT: Gyroscope, Accelerometer and Wheel Tick Solution....................................... 98

28.3 Installation Configuration........................................................................................................... 99

28.3.1 IMU-mount Alignment.......................................................................................................... 99

28.4 Sensor Configuration................................................................................................................. 102

28.4.1 Accelerometer Configuration............................................................................................ 102

28.4.2 Gyroscope Configuration................................................................................................... 103

28.4.3 Wheel-Tick/Speed Sensor Configuration...................................................................... 104

28.4.4 Sensor Time Tagging......................................................................................................... 106

28.5 ADR System Configuration...................................................................................................... 108

28.5.1 Enabling/Disabling Fusion Filter....................................................................................... 108

28.5.2 Recommended Configuration.......................................................................................... 108

28.6 Operation...................................................................................................................................... 108

28.6.1 Fusion Filter Modes............................................................................................................. 108

28.6.2 Accelerated Initialization and Calibration Procedure.................................................... 111

28.6.3 Automatic IMU-mount Alignment.................................................................................... 112

28.6.4 Navigation Output............................................................................................................... 114

28.6.5 Sensor Data Types............................................................................................................... 117

28.6.6 Raw Sensor Data Output................................................................................................... 118

28.6.7 Receiver Startup and Shutdown....................................................................................... 119

29 Untethered Dead Reckoning (UDR)................................................................................................... 119

29.1 Introduction................................................................................................................................... 119

29.2 Installation Configuration.......................................................................................................... 119

29.2.1 IMU-mount Alignment......................................................................................................... 119

29.3 Sensor Configuration................................................................................................................. 123

29.3.1 Accelerometer Configuration............................................................................................ 123

UBX-13003221 - R28 Early production information Page viii

## Page 9

u-blox 8 / u-blox M8 Receiver description - Manual

29.3.2 Gyroscope Configuration................................................................................................... 124

29.3.3 Sensor Time Tagging......................................................................................................... 125

29.4 UDR System Configuration....................................................................................................... 126

29.4.1 Enabling/Disabling Fusion Filter....................................................................................... 126

29.4.2 Recommended Configuration........................................................................................... 126

29.5 Operation...................................................................................................................................... 126

29.5.1 Fusion Filter Modes............................................................................................................. 127

29.5.2 Accelerated Initialization and Calibration Procedure.................................................. 129

29.5.3 Automatic IMU-mount Alignment................................................................................... 131

29.5.4 Navigation Output.............................................................................................................. 133

29.5.5 Sensor Data Types.............................................................................................................. 135

29.5.6 Raw Sensor Data Output................................................................................................... 136

29.5.7 Receiver Startup and Shutdown...................................................................................... 137

30 High Navigation Rate (HNR)................................................................................................................ 137

30.1 Introduction.................................................................................................................................. 137

30.2 Configuration............................................................................................................................... 137

Interface Description........................................................................................................................................ 138

31 NMEA Protocol......................................................................................................................................... 138

31.1 Protocol overview......................................................................................................................... 138

31.1.1 Message format.................................................................................................................... 138

31.1.2 Talker ID.................................................................................................................................. 138

31.1.3 Protocol configuration......................................................................................................... 139

31.1.4 Satellite numbering.............................................................................................................. 140

31.1.5 Latitude and longitude format........................................................................................... 141

31.1.6 Position fix flags.................................................................................................................... 141

31.1.7 Multi-GNSS considerations................................................................................................ 142

31.1.8 Output of invalid/unknown data........................................................................................ 143

31.1.9 Messages overview.............................................................................................................. 143

31.2 Standard Messages.................................................................................................................... 145

31.2.1 DTM......................................................................................................................................... 145

31.2.2 GBQ......................................................................................................................................... 146

31.2.3 GBS......................................................................................................................................... 146

31.2.4 GGA......................................................................................................................................... 147

31.2.5 GLL.......................................................................................................................................... 149

31.2.6 GLQ......................................................................................................................................... 150

31.2.7 GNQ......................................................................................................................................... 150

31.2.8 GNS.......................................................................................................................................... 151

31.2.9 GPQ......................................................................................................................................... 152

31.2.10 GRS........................................................................................................................................ 153

31.2.11 GSA........................................................................................................................................ 154

31.2.12 GST........................................................................................................................................ 155

31.2.13 GSV........................................................................................................................................ 156

31.2.14 RMC....................................................................................................................................... 157

UBX-13003221 - R28 Early production information Page ix

## Page 10

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.15 THS........................................................................................................................................ 158

31.2.16 TXT........................................................................................................................................ 159

31.2.17 VLW....................................................................................................................................... 160

31.2.18 VTG........................................................................................................................................ 161

31.2.19 ZDA........................................................................................................................................ 162

31.3 PUBX Messages........................................................................................................................... 163

31.3.1 CONFIG (PUBX,41)................................................................................................................. 163

31.3.2 POSITION (PUBX,00)........................................................................................................... 164

31.3.3 RATE (PUBX,40)................................................................................................................... 165

31.3.4 SVSTATUS (PUBX,03)........................................................................................................ 166

31.3.5 TIME (PUBX,04).................................................................................................................... 167

32 UBX Protocol............................................................................................................................................ 168

32.1 UBX Protocol Key Features........................................................................................................ 168

32.2 UBX Frame Structure................................................................................................................. 168

32.3 UBX Payload Definition Rules................................................................................................... 169

32.3.1 Structure Packing................................................................................................................ 169

32.3.2 Reserved Elements............................................................................................................. 169

32.3.3 Undefined Values................................................................................................................ 170

32.3.4 Message Naming................................................................................................................. 170

32.3.5 Number Formats................................................................................................................. 170

32.4 UBX Checksum............................................................................................................................. 171

32.5 UBX Message Flow..................................................................................................................... 172

32.5.1 Acknowledgement............................................................................................................... 172

32.5.2 Polling Mechanism.............................................................................................................. 172

32.6 UBX Class IDs............................................................................................................................... 172

32.7 UBX Messages Overview........................................................................................................... 173

32.8 UBX-ACK (0x05).......................................................................................................................... 179

32.8.1 UBX-ACK-ACK (0x05 0x01)................................................................................................ 179

32.8.2 UBX-ACK-NAK (0x05 0x00).............................................................................................. 179

32.9 UBX-AID (0x0B)........................................................................................................................... 180

32.9.1 UBX-AID-ALM (0x0B 0x30)................................................................................................ 180

32.9.2 UBX-AID-AOP (0x0B 0x33)................................................................................................ 182

32.9.3 UBX-AID-EPH (0x0B 0x31)................................................................................................. 184

32.9.4 UBX-AID-HUI (0x0B 0x02)................................................................................................. 186

32.9.5 UBX-AID-INI (0x0B 0x01).................................................................................................... 188

32.10 UBX-CFG (0x06)......................................................................................................................... 191

32.10.1 UBX-CFG-ANT (0x06 0x13)................................................................................................ 191

32.10.2 UBX-CFG-BATCH (0x06 0x93)........................................................................................ 192

32.10.3 UBX-CFG-CFG (0x06 0x09)............................................................................................. 193

32.10.4 UBX-CFG-DAT (0x06 0x06)............................................................................................. 195

32.10.5 UBX-CFG-DGNSS (0x06 0x70)........................................................................................ 197

32.10.6 UBX-CFG-DOSC (0x06 0x61)........................................................................................... 197

32.10.7 UBX-CFG-ESFALG (0x06 0x56)...................................................................................... 199

UBX-13003221 - R28 Early production information Page x

## Page 11

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.8 UBX-CFG-ESFA (0x06 0x4C).......................................................................................... 200

32.10.9 UBX-CFG-ESFG (0x06 0x4D)........................................................................................... 201

32.10.10 UBX-CFG-ESFWT (0x06 0x82)..................................................................................... 201

32.10.11 UBX-CFG-ESRC (0x06 0x60)......................................................................................... 204

32.10.12 UBX-CFG-GEOFENCE (0x06 0x69).............................................................................. 206

32.10.13 UBX-CFG-GNSS (0x06 0x3E)........................................................................................ 207

32.10.14 UBX-CFG-HNR (0x06 0x5C)........................................................................................... 210

32.10.15 UBX-CFG-INF (0x06 0x02)............................................................................................. 210

32.10.16 UBX-CFG-ITFM (0x06 0x39).......................................................................................... 212

32.10.17 UBX-CFG-LOGFILTER (0x06 0x47)............................................................................... 213

32.10.18 UBX-CFG-MSG (0x06 0x01)........................................................................................... 215

32.10.19 UBX-CFG-NAV5 (0x06 0x24)......................................................................................... 216

32.10.20 UBX-CFG-NAVX5 (0x06 0x23)...................................................................................... 219

32.10.21 UBX-CFG-NMEA (0x06 0x17)........................................................................................ 226

32.10.22 UBX-CFG-ODO (0x06 0x1E).......................................................................................... 234

32.10.23 UBX-CFG-PM2 (0x06 0x3B).......................................................................................... 235

32.10.24 UBX-CFG-PMS (0x06 0x86).......................................................................................... 242

32.10.25 UBX-CFG-PRT (0x06 0x00).......................................................................................... 243

32.10.26 UBX-CFG-PWR (0x06 0x57)......................................................................................... 253

32.10.27 UBX-CFG-RATE (0x06 0x08)........................................................................................ 254

32.10.28 UBX-CFG-RINV (0x06 0x34)......................................................................................... 255

32.10.29 UBX-CFG-RST (0x06 0x04).......................................................................................... 256

32.10.30 UBX-CFG-RXM (0x06 0x11)........................................................................................... 258

32.10.31 UBX-CFG-SBAS (0x06 0x16)......................................................................................... 259

32.10.32 UBX-CFG-SENIF (0x06 0x88)....................................................................................... 261

32.10.33 UBX-CFG-SLAS (0x06 0x8D)....................................................................................... 262

32.10.34 UBX-CFG-SMGR (0x06 0x62)....................................................................................... 263

32.10.35 UBX-CFG-SPT (0x06 0x64)........................................................................................... 266

32.10.36 UBX-CFG-TMODE2 (0x06 0x3D)................................................................................. 266

32.10.37 UBX-CFG-TMODE3 (0x06 0x71)................................................................................... 268

32.10.38 UBX-CFG-TP5 (0x06 0x31)............................................................................................ 270

32.10.39 UBX-CFG-TXSLOT (0x06 0x53)................................................................................... 274

32.10.40 UBX-CFG-USB (0x06 0x1B)........................................................................................... 275

32.11 UBX-ESF (0x10).......................................................................................................................... 277

32.11.1 UBX-ESF-ALG (0x10 0x14)................................................................................................. 277

32.11.2 UBX-ESF-INS (0x10 0x15)................................................................................................. 278

32.11.3 UBX-ESF-MEAS (0x10 0x02)........................................................................................... 280

32.11.4 UBX-ESF-RAW (0x10 0x03).............................................................................................. 281

32.11.5 UBX-ESF-STATUS (0x10 0x10)....................................................................................... 282

32.12 UBX-HNR (0x28)....................................................................................................................... 286

32.12.1 UBX-HNR-ATT (0x28 0x01).............................................................................................. 286

32.12.2 UBX-HNR-INS (0x28 0x02).............................................................................................. 287

32.12.3 UBX-HNR-PVT (0x28 0x00)............................................................................................ 288

UBX-13003221 - R28 Early production information Page xi

## Page 12

u-blox 8 / u-blox M8 Receiver description - Manual

32.13 UBX-INF (0x04).......................................................................................................................... 291

32.13.1 UBX-INF-DEBUG (0x04 0x04).......................................................................................... 291

32.13.2 UBX-INF-ERROR (0x04 0x00)......................................................................................... 291

32.13.3 UBX-INF-NOTICE (0x04 0x02)........................................................................................ 292

32.13.4 UBX-INF-TEST (0x04 0x03)............................................................................................ 292

32.13.5 UBX-INF-WARNING (0x04 0x01).................................................................................... 293

32.14 UBX-LOG (0x21)......................................................................................................................... 294

32.14.1 UBX-LOG-BATCH (0x21 0x11).......................................................................................... 294

32.14.2 UBX-LOG-CREATE (0x21 0x07)...................................................................................... 297

32.14.3 UBX-LOG-ERASE (0x21 0x03)........................................................................................ 298

32.14.4 UBX-LOG-FINDTIME (0x21 0x0E).................................................................................. 298

32.14.5 UBX-LOG-INFO (0x21 0x08)............................................................................................ 300

32.14.6 UBX-LOG-RETRIEVEBATCH (0x21 0x10)..................................................................... 302

32.14.7 UBX-LOG-RETRIEVEPOSEXTRA (0x21 0x0f).............................................................. 303

32.14.8 UBX-LOG-RETRIEVEPOS (0x21 0x0b).......................................................................... 303

32.14.9 UBX-LOG-RETRIEVESTRING (0x21 0x0d)................................................................... 304

32.14.10 UBX-LOG-RETRIEVE (0x21 0x09)................................................................................ 305

32.14.11 UBX-LOG-STRING (0x21 0x04)...................................................................................... 306

32.15 UBX-MGA (0x13)........................................................................................................................ 307

32.15.1 UBX-MGA-ACK (0x13 0x60)............................................................................................. 307

32.15.2 UBX-MGA-ANO (0x13 0x20)........................................................................................... 308

32.15.3 UBX-MGA-BDS (0x13 0x03)............................................................................................ 309

32.15.4 UBX-MGA-DBD (0x13 0x80)............................................................................................. 313

32.15.5 UBX-MGA-FLASH (0x13 0x21)......................................................................................... 314

32.15.6 UBX-MGA-GAL (0x13 0x02)............................................................................................. 316

32.15.7 UBX-MGA-GLO (0x13 0x06)............................................................................................ 320

32.15.8 UBX-MGA-GPS (0x13 0x00)............................................................................................ 323

32.15.9 UBX-MGA-INI (0x13 0x40)............................................................................................... 328

32.15.10 UBX-MGA-QZSS (0x13 0x05)........................................................................................ 334

32.16 UBX-MON (0x0A)...................................................................................................................... 338

32.16.1 UBX-MON-BATCH (0x0A 0x32)...................................................................................... 338

32.16.2 UBX-MON-GNSS (0x0A 0x28)........................................................................................ 339

32.16.3 UBX-MON-HW2 (0x0A 0x0B).......................................................................................... 341

32.16.4 UBX-MON-HW (0x0A 0x09)............................................................................................ 342

32.16.5 UBX-MON-IO (0x0A 0x02)............................................................................................... 343

32.16.6 UBX-MON-MSGPP (0x0A 0x06)..................................................................................... 344

32.16.7 UBX-MON-PATCH (0x0A 0x27)...................................................................................... 344

32.16.8 UBX-MON-RXBUF (0x0A 0x07)...................................................................................... 346

32.16.9 UBX-MON-RXR (0x0A 0x21)............................................................................................ 346

32.16.10 UBX-MON-SMGR (0x0A 0x2E)..................................................................................... 347

32.16.11 UBX-MON-SPT (0x0A 0x2F).......................................................................................... 350

32.16.12 UBX-MON-TXBUF (0x0A 0x08).................................................................................... 354

32.16.13 UBX-MON-VER (0x0A 0x04)......................................................................................... 355

UBX-13003221 - R28 Early production information Page xii

## Page 13

u-blox 8 / u-blox M8 Receiver description - Manual

32.17 UBX-NAV (0x01)......................................................................................................................... 357

32.17.1 UBX-NAV-AOPSTATUS (0x01 0x60).............................................................................. 357

32.17.2 UBX-NAV-ATT (0x01 0x05)............................................................................................. 358

32.17.3 UBX-NAV-CLOCK (0x01 0x22)........................................................................................ 359

32.17.4 UBX-NAV-COV (0x01 0x36)............................................................................................. 359

32.17.5 UBX-NAV-DGPS (0x01 0x31)........................................................................................... 360

32.17.6 UBX-NAV-DOP (0x01 0x04).............................................................................................. 361

32.17.7 UBX-NAV-EELL (0x01 0x3d)............................................................................................ 362

32.17.8 UBX-NAV-EOE (0x01 0x61).............................................................................................. 363

32.17.9 UBX-NAV-GEOFENCE (0x01 0x39)................................................................................ 363

32.17.10 UBX-NAV-HPPOSECEF (0x01 0x13)............................................................................. 364

32.17.11 UBX-NAV-HPPOSLLH (0x01 0x14)................................................................................ 365

32.17.12 UBX-NAV-NMI (0x01 0x28)............................................................................................ 367

32.17.13 UBX-NAV-ODO (0x01 0x09)........................................................................................... 370

32.17.14 UBX-NAV-ORB (0x01 0x34)............................................................................................. 371

32.17.15 UBX-NAV-POSECEF (0x01 0x01).................................................................................. 374

32.17.16 UBX-NAV-POSLLH (0x01 0x02).................................................................................... 374

32.17.17 UBX-NAV-PVT (0x01 0x07)............................................................................................ 375

32.17.18 UBX-NAV-RELPOSNED (0x01 0x3C)............................................................................ 379

32.17.19 UBX-NAV-RESETODO (0x01 0x10)............................................................................... 381

32.17.20 UBX-NAV-SAT (0x01 0x35)........................................................................................... 381

32.17.21 UBX-NAV-SBAS (0x01 0x32)......................................................................................... 383

32.17.22 UBX-NAV-SLAS (0x01 0x42)......................................................................................... 385

32.17.23 UBX-NAV-SOL (0x01 0x06)........................................................................................... 386

32.17.24 UBX-NAV-STATUS (0x01 0x03)................................................................................... 388

32.17.25 UBX-NAV-SVINFO (0x01 0x30).................................................................................... 390

32.17.26 UBX-NAV-SVIN (0x01 0x3B)......................................................................................... 392

32.17.27 UBX-NAV-TIMEBDS (0x01 0x24)................................................................................. 393

32.17.28 UBX-NAV-TIMEGAL (0x01 0x25)................................................................................. 394

32.17.29 UBX-NAV-TIMEGLO (0x01 0x23)................................................................................. 395

32.17.30 UBX-NAV-TIMEGPS (0x01 0x20)................................................................................. 397

32.17.31 UBX-NAV-TIMELS (0x01 0x26)..................................................................................... 398

32.17.32 UBX-NAV-TIMEUTC (0x01 0x21).................................................................................. 400

32.17.33 UBX-NAV-VELECEF (0x01 0x11)................................................................................... 401

32.17.34 UBX-NAV-VELNED (0x01 0x12).................................................................................... 402

32.18 UBX-RXM (0x02)....................................................................................................................... 403

32.18.1 UBX-RXM-IMES (0x02 0x61)........................................................................................... 403

32.18.2 UBX-RXM-MEASX (0x02 0x14)....................................................................................... 406

32.18.3 UBX-RXM-PMREQ (0x02 0x41)...................................................................................... 407

32.18.4 UBX-RXM-RAWX (0x02 0x15)......................................................................................... 409

32.18.5 UBX-RXM-RLM (0x02 0x59)............................................................................................ 416

32.18.6 UBX-RXM-RTCM (0x02 0x32)......................................................................................... 418

32.18.7 UBX-RXM-SFRBX (0x02 0x13)......................................................................................... 419

UBX-13003221 - R28 Early production information Page xiii

## Page 14

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.8 UBX-RXM-SVSI (0x02 0x20)............................................................................................ 421

32.19 UBX-SEC (0x27)........................................................................................................................ 423

32.19.1 UBX-SEC-UNIQID (0x27 0x03)........................................................................................ 423

32.20 UBX-TIM (0x0D)....................................................................................................................... 424

32.20.1 UBX-TIM-DOSC (0x0D 0x11)............................................................................................ 424

32.20.2 UBX-TIM-FCHG (0x0D 0x16).......................................................................................... 424

32.20.3 UBX-TIM-HOC (0x0D 0x17)............................................................................................. 425

32.20.4 UBX-TIM-SMEAS (0x0D 0x13)....................................................................................... 426

32.20.5 UBX-TIM-SVIN (0x0D 0x04)........................................................................................... 428

32.20.6 UBX-TIM-TM2 (0x0D 0x03)........................................................................................... 429

32.20.7 UBX-TIM-TOS (0x0D 0x12)............................................................................................. 430

32.20.8 UBX-TIM-TP (0x0D 0x01)................................................................................................ 432

32.20.9 UBX-TIM-VCOCAL (0x0D 0x15)..................................................................................... 434

32.20.10 UBX-TIM-VRFY (0x0D 0x06)........................................................................................ 437

32.21 UBX-UPD (0x09)........................................................................................................................ 438

32.21.1 UBX-UPD-SOS (0x09 0x14).............................................................................................. 438

33 RTCM Protocol........................................................................................................................................ 441

33.1 RTCM2........................................................................................................................................... 441

33.1.1 Introduction........................................................................................................................... 441

33.1.2 Supported Messages.......................................................................................................... 441

33.1.3 Configuration........................................................................................................................ 441

33.1.4 Output.................................................................................................................................... 441

33.1.5 Restrictions.......................................................................................................................... 442

33.1.6 Reference.............................................................................................................................. 442

33.2 RTCM version 3.......................................................................................................................... 442

33.2.1 Introduction.......................................................................................................................... 442

33.2.2 Supported Messages........................................................................................................ 443

33.2.3 u-blox Proprietary RTCM Messages............................................................................... 444

33.2.4 Configuration...................................................................................................................... 444

33.2.5 Output.................................................................................................................................. 445

33.2.6 Reference............................................................................................................................. 445

Appendix............................................................................................................................................................... 446

A Satellite Numbering................................................................................................................................ 446

B UBX and NMEA Signal Identifiers....................................................................................................... 446

C u-blox 8 / u-blox M8 Default Settings............................................................................................... 446

C.1 Antenna Supervisor Settings (UBX-CFG-ANT)...................................................................... 446

C.2 Data Batching Settings (UBX-CFG-BATCH)........................................................................... 447

C.3 Datum Settings (UBX-CFG-DAT).............................................................................................. 447

C.4 Geofencing Settings (UBX-CFG-GEOFENCE)........................................................................ 447

C.5 High Navigation Rate Settings (UBX-CFG-HNR)................................................................... 448

C.6 GNSS System Settings (UBX-CFG-GNSS)............................................................................. 448

C.7 INF Messages Settings (UBX-CFG-INF).................................................................................. 448

C.7.1 UBX Protocol........................................................................................................................... 448

UBX-13003221 - R28 Early production information Page xiv

## Page 15

u-blox 8 / u-blox M8 Receiver description - Manual

C.7.2 NMEA Protocol...................................................................................................................... 449

C.8 Jammer/Interference Monitor Settings (UBX-CFG-ITFM)................................................. 449

C.9 Logging Settings (UBX-CFG-LOGFILTER).............................................................................. 449

C.10 Navigation Settings (UBX-CFG-NAV5).................................................................................. 449

C.11 Navigation Settings (UBX-CFG-NAVX5)................................................................................ 450

C.12 NMEA Protocol Settings (UBX-CFG-NMEA)......................................................................... 452

C.13 Odometer Settings (UBX-CFG-ODO)...................................................................................... 452

C.14 Power Management 2 Configuration (UBX-CFG-PM2)....................................................... 452

C.15 Port Configuration (UBX-CFG-PRT)....................................................................................... 453

C.15.1 UART Port Configuration................................................................................................... 453

C.15.2 USB Port Configuration..................................................................................................... 453

C.15.3 SPI Port Configuration....................................................................................................... 454

C.15.4 DDC Port Configuration..................................................................................................... 454

C.16 Output Rate Settings (UBX-CFG-RATE)............................................................................... 454

C.17 Remote Inventory Settings (UBX-CFG-RINV)....................................................................... 455

C.18 Receiver Manager Configuration Settings (UBX-CFG-RXM)............................................ 455

C.19 SBAS Configuration Settings (UBX-CFG-SBAS)................................................................. 455

C.20 Timepulse Settings (UBX-CFG-TP5)..................................................................................... 456

C.21 USB Settings (UBX-CFG-USB)................................................................................................. 456

Related Documents........................................................................................................................................... 457

Overview........................................................................................................................................................... 457

Revision History................................................................................................................................................. 458

Contact................................................................................................................................................................. 460

u-blox Offices................................................................................................................................................. 460

UBX-13003221 - R28 Early production information Page xv

## Page 16

u-blox 8 / u-blox M8 Receiver description - Manual

Preface

1 Document Overview

The interface description including receiver description is an important resource for integrating

and configuring u-blox receivers. This document has a modular structure and it is not necessary

to read it from the beginning to the end. There are two main sections: The Receiver Description

and the Interface Description.

The Receiver Description describes the software aspects of system features and configuration of

u-blox receivers. The Receiver Description is structured according to areas of functionality, with

links provided to the corresponding NMEA and UBX messages, which are described in the

Interface Description.

The Interface Description is a reference describing the messages used by the u-blox receiver and

is organized by the specific NMEA, UBX, and RTCM messages.

This document provides general information on u-blox receivers. Some information

might not apply to certain products. Refer to the product data sheet and/or integration

manual for possible restrictions or limitations.

2 Firmware and Protocol Versions

The protocol version defines a set of messages that are applicable across various u-blox products.

Each firmware used by a u-blox receiver supports a specific protocol version, which is not

configurable.

The following sections will explain how to decode the shown information to get the firmware and

the protocol version.

2.1 How to Determine the Version and the Location of the Firmware

The u-blox receiver contains a firmware in two different locations:

• Internal ROM

• External flash memory

The location and the version of the currently running firmware can be found in the boot screen or

in the UBX-MON-VER message.

For firmware supporting Protocol Version 17 and below:

• Boot screen, Protocol Version 17 and below

• UBX-MON-VER, Protocol Version 17 and below

For firmware supporting Protocol Version from 18 to 23.01:

• Boot screen, Protocol Version from 18 to 23.01

• UBX-MON-VER, Protocol Version 18 to 23.01

2.1.1 Decoding the Boot Screen (for Protocol Version 17 and Below)

Boot screen for a u-blox receiver running from ROM:

UBX-13003221 - R28 Early production information Page 1 of 460

## Page 17

u-blox 8 / u-blox M8 Receiver description - Manual

Boot screen for a u-blox receiver running from flash:

Not every line is output by every u-blox receiver in the boot screen. This depends on the

product, the firmware location and the firmware version.

Possible lines in the boot screen and their meanings:

Entry Description

u-blox AG - www.u-blox.com Start of the boot screen

HW UBX-M80xx 00800000 Hardware version of the u-blox receiver (u-blox M8 receiver)

ROM CORE 2.01 (75331) Firmware version 2.01 running from ROM (revision number)

Oct 29 2013 13:28:17 compilation date/time

EXT CORE 2.01 (75350) Firmware version 2.01 running from flash (revision number)

Oct 29 2013 16:15:41 compilation date/time

ROM BASE 2.01 (75331) Underlying firmware version 2.01 in ROM (revision number)

Oct 29 2013 13:28:17 compilation date/time

PROTVER 15.00 Supported protocol version

GNSS OTP: GPS GLO, Default Major GNSS selection.

SEL: GPS GLO Current Major GNSS selection.

ANTSUPERV=AC SD PDoS SR Configuration of the Antenna supervisor where

AC: Active Antenna Control enabled

SD: Short Circuit Detection enabled

OD: Open Circuit Detection enabled

PDoS: Short Circuit Power Down Logic enabled

SR: Automatic Recovery from Short state

LLC FFFFFFFF-FF7F7C3F- Low-level configuration of the u-blox receiver.

FFFFFF96-FFFFFFFF-FFFFFF79

FIS 0xEF4015 (100111) found Flash Information Structure (FIS) file for flash memory with

JEDEC 0xEF4015 found in the external flash memory. Revision

number of the file is indicated in brackets.

UBX-13003221 - R28 Early production information Page 2 of 460

## Page 18

u-blox 8 / u-blox M8 Receiver description - Manual

Possible lines in the boot screen and their meanings: continued

Entry Description

RF0 dev ok RF channel 0 configured correctly.

The line containing the CORE indicates which version of the firmware is currently running.

The firmware is running either from ROM (indicated with ROM CORE) or from external

flash memory (indicated with EXT CORE).

The line containing the CORE is called firmware string in the rest of the document.

2.1.2 Decoding the Boot Screen (for Protocol Version from 18 to 23.01)

Boot screen for a u-blox receiver running from ROM:

Boot screen for a u-blox receiver running from flash:

Not every line is output by every u-blox receiver in the boot screen. This depends on the

product, the firmware location and the firmware version.

Possible lines in the boot screen and their meanings:

Entry Description

u-blox AG - www.u-blox.com Start of the boot screen

HW UBX-M8030 00800000 Hardware version of the u-blox receiver (u-blox M8 receiver)

HW UBX-G8020 00800000 Hardware version of the u-blox receiver (u-blox 8 receiver)

ROM CORE 3.01 (107888) Firmware version 3.01 running from ROM (revision number)

EXT CORE 3.01 (107900) Firmware version 3.01 running from flash (revision number)

ROM BASE 3.01 (107888) Underlying firmware version 3.01 in ROM (revision number)

UBX-13003221 - R28 Early production information Page 3 of 460

## Page 19

u-blox 8 / u-blox M8 Receiver description - Manual

Possible lines in the boot screen and their meanings: continued

Entry Description

FWVER=SPG 3.01 Firmware of product category and version where

SPG: Firmware of Standard Precision GNSS product

HPG: Firmware of High Precision GNSS product

ADR: Firmware of ADR product

UDR: Firmware of UDR product

TIM: Firmware of Time Sync product

FTS: Firmware of Time & Frequency Sync product

PROTVER=18.00 Supported protocol version

MOD=NEO-M8N-0 Module identification. Set in production.

FIS=0xEF4015 (100111) Flash Information Structure (FIS) file for flash memory with

JEDEC 0xEF4015 found in the external flash memory. Revision

number of the file is indicated in brackets.

GPS;GLO;GAL;BDS Supported Major GNSS.

SBAS;IMES;QZSS Supported Augmentation systems.

GNSS OTP=GPS;GLO Default Major GNSS selection.

LLC FFFFFFFF-FFFFFFFF- Low-level configuration of the u-blox receiver.

FFFFFFFF-FFFFFFFF-FFCFFFFF

ANTSUPERV=AC SD PDoS SR Configuration of the Antenna supervisor where

AC: Active Antenna Control enabled

SD: Short Circuit Detection enabled

OD: Open Circuit Detection enabled

PDoS: Short Circuit Power Down Logic enabled

SR: Automatic Recovery from Short state

PF=3FF Product configuration.

The line containing the FWVER indicates which version of the firmware is currently

running and is called firmware version in the rest of the document.

The numbers in parentheses (revision numbers) should only be used to identify a known

firmware version and are not guaranteed to increase over time.

2.1.3 Decoding the output of UBX-MON-VER (for Protocol Version 17 and below)

UBX-13003221 - R28 Early production information Page 4 of 460

## Page 20

u-blox 8 / u-blox M8 Receiver description - Manual

Possible fields in UBX-MON-VER and their meanings:

Entry Description

Software Version Currently running firmware version.

If no firmware version is shown in the first line of Extension(s),

then the u-blox receiver runs from ROM.

If a firmware version is shown in the first line of Extension(s),

then the u-blox receiver runs from flash.

Hardware Version The hardware version of the u-blox receiver.

Extension(s) Extended information about the u-blox receiver firmware. See

table below for the entries.

Not every entry is output by every u-blox receiver in the UBX-MON-VER extensions. This

depends on the product, the firmware location and the firmware version.

Possible entries in UBX-MON-VER Extension(s):

Entry Description

2.01 (75331) Underlying firmware version in ROM.

If such an entry is present, then the u-blox receiver runs from

flash.

PROTVER 15.00 Supported protocol version.

FIS 0xEF4015 (100111) Flash Information Structure (FIS) file for flash memory with

JEDEC 0xEF4015 found in the external flash memory. Revision

number of the file is indicated in brackets.

MOD NEO-M8N-0 Module identification. Set in production.

GPS;SBAS;GLO;BDS;QZSS Supported GNSS.

UBX-13003221 - R28 Early production information Page 5 of 460

## Page 21

u-blox 8 / u-blox M8 Receiver description - Manual

2.1.4 Decoding the output of UBX-MON-VER (for Protocol Version from 18 and 23.01)

Possible fields in UBX-MON-VER and their meanings:

Entry Description

Software Version Currently running firmware version.

ROM CORE 3.01 (107888) If ROM CORE, then the u-blox receiver runs from ROM.

EXT CORE 3.01 (107900) If EXT CORE, then the u-blox receiver runs from Flash.

Hardware Version The hardware version of the u-blox receiver.

Extension(s) Extended information about the u-blox receiver firmware. See

table below for the entries.

Not every entry is output by every u-blox receiver in the UBX-MON-VER extensions. This

depends on the product, the firmware location and the firmware version.

Possible entries in UBX-MON-VER Extension(s):

Entry Description

ROM BASE 3.01 (107888) Underlying firmware version in ROM.

If such an entry is present, then the u-blox receiver runs from

flash.

FWVER=SPG 3.01 Firmware of product category and version where

SPG: Firmware of Standard Precision GNSS product

HPG: Firmware of High Precision GNSS product

ADR: Firmware of ADR product

UDR: Firmware of UDR product

TIM: Firmware of Time Sync product

FTS: Firmware of Time & Frequency Sync product

PROTVER=18.00 Supported protocol version.

MOD=NEO-M8N-0 Module identification. Set in production.

UBX-13003221 - R28 Early production information Page 6 of 460

## Page 22

u-blox 8 / u-blox M8 Receiver description - Manual

Possible entries in UBX-MON-VER Extension(s): continued

Entry Description

FIS=0xEF4015 (100111) Flash Information Structure (FIS) file for flash memory with

JEDEC 0xEF4015 found in the external flash memory. Revision

number of the file is indicated in brackets.

GPS;GLO;GAL;BDS Supported Major GNSS.

SBAS;IMES;QZSS Supported Augmentation systems.

2.2 How to Determine the Supported Protocol Version of the u-blox Receiver

Each u-blox receiver reports its supported protocol version in the following ways:

• On start-up in the boot screen

• In the UBX-MON-VER message

with the line containing PROTVER (example: PROTVER=18.00).

Additionally, the firmware string, together with the firmware version, can be used to look up the

corresponding protocol version. The tables below give an overview of the released firmware and

their corresponding protocol versions.

2.2.1 u-blox 8 / u-blox M8 Firmware and Supported Protocol Versions

Firmware for Standard Precision GNSS products

Firmware version Firmware string Protocol Version

SPG 2.01 ROM CORE 2.01 (75331) Oct 29 2013 13:28:17 15.00

SPG 2.01 EXT CORE 2.01 (75350) Oct 29 2013 16:15:41 15.00

SPG 3.01 ROM CORE 3.01 (107888) 18.00

SPG 3.01 EXT CORE 3.01 (107900) 18.00

SPG 3.05 EXT CORE 3.05 (a5d3549) 18.00

SPG 3.50 EXT CORE 3.50 (190461) 23.00

SPG 3.51 ROM CORE 3.51 (19dc23) 23.01

SPG 3.51 EXT CORE 3.51 (19dc23) 23.01

Firmware for High Precision GNSS Products

Firmware version Firmware string Protocol Version

HPG 1.00 EXT CORE 3.01 (111160) 20.00

HPG 1.11 EXT CORE 3.01 (b8bc67) 20.01

HPG 1.20 EXT CORE 3.01 (d34ed4) 20.10

HPG 1.30 EXT CORE 3.01 (d080e3) 20.20

HPG 1.40 EXT CORE 3.01 (db0c89) 20.30

HPG 1.43 EXT CORE 3.05 (ff96ba) 20.30

Firmware for Dead Reckoning products

Firmware version Firmware string Protocol Version

ADR 3.00 EXT CORE 2.01 (77076) Dec 18 2013 09:40:24 ADR 3.00 15.00

ADR 3.10 EXT CORE 2.01 (87683) Nov 21 2014 14:03:10 ADR 3.10 15.01

M8L

ADR 3.11 EXT CORE 2.01 (89981) Jan 20 2015 17:22:06 ADR 3.11 15.01

M8L

ADR 4.00 EXT CORE 3.01 (16559bf) Apr 21 2016 15:49:07 ADR 4.00 19.00

UBX-13003221 - R28 Early production information Page 7 of 460

## Page 23

u-blox 8 / u-blox M8 Receiver description - Manual

Firmware for Dead Reckoning products continued

Firmware version Firmware string Protocol Version

ADR 4.10 EXT CORE 3.01 (c0c787c) Apr 24 2017 17:31:42 ADR 4.10 19.10

ADR 4.11 EXT CORE 3.01 (d189ff) Aug 22 2017 14:40:05 ADR 4.11 19.10

ADR 4.21 EXT CORE 3.01 (3620e2) 19.20

ADR 4.31 EXT CORE 3.01 (e3981c) 19.20

ADR 4.50 EXT CORE 3.01 (86c0ce) 19.20

UDR 1.00 EXT CORE 3.01 (16559bf) Apr 21 2016 15:50:59 UDR 1.00 19.00

UDR 1.21 EXT CORE 3.01 (3620e2) 19.20

UDR 1.31 EXT CORE 3.01 (e3981c) 19.20

UDR 1.50 EXT CORE 3.01 (86c0ce) 19.20

Firmware for Timing products

Firmware version Firmware string Protocol Version

FTS 1.01 EXT CORE 2.20 (81289) May 14 2014 14:11:24 16.00

TIM 1.00 EXT CORE 2.30 (85522) Sep 29 2014 09:40:12 17.00

TIM 1.01 EXT CORE 2.30 (86283) Oct 20 2014 13:51:49 17.00

TIM 1.02 EXT CORE 2.30 (93796) Apr 8 2015 15:53:38 17.00

TIM 1.10 EXT CORE 3.01 (111141) 22.00

TIM 1.11 EXT CORE 3.01 (29b2c9) 22.01

TIM 1.12 EXT CORE 3.01 (3e7a32) 22.01

UBX-13003221 - R28 Early production information Page 8 of 460

## Page 24

u-blox 8 / u-blox M8 Receiver description - Manual

Receiver Description

3 Receiver Configuration

3.1 Configuration Concept

u-blox receivers are fully configurable with UBX protocol configuration messages (message class

UBX-CFG). The configuration used by the u-blox receiver during normal operation is called

"Current Configuration". The Current Configuration can be changed during normal operation by

sending any UBX-CFG-XXX message to the u-blox receiver over an I/O port. The u-blox receiver will

change its Current Configuration immediately after receiving the configuration message. The u-

blox receiver always uses only the Current Configuration.

Unless the Current Configuration is made permanent by using UBX-CFG-CFG as described below,

the Current Configuration will be lost when there is:

• a power cycle

• a hardware reset

• a (complete) controlled software reset

See the section on resetting a u-blox receiver for details.

The Current Configuration can be made permanent (stored in a non-volatile memory) by saving it

to the "Permanent Configuration". This is done by sending a UBX-CFG-CFG message with an

appropriate saveMask (UBX-CFG-CFG/save).

The Permanent Configuration is copied to the Current Configuration during start-up or when a

UBX-CFG-CFG message with an appropriate loadMask (UBX-CFG-CFG/load) is sent to the u-blox

receiver.

The Permanent Configuration can be restored to the u-blox receiver's Default Configuration by

sending a UBX-CFG-CFG message with an appropriate clearMask (UBX-CFG-CFG/clear) to the u-

blox receiver. This only replaces the Permanent Configuration, not the Current Configuration. To

make the u-blox receiver operate with the Default Configuration which was restored to the

Permanent Configuration, a UBX-CFG-CFG/load command must be sent or the u-blox receiver

must be reset.

The mentioned masks (saveMask, loadMask, clearMask) are 4-byte bitfields. Every bit represents

one configuration sub-section. These sub-sections are defined in section "Organization of the

Configuration Sections". All three masks are part of every UBX-CFG-CFG message. Save, load and

clear commands can be combined in the same message. Order of execution is: clear, save, load.

The following diagram illustrates the process:

UBX-13003221 - R28 Early production information Page 9 of 460

## Page 25

u-blox 8 / u-blox M8 Receiver description - Manual

It is possible to change the current communications port settings using a UBX-CFG-CFG message.

This could affect baud rate and other transmission parameters. Because there may be messages

queued for transmission there may be uncertainty about which protocol applies to such

messages. In addition a message currently in transmission may be corrupted by a protocol

change. Host data reception parameters may have to be changed to be able to receive future

messages, including the acknowledge message associated with the UBX-CFG-CFG message.

3.2 Organization of the Configuration Sections

The configuration is divided into several sub-sections. Each of these sub-sections corresponds to

one or several UBX-CFG-XXX messages. The sub-section numbers in the following tables

correspond to the bit position in the masks mentioned above. All values not listed are reserved.

Configuration sub-sections

Number Name CFG messages Description

0 PRT UBX-CFG-PRT Port and USB settings

UBX-CFG-USB

1 MSG UBX-CFG-MSG Message settings (enable/disable, update rate)

2 INF UBX-CFG-INF Information output settings (Errors, Warnings, Notice,

Test etc.)

3 NAV UBX-CFG-NAV5 Settings for Navigation Parameters, Receiver Datum,

UBX-CFG-NAVX5 Measurement and Navigation Rate, SBAS, NMEA

UBX-CFG-DAT protocol and Time mode (Timing products only)

UBX-CFG-RATE

UBX-CFG-SBAS

UBX-CFG-NMEA

UBX-CFG-TMODE2

4 RXM UBX-CFG-GNSS GNSS Settings, Power Mode Settings, Time Pulse

UBX-CFG-TP5 Settings, Jamming/Interference Monitor Settings

UBX-CFG-RXM

UBX-CFG-PM2

UBX-CFG-ITFM

9 RINV UBX-CFG-RINV Remote Inventory configuration

10 ANT UBX-CFG-ANT Antenna configuration

11 LOG UBX-CFG- Logging configuration

LOGFILTER

UBX-13003221 - R28 Early production information Page 10 of 460

## Page 26

u-blox 8 / u-blox M8 Receiver description - Manual

Configuration sub-sections continued

Number Name CFG messages Description

12 FTS UBX-CFG-DOSC Disciplining configuration. Only applicable to the Time &

UBX-CFG-ESRC Frequency Sync product.

UBX-CFG-SMGR

3.3 Permanent Configuration Storage Media

The Current Configuration is stored in the volatile RAM of the u-blox receiver. Hence, any changes

made to the Current Configuration without saving will be lost if any of the reset events listed in

the section above occur. By using UBX-CFG-CFG/save, the selected configuration sub-sections are

saved to all non-volatile memories available:

• On-chip BBR (battery backed RAM). In order for the BBR to work, a backup battery must be

applied to the u-blox receiver.

• External flash memory, where available.

3.4 u-blox Receiver Default Configuration

The Permanent Configuration can be reset to Default Configuration through a UBX-CFG-CFG/clear

message. The Default Configuration of the u-blox receiver is normally determined when the u-blox

receiver is manufactured. Refer to specific product data sheet for further details.

3.5 Save-on-Shutdown Feature

The save-on-shutdown feature (SOS) enables the u-blox receiver to store the contents of the

battery-backed RAM to an external flash memory and restore it upon startup. This allows the u-

blox receiver to preserve some of the features available only with a battery backup (preserving

configuration and satellite orbit knowledge) without having a battery backup supply present. It

does not, however, preserve any kind of time knowledge. The save-on-shutdown must be

commanded by the host. The restore-on-startup is automatically done if the corresponding data

is present in the flash. No expiration check of the data is done.

The following outlines the suggested shutdown procedure when using the save-on-shutdown

feature:

• With the UBX-CFG-RST message, the host commands the u-blox receiver to stop, specifying

reset mode 0x08 ("Controlled GNSS stop") and a BBR mask of 0 ("Hotstart").

• The u-blox receiver confirms the reception of a valid / invalid request with a UBX-ACK-ACK / UBX-

ACK-NAK message.

• The host commands the saving of the contents of BBR to the flash memory using the UBX-UPD-

SOS-BACKUP message.

• The u-blox receiver confirms the reception of a valid / invalid request with a UBX-ACK-ACK / UBX-

ACK-NAK message.

• For a valid request the u-blox receiver reports on the success of the backup operation with a UBX-

UPD-SOS-ACK message.

• The host powers off the u-blox receiver.

Do not expect UBX-CFG-RST and UBX-UPD-SOS-BACKUP message to be acknowledged

with a UBX-ACK-ACK / UBX-ACK-NAK message by the receiver with newer FW versions.

And consequently the startup procedure is as follows:

UBX-13003221 - R28 Early production information Page 11 of 460

## Page 27

u-blox 8 / u-blox M8 Receiver description - Manual

• The host powers on the u-blox receiver.

• The u-blox receiver detects the previously stored data in flash. It restores the corresponding

memory and reports the success of the operation with a UBX-UPD-SOS-RESTORED message on

the port where it had received the save command message (if the output protocol filter on that

port allows it). It does not report anything if no stored data has been detected.

• Additionally the u-blox receiver outputs a UBX-INF-NOTICE and/or a NMEA-TXT message with

the contents RESTORED in the boot screen (depends on port and information messages

configuration) upon success.

• Optionally the host can deliver coarse time assistance using UBX-MGA-INI-TIME_UTC for better

startup performance.

Once the u-blox receiver has started up it is suggested to delete the stored data using a UBX-UPD-

SOS-CLEAR message. The u-blox receiver responds with a UBX-ACK-ACK or UBX-ACK-NAK message.

Note that this feature must not be used with power save mode and that saved data

must be deleted before switching to that mode.

4 Concurrent GNSS

Many u-blox positioning modules and chips are multi-GNSS receivers capable of receiving and

processing signals from multiple Global Navigation Satellite Systems (GNSS).

u-blox concurrent GNSS receivers are multi-GNSS receivers that can acquire and track satellites

from more than one GNSS system at the same time, and utilize them in positioning.

4.1 GNSS Types

u-blox receivers support a wide range of different GNSS. Some GNSS have large numbers of

satellites deployed globally and therefore are generally capable of providing navigation solutions

on their own. u-blox designates these as "major GNSS". By contrast, some are designed to be used

to enhance the use of one or more major GNSS and u-blox designates these "augmentation

systems".

In many cases, such as Satellite Numbering, this distinction does not matter as u-blox receivers

generally try to combine information from all available GNSS to create the best possible

navigation information. However, particularly in relation to configuring the receiver, the distinction

can be important.

4.1.1 Major GNSS

The major GNSS supported by u-blox receivers are described below.

4.1.1.1 GPS

The Global Positioning System (GPS) is a GNSS operated by the US department of defense. Its

purpose is to provide position, velocity and time for civilian and defense users on a global basis.

The system currently consists of 32 medium earth orbit satellites and several ground control

stations.

4.1.1.2 GLONASS

GLONASS is a GNSS operated by Russian Federation department of defense. Its purpose is to

provide position, velocity and time for civilian and defense users on a global basis. The system

consists of 24 medium earth orbit satellites and ground control stations.

UBX-13003221 - R28 Early production information Page 12 of 460

## Page 28

u-blox 8 / u-blox M8 Receiver description - Manual

It has a number of significant differences when compared to GPS. In most cases, u-blox receivers

operate in a very similar manner when they are configured to use GLONASS signals instead of

GPS. However some aspects of receiver output are likely to be noticeably affected.

4.1.1.3 Galileo

At the time of writing (early 2018), the Galileo system was still under development with

only a few fully operational SVs. Therefore, the precise performance and reliability of u-

blox receivers when receiving Galileo signals is effectively impossible to guarantee.

Galileo is a GNSS operated by the European Union. Its purpose is to provide position, velocity and

time for civilian users on a global basis. The system is currently not fully operational. It is

eventually expected to consist of 30 medium earth orbit satellites.

On u-blox M8 receivers a maximum of ten channels can be assigned to Galileo for signal

acquisition and tracking. Note that at most eight Galileo satellites will be used for navigation. It is

recommended not to set the number of Galileo channels higher than eight in UBX-CFG-GNSS.

4.1.1.3.1 Search and Rescue Return Link Message

The receiver supports reception and output of Search and Rescue (SAR) Return Link Messages

(RLM). When enabled, a UBX-RXM-RLM message will be generated whenever an RLM is detected by

the receiver.

At the time of writing (early 2018), no live transmission of RLMs by Galileo SVs had been

observed, so the details of their use was impossible to verify completely.

4.1.1.4 BeiDou

BeiDou is a GNSS operated by China. Its purpose is to initially provide position, velocity and time

for users in Asia. In a later stage when the system is fully deployed it will have worldwide coverage.

The full system will consist of five geostationary, five inclined geosynchronous and 27 medium

earth orbit satellites, as well as control, upload and monitoring stations. Although this implies a

full constellation of 37 SVs, only SVs numbered 1 to 30 are fully supported in the D1/D2 NAV

message described by the Interface Control Document version 2.0. For SVs numbered above 30,

there is currently no almanac or differential correction. Consequently, u-blox receivers only use

BeiDou SVs numbered 1 to 30.

4.1.2 Augmentation Systems

The augmentation systems supported by u-blox receivers are described below.

4.1.2.1 SBAS

There are a number of Space Based Augmentation Systems (SBAS) operated by different

countries using geostationary satellites. u-blox receivers currently support the following:

• WAAS (Wide Area Augmentation System) operated by the US.

• EGNOS (European Geostationary Navigation Overlay Service) operated by the EU.

• MSAS (Multi-functional Satellite Augmentation System) operated by Japan.

• GAGAN (GPS Aided Geo Augmented Navigation) operated by India.

See section SBAS for more details.

UBX-13003221 - R28 Early production information Page 13 of 460

## Page 29

u-blox 8 / u-blox M8 Receiver description - Manual

4.1.2.2 QZSS

The Quasi Zenith Satellite System (QZSS) is a regional satellite augmentation system operated

by Japan Aerospace Exploration Agency (JAXA). It is intended as an enhancement to GPS, to

increase availability and positional accuracy. The QZSS system achieves this by transmitting

GPS-compatible signals in the GPS bands.

NMEA messages will show the QZSS satellites only if configured to do so (see section Satellite

Numbering).

The QZSS L1SAIF is an additional signal broadcast by QZSS satellites that contains

augmentation and other data.

4.1.2.3 IMES

The Indoor MEssaging System (IMES) is an extension to the QZSS specification.

See section IMES for more details.

4.2 Configuration

The UBX-CFG-GNSS message allows the user to specify which GNSS signals should be processed

along with limits on how many tracking channels should be allocated to each GNSS. The receiver

will respond to such a request with a UBX-ACK-ACK message if it can support the requested

configuration or a UBX-ACK-NAK message if not.

Customers enabling BeiDou and/or Galileo who wish to use the NMEA protocol are

recommended to select NMEA version 4.1, as earlier versions have no support for these

two GNSS. See the NMEA protocol section for details on selecting NMEA versions.

The combinations of systems which can be configured simultaneously depends on the receiver's

capability to receive several carrier frequencies. The UBX-MON-GNSS message reports which major

GNSS can be selected. Refer to the data sheet of the corresponding u-blox receiver for full

information. Usually GPS, SBAS (e.g. WAAS, EGNOS, MSAS), QZSS and Galileo can be enabled

together, because they all use the 1575.42MHz L1 frequency. GLONASS and BeiDou both operate

on different frequencies, therefore the receiver must be able to receive a second or even third

carrier frequency in order to process these systems together with GPS.

It is recommended to disable GLONASS and BeiDou if a GPS-only antenna or GPS-only

SAW filter is used.

In all circumstances, it is necessary for at least one major GNSS to be enabled. It is also required

that at least 4 tracking channels are available to each enabled major GNSS, i.e. maxTrkCh must

have a minimum value of 4 for each enabled major GNSS. Further requirements on generating

configurations acceptable by the receiver can be found in UBX-CFG-GNSS.

4.2.1 Switching between GNSS

Users should be aware that switching between GNSS (and especially away from GPS) may affect

the long term accuracy of the receiver until the next cold start. In normal operation the receiver

selects the best models and corrections from the transmitted auxiliary data (e.g. UTC and

Ionospheric parameters), basing this selection on the configured GNSS. Disabling a major GNSS

prevents auxiliary data from that GNSS being refreshed and so it will become stale, resulting in

progressively degraded performance. This can occur even if the main power supply is removed, as

most receivers retain auxiliary data in non-volatile storage, e.g. battery backed RAM (BBR). For

this reason, u-blox recommends that receivers are cold started after any change that disables an

UBX-13003221 - R28 Early production information Page 14 of 460

## Page 30

u-blox 8 / u-blox M8 Receiver description - Manual

active GNSS, within a few weeks, but preferably immediately. This will ensure that the receiver

then uses only regularly refreshed information from the newly configured constellations.

4.2.2 Configuring QZSS L1SAIF

By default the receiver will be configured for QZSS L1C/A, this can be changed so the receiver can

be configured for QZSS L1SAIF also. See the table below for UBX-CFG-GNSS sigCfgMask settings

for signals on QZSS. For example, to enable QZSS L1C/A and QZSS L1SAIF, set the gnssId to 5

(for QZSS) and sigCfgMask to 0x05. If supported by the firmware, L1SAIF would then be enabled.

QZSS Signal configuration for UBX-CFG-GNSS

GnssId Description Signal mask

5 QZSS 0x01 = QZSS L1C/A

0x04 = QZSS L1SAIF

5 SBAS Configuration Settings Description

5.1 SBAS (Satellite Based Augmentation Systems)

SBAS (Satellite Based Augmentation System) is an augmentation technology for GPS, which

calculates GPS integrity and correction data with RIMS (Ranging and Integrity Monitoring

Stations) on the ground and uses geostationary satellites to broadcast GPS integrity and

correction data to GPS users. The correction data is transmitted on the GPS L1 frequency (1575.42

MHz), and therefore no additional receiver is required to make use of the correction and integrity

data.

u-blox receivers will only process corrections for GPS. Other corrections are not applied,

even if, as planned, some SBAS satellites start to transmit them (e.g. SDCM for

GLONASS).

UBX-13003221 - R28 Early production information Page 15 of 460

## Page 31

u-blox 8 / u-blox M8 Receiver description - Manual

SBAS Principle

There are several compatible SBAS systems available or in development all around the world:

• WAAS (Wide Area Augmentation System) for North America has been in operation since 2003.

• MSAS (Multi-Functional Satellite Augmentation System) for Japan has been in operation since

2007.

• EGNOS (European Geostationary Navigation Overlay Service) has been in operation since 2009.

• GAGAN (GPS Aided Geo Augmented Navigation), for India has been in operation since 2014.

• SDCM (System for Differential Corrections and Monitoring), for Russia is at the time of writing

in test mode.

Support of SBAS allows u-blox GPS technology to take full advantage of the augmentation

systems that are currently available (i.e. WAAS, EGNOS, MSAS, GAGAN). Signals from systems

currently being tested and/or planned (such as SDCM) may also work, when those systems

become fully operational, but this cannot be relied upon and u-blox receivers are not configured to

support them by default.

With SBAS enabled, the user benefits from additional satellites for ranging (navigation). u-blox

GPS technology uses the available SBAS satellites for navigation just like GPS satellites, if the

SBAS satellites offer this service.

To improve position accuracy, SBAS uses different types of correction data:

• Fast Corrections for short-term disturbances in GPS signals (due to clock problems, etc.).

• Long-term corrections for GPS clock problems, broadcast orbit errors etc.

• Ionosphere corrections for Ionosphere activity

Another benefit of SBAS is the use of GPS integrity information. In this way SBAS control stations

can 'disable' the use of GPS satellites within a 6-second alarm time in case of major GPS satellite

problems. If integrity monitoring is enabled, u-blox GPS technology only uses satellites, for which

UBX-13003221 - R28 Early production information Page 16 of 460

## Page 32

u-blox 8 / u-blox M8 Receiver description - Manual

integrity information is available.

For more information on SBAS and associated services, refer to the following resources:

• RTCA/DO-229D (MOPS). Available from www.rtca.org

• gps.faa.gov for information on WAAS.

• www.esa.int for information on EGNOS.

• www.essp-sas.eu for information about European Satellite Services Provider (ESSP), the

EGNOS operations manager.

• www.isro.org for information on GAGAN.

• www.sdcm.ru for information on SDCM.

SBAS satellites tracked (as of November 2015)

Identification Position GPS PRN SBAS Provider

AMR 98° W 133 WAAS

PanAmSat Galaxy XV 133.0° W 135 WAAS

TeleSat Anik F1R 107.3° W 138 WAAS

Inmarsat 3F2 AOR-E 15.5° W 120 EGNOS

Artemis 21.5° W 124 EGNOS

Inmarsat 3F5 IOR-W 25° E 126 EGNOS

MTSAT-1R 140.1° E 129 MSAS

MTSAT-2 145° E 137 MSAS

Inmarsat-4F1/IOR 64° E 127 GAGAN

GSAT-10 83° E 128 GAGAN

5.2 SBAS Features

This u-blox SBAS implementation is, in accordance with standard RTCA/DO-229D, a

class Beta-1 equipment. All timeouts etc. are chosen for the En Route Case. Do not use

this equipment under any circumstances for "safety of life" applications!

u-blox receivers are capable of receiving multiple SBAS signals concurrently, even from different

SBAS systems (WAAS, EGNOS, MSAS, etc.). They can be tracked and used for navigation

simultaneously. Every tracked SBAS satellite utilizes one vacant receiver tracking channel. Only

the number of receiver channels limits the total number of satellites used. Every SBAS satellite

that broadcasts ephemeris or almanac information can be used for navigation, just like a normal

GPS satellite.

For receiving correction data, the u-blox receiver automatically chooses the best SBAS satellite as

its primary source. It will select only one since the information received from other SBAS satellites

is redundant and/or could be inconsistent. The selection strategy is determined by the proximity

of the satellites, the services offered by the satellite, the configuration of the receiver (Testmode

allowed/disallowed, Integrity enabled/disabled) and the signal link quality to the satellite.

If corrections are available from the chosen SBAS satellite and used in the navigation calculation,

the DGPS flag is set in the receiver's output protocol messages (see UBX-NAV-PVT, UBX-NAV-SOL,

UBX-NAV-STATUS, UBX-NAV-SVINFO, NMEA Position Fix Flags description). The message UBX-

NAV-SBAS provides detailed information about which corrections are available and applied.

The most important SBAS feature for accuracy improvement is Ionosphere correction. The

measured data from regional RIMS stations are combined to make a TEC (Total Electron Content)

Map. This map is transferred to the receiver via the satellites to allow a correction of the

UBX-13003221 - R28 Early production information Page 17 of 460

## Page 33

u-blox 8 / u-blox M8 Receiver description - Manual

ionosphere error on each received satellite.

Supported SBAS messages

Message Type Message Content Source

0(0/2) Test Mode All

1 PRN Mask Assignment Primary

2, 3, 4, 5 Fast Corrections Primary

6 Integrity Primary

7 Fast Correction Degradation Primary

9 Satellite Navigation (Ephemeris) All

10 Degradation Primary

12 Time Offset Primary

17 Satellite Almanac All

18 Ionosphere Grid Point Assignment Primary

24 Mixed Fast / Long term Corrections Primary

25 Long term Corrections Primary

26 Ionosphere Delays Primary

Each satellite services a specific region and its correction signal is only useful within that region.

Planning is crucial to determine the best possible configuration, especially in areas where signals

from different SBAS systems can be received:

Example 1: SBAS Receiver in North America

In the eastern parts of North America, make sure that EGNOS satellites do not take preference

over WAAS satellites. The satellite signals from the EGNOS system should be disallowed by using

the PRN Mask.

Example 2: SBAS Receiver in Europe

Some WAAS satellite signals can be received in the western parts of Europe, therefore it is

recommended that the satellites from all but the EGNOS system should be disallowed using the

PRN Mask.

Although u-blox receivers try to select the best available SBAS correction data, it is

recommended to configure them to disallow using unwanted SBAS satellites.

The EGNOS SBAS system does not provide the satellite ranging function.

5.3 SBAS Configuration

To configure the SBAS functionalities use the UBX proprietary message UBX-CFG-SBAS (SBAS

Configuration).

SBAS Configuration parameters

Parameter Description

Mode - SBAS Subsystem Enabled / Disabled status of the SBAS subsystem. To

enable/disable SBAS operation use UBX-CFG-GNSS. The field in

UBX-CFG-SBAS is no longer supported.

Mode - Allow test mode usage Allow / Disallow SBAS usage from satellites in Test Mode

(Message 0)

Services/Usage - Ranging Use the SBAS satellites for navigation

UBX-13003221 - R28 Early production information Page 18 of 460

## Page 34

u-blox 8 / u-blox M8 Receiver description - Manual

SBAS Configuration parameters continued

Parameter Description

Services/Usage - Apply SBAS Combined enable/disable switch for Fast-, Long-Term and

correction data Ionosphere Corrections

Services/Usage - Apply integrity Use integrity data

information

Number of tracking channels Should be set using UBX-CFG-GNSS. The field in UBX-CFG-SBAS

is no longer supported.

PRN Mask Allows selectively enabling/disabling SBAS satellites (e.g.

restrict SBAS usage to WAAS-only).

By default, SBAS is enabled with three prioritized SBAS channels and it will use any received SBAS

satellites (except for those in test mode) for navigation, ionosphere parameters and corrections.

6 QZSS L1S SLAS Configuration Settings Description

6.1 QZSS L1S SLAS (Sub-meter Level Augmentation Service)

The L1S signal was formerly known as L1SAIF.

QZSS SLAS (Sub-meter Level Augmentation Service) is an augmentation technology, which

provides correction data for pseudoranges of GPS and QZSS satellites (as of October 2017).

Ground monitoring stations (GMS) positioned in Japan calculate independent corrections for each

visible satellite and broadcast this data to the user via QZSS satellites. The correction stream is

transmitted on the L1 frequency (1575.42 Mhz) and therefore no additional receiver is required to

make use of the correction data.

With QZSS SLAS enabled, u-blox receivers autonomously select the most suitable GMS based on

the user's location. The correction stream of this GMS will then be applied to the measurements in

order to improve position accuracy.

Furthermore, QZSS SLAS provides the user with reports for disaster and crisis management (DC

Reports) from the Japan Meteorological Agency (JMA) and other sources. Those reports are

provided by UBX-RXM-SFRBX messages.

For more information on QZSS SLAS, refer to the Interface Document IS-QZSS-L1S-001 (March

28, 2017) issued by the Cabinet Office, available from qzss.go.jp/en/.

6.2 QZSS L1S SLAS Features

Multiple SLAS signals can be tracked simultaneously. Only the number of receiver channels limits

the total number of satellites tracked.

The correction stream will be automatically detected from the most suitable ground monitoring

stations and QZSS satellites. The selection of the QZSS satellite is dependent on the quality of

the signals and the receiver configuration to allow satellites in test mode. The GMS that is not

flagged as unhealthy and is closest to the user will be selected. If the distance to the closest GMS

exceeds 200 km, no corrections will be used. The receiver might then fall back to using SBAS

corrections. Changes of the most suitable GMS or QZSS satellite as well as transitions in the

provided correction data stream will be handled in the background leading to a continuous set of

corrections for the navigation solution, if possible.

If corrections are available from the chosen QZSS satellite and used in the navigation calculation,

UBX-13003221 - R28 Early production information Page 19 of 460

## Page 35

u-blox 8 / u-blox M8 Receiver description - Manual

the DGNSS flag is set in the receiver's output protocol messages (see UBX-NAV-PVT, UBX-NAV-SOL,

UBX-NAV-STATUS, UBX-NAV-SVINFO, NMEA Position Fix Flags description). The message UBX-

NAV-SLAS provides detailed information about which corrections are available and applied.

By setting the RAIM feature (see UBX-CFG-SLAS), the user can setup the receiver to provide

DGPS-only solutions or to mix corrected and uncorrected measurements.

If in UBX-CFG-SLAS the RAIM option is set, other GNSS time systems than the QZSS

time system can't be observed by measurements.

Supported QZSS L1S SLAS messages for navigation enhancing

Message Type Message Content

0 Test Mode

47 Monitoring Station Information

48 PRN Mask

49 Data Issue Number

50 DGPS Correction

51 Satellite Health

6.3 QZSS L1S SLAS Configuration

To read and set the SLAS configurations use UBX-CFG-SLAS as follows:

QZSS L1S SLAS Configuration parameters

Parameter Description

Mode - enabled Apply QZSS SLAS corrections

Mode - test Allow the correction provided by QZSS satellites that are in

test mode

Mode - raim If this configuration is set, the receiver will try to estimate the

position by using only corrected measurements; if all corrected

measurements are not available, it won't use any corrections. If

this configuration is not set, the receiver will mix corrected and

uncorrected measurements for the navigation solution.

7 IMES Description

Indoor MEssaging System (IMES) is an extension to the QZSS specification using ground based

beacons that broadcast their location. Its purpose is to allow GNSS users to continue to navigate

inside buildings, when they can no longer reliably receive satellite based signals.

Operation of IMES beacons is only allowed within Japan.

u-blox receivers with IMES enabled conform to IS-QZSS v1.5 and do not support v1.4 or

earlier IMES signals. In particular, u-blox receivers rely on the IMES station's carrier

frequency being 1575.4282MHz ± 0.2ppm as specified in the IMES specification.

Transmissions from IMES stations that are not within this frequency range are unlikely

to be reliably received. Also the receiver expects the preamble 0x9E as well as the correct

sequence of CNT values as specified by the IS-QZSS.

u-blox receivers report the position information they receive from IMES transmitters directly with

UBX-RXM-IMES. They do not, however, combine this information with navigation solutions derived

from satellite signals (reported via various NMEA and UBX-NAV messages). Consequently, the

UBX-13003221 - R28 Early production information Page 20 of 460

## Page 36

u-blox 8 / u-blox M8 Receiver description - Manual

IMES position information may not always be consistent with satellite signal derived position

information.

7.1 IMES Features

• 50/250bps Auto-Detection: Both 50bps and 250bps IMES signals are supported by u-blox

receivers. The transmitter's data rate is detected automatically which allows the receiver to

even work in a mixed 50bps/250bps IMES environment.

• Dynamic Tracking Channel Allocation: The allocation of the tracking channels is done

dynamically, in the same way that channels are allocated to other GNSS. If sufficient IMES

stations are within reach of the receiver, it will track as many signals as it can up to the value of

maxTrkCh configured in UBX-CFG-GNSS (8 by default). To reserve a certain number of channels

for IMES only (preventing them from being dynamically allocated to other GNSS), set the

resTrkCh field in UBX-CFG-GNSS accordingly.

• Data summary: A summary of all the tracked IMES signals and what position information they

are providing is given in the UBX-RXM-IMES message.

• Raw IMES frames: The raw IMES subframes received from the IMES stations are reported as

they are received with UBX-RXM-SFRBX messages.

8 Navigation Configuration Settings Description

This section relates to the configuration message UBX-CFG-NAV5.

8.1 Platform settings

u-blox receivers support different dynamic platform models (see table below) to adjust the

navigation engine to the expected application environment. These platform settings can be

changed dynamically without performing a power cycle or reset. The settings improve the

receiver's interpretation of the measurements and thus provide a more accurate position output.

Setting the receiver to an unsuitable platform model for the given application environment is likely

to result in a loss of receiver performance and position accuracy.

Dynamic Platform Models

Platform Description

Portable Applications with low acceleration, e.g. portable devices. Suitable for most

situations.

Stationary Used in timing applications (antenna must be stationary) or other stationary

applications. Velocity restricted to 0 m/s. Zero dynamics assumed.

Pedestrian Applications with low acceleration and speed, e.g. how a pedestrian would move.

Low acceleration assumed.

Automotive Used for applications with equivalent dynamics to those of a passenger car. Low

vertical acceleration assumed.

At sea Recommended for applications at sea, with zero vertical velocity. Zero vertical

velocity assumed. Sea level assumed.

Airborne <1g Used for applications with a higher dynamic range and greater vertical

acceleration than a passenger car. No 2D position fixes supported.

Airborne <2g Recommended for typical airborne environments. No 2D position fixes

supported.

UBX-13003221 - R28 Early production information Page 21 of 460

## Page 37

u-blox 8 / u-blox M8 Receiver description - Manual

Dynamic Platform Models continued

Platform Description

Airborne <4g Only recommended for extremely dynamic environments. No 2D position fixes

supported.

Wrist Only recommended for wrist-worn applications. Receiver will filter out arm

motion (just available for protocol version > 17).

Bike Used for applications with equivalent dynamics to those of a motor bike. Low

vertical acceleration assumed.

Dynamic Platform Model Details

Platform Max Altitude MAX Horizontal MAX Vertical Sanity check type Max Position

[m] Velocity [m/s] Velocity [m/s] Deviation

Portable 12000 310 50 Altitude and Velocity Medium

Stationary 9000 10 6 Altitude and Velocity Small

Pedestrian 9000 30 20 Altitude and Velocity Small

Automotive 6000 100 15 Altitude and Velocity Medium

At sea 500 25 5 Altitude and Velocity Medium

Airborne <1g 50000 100 100 Altitude Large

Airborne <2g 50000 250 100 Altitude Large

Airborne <4g 50000 500 100 Altitude Large

Wrist 9000 30 20 Altitude and Velocity Medium

Bike 6000 100 15 Altitude and Velocity Medium

Dynamic platforms designed for high acceleration systems (e.g. airborne <2g) can result

in a higher standard deviation in the reported position.

If a sanity check against a limit of the dynamic platform model fails, then the position

solution is invalidated. The table above shows the types of sanity checks which are

applied for a particular dynamic platform model.

8.2 Navigation Input Filters

The navigation input filters in UBX-CFG-NAV5 mask the input data of the navigation engine.

These settings are already optimized. Do not change any parameters unless advised by

u-blox support engineers.

Navigation Input Filter parameters

Parameter Description

fixMode By default, the receiver calculates a 3D position fix if possible but reverts to 2D

position if necessary (Auto 2D/3D). The receiver can be forced to only calculate

2D (2D only) or 3D (3D only) positions.

fixedAlt and The fixed altitude is used if fixMode is set to 2D only. A variance greater than

fixedAltVar zero must also be supplied.

minElev Minimum elevation of a satellite above the horizon in order to be used in the

navigation solution. Low elevation satellites may provide degraded accuracy,

due to the long signal path through the atmosphere.

cnoThreshNum A navigation solution will only be attempted if there are at least the given

SVs and number of SVs with signals at least as strong as the given threshold.

cnoThresh

UBX-13003221 - R28 Early production information Page 22 of 460

## Page 38

u-blox 8 / u-blox M8 Receiver description - Manual

See also comments in section Degraded Navigation below.

8.3 Navigation Output Filters

The result of a navigation solution is initially classified by the fix type (as detailed in the fixType

field of UBX-NAV-PVT message). This distinguishes between failures to obtain a fix at all ("No Fix")

and cases where a fix has been achieved, which are further subdivided into specific types of fixes

(e.g. 2D, 3D, dead reckoning).

Where a fix has been achieved, a check is made to determine whether the fix should be classified

as valid or not. A fix is only valid if it passes the navigation output filters as defined in UBX-CFG-

NAV5. In particular, both PDOP and accuracy values must lie below the respective limits.

Valid fixes are marked using the valid flag in certain NMEA messages (see Position Fix Flags in

NMEA) and the gnssFixOK flag in UBX-NAV-PVT message.

Important: Users are recommended to check the gnssFixOK flag in the UBX-NAV-PVT or

the NMEA valid flag. Fixes not marked valid should not normally be used.

The UBX-NAV-SOL and UBX-NAV-STATUS messages also report whether a fix is valid in

their gpsFixOK and GPSfixOk flags. These messages have only been retained for

backwards compatibility and users are recommended to use the UBX-NAV-PVT message

in preference.

The UBX-CFG-NAV5 message also defines TDOP and time accuracy values that are used in order to

establish whether a fix is regarded as locked to GNSS or not, and as a consequence of this, which

time pulse setting has to be used. Fixes that do not meet both criteria will be regarded as

unlocked to GNSS, and the corresponding time pulse settings of UBX-CFG-TP5 will be used to

generate a time pulse.

8.3.1 Speed (3-D) Low-pass Filter

The UBX-CFG-ODO message offers the possibility to activate a speed (3-D) low-pass filter. The

output of the speed low-pass filter is published in the UBX-NAV-VELNED message (speed field). The

filtering level can be set via the UBX-CFG-ODO message (velLpGain field) and must be comprised

between 0 (heavy low-pass filtering) and 255 (weak low-pass filtering).

The internal filter gain is computed as a function of speed. Therefore, the level as defined

in the UBX-CFG-ODO message (velLpGain field) defines the nominal filtering level for

speeds below 5m/s.

8.3.2 Course over Ground Low-pass Filter

The UBX-CFG-ODO message offers the possibility to activate a course over ground low-pass filter

when the speed is below 8m/s. The output of the course over ground (also named heading of

motion 2-D) low-pass filter is published in the UBX-NAV-PVT message (headMot field), UBX-NAV-

VELNED message (heading field), NMEA-RMC message (cog field) and NMEA-VTG message (cogt

field). The filtering level can be set via the UBX-CFG-ODO message (cogLpGain field) and must be

comprised between 0 (heavy low-pass filtering) and 255 (weak low-pass filtering).

The filtering level as defined in the UBX-CFG-ODO message (cogLpGain field) defines the

filter gain for speeds below 8m/s. If the speed is higher than 8m/s, no course over ground

low-pass filtering is performed.

UBX-13003221 - R28 Early production information Page 23 of 460

## Page 39

u-blox 8 / u-blox M8 Receiver description - Manual

8.3.3 Low-speed Course Over Ground Filter

The UBX-CFG-ODO message offers the possibility to activate a low-speed course over ground filter

(also called heading of motion 2-D). This filter derives the course over ground from position at very

low speed. The output of the low-speed course over ground filter is published in the UBX-NAV-PVT

message (headMot field), UBX-NAV-VELNED message (heading field), NMEA-RMC message (cog

field) and NMEA-VTG message (cogt field). If the low-speed course over ground filter is not

activated or inactive, then the course over ground is computed as described in section Freezing

the Course Over Ground.

8.4 Static Hold

Static Hold Mode allows the navigation algorithms to decrease the noise in the position output

when the velocity is below a pre-defined 'Static Hold Threshold'. This reduces the position wander

caused by environmental factors such as multi-path and improves position accuracy especially in

stationary applications. By default, static hold mode is disabled.

If the speed drops below the defined 'Static Hold Threshold, the Static Hold Mode will be

activated. Once Static Hold Mode has been entered, the position output is kept static and the

velocity is set to zero until there is evidence of movement again. Such evidence can be velocity,

acceleration, changes of the valid flag (e.g. position accuracy estimate exceeding the Position

Accuracy Mask, see also section Navigation Output Filters), position displacement, etc.

The UBX-CFG-NAV5 message additionally allows for configuration of distance threshold (field

staticHoldMaxDist). If the estimated position is farther away from the static hold position than

this threshold, static mode will be quit.

8.5 Freezing the Course Over Ground

If the low-speed course over ground filter is deactivated or inactive (see section Low-speed Course

over Ground Filter), the receiver derives the course over ground from the GNSS velocity

information. If the velocity cannot be calculated with sufficient accuracy (e.g., with bad signals) or

if the absolute speed value is very low (under 0.1m/s) then the course over ground value becomes

inaccurate too. In this case the course over ground value is frozen, i.e. the previous value is kept

and its accuracy is degraded over time. These frozen values will not be output in the NMEA

messages NMEA-RMC and NMEA-VTG unless the NMEA protocol is explicitely configured to do so

(see NMEA Protocol Configuration).

8.6 Degraded Navigation

Degraded navigation describes all navigation modes which use less than four Satellite Vehicles

(SV).

8.6.1 2D Navigation

If the receiver only has three SVs for calculating a position, the navigation algorithm uses a

constant altitude to compensate for the missing fourth SV. When an SV is lost after a successful

3D fix (min. four SVs available), the altitude is kept constant at the last known value. This is called

a 2D fix.

u-blox receivers do not calculate any navigation solution with less than three SVs. Only u-

blox Timing products can calculate a timing solution with only one SV when they are in

stationary mode.

UBX-13003221 - R28 Early production information Page 24 of 460

## Page 40

u-blox 8 / u-blox M8 Receiver description - Manual

8.7 Geodetic Coordinate Systems and Ellipsoids

In order to have any useful meaning, the positions reported by a u-blox receiver must be

referenced to some coordinate system which defines the origin and, for example, which way is

"up". For many reasons, including history, practical autonomy and politics, all the major GNSS

define their own theoretical coordinate systems from which they realize a practical reference

frame by means of a network of reference points. Specifically:

• GPS uses WGS84

• GLONASS uses PZ90

• Galileo uses GTRF

• BeiDou uses CGCS2000

In practice, the relevant organisations choose to keep their respective frames very close to the

International Terrestrial Reference Frame (ITRF), defined and managed by the International Earth

Rotation and Reference Systems Service (IERS). However, because the Earth's tectonic plates and

even parts of the Earth's core move, new versions of ITRF are defined every few years, generally

with changes of the order of a few millimetres. Consequently, the major GNSS occasionally decide

that they need to update their reference frames to be better aligned to the latest ITRF. So, for

example, GPS switched to WGS84 (G1150) in GPS week 1150 (early 2002) based on ITRF2000, while

GLONASS switched from PZ90.02 to PZ90.11 at the end of 2013, based on ITRF2008. The net

effect of this, is that all the major GNSS use almost the same reference frame, but there are some

small (generally sub-cm) differences between them and these differences occasionally change.

In order to produce positions that can be shown on a map, it is necessary to translate between raw

coordinates (e.g. x, y, z) and a position relative to the Earth's surface (e.g. latitude, longitude and

altitude) and that requires defining the form of ellipsoid that best matches the shape of the Earth.

Historically many different ellipsoid definitions have been used for maps, many of which predate

the existence of GNSS and show quite significant differences, leading to discrepencies of as much

as 100 m in places. Fortunately, most digital maps now use the WGS84 ellipsoid, which is distinct

from the WGS84 coordinate system, but defined by the same body.

All u-blox receivers use (the current) version of WGS84 frame as their reference frame, carrying

out any necessary corrections internally. What is more, by default, u-blox receivers use the

WGS84 ellipsoid and therefore all positions communicated from/to a u-blox receiver will be relative

to that. However, users can alter this by specifying their chosen geodetic datum parameters using

the UBX-CFG-DAT message. The table below indicates the values u-blox recommends for use.

Recommended UBX-CFG-DAT parameters

Ellipsoid majA flat dX dY dZ rotX rotY rotZ

WGS84 (default) 6378137.0 298.257223563 0.0 0.0 0.0 0.0 0.0 0.0

PZ90 6378136.0 298.257839303 0.0 0.0 0.0 0.0 0.0 0.0

CGCS2000 6378137.0 298.257227101 0.0 0.0 0.0 0.0 0.0 0.0

Where the receiver is configured to use differential correction data (e.g. via an RTCM

stream), as a direct consequence, the receiver's coordinate frame will switch to whatever

frame the source of correction data is using.

UBX-13003221 - R28 Early production information Page 25 of 460

## Page 41

u-blox 8 / u-blox M8 Receiver description - Manual

9 Clocks and Time

9.1 Receiver Local Time

The receiver is dependent on a local oscillator (normally a TCXO or Crystal oscillator) for both the

operation of its radio parts and also for timing within its signal processing. No matter what

nominal frequency the local oscillator has (e.g. 26 MHz), u-blox receivers subdivide the oscillator

signal to provide a 1 kHz reference clock signal, which is used to drive many of the receiver's

processes. In particular, the measurement of satellite signals is arranged to be synchronised with

the "ticking" of this 1 kHz clock signal.

When the receiver first starts, it has no information about how these clock ticks relate to other

time systems; it can only count time in 1 millisecond steps. However, as the receiver derives

information from the satellites it is tracking or from aiding messages, it estimates the time that

each 1 kHz clock tick takes in the time-base of the relevant GNSS system. In previous generations

of u-blox receivers this was always the GPS time-base, but for this generation it could be GPS,

GLONASS, Galileo, or BeiDou. This estimate of GNSS time based on the local 1 kHz clock is called

receiver local time.

As receiver local time is a mapping of the local 1 kHz reference onto a GNSS time-base, it may

experience occasional discontinuities, especially when the receiver first starts up and the

information it has about the time-base is changing. Indeed after a cold start receiver local time will

initially indicate the length of time that the receiver has been running. However, when the receiver

obtains some credible timing information from a satellite or aiding message, it will jump to an

estimate of GNSS time.

9.2 Navigation Epochs

Each navigation solution is triggered by the tick of the 1 kHz clock nearest to the desired

navigation solution time. This tick is referred to as a navigation epoch. If the navigation solution

attempt is successful, one of the results is an accurate measurement of time in the time-base of

the chosen GNSS system, called GNSS system time. The difference between the calculated

GNSS system time and receiver local time is called the clock bias (and the clock drift is the rate at

which this bias is changing).

In practice the receiver's local oscillator will not be as stable as the atomic clocks to which GNSS

systems are referenced and consequently clock bias will tend to accumulate. However, when

selecting the next navigation epoch, the receiver will always try to use the 1 kHz clock tick which it

estimates to be closest to the desired fix period as measured in GNSS system time. Consequently

the number of 1 kHz clock ticks between fixes will occasionally vary (so when producing one fix per

second, there will normally be 1000 clock ticks between fixes, but sometimes, to correct drift away

from GNSS system time, there will be 999 or 1001).

The GNSS system time calculated in the navigation solution is always converted to a time in both

the GPS and UTC time-bases for output.

Clearly when the receiver has chosen to use the GPS time-base for its GNSS system time,

conversion to GPS time requires no work at all, but conversion to UTC requires knowledge of the

number of leap seconds since GPS time started (and other minor correction terms). The relevant

GPS to UTC conversion parameters are transmitted periodically (every 12.5 minutes) by GPS

satellites, but can also be supplied to the receiver via the UBX-MGA-GPS-UTC aiding message. By

contrast when the receiver has chosen to use the GLONASS time-base as its GNSS system time,

UBX-13003221 - R28 Early production information Page 26 of 460

## Page 42

u-blox 8 / u-blox M8 Receiver description - Manual

conversion to GPS time is more difficult as it requires knowledge of the difference between the

two time-bases, but conversion to UTC is easier (as GLONASS time is closely linked to UTC).

Where insufficient information is available for the receiver to perform any of these time-base

conversions precisely, pre-defined default offsets are used. Consequently plausible times are

nearly always generated, but they may be wrong by a few seconds (especially shortly after receiver

start). Depending on the configuration of the receiver, such "invalid" times may well be output, but

with flags indicating their state (e.g. the "valid" flags in UBX-NAV-PVT).

u-blox receivers employ multiple GNSS system times and/or receiver local times (in order

to support multiple GNSS systems concurrently), so users should not rely on UBX

messages that report GNSS system time or receiver local time being supported in

future. It is therefore recommended to give preference to those messages that report

UTC time.

9.3 iTOW Timestamps

All the main UBX-NAV messages (and some other messages) contain an iTOW field which

indicates the GPS time at which the navigation epoch occurred. Messages with the same iTOW

value can be assumed to have come from the same navigation solution.

Note that iTOW values may not be valid (i.e. they may have been generated with insufficient

conversion data) and therefore it is not recommended to use the iTOW field for any other purpose.

The original designers of GPS chose to express time/date as an integer week number

(starting with the first full week in January 1980) and a time of week (often abbreviated

to TOW) expressed in seconds. Manipulating time/date in this form is far easier for

digital systems than the more "conventional" year/month/day, hour/minute/second

representation. Consequently, most GNSS receivers use this representation internally,

only converting to a more "conventional form" at external interfaces. The iTOW field is

the most obvious externally visible consequence of this internal representation.

If reliable absolute time information is required, users are recommended to use the UBX-NAV-PVT

or UBX-HNR-PVT navigation solution messages which also contain additional fields that indicate

the validity (and accuracy in UBX-NAV-PVT) of the calculated times (see also the GNSS Times

section below for further messages containing time information).

9.4 GNSS Times

Each GNSS has its own time reference for which detailed and reliable information is provided in

the messages listed in the table below.

GNSS Times

Time Reference Message

GPS Time UBX-NAV-TIMEGPS

BeiDou Time UBX-NAV-TIMEBDS

GLONASS Time UBX-NAV-TIMEGLO

Galileo Time UBX-NAV-TIMEGAL

UTC Time UBX-NAV-TIMEUTC

UBX-13003221 - R28 Early production information Page 27 of 460

## Page 43

u-blox 8 / u-blox M8 Receiver description - Manual

9.5 Time Validity

Information about the validity of the time solution is given in the following form:

• Time validity: Information about time validity is provided in the valid flags (e.g. validDate and

validTime flags in the UBX-NAV-PVT message). If these flags are set, the time is known and

considered as valid for being used. These flags can be found in the GNSS Times table in the

GNSS Times section above as well as in the UBX-NAV-PVT and UBX-HNR-PVT messages.

• Time validity confirmation: Information about confirmed validity is provided in the

confirmedDate and confirmedTime flags in the UBX-NAV-PVT message. If these flags are set,

the time validity could be confirmed by using an additional independent source, meaning that

the probability of the time to be correct is very high. Note that information about time validity

confirmation is only available if the confirmedAvai bit in the UBX-NAV-PVT message is set.

Check UBX-NAV-PVT which Protocol Version supports this flag.

9.6 UTC Representation

UTC time is used in many NMEA and UBX messages. In NMEA messages it is always reported

rounded to the nearest hundredth of a second. Consequently, it is normally reported with two

decimal places (e.g. 124923.52). What is more, although compatibility mode (selected using UBX-

CFG-NMEA) requires three decimal places, rounding to the nearest hundredth of a second remains,

so the extra digit is always 0.

UTC time is is also reported within some UBX messages, such as UBX-NAV-TIMEUTC and UBX-NAV-

PVT. In these messages date and time are separated into seven distinct integer fields. Six of these

(year, month, day, hour, min and sec) have fairly obvious meanings and are all guaranteed to

match the corresponding values in NMEA messages generated by the same navigation epoch.

This facilitates simple synchronisation between associated UBX and NMEA messages.

The seventh field is called nano and it contains the number of nanoseconds by which the rest of

the time and date fields need to be corrected to get the precise time. So, for example, the UTC

time 12:49:23.521 would be reported as: hour: 12, min: 49, sec: 23, nano: 521000000.

It is however important to note that the first six fields are the result of rounding to the nearest

hundredth of a second. Consequently the nano value can range from -5000000 (i.e. -5 ms) to

+994999999 (i.e. nearly 995 ms).

When the nano field is negative, the number of seconds (and maybe minutes, hours, days, months

or even years) will have been rounded up. Therefore, some or all of them will need to be adjusted in

order to get the correct time and date. Thus in an extreme example, the UTC time 23:59:59.9993

on 31st December 2011 would be reported as: year: 2012, month: 1, day: 1, hour: 0, min: 0, sec: 0,

nano: -700000.

Of course, if a resolution of one hundredth of a second is adequate, negative nano values can

simply be rounded up to 0 and effectively ignored.

Which master clock the UTC time is referenced to is output in the message UBX-NAV-TIMEUTC.

For protocol versions 16 or greater, the preferred variant of UTC time can be specified using UBX-

CFG-NAV5.

9.7 Leap Seconds

Occasionally it is decided (by one of the international time keeping bodies) that, due to the slightly

uneven spin rate of the Earth, UTC has moved sufficiently out of alignment with mean solar time

(i.e. the Sun no longer appears directly overhead at 0 longitude at midday). A "leap second" is

UBX-13003221 - R28 Early production information Page 28 of 460

## Page 44

u-blox 8 / u-blox M8 Receiver description - Manual

therefore announced to bring UTC back into close alignment. This normally involves adding an

extra second to the last minute of the year, but it can also happen on 30th June. When this

happens UTC clocks are expected to go from 23:59:59 to 23:59:60 and only then on to 00:00:00.

It is also theoretically possible to have a negative leap second, in which case there will only be 59

seconds in a minute and 23:59:58 will be followed by 00:00:00.

u-blox receivers are designed to handle leap seconds in their UTC output and consequently users

processing UTC times from either NMEA and UBX messages should be prepared to handle

minutes that are either 59 or 61 seconds long.

Leap second information be be polled from the u-blox receiver with the message UBX-NAV-TIMELS

for Protocol Version 18 and above.

9.8 Real Time Clock

u-blox receivers contain circuitry to support a real time clock, which (if correctly fitted and

powered) keeps time while the receiver is otherwise powered off. When the receiver powers up, it

attempts to use the real time clock to initialise receiver local time and in most cases this leads to

appreciably faster first fixes.

9.9 Date

All GNSS frequently transmit information about the current time within their data message. In

most cases, this is a time of week (often abbreviated to TOW), which indicates the elapsed

number of seconds since the start of the week (midnight Saturday/Sunday). In order to map this

to a full date, it is necessary to know which week and so the GNSS also transmit a week number,

typically every 30 seconds. Unfortunately the GPS data message was designed in a way that only

allows the bottom 10 bits of the week number to be transmitted. This is not sufficient to yield a

completely unambiguous date as every 1024 weeks (a bit less than 20 years), the transmitted

week number value "rolls over" back to zero. Consequently, GPS receivers can't tell the difference

between, for example, 1980, 1999 or 2019 etc.

Fortunately, although BeiDou and Galileo have similar representations of time, they transmit

sufficient bits for the week number to be unambiguous for the forseeable future (the first

ambiguity will be in 2078 for Galileo and not until 2163 for BeiDou). GLONASS has a different

structure, based on a time of day, but again transmits sufficient information to avoid any

ambiguity during the expected lifetime of the system (the first ambiguous date will be in 2124).

Therefore, u-blox 8 / u-blox M8 receivers using Protocol Version 18 and above regard the date

information transmitted by GLONASS, BeiDou and Galileo to be unambiguous and, where

necessary, use this to resolve any ambiguity in the GPS date.

Customers attaching u-blox receivers to simulators should be aware that GPS time is

referenced to 6th January 1980, GLONASS to 1st January 1996, Galileo to 22nd August

1999 and BeiDou to 1st January 2006; the receiver cannot be expected to work reliably

with signals that appear to come from before these dates.

9.9.1 GPS-only Date Resolution

In circumstances where only GPS signals are available and for receivers with earlier firmware

versions, the receiver establishes the date by assuming that all week numbers must be at least as

large as a reference rollover week number. This reference rollover week number is hard-coded into

the firmware at compile time and is normally set a few weeks before the s/w is completed, but it

can be overridden by the wknRollover field of the UBX-CFG-NAVX5 message to any value the user

UBX-13003221 - R28 Early production information Page 29 of 460

## Page 45

u-blox 8 / u-blox M8 Receiver description - Manual

wishes.

The following example illustrates how this works: Assume that the reference rollover week

number set in the firmware at compile time is 1524 (which corresponds to a week in calendar year

2009, but would be transmitted by the satellites as 500). In this case, if the receiver sees

transmissions containing week numbers in the range 500 ... 1023, these will be interpreted as

week numbers 1524 ... 2047 (CY 2009 ... 2019), whereas transmissions with week numbers from 0

to 499 are interpreted as week numbers 2048 ... 2547 (CY 2019 ... 2028).

It is important to set the reference rollover week number appropriately when supplying u-

blox receivers with simulated signals, especially when the scenarios are in the past.

10 Broadcast Navigation Data

Reporting of broadcast navigation data is supported for products using protocol version

17 onwards.

The UBX-RXM-SFRBX reports the broadcast navigation data message collected by the receiver

from each tracked signal. When enabled, a separate message is generated every time the receiver

decodes a complete subframe of data from a tracked signal. The data bits are reported, as

received, including preambles and error checking bits as appropriate. However because there is

considerable variation in the data structure of the different GNSS signals, the form of the

reported data also varies. Indeed, although this document uses the term "subframe" generically, it

is not strictly the correct term for all GNSS (e.g. GLONASS has "strings" and Galileo has "pages").

10.1 Parsing Navigation Data Subframes

Each UBX-RXM-SFRBX message contains a subframe of data bits appropriate for the relevant

GNSS, delivered in a number of 32 bit words, as indicated by numWords field.

Due to the variation in data structure between different GNSS, the most important step in

parsing a UBX-RXM-SFRBX message is to identify the form of the data. This should be done by

reading the gnssId field, which indicates which GNSS the data was decoded from. In almost all

cases, this is sufficient to indicate the structure and the following sections are organised by GNSS

for that reason. However, in some cases the identity of the GNSS is not sufficient, and this is

described, where appropriate, in the following sections.

In most cases, the data does not map perfectly into a number of 32 bit words and, consequently,

some of the words reported in UBX-RXM-SFRBX messages contain fields marked as "Pad". These

fields should be ignored and no assumption should be made about their contents.

UBX-RXM-SFRBX messages are only generated when complete subframes are detected by the

receiver and all appropriate parity checks have passed.

Where the parity checking algorithm requires data to be inverted before it is decoded (e.g. GPS

L1C/A), the receiver carries this out before the message output. Therefore, users can process data

directly and do not need to worry about repeating any parity processing.

The meaning of the content of each subframe depends on the sending GNSS and is described in

the relevant Interface Control Documents (ICD).

10.2 GPS

The data structure in the GPS L1C/A and L2C signals is dissimilar and thus the UBX-RXM-SFRBX

message structure differs as well. For the GPS L1C/A and L2C signals it is as follows.

UBX-13003221 - R28 Early production information Page 30 of 460

## Page 46

u-blox 8 / u-blox M8 Receiver description - Manual

10.2.1 GPS L1C/A

For GPS L1C/A signals, there is a fairly straightforward mapping between the reported subframe

and the structure of subframe and words described in the GPS ICD. Each subframe comprises ten

data words, which are reported in the same order they are received.

Each word is arranged as follows:

Note that as the GPS data words only comprise 30 bits, the 2 most significant bits in each word

reported by UBX-RXM-SFRBX are padding and should be ignored.

10.3 GLONASS

For GLONASS L1OF and L2OF signals, each reported subframe contains a string as described in

the GLONASS ICD. This string comprises 85 data bits which are reported over three 32 bit words

in the UBX-RXM-SFRBX message. Data bits 1 to 8 are always a hamming code, whilst bits 81 to 84

are a string number and bit 85 is the idle chip, which should always have a value of zero. The

meaning of other bits vary with string and frame number.

The fourth and final 32 bit word in the UBX-RXM-SFRBX message contains frame and superframe

numbers (where available). These values aren't actually transmitted by the SVs, but are deduced

by the receiver and are included to aid decoding of the transmitted data. However, the receiver

does not always know these values, in which case a value of zero is reported.

The four words are arranged as follows:

In some circumstances, (especially on startup) the receiver may be able to decode data from a

GLONASS SV before it can identify the SV. When this occurs UBX-RXM-SFRBX messages will be

issued with an svId of 255 to indicate "unknown".

UBX-13003221 - R28 Early production information Page 31 of 460

## Page 47

u-blox 8 / u-blox M8 Receiver description - Manual

10.4 BeiDou

For BeiDou (B1I) signals, there is a fairly straightforward mapping between the reported subframe

and the structure of subframe and words described in the BeiDou ICD. Each subframe comprises

ten data words, which are reported in the same order they are received.

Each word is arranged as follows:

Note that as the BeiDou data words only comprise 30 bits, the 2 most significant bits in each word

reported by UBX-RXM-SFRBX are padding and should be ignored.

10.5 Galileo

The Galileo E1OS and E5b signals both transmit the I/NAV message but in different

configurations. The UBX-RXM-SFRBX structures for them are as follows.

10.5.1 Galileo E1OS

For Galileo E1OS signals, each reported subframe contains a pair of I/NAV pages as described in

the Galileo ICD.

Galileo pages can either be "Nominal" or "Alert" pages. For Nominal pages the eight words are

arranged as follows:

UBX-13003221 - R28 Early production information Page 32 of 460

## Page 48

u-blox 8 / u-blox M8 Receiver description - Manual

Alert pages are reported in very similar manner, but the page type bits will have value 1 and the

structure of the eight words will be slightly different (as indicated by the Galileo ICD).

10.6 SBAS

For SBAS (L1C/A) signals each reported subframe contains eight 32 data words to deliver the 250

bits transmitted in each SBAS data block.

The eight words are arranged as follows:

UBX-13003221 - R28 Early production information Page 33 of 460

## Page 49

u-blox 8 / u-blox M8 Receiver description - Manual

10.7 QZSS

The structure of the data delivered by QZSS L1C/A signals is effectively identical to that of GPS

(L1C/A). Similarly the QZSS L2C signal is effectively identical to the GPS (L2C).

The QZSS (L1SAIF) signal is different and uses the same data block format as used by SBAS

(L1C/A). QZSS (SAIF) signals can be distinguished from QZSS (L1C/A and L2C) by noting that they

have 8 words, instead of 10 for QZSS (L1C/A and L2C).

10.8 IMES

Data messages from IMES are of variable length and u-blox receivers currently support the

following varieties:

• Short - comprising of a single word

• Medium - comprising of two words

• Position 1 - comprising of three words

• Position 2 - comprising of four words

As a consequence, an IMES UBX-RXM-SFRBX message may have a numWords value of 1, 2, 3 or 4.

In all cases the structure of words follows the same pattern, with the first word being different

from any/all subsequent words as indicated by the following diagram:

10.9 Summary

The following table gives a summary of the different data message formats reported by the UBX-

RXM-SFRBX message.

UBX-13003221 - R28 Early production information Page 34 of 460

## Page 50

u-blox 8 / u-blox M8 Receiver description - Manual

Data message formats reported by UBX-RXM-SFRBX

GNSS Signal gnssId numWords period

GPS L1C/A 0 10 6s

SBAS L1C/A 1 8 1s

Galileo E1OS 2 8 2s

BeiDou B1I D1 3 10 6s

BeiDou B1I D2 3 10 0.6s

IMES Short 4 1 -

IMES Medium 4 2 -

IMES Position 1 4 3 -

IMES Position 2 4 4 -

QZSS L1C/A 5 10 6s

QZSS L1SAIF 5 8 1s

GLONASS L1OF 6 4 2s

11 Serial Communication Ports Description

u-blox receivers come with a highly flexible communication interface. It supports the NMEA and

the proprietary UBX protocols, and is truly multi-port and multi-protocol capable. Each protocol

(UBX, NMEA) can be assigned to several ports at the same time (multi-port capability) with

individual settings (e.g. baud rate, message rates, etc.) for each port. It is even possible to assign

more than one protocol (e.g. UBX protocol and NMEA at the same time) to a single port (multi-

protocol capability), which is particularly useful for debugging purposes.

To enable a message on a port, the UBX and/or NMEA protocol must be enabled on that port using

the UBX proprietary message UBX-CFG-PRT. This message also allows changing port-specific

settings (baud rate, address etc.). See UBX-CFG-MSG for a description of the mechanism for

enabling and disabling messages.

The following table shows the port numbers reported in the messages UBX-MON-IO, UBX-MON-

MSGPP, UBX-MON-TXBUF, UBX-MON-RXBUF. Note that any numbers not listed are reserved for future

use.

Port Number assignment

Port # Electrical Interface

0 DDC (I2C compatible)

1 UART 1

3 USB

4 SPI

11.1 TX-ready indication

This feature enables each port to define a corresponding pin, which indicates if bytes are ready to

be transmitted. By default, this feature is disabled. For USB, this feature is configurable but might

not behave as described below due to a different internal transmission mechanism. If the number

of pending bytes reaches the threshold configured for this port, the corresponding pin will become

active (configurable active-low or active-high), and stay active until the last bytes have been

transferred from software to hardware (note that this is not necessarily equal to all bytes

transmitted, i.e. after the pin has become inactive, up to 16 bytes can still need to be transferred

to the host).

UBX-13003221 - R28 Early production information Page 35 of 460

## Page 51

u-blox 8 / u-blox M8 Receiver description - Manual

The TX-ready pin can be selected from all PIOs which are not in use (see UBX-MON-HW for a list of

the PIOs and their mapping), each TX-ready pin is exclusively for one port and cannot be shared. If

the PIO is invalid or already in use, only the configuration for the TX-ready pin is ignored, the rest

of the port configuration is applied if valid. The acknowledge message does not indicate if the TX-

ready configuration is successfully set, it only indicates the successful configuration of the port.

To validate successful configuration of the TX-ready pin, the port configuration should be polled

and the settings of TX-ready feature verified (will be set to disabled/all zero if the settings are

invalid).

The threshold should not be set above 2 kB, as the internal message buffer limit can be reached

before this, resulting in the TX-ready pin never being set as messages are discarded before the

threshold is reached.

11.2 Extended TX timeout

If the host does not communicate over SPI or DDC for more than approximately 2 seconds, the

device assumes that the host is no longer using this interface and no more packets are scheduled

for this port. This mechanism can be changed by enabling "extended TX timeouts", in which case

the receiver delays idling the port until the allocated and undelivered bytes for this port reach 4 kB.

This feature is especially useful when using the TX-ready feature with a message output rate of

less than once per second, and polling data only when data is available, determined by the TX-

ready pin becoming active.

11.3 UART Ports

One or two Universal Asynchronous Receiver/Transmitter (UART) ports are featured, that can be

used to transmit GNSS measurements, monitor status information and configure the receiver.

See our online product descriptions for availability.

The serial ports consist of an RX and a TX line. Neither handshaking signals nor hardware flow

control signals are available. These serial ports operate in asynchronous mode. The baud rates

can be configured individually for each serial port. However, there is no support for setting

different baud rates for reception and transmission.

As of Protocol version 18+, the UART RX interface will be disabled when more than 100

frame errors are detected during a one-second period. This can happen if the wrong

baud rate is used or the UART RX pin is grounded. The error message appears when the

UART RX interface is re-enabled at the end of the one-second period.

Possible UART Interface Configurations

Baud Rate Data Bits Parity Stop Bits

4800 8 none 1

9600 8 none 1

19200 8 none 1

38400 8 none 1

57600 8 none 1

115200 8 none 1

230400 8 none 1

460800 8 none 1

Note that for protocols such as NMEA or UBX, it does not make sense to change the default word

length values (data bits) since these properties are defined by the protocol and not by the

UBX-13003221 - R28 Early production information Page 36 of 460

## Page 52

u-blox 8 / u-blox M8 Receiver description - Manual

electrical interface.

If the amount of data configured is too much for a certain port's bandwidth (e.g. all UBX messages

output on a UART port with a baud rate of 9600), the buffer will fill up. Once the buffer space is

exceeded, new messages to be sent will be dropped. To prevent message losses, the baud rate

and communication speed or the number of enabled messages should be selected so that the

expected number of bytes can be transmitted in less than one second.

See UBX-CFG-PRT for UART for a description of the contents of the UART port configuration

message.

11.4 USB Port

One Universal Serial Bus (USB) port is featured. See the data sheet for availability. This port can

be used for communication purposes and to power the positioning chip or module.

The USB interface supports two different power modes:

• In Self Powered Mode the receiver is powered by its own power supply. VDDUSB is used to

detect the availability of the USB port, i.e. whether the receiver is connected to a USB host.

• In Bus Powered Mode the device is powered by the USB bus, therefore no additional power

supply is needed. See the table below for the default maximum current that can be drawn by the

receiver. See UBX-CFG-USB for a description on how to change this maximum. Configuring Bus

Powered Mode indicates that the device will enter a low power state with disabled GNSS

functionality when the host suspends the device, e.g. when the host is put into stand-by mode.

Maximum Current in Bus Powered Mode

Generation Max Current

u-blox 8 / u-blox M8 100 mA

The voltage range for VDDUSB is specified from 3.0 V to 3.6 V, which differs slightly

from the specification for VCC.

The boot screen is retransmitted on the USB port after the enumeration. However,

messages generated between boot-up of the receiver and USB enumeration are not

visible on the USB port.

11.5 DDC Port

The Display Data Channel (DDC) bus is a two-wire communication interface compatible with the

I2C standard (Inter-Integrated Circuit). See our online product selector matrix for availability.

Unlike all other interfaces, the DDC is not able to communicate in full-duplex mode, i.e. TX and RX

are mutually exclusive. u-blox receivers act as a slave in the communication setup, therefore they

cannot initiate data transfers on their own. The host, which is always master, provides the data

clock (SCL), and the clock frequency is therefore not configurable on the slave.

The receiver's DDC address is set to 0x42 by default. This address can be changed by setting the

mode field in UBX-CFG-PRT for DDC accordingly.

As the receiver will be run in slave mode and the DDC physical layer lacks a handshake mechanism

to inform the master about data availability, a layer has been inserted between the physical layer

and the UBX and NMEA layer. The receiver DDC interface implements a simple streaming

interface that allows the constant polling of data, discarding everything that is not parse-able.

The receiver returns 0xFF if no data is available. The TX-ready feature can be used to inform the

master about data availability and can be used as a trigger for data transmission.

UBX-13003221 - R28 Early production information Page 37 of 460

## Page 53

u-blox 8 / u-blox M8 Receiver description - Manual

11.5.1 Read Access

The DDC interface allows 256 slave registers to be addressed. As shown in Figure DDC Register

Layout only three of these are currently implemented. The data registers 0 to 252, at addresses

0x00 to 0xFC, each 1 byte in size, contain information to be defined later - the result of reading

them is undefined. The currently available number of bytes in the message stream can be read at

addresses 0xFD and 0xFE. The register at address 0xFF allows the data stream to be read. If

there is no data awaiting transmission from the receiver, then this register will deliver the value

0xff, which cannot be the first byte of a valid message. If message data is ready for transmission,

then successive reads of register 0xff will deliver the waiting message data.

The registers 0x00 to 0xFC are reserved for future use and may be defined in a later

firmware release. Do not use them, as they don't provide any meaningful data!

DDC Register Layout

11.5.1.1 Read Access Forms

There are two forms of DDC read transfer. The 'random access' form includes a slave register

address and thus allows any register to be read. The second 'current address' form omits the

register address. If this second form is used, then an address pointer in the receiver is used to

determine which register to read. This address pointer will increment after each read unless it is

already pointing at register 0xff, the highest addressable register, in which case it remains

unaltered. The initial value of this address pointer at start-up is 0xff, so by default all current

address reads will repeatedly read register 0xff and receive the next byte of message data (or 0xff

if no message data is waiting). Figure DDC Random Read Access shows the format of the random

access form of the request. Following the start condition from the master, the 7-bit device

address and the RW bit (which is a logic low for write access) are clocked onto the bus by the

master transmitter. The receiver answers with an acknowledge (logic low) to indicate that it

UBX-13003221 - R28 Early production information Page 38 of 460

## Page 54

u-blox 8 / u-blox M8 Receiver description - Manual

recognises the address. Next, the 8-bit address of the register to be read must be written to the

bus. Following the receiver’s acknowledge, the master again triggers a start condition and writes

the device address, but this time the RW bit is a logic high to initiate the read access. Now, the

master can read 1 to N bytes from the receiver, generating a not-acknowledge and a stop condition

after the last byte being read.

DDC Random Read Access

The format of the current address read request is :

DDC Current Address Read Access

11.5.2 Write Access

The receiver does not provide any write access except for writing UBX and NMEA messages to the

receiver, such as configuration or aiding data. Therefore, the register set mentioned in section

Read Access is not writeable. Following the start condition from the master, the 7-bit device

address and the RW bit (which is a logic low for write access) are clocked onto the bus by the

master transmitter. The receiver answers with an acknowledge (logic low) to indicate that it is

responsible for the given address. Now, the master can write 2 to N bytes to the receiver,

generating a stop condition after the last byte being written. The number of data bytes must be

at least 2 to properly distinguish from the write access to set the address counter in random read

accesses.

UBX-13003221 - R28 Early production information Page 39 of 460

## Page 55

u-blox 8 / u-blox M8 Receiver description - Manual

DDC Write Access

11.6 SPI Port

A Serial Peripheral Interface (SPI) bus is available with selected receivers. See our online product

descriptions for availability.

SPI is a four-wire synchronous communication interface. In contrast to UART, the master provides

the clock signal, which therefore doesn't need to be specified for the slave in advance. Moreover, a

baud rate setting is not applicable for the slave. SPI modes 0-3 are implemented and can be

configured using the field mode.spiMode in CFG-PRT for SPI (default is SPI mode 0).

The SPI clock speed is limited depending on hardware and firmware versions!

11.6.1 Maximum SPI clock speed

u-blox 8 / u-blox M8 receivers support a maximum SPI clock speed of 5.5 MHz.

11.6.2 Read Access

As the register mode is not implemented for the SPI port, only the UBX/NMEA message stream is

provided. This stream is accessed using the Back-To-Back Read and Write Access (see section

Back-To-Back Read and Write Access). When no data is available to be written to the receiver,

MOSI should be held logic high, i.e. all bytes written to the receiver are set to 0xFF.

To prevent the receiver from being busy parsing incoming data, the parsing process is stopped

after 50 subsequent bytes containing 0xFF. The parsing process is re-enabled with the first byte

not equal to 0xFF. The number of bytes to wait for deactivation (50 by default) can be adjusted

using the field mode.ffCnt in CFG-PRT for SPI, which is only necessary when messages shall be

sent containing a large number of subsequent 0xFF bytes.

If the receiver has no more data to send, it sets MISO to logic high, i.e. all bytes transmitted decode

to 0xFF. An efficient parser in the host will ignore all 0xFF bytes which are not part of a message

and will resume data processing as soon as the first byte not equal to 0xFF is received.

11.6.3 Back-To-Back Read and Write Access

The receiver does not provide any write access except for writing UBX and NMEA messages to the

receiver, such as configuration or aiding data. For every byte written to the receiver, a byte will

simultaneously be read from the receiver. While the master writes to MOSI, at the same time it

UBX-13003221 - R28 Early production information Page 40 of 460

## Page 56

u-blox 8 / u-blox M8 Receiver description - Manual

needs to read from MISO, as any pending data will be output by the receiver with this access. The

data on MISO represents the results from a current address read, returning 0xFF when no more

data is available.

SPI Back-To-Back Read/Write Access

11.7 How to change between protocols

Reconfiguring a port from one protocol to another is a two-step process:

• Step 1: the preferred protocol(s) needs to be enabled on a port using UBX-CFG-PRT. One port can

handle several protocols at the same time (e.g. NMEA and UBX). By default, all ports are

configured for UBX and NMEA protocol so in most cases, it's not necessary to change the port

settings at all. Port settings can be viewed and changed using the UBX-CFG-PRT messages.

• Step 2: activate certain messages on each port using UBX-CFG-MSG.

12 Multiple GNSS assistance (MGA)

12.1 Introduction

Users would ideally like GNSS receivers to provide accurate position information the moment the

receivers are turned on. With standard GNSS receivers there can be a significant delay in

providing the first position fix, principally because the receiver needs to obtain data from several

satellites and the satellites transmit that data slowly. Under adverse signal conditions, data

downloads from the satellites to the receiver can take minutes, hours or even fail altogether.

Assisted GNSS (A-GNSS) is a common solution to this problem and involves some form of

reference network of receivers that collect data such as ephemeris, almanac, accurate time and

satellite status and pass this onto to the target receiver via any suitable communications link.

Such assistance data enables the receiver to compute a position within a few seconds, even under

poor signal conditions.

The UBX-MGA message class provides the means for delivering assistance data to u-blox

receivers and customers can obtain it from the u-blox AssistNow Online or AssistNow Offline

Services. Alternatively they can obtain assistance data from third-party sources (e.g. SUPL/RRLP)

and generate the appropriate UBX-MGA messages to send this data to the receiver.

UBX-13003221 - R28 Early production information Page 41 of 460

## Page 57

u-blox 8 / u-blox M8 Receiver description - Manual

12.2 Assistance Data

u-blox receivers currently accept the following types of assistance data:

• Position: Estimated receiver position can be submitted to the receiver using the UBX-MGA-INI-

POS_XYZ or UBX-MGA-INI-POS_LLH messages.

• Time: The current time can either be supplied as an inexact value via the standard

communication interfaces, suffering from latency depending on the baud rate, or using

hardware time synchronization where an accurate time pulse is connected to an external

interrupt. The preferred option is to supply UTC time using the UBX-MGA-INI-TIME_UTC

message, but times referenced to some GNSS can be delivered with the UBX-MGA-INI-TIME_

GNSS message.

• Clock drift: An estimate of the clock drift can be sent to the receiver using the UBX-MGA-INI-

CLKD message.

• Frequency: It is possible to supply hardware frequency aiding by connecting a periodic

rectangular signal with a frequency up to 500 kHz and arbitrary duty cycle (low/high phase

duration must not be shorter than 50 ns) to an external interrupt, and providing the applied

frequency value using the UBX-MGA-INI-FREQ message.

• Current orbit data: Each different GNSS transmits orbit data in slightly different forms. For

each system there are separate messages for delivering ephemeris and almanac. So for

example GPS ephemeris is delivered to the receiver using the UBX-MGA-GPS-EPH message, while

GLONASS almanac is delivered with the UBX-MGA-GLO-ALM message.

• Predicted orbit data: UBX-MGA-ANO messages can be used to supply predictions of future orbit

information to a u-blox receiver. These messages can be obtained from the AssistNow Offline

Service and allow a receiver to improve its TTFF even when it is no longer connected to the

internet.

• Auxiliary information: Each GNSS transmits some auxiliary data (such as SV health information

or UTC parameters) to the receiver. A selection of messages exist for providing such

information to the receiver, such as UBX-MGA-GPS-IONO for ionospheric data from GPS.

• EOP: Earth Orientation Parameters can be sent to the receiver using the UBX-MGA-INI-EOP

message. This will replace the default model used by the AssistNow Autonomous feature and

may improve performance (particularly as the receiver gets older and the built-in model decays).

• Navigation Database: u-blox receivers can be instructed to dump the current state of their

internal navigation database with the UBX-MGA-DBD-POLL message; sending this information

back to the receiver (e.g. after a period when the receiver was turned off) restores the database

to its former state, and thus allows the receiver to restart rapidly.

12.3 AssistNow Online

AssistNow Online is u-blox' end-to-end Assisted GNSS (A-GNSS) solution for receivers that have

access to the internet. Data supplied by the AssistNow Online Service can be directly uploaded to

a u-blox receiver in order to substantially reduce Time To First Fix (TTFF), even under poor signal

conditions. The system works by collecting data such as ephemeris and almanac from the

satellites through u-blox' Global Reference Network of receivers and providing this data to

customers in a convenient form that can be forwarded on directly to u-blox receivers.

The AssistNow Online Service uses a simple, stateless, HTTP interface. Therefore, it works on all

standard mobile communication networks that support internet access, including GPRS, UMTS

and Wireless LAN. No special arrangements need to be made with mobile network operators to

UBX-13003221 - R28 Early production information Page 42 of 460

## Page 58

u-blox 8 / u-blox M8 Receiver description - Manual

enable AssistNow Online.

The data returned by the AssistNow Online Service is a sequence of UBX-MGA messages, starting

with an estimate of the current time in the form of a UBX-MGA-INI-TIME_UTC message.

AssistNow Online currently supports GPS, GLONASS, BeiDou, Galileo, and QZSS.

Customers may choose to use third party sources of assistance data instead of using

the AssistNow Online Service. Customers choosing this option will need to ensure that

the data is converted from the format used by the third party source to the appropriate

MGA messages. However, it is important to ensure that the receiver has an estimate of

the current time before it processes any other assistance data. For this reason, it is

strongly recommended to send a UBX-MGA-INI-TIME_UTC or UBX-MGA-INI-TIME_GNSS

as the first message of any assistance.

12.3.1 Host Software

As u-blox receivers have no means to connect directly with the internet, the AssistNow Online

system can only work if the host system that contains the receiver can connect to the internet,

download the data from the AssistNow Online Service and forward it on to the receiver. In the

simplest case that may involve fetching the data from the AssistNow Online Service (by means of

a single HTTP GET request), and sending the resulting data to the receiver.

Depending on the circumstances, it may be beneficial for the host software to include:

• Creating an appropriate UBX-MGA-INI-TIME_UTC message to deliver a better sense of time to

the receiver, especially if the host system has a very good sense of the current time and can

deliver a time pulse to one of the receiver's EXTINT pins.

• Enable and use flow control to prevent loss of data due to buffer overflow in the receiver.

u-blox provides the source code for an example library, called libMGA, that provides all of

UBX-13003221 - R28 Early production information Page 43 of 460

## Page 59

u-blox 8 / u-blox M8 Receiver description - Manual

the functionality we expect in most host software.

12.3.2 AssistNow Online Sequence

A typical sequence of use of the AssistNow Online Service comprises the following steps:

• Power-up the u-blox receiver

• Request data from the AssistNow Online Service

• Optionally send UBX-MGA-INI-TIME_UTC followed by hardware time synchronization pulse if

hardware time synchronization is required.

• Send the UBX messages obtained from the AssistNow Online Service to the receiver.

12.3.3 Flow Control

u-blox receivers aim to process incoming messages as quickly as possible, but there will always be

a small delay in processing each message. Uploading assistance data to the receiver can involve

sending as many as one hundred individual messages to the receiver, one after the other. If the

communication link is fast, and/or the receiver is busy (trying to acquire new signals), it is possible

that the internal buffers will overflow and some messages will be lost. In order to combat this, u-

blox receivers support an optional flow control mechanism for assistance.

Flow control is activated by setting the ackAiding parameter in the UBX-CFG-NAVX5 message.

As a result the receiver will issue an acknowledgement message (UBX-MGA-ACK) for each

assistance message it successfully receives. The host software can examine these

acknowledgements to establish whether there were any problems with the data sent to the

receiver and deduce (by the lack of acknowledgement) if any messages have been lost. It may then

be appropriate to resend some of the assistance messages.

The simplest way to implement flow control would be to send one UBX-MGA assistance message

at a time, waiting for the acknowledgement, before sending the next. However, such a strategy is

likely to introduce significant delays into the whole assistance process. The best strategy will

depend on the amount of assistance data being sent and the nature of the communications link

(e.g. baud rate of serial link). u-blox recommends that when customers are developing their host

software they start by sending all assistance messages and then analyse the resulting

acknowledgements to see whether there have been significant losses. Adding small delays during

the transmission may be a simple but effective way to avoid substantial loss of data.

12.3.4 Authorization

The AssistNow Online Service is only available for use by u-blox customers. In order to use the

services, customers will need to obtain an authorization token from u-blox. This token must be

supplied as a parameter whenever a request is made to either service.

12.3.5 Service Parameters

The information exchange with the AssistNow Online Service is based on the HTTP protocol. Upon

reception of an HTTP GET request, the server will respond with the required messages in binary

format or with an error string in text format. After delivery of all data, the server will terminate the

connection.

The HTTP GET request from the client to the server should contain a standard HTTP query string

in the request URL. The query string consists of a set of "key=value" parameters in the following

form:

UBX-13003221 - R28 Early production information Page 44 of 460

## Page 60

u-blox 8 / u-blox M8 Receiver description - Manual

key=value;key=value;key=value;

The following rules apply:

• The order of keys is not important.

• Keys and values are case sensitive.

• Keys and values must be separated by an equals character ('=').

• Key/value pairs must be separated by semicolons (';').

• If a value contains a list, each item in the list must be separated by a comma (',').

The following table describes the keys that are supported.

AssistNow Online Parameter Keys

Key Name Unit/Range Optional Description

token String Mandatory The authorization token supplied by u-blox when a client

registers to use the service.

gnss String Mandatory A comma separated list of the GNSS for which data should be

returned. Valid GNSS are: gps, qzss and glo.

datatype String Mandatory A comma separated list of the data types required by the

client. Valid data types are: eph, alm, aux and pos. Time data

is always returned for each request. If the value of this

parameter is an empty string, only time data will be returned.

lat Numeric Optional Approximate user latitude in WGS 84 expressed in degrees

[degrees] and fractional degrees. Must be in range -90 to 90. Example:

lat=47.2.

lon Numeric Optional Approximate user longitude in WGS 84 expressed in degrees

[degrees] and fractional degrees. Must be in range -180 to 180. Example:

lon=8.55.

alt Numeric Optional Approximate user altitude above WGS 84 Ellipsoid. If this

[meters] value is not provided, the server assumes an altitude of 0

meters. Must be in range -1000 to 50000.

pacc Numeric Optional Approximate accuracy of submitted position (see position

[meters] parameters note below). If this value is not provided, the

server assumes an accuracy of 300 km. Must be in range 0 to

6000000.

tacc Numeric Optional The timing accuracy (see time parameters note below). If this

[seconds] value is not provided, the server assumes an accuracy of 10

seconds. Must be in range 0 to 3600.

latency Numeric Optional Typical latency between the time the server receives the

[seconds] request, and the time when the assistance data arrives at the

u-blox receiver. The server can use this value to correct the

time being transmitted to the client. If this value is not

provided, the server assumes a latency of 0. Must be in range

0 to 3600.

filteronpos (no value Optional If present, the ephemeris data returned to the client will only

required) contain data for the satellites which are likely to be visible

from the approximate position provided by the lat, lon, alt and

pacc parameters. If the lat and lon parameters are not

provided the service will return an error.

UBX-13003221 - R28 Early production information Page 45 of 460

## Page 61

u-blox 8 / u-blox M8 Receiver description - Manual

AssistNow Online Parameter Keys continued

Key Name Unit/Range Optional Description

filteronsv String Optional A comma separated list of u-blox gnssId:svId pairs. The

ephemeris data returned to the client will only contain data

for the listed satellites.

Thus, as an example, a valid parameter string would be:

token=XXXXXXXXXXXXXXXXXXXXXX;gnss=gps,qzss;datatype=eph,pos,aux;lat=47.28;lon=8.56;

pacc=1000

12.3.5.1 Position parameters (lat, lon, alt and pacc)

The position parameters (lat, lon, alt and pacc) are used by the server for two purposes:

• If the filteronpos parameter is provided, the server determines the currently visible satellites at

the user position, and only sends the ephemeris data of those satellites which should be in view

at the location of the user. This reduces bandwidth requirements. In this case the 'pacc' value is

taken into account, meaning that the server will return all SVs visible in the given uncertainty

region.

• If the datatype 'pos' is requested, the server will return the position and accuracy in the

response data. When this data is supplied to the u-blox receiver, depending on the accuracy of

the provided data, the receiver can then choose to select a better startup strategy. For example,

if the position is accurate to 100 km or better, the u-blox receiver will choose to go for a more

optimistic startup strategy. This will result in quicker startup time. The receiver will decide

which strategy to choose, depending on the 'pacc' parameter. If the submitted user position is

less accurate than what is being specified with the 'pacc' parameter, then the user will

experience prolonged or even failed startups.

12.3.5.2 Time parameters (tacc and latency)

Time data is always returned with each request. The time data refers to the time at which the

response leaves the server, corrected by an optional latency value. This time data provided by the

service is accurate to approximately 10 ms but by default the time accuracy is indicated to be +/-

10 seconds in order to account for network latency and any time between the client receiving the

data and it being provided to the receiver.

If both the network latency and the client latency can safely be assumed to be very low (or are

known), the client can choose to set the accuracy of the time message (tacc) to a much smaller

value (e.g. 0.5 s). This will result in a faster TTFF. The latency can also be adjusted as appropriate.

However, these fields should be used with caution: if the time accuracy is not correct when the

time data reaches the receiver, the receiver may experience prolonged or even failed start-ups.

For optimal results, the client should establish an accurate sense of time itself (e.g. by calibrating

its system clock using a local NTP service) and then modify the time data received from the

service as appropriate.

12.3.6 Multiple Servers

u-blox has designed and implemented the AssistNow Online Service in a way that should provide

very high reliability. Nonetheless, there will be rare occasions when a server is not available (e.g.

due to failure or some form of maintenance activity). In order to protect customers against the

impact of such outages, u-blox will run at least two instances of the AssistNow Online Service on

independent machines. Customers will have a free choice of requesting assistance data from any

UBX-13003221 - R28 Early production information Page 46 of 460

## Page 62

u-blox 8 / u-blox M8 Receiver description - Manual

of these servers, as all will provide the same information. However, should one fail for whatever

reason, it is highly unlikely that the other server(s) will also be unavailable. Therefore customers

requiring the best possible availability are recommended to implement a scheme where they

direct their requests to a chosen server, but, if that server fails to respond, have a fall-back

mechanism to use another server instead.

12.4 AssistNow Offline

AssistNow Offline is a feature that combines special firmware in u-blox receivers and a proprietary

service run by u-blox. It is targeted at receivers that only have occasional internet access and so

cannot use AssistNow Online. AssistNow Offline speeds up Time To First Fix (TTFF), typically to

considerably less than 10 s

AssistNow Offline currently supports GPS and GLONASS. u-blox intends to expand the

AssistNow Offline Service to support other GNSS (such as BeiDou and Galileo) in due

course.

The AssistNow Offline Service uses a simple, stateless, HTTP interface. Therefore, it works on all

standard mobile communication networks that support internet access, including GPRS, UMTS

and Wireless LAN. No special arrangements need to be made with mobile network operators to

enable AssistNow Offline.

Users of AssistNow Offline are expected to download data from the AssistNow Offline Service,

specifying the time period they want covered (1 to 5 weeks) and the types of GNSS. This data

must be uploaded to a u-blox receiver, so that it can estimate the positions of the satellites, when

no better data is available. Using these estimates will not provide as accurate a position fix as if

current ephemeris data is used, but it will allow much faster TTFFs in nearly all cases.

The data obtained from the AssistNow Offline Service is organised by date, normally a day at a

time. Consequently the more weeks for which coverage is requested, the larger the amount of

data to handle. Similarly, each different GNSS requires its own data and in the extreme cases,

several hundred kilobytes of data will be provided by the service. This amount can be reduced by

requesting lower resolution, but this will have a small negative impact on both position accuracy

and TTFF. See section Offline Service Parameters for details of how to specify these options.

The downloaded Offline data is encoded in a sequence of UBX-MGA-ANO messages, one for every

SV for every day of the period covered. Thus, for example, data for all GPS SVs for 4 weeks will

involve in excess of 900 separate messages, taking up around 70 kbytes. Where a u-blox receiver

has flash storage, all the data can be directly uploaded to be stored in the flash until it is needed.

In this case, the receiver will automatically select the most appropriate data to use at any time.

See section flash-based AssistNow Offline for further details.

AssistNow Offline can also be used where the receiver has no flash storage, or there is insufficient

spare flash memory. In this case the customer's system must store the AssistNow Offline data

until the receiver needs it and then upload only the appropriate part for immediate use. See

section host-based AssistNow Offline for further details.

12.4.1 Service Parameters

The information exchange with the AssistNow Offline Service is based on the HTTP protocol.

Upon reception of an HTTP GET request, the server will respond with the required messages in

binary format or with an error string in text format. After delivery of all data, the server will

terminate the connection.

The HTTP GET request from the client to the server should contain a standard HTTP querystring

UBX-13003221 - R28 Early production information Page 47 of 460

## Page 63

u-blox 8 / u-blox M8 Receiver description - Manual

in the request URL. The querystring consists of a set of "key=value" parameters in the following

form:

key=value;key=value;key=value;

The following rules apply:

• The order of keys is not important.

• Keys and values are case sensitive.

• Keys and values must be separated by an equals character ('=').

• Key/value pairs must be separated by semicolons (';').

• If a value contains a list, each item in the list must be separated by a comma (',').

The following table describes the keys that are supported.

AssistNow Offline Parameter Keys

Key Name Unit/Range Optional Description

token String Mandatory The authorization token supplied by u-blox when a client

registers to use the service.

gnss String Mandatory A comma separated list of the GNSS for which data should be

returned. The currently supported GNSS are: gps and glo.

period Numeric Optional The number of weeks into the future the data should be valid

[weeks] for. Data can be requested for up to 5 weeks in to the future. If

this value is not provided, the server assumes a period of 4

weeks.

resolution Numeric Optional The resolution of the data: 1=every day, 2=every other day,

[days] 3=every third day. If this value is not provided, the server

assumes a resolution of 1 day.

Thus, as an example, a valid parameter string would be:

token=XXXXXXXXXXXXXXXXXXXXXX;gnss=gps,glo;

12.4.2 Authorization

The AssistNow Offline Service uses the same authorization process as AssistNow Online; see

above for details.

12.4.3 Multiple Servers

The AssistNow Offline Service uses the same multiple server mechanism to provide high

availability as AssistNow Online; see above for details.

12.4.4 Time, Position and Almanac

While AssistNow Offline can be used on its own, it is expected that the user will provide estimates

of the receiver's current position, the current time and ensure that a reasonably up to date

almanac is available. In most cases this information is likely to be available without the user

needing to do anything. For example, where the receiver is connected to a battery backup power

supply and has a functioning real time clock (RTC), the receiver will keep its own sense of time and

will retain the last known position and any almanac. However, should the receiver be completely

unpowered before startup, then it will greatly improve TTFF if time, position and almanac can be

supplied in some form.

Almanac data has a validity period of several weeks, so it can be downloaded from the AssistNow

UBX-13003221 - R28 Early production information Page 48 of 460

## Page 64

u-blox 8 / u-blox M8 Receiver description - Manual

Online service at roughly the same time the Offline data is obtained. It can then be stored in the

host for uploading on receiver startup, or it can be transferred to the receiver straight away and

preserved there (provided suitable non-volatile storage is available).

Obviously, where a receiver has a functioning RTC, it should be able to keep its own sense of time,

but where no RTC is fitted (or power is completely turned off), providing a time estimate via the

UBX-MGA-INI-TIME_UTC message will be beneficial.

Similarly, where a receiver has effective non-volatile storage, the last known position will be

recalled, but if this is not the case, then it will help TTFF to provide a position estimate via one of

the UBX-MGA-INI-POS_XYZ or UBX-MGA-INI-POS_LLH messages.

Where circumstance prevent the provision of all three of these pieces of data, providing some is

likely to be better than none at all.

12.4.5 Flash-based AssistNow Offline

Flash-based AssistNow Offline functionality means that AssistNow Offline data is stored in the

flash memory connected to the chip.

The user's host system must download the data from the AssistNow Offline service when an

internet connection is available, and then deliver all of that data to the u-blox receiver. As the total

amount of data to be uploaded is large (typically around 100 kbytes) and writing to flash memory

is slow, the upload must be done in blocks of up to 512 bytes, one at a time. The UBX-MGA-FLASH-

DATA message is used to transmit each block to the receiver.

AssistNow Offline data stored in flash memory is not affected by any reset of the

receiver. The only simple ways to clear it are to completely erase the whole flash memory

or to overwrite it with a new set of AssistNow Offline data. Uploading a dummy block of

data (e.g. all zeros) will also have the effect of deleting the data, although a small

amount of flash storage will be used.

12.4.5.1 Flash-based Storage Procedure

The following steps are a typical sequence for transferring AssistNow Offline data into the

receiver's flash memory:

• The host downloads a copy of a latest data from the AssistNow Offline service and stores it

locally.

• It sends the first 512 bytes of that data using the UBX-MGA-FLASH-DATA message.

• It awaits a UBX-MGA-FLASH-ACK message in reply.

• Based on the contents of the UBX-MGA-FLASH-ACK message it, sends the next block, resends

the last block or aborts the whole process.

• The above three steps are repeated until all the rest of the data has been successfully

transferred (or the process has been aborted).

• The host sends an UBX-MGA-FLASH-STOP message to indicate completion of the upload.

• It awaits the final UBX-MGA-FLASH-ACK message in reply. Background processing in the receiver

prepares the downloaded data for use at this stage. Particularly if the receiver is currently busy,

this may take quite a few seconds, so the host has to be prepared for a delay before the UBX-

MGA-FLASH-ACK is seen.

Note that the final block may be smaller than 512 bytes (where the total data size is not perfectly

divisible by 512). Also, the UBX-MGA-FLASH-ACK messages are distinct from the UBX-MGA-ACK

messages used for other AssistNow functions.

UBX-13003221 - R28 Early production information Page 49 of 460

## Page 65

u-blox 8 / u-blox M8 Receiver description - Manual

Any existing data will be deleted as soon as the first block of new data arrives, so no useful data

will be available till the completion of the data transfer. Each block of data has a sequence

number, starting at zero for the first block. In order to guard against invalid partial data

downloads the receiver will not accept blocks which are out of sequence.

12.4.6 Host-based AssistNow Offline

Host-based AssistNow Offline involves AssistNow Offline data being stored until it is needed by

the user's host system in whatever memory it has available.

The user's host system must download the data from the AssistNow Offline service when an

internet connection is available, but retain it until the time the u-blox receiver needs it. At this

point, the host must upload just the relevant portion of the data to the receiver, so that the

receiver can start using it. This is achieved by parsing all the data and selecting for upload to the

receiver only those UBX-MGA-ANO messages with a date-stamp nearest the current time. As each

is a complete UBX message it can be sent directly to the receiver with no extra packaging. If

required the user can select to employ flow control, but in most cases this is likely to prove

unnecessary.

When parsing the data obtained from the AssistNow Offline service the following points should be

noted:

• The data is made up of a sequence of UBX-MGA-ANO messages.

• Customers should not rely on the messages all being of a fixed size, but should read their length

from the UBX header to work out where the message ends (and where the next begins).

• Each message indicates the SV for which it is applicable through the svId and gnssId fields.

• Each message contains a date-stamp within the year, month and day fields.

• Midday (UTC) on the day indicated should be considered to be the point at which the data is

most applicable.

• The messages will be ordered chronologically, earliest first.

• Messages with same date-stamp will be ordered by ascending gnssId and then ascending svId.

12.4.6.1 Host-based Procedure

The following steps are a typical sequence for host-based AssistNow Offline:

• The host downloads a copy of the latest data from the AssistNow Offline service and stores it

locally.

• Optionally it may also download a current set of almanac data from the AssistNow Online

service.

• It waits until it wants to use the u-blox receiver.

• If necessary it uploads any almanac, position estimate and/or time estimate to the receiver.

• The host scans through AssistNow Offline data looking for entries with a date-stamp that most

closely matches the current (UTC) time/date.

• The host sends each such UBX-MGA-ANO message to the receiver.

Note that when data has been downloaded from the AssistNow Offline service with the (default)

resolution of one day, the means for selecting the closest matching date-stamp is simply to look

for ones with the current (UTC) date.

UBX-13003221 - R28 Early production information Page 50 of 460

## Page 66

u-blox 8 / u-blox M8 Receiver description - Manual

12.5 Preserving Information During Power-off

The performance of u-blox receivers immediately after they are turned on is enhanced by

providing them with as much useful information as possible. Assistance (both Online and Offline)

is one way to achieve this, but retaining information from previous use of the receiver can be just

as valuable. All the types of data delivered by assistance can be retained while the receiver is

powered down for use when power is restored. Obviously the value of this data will diminish as

time passes, but in many cases it remains very useful and can significantly improve time to first

fix.

The are several ways in which a u-blox receiver can retain useful data while it is powered down,

including:

• Battery Backed RAM: The receiver can be supplied with sufficient power to maintain a small

portion of internal storage, while it is otherwise turned off. This is the best mechanism, provided

that the small amount of electrical power required can be supplied continuously.

• Save on Shutdown: The receiver can be instructed to dump its current state to the attached

flash memory (where fitted) as part of the shutdown procedure; this data is then automatically

retrieved when the receiver is restarted. See the description of the UBX-UPD-SOS messages for

more information.

• Database Dump: The receiver can be asked to dump the state of its internal database in the

form of a sequence of UBX messages reported to the host; these messages can be stored by

the host and then sent back to the receiver when it has been restarted. See the description of

the UBX-MGA-DBD messages for more information.

12.6 AssistNow Autonomous

(Note: some functionality described in this chapter may not be available in protocol versions less

than 18).

12.6.1 Introduction

The assistance scenarios covered by AssistNow Online and AssistNow Offline require an online

connection and a host that can use this connection to download aiding data and provide this to

the receiver when required.

The AssistNow Autonomous feature provides a functionality similar to AssistNow Offline without

the need for a host and a connection. Based on a broadcast ephemeris downloaded from the

satellite (or obtained by AssistNow Online) the receiver can autonomously (i.e. without any host

interaction or online connection) generate an accurate satellite orbit representation («AssistNow

Autonomous data») that is usable for navigation much longer than the underlying broadcast

ephemeris was intended for. This makes downloading new ephemeris or aiding data for the first

fix unnecessary for subsequent start-ups of the receiver.

The AssistNow Autonomous feature is disabled by default. It can be enabled using the

UBX-CFG-NAVX5 message.

12.6.2 Concept

The figure below illustrates the AssistNow Autonomous concept in a graphical way. Note that the

figure is a qualitative illustration and is not to scale.

• A broadcast ephemeris downloaded from the satellite is a precise representation of a part (for

GPS nominally four hours) of the satellite's true orbit (trajectory). It is not usable for positioning

UBX-13003221 - R28 Early production information Page 51 of 460

## Page 67

u-blox 8 / u-blox M8 Receiver description - Manual

beyond this validity period because it diverges dramatically from the true orbit afterwards.

• The AssistNow Autonomous orbit is an extension of one or more broadcast ephemerides. It

provides a long-term orbit for the satellite for several revolutions. Although this orbit is not

perfectly precise it is a sufficiently accurate representation of the true orbit to be used for

navigation.

• The AssistNow Autonomous data is automatically and autonomously generated from

downloaded (or assisted) ephemerides. The data is stored automatically in the on-chip battery-

backed memory (BBR). Optionally, the data can be backed-up in external flash memory or on the

host. The number of satellites for which data can be stored depends on the receiver

configuration and may change during operation.

• If no broadcast ephemeris is available for navigation AssistNow Autonomous automatically

generates the required parts of the orbits suitable for navigation from the stored data. The data

is also automatically kept current in order to minimize the calculation time once the navigation

engine needs orbits.

• The operation of the AssistNow Autonomous feature is transparent to the user and the

operation of the receiver. All calculations are done in background and do not affect the normal

operation of the receiver.

• The AssistNow Autonomous subsystem automatically invalidates data that has become too old

and that would introduce unacceptable positioning errors. This threshold is configurable (see

below).

• The prediction quality will be automatically improved if the satellite has been observed multiple

times. However, this requires the availability of a suitable flash memory (see the integration

manual for a list of supported devices). Improved prediction quality also positively affects the

maximum usability period of the data.

• AssistNow Autonomous considers GPS, GLONASS, Galileo and BeiDou satellites only. It will not

consider satellites on orbits with an eccentricity of >0.05 (e.g., Galileo E18). For GLONASS

support a suitable flash memory is mandatory because a single broadcast ephemeris spans to

little of the orbit (only approx. 30 minutes) in order to extend it in a usable way. Only multiple

observations of the same GLONASS satellite that span at least four hours will be used to

generate data.

UBX-13003221 - R28 Early production information Page 52 of 460

## Page 68

u-blox 8 / u-blox M8 Receiver description - Manual

12.6.3 Interface

Several UBX protocol messages provide interfaces to the AssistNow Autonomous feature. They

are:

• The UBX-CFG-NAVX5 message is used to enable or disable the AssistNow Autonomous feature.

It is disabled by default. Once enabled, the receiver will automatically produce AssistNow

Autonomous data for newly received broadcast ephemerides and, if that data is available,

automatically provide the navigation subsystem with orbits when necessary and adequate. The

message also allows for a configuration of the maximum acceptable orbit error. See the next

section for an explanation of this feature. It is recommended to use the firmware default value

that corresponds to a default orbit data validity of approximately three days (for GPS satellites

observed once) and up to six days (for GPS and GLONASS satellites observed multiple times

over a period of at least half a day).

• Note that disabling the AssistNow Autonomous feature will delete all previously collected

satellite observation data from the flash memory.

• The UBX-NAV-AOPSTATUS message provides information on the current state of the AssistNow

Autonomous subsystem. The status indicates whether the AssistNow Autonomous subsystem

is currently idle (or not enabled) or busy generating data or orbits. Hosts should monitor this

information and only power-off the receiver when the subsystem is idle (that is, when the

status field shows a steady zero).

• The UBX-NAV-SAT message indicates the use of AssistNow Autonomous orbits for individual

satellites.

UBX-13003221 - R28 Early production information Page 53 of 460

## Page 69

u-blox 8 / u-blox M8 Receiver description - Manual

• The UBX-NAV-ORB message indicates the availability of AssistNow Autonomous orbits for

individual satellites.

• The UBX-MGA-DBD message provides a means to retrieve the AssistNow Autonomous data from

the receiver in order to preserve the data in power-off mode where no battery backup is

available. Note that the receiver requires the absolute time (i.e. full date and time) to calculate

AssistNow Autonomous orbits. For best performance it is, therefore, recommended to supply

this information to the receiver using the UBX-MGA-INI-TIME_UTC message in this scenario.

• The Save-on-Shutdown feature preserves AssistNow Autonomous data.

12.6.4 Benefits and Drawbacks

AssistNow Autonomous can provide quicker start-up times (lower the TTFF) provided that data is

available for enough visible satellites. This is particularly true under weak signal conditions where

it might not be possible to download broadcast ephemerides at all, and, therefore, no fix at all

would be possible without AssistNow Autonomous (or A-GNSS). It is, however, required that the

receiver roughly knows the absolute time, either from an RTC or from time-aiding (see the

Interface section above), and that it knows which satellites are visible, either from the almanac or

from tracking the respective signals.

The AssistNow Autonomous orbit (satellite position) accuracy depends on various factors, such

as the particular type of satellite, the accuracy of the underlying broadcast ephemeris, or the

orbital phase of the satellite and Earth, and the age of the data (errors add up over time).

AssistNow Autonomous will typically extend a broadcast ephemeris for up to three to six days.

The UBX-CFG-NAVX5 (see above) message allows changing this threshold by setting the

«maximum acceptable modelled orbit error» (in meters). Note that this number does not reflect

the true orbit error introduced by extending the ephemeris. It is a statistical value that represents

a certain expected upper limit based on a number of parameters. A rough approximation that

relates the maximum extension time to this setting is: maxError [m] = maxAge [d] * f, where the

factor f is 30 for data derived from satellites seen once and and 16 for data derived for satellites

seen multiple times during a long enough time period (see the Concept section above).

There is no direct relation between (true and statistical) orbit accuracy and positioning accuracy.

The positioning accuracy depends on various factors, such as the satellite position accuracy, the

number of visible satellites, and the geometry (DOP) of the visible satellites. Position fixes that

include AssistNow Autonomous orbit information may be significantly worse than fixes using only

broadcast ephemerides. It might be necessary to adjust the limits of the Navigation Output

Filters.

A fundamental deficiency of any system to predict satellite orbits precisely is unknown future

events. Hence, the receiver will not be able to know about satellites that will have become

unhealthy, have undergone a clock swap, or have had a manoeuvre. This means that the

navigation engine might rarely mistake a wrong satellite position as the true satellite position.

However, provided that there are enough other good satellites, the navigation algorithms will

eventually eliminate a defective orbit from the navigation solution.

The repeatability of the satellite constellation is a potential pitfall for the use of the AssistNow

Autonomous feature. For a given location on Earth the (GPS) constellation (geometry of visible

satellites) repeats every 24 hours. Hence, when the receiver «learned» about a number of satellites

at some point in time the same satellites will in most places not be visible 12 hours later, and the

available AssistNow Autonomous data will not be of any help. Again 12 hours later, however, usable

data would be available because it had been generated 24 hours ago.

UBX-13003221 - R28 Early production information Page 54 of 460

## Page 70

u-blox 8 / u-blox M8 Receiver description - Manual

The longer a receiver observes the sky the more satellites it will have seen. At the equator, and

with full sky view, approximately ten (GPS) satellites will show up in a one hour window. After four

hours of observation approx. 16 satellites (i.e. half the constellation), after 10 hours approx. 24

satellites (2/3rd of the constellation), and after approx. 16 hours the full constellation will have

been observed (and AssistNow Autonomous data generated for). Lower sky visibility reduces

these figures. Further away from the equator the numbers improve because the satellites can be

seen twice a day. E.g. at 47 degrees north the full constellation can be observed in approx. 12 hours

with full sky view.

The calculations required for AssistNow Autonomous are carried out on the receiver. This requires

energy and users may therefore occasionally see increased power consumption during short

periods (several seconds, rarely more than 60 seconds) when such calculations are running.

Ongoing calculations will automatically prevent the power save mode from entering the power-off

state. The power-down will be delayed until all calculations are done.

The AssistNow Offline and AssistNow Autonomous features are exclusive and should

not be used at the same time. Every satellite will be ignored by AssistNow Autonomous if

there is AssistNow Offline data available for it.

13 Power Management

u-blox receivers support different power modes. These modes represent strategies of how to

control the acquisition and tracking engines in order to achieve either the best possible

performance or good performance with reduced power consumption.

Receiver power management can split into two categories:

• Externally Controlled Power Management: This includes various modes of power management

that are directly operated by the user or host device. These modes are: 1. External cycling of the

receiver main power supply. 2. Instruct the receiver to turn On/Off via the UBX-RXM-PMREQ

message. 3. Instruct the receiver to turn On/Off via external pins (EXTINT0 or EXTINT1).

• Internally Controlled Power Management: Here the receiver makes the decision when to power

down/up some/all of its internal components according to predefined parameters. It is also

referred to as Power Save Modes (PSM). In PSM one of three modes of operations can be

selected (not all are supported in a single firmware): 1. ON/OFF Operation (PSMOO) 2. Cyclic

Tracking (PSMCT) 3. Super-Efficient Mode (Super-E).

The following figure illustrates u-blox power management modes.

UBX-13003221 - R28 Early production information Page 55 of 460

## Page 71

u-blox 8 / u-blox M8 Receiver description - Manual

u-blox Power Management

The majority of the Power Management section is detailing the Power Save Mode (Internally

Controlled Power Management). However, some the concepts relevant to the Externally

Controlled Power Management are detailed, such as the EXTINT Control, Wake up and Power

On/Off Command.

Externally controlled power management operations can be used on top of the Internally

Controlled Power Management and they do override their operation.

13.1 Continuous Mode

u-blox receivers make use of dedicated signal processing engines optimized for signal acquisition

and tracking. The acquisition engine delivers rapid signal searches during cold starts or when

insufficient signals are available for navigation. The tracking engine delivers signal measurements

for navigation and acquires new signals as they become available during navigation. The

resources of both engines are deployed adaptively to minimize overall power consumption.

13.2 Power Save Mode

Power Save Mode (PSM) allows a reduction in system power consumption by selectively switching

parts of the receiver on and off. It is selected using the message UBX-CFG-RXM and configured

using UBX-CFG-PM2. It is recommended to use UBX-CFG-PMS instead if available (only supported in

protocol versions 18+) as it provides a simplified interface; see section Power mode setup for

details.

PSM is designed to only support the operation of GPS, GLONASS, BeiDou, Galileo and QZSS.

Enabling SBAS or IMES is possible only if at least one of the other systems is enabled. The PSM

state machine behavior will not be altered by enabling SBAS or IMES and it will not take them into

account in operation. Therefore, it is recommended to disable them (i.e., SBAS or IMES) when

operating in Power Save Mode. They can be disabled using UBX-CFG-GNSS.

The logic within Power Save Mode is designed so that Time Pulse operation is not

compromised. This means that entering all power saving states is delayed until the

conditions necessary to produce a Time Pulse have been met. Therefore, in order to

obtain good Power Save Mode operation, it is essential that any Time Pulse is correctly

UBX-13003221 - R28 Early production information Page 56 of 460

## Page 72

u-blox 8 / u-blox M8 Receiver description - Manual

configured with an appropriate time base, or that Time Pulses are turned off if not

needed (by clearing the active flag in UBX-CFG-TP5).

For protocol versions less than 18: Power Save Mode can only be selected with GPS

signals. Other GNSS are not supported.

Note: Power Save Mode is not supported in conjunction with the ADR, UDR and FTS

products.

13.2.1 Operation

Power Save Mode has two modes of operation:

• Power Save Mode Cyclic Tracking (PSMCT) Operation is used when position fixes are required in

short periods of 1 to 10s. In receivers that support Super-E Mode, Super-E replaces Cyclic

Tracking.

• Power Save Mode ON/OFF (PSMOO) Operation is used for periods longer than 10s, and can be in

the order of minutes, hours or days. (Not supported in protocol versions 23 to 23.01)

The mode of operation can be configured, and depending on the setting, the receiver

demonstrates different behavior: In ON/OFF operation the receiver switches between phases of

start-up/navigation and phases with low or almost no system activity (backup/sleep). In cyclic

tracking the receiver does not shut down completely between fixes, but uses low power tracking

instead.

Currently PSMCT is restricted to update period between 1 and 10 seconds and PSMOO is

restricted to update period over 10 seconds. However, this may change in future firmware

releases.

PSM is based on a state machine with five different states: (Inactive) Awaiting Next Fix and

(Inactive) Awaiting Next Search states, Acquisition state, Tracking state and Power Optimized

Tracking (POT) state.

• Inactive states: Most parts of the receiver are switched off.

• Acquisition state: The receiver actively searches for and acquires signals. Maximum power

consumption.

• Tracking state: The receiver continuously tracks and downloads data. Less power consumption

than in Acquisition state.

• POT state: The receiver repeatedly loops through a sequence of tracking (Track), calculating the

position fix (Fix), and entering an idle period (Idle). No new signals are acquired and no data is

downloaded. Much less power consumption than in Tracking state.

The following figure illustrates the PSM state machine:

UBX-13003221 - R28 Early production information Page 57 of 460

## Page 73

u-blox 8 / u-blox M8 Receiver description - Manual

State machine

13.2.1.1 Acquisition Timeout Logic

The receiver has internal, external and user-configurable mechanisms that determine the time to

be spent in acquisition state. This logic is put in place to ensure good performance and low power

consumption in different environments and scenarios. This collective logic is referred to as

Acquisition Timeout.

Internal mechanisms:

• If the receiver is able to acquire weak signals but not of the quality needed to get a fix, it will

transition to (Inactive) Awaiting Next Search state after the timeout configured in

maxStartupStateDur or earlier if too few signals are acquired.

• If the receiver is unable to acquire any signals or it acquires a small number of extremely bad

signals (e.g., no sky view), it will transition to (Inactive) Awaiting Next search state after 15

seconds or the timeout configured in maxStartupStateDur if shorter.

User-configurable mechanisms:

• minAcqTime is the minimum time that the receiver will spend in Acquisition state (see

minAcqTime for details.)

• maxStartupStateDur is the maximum time that the receiver will spend in Acquisition state (see

UBX-13003221 - R28 Early production information Page 58 of 460

## Page 74

u-blox 8 / u-blox M8 Receiver description - Manual

maxStartupStateDur for details).

• doNotEnterOff forces the receiver to stay awake and in Acquisition state even when a fix is not

possible (see doNotEnterOff for details).

External mechanisms:

• The receiver will be forced to stay awake if extintWake is enabled and the configured EXTINT pin

is set to "high" and it will be forced to stay in (Inactive) Awaiting Next Search/Fix states if

extintBackup is enabled and the configured EXTINT pin is set to "low" (see EXTINT pin control

for details).

13.2.1.2 ON/OFF operation - long update period

(Not supported in protocol versions 23 to 23.01).

When the receiver is switched on, it first enters Acquisition state. If it is able to obtain a valid

position fix within the time given by the Acquisition Timeout, it switches to Tracking state.

Otherwise it enters (Inactive) Awaiting Next Search state and re-starts after the configured

search period (minus a start-up margin). As soon as the receiver gets a valid position fix (one

passing the navigation output filters), it enters Tracking state. Upon entering Tracking state, the

onTime starts. Once the onTime is over, (Inactive) Awaiting Next Fix state is entered and the

receiver re-starts according to the configured update grid (see section Grid offset for an

explanation). If the signal is lost while in Tracking state, Acquisition state is entered. If the signal is

not found within the acquisition timeout, the receiver enters (Inactive) Awaiting Next Search state.

Otherwise the receiver will re-enter Tracking state and stay there until the newly started onTime is

over.

The diagram below illustrates how ON/OFF operation works:

Diagram of ON/OFF operation

13.2.1.3 Cyclic tracking operation - short update period

When the receiver is switched on, it first enters Acquisition state. If it is able to obtain a position

fix within the time given by the acquisition timeout, it switches to Tracking state. Otherwise, it will

enter (Inactive) Awaiting Next Search state and re-start within the configured search grid. After a

valid position fix, Tracking state is entered and the onTime starts. In other words the onTime

starts with the first valid position fix. Once the onTime is over, POT state is entered. In POT state

the receiver continues to output position fixes according to the updatePeriod. To have maximum

power savings, set the onTime to zero. This causes the receiver to enter POT state as soon as

possible. If the signal becomes weak or is lost during POT state, Tracking state is entered. Once

the signal is good again and the newly started onTime is over, the receiver will re-enter POT state.

If the receiver can't get a position fix in the Tracking state, it enters Acquisition state. Should the

acquisition fail as well, (Inactive) Awaiting Next Search state is entered. If doNotEnterOff is

UBX-13003221 - R28 Early production information Page 59 of 460

## Page 75

u-blox 8 / u-blox M8 Receiver description - Manual

enabled and no fix is possible, the receiver will remain in Acquisition state until a fix is possible and

it will never enter (Inactive) Awaiting Next Search state.

The diagram below illustrates how cyclic tracking operation works:

Diagram of cyclic tracking operation

13.2.1.4 Super-Efficient Mode

(Not supported in protocol versions less than 23).

Super-Efficient (Super-E) Mode is a power efficient mode of operation that replaces and improves

on cyclic tracking Power Save Mode (PSMCT). It uses improved clocking techiques to reduce

power consumption and more sophisticated decision making for switching between "Acquisition",

"Tracking" and "Power Optimized Tracking" states. This mode was developed and optimized to

provide a good compromise between power efficiency and positioning accuracy in wearable

applications.

13.2.1.5 User controlled operation - update and search period of zero

Setting the updatePeriod to zero causes the receiver to wait in the (Inactive) Awaiting Next Fix

state until woken up by the user. Setting the search period to zero causes the receiver to wait in

the (Inactive) Awaiting Next Search state indefinitely after an unsuccessful start-up. Any wake-up

event will re-start the receiver. See section Wake up for more information on wake-up events.

External wake-up is required when setting update or search period to zero.

13.2.1.6 Satellite data download

The receiver is not able to download satellite data (e.g. the ephemeris) while it is working in

ON/OFF or cyclic tracking operation. Therefore it has to temporarily switch to continuous

operation for the time the satellites transmit the desired data. To save power the receiver

schedules the downloads according to an internal timetable and only switches to continuous

operation while data of interest is being transmitted by the satellites.

Each SV transmits its own ephemeris data. Ephemeris data download is feasible when the

corresponding satellite has been tracked with a sufficient C/No over a certain period of time. The

download is scheduled in a 30 minute grid or immediately when fewer than a certain number of

visible satellites have valid ephemeris data.

Almanac, ionosphere, UTC correction and SV health data are transmitted by all SVs

simultaneously. Therefore these parameters can be downloaded when a single SV is tracked with

a high enough C/No.

Allowing more ephemerides to be downloaded before going into POT or (Inactive) Awaiting Next Fix

state can help improve the quality of the fixes and reduce the number of wake ups needed to

UBX-13003221 - R28 Early production information Page 60 of 460

## Page 76

u-blox 8 / u-blox M8 Receiver description - Manual

download ephemerides at the cost of extra time in Acquisition state (only when an inadequate

number of ephemerides are downloaded from tracked satellites).

13.2.2 Configuration

Power Save Mode is enabled and disabled with the UBX-CFG-RXM message and configured with the

UBX-CFG-PM2 message.

When enabling Power Save Mode, the receiver will be unable to download or process any

SBAS or IMES data. Therefore, there is no benefit in enabling them and it is

recommended to disable both systems. SBAS support and IMES support can be

disabled using UBX-CFG-GNSS.

A number of parameters can be used to customize PSM to any specific needs. These parameters

are listed in the following table:

Power Save Mode configuration options on UBX-CFG-PM2

Parameter Description

mode Receiver mode of operation

updatePeriod Time between two position fix attempts

searchPeriod Time between two acquisition attempts if the receiver is unable to get a

position fix

minAcqTime Minimum time the receiver spends in Acquisition state

onTime Time the receiver remains in Tracking state and produces position fixes

waitTimeFix Wait for time fix before entering Tracking state

doNotEnterOff Receiver does not enter (Inactive) Awaiting Next Search state if it can't get

a position fix but keeps indefinitely attempting a position fix instead

updateRTC Enables periodic Real Time Clock (RTC) update

updateEPH Enables periodic ephemeris update

extintSelect Selects EXTINT pin used with pin control feature

extintWake Enables force-ON pin control feature

extintBackup Enables force-OFF pin control feature

gridOffset Time offset of update grid with respect to start of week

maxStartupStateDur Maximum time in Acquisition state

optTarget The PSM settings will be weighed towards a specific target (only

supported in protocol versions 23 to 23.01)

13.2.2.1 Mode of operation (mode)

The mode of operation to use mainly depends on the update period: For short update periods (in

the range of a few seconds), cyclic tracking should be configured. For long update periods (in the

range of minutes or longer), only use ON/OFF operation.

See section ON/OFF operation - long update period and Cyclic tracking operation - short update

period for more information on the two modes of operation.

13.2.2.2 Reference Time Standard

In older versions ( in protocol versions less than 18), only GPS can be configured for PSM,

therefore, GPS time standard is used for the operation of PSM. Whereas, in newer versions where

multiple GNSS can operate simultaneously ( in protocol versions 18+), UTC time standard is used.

UBX-13003221 - R28 Early production information Page 61 of 460

## Page 77

u-blox 8 / u-blox M8 Receiver description - Manual

13.2.2.3 Update period (updatePeriod) and search period (searchPeriod)

The update period specifies the time between successive position fixes. If no position fix can be

obtained within the acquisition timeout, the receiver will retry after the time specified by the

search period. Update and search periods are fixed with respect to an absolute time grid based on

reference time standard (i.e., GPS Time or UTC. see Reference Time Standard). They do not refer

to the time of the last valid position fix or last position fix attempt.

New settings are ignored if the update period or the search period exceeds the maximum

number of milliseconds in a week. In that case the previously stored values remain

effective.

13.2.2.4 Minimum Acquisition Time (minAcqTime)

The receiver tries to obtain a position fix for at least the time given in minAcqTime. If the receiver

determines that it needs more time for the given starting conditions then it will automatically

prolong this time. If minAcqTime is set to zero then the minimum acquisition time is exclusively

determined by the receiver. Once the minAcqTime has expired, the receiver will terminate the

acquisition state if either a fix is achieved or if the receiver estimates that any signals received are

insufficient (too weak or too few) for a fix to be possible.

13.2.2.5 On time (onTime)

The onTime parameter specifies how long the receiver stays in Tracking state before switching to

the POT state (in PSMCT) or (Inactive) Awaiting Next Fix state (in PSMOO).

13.2.2.6 Wait for time fix (waitTimeFix)

A time fix is a fix type in which the receiver will ensure that the time is accurate and confirmed to

within the limits set in UBX-CFG-NAV5. Enabling the waitTimeFix option will force the receiver to

stay in Acquisition state until the time is known to within the configured limits then it will

transition to Tracking state. Enabling waitTimeFix will delay the transition from Acquisition state

to Tracking state by at least two extra seconds, thus, this should be taken into account (see

Acquisition Timeout). It is necessary to enable waitTimeFix in timing products.

The quality of the position fixes can also be configured by setting the limits in the message UBX-

CFG-NAV5. Setting harder limits in UBX-CFG-NAV5 will typically prolong the time in Acquisition

state. Thus, ensuring sufficient time is given to the receiver at start-up (when externally

controlled) is necessary (see Acquisition Timeout Logic). When internally controlled, the receiver

can make good judgement on the time needed in Acquisition state and no further adjustments will

be needed.

13.2.2.7 Maximum Startup State Duration (maxStartupStateDur)

(Only supported in protocol versions 17+).

The maxStartupStateDur is the maximum time that the receiver will spend in Startup state (i.e.,

Acquisition state). If the receiver is unable to acquire a valid position fix within this maximum time,

it will transition to (Inactive) Awaiting Next Search state (if doNotEnterOff is disabled).

Subsequently, the receiver will attempt to acquire another position fix according to the search

period (see Update period (updatePeriod) and search period (searchPeriod)). If

maxStartupStateDur is set to zero, the receiver will autonomously determine the maximum time

to spend in Acquisition state. Note that shorter settings (below about 45s) will degrade an

unaided receiver's ability to collect new Ephemeris data at low signal levels (see section Satellite

UBX-13003221 - R28 Early production information Page 62 of 460

## Page 78

u-blox 8 / u-blox M8 Receiver description - Manual

data download).

13.2.2.8 Do not enter '(Inactive) Awaiting Next Search' state when no fix (doNotEnterOff)

If this option is enabled, the receiver acts differently in case it cannot get a fix: instead of entering

(Inactive) Awaiting Next Search state, it keeps attempting to acquire a position fix. In other words,

the receiver will never be in (Inactive) Awaiting Next Search state and therefore searchPeriod and

minAcqTime will be ignored.

13.2.2.9 Update RTC (updateRTC) and Ephemeris (updateEPH)

To maintain the ability of a fast start-up, the receiver needs to calibrate its RTC and update its

ephemeris data on a regular basis. This can be ensured by activating the update RTC and update

Ephemeris option. The RTC is calibrated every 5 minutes and the ephemeris data is updated

approximately every 30 minutes. See section Satellite data download for more information.

13.2.2.10 EXTINT pin control

The operation of PSM can be externally controlled using either EXTINT0 or EXTINT1 pin. This

external control allows the user to decide when to wake up the receiver to obtain a fix and when to

force the receiver into sleep/backup mode to save power. Operating the receiver externally

through the EXTINT pins will override internal functions that coincide with that specific operation.

The choice of which pin to use can be configured through the extintSelect feature in UBX-CFG-PM2.

Only one pin can be selected at a time but it is sufficient to perform all the required tasks.

If the Force-ON (extintWake) feature in UBX-CFG-PM2 is enabled, the receiver will not enter Inactive

states for as long as the configured EXTINT pin (EXTINT0 or EXTINT1) is at 'high' level. The

receiver will therefore always be in Acquisition/Tracking state in PSMOO or in

Acquisition/Tracking/POT state in PSMCT. When the pin level changes to 'low' the receiver will

continue with its configured behavior.

If the Force-OFF (extintBackup) feature in UBX-CFG-PM2 is enabled, the receiver will enter Inactive

states for as long as the configured EXTINT pin is set to 'low' until the next wake up event. Any

wake-up event can wake up the receiver even while the EXTINT pin is set to 'low' (see Wake up).

However, if the pin stays at 'low' state, the receiver will only wake up for the time needed to read

the configuration pin settings then it will enter the Inactive state again.

If both Force-ON and Force-OFF features are enabled at the same time, the receiver PSM

operation will be completely in user control. Setting 'high' on the configured EXTINT pin will wake

up the receiver to get a position fix and setting 'low' will put the receiver into sleep/backup mode.

13.2.2.11 Grid offset (gridOffset)

Once the receiver has a valid time, the update grid is aligned to the start of the week of the

reference time standard (midnight between Saturday and Sunday). Before having a valid time, the

update grid is unaligned. A grid offset shifts the update grid with respect to the start of the week

of the reference time standard. An example of usage can be found in section Use grid offset.

The grid offset is not used in cyclic tracking operation.

UBX-13003221 - R28 Early production information Page 63 of 460

## Page 79

u-blox 8 / u-blox M8 Receiver description - Manual

13.2.2.12 Optimization target

In cyclic tracking operation, the behavior of the receiver can be tuned even more closely to the

application's need by choosing an appropriate optimization target.

In protocol version 23.01 two optimization targets are available:

• Performance: The receiver achieves a good GNSS performance while keeping the power

consumption low.

• Power save: The receiver might sacrifice GNSS performance in favor of a reduced power

consumption.

13.2.3 Features

13.2.3.1 Communication

When PSM is enabled, communication with the receiver (e.g. UBX message to disable PSM)

requires particular attention. This is because the receiver may be in Inactive state and therefore

unable to receive any message through its interfaces. To ensure that the configuration messages

are processed by the receiver, even while in Inactive state, the following steps need to be taken:

• Send a dummy sequence of 0xFF (one byte is sufficient) to the receiver's UART interface. This

will wake up the receiver if it is in Inactive state. If the receiver is not in Inactive state, the

sequence will be ignored.

• Send the configuration message about half a second after the dummy sequence. If the interval

between the dummy sequence and the configuration message is too short, the receiver may not

yet be ready. If the interval is too long, the receiver may return to Inactive state before the

configuration message was received. It is therefore important to check for a UBX-ACK-ACK reply

from the receiver to confirm that the configuration message was received.

• Send the configuration save message immediately after the configuration message.

Similarly, when configuring the receiver for PSMOO (and PSMCT when doNotEnterOff is disabled),

ensure that the configurations are saved. If they are not saved the receiver will enter backup mode

and when it wakes up again, it would have lost the configurations and even forgets it was in power

save mode. This can be avoided by using the UBX-CFG-CFG message (see Receiver Configuration

for details). When operating PSM from u-center and setting the receiver to Power Save Mode in

UBX-CFG-RXM, check the save configuration box. u-center will then send a UBX-CFG-CFG message

after the UBX-CFG-RXM to save the configurations.

13.2.3.2 Wake up

The receiver can be woken up by generating an edge on one of the following pins:

• rising or falling edge on one of the EXTINT pins

• rising or falling edge on the RXD1 pin

• rising or falling edge on the SPI CS pin

• rising edge on NRESET pin

All wake-up signals are interpreted as a position request, where the receiver wakes up and tries to

obtain a position fix. Wake-up signals have no effect if the receiver is already in Acquisition,

Tracking or POT state.

UBX-13003221 - R28 Early production information Page 64 of 460

## Page 80

u-blox 8 / u-blox M8 Receiver description - Manual

13.2.3.3 Behavior while USB host connected

As long as the receiver is connected to a USB host, it will not enter the lowest possible power

state. This is because it must retain a small level of CPU activity to avoid breaching requirements

of the USB specification. The drawback, however, is that power consumption is higher.

Wake up by pin/UART is possible even if the receiver is connected to a USB host. In this

case the state of the pin must be changed for a duration longer than one millisecond.

13.2.3.4 Cooperation with the AssistNow Autonomous feature

If both PSM and AssistNow Autonomous features are enabled, the receiver will not enter (Inactive)

Awaiting Next Fix state as long as AssistNow Autonomous carries out calculations. This prevents

losing data from unfinished calculations and, in the end, reduces the total extra power needed for

AssistNow Autonomous. The delay before entering (Inactive) Awaiting Next Fix state, if any, will be

in the range of several seconds, rarely more than 20 seconds.

Only entering (Inactive) Awaiting Next Fix state is affected by AssistNow Autonomous. In other

words: in cyclic tracking operation, AssistNow Autonomous will not interfere with the PSM (apart

from the increased power consumption).

Enabling the AssistNow Autonomous feature will lead to increased power consumption

while prediction is calculated. The main goal of PSM is to reduce the overall power

consumption. Therefore for each application special care must be taken to judge

whether AssistNow Autonomous is beneficial to the overall power consumption or not.

13.2.4 Examples

13.2.4.1 Use Grid Offset

Scenario: Get a position fix once a day at a fixed time. If the position fix cannot be obtained try

again every two hours.

Solution: First set the update period to 24x3600s and the search period to 2x3600s. Now a

position fix is obtained every 24 hours and if the position fix fails retrials are scheduled in two hour

intervals. As the update grid is aligned to midnight Saturday/Sunday reference time standard, the

position fixes happen at midnight reference time standard. By setting the grid offset to 12x3600s

the position fixes are shifted to once a day at noon reference time standard. If the position fix at

noon fails, retrials take place every two hours, the first at 14:00 reference time standard. Upon

successfully acquiring a position fix the next fix attempt is scheduled for noon the following day.

13.2.4.2 User controlled position fix

Scenario: Get a position fix on request.

Solution: Set updatePeriod and searchPeriod to zero. Set extintSelect to the desired EXTINT pin

to be used. Enable the extintWake and extintBackup features.

13.2.4.3 Use update periods of 30 minutes

Scenario: Get a position fix once every 30 minutes and acquire a fix needed for timing products.

Solution: Set mode of operation to PSMOO. Set updatePeriod to 1800 seconds. Set the search

period to 120 seconds. Enable waitTimeFix feature.

UBX-13003221 - R28 Early production information Page 65 of 460

## Page 81

u-blox 8 / u-blox M8 Receiver description - Manual

13.3 Peak current settings

The peak current during acquisition can be reduced by activating the corresponding option in UBX-

CFG-PM2. A peak current reduction will result in longer start-up times of the receiver.

This setting is independent of the activated mode (Continuous or Power Save Mode).

13.4 Power On/Off command

With message UBX-RXM-PMREQ the receiver can be forced to enter Inactive state (in Continuous

and Power Save Mode). It will stay in Inactive state for the time specified in the message or until it

is woken up by an EXTINT or activity on the RXD1, SPI CS, or NRESET pin.

Sending the message UBX-RXM-PMREQ while the receiver is in Power Save Mode will

overrule PSM and force the receiver to enter Inactive state. It will stay in Inactive state

until woken up. After wake-up the receiver continues working in Power Save Mode as

configured.

13.5 EXTINT pin control when Power Save Mode is not active

The receiver can be forced OFF also when the Power Save Mode is not active. This works the same

way as EXTINT pin control in Power Save Mode. Just as in Power Save Mode, this feature has to be

enabled and configured using UBX-CFG-PM2

13.6 Measurement and navigation rate with Power Save Mode

In Continuous Mode, measurement and navigation rate is configured using UBX-CFG-RATE. In

Power Save Mode however, measurement and navigation rate can differ from the configured rates

as follows:

• Cyclic Operation: When in state Power Optimized Tracking, the measurement and navigation

rate is determined by the updatePeriod configured in UBX-CFG-PM2. The receiver can however

switch to Tracking state (e.g. to download data). When in Tracking state, the measurement and

navigation rate is as configured with UBX-CFG-RATE. Note: When the receiver is no longer able to

produce position fixes, it can switch from Cyclic Operation to ON/OFF Operation (if this is not

disabled with the doNotEnterOff switch in UBX-CFG-PM2). In that case the remarks below are

relevant.

• ON/OFF Operation: ( in protocol versions less than 18) when in state Acquisition, the

measurement and navigation rate is fixed to 2 Hz. All NMEA (and UBX) messages that are

output upon a navigation fix are also output with a rate of 2 Hz. This must be considered when

choosing the baud rate of a receiver that uses Power Save Mode! Note that a receiver might

stay in Acquisition state for quite some time (can be tens of seconds under weak signal

conditions). When the receiver eventually switches to Tracking state, the measurement and

navigation rate will be as configured with UBX-CFG-RATE. However, ( in protocol versions 18+)

the measurement and navigation rate will be as configured with UBX-CFG-RATE in all active

states.

13.7 Power mode setup

(Not supported in protocol versions less than 18).

In order to simplify the power saving configuration of the receiver in typical circumstances, a set

of predefined setups can be selected using the message UBX-CFG-PMS.

UBX-13003221 - R28 Early production information Page 66 of 460

## Page 82

u-blox 8 / u-blox M8 Receiver description - Manual

Selecting one of the available setups (listed below) is the equivalent of using a combination of the

configuration messages with appropriate parameters that impact the power consumption of the

receiver.

Valid power mode setup in UBX-CFG-PMS

Setup Name Description

Full Power No compromises on power saves

Balanced Power savings without performance degradation

Aggressive 1 Hz Best power saving setup (1 Hz rate). This corresponds to Super-E mode

performance setting.

Aggressive 2 Hz Excellent power saving setup (2 Hz rate)

Aggressive 4 Hz Good power saving setup (4 Hz rate)

Interval ON OFF mode setup

u-blox recommends using these predefined settings, except where users have very specific power

saving requirements.

Note that polling UBX-CFG-PMS will return the setup only if the full configuration is consistent

with one of the predefined power mode setups.

In 4 Hz mode, when running a flash firmware, it is recommended to run with a subset of

GNSS systems, to avoid system overload.

Using UBX-CFG-PMS to set Super-E mode to 1, 2 or 4 Hz navigation rates sets

minAcqTime to 180 s instead the default 300 s in protocol version 23.01. 300 s is

recommended for the best performance.

14 Forcing a Receiver Reset

Typically, in GNSS receivers, one distinguishes between cold, warm, and hot starts, depending on

the type of valid information the receiver has at the time of the restart.

• Cold start In cold start mode, the receiver has no information from the last position (e.g. time,

velocity, frequency etc.) at startup. Therefore, the receiver must search the full time and

frequency space, and all possible satellite numbers. If a satellite signal is found, it is tracked to

decode the ephemeris (18-36 seconds under strong signal conditions), whereas the other

channels continue to search satellites. Once there is a sufficient number of satellites with valid

ephemeris, the receiver can calculate position and velocity data. Other GNSS receiver

manufacturers call this startup mode Factory Startup.

• Warm start In warm start mode, the receiver has approximate information for time, position,

and coarse satellite position data (Almanac). In this mode, after power-up, the receiver normally

needs to download ephemeris before it can calculate position and velocity data. As the

ephemeris data usually is outdated after 4 hours, the receiver will typically start with a Warm

start if it has been powered down for more than 4 hours. In this scenario, several augmentations

are possible. See section Multi-GNSS assistance.

• Hot start In hot start mode, the receiver was powered down only for a short time (4 hours or

less), so that its ephemeris is still valid. Since the receiver does not need to download ephemeris

again, this is the fastest startup method.

In the UBX-CFG-RST message, one can force the receiver to reset and clear data, in order to see the

effects of maintaining/losing such data between restarts. For this, the CFG-RST message offers

the navBbrMask field, where hot, warm and cold starts can be initiated, and also other

UBX-13003221 - R28 Early production information Page 67 of 460

## Page 83

u-blox 8 / u-blox M8 Receiver description - Manual

combinations thereof.

Data stored in flash memory is not cleared by any of the options provided by UBX-CFG-

RST. So, for example, if valid AssistNow Offline data stored in the flash it is likely to have

an impact on the cold start.

The Reset Type can also be specified. This is not related to GNSS, but to the way the software

restarts the system.

• Hardware Reset uses the on-chip Watchdog to electrically reset the chip. This is an immediate,

asynchronous reset. No Stop events are generated. This is equivalent to pulling the Reset signal

of the receiver to ground. This reset reloads the receiver configuration.

• Controlled Software Reset terminates all running processes in an orderly manner and, once the

system is idle, restarts operation, reloads its configuration and starts to acquire and track

GNSS satellites.

• Controlled Software Reset (GNSS only) only restarts the GNSS tasks, without reinitializing the

full system or reloading any stored configuration.

• Controlled GNSS Stop stops all GNSS tasks. The receiver will not be restarted, but will stop any

GNSS related processing.

• Controlled GNSS Start starts all GNSS tasks.

A reset may reload the receiver configuration. Use UBX-CFG-CFG to save runtime configuration

changes to BBR before the reset.

15 Receiver Status Monitoring

Messages in the UBX class UBX-MON are used to report the status of the parts of the embedded

computer system that are not GNSS specific.

The main purposes are

• Hardware and Software Versions, using UBX-MON-VER. See also the chapter decoding the output

of UBX-MON-VER

• Status of the Communications Input/Output system

• Status of various Hardware Sections with UBX-MON-HW

15.1 Input/Output system

The I/O system is a GNSS-internal layer where all data input- and output capabilities (such as

UART, DDC, SPI, USB) of the GNSS receiver are combined. Each communications task has buffers

assigned, where data is queued. For data originating at the receiver, to be communicated over one

or multiple communications queues, the message UBX-MON-TXBUF can be used. This message

shows the current and maximum buffer usage, as well as error conditions.

If the amount of data configured is too much for a certain port's bandwidth (e.g. all UBX

messages output on a UART port with a baud rate of 9600), the buffer will fill up. Once

the buffer space is exceeded, new messages to be sent will be dropped. For details see

section Serial Communication Ports Description.

Inbound data to the GNSS receiver is placed in buffers. Usage of these buffers is shown with the

message UBX-MON-RXBUF. Further, as data is then decoded within the receiver (e.g. to separate

UBX and NMEA data), the UBX-MON-MSGPP can be used. This message shows (for each port and

protocol) how many messages were successfully received. It also shows (for each port) how many

bytes were discarded because they were not in any of the supported protocol framings.

UBX-13003221 - R28 Early production information Page 68 of 460

## Page 84

u-blox 8 / u-blox M8 Receiver description - Manual

The following table shows the port numbers used. Note that any numbers not listed are reserved

for future use.

Port Number assignment

Port # Electrical Interface

0 DDC (I2C compatible)

1 UART 1

3 USB

4 SPI

Protocol numbers range from 0-7. All numbers not listed are reserved.

Protocol Number assignment

Protocol # Protocol Name

0 UBX Protocol

1 NMEA Protocol

2 RTCM Protocol

15.2 Jamming/Interference Indicator

The field jamInd of the UBX-MON-HW message can be used as an indicator for continuous wave

(narrowband) jammers/interference only. The interpretation of the value depends on the

application. It is necessary to run the receiver in an unjammed environment to determine an

appropriate value for the unjammed case. If the value rises significantly above this threshold, this

indicates that a continuous wave jammer is present.

This indicator is always enabled.

The indicator is reporting any currently detected narrowband interference over all currently

configured signal bands

15.3 Jamming/Interference Monitor (ITFM)

The field jammingState of the UBX-MON-HW message can be used as an indicator for both

broadband and continuous wave (CW) jammers/interference. It is independent of the (CW only)

jamming indicator described in Jamming/Interference Indicator above.

This monitor reports whether jamming has been detected or suspected by the receiver. The

receiver monitors the background noise and looks for significant changes. Normally, with no

interference detected, it will report 'OK'. If the receiver detects that the noise has risen above a

preset threshold, the receiver reports 'Warning'. If in addition, there is no current valid fix, the

receiver reports 'Critical'.

The monitor has four states as shown in the following table:

Jamming/Interference monitor reported states

Value Reported Description

state

0 Unknown Jamming/interference monitor not enabled, uninitialized

or antenna disconnected

1 OK no interference detected

2 Warning position ok but interference is visible (above the

thresholds)

UBX-13003221 - R28 Early production information Page 69 of 460

## Page 85

u-blox 8 / u-blox M8 Receiver description - Manual

Jamming/Interference monitor reported states continued

Value Reported Description

state

3 Critical no reliable position fix and interference is visible (above

the thresholds); interference is probable reason why

there is no fix

The monitor is disabled by default. The monitor is enabled by sending an appropriate UBX-CFG-

ITFM message with the enable bit set. In this message it is also possible to specify the thresholds

at which broadband and CW jamming are reported. These thresholds should be interpreted as the

dB level above 'normal'. It is also possible to specify whether the receiver expects an active or

passive antenna.

The monitor algorithm relies on comparing the currently measured spectrum with a

reference from when a good fix was obtained. Thus the monitor will only function when

the receiver has had at least one (good) first fix, and will report 'Unknown' before this

time.

Jamming/Interference monitor is not supported in power save mode (PSM) ON/OFF

mode.

The monitor is reporting any currently detected interference over all currently configured signal

bands.

16 Spoofing Detection

(Note: this feature is not supported in protocol versions less than 18).

16.1 Introduction

Spoofing is the process whereby someone tries to forge a GNSS signal with the intention of

fooling the receiver into calculating a different user position than the true one.

The spoofing detection feature monitors the GNSS signals for suspicious patterns indicating that

the receiver is being spoofed. A flag in UBX-NAV-STATUS alerts the user to potential spoofing.

16.2 Scope

The spoofing detection feature monitors suspicious changes in the GNSS signal indicating

external manipulation. Therefore the detection is only successful when the signal is genuine first

and when the transition to the spoofed signal is being observed directly. When a receiver is

started up to a spoofed signal the detection algorithms will be unable to recognize the spoofing.

Also, the algorithms rely on availability of signals from multiple GNSS; the detection does not work

in single GNSS mode.

17 Signal Attenuation Compensation

(not supported in protocol versions less than 19).

In normal operating conditions, low signal strength indicates likely contamination by multipath.

The receiver trusts such signals less in order to preserve the quality of the position solution in

poor signal environments. This feature can result in degraded performance in situations where

the signals are attenuated for another reason, for example due to antenna placement. In this

case, the signal attenuation compensation feature can be used to restore normal performance.

UBX-13003221 - R28 Early production information Page 70 of 460

## Page 86

u-blox 8 / u-blox M8 Receiver description - Manual

There are three possible modes:

• Disabled: no signal attenuation compensation is performed

• Automatic: the receiver automatically estimates and compensates for the signal attenuation

• Configured: the receiver compensates for the signal attenuation based on a configured value

These modes can be selected using UBX-CFG-NAVX5. In the case of the "configured" mode, the

user should input the maximum C/N0 observed in a clear-sky environment, excluding any outliers

or unusually high values. The configured value can have a large impact on the receiver

performance, so should be chosen carefully.

18 Remote Inventory

18.1 Description

The Remote Inventory enables storing user-defined data in the non-volatile memory of the

receiver. The data can be either binary or a string of ASCII characters. In the second case, it will be

output at startup after the boot screen.

18.2 Usage

• The contents of the Remote Inventory can be set and polled with the message UBX-CFG-RINV.

Refer to the message specification for a detailed description.

• If the contents of the Remote Inventory are polled without having been set before, the default

configuration (see table below) is output.

Default configuration

Parameter Value

flags 0x00

data "Notice: no data saved!"

As with all configuration changes, these must be saved in order to be made permanent.

Make sure to save the section RINV before resetting or switching off the receiver. For

more information about saving a configuration, see section Configuration Concept.

19 Time pulse

For protocol versions less than 18, functionality of the time pulse has not been

characterized when only BeiDou is enabled.

The time pulse feature is not available for protocol versions 23-23.01.

19.1 Introduction

u-blox receivers include a time pulse function providing clock pulses with configurable duration

and frequency. The time pulse function can be configured using the UBX-CFG-TP5 message. The

UBX-TIM-TP message provides time information for the next pulse, time source and the

quantization error of the output pin.

UBX-13003221 - R28 Early production information Page 71 of 460

## Page 87

u-blox 8 / u-blox M8 Receiver description - Manual

19.2 Recommendations

• The time pulse can be aligned to a wide variety of GNSS times or to variants of UTC derived from

them (see the section on time bases). However, it is strongly recommended that the choice of

time base is aligned with the available GNSS signals (so to produce GPS time or UTC(USNO),

ensure GPS signals are available, and for GLONASS time or UTC(SU) ensure the presence

GLONASS signals). This will involve coordinating that the setting of UBX-CFG-GNSS with the

choice of time pulse time base.

• For best time pulse performance it is recommended to disable the SBAS subsystem.

• When using time pulse for precision timing applications it is recommended to calibrate the

antenna cable delay against a reference-timing source.

• Care needs to be given to the cable delay settings in the receiver configuration.

• In order to get the best timing accuracy with the antenna, a fixed and accurate position is

needed.

• If relative time accuracy between multiple receivers is required, do not mix receivers of different

product families. If this is required, the receivers must be calibrated accordingly, by setting

cable delay and user delay.

• The recommended configuration when using the UBX-TIM-TP message is to set both the

measurement rate (UBX-CFG-RATE) and the time pulse frequency (UBX-CFG-TP5) to 1 Hz.

Since the rate of UBX-TIM-TP is bound to the measurement rate, more than one UBX-

TIM-TP message can appear between two pulses if the measurement rate is set larger

than the time pulse frequency. In this case all UBX-TIM-TP messages in between a time

pulse T1 and T2 belong to T2 and the last UBX-TIM-TP before T2 reports the most

accurate quantization error. In general, if the navigation solution rate and time pulse rate

are configured to different values, there will not be a single UBX-TIM-TP message for

each time pulse.

The sequential order of the signal present at the TIMEPULSE pin and the respective output

message for the simple case of 1 pulse per second (1PPS) and a one second navigation update rate

is shown in the following figure.

UBX-13003221 - R28 Early production information Page 72 of 460

## Page 88

u-blox 8 / u-blox M8 Receiver description - Manual

19.3 GNSS time bases

GNSS receivers must handle a variety of different time bases as each GNSS has its own reference

system time. What is more, although each GNSS provides a model for converting their system

time into UTC, they all support a slightly different variant of UTC. So, for example, GPS supports a

variant of UTC as defined by the US National Observatory, while BeiDou uses UTC from the

National Time Service Center, China (NTSC) and NavIC uses UTC from National Physics

Laboratory, India (NPLI). While the different UTC variants are normally closely aligned, they can

differ by as much as a few hundreds of nanoseconds.

Although u-blox receivers can combine a variety of different GNSS times internally, the user must

choose a single type of GNSS time and, separately, a single type of UTC for input (on EXTINTs)

and output (via the Time Pulse) and the parameters reported in corresponding messages.

For protocol versions 16 or greater, the UBX-CFG-TP5 message allows the user to choose between

any of the supported GNSS (GPS, GLONASS, BeiDou, etc.) times and UTC. Also, the UBX-CFG-

NAV5 message allows the user to select which variant of UTC the receiver should use. This

includes an "automatic" option which causes the receiver to select an appropriate UTC version

itself, based on the GNSS configuration, using, in order of preference, USNO if GPS is enabled, SU

if GLONASS is enabled, NTSC if BeiDou is enabled, European if Galileo is enabled and, finally, NPLI

if NavIC is enabled.

Note that for protocol versions prior to 16, no choice of UTC variant is supported and the UBX-CFG-

TP5 message only allows the user to choose between GPS and UTC as the time system the

generated time pulse will be aligned to.

The receiver will assume that the input time pulse uses the same GNSS time base as specified for

the output using UBX-CFG-TP5. So if the user selects GLONASS time for time pulse output, any

time pulse input must also be aligned to GLONASS time (or to the separately chosen variant of

UTC). Where UTC is selected for time pulse output, any GNSS time pulse input will be assumed to

be aligned to GPS time.

u-blox receivers allow users to choose independently GNSS signals used in the receiver

(using UBX-CFG-GNSS) and the input/output time base (using UBX-CFG-TP5). For

example it is possible to instruct the receiver to use GPS and GLONASS satellite signals

to generate BeiDou time. This practice will compromise time-pulse accuracy if the

receiver cannot measure the timing difference between the constellations directly and is

not recommended.

The information that allows GNSS times to be converted to the associated UTC times is

UBX-13003221 - R28 Early production information Page 73 of 460

## Page 89

u-blox 8 / u-blox M8 Receiver description - Manual

only transmitted by the GNSS at relatively infrequent periods. For example GPS

transmits UTC(USNO) information only once every 12.5 minutes. Therefore, if a Time

Pulse is configured to use a variant of UTC time, after a cold start, substantial delays

before the receiver has sufficient information to start outputing the Time Pulse can be

expected.

19.4 Time pulse configuration

u-blox receivers provide one or two TIMEPULSE pins (dependent on product variant) delivering a

time pulse (TP) signal with a configurable pulse period, pulse length and polarity (rising or falling

edge). Check the product data sheet for detailed specification of configurable values.

It is possible to define different signal behavior (i.e. output frequency and pulse length) depending

on whether or not the receiver is locked to a reliable time source. Time pulse signals can be

configured using the UBX proprietary message UBX-CFG-TP5.

19.5 Configuring time pulse with UBX-CFG-TP5

The UBX message UBX-CFG-TP5 can be used to change the time pulse settings, and includes the

following parameters defining the pulse:

• time pulse index - Index of time pulse output pin to be configured. If a product only has one time

pulse output it is typically configurable with index 0. Exceptions to this include LEA-M8F,

M8030-KT-FT and NEO-M8L. Refer to specific product documentation.

• antenna cable delay - Signal delay due to the cable between antenna and receiver.

• RF group delay - Signal delay in the RF module of the receiver (read-only).

• pulse frequency/period - Frequency or period time of the pulse when locked mode is not

configured or active.

• pulse frequency/period lock - Frequency or period time of the pulse, as soon as receiver has

calculated a valid time from a received signal. Only used if the corresponding flag is set to use

another setting in locked mode.

• pulse length/ratio - Length or duty cycle of the generated pulse, either specifies a time or ratio

for the pulse to be on/off.

• pulse length/ratio lock - Length or duty cycle of the generated pulse, as soon as receiver has

calculated a valid time from a received signal. Only used if the corresponding flag is set to use

another setting in locked mode.

• user delay - The cable delay from the receiver to the user device plus signal delay of any user

application.

• active - time pulse will be active if this bit is set.

• lock to gps freq - Use frequency gained from GPS signal information rather than local

oscillator's frequency if flag is set.

• lock to gnss freq - Use frequency gained from GNSS signal information rather than local

oscillator's frequency if flag is set.

• locked other setting - If this bit is set, as soon as the receiver can calculate a valid time, the

alternative setting is used. This mode can be used for example to disable time pulse if time is

not locked, or indicate lock with different duty cycles.

• is frequency - Interpret the 'Frequency/Period' field as frequency rather than period if flag is set.

• is length - Interpret the 'Length/Ratio' field as length rather than ratio if flag is set.

UBX-13003221 - R28 Early production information Page 74 of 460

## Page 90

u-blox 8 / u-blox M8 Receiver description - Manual

• align to TOW - If this bit is set, pulses are aligned to the top of a second.

• polarity - If set, the first edge of the pulse is a rising edge (Pulse Mode: Rising).

• grid UTC/GPS - Selection between UTC (0) or GPS (1) timegrid. Also effects the time output by

UBX-TIM-TP message.

• grid UTC/GNSS - Selection between UTC (0), GPS (1), GLONASS (2) and Beidou (3) timegrid.

Also effects the time output by UBX-TIM-TP message.

The maximum pulse length can't exceed the pulse period.

Time pulse settings shall be chosen in such a way, that neither the high nor the low

period of the output is less than 50 ns (except when disabling it completely), otherwise

pulses can be lost.

The maximum frequency of the second time pulse pin (TIMEPULSE2) is limited to 1 kHz

for protocol versions less than 18 unless using a Timing product variant.

19.5.1 Example 1

The example below shows the 1PPS TP signal generated on the time pulse output according to the

specific parameters of the UBX-CFG-TP5 message:

• tpIdx = 0

• freqPeriod = 1 s

• pulseLenRatio = 100 ms

• active = 1

• lockGpsFreq = lockGnssFreq = 1

• isLength = 1

• alignToTow = 1

• polarity = 1

• gridUtcGps = gridUtcGnss = 1

The 1 Hz output is maintained whether or not the receiver is locked to GPS time. The alignment to

TOW can only be maintained when GPS time is locked.

19.5.2 Example 2

This example only works with a Timing product variant or for protocol versions greater than 17.

The following example shows a 10 MHz TP signal generated on the TIMEPULSE2 output when the

receiver is locked to GPS time. Without the lock to GPS time no frequency is output.

UBX-13003221 - R28 Early production information Page 75 of 460

## Page 91

u-blox 8 / u-blox M8 Receiver description - Manual

• tpIdx = 1

• freqPeriod = 1 Hz

• pulseLenRatio = 0

• freqPeriodLock = 10 MHz

• pulseLenRatioLock = 50%

• active = 1

• lockGpsFreq = lockGnssFreq = 1

• lockedOtherSet = 1

• isFreq = 1

• alignToTow = 1

• polarity = 1

• gridUtcGps = gridUtcGnss = 1

20 Timemark

The receiver can be used to provide an accurate measurement of the time at which a pulse was

detected on the external interrupt pin. The reference time can be chosen by setting the time

source parameter to UTC, GPS, GLONASS, BeiDou, Galileo or local time in the UBX-CFG-TP5

configuration message. The UTC standard can be set in the UBX-CFG-NAV5 configuration

message. The delay figures defined with UBX-CFG-TP5 are also applied to the results output in the

UBX-TIM-TM2 message.

A UBX-TIM-TM2 message is output at the next epoch if

• the UBX-TIM-TM2 message is enabled

• a rising or falling edge was triggered since last epoch on one of the EXTINT channels

The UBX-TIM-TM2 messages include time of the last timemark, new rising/falling edge indicator,

time source, validity, number of marks and a quantization error. The timemark is triggered

continuously.

Only the last rising and falling edge detected between two epochs is reported since the

output rate of the UBX-TIM-TM2 message corresponds to the measurement rate

configured with UBX-CFG-RATE (see Figure below).

UBX-13003221 - R28 Early production information Page 76 of 460

## Page 92

u-blox 8 / u-blox M8 Receiver description - Manual

21 Odometer

21.1 Introduction

The odometer provides information on travelled ground distance (in meter) using solely the

position and Doppler-based velocity of the navigation solution. For each computed travelled

distance since the last odometer reset, the odometer estimates a 1-sigma accuracy value. The

total cumulative ground distance is maintained and saved in the BBR memory.

The odometer feature is disabled by default. It can be enabled using the UBX-CFG-ODO

message.

21.2 Odometer Output

The odometer output is published in the UBX-NAV-ODO message. This message contains the

following elements:

• Ground distance since last reset (distance field): this distance is defined as the total cumulated

distance in meters since the last time the odometer was reset (see section Resetting the

Odometer);

• Ground distance accuracy (distanceStd field): this quantity is defined as the 1-sigma accuracy

estimate (in meters) associated to the Ground distance since last reset value;

• Total cumulative ground distance (totalDistance field): this quantity is defined as the total

cumulated distance in meters since the last time the receiver was cold started (see section

Resetting the Odometer).

If logging is enabled, then the odometer's ground distance since last reset value will be included in

UBX-13003221 - R28 Early production information Page 77 of 460

## Page 93

u-blox 8 / u-blox M8 Receiver description - Manual

the logged position data (see section Logging).

21.3 Odometer Configuration

The odometer can be enabled/disabled by setting the appropriate flag in UBX-CFG-ODO (flags

field). The algorithm behaviour can be optimized by setting up a profile (odoCfg field)

representative of the context in which the receiver is operated. The implemented profiles together

with their meanings are listed below:

• Running: the algorithm is optimized for typical dynamics encountered while running, i.e the

Doppler-based velocity solution is assumed to be of lower quality;

• Cycling: the algorithm is optimized for typical dynamics encountered while cycling;

• Swimming: the algorithm is optimized for very slow and smooth trajectories typically

encountered while swimming;

• Car: the algorithm assumes that good Doppler measurements are available (i.e. the antenna is

subject to low vibrations) and is optimized for typical dynamics encountered by cars.

The odometer can only be reliably operated in a swimming context if satellite signals are

available and the antenna is not immersed.

21.4 Resetting the Odometer

The odometer outputs (see UBX-NAV-ODO message) can be reset by the following means:

• Ground distance since last reset (distance field): by sending a UBX-NAV-RESETODO message;

• Ground distance accuracy (distanceStd field): by sending a UBX-NAV-RESETODO message;

• Total cumulative ground distance (totalDistance): by a cold start of the receiver (this erases the

BBR memory);

22 Logging

22.1 Introduction

The logging feature allows position fixes and arbitrary byte strings from the host to be logged in

flash memory attached to the receiver. Logging of position fixes happens independently of the

host system, and can continue while the host is powered down.

The following tables list all the logging related messages:

Logging control and configuration messages

Message Description

UBX-LOG-CREATE Creates a log file and activates the logging subsystem

UBX-LOG-ERASE Erases a log file and deactivates the logging subsystem

UBX-CFG-LOGFILTER Used to start/stop recording and set/get the logging configuration

UBX-LOG-INFO Provides information about the logging system

UBX-LOG-STRING Enables a host process to write a string of bytes to the log file

Logging retrieval messages

Message Description

UBX-LOG-RETRIEVE Starts the log retrieval process

UBX-LOG-RETRIEVEPOS A position log entry returned by the receiver

UBX-13003221 - R28 Early production information Page 78 of 460

## Page 94

u-blox 8 / u-blox M8 Receiver description - Manual

Logging retrieval messages continued

Message Description

UBX-LOG- Odometer position data

RETRIEVEPOSEXTRA

UBX-LOG-RETRIEVESTRING A byte string log entry returned by the receiver

UBX-LOG-FINDTIME Finds the index of the first entry <= given time

22.2 Setting the logging system up

An empty log can be created using the UBX-LOG-CREATE message and a log can be deleted with

the UBX-LOG-ERASE message. The logging system will only be running if a log is in existence, so

most logging messages will be rejected with an UBX-ACK-NAK message if there is no log present.

Only one log can be created at any one time so an UBX-ACK-NAK message will be returned if a log

already exists. The message specifies the maximum size of the log in bytes (with some pre-set

values provided). Both the logging subsystem and the receiver file-store have implementation

overheads, so total space available for log entries will be somewhat smaller than the size

specified.

UBX-LOG-CREATE also allows the log to be specified as a circular log. If the log is circular, then

when it fills up, a set of older log entries will be deleted and the space freed up used for new log

entries. By contrast, if a non-circular log becomes full then new entries which do not fit will be

rejected. UBX-LOG-CREATE also causes the logging system to start up so that further logging

messages can be processed. The logging system will start up automatically on power-up if there is

a log in existence. The log will remain in the receiver until specifically erased using the UBX-LOG-

ERASE message.

UBX-CFG-LOGFILTER controls whether logging of entries is currently enabled and selects position

fix messages for logging. These configuration settings will be saved if the configuration is saved

to flash. If this is done, then entry logging will continue on power-up in the same manner that it did

before power-down.

The top level active/inactive states of the logging subsystem.

22.3 Information about the log

The receiver can be polled for a UBX-LOG-INFO message which will give information about the log.

This will include the maximum size that the log can grow to (which, due to overheads, will be

smaller than that requested in UBX-LOG-CREATE) and the amount of log space currently occupied.

It will also report the number of entries currently in the log together with the time and date of the

newest and oldest messages which have a valid time stamp.

Log entries are compressed and have housekeeping information associated with them, so the

actual space occupied by log messages may be difficult to predict. The minimum size for a

UBX-13003221 - R28 Early production information Page 79 of 460

## Page 95

u-blox 8 / u-blox M8 Receiver description - Manual

position fix entry is 9 bytes and the maximum 24 bytes, the typical size is 10 or 11 bytes. If the

odometer is enabled then this will use at least another three bytes per fix.

Each log also has a fixed overhead which is dependent on the log type. The approximate size of

this overhead is shown in the following table.

Log overhead size

Log type Overhead

circular Up to 40 kB

non-circular Up to 8 kB

The number of entries that can be logged in any given flash size can be estimated as follows:

Approx. number of entries = (flash size available for logging - log

overhead)/typical entry size

For example, if 1500 kB of flash is available for logging (after other flash usage such as the

firmware image is taken into account) a non-circular log would be able to contain approximately

139000 entries ((1500*1024)-(8*1024))/11 = 138891.

22.4 Recording

The UBX-CFG-LOGFILTER message specifies the conditions under which entries are recorded.

Nothing will be recorded if recording is disabled, otherwise position fix and UBX-LOG-STRING

entries can be recorded. When recording is enabled an entry will also be created from each UBX-

LOG-STRING message. These will be timestamped if the receiver has current knowledge of time.

The UBX-CFG-LOGFILTER message has several values which can be used to select position fix

entries for logging. If all of these values are zero, then all position fixes will be logged (subject to a

maximum rate of 1Hz). A position is logged if any of the thresholds are exceeded. If a threshold is

set to zero it is ignored. In addition the position difference and current speed thresholds also have

a minimum time threshold.

Position fixes are only recorded if a valid fix is obtained - failed and invalid fixes are not recorded.

Position fixes are compressed to economise on the amount of flash space used. In order to

improve the compression, the fix values are rounded to improve their compression. This means

that the values returned by the logging system may differ slightly from any which are gathered in

real time.

In On/Off power save mode it is possible to configure the logging system so that only one fix is

recorded for each on period. This will be recorded immediately before the receiver powers off and

will be the best fix seen during the on period (in this case, "best" is defined as being the fix with the

lowest horizontal accuracy figure).

The recorded data for a fix comprises :

• The time and date of the fix recorded to a precision of one second.

• Latitude and longitude to a precision of one millionth of a degree. Depending on position on

Earth this is a precision in the order of 0.1 m.

• Altitude (height above mean sea level) to a precision of 0.1 m. Entries with an altitude lower than

-470 m (lower than the lowest point on earth) or higher than 20,000 m may not be recorded in

the log.

• Ground speed to a precision of 1 cm/s

• The fix type (only successful fix types, since these are the only ones recorded).

UBX-13003221 - R28 Early production information Page 80 of 460

## Page 96

u-blox 8 / u-blox M8 Receiver description - Manual

• The number of satellites used in the fix is recorded, but there is a maximum count which can be

recorded. If the actual count exceeds this maximum count then the maximum count will be

recorded. If a log entry is retrieved with a satellite count equal to the maximum this means that

value or more. The maximum count is 51. (The maximum count is 19 in protocol versions less

than 24).

• A horizontal accuracy estimate is recorded to give an indication of fix quality. This is an

approximate compressed representation of the accuracy as determined by the fix process. Any

accuracy less than 0.7 m will be recorded as 0.7 m and any value above 1 km will be recorded as

1km. Within these limits, the recorded accuracy will always be greater than the fix accuracy

number (by up to 40%).

• Heading to a precision of one degree.

• Odometer distance data (if odometer is enabled).

The states of the active logging subsystem

22.5 Retrieval

UBX-LOG-RETRIEVE starts the process which allows the receiver to output log entries. Log

recording must be stopped using UBX-CFG-LOGFILTER before this can be done. UBX-LOG-INFO

may be helpful to a host system in order to understand the current log status before retrieval is

started.

Once retrieval has started, one message will be output from the receiver for each log entry

requested. Sending any logging message to the receiver during retrieval will cause the retrieval to

stop before the message is processed.

To maximise the speed of transfer it is recommended that a high communications data rate is

used and GNSS processing is stopped during the transfer (see UBX-CFG-RST)

UBX-LOG-RETRIEVE can specify a start-entry index and entry-count. The maximum number of

entries that can be returned in response to a single UBX-LOG-RETRIEVE message is 256. If more

entries than this are required the message will need to be sent multiple times with different

startEntry indices.

UBX-13003221 - R28 Early production information Page 81 of 460

## Page 97

u-blox 8 / u-blox M8 Receiver description - Manual

The receiver will send a UBX-LOG-RETRIEVEPOS message for each position fix log entry and a UBX-

LOG-RETRIEVESTRING message for each string log entry. If the odometer was enabled at the time

a position was logged, then a UBX-LOG-RETRIEVEPOSEXTRA will also be sent. Messages will be

sent in the order in which they were logged, so UBX-LOG-RETRIEVEPOS and UBX-LOG-

RETRIEVESTRING messages may be interspersed in the message stream.

The UBX-LOG-FINDTIME message can be used to search a log for the index of the first entry less

than or equal to the given time. This index can then be used with the UBX-LOG-RETRIEVE message

to provide time-based retrieval of log entries.

22.6 Command message acknowledgement

Some log operations may take a long time to execute because of the time taken to write to flash

memory. The time for some operations may be unpredictable since the number and timing of

flash operations may vary. In order to allow host software to synchronise to these delays logging

messages will always produce a response. This will be UBX-ACK-NAK in case of error, otherwise UBX-

ACK-ACK unless there is some other defined response to the message.

It is possible to send a small number of logging commands without waiting for acknowledgement,

since there is a command queue, but this risks confusion between the acknowledgements for the

commands. Also a command queue overflow would result in commands being lost.

23 Data Batching

(Note: this functionality is supported only in protocol versions 23.01).

23.1 Introduction

The data batching feature allows position fixes to be stored in the RAM of the receiver to be

retrieved later in one batch. Batching of position fixes happens independently of the host system,

and can continue while the host is powered down.

The following tables list all the batching related messages:

Batching control and configuration messages

Message Description

UBX-CFG-BATCH Used to enable and configure the batching feature

UBX-MON-BATCH Provides information about the buffer fill level and dropped data due

to overrun

Batch retrieval messages

Message Description

UBX-LOG-RETRIEVEBATCH Starts the batch retrieval process

UBX-LOG-BATCH A batch entry returned by the receiver

23.2 Setting up the data batching

Data batching is disabled per default and it has to be configured before use via UBX-CFG-BATCH.

The feature must be enabled and the buffer size must be set to greater than 0. It is possible to set

up a PIO as a flag that indicates when the buffer is close to filling up. The fill level when this PIO is

asserted can be set by the user separately from the buffer size. The notification fill level must not

be larger than the buffer size.

If the host does not retrieve the batched fixes before the buffer fills up the oldest fix will be

UBX-13003221 - R28 Early production information Page 82 of 460

## Page 98

u-blox 8 / u-blox M8 Receiver description - Manual

dropped and replaced with the newest.

The RAM available in the chip limits the size of the buffer. To make the best use of the available

space users can select what data they want to batch. When batching is enabled a basic set of

data is stored and the configuration flags extraPvt and extraOdo can be used to store more

detailed information about the position fixes. Doing so reduces the number of fixes that can be

batched.

The receiver will reject configuration if it cannot allocate the required buffer memory. To ensure

robust operation of the receiver the following limits are enforced:

Maximum number of batched epochs

extraPvt extraOdo Maximum number of epochs

0 0 300

0 1 221

1 0 156

1 1 132

It is recommended to disable all periodic output messages when using batching. This

improves system robustness and also helps ensure that the output of batched data is

not delayed by other messages.

The buffer size is set up in terms of navigation epochs. This means that the time that

can be covered with a certain buffer depends on the navigation rate. This rate can be set

separately for full power operation via UBX-CFG-RATE and for power save mode via the

updatePeriod in UBX-CFG-PM2.

Data batching settings should not be re-configured while retrieving data from the buffer.

23.3 Retrieval

UBX-LOG-RETRIEVEBATCH starts the process which allows the receiver to output batch entries.

Batching must not be stopped for readout; all batched data is lost when the feature is disabled.

Batched fixes are always retrieved starting with the oldest fix in the buffer and progressing

towards newer ones. There is no way to skip certain fixes during retrieval.

When a UBX-LOG-RETRIEVEBATCH message is sent the receiver transmits all batched fixes. It is

recommended to send a retrieval request with sendMonFirst set. This way the receiver will send a

UBX-MON-BATCH message first that contains the number of fixes in the batching buffer. This

information can be used to detect when the u-blox receiver finished sending data.

Once retrieval has started, the receiver will first send UBX-MON-BATCH if sendMonFirst option was

selected in the UBX-LOG-RETRIEVEBATCH. After that, it will send UBX-LOG-BATCH messages with

the batched fixes.

To maximise the speed of transfer it is recommended that a high communications data rate is

used.

The receiver will discard retrieval request while processing a previous UBX-LOG-

RETRIEVEBATCH message.

The receiver does not acknowledge the reception of UBX-LOG-RETRIEVEBATCH; the

response that the host should expect are the reply messages.

UBX-13003221 - R28 Early production information Page 83 of 460

## Page 99

u-blox 8 / u-blox M8 Receiver description - Manual

24 Geofencing

(Note: this feature is not supported in protocol versions less than 18).

24.1 Introduction

The geofencing feature allows for the configuration of up to four circular areas (geofences) on the

Earth's surface. The receiver will then evaluate for each of these areas whether the current

position lies within the area or not and signal the state via UBX messaging and PIO toggling.

24.2 Interface

Geofencing can be configured using the UBX-CFG-GEOFENCE message. The geofence evaluation is

active whenever there is at least one geofence configured.

The current state of each geofence plus the combined state is output in UBX-NAV-GEOFENCE with

every navigation epoch.

Additionally the user can configure the receiver to output the combined geofence state on a

physical pin.

24.3 Geofence state evaluation

With every navigation epoch the receiver will evaluate the current solution's position versus the

configured geofences. There are three possible outcomes for each geofence:

• Inside - The position is inside the geofence with the configured confidence level

• Outside - The position lies outside of the geofence with the configured confidence level

• Unknown - There is no valid position solution or the position uncertainty does not allow for

unambiguous state evaluation

The position solution uncertainty (standard deviation) is multiplied with the configured

confidence sigma level number and taken into account when evaluating the geofence state (red

circle in figure below).

The combined state for all geofences is evaluated as the combination (logical OR) of all geofences:

• Inside - The position lies inside of at least one geofence

• Outside - The position lies outside of all geofences

• Unknown - All remaining states

UBX-13003221 - R28 Early production information Page 84 of 460

## Page 100

u-blox 8 / u-blox M8 Receiver description - Manual

24.4 Using a PIO for Geofence State Output

This feature can be used for example for waking up a sleeping host when a defined geofence

condition is reached. The receiver will toggle the assigned pin according to the combined geofence

state. Due to hardware restrictions the unknown state will always be represented as HIGH. If the

receiver is in software backup or in a reset, the pin will go to HIGH accordingly. The meaning of the

LOW state can be configured using UBX-CFG-GEOFENCE.

25 Time Mode Configuration

This feature is only available with Timing, FTS or High Precision GNSS (HPG) products

This section relates to the configuration message UBX-CFG-TMODE2 (for Timing or FTS products)

and to the configuration message UBX-CFG-TMODE3 (for HPG products).

25.1 Introduction

Time Mode is a special receiver mode where the position of the receiver is known and fixed and

only the time is calculated using all available satellites. This mode allows for maximum time

accuracy, for single-SV solutions, and also for using the receiver as a stationary reference station.

25.2 Fixed Position

In order to use the Time Mode, the receiver's position must be known as exactly as possible. Either

the user already knows and enters the position, or it is determined using Survey-in. Errors in the

fixed position will translate into time errors depending on the satellite constellation.

For Timing products, as a rule of thumb the position should be known with an accuracy of better

than 1 m for a timing accuracy in the order of nanoseconds. If an accuracy is required only in the

order of microseconds, a position accuracy of roughly 300 m is sufficient.

For HPG products, errors in the reference station position will directly translate into rover position

errors. The reference station position accuracy should therefore be at least as good as the desired

rover absolute position accuracy.

25.3 Survey-in

Survey-in is the procedure that is carried out prior to using Time Mode. It determines a stationary

receiver's position by building a weighted mean of all valid 3D position solutions.

Two requirements for stopping the procedure must be specified:

• The minimum observation time defines a minimum amount of observation time regardless of

the actual number of valid fixes that were used for the position calculation. Reasonable values

range from one day for high accuracy requirements to a few minutes for coarse position

determination.

• The required 3D position standard deviation defines a limit on the spread of positions that

contribute to the calculated mean. As the position error translates into a time error when using

Time Mode (see above), one should carefully evaluate the time accuracy requirements and

choose an appropriate value.

Survey-in ends, when both requirements are met. After Survey-in has finished successfully, the

receiver will automatically enter fixed position Time Mode.

The Survey-in status can queried using the UBX-TIM-SVIN message for Timing or FTS products or

UBX-13003221 - R28 Early production information Page 85 of 460

## Page 101

u-blox 8 / u-blox M8 Receiver description - Manual

the UBX-NAV-SVIN message for HPG products.

The "Standard Deviation" parameter defines uncertainty of the manually provided "True

Position" set of parameters. This uncertainty directly affects the accuracy of the

timepulse. This is to prevent an error that would otherwise be present in the timepulse

because of the initially inaccurate position (assumed to be correct by the receiver)

without users being aware of it. The "3D accuracy" parameter in "Fixed Position" as well

as the "Position accuracy limit" in "Survey-in" affect the produced time information and

the timepulse in the same way. Note that the availability of the position accuracy does

not mitigate the error in the timepulse but only accounts for it when calculating the

resulting time accuracy.

Once a survey-in has been started, its progress is saved in non-volatile memory, and

hence continues over events such as a reset, receiver restart, or change of satellite

constellation. If a survey-in position is required using data only for a particular receiver

configuration, then any on-going survey-in should be stopped by either a UBX-CFG-

TMODE2 or a UBX-CFG-TMODE3 message with the timeMode field set to 0, then the

receiver configured as required, and then a new UBX-CFG-TMODE2 or UBX-CFG-TMODE3

message sent with the new survey-in parameters.

26 Time & Frequency Sync (FTS)

The features described in this section are only available with the FTS products

26.1 Introduction

An FTS configured receiver provides an accurate, low phase-noise reference frequency as well as

phase reference pulse (typically at one pulse per second). An FTS receiver also implements

automatic hold-over capability based on a stable VCTCXO in modules and the customer's choice

of reference oscillator in chip-based designs. It offers generic interfaces for external sources of

synchronization (suitable for external OCXOs, IEEE1588 or Synchronous Ethernet). The receiver is

optimized for stationary applications and delivers excellent GNSS sensitivity in conjunction with

assistance data.

In the rest of this description the following terminology will be used:

• Disciplined oscillator: an oscillator whose frequency is corrected by a more stable frequency

reference, such as a GNSS system.

• Internal oscillator: the mandatory disciplined oscillator which is used as the reference frequency

for the GNSS receiver subsystem. The output from this oscillator is also available to the

application as an output from the module.

• External oscillator: an optional oscillator, disciplined by the receiver, either via I2C DAC or via

UBX messages handle by a host.

• Source: a source of frequency and/or phase synchronization either measured by the receiver

based on direct hardware input or an offset estimated by an external timing sub-system with

respect to the receiver output. Sources are handled according to related estimates of

uncertainty delivered by the application or (for oscillators) configurable models provided by the

receiver.

• Holdover: periods when GNSS measurements of sufficient quality to maintain time/frequency

are not available.

UBX-13003221 - R28 Early production information Page 86 of 460

## Page 102

u-blox 8 / u-blox M8 Receiver description - Manual

In all FTS related messages the above sources are indexed as follows:

Synchronization source indexing

Source Index

Internal oscillator 0

GNSS 1

EXTINT0 (external input) 2

EXTINT1 (external input) 3

Internal oscillator measured by the 4

host

External oscillator measured by the 5

host

The following table lists FTS related messages:

FTS message summary

Message Description

UBX-CFG-SMGR Synchronization manager configuration

UBX-CFG-ESRC External source configuration

UBX-CFG-DOSC Disciplined oscillator configuration

UBX-CFG-TP5 Configures the output pulse parameters

UBX-CFG-NAV5 Configures which variant of UTC is used by the receiver

UBX-MON-SMGR SMGR monitoring message

UBX-TIM-DOSC Message containing disciplining command for external oscillators

controlled through the host

UBX-TIM-HOC Message allowing the host to directly control the module's

oscillators

UBX-TIM-TOS Message containing information about the preceding time-pulse

output by the receiver

UBX-TIM-SMEAS Message containing measurements of phase/frequency inputs

UBX-TIM-VCOCAL Oscillator calibration command and result report

UBX-TIM-FCHG Information about latest frequency change to an oscillator

The remainder of this chapter describes some typical use cases, introduces the Synchronization

Manager (SMGR) functionality unique to FTS products and describes the use of related

messages.

26.2 Example use cases

In this section some typical use cases are described.

26.2.1 Stand-alone synchronization system

UBX-13003221 - R28 Early production information Page 87 of 460

## Page 103

u-blox 8 / u-blox M8 Receiver description - Manual

In this example, the FTS device provides a stand-alone synchronization sub-system in the context

of, say, a small cell. The module's internal 30.72MHz VCTCXO is disciplined by the module and

provides the frequency reference to the platform. The module provides a PPS signal to

synchronize the platform's physical layer. A 1PPS (or frequency) input to the module provides

frequency and/or phase information from host timing sub-systems such as PTP or Sync-E. In the

absence of phase information from GNSS or any other source, the module relies on the VCTCXO

for synchronization holdover, augmented by any reliable source of frequency control. In the

absence of frequency control, the holdover performance is determined entirely by the VCTCXO.

In some applications holdover performance will be enhanced by using an external stable (but not

necessarily accurate) frequency reference.

26.2.2 Oscillator control via host

UBX-13003221 - R28 Early production information Page 88 of 460

## Page 104

u-blox 8 / u-blox M8 Receiver description - Manual

The frequency offset of the external oscillator is measured by the FTS device and communicated

to the host which can then make any corrections necessary. The FTS device also generates a PPS

phase reference internally (with no guarantee of coherence with the external oscillator). During

holdover, the phase of 1PPS signal is maintained using either the primary reference oscillator or

the 1PPS_In signal, according to their respective uncertainty.

26.2.3 Oscillator control via directly-connected DAC

In this use case, the FTS device disciplines an external oscillator via an external DAC. During

holdover the input to the external DAC is frozen and the phase of the time pulse output is

maintained by the primary reference oscillator, but only guaranteed to be fully coherent with the

internal oscillator. The FTS receiver can also be commanded to perform a one-off calibration of

the tuning slope of external oscillator if necessary.

26.2.4 External (coherent) PPS

UBX-13003221 - R28 Early production information Page 89 of 460

## Page 105

u-blox 8 / u-blox M8 Receiver description - Manual

In this use case, the system PPS is generated by an external device from the output of the primary

reference oscillator. The FTS receiver measures the phase of this PPS input against GNSS time or

the best available source. Any small phase corrections necessary can be made by the receiver via

adjustments to the oscillator frequency or directly by the host to the PPS generator (e.g. to

accelerate removal of large phase errors). During holdover the DAC input is frozen.

26.3 Synchronization Manager Concept

The Synchronization Manager (SMGR) assumes the frequency and phase control functions in FTS

configured devices. The SMGR uses internal and external phase and frequency measurements to

derive the disciplining values (necessary frequency changes) and to assess the quality

(uncertainty) of the time pulse signal and the frequency outputs. The SMGR considers the

following synchronization sources:

• The GNSS solutions

• Internal oscillator

• Up to two external signals: frequency or time pulse (e.g. 1PPS) reference signals on EXTINT0

and/or EXTINT1

• Externally conducted measurements, from which the results are sent to the receiver through

one of the host interfaces

Each measurement provides frequency offset and/or phase information along with an estimate of

the uncertainty of each. The SMGR functional block diagram is given below:

UBX-13003221 - R28 Early production information Page 90 of 460

## Page 106

u-blox 8 / u-blox M8 Receiver description - Manual

The user has the option to configure how the SMGR considers the external signals, e.g. time or

frequency source, disciplined or not, etc... The user must also configure the uncertainty of the

signals along with their nominal characteristics. One of the external signals may be configured as

the feedback path of a disciplined external oscillator.

The SMGR can operate in frequency locked or in phase locked mode. In frequency locked mode the

target of the SMGR is to eliminate frequency error. In phase locked mode the elimination of time

error is the goal; this may lead to intentional deviation from the correct oscillator frequency. The

correction rate in both of these modes is subject to configurable limits (see UBX-CFG-SMGR). The

SMGR runs periodically (typically once a second). Its operation consists of the following stages

each time it is executed:

• Choose the best source to be the reference, given the characteristics (phase noise and stability)

of each of the sources and the uncertainty of their measurements.

• Calculate the phase and/or frequency errors as well as their uncertainty for each of the

disciplined oscillators with respect to the reference source.

• Calculate correction for disciplined oscillators; time and/or frequency corrections are limited to

the configured limits.

• Map frequency adjustment to physical output.

The SMGR runs periodically and retrieves the most recent measurements for each source along

with the estimates about their respective uncertainty. The relative phase and/or frequency errors

of disciplined oscillators with respect to the reference are calculated from incoming

measurements and used to discipline them. The decision-making process as such does not

depend on decisions made previously, however it does rely on the estimated uncertainty for each

source, which is determined by comparing predicted and measured values over some moderate

period of time. The SMGR only uses a single reference source at any one time. It does not combine

measurements from different sources in any way. If the selected reference provides a time error

measurement then a phase locked loop is possible, otherwise the receiver automatically enters

frequency lock even if configured to maintain a phase lock.

In some cases the host software might choose to drive an oscillator directly. This may be useful

UBX-13003221 - R28 Early production information Page 91 of 460

## Page 107

u-blox 8 / u-blox M8 Receiver description - Manual

where a large timing error has accumulated (e.g. after a long period of holdover) and normal

operation would prevent the error being corrected swiftly. In this case, the host can deliberately

steer the oscillator to correct timing in large steps as configured maximum phase and frequency

change limits are not applied to adjustments commanded by the host. Another use of the direct

host-driven steering may be the calibration of other parts of the system. Use UBX-TIM-HOC

message for this functionality.

If the time error is so large that its correction would take prohibitively long even with maximum

frequency offset of the oscillator the receiver can be switched to non-coherent time pulse output

mode. In this case the sync manager is temporarily reconfigured to allow time pulse intervals that

are not coherent with the frequency output, i.e. there are more or less than the nominal number of

cycles between two pulses. The user may optionally specify a limit on time adjustments. The

output mode can be set to coherent again once the time error is sufficiently small.

A SMGR summary status is provided by UBX-MON-SMGR message.

The SMGR runs at the navigation rate set by UBX-CFG-RATE. For FTS configured

devices, it is not recommended to use navigation rates higher than 1Hz.

26.4 Oscillator and source specification

For correct operation, the frequency, phase and stability characteristics of all sources and

disciplined oscillators must be described. External synchronization sources are configured with

UBX-CFG-ESRC and disciplined oscillators with UBX-CFG-DOSC. The models (short and long term

stability behavior) specified by these messages provide the SMGR with the knowledge necessary

to its decision making.

The user must also configure the method (coherent or non-coherent) used for frequency

adjustment, the maximum frequency adjustment and other parameters contained in UBX-CFG-

DOSC.

It is assumed that an external voltage-controlled oscillator has a constant ratio of relative

frequency change to control voltage change. The oscillator is therefore characterized by two

metrics: an offset (control voltage for nominal frequency) and a gain (relative frequency change

per control step). Each of these parameters are known along with their uncertainty. It is assumed

that the oscillator control gain is stable over time but its offset may change significantly with

aging. Because of the drift of the offset, its saved value is regularly updated in the model. The

gain, on the other hand, is only updated on demand by the host application by re-configuration or

calibration. For the measurement of the gain a special auto-calibration is available, described in

the calibration section.

External oscillator stability (frequency changes) is described by four parameters (see UBX-CFG-

DOSC):

• changes with temperature: withTemp is the maximum deviation limit from the nominal

frequency at the reference temperature over the supported temperature range (in ppb) and

timeToTemp (in s) which is a period after which the maximum deviation limit is reached.

• aging: maxDevLifeTime is the maximum deviation from the nominal frequency (in ppb) and

withAge is the oscillator stability with age (in ppb/year).

UBX-13003221 - R28 Early production information Page 92 of 460

## Page 108

u-blox 8 / u-blox M8 Receiver description - Manual

26.5 Calibration

Prior to disciplining an oscillator, the SMGR must have an accurate knowledge of the controlled

oscillator's frequency control gain and initial frequency offset (oscillator gains may differ

significantly from unit to unit and batch to batch, largely as a result of different crystal Q). The

receiver provides a slope measurement utility to aid the calibration process.

The calibration utility is a special mode where all disciplining operations are suspended and

therefore all disciplined oscillators, internal or external, cease to produce usable outputs. It takes

place in response to a specific request (UBX-TIM-VCOCAL message) from the host to do so for a

particular oscillator and only one oscillator can be calibrated at a time. During this phase, the

SMGR forces large frequency variations by changing the input of the digital to analogue

conversion device whose output is driving the oscillator. Several frequency measurements are

performed and a gain is estimated.

Calibration parameters must be configured or the calibration utility called before disciplining

operation is possible. Once calibrated, the calibStatus flag in UBX-CFG-DOSC is set. The

calibration utility can be re-triggered at any time by issuing the appropriate command through the

UBX-TIM-VCOCAL message (not recommended during normal operation). An ongoing calibration

process can be aborted using the same message with the appropriate flags. It can also be

bypassed if the calibStatus flag in the UBX-CFG-DOSC message is set to 1 (oscillator is calibrated

independently with results saved using the UBX-CFG-DOSC message).

In order to enter the calibration mode it is required that:

• A stable frequency source is available for the duration of the calibration. This source may be a

GNSS solution or a frequency signal on an EXTINT pin.

• The oscillator subject to calibration is configured through the UBX-CFG-DOSC message

(including an initial estimate of gain) and available for the duration of the process.

For an external oscillator it is also assumed that the useful range of the input is covered by the

output of the DAC and that the relation frequency versus DAC input is linear. Once the calibration

operation is complete the receiver will issue a UBX message to indicate that the SMGR is reverting

to normal operation and to report the results of the calibration. A default for the internal oscillator

is available in the firmware.

Note that it is important that only the chosen frequency source is enabled during the calibration

process and that it remains stable throughout the calibration period; otherwise incorrect

oscillator measurements will be made and this will lead to miscalibration and poor subsequent

operation of the receiver.

UBX-13003221 - R28 Early production information Page 93 of 460

## Page 109

u-blox 8 / u-blox M8 Receiver description - Manual

26.6 FTS device Output and Top Of Second (TOS) message

The outputs available from an FTS device can be one or all of the following:

• A disciplined frequency source at the same frequency as the internal oscillator.

• A 1PPS or an even second signal (other similar rates are possible) coherent with the internal

oscillator, configured by UBX-CFG-TP5.

• Messages reporting measurement results (for example for a host disciplined external

oscillator).

• A UBX-TIM-TOS message which describes the current condition (accuracy, coherent or non-

coherent, etc...) of the frequency and PPS outputs.

• DAC command for disciplined external oscillators.

The top of second (TOS) message is a summary of the FTS device's status. It is output shortly

after each time pulse and so will normally be aligned to the second of the reference time (if

available). To guarantee that this message is output as the first message after the time pulse a

system of time slot reservation is provided for all communication interfaces towards the host. For

more information on this mechanism refer to the description of TX time slots

Users of the FTS variant are expected to use the UBX-TIM-TOS message to obtain key

parameters for each time pulse. The UBX-TIM-TP message is only supported for

compatibility with timing receivers and is not guaranteed to provide the most

appropriate information in all FTS use cases.

The time pulse of an FTS device is generated differently from that of other u-blox receivers.

FTS products support two modes of time pulse generation: "coherent" and "non-coherent" pulses.

"Coherent" pulse generation means that the number of clock cycles between two pulses is always

the same. When in "non-coherent" pulse mode the receiver may change the number of clock cycles

between two pulses if it can thus reduce the phase error of the time pulse. The receiver can be

configured (using UBX-CFG-SMGR) to operate in either of these modes or to switch from "non-

coherent" to coherent mode after initial frequency and phase error has been eliminated.

It can be useful to instruct the receiver to enter the "non-coherent" pulse mode during startup or

while recovering from holdover; it reduces the time necessary for phase convergence. After the

phase error is reduced the host can instruct the FTS receiver to switch back to "coherent" mode

again.

The UBX-TIM-TOS message, when enabled, indicates the actual mode of pulse generation.

Depending on the time pulse generation mode, the time pulse can be forced to be phase aligned to

the oscillators. In coherent output mode the phase offset of the oscillator at the rising edge of the

time pulse is defined by the phaseOffset field of UBX-CFG-DOSC. In "non-coherent" mode this

constraint is ignored.

The phase offset is handled differently for both oscillators. Whereas phase lock between

the internal oscillator and the time pulse is guaranteed by hardware, in the case of the

external oscillator the lock is achieved by software and that lock is therefore the lock

behavior is expected to be different.

The frequency, shape and offset of the time pulse can be configured with the UBX-CFG-TP5

message. Some of the fields are interpreted differently by FTS devices compared to other u-blox

receivers. Among others the lockGnssFreq flag is ignored and the time pulse is always aligned to

the best synchronization source. Furthermore, switching between the two time pulse frequency

and length parameters is not governed by GNSS alone but by the condition selected in the

UBX-13003221 - R28 Early production information Page 94 of 460

## Page 110

u-blox 8 / u-blox M8 Receiver description - Manual

syncMode field.

Two delay parameters can be configured using UBX-CFG-TP5, antCableDelay and

userConfigDelay. In an FTS product care should be taken what delays are attributed to

which of the delay terms. The antenna cable delay is only relevant when the receiver is

following GNSS as reference; the user-configurable delay is applied regardless of the

active reference signal.

In current FTS products only TIMEPULSE 2 can be used for pulse generation.

Additionally, just 0.5 Hz, 1 Hz and 2 Hz time pulse output is supported by current FTS

products. Other output frequencies may be configured with UBX-CFG-TP5 but are not

guaranteed to work properly.

26.7 Message transmission time slot reservations on host interfaces

The firmware provides three message transmission time slots that are aligned to the time pulse

output of the receiver. No message is scheduled for transmission in the first slot after the leading

edge of the time pulse. The second slot is reserved for the UBX-TIM-TOS message and the third

slot is used for outputting other messages. However, any message transmission that was started

will be finished before a new message is started.

The time slots can be enabled and configured using UBX-CFG-TXSLOT.

When the reference time pulse is disabled or runs at a high frequency it may happen

that many or all outgoing messages are lost. Therefore the time slot mechanism should

be configured to match the time pulse behavior or disabled altogether.

This mechanism only controls when a message transmission may start and does not guarantee

that the message transmission will finish before the end of the corresponding slot. Therefore the

end of the last slot should be configured such that the longest enabled message can still be

transmitted before the period starts when the receiver must not transmit messages.

The timing of the actual message output is also dependent on the communication

interface and its clocking. On the slave interfaces (DDC and SPI) the host must provide

clock in all time slots for this feature to work.

26.7.1 Example setup

Following is an example scenario. The receiver is set up to output a time pulse at a 1 Hz rate.

Suppose that the following requirements are given for system integration:

• The TOS message should be output 10 to 50 ms after the time pulse.

• No other message should be output from the leading edge of the time pulse until 50 ms after

the time pulse.

• The longest enabled message takes up to 100 ms to transmit through the chosen interface with

the configured speed.

Then the time slots are enabled and the three slots are configured to end 10, 50 and 900 ms after

the pulse respectively. The following figure indicates time pulses with upwards pointing arrows.

Slot 0 (the first one active immediately after the time pulse) is active and thus blocks the

transmission of new messages from 100 ms before the time pulse until 10 ms after it. Time slot 1,

i.e. the time between 10 and 50 ms after the pulse, is reserved for the top-of-second message. All

other messages are output in slot 2.

UBX-13003221 - R28 Early production information Page 95 of 460

## Page 111

u-blox 8 / u-blox M8 Receiver description - Manual

27 RTK Mode Configuration

This feature is only available with the High Precision GNSS products

u-blox RTK technology introduces the concept of a reference station and a rover. Using the

RTCM3 protocol, the reference station sends corrections to the rover via a communication link

enabling the rover to compute its position relative to the reference with high accuracy.

In the high precision GNSS context, the terms reference station and base station can be

used interchangeably.

The distance between the reference station and the rover is called baseline length.

The reference station can provide correction to several rovers but the rover cannot

concurrently process corrections from several reference stations.

The remainder of this chapter describes how to configure the reference station and the rover.

More details about the RTCM3 protocol can be found in the RTCM3 section.

27.1 Reference Station Mode Configuration

Reference Station Mode is a special receiver mode where the receiver uses measurements from all

available satellites to broadcast corrections. Configuring a stationary reference station is done in

two steps:

• The receiver must be set in Time Mode using the configuration steps described in the Time

Mode Configuration section.

• The RTCM3 correction stream must be configured following the rules detailed in the RTCM3

Configuration section. Each RTCM message must be individually enabled using UBX-CFG-MSG.

By default the reference station will begin operation in standard GNSS mode without any

RTCM output. Messages for observations will be streamed as soon as they are

configured for output. However messages for the reference station position will only be

output when both the reference station is in fixed position mode, and the message is

configured for output. As explained in the Time Mode Configuration section, this mode

can be directly configured or reached at the end of a successful survey-in.

The rover will need to have received both reference station observation messages and

reference station position messages in order to attempt ambiguity fixes.

When the reference station is in Time Mode, some error checking is performed on the

entered, or surveyed-in, fixed position. If the result of these checks indicates that the

fixed position may be incorrect, then a UBX-INF-WARNING message will be sent, with the

text "Reference Station position seems incorrect".

UBX-13003221 - R28 Early production information Page 96 of 460

## Page 112

u-blox 8 / u-blox M8 Receiver description - Manual

27.2 Rover Mode Configuration

The RTK rover can be configured to work in either of these two differential modes using UBX-CFG-

DGNSS:

• RTK fixed: In this mode, the rover will attempt to fix ambiguities whenever possible.

• RTK float: In this mode, the rover will estimate the ambiguities as float but will make no

attempts at fixing them.

The time after which old RTCM data will be discarded can be specified using the dgnssTimeout

field in UBX-CFG-NAV5.

By default the rover will begin operation in RTK fixed mode. Upon receiving an RTCM3

correction stream on any of its communication interfaces, the rover will parse the data,

apply the correction and, if possible, fix ambiguities. In absence of correction data or if

the correction data times out, the rover will operate in standard GNSS mode.

The time needed to resolve the ambiguity is affected by the baseline length as well as by

multipath and satellite visibility at both rover and reference station.

27.3 Moving Baseline RTK Configuration

The moving baseline (MB) RTK mode differs from the standard RTK mode in that it does not

require the reference to be stationary at a known location. In MB RTK mode, both the reference

station and rover receivers can move while computing a centimeter-level accurate 3D vector

between them. This is ideal for applications where the relative position offset between two moving

vehicles is required such as, for example, the follow-me feature on a UAV.

For the sake of conciseness, in the moving baseline RTK context, the reference station

and rover receivers are referred to as MB reference and MB rover, respectively.

27.3.1 MB Reference Configuration

Configuring a receiver to operate in MB reference mode is done in two steps:

• The receiver must be set in Time Mode disabled using the configuration message UBX-CFG-

TMODE3.

• The RTCM3 correction stream must be configured following the rules detailed in the RTCM3

Configuration section. Each RTCM message must be individually enabled using UBX-CFG-MSG.

If the MB reference moves, then its position changes over time. To ensure that the baseline is as

accurate as possible:

• The MB reference position must be sent for each epoch the MB reference observations are sent.

• The MB reference and rover must use the same navigation update rate.

27.3.2 MB Rover Configuration

As in the standard RTK mode, it is possible to configure the MB rover to operate in RTK fixed or

RTK float using the UBX-CFG-DGNSS message.

By default the MB rover will begin operation in RTK fixed mode.

As discussed in the Moving Baseline Expected Performance section, RTCM corrections

can only be extrapolated over a few seconds when both reference and rover receivers are

moving. Therefore, any dgnssTimeout value configured using the UBX-CFG-NAV5

message will be ignored by the MB rover.

UBX-13003221 - R28 Early production information Page 97 of 460

## Page 113

u-blox 8 / u-blox M8 Receiver description - Manual

27.3.3 Expected Performance

While the MB RTK solution aims at estimating the relative position with centimeter-level

accuracy, the absolute position of each receiver is expected to be known with a standard GNSS

accuracy of a few meters. Additionally, the performance of the MB RTK solution is limited by the

following:

• A moving reference receiver typically experiences worse GNSS tracking than a static reference

receiver in an open-sky environment and therefore the MB RTK performance may be degraded.

• The MB rover can only compute an optimal MB RTK solution if the time-matched RTCM

observation and position messages are received within a predefined time limit. The MB rover will

wait up to 700 ms for messages before falling back to an extrapolated MB RTK solution. The MB

rover will extrapolate the MB reference observations and/or position for up to 3 s before falling

back to standard GNSS operation.

• The achievable update rate of the MB RTK solution is limited by the communication link latency.

As a rule of thumb, the communication link latency should be about half the desired navigation

update period. If it exceeds 700 ms, the MB rover will not be able to compute an MB RTK

solution, even at 1 Hz.

• Since the MB rover must wait for time-matched RTCM corrections from the MB RTK reference

to compute its position, the overall latency of the MB RTK solution will be the sum of the

communication link latency plus the MB RTK computation time.

When falling back to standard GNSS operation, the MB rover will automatically adjust

the accuracy and status flag information contained in the messages listed in the

RTCM3 Output section.

Upon recovering the RTCM correction stream, the MB rover will automatically try to

revert to MB RTK operation.

28 Automotive Dead Reckoning (ADR)

This feature is only available with the ADR products.

28.1 Introduction

u-blox solutions for Automotive Dead Reckoning (ADR) allow high-accuracy positioning in places

with poor or no GNSS coverage. ADR is based on Sensor Fusion Dead Reckoning (SFDR)

technology, which combines GNSS measurements with those from external sensors.

ADR solutions use the messages of the External Sensor Fusion (ESF) class.

28.2 Solution Types

28.2.1 GAWT: Gyroscope, Accelerometer and Wheel Tick Solution

The GAWT solution combines data from wheel-tick sensors, accelerometers and gyroscopes to

compute a fused navigation solution. There are several different possible GAWT variants,

depending on which sensors are available. The minimum set of sensors required for computing

GAWT solutions is:

• A speed/distance sensor providing a single wheel tick (sometimes called a speed tick) or speed

measurement;

• A z-axis gyroscope measuring the vehicle yaw rate;

UBX-13003221 - R28 Early production information Page 98 of 460

## Page 114

u-blox 8 / u-blox M8 Receiver description - Manual

• An x-axis accelerometer measuring the vehicle forward-backward acceleration.

The solution may be further improved by using the following additional sensors:

• A 3-axis accelerometer can improve the height estimation accuracy;

• If the z-axis gyroscope is not aligned to the vehicle vertical axis then a 3-axis gyroscope with

IMU-mount misalignment configuration (UBX-CFG-ESFALG) will allow the receiver to re-create

the output of a correctly aligned z-axis gyroscope. This will result in improved planimetric

accuracy compared to a single mis-aligned z-axis gyroscope.

• A temperature sensor can be used to compensate for temperature-dependent gyroscope

errors. Depending on the sensor specification and temperature variation, this can significantly

improve performance during periods of dead reckoning (see Gyroscope Configuration section for

more details).

To operate ADR products in GAWT mode, the following tasks need to be completed:

• Sensor configuration (only for chipset products): the Wheel-Tick/Speed Sensor, the Gyroscope

and the Accelerometers settings must be set up, and the Sensor Time Tagging must be

properly configured. If the sensors data are properly fed to the receiver and configuration is

successful, the sensors should appear in the UBX-ESF-STATUS message.

In ADR module products (NEO-M8L), the receiver is ready to operate in ADR (GAWT)

navigation mode (this note is only valid in protocol versions 15.01+).

• Installation configuration: the IMU-mount Misalignment should be accurately configured for

the receiver to achieve fusion solution.

Once these steps are completed, the firmware is ready to be operated in ADR GAWT navigation

mode.

28.3 Installation Configuration

If the GNSS antenna is placed at a significant distance from the receiver, position

offsets can be introduced which might affect the accuracy of the navigation solution. In

order to compensate for the position offset advanced configurations can be applied.

Contact u-blox support for more information on advanced configurations.

28.3.1 IMU-mount Alignment

(This feature is not supported in protocol versions less than 15.01).

The default assumption is that the IMU-frame and the installation-frame have the same

orientation (i.e. all axes are parallel). If this assumption is not valid, the positioning solution can be

degraded if the IMU-mount misalignment angles are small (typically few degrees) or can even fail

in case of large (tens of degrees) IMU-mount misalignments. Therefore, it is important to correctly

configure the IMU-mount misalignment settings by using the UBX-CFG-ESFALG configuration

message.

This section describes how IMU-mount misalignment angles, i.e. the angles which rotate the

installation-frame to the IMU-frame, can be configured using the UBX-CFG-ESFALG configuration

message (see User-defined Configuration section below).

If the IMU-mount misalignment angles are unknown, they can be estimated during a dedicated

initialization drive through an automatic alignment procedure. This is described in the Automatic

IMU-Mount Alignment section below.

In u-blox module products containing an internal IMU (e.g. NEO-M8U modules), the IMU-

UBX-13003221 - R28 Early production information Page 99 of 460

## Page 115

u-blox 8 / u-blox M8 Receiver description - Manual

mount misalignment angles are estimated automatically by default (see Automatic

IMU-Mount Alignment section below for further details).

28.3.1.1 Definitions

The IMU-mount misalignment angles are defined as follows:

• The transformation from the installation-frame to the IMU-frame is described by three Euler

angles about the installation-frame axes denoted as IMU-mount roll, IMU-mount pitch and IMU-

mount yaw angles. All three angles are referred as the IMU-mount misalignment angles.

There is a single IMU-mount misalignment configuration that applies to both

gyroscopes and accelerometers, so these sensors must be aligned with each other if

both types are present.

28.3.1.2 User-defined IMU-mount Alignment

The user can configure manually some IMU-mount roll, pitch and yaw angles using the UBX-CFG-

ESFALG configuration message. The values that should be set in the configuration message are

the Euler angles required to rotate the installation-frame to the IMU-frame. The IMU-mount yaw

rotation should be performed first, then the IMU-mount pitch and finally the IMU-mount roll. At

each stage, the rotation is around the appropriate axis of the transformed installation-frame,

meaning that the order of the rotation sequence is important (see figure below).

If there is only a single IMU-mount misalignment angle then it may be measured as shown in the

three examples below.

UBX-13003221 - R28 Early production information Page 100 of 460

## Page 116

u-blox 8 / u-blox M8 Receiver description - Manual

In order to prevent significant degradation of the positioning solution the IMU-mount

misalignment angles should be configured with an accuracy of at least 5 degrees.

The list below describes in details how the fields in the UBX-CFG-ESFALG message must be

interpreted with respect to example illustrated in the figure above:

• User-defined IMU-mount yaw angle: The IMU-mount yaw angle (yaw) corresponds to the

rotation around the installation-frame z-axis (vertical) required for aligning the installation-

frame to the IMU-frame (yaw = 344.0 deg if the IMU-mount misalignment is composed of a

single rotation around the installation-frame z-axis, i.e. with no IMU-mount roll and IMU-mount

pitch rotation).

• User-defined IMU-mount pitch angle: The IMU-mount pitch angle (pitch) corresponds to the

rotation around the installation-frame y-axis required for aligning the installation-frame to the

IMU-frame (pitch = 26.5 deg if the IMU-mount alignment is composed of a single rotation

around the installation-frame y-axis, i.e. with no IMU-mount roll and IMU-mount yaw rotation).

• User-defined IMU-mount roll angle: The IMU-mount roll angle (roll) corresponds to the

rotation around the installation-frame x-axis required for aligning the installation-frame to the

IMU-frame (roll = -23.5 deg if the IMU-mount misalignment is composed of a single rotation

around installation-frame x-axis, i.e. with no IMU-mount pitch and IMU-mount yaw rotation).

UBX-13003221 - R28 Early production information Page 101 of 460

## Page 117

u-blox 8 / u-blox M8 Receiver description - Manual

If automatic alignment is turned-on (see Automatic IMU-mount Alignment section), the

angles obtained by polling UBX-CFG-ESFALG are still the user-defined angles which do

not correspond to the result of the automatic IMU-mount alignment engine as output in

UBX-ESF-ALG (see IMU-mount Misalignment Angles Output section for more details).

28.3.1.3 Automatic IMU-mount Alignment

The automatic IMU-mount alignment engine estimates automatically the IMU-mount roll, pitch

and yaw angles. It requires an initialization phase during which no INS/GNSS fusion can be

achieved (see Filter Modes section for further details). The progress of the automatic alignment

initialization can be monitored with the UBX-ESF-STATUS message, and/or with the UBX-ESF-ALG

message providing more details. When the vehicle is subject to sufficient dynamics (i.e. left and

right turns during a normal drive), the automatic IMU-mount alignment engine will estimate the

IMU-mount misalignment angles which have the same meaning as defined in the Definitions

section, regardless whether the user did or not enter manually some IMU-mount misalignment

angles (see User-defined Configuration section). Once the automatic IMU-mount alignment

engine has sufficient confidence in the estimated initial IMU-mount misalignment angles, the

IMU-mount misalignment angles initialization phase is completed. The raw accelerometer and

gyroscope data (i.e. the IMU observations) are then compensated for IMU-mount misalignment

and sensor fusion can be done. The resulting IMU-mount misalignment angles are output in the

UBX-ESF-ALG message.

For automatic IMU-mount alignemnt a 3-axis gyroscope and 3-axis accelerometer is

required (only valid in protocol versions 19.2+).

28.3.1.3.1 Enabling/Disabling Automatic IMU-mount Alignment

The user can activate/deactivate the automatic IMU-mount alignment by setting the

doAutoMntAlg bit in the UBX-CFG-ESFALG configuration message.

If automatic IMU-mount alignment is deactivated while aligning, the estimated

misalignment angles that were available at deactivation time are used (only if they were

initialized, see next section). If automatic IMU-mount alignment is re-activated,

alignment is pursued by starting from the state where deactivation happened (only valid

in protocol versions 19+).

28.3.1.4 Limitation with Single-Axis Gyroscope

Gyroscope-mount misalignment is only supported when a three-axis gyroscope is available. In

case of a single-axis gyroscope, the sensor should be physically aligned along the installation-

frame z-axis. This is needed to avoid a scale factor error which will affect the accuracy of the

output due to the two missing gyroscopes.

28.4 Sensor Configuration

This section describes the external sensor configuration parameters.

28.4.1 Accelerometer Configuration

The accelerometer sensor senses specific forces, expressed in meters per seconds squared, along

its input axis. In the full configuration, an IMU contains a three-axis accelerometer whose sensitive

axes are assumed to be mutually orthogonal in a Cartesian frame.

UBX-13003221 - R28 Early production information Page 102 of 460

## Page 118

u-blox 8 / u-blox M8 Receiver description - Manual

28.4.1.1 Messages

The accelerometer sensor can be configured in the following messages (only supported in protocol

versions 15.01+):

Configuration Messages for ADR Products

Product Type Message Solution Type

Chipset UBX-CFG-ESFA UDR( only supported in protocol

versions 19.2+)

28.4.2 Gyroscope Configuration

The gyroscope sensor senses angular rates, expressed in radians per seconds or degrees per

second, along its input axis. In the full configuration, an IMU contains a three-axis gyroscope

whose sensitive axes are assumed to be mutually orthogonal in a Cartesian frame.

28.4.2.1 Messages

The gyroscope sensor can be configured in the following messages (only supported in protocol

versions 15.01+):

Configuration Messages for ADR Products

Product Type Message Solution Type

Chipset UBX-CFG-ESFG UDR( only supported in protocol

versions 19.2+)

28.4.2.2 Temperature Compensation

Gyroscope sensors generally exhibit a temperature-dependent bias that varies from unit to unit.

To help compensate for this variation the receiver builds up a table of gyroscope bias versus

temperature measurements which are often available from the gyroscope sensor itself. This is

particularly valuable to dead-reckoning-only navigation after the vehicle has been left for some

time in parking garage.

The gyroscope temperature compensation engine has the following settings:

• Gyroscope RMS threshold above which temperature table is not updated: The gyroscope

temperature-dependent bias is only updated if the measured gyroscope angular rate RMS is

below the given threshold. This avoids artificially high estimates of the gyroscope temperature-

dependent bias from transient events such as vehicle engine starts or nearby heavy

construction. This threshold can be configured in the gyroRmsThdl field and is shared with the

sensor accuracy estimation engine (see above);

• Temperature-dependent bias table saving rate: Gyroscope temperature compensation data

are saved to non-volatile storage at intervals that can be configured by the tcTableSaveRate

field.

The gyroscope temperature-dependent bias table is revised under the following conditions:

• The vehicle is stationary (without wheel-tick measurements or at zero speed);

• The RMS of the measured gyroscope angular rates and accelerometer specific forces is below a

given threshold (see above);

• Turntable mode is not engaged (only for ADR products, see Ferry and Turntable Modes section);

Gyroscope temperature compensation is effective if the gyroscope(s) exhibits

repeatable characteristics with temperature and is not unduly affected by external

UBX-13003221 - R28 Early production information Page 103 of 460

## Page 119

u-blox 8 / u-blox M8 Receiver description - Manual

factors (such as supply voltage or mechanical stress).

28.4.3 Wheel-Tick/Speed Sensor Configuration

28.4.3.1 Messages

The wheel-tick sensor can be configured in the following messages:

Configuration Messages for ADR Products

Product Type Message Solution Type

Module (e.g. NEO-M8L) UBX-CFG-ESFWT GAWT

28.4.3.2 Sensor Types

u-blox products support sensors delivering the following types of data:

• Relative wheel-tick data: If the wheel-tick sensor delivers relative wheel-tick counts (i.e. wheel-

tick count since the previous measurement), the wtCountMax value must be set to 0.

• Absolute wheel-tick data: If the wheel-tick sensor delivers absolute wheel-tick counts (i.e.

wheel-tick count since startup at time tag 0) that always increase, regardless of driving forward

or backward (driving direction is indicated separately, see the ESF Measurement Data section),

the wtCountMax value must be set to any non-zero value.

By default, the maximum absolute wheel-tick counter value is automatically estimated

by the receiver for a maximum counter value that can be represented as a 2^N value.

Other maximum counter values must be manually configured. For example, a

wtCountMax=1024 roll-over value would be automatically estimated, but a wtCountMax=1

000 must be configured. The maximum counter value is configured by setting the

autoWtCountMaxOff bit and setting the wtCountMax value to the upper threshold of the

absolute wheel-tick sensor count before starting again from zero (roll-over). (This note is

only valid in protocol versions 19+).

If absolute wheel-tick data are used, the upper threshold towards which the absolute

wheel-tick sensor counts ticks before starting again from zero (roll-over) must be

configured in the wtCountMax field (This note is only valid in protocol versions less than

19).

• Speed data: The sensor delivers speed data in meters per second (data type 11 in ESF-MEAS).

Data coming from this sensor type can only be delivered to the receiver via serial port (software

interface).

If speed data but no absolute or relative wheel-tick data are detected, the receiver

automatically uses the speed data without the need of reconfiguring the useWtSpeed

bit. This behaviour can be deactivated by setting the autoUseWtSpeedOff bit and by

manually setting or clearing the useWtSpeed bit. If wheel-tick data (or both wheel-tick

and speed data) are detected on the software interface, the receiver uses the data type

(by default wheel-tick data) corresponding to the configured useWtSpeed bit value (This

note is only valid in protocol versions 19+).

To make the receiver interpret incoming speed data (data type 11 in ESF-MEAS) instead

of the single wheel-tick data (data type 10 in ESF-MEAS) on the software interface, the

useWtSpeed bit must be set (This note is only valid in protocol versions less than 19).

It is strongly recommended to use absolute wheel-tick sensors in order to ensure robust

measurement processing even after sensor failures or outages.

UBX-13003221 - R28 Early production information Page 104 of 460

## Page 120

u-blox 8 / u-blox M8 Receiver description - Manual

28.4.3.3 Interface

Wheel-tick/speed data can be delivered to u-blox products via the following interfaces:

• Hardware interface: Some u-blox products (e.g. NEO-M8L modules) have a pin dedicated to

analog wheel-tick signal input and a pin dedicated to the wheel-tick direction signal. The

receiver checks for analog wheel-tick signal input and will use it if the pin is correctly connected,

the useWtPin flag is set (this is the default configuration for products having a pin dedicated to

analog wheel-tick signal input), and the analog direction pin polarity is configured.

The analog direction signal polarity is automatically detected by the receiver. To

manually configure the polarity, automatic detection must be turned-off by setting the

autoDirPinPolOff bit and the polarity must be defined in the dirPinPol field (This

note is only valid in protocol versions 19+).

The analog direction signal polarity must be configured in the dirPinPol field (This note

is only valid in protocol versions less than 19).

Double edge counting can be enabled via the cntBothEdges flag. It can increase performance with

low resolution wheel ticks. It does not fit all kinds of wheel tick signals. It must not be used with

signals that are not generated with approximately 50% duty signal as it would worsen

performance.

• Software interface: The sensor data are delivered to the receiver on the serial port (software

interface) in the form of UBX-ESF-MEAS messages. Serial port can be configured for UART using

the UBX-CFG-PRT message. For products with a hardware interface for analog wheel-tick signal

input (e.g. NEO-M8L modules), the useWtPin bit must not be set if sensor data delivered via

serial port should be used (only in protocol versions less than 19).

By default, the receiver automatically switches-off the hardware interface (i.e. ignores

the useWtPin flag) if wheel-tick/speed data are detected on the software interface.

Therefore data coming from the software interface will be prioritized over data coming

from the hardware interface. To disable the automatic use of data detected on the

software interface, the autoSoftwareWtOff bit must be set (This note is only valid in

protocol versions 19+).

28.4.3.4 Settings

The following sensor settings can be configured:

• Sampling Frequency: The wheel-tick/speed data sampling frequency (wtFrequency) should be

provided with an accuracy of about 10%. If not provided, it is automatically determined during

initialization phase: this requires a consistent data rate and can take several minutes. Once

initialized, the sampling frequency will be stored in non-volatile storage. For optimal navigation

performance, the standard wheel-tick/speed input at 10 [Hz] is recommended.

• Accuracy: The wheel-tick/speed data accuracy (wtAccuracy) is defined as the standard

deviation under normal operating conditions. Wheel-tick/speed data are corrupted by noise

from sources inherent to the sensor. The accuracy is automatically determined and will then be

stored in non-volatile storage.

• Latency: For best positioning performance, the latency of the wheel-tick/speed data (wtLatenc

y) should be given as accurately as possible (to within at least 10 ms). If not provided, the wheel-

tick/speed data latency is assumed zero. More details about latency can be found in the Sensor

Time Tagging section.

• Quantization error: If absolute/relative wheel-tick data are used and the tick data do not contain

UBX-13003221 - R28 Early production information Page 105 of 460

## Page 121

u-blox 8 / u-blox M8 Receiver description - Manual

raw tick counts (e.g. if the tick data is a distance), the quantization error can be defined in the

wtQuantError or quantError fields. The quantization error can be calculated as 2*Pi*R / T

with R the wheel radius, T the number of ticks per wheel rotation. If the quantization error is not

provided, it is automatically initialized by the receiver.

• Sensor dead band: Some wheel-tick or speed sensors have a dead band which is the value below

which no speed is reported. If this is the case, the value needs to be configured in the

speedDeadBand field. However, the performance will still be degraded compared to having no

dead band. If not provided, the receiver assumes the sensor has no dead band.

• Speed data accuracy: If speed data are used, the speed data accuracy can be set in the

wtQuantError or quantError field. If not provided, the speed data accuracy is automatically

initialized by the receiver.

• Scale factors: If the coarse scale factors are not configured by the user (wtFactor, factorR,

factorF), they are estimated automatically during initialization (see Initialization Mode section

for more details).

• Combination of multiple rear wheel-ticks: The receiver can be configured to use the combined

rear wheel-ticks rather than the single-tick. It is recommended to use combined rear wheel-ticks

if available, as they are often of higher quality than the single-ticks. If DWT, GWT and GAWT

solutions are configured concurrently, combineTicks must be set to provide a consistent

configuration. If combineTicks is set, the wheel-ticks basis settings (maximum value of the

wheel-ticks counter, wheel-ticks sensor frequency, scale factors and quantization error) must

reflect the properties of the rear wheel-ticks.

28.4.4 Sensor Time Tagging

In order to achieve optimal performance with the fusion solution it is essential to determine the

epoch in the receiver time frame when the external sensor measurements were generated. This

may be done in one of the following ways:

• First Byte Reception: reception time of first byte of UBX-ESF-MEAS message

• Time Mark on External Input: reception time of time mark signal sent to external input

The latency of the sensor data is the time between when the sensor measurement was taken and

the detection at the receiver of either the first byte of the UBX-ESF-MEAS message or the pre-

processor's time mark, depending on the timing approach chosen. Increased latency reduces the

navigation performance.

In ADR, the latency can be set by using the latency, wtLatency, gyroLatency and accelLatency

parameters in the appropriate configuration message, as discussed in the Automotive Dead

Reckoning (ADR) chapter.

In UDR, the latency can be set by using the latency parameter in the appropriate sensor

configuration message, as discussed in the Untethered Dead Reckoning (UDR) chapter.

28.4.4.1 First Byte Reception

The easiest way to determine the sensor measurement generation time is to have the GNSS

receiver assume the time of reception of the first byte of the UBX-ESF-MEAS message (minus a

constant configured latency) to be the time of sensor measurement. This approach is the

simplest to implement, but Time Mark on External Input can yield better latency control and

compensation.

UBX-13003221 - R28 Early production information Page 106 of 460

## Page 122

u-blox 8 / u-blox M8 Receiver description - Manual

28.4.4.2 Time Mark on External Input

In this case, the preprocessor unit generating the measurements sends a signal to the EXTINT

input of the GNSS receiver, marking the moment of measurement generation. The subsequent

UBX-ESF-MEAS message is then flagged accordingly, and the measurements in the message will

be assumed to have been generated at the time of external signal reception (minus a constant

configured latency). This approach is the preferred solution, but it can be difficult to realize an

exact analog time signal for the preprocessor unit.

28.4.4.3 Sensor Time Tagging Configuration

The receiver requires external sensor packets time tagged in seconds.

The external sensor time tagging for WT can be configured in the UBX-CFG-ESFWT (not supported

in protocol versions less than 15.01).

The following sensor time tagging settings need to be specified:

• Sensor time tag scale factor to seconds: (timeTagFactor): This parameters converts the

sensor time tags from their original time unit into the required seconds. For example if the IMU

raw packets are time-tagged in milliseconds, the scale factor for converting one millisecond into

one seconds is 0.001.

• Sensor time tag maximum value: (timeTagMax): External sensor time tags are encoded in

different data types (signed/unsigned, varying number of bytes) which might vary across sensor

types. For example if the IMU raw packet's time-tag field is encoded into an unsigned long

integer (4 bytes), the maximum possible time-tag value is 4294967295 (0xFFFFFFFFin

hexadecimal).

UBX-13003221 - R28 Early production information Page 107 of 460

## Page 123

u-blox 8 / u-blox M8 Receiver description - Manual

28.5 ADR System Configuration

28.5.1 Enabling/Disabling Fusion Filter

The ADR fusion filter can be turned off by means of the useAdr bit in the UBX-CFG-NAVX5

configuration message. If fusion is turned off, the receiver outputs a GNSS-only solution.

28.5.2 Recommended Configuration

For an optimum ADR navigation performance, the recommended general configuration is the

following:

• Navigation Rate: the standard navigation solution update rate of 1 Hz (see UBX-CFG-RATE

message) is recommended. The wheel tick quantization error is a limiting factor when using

high frequency updates. This means that navigation rates higher than 1 Hz may result in lower

position accuracies.

Reconsider the enabled messages and features (e.g logging) at higher navigation rates

to meet CPU load, memory and interface bandwidth constraints (Valid in protocol

versions 19.2).

28.6 Operation

This section describes how the ADR receiver operates.

28.6.1 Fusion Filter Modes

The fusion filter operates in different modes which are output in the UBX-ESF-STATUS message.

The table below summarizes the different fusion filter modes with the associated tasks the

receiver is doing.

Fusion Modes

Mode Performed Tasks / Possible Causes Published Fix

Type

Initialization Initialization of IMU 3D-Fix

Initialization of IMU-mount alignment (GNSS)

Initialization of INS (position, velocity, attitude)

Initialization of wheel-tick sensor (ADR only)

IMU sensor error (e.g. missing data) detected (only supported

in protocol versions 19.2+)

Fusion Fine-calibration of IMU-mount misalignment angles (not GNSS/DR Fix

supported in protocol versions less than 19)

Fine-calibration of IMU sensors

Fine-calibrating of wheel-tick factors (ADR only)

UDR mode under ADR / WT sensor error (e.g. missing data)

detected (ADR only)(only supported in protocol versions 19.2+)

Suspended Fusion Sensor error (e.g. missing data) detected (only supported in 3D-Fix

protocol versions less than 19.2) (GNSS)

Ferry detected (ADR only)

Disabled Fusion Fatal fusion filter error occurred 3D-Fix

Fusion filter turned-off by user (GNSS)

More details about each fusion mode are given in the following sections.

UBX-13003221 - R28 Early production information Page 108 of 460

## Page 124

u-blox 8 / u-blox M8 Receiver description - Manual

28.6.1.1 Initialization Mode

The purpose of the initialization phase is to estimate all unknown parameters which are required

for achieving fusion. The initialization phase is triggered after a receiver cold start or a filter reset

in case of fusion failure. The receiver is in initialization mode if the fusionMode field in the UBX-

ESF-STATUS message is 0:INITIALIZING. In this case the required sensor calibration status (cal

ibStatus) is flagged as 0: NOT CALIBRATED and the navigation solution output during

initialization is based on GNSS solely.

The initialization phase comprises the following internal steps whose status is published in the

initStatus field of the UBX-ESF-STATUS message:

• IMU initialization: Unknown crucial IMU parameters such as sensor sampling frequency are

estimated during initialization. As long as all required IMU parameters are not initialized, the

status of the IMU initialization (imuInitStatus) is flagged as 1:INITIALIZING in the UBX-ESF-

STATUS message. Moreover, the required sensor calibration statuses (calibStatus) are flagged

as 0:NOT CALIBRATED in the UBX-ESF-STATUS message. Note that if the user configured all

required sensor settings, this step is skipped and IMU initialization is flagged as 2:

INITIALIZED (not supported in protocol versions less than 19).

• IMU-mount alignment initialization: If automatic IMU-mount alignment is enabled (see the

Automatic IMU-mount Alignment Configuration section), initial IMU-mount roll, IMU-mount

pitch and IMU-mount yaw angles need to be estimated. For that, good GNSS signal reception as

well as sufficient vehicle dynamics (i.e. a series of left and right turns during a normal drive)

need to be at hand. As long as the IMU-mount alignment is not initialized, the status of the IMU-

mount alignment (mntAlgStatus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS

message. Once initialized, the IMU-mount alignment status is flagged as 2:INITIALIZED. If no

IMU-mount alignment is required, the IMU-mount alignment is flagged as 0:OFF. A detailed

description of the automatic IMU-mount alignment operation can be found in the Automatic

IMU-mount Alignment Operation section (not supported in protocol versions less than 15.01).

• INS initialization: Before entering fusion mode, the initial vehicle position, velocity and especially

attitude (vehicle roll, pitch heading angles) needs to be known with sufficient accuracy. This is

achieved during INS initialization phase (which comprises an INS coarse alignment step) using

GNSS. As long as the fusion filter isn't initialized, the status of the INS initialization (insInitSt

atus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the INS

initialization is flagged as 2:INITIALIZED (not supported in protocol versions less than 15.01).

This section is valid only for protocol versions less than 19.2

• Wheel-tick sensor initialization (ADR products only): Before entering fusion mode, some

parameters like initial wheel-tick factors need to be estimated with sufficient accuracy. This is

achieved during wheel-tick sensor initialization phase using GNSS. As long as the wheel-tick

parameters are not initialized, the status of the wheel-tick initialization (wtInitStatus) is

flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the wheel-tick

sensor initialization is flagged as 2:INITIALIZED and the parameters are stored in non-volatile

storage. If no wheel-tick data are required (in UDR products), the wheel-tick initialization is

flagged as 0:OFF (only valid in protocol versions less than 19.2).

This section is valid only for protocol versions 19.2+

• Wheel-tick sensor initialization (ADR products only): Solution enters fusion mode (fusionMode

field in the UBX-ESF-STATUS message is on 1:FUSION), even when wheel-tick is not yet

initialized, following a UDR mode approach. WT sensor parameters, like initial wheel-tick

UBX-13003221 - R28 Early production information Page 109 of 460

## Page 125

u-blox 8 / u-blox M8 Receiver description - Manual

factors, are estimated in parallel and are used once estimated with sufficient accuracy. As long

as the wheel-tick parameters are not initialized, the status of the wheel-tick initialization (wtIni

tStatus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the

wheel-tick sensor initialization is flagged as 2:INITIALIZED, WT data are used by the filter and

the parameters are stored in non-volatile storage. If no wheel-tick data are required (in UDR

products), the wheel-tick initialization is flagged as 0:OFF (only valid in protocol versions 19.2+).

Beside the wheel-tick factors, other parameters like direction pin polarity are initialized if

requested.

• Sensor error (e.g. missing data) detected: Sensor timeout of more than 500ms will trigger an

INS re-initialization (not supported in protocol versions less than 19.2).

Note that initialization phase requires good GNSS signal conditions as well as periods during

which vehicle is stationary and moving (including turns). Once all required initialization steps are

achieved, fusion mode is triggered and the calibration phase begins.

28.6.1.2 Fusion Mode

Once initialization phase is achieved, the receiver enters navigation mode. The receiver is in fusion

mode if the fusionMode field in the UBX-ESF-STATUS message is set on 1:FUSION. The fusion

filter then starts to compute combined GNSS/dead-reckoning fixes (fused solutions) and to

calibrate the sensors required for computing the fused navigation solution (used bit set). This is

the case when the sensor calibration status (calibStatus) is flagged as 1:CALIBRATING. As soon

as the calibration reaches a status where optimal fusion performance can be expected, the sensor

calibration status is flagged as 2/3:CALIBRATED.

28.6.1.3 Suspended Fusion Mode

Sensor fusion can be temporarily suspended in cases where no fused solution should/can be

computed. The receiver is in the temporarily disabled fusion mode if the fusionMode field in the

UBX-ESF-STATUS message is set on 2:SUSPENDED. In this case, the receiver computes a GNSS-

only solution.

Fusion is suspended if:

• One or several sensors deliver erroneous data or no data at all, the fusion is suspended during

the sensor failure period. The receiver automatically recovers once the affected sensor(s) is/are

back to normal operation (only supported in protocol versions less than 19.2).

• The vehicle is detected to be on a ferry where wheel-ticks do not detect any displacement (in

ADR products only).

28.6.1.4 Disabled Fusion Mode

Sensor fusion can be permanently switched off in cases where recurrent fusion failures happen or

user turned off manually fusion. The receiver is in the permanently disabled fusion mode if the

fusionMode field in the UBX-ESF-STATUS message is set on 3:DISABLED. In such a case, the

receiver computes a GNSS-only solution.

Fusion is permanently disabled in the following cases:

• If the fusion filter was manually turned off by the user (useAdr bit in the UBX-CFG-NAVX5

message is not set).

• If significantly wrong installation or filter parameters causing filter divergence are sent to the

receiver.

UBX-13003221 - R28 Early production information Page 110 of 460

## Page 126

u-blox 8 / u-blox M8 Receiver description - Manual

• If the fusion filter encountered too many errors.

An IMU-mount alignment error is output in the error field in the UBX-ESF-ALG message.

28.6.2 Accelerated Initialization and Calibration Procedure

This section describes how to perform fast initialization and calibration of the ADR receiver for the

purpose of evaluation.

The duration of the initialization phase mostly depends on the quality of the GNSS signals and the

dynamics encountered by the vehicle. Therefore the car should be driven to an open and flat area

like an empty open-sky parking area for example. The initialization and calibration drive should

contain phases where the car is stopped during a few minutes (with engine turned on), phases

where the car is doing normal left and right turns and phases where speed is above 30 km/h under

good GNSS reception conditions.

The initialization time required for reaching fused navigation mode can be shortened by following

the procedure in the order described in the table below.

Accelerated Initialization Procedure

Phase Procedure Indicator of Success

IMU initialization After receiver coldstart or first IMU initialization status (imuInitStat

receiver use, turn-on car engine and us) is flagged as 2:INITIALIZED in the

stay stationary under good GNSS UBX-ESF-STATUS message.

signal reception conditions during at

least 3 minutes.

This step can be skipped in DWT

navigation mode.

INS initialization Once IMU is initialized, stay GNSS 3D fix achieved, good 3D position

(position and stationary under good GNSS signal accuracy (at least 5 m), high number of

velocity) reception conditions until a reliable used SVs (check UBX-NAV-PVT

GNSS fix could be achieved. message).

IMU-mount Start driving with a minimum speed IMU-mount alignment status (mntAlgS

alignment of 30 km/h and do a series of tatus) is flagged as 2:INITIALIZED in

initialization approximately 10 left and right turns the UBX-ESF-STATUS message, the

(at least 90 degrees turns). Each IMU-mount alignment status (status)

turn should be completed as if the is flagged as 3:COARSE ALIGNED in the

vehicle would drive in a sharp UBX-ESF-ALG message.

roundabout.

This step can be skipped if

automatic IMU-mount alignment is

turned off.

Wheel-tick sensor Drive for at least 500 meters at a Wheel-tick sensor initialization status (

initialization minimum speed of 20 km/h. To wtInitStatus) is flagged as 2:

shorten this calibration step, the car INITIALIZED in the UBX-ESF-STATUS

should be driven at higher speed message.

(around 50 km/h) for at least 10

seconds under good GNSS visibility.

INS initialization Drive straight for at least 100 INS initialization status (insInitStatu

(attitude) meters at a minimum speed of 40 s) is flagged as 2:INITIALIZED in the

km/h. UBX-ESF-STATUS message.

UBX-13003221 - R28 Early production information Page 111 of 460

## Page 127

u-blox 8 / u-blox M8 Receiver description - Manual

Once initialization is completed, the fusionMode field in the UBX-ESF-STATUS message switches

to 1:FUSION, combined GNSS/dead-reckoning fixes (fused solutions) are output and the sensors

used in the navigation filter start to get calibrated. Calibration is a continuous process running in

the background and directly impacting the navigation solution quality.

The calibration time required for reaching optimal ADR navigation performance can be shortened

by following the procedure described in the table below.

Accelerated Calibration Procedure

Phase Procedure Indicator of Success

IMU-mount Keep driving with a minimum speed Once the IMU-mount alignment engine

alignment of 30 km/h and do a series of left has high confidence in its

calibration and right turns (at least 90 degrees misalignment angle estimates, the

with similar sharpness as when IMU-mount alignment status (status)

driving in a sharp roundabout). At is flagged as 4:FINE ALIGNED in the

each turn the estimated IMU-mount UBX-ESF-ALG message.

misalignment angles are refined and

their accuracy increased.

This step can be skipped if

automatic IMU-mount alignment is

turned-off.

IMU calibration Drive curves and straight segments The calibration status of the used

(gyroscope and during a few minutes by including a sensors (calibStatus) is flagged as

accelerometer) few stops lasting at least 30 2/3:CALIBRATED in the UBX-ESF-

seconds each. This drive should also STATUS message.

include some periods with higher

speed (at least 50 km/h) and can

typically be carried out on normal

open-sky roads with good GNSS

signal reception conditions.

Note that the calibration status (calibStatus in UBX-ESF-STATUS message) of some used

sensors might fall back to 1:CALIBRATING if the receiver is operated in challenging conditions. In

such a case, fused navigation solution uncertainty increases until optimal conditions are observed

again for re-calibrating the sensors.

The fused navigation performance quality might also depend on how well the gyroscope

temperature compensation table is populated. The table gradually fills in while the

vehicle is stationary and by observing gyroscope biases at different temperatures.

Therefore the quality of the gyroscope temperature compensation depends on how

many temperature bins could be observed while the vehicle was stationary and on the

duration of observation for each bin.

28.6.3 Automatic IMU-mount Alignment

(This feature is not supported in protocol versions less than 15.01).

28.6.3.1 Alignment Solution Output

The IMU-mount misalignment angles are output in the UBX-ESF-ALG message. They have the

following meaning:

• IMU-mount yaw angle: During IMU-mount yaw angle initialization (status field is equal to 2),

UBX-13003221 - R28 Early production information Page 112 of 460

## Page 128

u-blox 8 / u-blox M8 Receiver description - Manual

the published angle (yaw) corresponds to the current estimated value but is not yet applied for

rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount yaw angle configured by the user (see User-defined Configuration section)

and is applied for rotating the IMU observations.

• IMU-mount pitch angle: During IMU-mount pitch angle initialization (status field is equal to 1),

the published angle (pitch) corresponds to the current estimated value but is not yet applied

for rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount pitch angle configured by the user (see User-defined Configuration section)

and is applied for rotating the IMU observations.

• IMU-mount roll angle: During IMU-mount roll angle initialization (status field is equal to 1), the

published angle (roll) corresponds to the current estimated value but is not yet applied for

rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount roll angle configured by the user (see User-defined Configuration section) and

is applied for rotating the IMU observations.

If user-defined IMU-mount misalignment angles were configured by the user using UBX-

CFG-ESFALG (see User-defined Configuration section) and automatic IMU-mount

alignment is active, the angles output in the UBX-ESF-ALG message still correspond to

the definition given above: they represent the full rotation required for transforming IMU

data from installation-frame to IMU-frame. This means that the output misalignment

angles are computed from the composed rotation of the user-defined rotation and the

internally-estimated rotation.

28.6.3.2 Alignment Progress

The progress of the automatic IMU-mount alignment can be monitored by checking the status

field in the UBX-ESF-ALG message (see the UBX-ESF-ALG message description for the meaning of

the values output in the status field).

• IMU-mount roll/pitch angle initialization ongoing: The alignment engine is initializing the IMU-

mount roll and pitch angles (status is 1). Both angles can only be initialized if vehicle encounters

left and right turns (as occurring during a normal drive).

• IMU-mount yaw angle initialization ongoing: The alignment engine is initializing the IMU-mount

yaw angle (status is 2). IMU-mount yaw angle can only be initialized once IMU-mount roll and

pitch angles are initialized and if vehicle encounters left and right turns (as occurring during a

normal drive).

• IMU-mount misalignment angles are initialized (only supported in protocol versions 15.01 to 17):

The alignment engine has sufficient confidence in all IMU-mount misalignment angles and

validates their use for compensating the accelerometer and gyroscope data, i.e. fused

navigation solutions can be computed (status is 3).

• IMU-mount alignment coarse calibration ongoing (only supported in protocol versions 19+):

Once initialized (status is 3), the automatic IMU-mount alignment engine has sufficient

confidence in all IMU-mount misalignment angles and validates their use for compensating the

UBX-13003221 - R28 Early production information Page 113 of 460

## Page 129

u-blox 8 / u-blox M8 Receiver description - Manual

accelerometer and gyroscope data (fused navigation solutions can be computed). The engine

keeps filtering the IMU-mount misalignment angles every time the observed vehicle dynamics

allows for it.

• IMU-mount alignment fine calibration ongoing (only supported in protocol versions 19+): Once

the IMU-mount misalignment angles are estimated with a good accuracy, the automatic IMU-

mount alignment engine becomes more conservative in updating the IMU-mount misalignment

angles (status is 4).

28.6.3.3 Alignment Errors

The following errors might be output in the error bitfield of the UBX-ESF-ALG message:

• IMU-mount misalignment angle error (only supported in protocol versions 15.01 to 17): If the

automatic IMU-mount alignment engine suspects wrong IMU-mount misalignment angles

(either due to a wrong initialization or a change in the physical mounting of the device), the

error bit 0 in the UBX-ESF-ALG message is set.

• IMU-mount roll/pitch angle error (only supported in protocol versions 19+): If the automatic

IMU-mount alignment engine suspects wrong IMU-mount roll and/or IMU-mount pitch

misalignment angles (either due to a wrong initialization or a change in the physical mounting of

the device), the error bit 0 in the UBX-ESF-ALG message is set.

• IMU-mount yaw angle error (only supported in protocol versions 19+): If the automatic IMU-

mount alignment engine suspects wrong IMU-mount yaw misalignment angle (either due to a

wrong initialization or a change in the physical mounting of the device), the error bit 1 in the

UBX-ESF-ALG message is set.

• Euler Angle singularity ('gimbal-lock') error (only supported in protocol versions 19+): The Euler

angle singularity error bit 2 is set when the automatic IMU-mount alignment engine detects an

installation where the IMU-frame is misaligned in such a way that a degree of freedom is lost

when two IMU-mount misalignment (Euler) angles begin to describe the same rotations (or

axes). This happens for example with an IMU-mount misalignment of +/- 90 degrees around the

IMU-mount pitch axis, where IMU-mount roll and IMU-mount yaw cannot be distinguished from

each other. In such a case, these IMU-mount misalignment angles start to heavily fluctuate with

time due to the mathematical singularity occurring at these points, meaning that the IMU-

mount misalignment angles output in the UBX-ESF-ALG are not stable in time. Note however

that each individual set of IMU-mount misalignment angles output in such a case still describes

the correct rotation. Moreover, the internal rotation applied for aligning the IMU readings

doesn't suffer from this singularity issue and optimal fusion can still be achieved.

28.6.4 Navigation Output

28.6.4.1 Local-level North-East-Down (NED) Frame

The local-level frame is a geodetic frame with following features:

• The origin (O) is a point on the Earth surface;

• The x-axis points to North;

• the y-axis points to East;

• the z-axis completes the right-handed reference system by pointing down.

The frame is referred to as North-East-Down (NED) since its axes are aligned with the North, East

and Down directions.

UBX-13003221 - R28 Early production information Page 114 of 460

## Page 130

u-blox 8 / u-blox M8 Receiver description - Manual

28.6.4.2 Vehicle-Frame

The vehicle-frame is a right-handed 3D Cartesian frame rigidly connected with the vehicle and is

used to determine the attitude of the vehicle with respect to the local-level frame. It has the

following features:

• The origin (O) is the VRP in protocol versions less than 19.2, otherwise, is the origin of the IMU

instrumental frame;

• The x-axis points towards the front of the vehicle;

• the y-axis points towards the right of the vehicle;

• the z-axis completes the right-handed reference system by pointing down.

28.6.4.3 Vehicle Position and Velocity Output

The position and velocity information is output in several messages like UBX-NAV-PVT for

example. In protocol versions less than 19.2, position and velocity computed by the ADR

navigation filter are referenced to the VRP. For protocol versions 19.2+, position and velocity are

referenced to the origin of the IMU instrumental frame.

28.6.4.4 Vehicle Attitude Output

(Only supported in protocol versions 19+).

The transformation between the vehicle-frame and the local-level frame is described by three

attitude angles about the local-level axes denoted as vehicle roll, vehicle pitch and vehicle heading.

All three angles are referred as vehicle attitude and are illustrated in the figure below:

UBX-13003221 - R28 Early production information Page 115 of 460

## Page 131

u-blox 8 / u-blox M8 Receiver description - Manual

The order of the sequence of rotations around the navigation axes defining the vehicle attitude

matrix in terms of vehicle attitude angles is illustrated below:

UBX-13003221 - R28 Early production information Page 116 of 460

## Page 132

u-blox 8 / u-blox M8 Receiver description - Manual

Note that in this figure the body-frame corresponds to the vehicle-frame.

The vehicle attitude is output in the UBX-NAV-ATT message. The message provides all three

angles together with their accuracy estimates.

Roll angle estimation only supported in protocol versions 19.2+.

28.6.4.5 Vehicle Dynamics Output

(Only supported in protocol versions 19+).

The UBX-ESF-INS message outputs information about vehicle dynamics provided by the INS:

compensated vehicle angular rates and compensated vehicle accelerations. The acceleration data

is free of any gravitational acceleration. Its accuracy is directly dependent on the filter attitude

estimation accuracy.

Compensated vehicle dynamics information is output with respect to the vehicle-frame.

The message outputs only dynamics information that is directly compensated by the

fusion filter. This implies that depending on the solution type and the sensor availability,

dynamics along some axes of the vehicle-frame might not be available.

28.6.5 Sensor Data Types

The supported sensor data types are:

Definition of Data Types

Type Description Unit Format of the 24 data bits

0 none, data field contains no data

1..4 reserved

5 z-axis gyroscope angular rate deg/s *2^-12 signed

UBX-13003221 - R28 Early production information Page 117 of 460

## Page 133

u-blox 8 / u-blox M8 Receiver description - Manual

Definition of Data Types continued

Type Description Unit Format of the 24 data bits

6 front-left wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

7 front-right wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

8 rear-left wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

9 rear-right wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

10 single tick (speed tick) Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

11 speed m/s * 1e-3 signed

12 gyroscope temperature deg Celsius * 1e- signed

2

13 y-axis gyroscope angular rate deg/s *2^-12 signed

14 x-axis gyroscope angular rate deg/s *2^-12 signed

16 x-axis accelerometer specific force m/s^2 *2^-10 signed

17 y-axis accelerometer specific force m/s^2 *2^-10 signed

18 z-axis accelerometer specific force m/s^2 *2^-10 signed

28.6.6 Raw Sensor Data Output

(This feature is not supported in protocol versions less than 15.01).

Some u-blox module products contain inertial sensors (IMU) that are directly connected to the

GNSS and cannot be directly accessed from outside the module. The UBX-ESF-RAW message can

be used to access raw measurements of these sensors. A variable number of data fields may be

used in a single message and these can contain different types of measurements. The type of

each measurement is specified in the dataType field. The possible data types are x, y and z-axis

measurements on gyroscope or accelerometer and gyroscope temperature measurements as

described in the ESF Measurement Data section. One UBX-ESF-RAW message can contain multiple

samples from the same sensor. The user can separate and order these using the time tags

attached to each of the measurements.

UBX-13003221 - R28 Early production information Page 118 of 460

## Page 134

u-blox 8 / u-blox M8 Receiver description - Manual

The measurements are made at a fixed rate. The sampling rate or other sensor configuration

options can not be changed.

To turn on this feature the UBX-ESF-RAW message must be enabled using UBX-CFG-MSG. If non-

zero rate is selected the message will be output but the selected rate does not otherwise have an

influence at the rate of the messages.

Turning on this feature does not disable sensor fusion in the receiver. To use an external

fusion algorithm consider disabling the automotive dead reckoning mode using UBX-

CFG-NAVX5.

28.6.7 Receiver Startup and Shutdown

Continuous dead reckoning is possible over receiver restarts if the following conditions are true:

• Non-volatile storage is available, or the save-on-shutdown feature (SOS) is used

• The vehicle is not moved while the receiver is off

During periods of external sensor data unavailability the receiver switches to GNSS-only

navigation if the last sensor information indicated the vehicle was moving.

29 Untethered Dead Reckoning (UDR)

This feature is only available with the UDR products.

29.1 Introduction

u-blox solution for Untethered Dead Reckoning (UDR) allows improved navigation performance in

places with GNSS-denied conditions as well as during short GNSS outages. UDR is based on

Sensor Fusion Dead Reckoning (SFDR) technology, which integrates an Inertial Navigation

System (INS) with GNSS measurements. The INS integrates angular rates and specific forces

sensed by an Inertial Measurement Unit (IMU). The INS computes position, velocity and attitude

changes and can, once initialized, provide accurate navigation information. However, an inertial-

only navigation solution would degrade quickly with time due to the errors corrupting the IMU

observations. The integration of the INS with GNSS measurements bounds these time-growing

errors by calibrating the INS. The resulting integrated INS/GNSS filter, called fusion filter below,

has the following advantages compared to standalone GNSS positioning:

• Improved navigation performance in GNSS-denied conditions: errors caused by multipath or

weak signal conditions are mitigated though the aid brought by the IMU.

• Navigation solution during short GNSS-outages: the INS bridges short GNSS gaps which might

be caused by tunnels or parking garages.

UDR solution uses the messages of the External Sensor Fusion (ESF) class.

29.2 Installation Configuration

(The features in this section are not supported in protocol versions less than 19).

29.2.1 IMU-mount Alignment

(This feature is not supported in protocol versions less than 15.01).

The default assumption is that the IMU-frame and the installation-frame have the same

orientation (i.e. all axes are parallel). If this assumption is not valid, the positioning solution can be

degraded if the IMU-mount misalignment angles are small (typically few degrees) or can even fail

UBX-13003221 - R28 Early production information Page 119 of 460

## Page 135

u-blox 8 / u-blox M8 Receiver description - Manual

in case of large (tens of degrees) IMU-mount misalignments. Therefore, it is important to correctly

configure the IMU-mount misalignment settings by using the UBX-CFG-ESFALG configuration

message.

This section describes how IMU-mount misalignment angles, i.e. the angles which rotate the

installation-frame to the IMU-frame, can be configured using the UBX-CFG-ESFALG configuration

message (see User-defined Configuration section below).

If the IMU-mount misalignment angles are unknown, they can be estimated during a dedicated

initialization drive through an automatic alignment procedure. This is described in the Automatic

IMU-Mount Alignment section below.

In u-blox module products containing an internal IMU (e.g. NEO-M8U modules), the IMU-

mount misalignment angles are estimated automatically by default (see Automatic

IMU-Mount Alignment section below for further details).

29.2.1.1 Definitions

The IMU-mount misalignment angles are defined as follows:

• The transformation from the installation-frame to the IMU-frame is described by three Euler

angles about the installation-frame axes denoted as IMU-mount roll, IMU-mount pitch and IMU-

mount yaw angles. All three angles are referred as the IMU-mount misalignment angles.

There is a single IMU-mount misalignment configuration that applies to both

gyroscopes and accelerometers, so these sensors must be aligned with each other if

both types are present.

29.2.1.2 User-defined IMU-mount Alignment

The user can configure manually some IMU-mount roll, pitch and yaw angles using the UBX-CFG-

ESFALG configuration message. The values that should be set in the configuration message are

the Euler angles required to rotate the installation-frame to the IMU-frame. The IMU-mount yaw

rotation should be performed first, then the IMU-mount pitch and finally the IMU-mount roll. At

each stage, the rotation is around the appropriate axis of the transformed installation-frame,

meaning that the order of the rotation sequence is important (see figure below).

UBX-13003221 - R28 Early production information Page 120 of 460

## Page 136

u-blox 8 / u-blox M8 Receiver description - Manual

If there is only a single IMU-mount misalignment angle then it may be measured as shown in the

three examples below.

UBX-13003221 - R28 Early production information Page 121 of 460

## Page 137

u-blox 8 / u-blox M8 Receiver description - Manual

In order to prevent significant degradation of the positioning solution the IMU-mount

misalignment angles should be configured with an accuracy of at least 5 degrees.

The list below describes in details how the fields in the UBX-CFG-ESFALG message must be

interpreted with respect to example illustrated in the figure above:

• User-defined IMU-mount yaw angle: The IMU-mount yaw angle (yaw) corresponds to the

rotation around the installation-frame z-axis (vertical) required for aligning the installation-

frame to the IMU-frame (yaw = 344.0 deg if the IMU-mount misalignment is composed of a

single rotation around the installation-frame z-axis, i.e. with no IMU-mount roll and IMU-mount

pitch rotation).

• User-defined IMU-mount pitch angle: The IMU-mount pitch angle (pitch) corresponds to the

rotation around the installation-frame y-axis required for aligning the installation-frame to the

IMU-frame (pitch = 26.5 deg if the IMU-mount alignment is composed of a single rotation

around the installation-frame y-axis, i.e. with no IMU-mount roll and IMU-mount yaw rotation).

• User-defined IMU-mount roll angle: The IMU-mount roll angle (roll) corresponds to the

rotation around the installation-frame x-axis required for aligning the installation-frame to the

IMU-frame (roll = -23.5 deg if the IMU-mount misalignment is composed of a single rotation

around installation-frame x-axis, i.e. with no IMU-mount pitch and IMU-mount yaw rotation).

UBX-13003221 - R28 Early production information Page 122 of 460

## Page 138

u-blox 8 / u-blox M8 Receiver description - Manual

If automatic alignment is turned-on (see Automatic IMU-mount Alignment section), the

angles obtained by polling UBX-CFG-ESFALG are still the user-defined angles which do

not correspond to the result of the automatic IMU-mount alignment engine as output in

UBX-ESF-ALG (see IMU-mount Misalignment Angles Output section for more details).

29.2.1.3 Automatic IMU-mount Alignment

The automatic IMU-mount alignment engine estimates automatically the IMU-mount roll, pitch

and yaw angles. It requires an initialization phase during which no INS/GNSS fusion can be

achieved (see Filter Modes section for further details). The progress of the automatic alignment

initialization can be monitored with the UBX-ESF-STATUS message, and/or with the UBX-ESF-ALG

message providing more details. When the vehicle is subject to sufficient dynamics (i.e. left and

right turns during a normal drive), the automatic IMU-mount alignment engine will estimate the

IMU-mount misalignment angles which have the same meaning as defined in the Definitions

section, regardless whether the user did or not enter manually some IMU-mount misalignment

angles (see User-defined Configuration section). Once the automatic IMU-mount alignment

engine has sufficient confidence in the estimated initial IMU-mount misalignment angles, the

IMU-mount misalignment angles initialization phase is completed. The raw accelerometer and

gyroscope data (i.e. the IMU observations) are then compensated for IMU-mount misalignment

and sensor fusion can be done. The resulting IMU-mount misalignment angles are output in the

UBX-ESF-ALG message.

For automatic IMU-mount alignemnt a 3-axis gyroscope and 3-axis accelerometer is

required (only valid in protocol versions 19.2+).

29.2.1.3.1 Enabling/Disabling Automatic IMU-mount Alignment

The user can activate/deactivate the automatic IMU-mount alignment by setting the

doAutoMntAlg bit in the UBX-CFG-ESFALG configuration message.

If automatic IMU-mount alignment is deactivated while aligning, the estimated

misalignment angles that were available at deactivation time are used (only if they were

initialized, see next section). If automatic IMU-mount alignment is re-activated,

alignment is pursued by starting from the state where deactivation happened (only valid

in protocol versions 19+).

29.2.1.4 Limitation with Single-Axis Gyroscope

Gyroscope-mount misalignment is only supported when a three-axis gyroscope is available. In

case of a single-axis gyroscope, the sensor should be physically aligned along the installation-

frame z-axis. This is needed to avoid a scale factor error which will affect the accuracy of the

output due to the two missing gyroscopes.

29.3 Sensor Configuration

This section describes the external sensor configuration parameters.

29.3.1 Accelerometer Configuration

The accelerometer sensor senses specific forces, expressed in meters per seconds squared, along

its input axis. In the full configuration, an IMU contains a three-axis accelerometer whose sensitive

axes are assumed to be mutually orthogonal in a Cartesian frame.

UBX-13003221 - R28 Early production information Page 123 of 460

## Page 139

u-blox 8 / u-blox M8 Receiver description - Manual

29.3.1.1 Messages

The accelerometer sensor can be configured in the following message:

Configuration Messages for UDR Products

Product Type Message

Chipset UBX-CFG-ESFA

29.3.2 Gyroscope Configuration

The gyroscope sensor senses angular rates, expressed in radians per seconds or degrees per

second, along its input axis. In the full configuration, an IMU contains a three-axis gyroscope

whose sensitive axes are assumed to be mutually orthogonal in a Cartesian frame.

29.3.2.1 Messages

The gyroscope sensor can be configured in the following message:

Configuration Messages for UDR Products

Product Type Message

Chipset UBX-CFG-ESFG

29.3.2.2 Temperature Compensation

Gyroscope sensors generally exhibit a temperature-dependent bias that varies from unit to unit.

To help compensate for this variation the receiver builds up a table of gyroscope bias versus

temperature measurements which are often available from the gyroscope sensor itself. This is

particularly valuable to dead-reckoning-only navigation after the vehicle has been left for some

time in parking garage.

The gyroscope temperature compensation engine has the following settings:

• Gyroscope RMS threshold above which temperature table is not updated: The gyroscope

temperature-dependent bias is only updated if the measured gyroscope angular rate RMS is

below the given threshold. This avoids artificially high estimates of the gyroscope temperature-

dependent bias from transient events such as vehicle engine starts or nearby heavy

construction. This threshold can be configured in the gyroRmsThdl field and is shared with the

sensor accuracy estimation engine (see above);

• Temperature-dependent bias table saving rate: Gyroscope temperature compensation data

are saved to non-volatile storage at intervals that can be configured by the tcTableSaveRate

field.

The gyroscope temperature-dependent bias table is revised under the following conditions:

• The vehicle is stationary (without wheel-tick measurements or at zero speed);

• The RMS of the measured gyroscope angular rates and accelerometer specific forces is below a

given threshold (see above);

• Turntable mode is not engaged (only for ADR products, see Ferry and Turntable Modes section);

Gyroscope temperature compensation is effective if the gyroscope(s) exhibits

repeatable characteristics with temperature and is not unduly affected by external

factors (such as supply voltage or mechanical stress).

UBX-13003221 - R28 Early production information Page 124 of 460

## Page 140

u-blox 8 / u-blox M8 Receiver description - Manual

29.3.3 Sensor Time Tagging

In order to achieve optimal performance with the fusion solution it is essential to determine the

epoch in the receiver time frame when the external sensor measurements were generated. This

may be done in one of the following ways:

• First Byte Reception: reception time of first byte of UBX-ESF-MEAS message

• Time Mark on External Input: reception time of time mark signal sent to external input

The latency of the sensor data is the time between when the sensor measurement was taken and

the detection at the receiver of either the first byte of the UBX-ESF-MEAS message or the pre-

processor's time mark, depending on the timing approach chosen. Increased latency reduces the

navigation performance.

In ADR, the latency can be set by using the latency, wtLatency, gyroLatency and accelLatency

parameters in the appropriate configuration message, as discussed in the Automotive Dead

Reckoning (ADR) chapter.

In UDR, the latency can be set by using the latency parameter in the appropriate sensor

configuration message, as discussed in the Untethered Dead Reckoning (UDR) chapter.

29.3.3.1 First Byte Reception

The easiest way to determine the sensor measurement generation time is to have the GNSS

receiver assume the time of reception of the first byte of the UBX-ESF-MEAS message (minus a

constant configured latency) to be the time of sensor measurement. This approach is the

simplest to implement, but Time Mark on External Input can yield better latency control and

compensation.

29.3.3.2 Time Mark on External Input

In this case, the preprocessor unit generating the measurements sends a signal to the EXTINT

input of the GNSS receiver, marking the moment of measurement generation. The subsequent

UBX-ESF-MEAS message is then flagged accordingly, and the measurements in the message will

be assumed to have been generated at the time of external signal reception (minus a constant

configured latency). This approach is the preferred solution, but it can be difficult to realize an

exact analog time signal for the preprocessor unit.

UBX-13003221 - R28 Early production information Page 125 of 460

## Page 141

u-blox 8 / u-blox M8 Receiver description - Manual

29.3.3.3 Sensor Time Tagging Configuration

The receiver requires external sensor packets time tagged in seconds.

The external sensor time tagging for WT can be configured in the UBX-CFG-ESFWT (not supported

in protocol versions less than 15.01).

The following sensor time tagging settings need to be specified:

• Sensor time tag scale factor to seconds: (timeTagFactor): This parameters converts the

sensor time tags from their original time unit into the required seconds. For example if the IMU

raw packets are time-tagged in milliseconds, the scale factor for converting one millisecond into

one seconds is 0.001.

• Sensor time tag maximum value: (timeTagMax): External sensor time tags are encoded in

different data types (signed/unsigned, varying number of bytes) which might vary across sensor

types. For example if the IMU raw packet's time-tag field is encoded into an unsigned long

integer (4 bytes), the maximum possible time-tag value is 4294967295 (0xFFFFFFFFin

hexadecimal).

29.4 UDR System Configuration

(These features are not supported in protocol versions less than 19).

29.4.1 Enabling/Disabling Fusion Filter

The UDR fusion filter can be turned off by means of the useAdr bit in the UBX-CFG-NAVX5

configuration message. If fusion is turned off, the receiver outputs a GNSS-only solution.

29.4.2 Recommended Configuration

For an optimum navigation performance, the recommended general configuration is the following:

• Navigation Rate: the standard navigation solution update rate of 1 Hz (see UBX-CFG-RATE

message) is recommended.

Reconsider the enabled messages and features (e.g logging) at higher navigation rates

to meet CPU load, memory and interface bandwidth constraints (Valid in protocol

versions 19.2).

29.5 Operation

This section describes how the UDR receiver operates.

UBX-13003221 - R28 Early production information Page 126 of 460

## Page 142

u-blox 8 / u-blox M8 Receiver description - Manual

29.5.1 Fusion Filter Modes

The fusion filter operates in different modes which are output in the UBX-ESF-STATUS message.

The table below summarizes the different fusion filter modes with the associated tasks the

receiver is doing.

Fusion Modes

Mode Performed Tasks / Possible Causes Published Fix

Type

Initialization Initialization of IMU 3D-Fix

Initialization of IMU-mount alignment (GNSS)

Initialization of INS (position, velocity, attitude)

Initialization of wheel-tick sensor (ADR only)

IMU sensor error (e.g. missing data) detected (only supported

in protocol versions 19.2+)

Fusion Fine-calibration of IMU-mount misalignment angles (not GNSS/DR Fix

supported in protocol versions less than 19)

Fine-calibration of IMU sensors

Fine-calibrating of wheel-tick factors (ADR only)

UDR mode under ADR / WT sensor error (e.g. missing data)

detected (ADR only)(only supported in protocol versions 19.2+)

Suspended Fusion Sensor error (e.g. missing data) detected (only supported in 3D-Fix

protocol versions less than 19.2) (GNSS)

Ferry detected (ADR only)

Disabled Fusion Fatal fusion filter error occurred 3D-Fix

Fusion filter turned-off by user (GNSS)

More details about each fusion mode are given in the following sections.

29.5.1.1 Initialization Mode

The purpose of the initialization phase is to estimate all unknown parameters which are required

for achieving fusion. The initialization phase is triggered after a receiver cold start or a filter reset

in case of fusion failure. The receiver is in initialization mode if the fusionMode field in the UBX-

ESF-STATUS message is 0:INITIALIZING. In this case the required sensor calibration status (cal

ibStatus) is flagged as 0: NOT CALIBRATED and the navigation solution output during

initialization is based on GNSS solely.

The initialization phase comprises the following internal steps whose status is published in the

initStatus field of the UBX-ESF-STATUS message:

• IMU initialization: Unknown crucial IMU parameters such as sensor sampling frequency are

estimated during initialization. As long as all required IMU parameters are not initialized, the

status of the IMU initialization (imuInitStatus) is flagged as 1:INITIALIZING in the UBX-ESF-

STATUS message. Moreover, the required sensor calibration statuses (calibStatus) are flagged

as 0:NOT CALIBRATED in the UBX-ESF-STATUS message. Note that if the user configured all

required sensor settings, this step is skipped and IMU initialization is flagged as 2:

INITIALIZED (not supported in protocol versions less than 19).

• IMU-mount alignment initialization: If automatic IMU-mount alignment is enabled (see the

Automatic IMU-mount Alignment Configuration section), initial IMU-mount roll, IMU-mount

pitch and IMU-mount yaw angles need to be estimated. For that, good GNSS signal reception as

UBX-13003221 - R28 Early production information Page 127 of 460

## Page 143

u-blox 8 / u-blox M8 Receiver description - Manual

well as sufficient vehicle dynamics (i.e. a series of left and right turns during a normal drive)

need to be at hand. As long as the IMU-mount alignment is not initialized, the status of the IMU-

mount alignment (mntAlgStatus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS

message. Once initialized, the IMU-mount alignment status is flagged as 2:INITIALIZED. If no

IMU-mount alignment is required, the IMU-mount alignment is flagged as 0:OFF. A detailed

description of the automatic IMU-mount alignment operation can be found in the Automatic

IMU-mount Alignment Operation section (not supported in protocol versions less than 15.01).

• INS initialization: Before entering fusion mode, the initial vehicle position, velocity and especially

attitude (vehicle roll, pitch heading angles) needs to be known with sufficient accuracy. This is

achieved during INS initialization phase (which comprises an INS coarse alignment step) using

GNSS. As long as the fusion filter isn't initialized, the status of the INS initialization (insInitSt

atus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the INS

initialization is flagged as 2:INITIALIZED (not supported in protocol versions less than 15.01).

This section is valid only for protocol versions less than 19.2

• Wheel-tick sensor initialization (ADR products only): Before entering fusion mode, some

parameters like initial wheel-tick factors need to be estimated with sufficient accuracy. This is

achieved during wheel-tick sensor initialization phase using GNSS. As long as the wheel-tick

parameters are not initialized, the status of the wheel-tick initialization (wtInitStatus) is

flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the wheel-tick

sensor initialization is flagged as 2:INITIALIZED and the parameters are stored in non-volatile

storage. If no wheel-tick data are required (in UDR products), the wheel-tick initialization is

flagged as 0:OFF (only valid in protocol versions less than 19.2).

This section is valid only for protocol versions 19.2+

• Wheel-tick sensor initialization (ADR products only): Solution enters fusion mode (fusionMode

field in the UBX-ESF-STATUS message is on 1:FUSION), even when wheel-tick is not yet

initialized, following a UDR mode approach. WT sensor parameters, like initial wheel-tick

factors, are estimated in parallel and are used once estimated with sufficient accuracy. As long

as the wheel-tick parameters are not initialized, the status of the wheel-tick initialization (wtIni

tStatus) is flagged as 1:INITIALIZING in the UBX-ESF-STATUS message. Once initialized, the

wheel-tick sensor initialization is flagged as 2:INITIALIZED, WT data are used by the filter and

the parameters are stored in non-volatile storage. If no wheel-tick data are required (in UDR

products), the wheel-tick initialization is flagged as 0:OFF (only valid in protocol versions 19.2+).

Beside the wheel-tick factors, other parameters like direction pin polarity are initialized if

requested.

• Sensor error (e.g. missing data) detected: Sensor timeout of more than 500ms will trigger an

INS re-initialization (not supported in protocol versions less than 19.2).

Note that initialization phase requires good GNSS signal conditions as well as periods during

which vehicle is stationary and moving (including turns). Once all required initialization steps are

achieved, fusion mode is triggered and the calibration phase begins.

29.5.1.2 Fusion Mode

Once initialization phase is achieved, the receiver enters navigation mode. The receiver is in fusion

mode if the fusionMode field in the UBX-ESF-STATUS message is set on 1:FUSION. The fusion

filter then starts to compute combined GNSS/dead-reckoning fixes (fused solutions) and to

calibrate the sensors required for computing the fused navigation solution (used bit set). This is

UBX-13003221 - R28 Early production information Page 128 of 460

## Page 144

u-blox 8 / u-blox M8 Receiver description - Manual

the case when the sensor calibration status (calibStatus) is flagged as 1:CALIBRATING. As soon

as the calibration reaches a status where optimal fusion performance can be expected, the sensor

calibration status is flagged as 2/3:CALIBRATED.

29.5.1.3 Suspended Fusion Mode

Sensor fusion can be temporarily suspended in cases where no fused solution should/can be

computed. The receiver is in the temporarily disabled fusion mode if the fusionMode field in the

UBX-ESF-STATUS message is set on 2:SUSPENDED. In this case, the receiver computes a GNSS-

only solution.

Fusion is suspended if:

• One or several sensors deliver erroneous data or no data at all, the fusion is suspended during

the sensor failure period. The receiver automatically recovers once the affected sensor(s) is/are

back to normal operation (only supported in protocol versions less than 19.2).

• The vehicle is detected to be on a ferry where wheel-ticks do not detect any displacement (in

ADR products only).

29.5.1.4 Disabled Fusion Mode

Sensor fusion can be permanently switched off in cases where recurrent fusion failures happen or

user turned off manually fusion. The receiver is in the permanently disabled fusion mode if the

fusionMode field in the UBX-ESF-STATUS message is set on 3:DISABLED. In such a case, the

receiver computes a GNSS-only solution.

Fusion is permanently disabled in the following cases:

• If the fusion filter was manually turned off by the user (useAdr bit in the UBX-CFG-NAVX5

message is not set).

• If significantly wrong installation or filter parameters causing filter divergence are sent to the

receiver.

• If the fusion filter encountered too many errors.

An IMU-mount alignment error is output in the error field in the UBX-ESF-ALG message.

29.5.2 Accelerated Initialization and Calibration Procedure

This section describes how to perform fast initialization and calibration of the UDR receiver for the

purpose of evaluation.

The duration of the initialization phase mostly depends on the quality of the GNSS signals and the

dynamics encountered by the vehicle. Therefore the car should be driven to an open and flat area

like an empty open-sky parking area for example. The initialization and calibration drive should

contain phases where the car is stopped during a few minutes (with engine turned-on), phases

where the car is doing normal left and right turns and phases where speed is above 30 km/h under

good GNSS reception conditions.

The initialization time required for reaching fused navigation mode can be shortened by following

the procedure in the order described in the table below.

Accelerated Initialization Procedure

Phase Procedure Indicator of Success

UBX-13003221 - R28 Early production information Page 129 of 460

## Page 145

u-blox 8 / u-blox M8 Receiver description - Manual

Accelerated Initialization Procedure continued

Phase Procedure Indicator of Success

IMU initialization After receiver coldstart or first IMU initialization status (imuInitStat

receiver use, turn-on car engine and us) is flagged as 2:INITIALIZED in the

stay stationary under good GNSS UBX-ESF-STATUS message.

signal reception conditions during at

least 3 minutes.

INS initialization Once IMU is initialized, stay GNSS 3D fix achieved, good 3D position

(position and stationary under good GNSS signal accuracy (at least 5 m), high number of

velocity) reception conditions until a reliable used SVs (check UBX-NAV-PVT

GNSS fix could be achieved. message).

IMU-mount Start driving with a minimum speed IMU-mount alignment status (mntAlgS

alignment of 12 km/h and do a series of tatus) is flagged as 2:INITIALIZED in

initialization approximately 10 left and right turns the UBX-ESF-STATUS message, the

(at least 90 degrees turns). Each IMU-mount alignment status (status)

turn should be completed as if the is flagged as 3:COARSE ALIGNED in the

vehicle would drive in a sharp UBX-ESF-ALG message.

roundabout.

This step can be skipped if

automatic IMU-mount alignment is

turned-off.

INS initialization Drive straight for at least 100 INS initialization status (insInitStatu

(attitude) meters at a minimum speed of 40 s) is flagged as 2:INITIALIZED in the

km/h. UBX-ESF-STATUS message.

Once initialization is completed, the fusionMode field in the UBX-ESF-STATUS message switches

to 1:FUSION, combined GNSS/Dead-reckoning fixes (fused solutions) are output and the sensors

used in the navigation filter start to get calibrated. Calibration is a continuous process running in

the background and improving the navigation solution quality.

The calibration time required for reaching optimal UDR navigation performance can be shortened

by following the procedure described in the table below.

Accelerated Calibration Procedure

Phase Procedure Indicator of Success

IMU-mount Keep driving with a minimum speed Once the IMU-mount alignment engine

alignment of 30 km/h and do a series of left has high confidence in its

calibration and right turns (at least 90 degrees misalignment angle estimates, the

with similar sharpness as when IMU-mount alignment status (status)

driving in a sharp roundabout). At is flagged as 4:FINE ALIGNED in the

each turn the estimated IMU-mount UBX-ESF-ALG message.

misalignment angles are refined and

their accuracy increased.

This step can be skipped if

automatic IMU-mount alignment is

turned-off.

UBX-13003221 - R28 Early production information Page 130 of 460

## Page 146

u-blox 8 / u-blox M8 Receiver description - Manual

Accelerated Calibration Procedure continued

Phase Procedure Indicator of Success

IMU calibration Drive curves and straight segments The calibration status of the used

(gyroscope and during a few minutes by including a sensors (calibStatus) is flagged as

accelerometer) few stops lasting at least 30 2/3:CALIBRATED in the UBX-ESF-

seconds each. This drive should also STATUS message.

include some periods with higher

speed (at least 50 km/h) and can

typically be carried out on normal

open-sky roads with good GNSS

signal reception conditions.

Note that the calibration status (calibStatus in UBX-ESF-STATUS message) of some used

sensors might fall back to 1:CALIBRATING if the receiver is operated in challenging conditions. In

such a case, fused navigation solution uncertainty increases until optimal conditions are observed

again for re-calibrating the sensors.

The fused navigation performance quality might also depend on how well the gyroscope

temperature compensation table is populated. The table gradually fills in while the

vehicle is stationary and by observing gyroscope biases at different temperatures.

Therefore the quality of the gyroscope temperature compensation depends on how

many temperature bins could be observed while the vehicle was stationary and on the

duration of observation for each bin.

29.5.3 Automatic IMU-mount Alignment

(This feature is not supported in protocol versions less than 15.01).

29.5.3.1 Alignment Solution Output

The IMU-mount misalignment angles are output in the UBX-ESF-ALG message. They have the

following meaning:

• IMU-mount yaw angle: During IMU-mount yaw angle initialization (status field is equal to 2),

the published angle (yaw) corresponds to the current estimated value but is not yet applied for

rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount yaw angle configured by the user (see User-defined Configuration section)

and is applied for rotating the IMU observations.

• IMU-mount pitch angle: During IMU-mount pitch angle initialization (status field is equal to 1),

the published angle (pitch) corresponds to the current estimated value but is not yet applied

for rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount pitch angle configured by the user (see User-defined Configuration section)

and is applied for rotating the IMU observations.

• IMU-mount roll angle: During IMU-mount roll angle initialization (status field is equal to 1), the

published angle (roll) corresponds to the current estimated value but is not yet applied for

rotating the IMU observations. After initialization (status field is equal or higher than 3), the

published angle corresponds to the estimated value and is applied for rotating the IMU

UBX-13003221 - R28 Early production information Page 131 of 460

## Page 147

u-blox 8 / u-blox M8 Receiver description - Manual

observations. If automatic IMU-mount alignment is disabled, the published angle corresponds

to the IMU-mount roll angle configured by the user (see User-defined Configuration section) and

is applied for rotating the IMU observations.

If user-defined IMU-mount misalignment angles were configured by the user using UBX-

CFG-ESFALG (see User-defined Configuration section) and automatic IMU-mount

alignment is active, the angles output in the UBX-ESF-ALG message still correspond to

the definition given above: they represent the full rotation required for transforming IMU

data from installation-frame to IMU-frame. This means that the output misalignment

angles are computed from the composed rotation of the user-defined rotation and the

internally-estimated rotation.

29.5.3.2 Alignment Progress

The progress of the automatic IMU-mount alignment can be monitored by checking the status

field in the UBX-ESF-ALG message (see the UBX-ESF-ALG message description for the meaning of

the values output in the status field).

• IMU-mount roll/pitch angle initialization ongoing: The alignment engine is initializing the IMU-

mount roll and pitch angles (status is 1). Both angles can only be initialized if vehicle encounters

left and right turns (as occurring during a normal drive).

• IMU-mount yaw angle initialization ongoing: The alignment engine is initializing the IMU-mount

yaw angle (status is 2). IMU-mount yaw angle can only be initialized once IMU-mount roll and

pitch angles are initialized and if vehicle encounters left and right turns (as occurring during a

normal drive).

• IMU-mount misalignment angles are initialized (only supported in protocol versions 15.01 to 17):

The alignment engine has sufficient confidence in all IMU-mount misalignment angles and

validates their use for compensating the accelerometer and gyroscope data, i.e. fused

navigation solutions can be computed (status is 3).

• IMU-mount alignment coarse calibration ongoing (only supported in protocol versions 19+):

Once initialized (status is 3), the automatic IMU-mount alignment engine has sufficient

confidence in all IMU-mount misalignment angles and validates their use for compensating the

accelerometer and gyroscope data (fused navigation solutions can be computed). The engine

keeps filtering the IMU-mount misalignment angles every time the observed vehicle dynamics

allows for it.

• IMU-mount alignment fine calibration ongoing (only supported in protocol versions 19+): Once

the IMU-mount misalignment angles are estimated with a good accuracy, the automatic IMU-

mount alignment engine becomes more conservative in updating the IMU-mount misalignment

angles (status is 4).

29.5.3.3 Alignment Errors

The following errors might be output in the error bitfield of the UBX-ESF-ALG message:

• IMU-mount misalignment angle error (only supported in protocol versions 15.01 to 17): If the

automatic IMU-mount alignment engine suspects wrong IMU-mount misalignment angles

(either due to a wrong initialization or a change in the physical mounting of the device), the

error bit 0 in the UBX-ESF-ALG message is set.

• IMU-mount roll/pitch angle error (only supported in protocol versions 19+): If the automatic

IMU-mount alignment engine suspects wrong IMU-mount roll and/or IMU-mount pitch

misalignment angles (either due to a wrong initialization or a change in the physical mounting of

UBX-13003221 - R28 Early production information Page 132 of 460

## Page 148

u-blox 8 / u-blox M8 Receiver description - Manual

the device), the error bit 0 in the UBX-ESF-ALG message is set.

• IMU-mount yaw angle error (only supported in protocol versions 19+): If the automatic IMU-

mount alignment engine suspects wrong IMU-mount yaw misalignment angle (either due to a

wrong initialization or a change in the physical mounting of the device), the error bit 1 in the

UBX-ESF-ALG message is set.

• Euler Angle singularity ('gimbal-lock') error (only supported in protocol versions 19+): The Euler

angle singularity error bit 2 is set when the automatic IMU-mount alignment engine detects an

installation where the IMU-frame is misaligned in such a way that a degree of freedom is lost

when two IMU-mount misalignment (Euler) angles begin to describe the same rotations (or

axes). This happens for example with an IMU-mount misalignment of +/- 90 degrees around the

IMU-mount pitch axis, where IMU-mount roll and IMU-mount yaw cannot be distinguished from

each other. In such a case, these IMU-mount misalignment angles start to heavily fluctuate with

time due to the mathematical singularity occurring at these points, meaning that the IMU-

mount misalignment angles output in the UBX-ESF-ALG are not stable in time. Note however

that each individual set of IMU-mount misalignment angles output in such a case still describes

the correct rotation. Moreover, the internal rotation applied for aligning the IMU readings

doesn't suffer from this singularity issue and optimal fusion can still be achieved.

29.5.4 Navigation Output

(Only supported in protocol versions 19+).

29.5.4.1 Local-level North-East-Down (NED) Frame

The local-level frame is a geodetic frame with following features:

• The origin (O) is a point on the Earth surface;

• The x-axis points to North;

• the y-axis points to East;

• the z-axis completes the right-handed reference system by pointing down.

The frame is referred to as North-East-Down (NED) since its axes are aligned with the North, East

and Down directions.

29.5.4.2 Body-Frame

The body-frame is a right-handed 3D Cartesian frame rigidly connected with the vehicle and is

used to determine the attitude of the vehicle with respect to the local-level frame. It has the

following features:

• The origin (O) is the origin of the IMU instrumental frame;

• The x-axis points towards the front of the vehicle;

• the y-axis points towards the right of the vehicle;

• the z-axis completes the right-handed reference system by pointing down.

29.5.4.3 Vehicle Position and Velocity Output

The position and velocity information is output in several messages like UBX-NAV-PVT for

example. The position computed by the UDR navigation filter is referenced to the origin (O) of the

body-frame.

UBX-13003221 - R28 Early production information Page 133 of 460

## Page 149

u-blox 8 / u-blox M8 Receiver description - Manual

29.5.4.4 Vehicle Attitude Output

The transformation between the body-frame and the local-level frame is described by three

attitude angles about the local-level axes denoted as vehicle roll, vehicle pitch and vehicle heading.

All three angles are referred as vehicle attitude and are illustrated in the figure below:

The order of the sequence of rotations around the navigation axes defining the vehicle attitude

matrix in terms of vehicle attitude angles is illustrated below:

UBX-13003221 - R28 Early production information Page 134 of 460

## Page 150

u-blox 8 / u-blox M8 Receiver description - Manual

The vehicle attitude is output in the UBX-NAV-ATT message. The message provides all three

angles together with their accuracy estimates. Note that since no backwards motion information

is measured, no heading of motion information is output in the UBX-NAV-PVT message (heading of

vehicle is provided in a separate field within the same message).

29.5.4.5 Vehicle Dynamics Output

The UBX-ESF-INS message outputs information about vehicle dynamics provided by the INS:

compensated vehicle angular rates and compensated vehicle accelerations. The acceleration data

is free of any gravitational acceleration. It's accuracy is directly dependent on the filter attitude

estimation accuracy.

Compensated vehicle dynamics information is output with respect to the body-frame.

29.5.5 Sensor Data Types

The supported sensor data types are:

Definition of Data Types

Type Description Unit Format of the 24 data bits

0 none, data field contains no data

1..4 reserved

5 z-axis gyroscope angular rate deg/s *2^-12 signed

6 front-left wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

UBX-13003221 - R28 Early production information Page 135 of 460

## Page 151

u-blox 8 / u-blox M8 Receiver description - Manual

Definition of Data Types continued

Type Description Unit Format of the 24 data bits

7 front-right wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

8 rear-left wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

9 rear-right wheel ticks Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

10 single tick (speed tick) Bits 0-22: unsigned

tick value. Bit 23:

direction indicator

(0=forward,

1=backward)

11 speed m/s * 1e-3 signed

12 gyroscope temperature deg Celsius * 1e- signed

2

13 y-axis gyroscope angular rate deg/s *2^-12 signed

14 x-axis gyroscope angular rate deg/s *2^-12 signed

16 x-axis accelerometer specific force m/s^2 *2^-10 signed

17 y-axis accelerometer specific force m/s^2 *2^-10 signed

18 z-axis accelerometer specific force m/s^2 *2^-10 signed

29.5.6 Raw Sensor Data Output

(This feature is not supported in protocol versions less than 15.01).

Some u-blox module products contain inertial sensors (IMU) that are directly connected to the

GNSS and cannot be directly accessed from outside the module. The UBX-ESF-RAW message can

be used to access raw measurements of these sensors. A variable number of data fields may be

used in a single message and these can contain different types of measurements. The type of

each measurement is specified in the dataType field. The possible data types are x, y and z-axis

measurements on gyroscope or accelerometer and gyroscope temperature measurements as

described in the ESF Measurement Data section. One UBX-ESF-RAW message can contain multiple

samples from the same sensor. The user can separate and order these using the time tags

attached to each of the measurements.

The measurements are made at a fixed rate. The sampling rate or other sensor configuration

options can not be changed.

To turn on this feature the UBX-ESF-RAW message must be enabled using UBX-CFG-MSG. If non-

zero rate is selected the message will be output but the selected rate does not otherwise have an

UBX-13003221 - R28 Early production information Page 136 of 460

## Page 152

u-blox 8 / u-blox M8 Receiver description - Manual

influence at the rate of the messages.

Turning on this feature does not disable sensor fusion in the receiver. To use an external

fusion algorithm consider disabling the automotive dead reckoning mode using UBX-

CFG-NAVX5.

29.5.7 Receiver Startup and Shutdown

Continuous dead reckoning is possible over receiver restarts if the following conditions are true:

• Non-volatile storage is available, or the save-on-shutdown feature (SOS) is used

• The vehicle is not moved while the receiver is off

During periods of external sensor data unavailability the receiver switches to GNSS-only

navigation if the last sensor information indicated the vehicle was moving.

30 High Navigation Rate (HNR)

This feature is only available with the ADR products.

This feature is only available with the UDR products.

30.1 Introduction

u-blox DR solutions allow a low latency position and velocity to be output at up to 30 Hz. The

maximum GNSS rate is 2 Hz. Sensors measurements are used to propagate the solution at the

higher rate between GNSS epochs.

The high navigation rate solution is output using the UBX-HNR-PVT message for firmwares using

protocol version 19+.

30.2 Configuration

The high navigation rate output can be configured using the UBX-CFG-HNR message.

If a high navigation rate has been configured with UBX-CFG-HNR then the number of

enabled output messages must be adjusted to keep within the maximum throughput of

the interface used.

UBX-13003221 - R28 Early production information Page 137 of 460

## Page 153

u-blox 8 / u-blox M8 Receiver description - Manual

Interface Description

31 NMEA Protocol

31.1 Protocol overview

31.1.1 Message format

NMEA messages sent by the GNSS receiver are based on NMEA 0183 Version 4.10. The following

figure shows the structure of a NMEA protocol message.

For further information on the NMEA Standard, refer to NMEA 0183 Standard For Interfacing

Marine Electronic Devices, Version 4.10, June, 2012. See http://www.nmea.org/ for ordering

instructions.

The NMEA standard allows for proprietary, manufacturer-specific messages to be added. These

shall be marked with a manufacturer mnemonic. The mnemonic assigned to u-blox is UBX and is

used for all non-standard messages. These proprietary NMEA messages therefore have the

address field set to PUBX. The first data field in a PUBX message identifies the message number

with two digits.

31.1.2 Talker ID

One of the ways the NMEA standard differentiates between GNSS is by using a two-letter

message identifier, the 'Talker ID'. The specific Talker ID used by a u-blox receiver will depend on

the device model and system configuration. The table below shows the Talker ID that will be used

for various GNSS configurations.

UBX-13003221 - R28 Early production information Page 138 of 460

## Page 154

u-blox 8 / u-blox M8 Receiver description - Manual

NMEA Talker IDs

Configured GNSS Talker ID

GPS, SBAS, QZSS GP

GLONASS GL

Galileo GA

BeiDou GB*

Any combination of GNSS GN

*This is a u-blox extension to the NMEA 4.10 standard. Only NMEA 4.11 defines the GB talker ID.

See also Extended Configuration in Protocol Configuration.

31.1.3 Protocol configuration

The NMEA protocol on u-blox receivers can be configured to the need of customer applications

using UBX-CFG-NMEA. For backwards compatibility various versions of this message are

supported, however, any new users should use the version that is not marked as deprecated.

There are four NMEA standards supported. The default NMEA version is 4.10. Alternatively

versions 4.00, 2.3, and 2.1 can be enabled (for details on how this affects the output refer to

section Position Fix Flags in NMEA Mode).

Customers using BeiDou and/or Galileo are recommended to select NMEA version 4.10,

as earlier versions have no support for these two GNSS.

Customers using High Precision GNSS (HPG) products are recommended to select

NMEA version 4.10, as earlier versions do no support the Float RTK (F) and Real Time

Kinematic (R) mode indicator flags in all messages.

NMEA defines satellite numbering systems for some, but not all GNSS (this is partly dependent

on the NMEA version). Satellite numbers for unsupported GNSS can be configured using UBX-

CFG-NMEA. Unknown satellite numbers are always reported as a null NMEA field (i.e. an empty

string).

The NMEA specification indicates that the GGA message is GPS-specific. However, u-blox

receivers support the output of a GGA message for each of the Talker IDs.

NMEA filtering flags

Parameter Description

Position filtering Enable positions from failed or invalid fixes to be reported (with the "V" status

flag to indicate that the data is not valid).

Valid position Enable positions from invalid fixes to be reported (with the "V" status flag to

filtering indicate that the data is not valid).

Time filtering Enable the receiver's best knowledge of time to be output, even though it

might be wrong.

Date filtering Enable the receiver's best knowledge of date to be output, even though it

might be wrong.

GPS-only filtering Restrict output to GPS satellites only.

Track filtering Permit course over ground (COG) to be reported even when it would otherwise

be frozen.

NMEA flags

Parameter Description

UBX-13003221 - R28 Early production information Page 139 of 460

## Page 155

u-blox 8 / u-blox M8 Receiver description - Manual

NMEA flags continued

Parameter Description

Compatibility Some older NMEA applications expect the NMEA output to be formatted in a

Mode specific way, for example, they will only work if the latitude and longitude have

exactly four digits behind the decimal point. u-blox receivers offer a

compatibility mode to support these legacy applications.

Consideration u-blox receivers use a sophisticated signal quality detection scheme, in order

Mode to produce the best possible position output. This algorithm considers all SV

measurements, and may eventually decide to only use a subset thereof, if it

improves the overall position accuracy. If Consideration Mode is enabled, all

satellites, which were considered for navigation, are communicated as being

used for the position determination. If Consideration Mode is disabled, only

those satellites which after the consideration step remained in the position

output are marked as being used.

Limit82 Mode Enabling this mode will limit the NMEA sentence length to a maximum of 82

characters.

High Precision Enabling this mode increases precision of the position output. Latitude and

Mode longitude then have seven digits after the decimal point, and altitude has

three digits after the decimal point. Note: The High Precision Mode cannot be

set in conjunction with either Compatibility Mode or Limit82 Mode.

Extended configuration

Option Description

GNSS to filter Filters satellites based on their GNSS

Satellite This field configures the display of satellites that do not have an NMEA-

numbering defined value. Note: this does not apply to satellites with an unknown ID.

Main Talker ID By default the main Talker ID (i.e. the Talker ID used for all messages other

than GSV) is determined by the GNSS assignment of the receiver's channels

(see UBX-CFG-GNSS). This field enables the main Talker ID to be overridden.

GSV Talker ID By default the Talker ID for GSV messages is GNSS-specific (as defined by

NMEA). This field enables the GSV Talker ID to be overridden.

BDS Talker ID By default the Talker ID for BeiDou is 'GB'. This field enables the BeiDou Talker

ID to be overridden.

Extra fields in NMEA 4.10 and above

Message Extra fields

GBS systemId, signalId

GNS navStatus

GRS systemId, signalId

GSA systemId

GSV signalId

RMC navStatus

31.1.4 Satellite numbering

The NMEA protocol (V4.10) identifies GNSS satellites with a one digit system ID and a two digit

satellite number. u-blox receivers support this method in their NMEA output when "strict" SV

numbering is selected.

UBX-13003221 - R28 Early production information Page 140 of 460

## Page 156

u-blox 8 / u-blox M8 Receiver description - Manual

In most cases this is the default setting, but can be checked or set using UBX-CFG-NMEA.

In order to support QZSS within current receivers and prepare for support of other systems (e.g.

Galileo) in future receivers, an "extended" SV numbering scheme can be enabled (using UBX-CFG-

NMEA).

This uses the NMEA-defined numbers where possible, but adds other number ranges to support

other GNSS. Note however that these non-standard extensions require 3 digit numbers, which

may not be supported by some NMEA parsing software. For example QZSS satellites are reported

using numbers in the range 193 to 197.

See Satellite Numbering for a complete list of satellite numbers.

GLONASS satellites can be tracked before they have been identified. In NMEA output,

such unknown satellite numbers are always reported as a null field (i.e. an empty string).

31.1.5 Latitude and longitude format

According to the NMEA Standard, Latitude and Longitude are output in the format of Degrees,

Minutes and (Decimal) Fractions of Minutes. To convert to Degrees and Fractions of Degrees, or

Degrees, Minutes, Seconds and Fractions of seconds, the 'Minutes' and 'Fractional Minutes' parts

need to be converted. In other words: If the GPS Receiver reports a Latitude of 4717.112671 North

and Longitude of 00833.914843 East, this is

Latitude 47 Degrees, 17.112671 Minutes

Longitude 8 Degrees, 33.914843 Minutes

or

Latitude 47 Degrees, 17 Minutes, 6.76026 Seconds

Longitude 8 Degrees, 33 Minutes, 54.89058 Seconds

or

Latitude 47.28521118 Degrees

Longitude 8.56524738 Degrees

31.1.6 Position fix flags

This section shows how u-blox implements the NMEA protocol and the conditions determining

how flags are set.

Flags in NMEA 4.10 and above

NMEA Message GLL, RMC GGA GLL, VTG RMC, GNS

Field status quality posMode posMode

No position fix (at power-up, after losing satellite lock) V 0 N N

GNSS fix, but user limits exceeded V 0 N N

Dead reckoning fix, but user limits exceeded V 6 E E

Dead reckoning fix A 6 E E

RTK float A 5 D F

RTK fixed A 4 D R

2D GNSS fix A 1 / 2 A / D A / D

3D GNSS fix A 1 / 2 A / D A / D

Combined GNSS/dead reckoning fix A 1 / 2 A / D A / D

See below (1) See below See below See below

(2) (3) (3)

UBX-13003221 - R28 Early production information Page 141 of 460

## Page 157

u-blox 8 / u-blox M8 Receiver description - Manual

(1) Possible values for status: V = Data invalid, A = Data valid

(2) Possible values for quality: 0 = No fix, 1 = Autonomous GNSS fix, 2 = Differential GNSS fix, 4 =

RTK fixed, 5 = RTK float, 6 = Estimated/Dead reckoning fix

(3) Possible values for posMode: N = No fix, E = Estimated/Dead reckoning fix, A = Autonomous

GNSS fix, D = Differential GNSS fix, F = RTK float, R = RTK fixed

Flags in NMEA 2.3 and above

NMEA Message GLL, RMC GGA GSA GLL, VTG,

RMC, GNS

Field status quality navMode posMode

No position fix (at power-up, after losing satellite lock) V 0 1 N

GNSS fix, but user limits exceeded V 0 1 N

Dead reckoning fix, but user limits exceeded V 6 2 E

Dead reckoning fix A 6 2 E

2D GNSS fix A 1 / 2 2 A / D

3D GNSS fix A 1 / 2 3 A / D

Combined GNSS/dead reckoning fix A 1 / 2 3 A / D

See below (1) See below See below See below

(2) (3) (4)

(1) Possible values for status: V = Data invalid, A = Data valid

(2) Possible values for quality: 0 = No fix, 1 = Autonomous GNSS fix, 2 = Differential GNSS fix, 4 =

RTK fixed, 5 = RTK float, 6 = Estimated/Dead reckoning fix

(3) Possible values for navMode: 1 = No fix, 2 = 2D fix, 3 = 3D fix

(4) Possible values for posMode: N = No fix, E = Estimated/Dead reckoning fix, A = Autonomous

GNSS fix, D = Differential GNSS fix, F = RTK float, R = RTK fixed

Flags in NMEA 2.1 and below

The flags in NMEA 2.1 and below are the same as NMEA 2.3 and above but with the following

differences:

• The posMode field is not output for GLL, RMC and VTG messages (each message has one field

less).

• The GGA quality field is set to 1 (instead of 6) for both types of dead reckoning fix.

31.1.7 Multi-GNSS considerations

Many applications which process NMEA messages assume that only a single GNSS is active.

However, when multiple GNSS are configured, the NMEA specification requires the output to

change in the following ways:

NMEA output for Multi-GNSS

Change Description

Main Talker ID The main Talker ID will be 'GN' (e.g. instead of 'GP' for a GPS receiver)

GSV Talker IDs The GSV message reports the signal strength of the visible

satellites. However, the Talker ID it uses is specific to the GNSS it is

reporting information for, so for a multi-GNSS receiver it will not be

the same as the main Talker ID (e.g. other messages will be using the

'GN' Talker ID but the GSV message will use GNSS-specific Talker

IDs).

UBX-13003221 - R28 Early production information Page 142 of 460

## Page 158

u-blox 8 / u-blox M8 Receiver description - Manual

NMEA output for Multi-GNSS continued

Change Description

Multiple GSA and GRS Multiple GSA and GRS messages are output for each fix, one for

Messages each GNSS. This may confuse applications which assume they are

output only once per position fix (as is the case for a single GNSS

receiver).

31.1.8 Output of invalid/unknown data

By default the receiver will not output invalid data. In such cases, it will output empty fields.

A valid position fix is reported as follows:

$GPGLL,4717.11634,N,00833.91297,E,124923.00,A,A*6E

An invalid position fix (but time valid) is reported as follows:

$GPGLL,,,,,124924.00,V,N*42

If Time is unknown (e.g. during a cold start):

$GPGLL,,,,,,V,N*64

Note:

An exception from the above default are dead reckoning fixes, which are also output

when invalid (user limits exceeded).

Differing from the NMEA standard, u-blox reports valid dead reckoning fixes with user

limits met (not exceeded) as valid (A) instead of invalid (V).

Output of invalid data marked with the 'Invalid/Valid' Flags can be enabled using the UBX

protocol message UBX-CFG-NMEA.

31.1.9 Messages overview

When configuring NMEA messages using the UBX protocol message UBX-CFG-MSG, the Class/Ids

shown in the table shall be used.

Page Mnemonic Cls/ID Description

NMEA Standard Messages Standard messages

145 DTM 0xF0 0x0A Datum reference

146 GBQ 0xF0 0x44 Poll a standard message (Talker ID GB)

146 GBS 0xF0 0x09 GNSS satellite fault detection

147 GGA 0xF0 0x00 Global positioning system fix data

149 GLL 0xF0 0x01 Latitude and longitude, with time of position fix and status

150 GLQ 0xF0 0x43 Poll a standard message (Talker ID GL)

150 GNQ 0xF0 0x42 Poll a standard message (Talker ID GN)

151 GNS 0xF0 0x0D GNSS fix data

152 GPQ 0xF0 0x40 Poll a standard message (Talker ID GP)

153 GRS 0xF0 0x06 GNSS range residuals

154 GSA 0xF0 0x02 GNSS DOP and active satellites

155 GST 0xF0 0x07 GNSS pseudorange error statistics

156 GSV 0xF0 0x03 GNSS satellites in view

157 RMC 0xF0 0x04 Recommended minimum data

UBX-13003221 - R28 Early production information Page 143 of 460

## Page 159

u-blox 8 / u-blox M8 Receiver description - Manual

NMEA Messages Overview continued

Page Mnemonic Cls/ID Description

158 THS 0xF0 0x0E True heading and status

159 TXT 0xF0 0x41 Text transmission

160 VLW 0xF0 0x0F Dual ground/water distance

161 VTG 0xF0 0x05 Course over ground and ground speed

162 ZDA 0xF0 0x08 Time and date

NMEA PUBX Messages Proprietary messages

163 CONFIG 0xF1 0x41 Set protocols and baud rate

164 POSITION 0xF1 0x00 Lat/Long position data

165 RATE 0xF1 0x40 Set NMEA message output rate

166 SVSTATUS 0xF1 0x03 Satellite status

167 TIME 0xF1 0x04 Time of day and clock information

UBX-13003221 - R28 Early production information Page 144 of 460

## Page 160

u-blox 8 / u-blox M8 Receiver description - Manual

31.2 Standard Messages

Standard messages: i.e. Messages as defined in the NMEA standard.

31.2.1 DTM

31.2.1.1 Datum reference

Message DTM

Description Datum reference

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message gives the difference between the current datum and the reference

datum.

The current datum is set to WGS84 by default.

The reference datum cannot be changed and is always set to WGS84.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x0A 11

Message Structure:

$xxDTM,datum,subDatum,lat,NS,lon,EW,alt,refDatum*cs<CR><LF>

Example:

$GPDTM,W84,,0.0,N,0.0,E,0.0,W84*6F

$GPDTM,999,,0.08,N,0.07,E,-47.7,W84*1C

Field Name Unit Format Example Description

No.

0 xxDTM - string $GPDTM DTM Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 datum - string W84 Local datum code: W84 = WGS84, P90 =

PZ90 (supported in protocol versions greater

than 19.1), 999 = user-defined

2 subDatum - string - A null field

3 lat min numeric 0.08 Offset in Latitude

4 NS - character S North/South indicator

5 lon min numeric 0.07 Offset in Longitude

6 EW - character E East/West indicator

7 alt m numeric -2.8 Offset in altitude

8 refDatum - string W84 Reference datum code: W84 (WGS 84, fixed

field)

9 cs - hexadecimal *67 Checksum

10 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 145 of 460

## Page 161

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.2 GBQ

31.2.2.1 Poll a standard message (Talker ID GB)

Message GBQ

Description Poll a standard message (Talker ID GB)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Polls a standard NMEA message if the current Talker ID is GB

ID for CFG-MSG Number of fields

Message Info 0xF0 0x44 4

Message Structure:

$xxGBQ,msgId*cs<CR><LF>

Example:

$EIGBQ,RMC*28

Field Name Unit Format Example Description

No.

0 xxGBQ - string $EIGBQ GBQ Message ID (xx = Talker ID of the device

requesting the poll)

1 msgId - string RMC Message ID of the message to be polled

2 cs - hexadecimal *28 Checksum

3 <CR><LF> - character - Carriage return and line feed

31.2.3 GBS

31.2.3.1 GNSS satellite fault detection

Message GBS

Description GNSS satellite fault detection

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message outputs the results of the Receiver Autonomous Integrity

Monitoring Algorithm (RAIM).

• The fields errLat, errLon and errAlt output the standard deviation of the

position calculation, using all satellites that pass the RAIM test successfully.

• The fields errLat, errLon and errAlt are only output if the RAIM process

passed successfully (i.e. no or successful edits happened). These fields are

never output if 4 or fewer satellites are used for the navigation calculation

(because, in such cases, integrity cannot be determined by the receiver

autonomously).

• The fields prob, bias and stdev are only output if at least one satellite failed in

the RAIM test.

If more than one satellites fail the RAIM test, only the information for the worst

satellite is output in this message.

ID for CFG-MSG Number of fields

UBX-13003221 - R28 Early production information Page 146 of 460

## Page 162

u-blox 8 / u-blox M8 Receiver description - Manual

Message Info 0xF0 0x09 13

Message Structure:

$xxGBS,time,errLat,errLon,errAlt,svid,prob,bias,stddev,systemId,signalId*cs<CR><LF>

Example:

$GPGBS,235503.00,1.6,1.4,3.2,,,,,,*40

$GPGBS,235458.00,1.4,1.3,3.1,03,,-21.4,3.8,1,0*5B

Field Name Unit Format Example Description

No.

0 xxGBS - string $GPGBS GBS Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 235503.00 UTC time to which this RAIM sentence

belongs. See section UTC representation in

the integration manual for details.

2 errLat m numeric 1.6 Expected error in latitude

3 errLon m numeric 1.4 Expected error in longitude

4 errAlt m numeric 3.2 Expected error in altitude

5 svid - numeric 03 Satellite ID of most likely failed satellite

6 prob - numeric - Probability of missed detection: null (not

supported, fixed field)

7 bias m numeric -21.4 Estimated bias of most likely failed satellite (a

priori residual)

8 stddev m numeric 3.8 Standard deviation of estimated bias

9 systemId - hexadecimal 1 NMEA-defined GNSS system ID, see Signal

Identifiers table (only available in NMEA 4.10

and later)

10 signalId - hexadecimal 0 NMEA-defined GNSS signal ID, see Signal

Identifiers table (only available in NMEA 4.10

and later)

11 cs - hexadecimal *5B Checksum

12 <CR><LF> - character - Carriage return and line feed

31.2.4 GGA

31.2.4.1 Global positioning system fix data

Message GGA

Description Global positioning system fix data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The output of this message is dependent on the currently selected datum

(default: WGS84). The NMEA specification indicates that the GGA message is

GPS-specific. However, when the receiver is configured for multi-GNSS, the

GGA message contents will be generated from the multi-GNSS solution. For

multi-GNSS use, it is recommended that the NMEA-GNS message is used

instead.

UBX-13003221 - R28 Early production information Page 147 of 460

## Page 163

u-blox 8 / u-blox M8 Receiver description - Manual

Time and position, together with GPS fixing-related data (number of satellites in

use, and the resulting HDOP, age of differential data if in use, etc.).

ID for CFG-MSG Number of fields

Message Info 0xF0 0x00 17

Message Structure:

$xxGGA,time,lat,NS,lon,EW,quality,numSV,HDOP,alt,altUnit,sep,sepUnit,diffAge,diffStation*cs<CR><LF>

Example:

$GPGGA,092725.00,4717.11399,N,00833.91590,E,1,08,1.01,499.6,M,48.0,M,,*5B

Field Name Unit Format Example Description

No.

0 xxGGA - string $GPGGA GGA Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 092725.00 UTC time. See section UTC representation in

the integration manual for details.

2 lat - ddmm. 4717.11399 Latitude (degrees and minutes), see format

mmmmm description

3 NS - character N North/South indicator

4 lon - dddmm. 00833.91590 Longitude (degrees and minutes), see format

mmmmm description

5 EW - character E East/West indicator

6 quality - digit 1 Quality indicator for position fix, see position

fix flags description

7 numSV - numeric 08 Number of satellites used (range: 0-12)

8 HDOP - numeric 1.01 Horizontal Dilution of Precision

9 alt m numeric 499.6 Altitude above mean sea level

10 altUnit - character M Altitude units: M (meters, fixed field)

11 sep m numeric 48.0 Geoid separation: difference between ellipsoid

and mean sea level

12 sepUnit - character M Geoid separation units: M (meters, fixed field)

13 diffAge s numeric - Age of differential corrections (null when

DGPS is not used)

14 diffStat - numeric - ID of station providing differential corrections

ion (null when DGPS is not used)

15 cs - hexadecimal *5B Checksum

16 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 148 of 460

## Page 164

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.5 GLL

31.2.5.1 Latitude and longitude, with time of position fix and status

Message GLL

Description Latitude and longitude, with time of position fix and status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The output of this message is dependent on the currently selected datum

(default: WGS84)

-

ID for CFG-MSG Number of fields

Message Info 0xF0 0x01 10

Message Structure:

$xxGLL,lat,NS,lon,EW,time,status,posMode*cs<CR><LF>

Example:

$GPGLL,4717.11364,N,00833.91565,E,092321.00,A,A*60

Field Name Unit Format Example Description

No.

0 xxGLL - string $GPGLL GLL Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 lat - ddmm. 4717.11364 Latitude (degrees and minutes), see format

mmmmm description

2 NS - character N North/South indicator

3 lon - dddmm. 00833.91565 Longitude (degrees and minutes), see format

mmmmm description

4 EW - character E East/West indicator

5 time - hhmmss.ss 092321.00 UTC time. See section UTC representation in

the integration manual for details.

6 status - character A Data validity status, see position fix flags

description

7 posMode - character A Positioning mode, see position fix flags

description (only available in NMEA 2.3 and

later)

8 cs - hexadecimal *60 Checksum

9 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 149 of 460

## Page 165

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.6 GLQ

31.2.6.1 Poll a standard message (Talker ID GL)

Message GLQ

Description Poll a standard message (Talker ID GL)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Polls a standard NMEA message if the current Talker ID is GL

ID for CFG-MSG Number of fields

Message Info 0xF0 0x43 4

Message Structure:

$xxGLQ,msgId*cs<CR><LF>

Example:

$EIGLQ,RMC*3A

Field Name Unit Format Example Description

No.

0 xxGLQ - string $EIGLQ GLQ Message ID (xx = Talker ID of the device

requesting the poll)

1 msgId - string RMC Message ID of the message to be polled

2 cs - hexadecimal *3A Checksum

3 <CR><LF> - character - Carriage return and line feed

31.2.7 GNQ

31.2.7.1 Poll a standard message (Talker ID GN)

Message GNQ

Description Poll a standard message (Talker ID GN)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Polls a standard NMEA message if the current Talker ID is GN

ID for CFG-MSG Number of fields

Message Info 0xF0 0x42 4

Message Structure:

$xxGNQ,msgId*cs<CR><LF>

Example:

$EIGNQ,RMC*3A

Field Name Unit Format Example Description

No.

0 xxGNQ - string $EIGNQ GNQ Message ID (xx = Talker ID of the device

requesting the poll)

1 msgId - string RMC Message ID of the message to be polled

2 cs - hexadecimal *3A Checksum

UBX-13003221 - R28 Early production information Page 150 of 460

## Page 166

u-blox 8 / u-blox M8 Receiver description - Manual

GNQ continued

Field Name Unit Format Example Description

No.

3 <CR><LF> - character - Carriage return and line feed

31.2.8 GNS

31.2.8.1 GNSS fix data

Message GNS

Description GNSS fix data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The output of this message is dependent on the currently selected datum

(default: WGS84)

Time and position, together with GNSS fixing-related data (number of satellites

in use, and the resulting HDOP, age of differential data if in use, etc.).

ID for CFG-MSG Number of fields

Message Info 0xF0 0x0D 16

Message Structure:

$xxGNS,time,lat,NS,lon,EW,posMode,numSV,HDOP,alt,sep,diffAge,diffStation,navStatus*cs<CR><LF>

Example:

$GNGNS,103600.01,5114.51176,N,00012.29380,W,ANNN,07,1.18,111.5,45.6,,,V*00

$GNGNS,122310.2,3722.425671,N,12258.856215,W,DAAA,14,0.9,1005.543,6.5,,,V*0E

$GPGNS,122310.2,,,,,,07,,,,5.2,23,V*02

Field Name Unit Format Example Description

No.

0 xxGNS - string $GPGNS GNS Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 091547.00 UTC time. See section UTC representation in

the integration manual for details.

2 lat - ddmm. 5114.50897 Latitude (degrees and minutes), see format

mmmmm description

3 NS - character N North/South indicator

4 lon - dddmm. 00012.28663 Longitude (degrees and minutes), see format

mmmmm description

5 EW - character E East/West indicator

6 posMode - character AAAA Positioning mode, see position fix flags

description. Four first characters are in the

following order for GPS, GLONASS, Galileo

and BeiDou. In NMEA GNS, u-blox uses a non-

standard implementation where same single

status is reported for all enabled and not

filtered out constellations.

7 numSV - numeric 10 Number of satellites used (range: 0-99)

UBX-13003221 - R28 Early production information Page 151 of 460

## Page 167

u-blox 8 / u-blox M8 Receiver description - Manual

GNS continued

Field Name Unit Format Example Description

No.

8 HDOP - numeric 0.83 Horizontal Dilution of Precision

9 alt m numeric 111.1 Altitude above mean sea level

10 sep m numeric 45.6 Geoid separation: difference between ellipsoid

and mean sea level

11 diffAge s numeric - Age of differential corrections (null when

DGPS is not used)

12 diffStat - numeric - ID of station providing differential corrections

ion (null when DGPS is not used)

13 navStatu - character V Navigational status indicator: V (Equipment is

s not providing navigational status information,

fixed field, only available in NMEA 4.10 and

later)

14 cs - hexadecimal *71 Checksum

15 <CR><LF> - character - Carriage return and line feed

31.2.9 GPQ

31.2.9.1 Poll a standard message (Talker ID GP)

Message GPQ

Description Poll a standard message (Talker ID GP)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Polls a standard NMEA message if the current Talker ID is GP

ID for CFG-MSG Number of fields

Message Info 0xF0 0x40 4

Message Structure:

$xxGPQ,msgId*cs<CR><LF>

Example:

$EIGPQ,RMC*3A

Field Name Unit Format Example Description

No.

0 xxGPQ - string $EIGPQ GPQ Message ID (xx = Talker ID of the device

requesting the poll)

1 msgId - string RMC Message ID of the message to be polled

2 cs - hexadecimal *3A Checksum

3 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 152 of 460

## Page 168

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.10 GRS

31.2.10.1 GNSS range residuals

Message GRS

Description GNSS range residuals

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message relates to associated GGA and GSA messages.

If less than 12 SVs are available, the remaining fields are output empty. If more

than 12 SVs are used, only the residuals of the first 12 SVs are output, in order to

remain consistent with the NMEA standard.

In a multi-GNSS system this message will be output multiple times, once for

each GNSS.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x06 19

Message Structure:

$xxGRS,time,mode{,residual},systemId,signalId*cs<CR><LF>

Example:

$GNGRS,104148.00,1,2.6,2.2,-1.6,-1.1,-1.7,-1.5,5.8,1.7,,,,,1,1*52

$GNGRS,104148.00,1,,0.0,2.5,0.0,,2.8,,,,,,,1,5*52

Field Name Unit Format Example Description

No.

0 xxGRS - string $GPGRS GRS Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 082632.00 UTC time of associated position fix. See

section UTC representation in the integration

manual for details.

2 mode - digit 1 Computation method used:

1 = Residuals were recomputed after the GGA

position was computed (fixed)

Start of repeated block (12 times)

3 + residual m numeric 0.54 Range residuals for SVs used in navigation.

1*N The SV order matches the order from the

GSA sentence

End of repeated block

15 systemId - hexadecimal 1 NMEA-defined GNSS system ID, see Signal

Identifiers table (only available in NMEA 4.10

and later)

16 signalId - hexadecimal 0 NMEA-defined GNSS signal ID, see Signal

Identifiers table (only available in NMEA 4.10

and later)

17 cs - hexadecimal *70 Checksum

18 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 153 of 460

## Page 169

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.11 GSA

31.2.11.1 GNSS DOP and active satellites

Message GSA

Description GNSS DOP and active satellites

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The GNSS receiver operating mode, satellites used for navigation, and DOP

values.

• If less than 12 SVs are used for navigation, the remaining fields are left empty.

If more than 12 SVs are used for navigation, only the IDs of the first 12 are

output.

• The SV numbers (fields 'svid') are in the range of 1 to 32 for GPS satellites, and

33 to 64 for SBAS satellites (33 = SBAS PRN 120, 34 = SBAS PRN 121, and so

on)

In a multi-GNSS system this message will be output multiple times, once for

each GNSS.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x02 21

Message Structure:

$xxGSA,opMode,navMode{,svid},PDOP,HDOP,VDOP,systemId*cs<CR><LF>

Example:

$GPGSA,A,3,23,29,07,08,09,18,26,28,,,,,1.94,1.18,1.54,1*0D

Field Name Unit Format Example Description

No.

0 xxGSA - string $GPGSA GSA Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 opMode - character A Operation mode:

M = Manually set to operate in 2D or 3D mode

A = Automatically switching between 2D or

3D mode

2 navMode - digit 3 Navigation mode, see position fix flags

description

Start of repeated block (12 times)

3 + svid - numeric 29 Satellite number

1*N

End of repeated block

15 PDOP - numeric 1.94 Position dilution of precision

16 HDOP - numeric 1.18 Horizontal dilution of precision

17 VDOP - numeric 1.54 Vertical dilution of precision

18 systemId - hexadecimal 1 NMEA-defined GNSS system ID, see Signal

Identifiers table (only available in NMEA 4.10

and later)

19 cs - hexadecimal *0D Checksum

UBX-13003221 - R28 Early production information Page 154 of 460

## Page 170

u-blox 8 / u-blox M8 Receiver description - Manual

GSA continued

Field Name Unit Format Example Description

No.

20 <CR><LF> - character - Carriage return and line feed

31.2.12 GST

31.2.12.1 GNSS pseudorange error statistics

Message GST

Description GNSS pseudorange error statistics

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message reports statistical information on the quality of the position

solution.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x07 11

Message Structure:

$xxGST,time,rangeRms,stdMajor,stdMinor,orient,stdLat,stdLong,stdAlt*cs<CR><LF>

Example:

$GPGST,082356.00,1.8,,,,1.7,1.3,2.2*7E

Field Name Unit Format Example Description

No.

0 xxGST - string $GPGST GST Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 082356.00 UTC time of associated position fix. See

section UTC representation in the integration

manual for details.

2 rangeRms m numeric 1.8 RMS value of the standard deviation of the

ranges

3 stdMajor m numeric - Standard deviation of semi-major axis (only

supported in ADR 4.10 and later)

4 stdMinor m numeric - Standard deviation of semi-minor axis (only

supported in ADR 4.10 and later)

5 orient deg numeric - Orientation of semi-major axis (only

supported in ADR 4.10 and later)

6 stdLat m numeric 1.7 Standard deviation of latitude error

7 stdLong m numeric 1.3 Standard deviation of longitude error

8 stdAlt m numeric 2.2 Standard deviation of altitude error

9 cs - hexadecimal *7E Checksum

10 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 155 of 460

## Page 171

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.13 GSV

31.2.13.1 GNSS satellites in view

Message GSV

Description GNSS satellites in view

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The number of satellites in view, together with each SV ID, elevation azimuth,

and signal strength (C/No) value. Only four satellite details are transmitted in

one message.

In a multi-GNSS system sets of GSV messages will be output multiple times,

one set for each GNSS.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x03 8..16

Message Structure:

$xxGSV,numMsg,msgNum,numSV{,svid,elv,az,cno},signalId*cs<CR><LF>

Example:

$GPGSV,3,1,09,09,,,17,10,,,40,12,,,49,13,,,35,1*6F

$GPGSV,3,2,09,15,,,44,17,,,45,19,,,44,24,,,50,1*64

$GPGSV,3,3,09,25,,,40,1*6E

$GPGSV,1,1,03,12,,,42,24,,,47,32,,,37,5*66

$GAGSV,1,1,00,2*76

Field Name Unit Format Example Description

No.

0 xxGSV - string $GPGSV GSV Message ID (xx = GSV Talker ID, see

NMEA Talker IDs table). Talker ID GN shall not

be used.

1 numMsg - digit 3 Number of messages, total number of GSV

messages being output (range: 1-9)

2 msgNum - digit 1 Number of this message (range: 1-numMsg)

3 numSV - numeric 10 Number of known satellites in view regarding

both the talker ID and the signalId

Start of repeated block (1..4 times)

4 + svid - numeric 23 Satellite ID

4*N

5 + elv deg numeric 38 Elevation (<= 90)

4*N

6 + az deg numeric 230 Azimuth (range: 0-359)

4*N

7 + cno dB numeric 44 Signal strength (C/N0, range: 0-99), null when

4*N Hz not tracking

End of repeated block

UBX-13003221 - R28 Early production information Page 156 of 460

## Page 172

u-blox 8 / u-blox M8 Receiver description - Manual

GSV continued

Field Name Unit Format Example Description

No.

5.. signalId - hexadecimal 0 NMEA-defined GNSS signal ID, see Signal

16 Identifiers table (only available in NMEA 4.10

and later)

6.. cs - hexadecimal *7F Checksum

16

7.. <CR><LF> - character - Carriage return and line feed

16

31.2.14 RMC

31.2.14.1 Recommended minimum data

Message RMC

Description Recommended minimum data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The output of this message is dependent on the currently selected datum

(default: WGS84)

The recommended minimum sentence defined by NMEA for GNSS system data.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x04 16

Message Structure:

$xxRMC,time,status,lat,NS,lon,EW,spd,cog,date,mv,mvEW,posMode,navStatus*cs<CR><LF>

Example:

$GPRMC,083559.00,A,4717.11437,N,00833.91522,E,0.004,77.52,091202,,,A,V*57

Field Name Unit Format Example Description

No.

0 xxRMC - string $GPRMC RMC Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 083559.00 UTC time. See section UTC representation in

the integration manual for details.

2 status - character A Data validity status, see position fix flags

description

3 lat - ddmm. 4717.11437 Latitude (degrees and minutes), see format

mmmmm description

4 NS - character N North/South indicator

5 lon - dddmm. 00833.91522 Longitude (degrees and minutes), see format

mmmmm description

6 EW - character E East/West indicator

7 spd kno numeric 0.004 Speed over ground

ts

8 cog deg numeric 77.52 Course over ground

UBX-13003221 - R28 Early production information Page 157 of 460

## Page 173

u-blox 8 / u-blox M8 Receiver description - Manual

RMC continued

Field Name Unit Format Example Description

No.

9 date - ddmmyy 091202 Date in day, month, year format. See section

UTC representation in the integration manual

for details.

10 mv deg numeric - Magnetic variation value. Only supported in

ADR 4.10 and later

11 mvEW - character - Magnetic variation E/W indicator. Only

supported in ADR 4.10 and later

12 posMode - character A Mode Indicator, see position fix flags

description (only available in NMEA 2.3 and

later)

13 navStatu - character V Navigational status indicator: V (Equipment is

s not providing navigational status information,

fixed field, only available in NMEA 4.10 and

later)

14 cs - hexadecimal *57 Checksum

15 <CR><LF> - character - Carriage return and line feed

31.2.15 THS

31.2.15.1 True heading and status

Message THS

Description True heading and status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with ADR products)

Type Output

Comment Actual vehicle heading in degrees produced by any device or system producing

true heading. This sentence includes a Mode indicator field providing critical

safety-related information about the heading data, and replaces the HDT

sentence.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x0E 5

Message Structure:

$xxTHS,headt,mi*cs<CR><LF>

Example:

$GPTHS,77.52,E*32

Field Name Unit Format Example Description

No.

0 xxTHS - string $GPTHS THS Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 headt deg numeric 77.52 Heading of vehicle (true)

ree

s

UBX-13003221 - R28 Early production information Page 158 of 460

## Page 174

u-blox 8 / u-blox M8 Receiver description - Manual

THS continued

Field Name Unit Format Example Description

No.

2 mi - character E Mode indicator:

A = Autonomous

E = Estimated (dead reckoning)

M = Manual input

S = Simulator

V = Data not valid

3 cs - hexadecimal *32 Checksum

4 <CR><LF> - character - Carriage return and line feed

31.2.16 TXT

31.2.16.1 Text transmission

Message TXT

Description Text transmission

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment -

ID for CFG-MSG Number of fields

Message Info 0xF0 0x41 7

Message Structure:

$xxTXT,numMsg,msgNum,msgType,text*cs<CR><LF>

Example:

$GPTXT,01,01,02,u-blox ag - www.u-blox.com*50

$GPTXT,01,01,02,ANTARIS ATR0620 HW 00000040*67

Field Name Unit Format Example Description

No.

0 xxTXT - string $GPTXT TXT Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 numMsg - numeric 01 Total number of messages in this

transmission (range: 1-99)

2 msgNum - numeric 01 Message number in this transmission (range:

1-numMsg)

3 msgType - numeric 02 Text identifier (u-blox receivers specify the

type of the message with this number):

00: Error

01: Warning

02: Notice

07: User

4 text - string www.u-blox. Any ASCII text

com

5 cs - hexadecimal *67 Checksum

UBX-13003221 - R28 Early production information Page 159 of 460

## Page 175

u-blox 8 / u-blox M8 Receiver description - Manual

TXT continued

Field Name Unit Format Example Description

No.

6 <CR><LF> - character - Carriage return and line feed

31.2.17 VLW

31.2.17.1 Dual ground/water distance

Message VLW

Description Dual ground/water distance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The distance traveled, relative to the water and over the ground. This message

relates to the odometer feature detailed in the integration manual.

Contrarily to the NMEA standard, if NMEA 2.1 or 2.3 are configured, the

sentence will additionally contain tgd, tgdUnit, gd and gdUnit fields.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x0F 11

Message Structure:

$xxVLW,twd,twdUnit,wd,wdUnit,tgd,tgdUnit,gd,gdUnit*cs<CR><LF>

Example:

$GPVLW,,N,,N,15.8,N,1.2,N*06

Field Name Unit Format Example Description

No.

0 xxVLW - string $GPVLW VLW Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 twd nmi numeric - Total cumulative water distance: null (fixed

field)

2 twdUnit - character N Total cumulative water distance units: N

(nautical miles, fixed field)

3 wd nmi numeric - Water distance since reset: null (fixed field)

4 wdUnit - character N Water distance since reset units: N (nautical

miles, fixed field)

5 tgd nmi numeric 15.8 Total cumulative ground distance (only

available in NMEA 4.00 and later)

6 tgdUnit - character N Total cumulative ground distance units: N

(nautical miles, fixed field, only available in

NMEA 4.00 and later)

7 gd nmi numeric 1.2 Ground distance since reset (only available in

NMEA 4.00 and later)

8 gdUnit - character N Ground distance since reset units: N (nautical

miles, fixed field, only available in NMEA 4.00

and later)

9 cs - hexadecimal *06 Checksum

10 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 160 of 460

## Page 176

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.18 VTG

31.2.18.1 Course over ground and ground speed

Message VTG

Description Course over ground and ground speed

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment Velocity is given as course over ground (COG) and speed over ground (SOG).

ID for CFG-MSG Number of fields

Message Info 0xF0 0x05 12

Message Structure:

$xxVTG,cogt,cogtUnit,cogm,cogmUnit,sogn,sognUnit,sogk,sogkUnit,posMode*cs<CR><LF>

Example:

$GPVTG,77.52,T,,M,0.004,N,0.008,K,A*06

Field Name Unit Format Example Description

No.

0 xxVTG - string $GPVTG VTG Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 cogt deg numeric 77.52 Course over ground (true)

ree

s

2 cogtUnit - character T Course over ground units: T (degrees true,

fixed field)

3 cogm deg numeric - Course over ground (magnetic). Only

ree supported in ADR 4.10 and above

s

4 cogmUnit - character M Course over ground units: M (degrees

magnetic, fixed field)

5 sogn kno numeric 0.004 Speed over ground

ts

6 sognUnit - character N Speed over ground units: N (knots, fixed field)

7 sogk km/ numeric 0.008 Speed over ground

h

8 sogkUnit - character K Speed over ground units: K (kilometers per

hour, fixed field)

9 posMode - character A Mode indicator, see position fix flags

description (only available in NMEA 2.3 and

later)

10 cs - hexadecimal *06 Checksum

11 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 161 of 460

## Page 177

u-blox 8 / u-blox M8 Receiver description - Manual

31.2.19 ZDA

31.2.19.1 Time and date

Message ZDA

Description Time and date

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment UTC, day, month, year and local time zone.

ID for CFG-MSG Number of fields

Message Info 0xF0 0x08 9

Message Structure:

$xxZDA,time,day,month,year,ltzh,ltzn*cs<CR><LF>

Example:

$GPZDA,082710.00,16,09,2002,00,00*64

Field Name Unit Format Example Description

No.

0 xxZDA - string $GPZDA ZDA Message ID (xx = current Talker ID, see

NMEA Talker IDs table)

1 time - hhmmss.ss 082710.00 UTC Time. See section UTC representation in

the integration manual for details.

2 day day dd 16 UTC day (range: 1-31)

3 month mo mm 09 UTC month (range: 1-12)

nth

4 year yea yyyy 2002 UTC year

r

5 ltzh - xx 00 Local time zone hours (fixed field, always 00)

6 ltzn - zz 00 Local time zone minutes (fixed field, always

00)

7 cs - hexadecimal *64 Checksum

8 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 162 of 460

## Page 178

u-blox 8 / u-blox M8 Receiver description - Manual

31.3 PUBX Messages

Proprietary messages: i.e. Messages defined by u-blox.

31.3.1 CONFIG (PUBX,41)

31.3.1.1 Set protocols and baud rate

Message CONFIG

Description Set protocols and baud rate

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Set

Comment -

ID for CFG-MSG Number of fields

Message Info 0xF1 0x41 9

Message Structure:

$PUBX,41,portId,inProto,outProto,baudrate,autobauding*cs<CR><LF>

Example:

$PUBX,41,1,0007,0003,19200,0*25

Field Name Unit Format Example Description

No.

0 $PUBX - string $PUBX Message ID, UBX protocol header, proprietary

sentence

1 msgId - numeric 41 Proprietary message identifier

2 portId - numeric 1 ID of communication port. See section

Communication ports in the integration

manual for details.

3 inProto - hexadecimal 0007 Input protocol mask. Bitmask, specifying

which protocols(s) are allowed for input. See

section Communication ports in the

integration manual for details.

4 outProto - hexadecimal 0003 Output protocol mask. Bitmask, specifying

which protocols(s) are allowed for input. See

section Communication ports in the

integration manual for details.

5 baudrate bits numeric 19200 Baud rate

/s

6 autobaud - numeric 0 Autobauding: 1=enable, 0=disable (not

ing supported on u-blox 5, set to 0)

7 cs - hexadecimal *25 Checksum

8 <CR><LF> - character - Carriage return and line feed

UBX-13003221 - R28 Early production information Page 163 of 460

## Page 179

u-blox 8 / u-blox M8 Receiver description - Manual

31.3.2 POSITION (PUBX,00)

31.3.2.1 Lat/Long position data

Message POSITION

Description Lat/Long position data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The output of this message is dependent on the currently selected datum

(default: WGS84).

This message contains position solution data. The datum selection may be

changed using the message UBX-CFG-DAT.

ID for CFG-MSG Number of fields

Message Info 0xF1 0x00 23

Message Structure:

$PUBX,00,time,lat,NS,long,EW,altRef,navStat,hAcc,vAcc,SOG,COG,vVel,diffAge,HDOP,VDOP,TDOP,numSvs,re

served,DR,*cs<CR><LF>

Example:

$PUBX,00,081350.00,4717.113210,N,00833.915187,E,546.589,G3,2.1,2.0,0.007,77.52,0.007,,0.92,1.19,0.7

7,9,0,0*5F

Field Name Unit Format Example Description

No.

0 $PUBX - string $PUBX Message ID, UBX protocol header, proprietary

sentence

1 msgId - numeric 00 Proprietary message identifier: 00

2 time - hhmmss.ss 081350.00 UTC time. See section UTC representation in

the integration manual for details.

3 lat - ddmm. 4717.113210 Latitude (degrees and minutes), see format

mmmmm description

4 NS - character N North/South Indicator

5 long - dddmm. 00833. Longitude (degrees and minutes), see format

mmmmm 915187 description

6 EW - character E East/West indicator

7 altRef m numeric 546.589 Altitude above user datum ellipsoid

8 navStat - string G3 Navigation Status:

NF = No Fix

DR = Dead reckoning only solution

G2 = Stand alone 2D solution

G3 = Stand alone 3D solution

D2 = Differential 2D solution

D3 = Differential 3D solution

RK = Combined GPS + dead reckoning

solution

TT = Time only solution

9 hAcc m numeric 2.1 Horizontal accuracy estimate

UBX-13003221 - R28 Early production information Page 164 of 460

## Page 180

u-blox 8 / u-blox M8 Receiver description - Manual

POSITION continued

Field Name Unit Format Example Description

No.

10 vAcc m numeric 2.0 Vertical accuracy estimate

11 SOG km/ numeric 0.007 Speed over ground

h

12 COG deg numeric 77.52 Course over ground

13 vVel m/s numeric 0.007 Vertical velocity (positive downwards)

14 diffAge s numeric - Age of differential corrections (blank when

DGPS is not used)

15 HDOP - numeric 0.92 HDOP, Horizontal Dilution of Precision

16 VDOP - numeric 1.19 VDOP, Vertical Dilution of Precision

17 TDOP - numeric 0.77 TDOP, Time Dilution of Precision

18 numSvs - numeric 9 Number of satellites used in the navigation

solution

19 reserved - numeric 0 Reserved, always set to 0

20 DR - numeric 0 DR used

21 cs - hexadecimal *5B Checksum

22 <CR><LF> - character - Carriage return and line feed

31.3.3 RATE (PUBX,40)

31.3.3.1 Set NMEA message output rate

Message RATE

Description Set NMEA message output rate

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Set

Comment Set/Get message rate configuration (s) to/from the receiver.

• Send rate is relative to the event a message is registered on. For example, if

the rate of a navigation message is set to 2, the message is sent every second

navigation solution.

ID for CFG-MSG Number of fields

Message Info 0xF1 0x40 11

Message Structure:

$PUBX,40,msgId,rddc,rus1,rus2,rusb,rspi,reserved*cs<CR><LF>

Example:

$PUBX,40,GLL,1,0,0,0,0,0*5D

Field Name Unit Format Example Description

No.

0 $PUBX - string $PUBX Message ID, UBX protocol header, proprietary

sentence

1 ID - numeric 40 Proprietary message identifier

2 msgId - string GLL NMEA message identifier

UBX-13003221 - R28 Early production information Page 165 of 460

## Page 181

u-blox 8 / u-blox M8 Receiver description - Manual

RATE continued

Field Name Unit Format Example Description

No.

3 rddc cycl numeric 1 output rate on DDC

es 0 disables that message from being output

on this port

1 means that this message is output every

epoch

4 rus1 cycl numeric 1 output rate on USART 1

es 0 disables that message from being output

on this port

1 means that this message is output every

epoch

5 rus2 cycl numeric 1 output rate on USART 2

es 0 disables that message from being output

on this port

1 means that this message is output every

epoch

6 rusb cycl numeric 1 output rate on USB

es 0 disables that message from being output

on this port

1 means that this message is output every

epoch

7 rspi cycl numeric 1 output rate on SPI

es 0 disables that message from being output

on this port

1 means that this message is output every

epoch

8 reserved - numeric 0 Reserved: always fill with 0

9 cs - hexadecimal *5D Checksum

10 <CR><LF> - character - Carriage return and line feed

31.3.4 SVSTATUS (PUBX,03)

31.3.4.1 Satellite status

Message SVSTATUS

Description Satellite status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The PUBX,03 message contains satellite status information.

ID for CFG-MSG Number of fields

Message Info 0xF1 0x03 5 + 6*n

Message Structure:

$PUBX,03,GT{,sv,s,az,el,cno,lck},*cs<CR><LF>

Example:

UBX-13003221 - R28 Early production information Page 166 of 460

## Page 182

u-blox 8 / u-blox M8 Receiver description - Manual

$PUBX,03,11,23,-,,,45,010,29,-,,,46,013,07,-,,,42,015,08,U,067,31,42,025,10,U,195,33,46,026,18,U,32

6,08,39,026,17,-,,,32,015,26,U,306,66,48,025,27,U,073,10,36,026,28,U,089,61,46,024,15,-,,,39,014*0D

Field Name Unit Format Example Description

No.

0 $PUBX - string $PUBX Message ID, UBX protocol header, proprietary

sentence

1 msgId - numeric 03 Proprietary message identifier: 03

2 n - numeric 11 Number of GNSS satellites tracked

Start of repeated block (n times)

3 + sv - numeric 23 Satellite ID according to UBX svId mapping

6*N (see Satellite Numbering)

4 + s - character - Satellite status:

6*N - = Not used

U = Used in solution

e = Ephemeris available, but not used for

navigation

5 + az deg numeric - Satellite azimuth (range: 0-359)

6*N

6 + el deg numeric - Satellite elevation (<= 90)

6*N

7 + cno dB numeric 45 Signal strength (C/N0, range 0-99), blank

6*N Hz when not tracking

8 + lck s numeric 010 Satellite carrier lock time (range: 0-64)

6*N 0: code lock only

64: lock for 64 seconds or more

End of repeated block

3 + cs - hexadecimal *0D Checksum

6*n

4 + <CR><LF> - character - Carriage return and line feed

6*n

31.3.5 TIME (PUBX,04)

31.3.5.1 Time of day and clock information

Message TIME

Description Time of day and clock information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment -

ID for CFG-MSG Number of fields

Message Info 0xF1 0x04 12

Message Structure:

$PUBX,04,time,date,utcTow,utcWk,leapSec,clkBias,clkDrift,tpGran,*cs<CR><LF>

Example:

UBX-13003221 - R28 Early production information Page 167 of 460

## Page 183

u-blox 8 / u-blox M8 Receiver description - Manual

$PUBX,04,073731.00,091202,113851.00,1196,15D,1930035,-2660.664,43,*3C

Field Name Unit Format Example Description

No.

0 $PUBX - string $PUBX Message ID, UBX protocol header, proprietary

sentence

1 msgId - numeric 04 Proprietary message identifier: 04

2 time - hhmmss.ss 073731.00 UTC time. See section UTC representation in

the integration manual for details.

3 date - ddmmyy 091202 UTC date, day, month, year. See section UTC

representation in the integration manual for

details.

4 utcTow s numeric 113851.00 UTC time of week

5 utcWk - numeric 1196 UTC week number, continues beyond 1023

6 leapSec s numeric/text 15D Leap seconds

The number is marked with a D if the value is

the firmware default value. If the value is not

marked it has been received from a satellite.

7 clkBias ns numeric 1930035 Receiver clock bias

8 clkDrift ns/ numeric -2660.664 Receiver clock drift

s

9 tpGran ns numeric 43 Time pulse granularity, the quantization error

of the TIMEPULSE pin

10 cs - hexadecimal *3C Checksum

11 <CR><LF> - character - Carriage return and line feed

32 UBX Protocol

32.1 UBX Protocol Key Features

u-blox receivers support a u-blox proprietary protocol to communicate with a host. This protocol

has the following key features:

• Compact - uses 8-bit binary data.

• Checksum protected - uses a low-overhead checksum algorithm

• Modular - uses a 2-stage message identifier (Class and Message ID)

32.2 UBX Frame Structure

The structure of a basic UBX Frame is shown in the following diagram.

UBX-13003221 - R28 Early production information Page 168 of 460

## Page 184

u-blox 8 / u-blox M8 Receiver description - Manual

• Every Frame starts with a 2-byte Preamble consisting of two synchronization characters: 0xB5

0x62.

• A 1-byte Message Class field follows. A Class is a group of messages that are related to each

other.

• A 1-byte Message ID field defines the message that is to follow.

• A 2-byte Length field follows. The length is defined as being that of the payload only. It does not

include the Preamble, Message Class, Message ID, Length, or Cyclic Redundancy Check (CRC)

fields. The number format of the length field is a Little-Endian unsigned 16-bit integer.

• The Payload field contains a variable number of bytes.

• The two 1-byte CK_A and CK_B fields hold a 16-bit checksum whose calculation is defined below.

This concludes the Frame.

32.3 UBX Payload Definition Rules

32.3.1 Structure Packing

Values are placed in such an order that structure packing is not a problem. This means that 2-

byte values shall start on offsets which are a multiple of 2; 4-byte values shall start at a multiple

of 4; and so on.

32.3.2 Reserved Elements

Some messages contain reserved fields or bits to allow for future expansion. The contents of

these elements should be ignored in output messages and must be set to zero in input messages.

Where a message is output and subsequently returned to the receiver as an input message,

reserved elements can either be explicitly set to zero or left with whatever value they were output

with.

UBX-13003221 - R28 Early production information Page 169 of 460

## Page 185

u-blox 8 / u-blox M8 Receiver description - Manual

32.3.3 Undefined Values

The description of some fields provides specific meanings for specific values. For example, the

field gnssId appears in many UBX messages and uses 0 to indicate GPS, 1 for SBAS and so on (see

Satellite Numbering for details); however it is usually stored in a byte with far more possible values

than the handful currently defined. All such undefined values are reserved for future expansion

and therefore should not be used.

32.3.4 Message Naming

Referring to messages is done by adding the class name and a dash in front of the message name.

For example, the version information message is referred to as UBX-MON-VER. Referring to

message fields or their values is done by adding a dot and the name, e.g. UBX-MON-VER.

swVersion.

32.3.5 Number Formats

All multi-byte values are ordered in Little Endian format, unless otherwise indicated.

All floating point values are transmitted in IEEE754 single or double precision.

Variable Type Definitions

Short Type Size Comment Min/Max Resolution

(Bytes)

U1 Unsigned Char 1 0..255 1

RU1_3 Unsigned Char 1 Binary floating 0..(31*2^7) non- ~ 2^(value >> 5)

point with 3 bits continuous

exponent, eeeb

bbbb with b the

base and e the

exponent,

(value & 0x1F)

<< (value >> 5)

I1 Signed Char 1 2's complement -128 .. 127 1

X1 Bitfield 1 n/a n/a

U2 Unsigned Short 2 0 .. 65535 1

RU2_5 Unsigned Short 2 Binary floating 0 .. (2047*2^31) ~ 2^(value >> 11)

point with 5 bits non-continuous

exponent, eeee

ebbb bbbb

bbbb with b the

base and e the

exponent,

(value & 0x7FF)

<< (value >> 11)

I2 Signed Short 2 2's complement -32768 .. 32767 1

X2 Bitfield 2 n/a n/a

U4 Unsigned Long 4 0 .. 1

4'294'967'295

I4 Signed Long 4 2's complement -2'147'483'648 .. 1

2'147'483'647

UBX-13003221 - R28 Early production information Page 170 of 460

## Page 186

u-blox 8 / u-blox M8 Receiver description - Manual

Variable Type Definitions continued

Short Type Size Comment Min/Max Resolution

(Bytes)

X4 Bitfield 4 n/a n/a

R4 IEEE 754 Single Precision 4 -1*2^+127 .. ~ Value * 2^-24

2^+127

I8 Signed Long Long 8 2's complement -1*2^+63 .. 1

2^+63 - 1

R8 IEEE 754 Double Precision 8 -1*2^+1023 .. ~ Value * 2^-53

2^+1023

CH ASCII / ISO 8859.1 1

Encoding

The description of some integer values (e.g. U2, I4 or I8) indicates a fixed-point format

(e.g. [UU.FF], [IIIII.FFF] or [IIIIIII.FFFFFFFF]). The fixed-point value can be

retrieved from the integer value by first casting it to appropriate type (e.g. as a floating-

point number) and then scaling it with the indicated scaling factor.

32.4 UBX Checksum

The checksum is calculated over the Message, starting and including the CLASS field up until, but

excluding, the Checksum Field:

The checksum algorithm used is the 8-Bit Fletcher Algorithm, which is used in the TCP standard (

RFC 1145). This algorithm works as follows:

• Buffer[N] contains the data over which the checksum is to be calculated.

• The two CK_ values are 8-Bit unsigned integers only! If implementing with larger-sized integer

values, make sure to mask both CK_A and CK_B with 0xFF after both operations in the loop.

CK_A = 0, CK_B = 0

For(I=0;I<N;I++)

{

UBX-13003221 - R28 Early production information Page 171 of 460

## Page 187

u-blox 8 / u-blox M8 Receiver description - Manual

CK_A = CK_A + Buffer[I]

CK_B = CK_B + CK_A

}

• After the loop, the two U1 values contain the checksum, transmitted after the Message, which

conclude the Frame.

32.5 UBX Message Flow

There are certain features associated with the messages being sent back and forth:

32.5.1 Acknowledgement

When messages from the class CFG are sent to the receiver, the receiver will send an

"acknowledge" (UBX-ACK-ACK) or a "not acknowledge" (UBX-ACK-NAK) message back to the sender,

depending on whether or not the message was processed correctly.

Some messages from other classes (e.g. LOG) also use the same acknowledgement mechanism.

32.5.2 Polling Mechanism

All messages that are output by the receiver in a periodic manner (i.e. messages in classes MON,

NAV and RXM) and Get/Set type messages, such as the messages in the CFG class, can also be

polled.

The UBX protocol is designed so that messages can be polled by sending the message required to

the receiver but without a payload (or with just a single parameter that identifies the poll request).

The receiver then responds with the same message with the payload populated.

32.6 UBX Class IDs

A class is a grouping of messages which are related to each other. The following table lists all the

current message classes.

Name Class Description

NAV 0x01 Navigation Results Messages: Position, Speed, Time, Acceleration, Heading, DOP,

SVs used

RXM 0x02 Receiver Manager Messages: Satellite Status, RTC Status

INF 0x04 Information Messages: Printf-Style Messages, with IDs such as Error, Warning,

Notice

ACK 0x05 Ack/Nak Messages: Acknowledge or Reject messages to UBX-CFG input messages

CFG 0x06 Configuration Input Messages: Configure the receiver

UPD 0x09 Firmware Update Messages: Memory/Flash erase/write, Reboot, Flash

identification, etc

MON 0x0A Monitoring Messages: Communication Status, Stack Usage, Task Status

AID 0x0B AssistNow Aiding Messages: Ephemeris, Almanac, other A-GPS data input

TIM 0x0D Timing Messages: Time Pulse Output, Time Mark Results

ESF 0x10 External Sensor Fusion Messages: External Sensor Measurements and Status

Information

MGA 0x13 Multiple GNSS Assistance Messages: Assistance data for various GNSS

LOG 0x21 Logging Messages: Log creation, deletion, info and retrieval

SEC 0x27 Security Feature Messages

HNR 0x28 High Rate Navigation Results Messages: High rate time, position, speed, heading

All remaining class IDs are reserved.

UBX-13003221 - R28 Early production information Page 172 of 460

## Page 188

u-blox 8 / u-blox M8 Receiver description - Manual

32.7 UBX Messages Overview

Page Mnemonic Cls/ID Length Type Description

UBX Class ACK Ack/Nak Messages

179 ACK-ACK 0x05 0x01 2 Output Message acknowledged

179 ACK-NAK 0x05 0x00 2 Output Message not acknowledged

UBX Class AID AssistNow Aiding Messages

180 AID-ALM 0x0B 0x30 0 Poll Request Poll GPS aiding almanac data

180 AID-ALM 0x0B 0x30 1 Poll Request Poll GPS aiding almanac data for a SV

181 AID-ALM 0x0B 0x30 (8) or (40) Input/Output GPS aiding almanac input/output...

182 AID-AOP 0x0B 0x33 0 Poll Request Poll AssistNow Autonomous data, all...

182 AID-AOP 0x0B 0x33 1 Poll Request Poll AssistNow Autonomous data, one...

183 AID-AOP 0x0B 0x33 68 Input/Output AssistNow Autonomous data

184 AID-EPH 0x0B 0x31 0 Poll Request Poll GPS aiding ephemeris data

184 AID-EPH 0x0B 0x31 1 Poll Request Poll GPS aiding ephemeris data for a SV

185 AID-EPH 0x0B 0x31 (8) or (104) Input/Output GPS aiding ephemeris input/output...

186 AID-HUI 0x0B 0x02 0 Poll Request Poll GPS health, UTC, ionosphere...

186 AID-HUI 0x0B 0x02 72 Input/Output GPS health, UTC and ionosphere...

188 AID-INI 0x0B 0x01 0 Poll Request Poll GPS initial aiding data

188 AID-INI 0x0B 0x01 48 Input/Output Aiding position, time, frequency, clock...

UBX Class CFG Configuration Input Messages

191 CFG-ANT 0x06 0x13 4 Get/set Antenna control settings

192 CFG-BATCH 0x06 0x93 8 Get/set Get/set data batching configuration

193 CFG-CFG 0x06 0x09 (12) or (13) Command Clear, save and load configurations

195 CFG-DAT 0x06 0x06 44 Set Set user-defined datum

196 CFG-DAT 0x06 0x06 52 Get Get currently defined datum

197 CFG-DGNSS 0x06 0x70 4 Get/set DGNSS configuration

197 CFG-DOSC 0x06 0x61 4 + 32*numO...Get/set Disciplined oscillator configuration

199 CFG-ESFALG 0x06 0x56 12 Get/set Get/set IMU-mount misalignment...

200 CFG-ESFA 0x06 0x4C 20 Get/set Get/set the Accelerometer (A) sensor...

201 CFG-ESFG 0x06 0x4D 20 Get/set Get/set the Gyroscope (G) sensor...

201 CFG-ESFWT 0x06 0x82 32 Get/set Get/set wheel-tick configuration

204 CFG-ESRC 0x06 0x60 4 + 36*numS...Get/set External synchronization source...

206 CFG-GEOFENCE 0x06 0x69 8 + 12*numF... Get/set Geofencing configuration

207 CFG-GNSS 0x06 0x3E 4 + 8*numCo...Get/set GNSS system configuration

210 CFG-HNR 0x06 0x5C 4 Get/set High navigation rate settings

210 CFG-INF 0x06 0x02 1 Poll Request Poll configuration for one protocol

211 CFG-INF 0x06 0x02 0 + 10*N Get/set Information message configuration

212 CFG-ITFM 0x06 0x39 8 Get/set Jamming/interference monitor...

213 CFG-LOGFILTER 0x06 0x47 12 Get/set Data logger configuration

UBX-13003221 - R28 Early production information Page 173 of 460

## Page 189

u-blox 8 / u-blox M8 Receiver description - Manual

UBX Messages Overview continued

Page Mnemonic Cls/ID Length Type Description

215 CFG-MSG 0x06 0x01 2 Poll Request Poll a message configuration

215 CFG-MSG 0x06 0x01 8 Get/set Set message rate(s)

216 CFG-MSG 0x06 0x01 3 Get/set Set message rate

216 CFG-NAV5 0x06 0x24 36 Get/set Navigation engine settings

219 CFG-NAVX5 0x06 0x23 40 Get/set Navigation engine expert settings

221 CFG-NAVX5 0x06 0x23 40 Get/set Navigation engine expert settings

224 CFG-NAVX5 0x06 0x23 44 Get/set Navigation engine expert settings

226 CFG-NMEA 0x06 0x17 4 Get/set NMEA protocol configuration...

228 CFG-NMEA 0x06 0x17 12 Get/set NMEA protocol configuration V0...

231 CFG-NMEA 0x06 0x17 20 Get/set Extended NMEA protocol configuration V1

234 CFG-ODO 0x06 0x1E 20 Get/set Odometer, low-speed COG engine...

235 CFG-PM2 0x06 0x3B 44 Get/set Extended power management...

237 CFG-PM2 0x06 0x3B 48 Get/set Extended power management...

239 CFG-PM2 0x06 0x3B 48 Get/set Extended power management...

242 CFG-PMS 0x06 0x86 8 Get/set Power mode setup

243 CFG-PRT 0x06 0x00 1 Poll Request Polls the configuration for one I/O port

243 CFG-PRT 0x06 0x00 20 Get/set Port configuration for UART ports

246 CFG-PRT 0x06 0x00 20 Get/set Port configuration for USB port

248 CFG-PRT 0x06 0x00 20 Get/set Port configuration for SPI port

251 CFG-PRT 0x06 0x00 20 Get/set Port configuration for I2C (DDC) port

253 CFG-PWR 0x06 0x57 8 Set Put receiver in a defined power state

254 CFG-RATE 0x06 0x08 6 Get/set Navigation/measurement rate settings

255 CFG-RINV 0x06 0x34 1 + 1*N Get/set Contents of remote inventory

256 CFG-RST 0x06 0x04 4 Command Reset receiver / Clear backup data...

258 CFG-RXM 0x06 0x11 2 Get/set RXM configuration

258 CFG-RXM 0x06 0x11 2 Get/set RXM configuration

259 CFG-SBAS 0x06 0x16 8 Get/set SBAS configuration

261 CFG-SENIF 0x06 0x88 6 Get/set I2C sensor interface configuration

262 CFG-SLAS 0x06 0x8D 4 Get/set SLAS configuration

263 CFG-SMGR 0x06 0x62 20 Get/set Synchronization manager configuration

266 CFG-SPT 0x06 0x64 12 Get/set Configure and start a sensor...

266 CFG-TMODE2 0x06 0x3D 28 Get/set Time mode settings 2

268 CFG-TMODE3 0x06 0x71 40 Get/set Time mode settings 3

270 CFG-TP5 0x06 0x31 0 Poll Request Poll time pulse parameters for time...

270 CFG-TP5 0x06 0x31 1 Poll Request Poll time pulse parameters

271 CFG-TP5 0x06 0x31 32 Get/set Time pulse parameters

272 CFG-TP5 0x06 0x31 32 Get/set Time pulse parameters

274 CFG-TXSLOT 0x06 0x53 16 Set TX buffer time slots configuration

UBX-13003221 - R28 Early production information Page 174 of 460

## Page 190

u-blox 8 / u-blox M8 Receiver description - Manual

UBX Messages Overview continued

Page Mnemonic Cls/ID Length Type Description

275 CFG-USB 0x06 0x1B 108 Get/set USB configuration

UBX Class ESF External Sensor Fusion Messages

277 ESF-ALG 0x10 0x14 16 Periodic/Polled IMU alignment information

278 ESF-INS 0x10 0x15 36 Periodic/Polled Vehicle dynamics information

280 ESF-MEAS 0x10 0x02 (8 + 4*numM...Input/Output External sensor fusion measurements

281 ESF-RAW 0x10 0x03 4 + 8*N Output Raw sensor measurements

282 ESF-STATUS 0x10 0x10 16 + 4*numS... Periodic/Polled External sensor fusion status

UBX Class HNR High Rate Navigation Results Messages

286 HNR-ATT 0x28 0x01 32 Periodic/Polled Attitude solution

287 HNR-INS 0x28 0x02 36 Periodic/Polled Vehicle dynamics information

288 HNR-PVT 0x28 0x00 72 Periodic/Polled High rate output of PVT solution

UBX Class INF Information Messages

291 INF-DEBUG 0x04 0x04 0 + 1*N Output ASCII output with debug contents

291 INF-ERROR 0x04 0x00 0 + 1*N Output ASCII output with error contents

292 INF-NOTICE 0x04 0x02 0 + 1*N Output ASCII output with informational contents

292 INF-TEST 0x04 0x03 0 + 1*N Output ASCII output with test contents

293 INF-WARNING 0x04 0x01 0 + 1*N Output ASCII output with warning contents

UBX Class LOG Logging Messages

294 LOG-BATCH 0x21 0x11 100 Polled Batched data

297 LOG-CREATE 0x21 0x07 8 Command Create log file

298 LOG-ERASE 0x21 0x03 0 Command Erase logged data

298 LOG-FINDTIME 0x21 0x0E 10 Input Find index of a log entry based on a...

299 LOG-FINDTIME 0x21 0x0E 8 Output Response to FINDTIME request

300 LOG-INFO 0x21 0x08 0 Poll Request Poll for log information

300 LOG-INFO 0x21 0x08 48 Output Log information

302 LOG-RETRIEVEBA... 0x21 0x10 4 Command Request batch data

303 LOG-RETRIEVEPO... 0x21 0x0f 32 Output Odometer log entry

303 LOG-RETRIEVEPOS 0x21 0x0b 40 Output Position fix log entry

304 LOG-RETRIEVEST... 0x21 0x0d 16 + 1*byteCo...Output Byte string log entry

305 LOG-RETRIEVE 0x21 0x09 12 Command Request log data

306 LOG-STRING 0x21 0x04 0 + 1*N Command Store arbitrary string in on-board flash

UBX Class MGA Multiple GNSS Assistance Messages

307 MGA-ACK-DATA0 0x13 0x60 8 Output Multiple GNSS acknowledge message

308 MGA-ANO 0x13 0x20 76 Input Multiple GNSS AssistNow Offline...

309 MGA-BDS-EPH 0x13 0x03 88 Input BeiDou ephemeris assistance

310 MGA-BDS-ALM 0x13 0x03 40 Input BeiDou almanac assistance

311 MGA-BDS-HEALTH 0x13 0x03 68 Input BeiDou health assistance

312 MGA-BDS-UTC 0x13 0x03 20 Input BeiDou UTC assistance

UBX-13003221 - R28 Early production information Page 175 of 460

## Page 191

u-blox 8 / u-blox M8 Receiver description - Manual

UBX Messages Overview continued

Page Mnemonic Cls/ID Length Type Description

312 MGA-BDS-IONO 0x13 0x03 16 Input BeiDou ionosphere assistance

313 MGA-DBD 0x13 0x80 0 Poll Request Poll the navigation database

313 MGA-DBD 0x13 0x80 12 + 1*N Input/Output Navigation database dump entry

314 MGA-FLASH-DATA 0x13 0x21 6 + 1*size Input Transfer MGA-ANO data block to flash

315 MGA-FLASH-STOP 0x13 0x21 2 Input Finish flashing MGA-ANO data

315 MGA-FLASH-ACK 0x13 0x21 6 Output Acknowledge last FLASH-DATA or -STOP

316 MGA-GAL-EPH 0x13 0x02 76 Input Galileo ephemeris assistance

318 MGA-GAL-ALM 0x13 0x02 32 Input Galileo almanac assistance

319 MGA-GAL-TIMEO... 0x13 0x02 12 Input Galileo GPS time offset assistance

319 MGA-GAL-UTC 0x13 0x02 20 Input Galileo UTC assistance

320 MGA-GLO-EPH 0x13 0x06 48 Input GLONASS ephemeris assistance

321 MGA-GLO-ALM 0x13 0x06 36 Input GLONASS almanac assistance

322 MGA-GLO-TIMEO... 0x13 0x06 20 Input GLONASS auxiliary time offset assistance

323 MGA-GPS-EPH 0x13 0x00 68 Input GPS ephemeris assistance

325 MGA-GPS-ALM 0x13 0x00 36 Input GPS almanac assistance

326 MGA-GPS-HEALTH 0x13 0x00 40 Input GPS health assistance

326 MGA-GPS-UTC 0x13 0x00 20 Input GPS UTC assistance

327 MGA-GPS-IONO 0x13 0x00 16 Input GPS ionosphere assistance

328 MGA-INI-POS_XYZ 0x13 0x40 20 Input Initial position assistance

329 MGA-INI-POS_LLH 0x13 0x40 20 Input Initial position assistance

329 MGA-INI-TIME_UTC 0x13 0x40 24 Input Initial time assistance

331 MGA-INI-TIME_GN... 0x13 0x40 24 Input Initial time assistance

332 MGA-INI-CLKD 0x13 0x40 12 Input Initial clock drift assistance

333 MGA-INI-FREQ 0x13 0x40 12 Input Initial frequency assistance

334 MGA-INI-EOP 0x13 0x40 72 Input Earth orientation parameters assistance

334 MGA-QZSS-EPH 0x13 0x05 68 Input QZSS ephemeris assistance

336 MGA-QZSS-ALM 0x13 0x05 36 Input QZSS almanac assistance

337 MGA-QZSS-HEAL... 0x13 0x05 12 Input QZSS health assistance

UBX Class MON Monitoring Messages

338 MON-BATCH 0x0A 0x32 12 Polled Data batching buffer status

339 MON-GNSS 0x0A 0x28 8 Polled Information message major GNSS...

341 MON-HW2 0x0A 0x0B 28 Periodic/Polled Extended hardware status

342 MON-HW 0x0A 0x09 60 Periodic/polled Hardware status

343 MON-IO 0x0A 0x02 0 + 20*N Periodic/Polled I/O system status

344 MON-MSGPP 0x0A 0x06 120 Periodic/Polled Message parse and process status

344 MON-PATCH 0x0A 0x27 0 Poll Request Poll request for installed patches

345 MON-PATCH 0x0A 0x27 4 + 16*nEntriesPolled Installed patches

346 MON-RXBUF 0x0A 0x07 24 Periodic/Polled Receiver buffer status

UBX-13003221 - R28 Early production information Page 176 of 460

## Page 192

u-blox 8 / u-blox M8 Receiver description - Manual

UBX Messages Overview continued

Page Mnemonic Cls/ID Length Type Description

346 MON-RXR 0x0A 0x21 1 Output Receiver status information

347 MON-SMGR 0x0A 0x2E 16 Periodic/Polled Synchronization manager status

350 MON-SPT 0x0A 0x2F 4 + 12*numR... Polled Sensor production test

354 MON-TXBUF 0x0A 0x08 28 Periodic/Polled Transmitter buffer status

355 MON-VER 0x0A 0x04 0 Poll Request Poll receiver and software version

355 MON-VER 0x0A 0x04 40 + 30*N Polled Receiver and software version

UBX Class NAV Navigation Results Messages

357 NAV-AOPSTATUS 0x01 0x60 16 Periodic/Polled AssistNow Autonomous status

358 NAV-ATT 0x01 0x05 32 Periodic/Polled Attitude solution

359 NAV-CLOCK 0x01 0x22 20 Periodic/Polled Clock solution

359 NAV-COV 0x01 0x36 64 Periodic/Polled Covariance matrices

360 NAV-DGPS 0x01 0x31 16 + 12*numChPeriodic/Polled DGPS data used for NAV

361 NAV-DOP 0x01 0x04 18 Periodic/Polled Dilution of precision

362 NAV-EELL 0x01 0x3d 16 Periodic/Polled Position error ellipse parameters

363 NAV-EOE 0x01 0x61 4 Periodic End of epoch

363 NAV-GEOFENCE 0x01 0x39 8 + 2*numFe... Periodic/Polled Geofencing status

364 NAV-HPPOSECEF 0x01 0x13 28 Periodic/Polled High precision position solution in ECEF

365 NAV-HPPOSLLH 0x01 0x14 36 Periodic/Polled High precision geodetic position solution

367 NAV-NMI 0x01 0x28 16 Periodic/Polled Navigation message cross-check...

370 NAV-ODO 0x01 0x09 20 Periodic/Polled Odometer solution

371 NAV-ORB 0x01 0x34 8 + 6*numSv Periodic/Polled GNSS orbit database info

374 NAV-POSECEF 0x01 0x01 20 Periodic/Polled Position solution in ECEF

374 NAV-POSLLH 0x01 0x02 28 Periodic/Polled Geodetic position solution

375 NAV-PVT 0x01 0x07 92 Periodic/Polled Navigation position velocity time solution

379 NAV-RELPOSNED 0x01 0x3C 40 Periodic/Polled Relative positioning information in...

381 NAV-RESETODO 0x01 0x10 0 Command Reset odometer

381 NAV-SAT 0x01 0x35 8 + 12*numSvsPeriodic/Polled Satellite information

383 NAV-SBAS 0x01 0x32 12 + 12*cnt Periodic/Polled SBAS status data

385 NAV-SLAS 0x01 0x42 20 + 8*cnt Periodic/Polled QZSS L1S SLAS status data

386 NAV-SOL 0x01 0x06 52 Periodic/Polled Navigation solution information

388 NAV-STATUS 0x01 0x03 16 Periodic/Polled Receiver navigation status

390 NAV-SVINFO 0x01 0x30 8 + 12*numCh Periodic/Polled Space vehicle information

392 NAV-SVIN 0x01 0x3B 40 Periodic/Polled Survey-in data

393 NAV-TIMEBDS 0x01 0x24 20 Periodic/Polled BeiDou time solution

394 NAV-TIMEGAL 0x01 0x25 20 Periodic/Polled Galileo time solution

395 NAV-TIMEGLO 0x01 0x23 20 Periodic/Polled GLONASS time solution

397 NAV-TIMEGPS 0x01 0x20 16 Periodic/Polled GPS time solution

398 NAV-TIMELS 0x01 0x26 24 Periodic/Polled Leap second event information

UBX-13003221 - R28 Early production information Page 177 of 460

## Page 193

u-blox 8 / u-blox M8 Receiver description - Manual

UBX Messages Overview continued

Page Mnemonic Cls/ID Length Type Description

400 NAV-TIMEUTC 0x01 0x21 20 Periodic/Polled UTC time solution

401 NAV-VELECEF 0x01 0x11 20 Periodic/Polled Velocity solution in ECEF

402 NAV-VELNED 0x01 0x12 36 Periodic/Polled Velocity solution in NED frame

UBX Class RXM Receiver Manager Messages

403 RXM-IMES 0x02 0x61 4 + 44*numTx Periodic/Polled Indoor Messaging System information

406 RXM-MEASX 0x02 0x14 44 + 24*num... Periodic/Polled Satellite measurements for RRLP

407 RXM-PMREQ 0x02 0x41 8 Command Power management request

408 RXM-PMREQ 0x02 0x41 16 Command Power management request

409 RXM-RAWX 0x02 0x15 16 + 32*num... Periodic/Polled Multi-GNSS raw measurement data

413 RXM-RAWX 0x02 0x15 16 + 32*num... Periodic/Polled Multi-GNSS raw measurements

416 RXM-RLM 0x02 0x59 16 Output Galileo SAR short-RLM report

417 RXM-RLM 0x02 0x59 28 Output Galileo SAR long-RLM report

418 RXM-RTCM 0x02 0x32 8 Output RTCM input status

419 RXM-SFRBX 0x02 0x13 8 + 4*numW... Output Broadcast navigation data subframe

420 RXM-SFRBX 0x02 0x13 8 + 4*numW... Output Broadcast navigation data subframe

421 RXM-SVSI 0x02 0x20 8 + 6*numSV Periodic/Polled SV status info

UBX Class SEC Security Feature Messages

423 SEC-UNIQID 0x27 0x03 9 Output Unique chip ID

UBX Class TIM Timing Messages

424 TIM-DOSC 0x0D 0x11 8 Output Disciplined oscillator control

424 TIM-FCHG 0x0D 0x16 32 Periodic/Polled Oscillator frequency changed notification

425 TIM-HOC 0x0D 0x17 8 Input Host oscillator control

426 TIM-SMEAS 0x0D 0x13 12 + 24*num... Input/Output Source measurement

428 TIM-SVIN 0x0D 0x04 28 Periodic/Polled Survey-in data

429 TIM-TM2 0x0D 0x03 28 Periodic/Polled Time mark data

430 TIM-TOS 0x0D 0x12 56 Periodic Time pulse time and frequency data

432 TIM-TP 0x0D 0x01 16 Periodic/Polled Time pulse time data

434 TIM-VCOCAL 0x0D 0x15 1 Command Stop calibration

435 TIM-VCOCAL 0x0D 0x15 12 Command VCO calibration extended command

436 TIM-VCOCAL 0x0D 0x15 12 Periodic/Polled Results of the calibration

437 TIM-VRFY 0x0D 0x06 20 Periodic/Polled Sourced time verification

UBX Class UPD Firmware Update Messages

438 UPD-SOS 0x09 0x14 0 Poll Request Poll backup restore status

438 UPD-SOS 0x09 0x14 4 Command Create backup in flash

439 UPD-SOS 0x09 0x14 4 Command Clear backup in flash

439 UPD-SOS 0x09 0x14 8 Output Backup creation acknowledge

440 UPD-SOS 0x09 0x14 8 Output System restored from backup

UBX-13003221 - R28 Early production information Page 178 of 460

## Page 194

u-blox 8 / u-blox M8 Receiver description - Manual

32.8 UBX-ACK (0x05)

Ack/Nak Messages: i.e. Acknowledge or Reject messages to UBX-CFG input messages.

Messages in the UBX-ACK class output the processing results to UBX-CFG and some other

messages.

32.8.1 UBX-ACK-ACK (0x05 0x01)

32.8.1.1 Message acknowledged

Message UBX-ACK-ACK

Description Message acknowledged

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment Output upon processing of an input message. A UBX-ACK-ACK is sent as soon

as possible but at least within one second.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x05 0x01 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - clsID - Class ID of the Acknowledged Message

1 U1 - msgID - Message ID of the Acknowledged

Message

32.8.2 UBX-ACK-NAK (0x05 0x00)

32.8.2.1 Message not acknowledged

Message UBX-ACK-NAK

Description Message not acknowledged

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment Output upon processing of an input message. A UBX-ACK-NAK is sent as soon

as possible but at least within one second.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x05 0x00 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - clsID - Class ID of the Not-Acknowledged

Message

1 U1 - msgID - Message ID of the Not-Acknowledged

Message

UBX-13003221 - R28 Early production information Page 179 of 460

## Page 195

u-blox 8 / u-blox M8 Receiver description - Manual

32.9 UBX-AID (0x0B)

AssistNow Aiding Messages: i.e. Ephemeris, Almanac, other A-GPS data input.

Messages in the AID class are used to send GPS aiding data to the receiver.

32.9.1 UBX-AID-ALM (0x0B 0x30)

32.9.1.1 Poll GPS aiding almanac data

Message UBX-AID-ALM

Description Poll GPS aiding almanac data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll GPS aiding data (Almanac) for all 32 SVs by sending this message to the

receiver without any payload. The receiver will return 32 messages of type AID-

ALM as defined below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x30 0 see below CK_A CK_B

No payload

32.9.1.2 Poll GPS aiding almanac data for a SV

Message UBX-AID-ALM

Description Poll GPS aiding almanac data for a SV

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll GPS aiding data (Almanac) for an SV by sending this message to the

receiver. The receiver will return one message of type AID-ALM as defined below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x30 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - svid - SV ID for which the receiver shall return its

Almanac Data (Valid Range: 1 .. 32 or 51,

56, 63).

UBX-13003221 - R28 Early production information Page 180 of 460

## Page 196

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.1.3 GPS aiding almanac input/output message

Message UBX-AID-ALM

Description GPS aiding almanac input/output message

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

• If the WEEK Value is 0, DWRD0 to DWRD7 are not sent as the Almanac is not

available for the given SV. This may happen even if NAV-SVINFO and RXM-

SVSI are indicating almanac availability as the internal data may not represent

the content of an original broadcast almanac (or only parts thereof).

• DWORD0 to DWORD7 contain the 8 words following the Hand-Over Word (

HOW ) from the GPS navigation message, either pages 1 to 24 of sub-frame 5

or pages 2 to 10 of subframe 4. See IS-GPS-200 for a full description of the

contents of the Almanac pages.

• In DWORD0 to DWORD7, the parity bits have been removed, and the 24 bits of

data are located in Bits 0 to 23. Bits 24 to 31 shall be ignored.

• Example: Parameter e (Eccentricity) from Almanac Subframe 4/5, Word 3, Bits

69-84 within the subframe can be found in DWRD0, Bits 15-0 whereas Bit 0 is

the LSB.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x30 (8) or (40) see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - svid - SV ID for which this

Almanac Data is (Valid Range: 1 .. 32 or 51,

56, 63).

4 U4 - week - Issue Date of Almanac (GPS week number)

Start of optional block

8 U4[8] - dwrd - Almanac Words

End of optional block

UBX-13003221 - R28 Early production information Page 181 of 460

## Page 197

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.2 UBX-AID-AOP (0x0B 0x33)

32.9.2.1 Poll AssistNow Autonomous data, all satellites

Message UBX-AID-AOP

Description Poll AssistNow Autonomous data, all satellites

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll AssistNow Autonomous aiding data for all GPS satellites by sending this

empty message. The receiver will return an AID-AOP message (see definition

below) for each GPS satellite for which data is available.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x33 0 see below CK_A CK_B

No payload

32.9.2.2 Poll AssistNow Autonomous data, one GPS satellite

Message UBX-AID-AOP

Description Poll AssistNow Autonomous data, one GPS satellite

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll the AssistNow Autonomous data for the specified GPS satellite. The

receiver will return an AID-AOP message (see definition below) if data is available

for the requested satellite.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x33 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - svid - GPS SV ID for which the data is requested

(valid range: 1..32).

UBX-13003221 - R28 Early production information Page 182 of 460

## Page 198

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.2.3 AssistNow Autonomous data

Message UBX-AID-AOP

Description AssistNow Autonomous data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

If enabled, this message is output at irregular intervals. It is output whenever

AssistNow Autonomous has produced new data for a satellite. Depending on the

availability of the optional data the receiver will output either version of the

message. If this message is polled using one of the two poll requests described

above, the receiver will send this message if AssistNow Autonomous data is

available, or it will send the corresponding poll request message if no AssistNow

Autonomous data is available for each satellite (i.e. svid 1..32). At the user's

choice the optional data may be chopped from the payload of a previously polled

message when sending the message back to the receiver. Sending a valid AID-

AOP message to the receiver will automatically enable the AssistNow

Autonomous feature on the receiver. See section AssistNow Autonomous in the

receiver description for details on this feature.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x33 68 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - gnssId - GNSS identifier (see Satellite Numbering)

1 U1 - svId - Satellite identifier (see Satellite

Numbering)

2 U1[2] - reserved1 - Reserved

4 U1[64] - data - assistance data

UBX-13003221 - R28 Early production information Page 183 of 460

## Page 199

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.3 UBX-AID-EPH (0x0B 0x31)

32.9.3.1 Poll GPS aiding ephemeris data

Message UBX-AID-EPH

Description Poll GPS aiding ephemeris data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll GPS Aiding Data (Ephemeris) for all 32 SVs by sending this message to the

receiver without any payload. The receiver will return 32 messages of type AID-

EPH as defined below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x31 0 see below CK_A CK_B

No payload

32.9.3.2 Poll GPS aiding ephemeris data for a SV

Message UBX-AID-EPH

Description Poll GPS aiding ephemeris data for a SV

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

Poll GPS Constellation Data (Ephemeris) for an SV by sending this message to

the receiver. The receiver will return one message of type AID-EPH as defined

below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x31 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - svid - SV ID for which the receiver shall return its

Ephemeris Data (Valid Range: 1 .. 32).

UBX-13003221 - R28 Early production information Page 184 of 460

## Page 200

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.3.3 GPS aiding ephemeris input/output message

Message UBX-AID-EPH

Description GPS aiding ephemeris input/output message

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

• SF1D0 to SF3D7 is only sent if ephemeris is available for this SV. If not, the

payload may be reduced to 8 Bytes, or all bytes are set to zero, indicating that

this SV Number does not have valid ephemeris for the moment. This may

happen even if NAV-SVINFO and RXM-SVSI are indicating ephemeris

availability as the internal data may not represent the content of an original

broadcast ephemeris (or only parts thereof).

• SF1D0 to SF3D7 contain the 24 words following the Hand-Over Word ( HOW )

from the GPS navigation message, subframes 1 to 3. The Truncated TOW

Count is not valid and cannot be used. See IS-GPS-200 for a full description of

the contents of the Subframes.

• In SF1D0 to SF3D7, the parity bits have been removed, and the 24 bits of data

are located in Bits 0 to 23. Bits 24 to 31 shall be ignored.

• When polled, the data contained in this message does not represent the full

original ephemeris broadcast. Some fields that are irrelevant to u-blox

receivers may be missing. The week number in Subframe 1 has already been

modified to match the Time Of Ephemeris (TOE).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x31 (8) or (104) see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - svid - SV ID for which this ephemeris data is

(Valid Range: 1 .. 32).

4 U4 - how - Hand-Over Word of first Subframe. This is

required if data is sent to the receiver.

0 indicates that no Ephemeris Data is

following.

Start of optional block

8 U4[8] - sf1d - Subframe 1 Words 3..10 (SF1D0..SF1D7)

40 U4[8] - sf2d - Subframe 2 Words 3..10 (SF2D0..SF2D7)

72 U4[8] - sf3d - Subframe 3 Words 3..10 (SF3D0..SF3D7)

End of optional block

UBX-13003221 - R28 Early production information Page 185 of 460

## Page 201

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.4 UBX-AID-HUI (0x0B 0x02)

32.9.4.1 Poll GPS health, UTC, ionosphere parameters

Message UBX-AID-HUI

Description Poll GPS health, UTC, ionosphere parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x02 0 see below CK_A CK_B

No payload

32.9.4.2 GPS health, UTC and ionosphere parameters

Message UBX-AID-HUI

Description GPS health, UTC and ionosphere parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

This message contains a health bit mask, UTC time and Klobuchar parameters.

For more information on these parameters, see the ICD-GPS-200

documentation.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x02 72 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X4 - health - Bitmask, every bit represenst a GPS SV (1-

32). If the bit is set the SV is healthy.

4 R8 - utcA0 - UTC - parameter A0

12 R8 - utcA1 - UTC - parameter A1

20 I4 - utcTOW - UTC - reference time of week

24 I2 - utcWNT - UTC - reference week number

26 I2 - utcLS - UTC - time difference due to leap seconds

before event

28 I2 - utcWNF - UTC - week number when next leap

second event occurs

30 I2 - utcDN - UTC - day of week when next leap second

event occurs

32 I2 - utcLSF - UTC - time difference due to leap seconds

after event

UBX-13003221 - R28 Early production information Page 186 of 460

## Page 202

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-AID-HUI continued

Byte Offset Number Scaling Name Unit Description

Format

34 I2 - utcSpare - UTC - Spare to ensure structure is a

multiple of 4 bytes

36 R4 - klobA0 s Klobuchar - alpha 0

40 R4 - klobA1 s/semi Klobuchar - alpha 1

circle

44 R4 - klobA2 s/semi Klobuchar - alpha 2

circle^

2

48 R4 - klobA3 s/semi Klobuchar - alpha 3

circle^

3

52 R4 - klobB0 s Klobuchar - beta 0

56 R4 - klobB1 s/semi Klobuchar - beta 1

circle

60 R4 - klobB2 s/semi Klobuchar - beta 2

circle^

2

64 R4 - klobB3 s/semi Klobuchar - beta 3

circle^

3

68 X4 - flags - flags (see graphic below)

Bitfield flags

This graphic explains the bits of flags

Name Description

healthValid Healthmask field in this message is valid

utcValid UTC parameter fields in this message are valid

klobValid Klobuchar parameter fields in this message are valid

UBX-13003221 - R28 Early production information Page 187 of 460

## Page 203

u-blox 8 / u-blox M8 Receiver description - Manual

32.9.5 UBX-AID-INI (0x0B 0x01)

32.9.5.1 Poll GPS initial aiding data

Message UBX-AID-INI

Description Poll GPS initial aiding data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x01 0 see below CK_A CK_B

No payload

32.9.5.2 Aiding position, time, frequency, clock drift

Message UBX-AID-INI

Description Aiding position, time, frequency, clock drift

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment All UBX-AID messages are deprecated; use UBX-MGA messages instead

This message contains position, time and clock drift information. The position

can be input in either the ECEF X/Y/Z coordinate system or as lat/lon/height. The

time can either be input as inexact value via the standard communication

interface, suffering from latency depending on the baud rate, or using hardware

time synchronization where an accurate time pulse is input on the external

interrupts. It is also possible to supply hardware frequency aiding by connecting

a continuous signal to an external interrupt.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0B 0x01 48 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 I4 - ecefXOrLat cm_ WGS84 ECEF X coordinate or latitude,

or_ depending on flags below

deg*1e-

7

4 I4 - ecefYOrLon cm_ WGS84 ECEF Y coordinate or longitude,

or_ depending on flags below

deg*1e-

7

8 I4 - ecefZOrAlt cm WGS84 ECEF Z coordinate or altitude,

depending on flags below

12 U4 - posAcc cm Position accuracy (stddev)

UBX-13003221 - R28 Early production information Page 188 of 460

## Page 204

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-AID-INI continued

Byte Offset Number Scaling Name Unit Description

Format

16 X2 - tmCfg - Time mark configuration (see graphic

below)

18 U2 - wnoOrDate week_ Actual week number or

or_ yearSince2000/Month (YYMM),

yearM depending on flags below

onth

20 U4 - towOrTime ms_ Actual time of week or

or_ DayOfMonth/Hour/Minute/Second

dayHo (DDHHMMSS), depending on flags below

urMin

uteSe

c

24 I4 - towNs ns Fractional part of time of week

28 U4 - tAccMs ms Milliseconds part of time accuracy

32 U4 - tAccNs ns Nanoseconds part of time accuracy

36 I4 - clkDOrFreq ns/s_ Clock drift or frequency, depending on

or_ flags below

Hz*1e-

2

40 U4 - clkDAccOrFreq ns/s_ Accuracy of clock drift or frequency,

Acc or_ppb depending on flags below

44 X4 - flags - Bitmask with the following flags (see

graphic below)

Bitfield tmCfg

This graphic explains the bits of tmCfg

Name Description

fEdge use falling edge (default rising)

tm1 time mark on extint 1 (default extint 0)

f1 frequency on extint 1 (default extint 0)

UBX-13003221 - R28 Early production information Page 189 of 460

## Page 205

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

pos Position is valid

time Time is valid

clockD Clock drift data contains valid clock drift, must not be set together with clockF

tp Use time pulse

clockF Clock drift data contains valid frequency, must not be set together with clockD

lla Position is given in lat/long/alt (default is ECEF)

altInv Altitude is not valid, if lla was set

prevTm Use time mark received before AID-INI message (default uses mark received after message)

utc Time is given as UTC date/time (default is GPS wno/tow)

UBX-13003221 - R28 Early production information Page 190 of 460

## Page 206

u-blox 8 / u-blox M8 Receiver description - Manual

32.10 UBX-CFG (0x06)

Configuration Input Messages: i.e. Configure the receiver.

Messages in the CFG class can be used to configure the receiver and poll current configuration

values. Any messages in the CFG class sent to the receiver are either acknowledged (with

message UBX-ACK-ACK) if processed successfully or rejected (with message UBX-ACK-NAK) if

processing unsuccessfully.

32.10.1 UBX-CFG-ANT (0x06 0x13)

32.10.1.1 Antenna control settings

Message UBX-CFG-ANT

Description Antenna control settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message allows the user to configure the antenna supervisor.

The antenna supervisor can be used to detect the status of an active antenna

and control it. It can be used to turn off the supply to the antenna in the event of

a short cirquit (for example) or to manage power consumption in power save

mode.

Refer to antenna supervisor configuration in the integration manual for more

information regarding the behavior of the antenna supervisor.

Refer to UBX-MON-HW for a description of the fields in the message used to

obtain the status of the antenna.

Note that not all pins can be used for antenna supervisor operation, the default

pins are recommended. Consult the integration manual if you need to use the

other pins.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x13 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X2 - flags - Antenna flag mask (see graphic below)

2 X2 - pins - Antenna pin configuration (see graphic

below)

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 191 of 460

## Page 207

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

svcs Enable antenna supply voltage control signal

scd Enable short circuit detection

ocd Enable open circuit detection

pdwnOnSCD Power down antenna supply if short circuit is detected. (only in combination with bit 1)

recovery Enable automatic recovery from short state

Bitfield pins

This graphic explains the bits of pins

Name Description

pinSwitch PIO-pin used for switching antenna supply

pinSCD PIO-pin used for detecting a short in the antenna supply

pinOCD PIO-pin used for detecting open/not connected antenna

reconfig if set to one, and this command is sent to the receiver, the receiver will reconfigure the pins as

specified.

32.10.2 UBX-CFG-BATCH (0x06 0x93)

32.10.2.1 Get/set data batching configuration

Message UBX-CFG-BATCH

Description Get/set data batching configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 23.01

Type Get/set

Comment Gets or sets the configuration for data batching.

See Data Batching for more information.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x93 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - flags - Flags (see graphic below)

2 U2 - bufSize - Size of buffer in number of epochs to store

4 U2 - notifThrs - Buffer fill level that triggers PIO

notification, in number of epochs stored

6 U1 - pioId - PIO ID to use for buffer level notification

7 U1 - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 192 of 460

## Page 208

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

enable Enable data batching

extraPvt Store extra PVT information

The fields iTOW, tAcc, numSV, hMSL, vAcc, velN, velE, velD, sAcc, headAcc and pDOP in UBX-LOG-

BATCH are only valid if this flag is set.

extraOdo Store odometer data

The fields distance, totalDistance and distanceStd in UBX-LOG-BATCH are only valid if this flag is

set.

Note: the odometer feature itself must also be enabled.

pioEnable Enable PIO notification

pioActiveLow PIO is active low

32.10.3 UBX-CFG-CFG (0x06 0x09)

32.10.3.1 Clear, save and load configurations

Message UBX-CFG-CFG

Description Clear, save and load configurations

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment See Receiver configuration for a detailed description on how receiver

configuration should be used. The three masks are made up of individual bits,

each bit indicating the sub-section of all configurations on which the

corresponding action shall be carried out. The reserved bits in the masks must

be set to '0'. For detailed information refer to the Organization of the

configuration sections. Note that commands can be combined. The sequence of

execution is clear, save, load.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x09 (12) or (13) see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X4 - clearMask - Mask with configuration sub-sections to

clear (i.e. load default configurations to

permanent configurations in non-volatile

memory) (see graphic below)

UBX-13003221 - R28 Early production information Page 193 of 460

## Page 209

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-CFG continued

Byte Offset Number Scaling Name Unit Description

Format

4 X4 - saveMask - Mask with configuration sub-sections to

save (i.e. save current configurations to

non-volatile memory), see ID description of

clearMask

8 X4 - loadMask - Mask with configuration sub-sections to

load (i.e. load permanent configurations

from non-volatile memory to current

configurations), see ID description of

clearMask

Start of optional block

12 X1 - deviceMask - Mask which selects the memory devices

for this command. (see graphic below)

End of optional block

Bitfield clearMask

This graphic explains the bits of clearMask

Name Description

ioPort Communications port settings. Modifying this sub-section results in an IO system reset. Because of

this undefined data may be output for a short period of time after receiving the message.

msgConf Message configuration

infMsg INF message configuration

navConf Navigation configuration

rxmConf Receiver Manager configuration

senConf Sensor interface configuration (not supported in protocol versions less than 19)

rinvConf Remote inventory configuration

antConf Antenna configuration

logConf Logging configuration

ftsConf FTS configuration. Only applicable to the FTS product variant.

UBX-13003221 - R28 Early production information Page 194 of 460

## Page 210

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield deviceMask

This graphic explains the bits of deviceMask

Name Description

devBBR Battery backed RAM

devFlash Flash

devEEPROM EEPROM

devSpiFlash SPI Flash

32.10.4 UBX-CFG-DAT (0x06 0x06)

32.10.4.1 Set user-defined datum

Message UBX-CFG-DAT

Description Set user-defined datum

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Set

Comment For more information see the description of Geodetic Systems and Frames.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x06 44 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 R8 - majA m Semi-major axis ( accepted range = 6,300,

000.0 to 6,500,000.0 meters ).

8 R8 - flat - 1.0 / flattening ( accepted range is 0.0 to

500.0 ).

16 R4 - dX m X axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

20 R4 - dY m Y axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

24 R4 - dZ m Z axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

28 R4 - rotX s Rotation about the X axis ( accepted range

is +/- 20.0 milli-arc seconds ).

32 R4 - rotY s Rotation about the Y axis ( accepted range

is +/- 20.0 milli-arc seconds ).

36 R4 - rotZ s Rotation about the Z axis ( accepted range

is +/- 20.0 milli-arc seconds ).

40 R4 - scale ppm Scale change ( accepted range is 0.0 to

50.0 parts per million ).

UBX-13003221 - R28 Early production information Page 195 of 460

## Page 211

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.4.2 Get currently defined datum

Message UBX-CFG-DAT

Description Get currently defined datum

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get

Comment Returns the parameters of the currently defined datum. If no user-defined

datum has been set, this will default to WGS84.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x06 52 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - datumNum - Datum number: 0 = WGS84, 0xFFFF =

user-defined

2 CH[6] - datumName - ASCII string: WGS84 or USER

8 R8 - majA m Semi-major axis ( accepted range = 6,300,

000.0 to 6,500,000.0 meters ).

16 R8 - flat - 1.0 / flattening ( accepted range is 0.0 to

500.0 ).

24 R4 - dX m X axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

28 R4 - dY m Y axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

32 R4 - dZ m Z axis shift at the origin ( accepted range

is +/- 5000.0 meters ).

36 R4 - rotX s Rotation about the X axis ( accepted range

is +/- 20.0 milli-arc seconds ).

40 R4 - rotY s Rotation about the Y axis ( accepted range

is +/- 20.0 milli-arc seconds ).

44 R4 - rotZ s Rotation about the Z axis ( accepted range

is +/- 20.0 milli-arc seconds ).

48 R4 - scale ppm Scale change ( accepted range is 0.0 to

50.0 parts per million ).

UBX-13003221 - R28 Early production information Page 196 of 460

## Page 212

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.5 UBX-CFG-DGNSS (0x06 0x70)

32.10.5.1 DGNSS configuration

Message UBX-CFG-DGNSS

Description DGNSS configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with High Precision GNSS

products)

Type Get/set

Comment This message allows the user to configure the DGNSS configuration of the

receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x70 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - dgnssMode - Specifies differential mode:

2: RTK float: No attempts are made to fix

ambiguities.

3: RTK fixed: Ambiguities are fixed

whenever possible.

1 U1[3] - reserved1 - Reserved

32.10.6 UBX-CFG-DOSC (0x06 0x61)

32.10.6.1 Disciplined oscillator configuration

Message UBX-CFG-DOSC

Description Disciplined oscillator configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Get/set

Comment This message allows the characteristics of the internal or external oscillator to

be described to the receiver.

The gainVco and gainUncertainty parameters are normally set using the

calibration process initiated using UBX-TIM-VCOCAL.

The behavior of the system can be badly affected by setting the wrong values,

so customers are advised to only change these parameters with care.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x61 4 + 32*numOsc see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - numOsc - Number of oscillators to configure (affects

length of this message)

UBX-13003221 - R28 Early production information Page 197 of 460

## Page 213

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-DOSC continued

Byte Offset Number Scaling Name Unit Description

Format

2 U1[2] - reserved1 - Reserved

Start of repeated block (numOsc times)

4 + 32*N U1 - oscId - Id of oscillator.

0 - internal oscillator

1 - external oscillator

5 + 32*N U1 - reserved2 - Reserved

6 + 32*N X2 - flags - flags (see graphic below)

8 + 32*N U4 2^-2 freq Hz Nominal frequency of source

12 + 32*N I4 - phaseOffset ps Intended phase offset of the oscillator

relative to the leading edge of the time

pulse

16 + 32*N U4 2^-8 withTemp ppb Oscillator stability limit over operating

temperature range (must be > 0)

20 + 32*N U4 2^-8 withAge ppb/ye Oscillator stability with age (must be > 0)

ar

24 + 32*N U2 - timeToTemp s The minimum time that it could take for a

temperature variation to move the

oscillator frequency by 'withTemp' (must

be > 0)

26 + 32*N U1[2] - reserved3 - Reserved

28 + 32*N I4 2^-16 gainVco ppb/ra Oscillator control gain/slope; change of

w LSB frequency per unit change in raw control

change

32 + 32*N U1 2^-8 gainUncertain - Relative uncertainty (1 standard deviation)

ty of oscillator control gain/slope

33 + 32*N U1[3] - reserved4 - Reserved

End of repeated block

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 198 of 460

## Page 214

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

isCalibrated 1 if the oscillator gain is calibrated, 0 if not

controlIf Communication interface for oscillator control:

0: Custom DAC attached to receiver's I2C

1: Microchip MCP4726 (12 bit DAC) attached to receiver's I2C

2: TI DAC8571 (16 bit DAC) attached to receiver's I2C

13: 12 bit DAC attached to host

14: 14 bit DAC attached to host

15: 16 bit DAC attached to host

Note that for DACs attached to the host, the host must monitor UBX-TIM-DOSC messages and pass

the supplied raw values on to the DAC.

32.10.7 UBX-CFG-ESFALG (0x06 0x56)

32.10.7.1 Get/set IMU-mount misalignment configuration

Message UBX-CFG-ESFALG

Description Get/set IMU-mount misalignment configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16 and 17 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Get/set

Comment Get/set the IMU-mount misalignment configuration (rotation from installation-

frame to the IMU-frame).

A detailed description on how to compose this configuration is given in the ADR

Installation section for ADR products.

A detailed description on how to compose this configuration is given in the UDR

Installation section for UDR products.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x56 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - bitfield - Bitfield (see graphic below)

4 U4 1e-2 yaw deg User-defined IMU-mount yaw angle [0,

36000], e.g. for 60.00 degree yaw angle

the configured value would be 6000

8 I2 1e-2 pitch deg User-defined IMU-mount pitch angle [-

9000, 9000], e.g. for 60.00 degree pitch

angle the configured value would be 6000

10 I2 1e-2 roll deg User-defined IMU-mount roll angle [-

18000, 18000], e.g. for 60.00 degree roll

angle the configured value would be 6000

UBX-13003221 - R28 Early production information Page 199 of 460

## Page 215

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield bitfield

This graphic explains the bits of bitfield

Name Description

version Message version (0x00 for this version)

doAutoMntAlg Only supported on certain products.

Enable/disable automatic IMU-mount alignment (0: Disabled, 1: Enabled). This flag can only be used

with modules containing an internal IMU.

32.10.8 UBX-CFG-ESFA (0x06 0x4C)

32.10.8.1 Get/set the Accelerometer (A) sensor configuration

Message UBX-CFG-ESFA

Description Get/set the Accelerometer (A) sensor configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with UDR products)

Type Get/set

Comment Get/set the configuration for the accelerometer sensor required for External

Sensor Fusion (ESF) based navigation. More details can be found in the

Accelerometer Configuration section.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x4C 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[9] - reserved1 - Reserved

10 U1 2^-6 accelRmsThdl m/s^2 Accelerometer RMS threshold below

which automatically estimated

accelerometer noise-level (accuracy) is

updated.

11 U1 - frequency Hz Nominal accelerometer sensor data

sampling frequency.

12 U2 - latency ms Accelerometer sensor data latency due to

e.g. CAN bus.

14 U2 1e-4 accuracy m/s^2 Accelerometer sensor data accuracy.

16 U1[4] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 200 of 460

## Page 216

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.9 UBX-CFG-ESFG (0x06 0x4D)

32.10.9.1 Get/set the Gyroscope (G) sensor configuration

Message UBX-CFG-ESFG

Description Get/set the Gyroscope (G) sensor configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with UDR products)

Type Get/set

Comment Get/set the configuration for the gyroscope sensor required for External Sensor

Fusion (ESF) based navigation. More details can be found in the Gyroscope

Configuration section.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x4D20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[7] - reserved1 - Reserved

8 U2 - tcTableSaveRa s Temperature-dependent gyroscope bias

te table saving update rate.

10 U1 2^-8 gyroRmsThdl deg/s Gyroscope sensor RMS threshold below

which automatically estimated gyroscope

noise-level (accuracy) is updated.

11 U1 - frequency Hz Nominal gyroscope sensor data sampling

frequency.

12 U2 - latency ms Gyroscope sensor data latency due to e.g.

CAN bus.

14 U2 1e-3 accuracy deg/s Gyroscope sensor data accuracy.

16 U1[4] - reserved2 - Reserved

32.10.10 UBX-CFG-ESFWT (0x06 0x82)

32.10.10.1 Get/set wheel-tick configuration

Message UBX-CFG-ESFWT

Description Get/set wheel-tick configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with ADR products)

Type Get/set

Comment Get/set the wheel-tick configuration for GWT or GAWT solution. Further

information on the configuration parameters is given in the Automotive Dead

Reckoning (ADR) chapter.

This field can only be used with modules supporting analog wheel-tick signals

and containing an internal IMU.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x82 32 see below CK_A CK_B

UBX-13003221 - R28 Early production information Page 201 of 460

## Page 217

u-blox 8 / u-blox M8 Receiver description - Manual

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - flags1 - Flags (see graphic below)

2 X1 - flags2 - Flags (see graphic below)

3 U1[1] - reserved1 - Reserved

4 U4 1e-6 wtFactor - Wheel-tick scale factor to obtain distance

[m] from wheel-ticks (0 = not set)

8 U4 1e-6 wtQuantError m (or Wheel-tick quantization. If useWtSpeed is

m/s) set then this is interpreted as the speed

measurement error RMS.

12 U4 - wtCountMax - Wheel-tick counter maximum value

(rollover - 1). If null, relative wheel-tick

counts are assumed (and therefore no

rollover). If not null, absolute wheel-tick

counts are assumed and the value

corresponds to the highest tick count

value before rollover happens. If

useWtSpeed is set then this value is

ignored.

If value is set to 1, absolute wheel-tick

counts are assumed and the value will be

automatic calculated if possible. It is only

possible for automatic calibration to

calculate wtCntMax if it can be

represented as a number of set bits (i.e.

2^N). If it cannot be represented in this

way it must be set to the correct absolute

tick value manually.

16 U2 - wtLatency ms Wheel-tick data latency due to e.g. CAN

bus

18 U1 - wtFrequency Hz Nominal wheel-tick data frequency (0 =

not set)

19 X1 - flags3 - Flags (see graphic below)

20 U2 - speedDeadBand cm/s Speed sensor dead band (0 = not set)

22 U1[10] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 202 of 460

## Page 218

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags1

This graphic explains the bits of flags1

Name Description

combineTicks Use combined rear wheel-ticks instead of the single tick

useWtSpeed Use speed measurements (data type 11 in ESF-MEAS) instead of single ticks (data type 10)

dirPinPol Only supported on certain products.

Direction pin polarity

0: High signal level means forward direction,

1: High signal level means backward direction.

useWtPin Use wheel-tick pin for speed measurement.

Bitfield flags2

This graphic explains the bits of flags2

Name Description

autoWtCountMa Disable automatic estimation of maximum absolute wheel-tick counter value (0: enabled, 1:

xOff disabled). See wtCountMax field description for more details.

(Not supported in protocol versions less than 19)

autoDirPinPol Only supported on certain products.

Off Disable automatic wheel-tick direction pin polarity detection (0: enabled, 1: disabled). See dirPinPol

field description for more details.

(Not supported in protocol versions less than 19)

autoSoftwareW Only supported on certain products.

tOff Disable automatic use of wheel-tick or speed data received over the software interface if available (0:

enabled, 1: disabled). In this case, data coming from the hardware interface (wheel-tick pins) will

automatically be ignored if wheel-tick/speed data are available from the software interface. See

useWtPin field description for more details.

(Not supported in protocol versions less than 19)

UBX-13003221 - R28 Early production information Page 203 of 460

## Page 219

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags2 Description continued

Name Description

autoUseWtSpee Disable automatic receiver reconfiguration for processing speed data instead of wheel-tick data if no

dOff wheel-tick data are available but speed data were detected (0: enabled, 1: disabled). See useWtSpeed

field description for more details.

(Not supported in protocol versions less than 19)

Bitfield flags3

This graphic explains the bits of flags3

Name Description

cntBothEdges Only supported on certain products.

Count both rising and falling edges on wheel-tick signal (only relevant if wheel-tick is measured by

the u-blox receiver).

Only turn on this feature if the wheel-tick signal has 50 % duty cycle. Turning on this feature with

fixed-width pulses can lead to severe degradation of performance.

Use wheel-tick pin for speed measurement. This field can only be used with modules supporting

analog wheel-tick signals.

32.10.11 UBX-CFG-ESRC (0x06 0x60)

32.10.11.1 External synchronization source configuration

Message UBX-CFG-ESRC

Description External synchronization source configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Get/set

Comment External time or frequency source configuration. The stability of time and

frequency sources is described using different fields, see sourceType field

documentation.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x60 4 + 36*numSources see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - numSources - Number of sources (affects length of this

message)

2 U1[2] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 204 of 460

## Page 220

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-ESRC continued

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (numSources times)

4 + 36*N U1 - extInt - EXTINT index of this source (0 for

EXTINT0 and 1 for EXTINT1)

5 + 36*N U1 - sourceType - Source type:

0: none

1: frequency source; use withTemp,

withAge, timeToTemp and

maxDevLifeTime to describe the stability

of the source

2: time source; use offset,

offsetUncertainty and jitter fields to

describe the stability of the source

3: feedback from external oscillator;

stability data is taken from the external

oscillator's configuration

6 + 36*N X2 - flags - Flags (see graphic below)

8 + 36*N U4 2^-2 freq Hz Nominal frequency of source

12 + 36*N U1[4] - reserved2 - Reserved

16 + 36*N U4 2^-8 withTemp ppb Oscillator stability limit over operating

temperature range (must be > 0)

Only used if sourceType is 1.

20 + 36*N U4 2^-8 withAge ppb/ye Oscillator stability with age (must be > 0)

ar Only used if sourceType is 1.

24 + 36*N U2 - timeToTemp s The minimum time that it could take for a

temperature variation to move the

oscillator frequency by 'withTemp' (must

be > 0)

Only used if sourceType is 1.

26 + 36*N U2 - maxDevLifeTim ppb Maximum frequency deviation during

e lifetime (must be > 0)

Only used if sourceType is 1.

28 + 36*N I4 - offset ns Phase offset of signal

Only used if sourceType is 2.

32 + 36*N U4 - offsetUncerta ns Uncertainty of phase offset (one standard

inty deviation)

Only used if sourceType is 2.

36 + 36*N U4 - jitter ns/s Phase jitter (must be > 0)

Only used if sourceType is 2.

End of repeated block

UBX-13003221 - R28 Early production information Page 205 of 460

## Page 221

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

polarity Polarity of signal:

0: leading edge is rising edge

1: leading edge is falling edge

gnssUtc Time base of timing signal:

0: GNSS - as specified in CFG-TP5 (or GPS if CFG-TP5 indicates UTC)

1: UTC

Only used if sourceType is 2.

32.10.12 UBX-CFG-GEOFENCE (0x06 0x69)

32.10.12.1 Geofencing configuration

Message UBX-CFG-GEOFENCE

Description Geofencing configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Gets or sets the geofencing configuration.

See the Geofencing description for feature details.

If the receiver is sent a valid new configuration, it will respond with a UBX-ACK-

ACK message and immediately change to the new configuration. Otherwise the

receiver will reject the request, by issuing a UBX-ACK-NAK and continuing

operation with the previous configuration.

Note that the acknowledge message does not indicate whether the PIO

configuration has been successfully applied (pin assigned), it only indicates the

successful configuration of the feature. The configured PIO must be previously

unoccupied for successful assignment.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x69 8 + 12*numFences see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - numFences - Number of geofences contained in this

message. Note that the receiver can only

store a limited number of geofences

(currently 4).

UBX-13003221 - R28 Early production information Page 206 of 460

## Page 222

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-GEOFENCE continued

Byte Offset Number Scaling Name Unit Description

Format

2 U1 - confLvl - Required confidence level for state

evaluation. This value times the position's

standard deviation (sigma) defines the

confidence band.

0 = no confidence required

1 = 68%

2 = 95%

3 = 99.7%

4 = 99.99%

3 U1[1] - reserved1 - Reserved

4 U1 - pioEnabled - 1 = Enable PIO combined fence state

output, 0 = disable

5 U1 - pinPolarity - PIO pin polarity. 0 = Low means inside, 1 =

Low means outside. Unknown state is

always high.

6 U1 - pin - PIO pin number

7 U1[1] - reserved2 - Reserved

Start of repeated block (numFences times)

8 + 12*N I4 1e-7 lat deg Latitude of the geofence circle center

12 + 12*N I4 1e-7 lon deg Longitude of the geofence circle center

16 + 12*N U4 1e-2 radius m Radius of the geofence circle

End of repeated block

32.10.13 UBX-CFG-GNSS (0x06 0x3E)

32.10.13.1 GNSS system configuration

Message UBX-CFG-GNSS

Description GNSS system configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Gets or sets the GNSS system channel sharing configuration.

If the receiver is sent a valid new configuration, it will respond with a UBX-ACK-

ACK message and immediately change to the new configuration. Otherwise the

receiver will reject the request, by issuing a UBX-ACK-NAK and continuing

operation with the previous configuration.

Configuration requirements:

• It is necessary for at least one major GNSS to be enabled, after applying the

new configuration to the current one.

• It is also required that at least 4 tracking channels are available to each

enabled major GNSS, i.e. maxTrkCh must have a minimum value of 4 for each

enabled major GNSS.

• The number of tracking channels in use must not exceed the number of

UBX-13003221 - R28 Early production information Page 207 of 460

## Page 223

u-blox 8 / u-blox M8 Receiver description - Manual

tracking channels available in hardware, and the sum of all reserved tracking

channels needs to be less than or equal to the number of tracking channels in

use.

Notes:

• To avoid cross-correlation issues, it is recommended that GPS and QZSS are

always both enabled or both disabled.

• Polling this message returns the configuration of all supported GNSS, whether

enabled or not; it may also include GNSS unsupported by the particular

product, but in such cases the enable flag will always be unset.

• See section GNSS Configuration for a discussion of the use of this message.

• See section Satellite Numbering for a description of the GNSS IDs available.

• Applying the GNSS system configuration takes some time. After issuing UBX-

CFG-GNSS, wait first for the acknowledgement from the receiver and then 0.5

seconds before sending the next command.

• If Galileo is enabled, UBX-CFG-GNSS must be followed by UBX-CFG-CFG to save

current configuration to BBR and then by UBX-CFG-RST with resetMode set to

Hardware reset.

• Configuration specific to the GNSS system can be done via other messages (e.

g. UBX-CFG-SBAS).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x3E 4 + 8*numConfigBlocks see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - msgVer - Message version (0x00 for this version)

1 U1 - numTrkChHw - Number of tracking channels available in

hardware (read only)

2 U1 - numTrkChUse - (Read only in protocol versions greater

than 23) Number of tracking channels to

use. Must be > 0, <= numTrkChHw. If

0xFF, then number of tracking channels to

use will be set to numTrkChHw.

3 U1 - numConfigBloc - Number of configuration blocks following

ks

Start of repeated block (numConfigBlocks times)

4 + 8*N U1 - gnssId - System identifier (see Satellite Numbering

)

5 + 8*N U1 - resTrkCh - (Read only in protocol versions greater

than 23) Number of reserved (minimum)

tracking channels for this system.

6 + 8*N U1 - maxTrkCh - (Read only in protocol versions greater

than 23) Maximum number of tracking

channels used for this system. Must be >

0, >= resTrkChn, <= numTrkChUse and <=

maximum number of tracking channels

supported for this system.

7 + 8*N U1 - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 208 of 460

## Page 224

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-GNSS continued

Byte Offset Number Scaling Name Unit Description

Format

8 + 8*N X4 - flags - Bitfield of flags. At least one signal must

be configured in every enabled system.

(see graphic below)

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

enable Enable this system

sigCfgMask Signal configuration mask

When gnssId is 0 (GPS)

0x01 = GPS L1C/A

0x10 = GPS L2C

0x20 = GPS L5

When gnssId is 1 (SBAS)

0x01 = SBAS L1C/A

When gnssId is 2 (Galileo)

0x01 = Galileo E1 (not supported in protocol versions less than 18)

0x10 = Galileo E5a

0x20 = Galileo E5b

When gnssId is 3 (BeiDou)

0x01 = BeiDou B1I

0x10 = BeiDou B2I

0x80 = BeiDou B2A

When gnssId is 5 (QZSS)

0x01 = QZSS L1C/A

0x04 = QZSS L1S

0x10 = QZSS L2C

0x20 = QZSS L5

When gnssId is 6 (GLONASS)

0x01 = GLONASS L1

0x10 = GLONASS L2

UBX-13003221 - R28 Early production information Page 209 of 460

## Page 225

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.14 UBX-CFG-HNR (0x06 0x5C)

32.10.14.1 High navigation rate settings

Message UBX-CFG-HNR

Description High navigation rate settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16 and 17 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Get/set

Comment The u-blox receivers support high rates of navigation update up to 30 Hz. The

navigation solution output UBX-NAV-HNR will not be aligned to the top of a

second.

• The update rate has a direct influence on the power consumption. The more

fixes that are required, the more CPU power and communication resources are

required.

• For most applications a 1 Hz update rate would be sufficient.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x5C 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - highNavRate Hz Rate of navigation solution output

1 U1[3] - reserved1 - Reserved

32.10.15 UBX-CFG-INF (0x06 0x02)

32.10.15.1 Poll configuration for one protocol

Message UBX-CFG-INF

Description Poll configuration for one protocol

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x02 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - protocolID - Protocol identifier, identifying the output

protocol for this poll request. The

following are valid protocol identifiers:

0: UBX protocol

1: NMEA protocol

2-255: Reserved

UBX-13003221 - R28 Early production information Page 210 of 460

## Page 226

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.15.2 Information message configuration

Message UBX-CFG-INF

Description Information message configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment The value of infMsgMask[x] below is formed so that each bit represents one of

the INF class messages (bit 0 for ERROR, bit 1 for WARNING and so on). For a

complete list, see the Message class INF. Several configurations can be

concatenated to one input message. In this case the payload length can be a

multiple of the normal length. Output messages from the module contain only

one configuration unit.

Note that:

• I/O ports 1 and 2 correspond to serial ports 1 and 2.

• I/O port 0 is I2C (DDC).

• I/O port 3 is USB.

• I/O port 4 is SPI.

• I/O port 5 is reserved for future use.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x02 0 + 10*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*10 U1 - protocolID - Protocol identifier, identifying for which

protocol the configuration is set/get. The

following are valid protocol identifiers:

0: UBX protocol

1: NMEA protocol

2-255: Reserved

1 + 10*N U1[3] - reserved1 - Reserved

4 + 10*N X1[6] - infMsgMask - A bit mask, saying which information

messages are enabled on each I/O port

(see graphic below)

End of repeated block

Bitfield infMsgMask

This graphic explains the bits of infMsgMask

UBX-13003221 - R28 Early production information Page 211 of 460

## Page 227

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

ERROR enable ERROR

WARNING enable WARNING

NOTICE enable NOTICE

TEST enable TEST

DEBUG enable DEBUG

32.10.16 UBX-CFG-ITFM (0x06 0x39)

32.10.16.1 Jamming/interference monitor configuration

Message UBX-CFG-ITFM

Description Jamming/interference monitor configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x39 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X4 - config - Interference config word (see graphic

below)

4 X4 - config2 - Extra settings for jamming/interference

monitor (see graphic below)

Bitfield config

This graphic explains the bits of config

UBX-13003221 - R28 Early production information Page 212 of 460

## Page 228

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

bbThreshold Broadband jamming detection threshold (unit = dB)

cwThreshold CW jamming detection threshold (unit = dB)

algorithmBits Reserved algorithm settings - should be set to 0x16B156 in hex for correct settings

enable Enable interference detection

Bitfield config2

This graphic explains the bits of config2

Name Description

generalBits General settings - should be set to 0x31E in hex for correct setting

antSetting Antenna setting, 0=unknown, 1=passive, 2=active

enable2 Set to 1 to scan auxiliary bands (u-blox 8 / u-blox M8 only, otherwise ignored)

32.10.17 UBX-CFG-LOGFILTER (0x06 0x47)

32.10.17.1 Data logger configuration

Message UBX-CFG-LOGFILTER

Description Data logger configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message can be used to configure the data logger, i.e. to enable/disable the

log recording and to get/set the position entry filter settings.

Position entries can be filtered based on time difference, position difference or

current speed thresholds. Position and speed filtering also have a minimum time

interval. A position is logged if any of the thresholds are exceeded. If a threshold

is set to zero it is ignored. The maximum rate of position logging is 1 Hz.

The filter settings will be configured to the provided values only if the

'applyAllFilterSettings' flag is set. This allows the recording to be

enabled/disabled independently of configuring the filter settings.

Configuring the data logger in the absence of a logging file is supported. By

doing so, once the logging file is created, the data logger configuration will take

effect immediately and logging recording and filtering will activate according to

the configuration.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x47 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

UBX-13003221 - R28 Early production information Page 213 of 460

## Page 229

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-LOGFILTER continued

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 X1 - flags - Flags (see graphic below)

2 U2 - minInterval s Minimum time interval between logged

positions (0 = not set). This is only applied

in combination with the speed and/or

position thresholds. If both minInterval

and timeThreshold are set, minInterval

must be less than or equal to

timeThreshold.

4 U2 - timeThreshold s If the time difference is greater than the

threshold, then the position is logged (0 =

not set).

6 U2 - speedThreshol m/s If the current speed is greater than the

d threshold, then the position is logged (0 =

not set). minInterval also applies.

8 U4 - positionThres m If the 3D position difference is greater

hold than the threshold, then the position is

logged (0 = not set). minInterval also

applies.

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 214 of 460

## Page 230

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

recordEnabled 1 = enable recording, 0 = disable recording

psmOncePerWak 1 = enable recording only one single position per PSM on/off mode wake-up period, 0 = disable once

upEnabled per wake-up

applyAllFilte 1 = apply all filter settings, 0 = only apply recordEnabled

rSettings

32.10.18 UBX-CFG-MSG (0x06 0x01)

32.10.18.1 Poll a message configuration

Message UBX-CFG-MSG

Description Poll a message configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x01 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - msgClass - Message class

1 U1 - msgID - Message identifier

32.10.18.2 Set message rate(s)

Message UBX-CFG-MSG

Description Set message rate(s)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Get/set message rate configuration (s) to/from the receiver.

See also section How to change between protocols.

• Send rate is relative to the event a message is registered on. For example, if

the rate of a navigation message is set to 2, the message is sent every second

navigation solution. For configuring NMEA messages, the section NMEA

Messages Overview describes class and identifier numbers used.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x01 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - msgClass - Message class

1 U1 - msgID - Message identifier

2 U1[6] - rate - Send rate on I/O port (6 ports)

UBX-13003221 - R28 Early production information Page 215 of 460

## Page 231

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.18.3 Set message rate

Message UBX-CFG-MSG

Description Set message rate

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Set message rate configuration for the current port.

See also section How to change between protocols.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x01 3 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - msgClass - Message class

1 U1 - msgID - Message identifier

2 U1 - rate - Send rate on current port

32.10.19 UBX-CFG-NAV5 (0x06 0x24)

32.10.19.1 Navigation engine settings

Message UBX-CFG-NAV5

Description Navigation engine settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment See the Navigation Configuration Settings Description for a detailed description

of how these settings affect receiver operation.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x24 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X2 - mask - Parameters bitmask. Only the masked

parameters will be applied. (see graphic

below)

UBX-13003221 - R28 Early production information Page 216 of 460

## Page 232

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NAV5 continued

Byte Offset Number Scaling Name Unit Description

Format

2 U1 - dynModel - Dynamic platform model:

0: portable

2: stationary

3: pedestrian

4: automotive

5: sea

6: airborne with <1g acceleration

7: airborne with <2g acceleration

8: airborne with <4g acceleration

9: wrist-worn watch (not supported in

protocol versions less than 18)

10: motorbike (supported in protocol

versions 19.2, and 35.10, and 35.15, and

35.20)

11: robotic lawn mower

12: electric kick scooter

3 U1 - fixMode - Position fixing mode:

1: 2D only

2: 3D only

3: auto 2D/3D

4 I4 0.01 fixedAlt m Fixed altitude (mean sea level) for 2D fix

mode

8 U4 0.0001 fixedAltVar m^2 Fixed altitude variance for 2D mode

12 I1 - minElev deg Minimum elevation for a GNSS satellite to

be used in NAV

13 U1 - drLimit s Reserved

14 U2 0.1 pDop - Position DOP mask to use

16 U2 0.1 tDop - Time DOP mask to use

18 U2 - pAcc m Position accuracy mask

20 U2 - tAcc m Time accuracy mask

22 U1 - staticHoldThr cm/s Static hold threshold

esh

23 U1 - dgnssTimeout s DGNSS timeout

24 U1 - cnoThreshNumS - Number of satellites required to have

Vs C/N0 above cnoThresh for a fix to be

attempted

25 U1 - cnoThresh dBHz C/N0 threshold for deciding whether to

attempt a fix

26 U1[2] - reserved1 - Reserved

28 U2 - staticHoldMax m Static hold distance threshold (before

Dist quitting static hold)

UBX-13003221 - R28 Early production information Page 217 of 460

## Page 233

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NAV5 continued

Byte Offset Number Scaling Name Unit Description

Format

30 U1 - utcStandard - UTC standard to be used (see GNSS time

bases):

0: Automatic; receiver selects based on

GNSS configuration

3: UTC as operated by the U.S. Naval

Observatory (USNO); derived from GPS

time

5: UTC as combined from multiple

European laboratories; derived from

Galileo time

6: UTC as operated by the former Soviet

Union (SU); derived from GLONASS time

7: UTC as operated by the National Time

Service Center (NTSC), China; derived

from BeiDou time

8: UTC as operated by the National

Physics Laboratory, India (NPLI); derived

from NavIC time

(not supported in protocol versions less

than 16).

31 U1[5] - reserved2 - Reserved

Bitfield mask

This graphic explains the bits of mask

Name Description

dyn Apply dynamic model settings

minEl Apply minimum elevation settings

posFixMode Apply fix mode settings

drLim Reserved

posMask Apply position mask settings

timeMask Apply time mask settings

staticHoldMas Apply static hold settings

k

dgpsMask Apply DGPS settings

cnoThreshold Apply CNO threshold settings (cnoThresh, cnoThreshNumSVs)

UBX-13003221 - R28 Early production information Page 218 of 460

## Page 234

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield mask Description continued

Name Description

utc Apply UTC settings

(not supported in protocol versions less than 16).

32.10.20 UBX-CFG-NAVX5 (0x06 0x23)

32.10.20.1 Navigation engine expert settings

Message UBX-CFG-NAVX5

Description Navigation engine expert settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16 and 17

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x23 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - version - Message version (0x0000 for this version)

2 X2 - mask1 - First parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

4 X4 - mask2 - Second parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

8 U1[2] - reserved1 - Reserved

10 U1 - minSVs #SVs Minimum number of satellites for

navigation

11 U1 - maxSVs #SVs Maximum number of satellites for

navigation

12 U1 - minCNO dBHz Minimum satellite signal level for

navigation

13 U1 - reserved2 - Reserved

14 U1 - iniFix3D - 1 = initial fix must be 3D

15 U1[2] - reserved3 - Reserved

17 U1 - ackAiding - 1 = issue acknowledgements for

assistance message input

18 U2 - wknRollover - GPS week rollover number; GPS week

numbers will be set correctly from this

week up to 1024 weeks after this week.

Setting this to 0 reverts to firmware

default.

20 U1[6] - reserved4 - Reserved

UBX-13003221 - R28 Early production information Page 219 of 460

## Page 235

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NAVX5 continued

Byte Offset Number Scaling Name Unit Description

Format

26 U1 - usePPP - 1 = use Precise Point Positioning (only

available with the PPP product variant)

27 U1 - aopCfg - AssistNow Autonomous configuration

(see graphic below)

28 U1[2] - reserved5 - Reserved

30 U2 - aopOrbMaxErr m Maximum acceptable (modeled)

AssistNow Autonomous orbit error (valid

range = 5..1000, or 0 = reset to firmware

default)

32 U1[4] - reserved6 - Reserved

36 U1[3] - reserved7 - Reserved

39 U1 - useAdr - Only supported on certain products

Enable/disable ADR sensor fusion (if 0:

sensor fusion is disabled - if 1: sensor

fusion is enabled).

Bitfield mask1

This graphic explains the bits of mask1

Name Description

minMax 1 = apply min/max SVs settings

minCno 1 = apply minimum C/N0 setting

initial3dfix 1 = apply initial 3D fix settings

wknRoll 1 = apply GPS weeknumber rollover settings

ackAid 1 = apply assistance acknowledgement settings

ppp 1 = apply usePPP flag

aop 1 = apply aopCfg (useAOP flag) and aopOrbMaxErr settings (AssistNow Autonomous)

Bitfield mask2

This graphic explains the bits of mask2

UBX-13003221 - R28 Early production information Page 220 of 460

## Page 236

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

adr Apply ADR sensor fusion on/off setting (useAdr flag)

Bitfield aopCfg

This graphic explains the bits of aopCfg

Name Description

useAOP 1 = enable AssistNow Autonomous

32.10.20.2 Navigation engine expert settings

Message UBX-CFG-NAVX5

Description Navigation engine expert settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Get/set

Comment (Polling will send back a version 3 message in protocol versions 19.2).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x23 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - version - Message version (0x0002 for this version)

2 X2 - mask1 - First parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

4 X4 - mask2 - Second parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

8 U1[2] - reserved1 - Reserved

10 U1 - minSVs #SVs Minimum number of satellites for

navigation

11 U1 - maxSVs #SVs Maximum number of satellites for

navigation

12 U1 - minCNO dBHz Minimum satellite signal level for

navigation

13 U1 - reserved2 - Reserved

14 U1 - iniFix3D - 1 = initial fix must be 3D

15 U1[2] - reserved3 - Reserved

UBX-13003221 - R28 Early production information Page 221 of 460

## Page 237

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NAVX5 continued

Byte Offset Number Scaling Name Unit Description

Format

17 U1 - ackAiding - 1 = issue acknowledgements for

assistance message input

18 U2 - wknRollover - GPS week rollover number; GPS week

numbers will be set correctly from this

week up to 1024 weeks after this week.

Setting this to 0 reverts to firmware

default.

20 U1 - sigAttenCompM dBHz Only supported on certain products

ode Permanently attenuated signal

compensation (0 = disabled, 255 =

automatic, 1..63 = maximum expected

C/N0 value)

21 U1 - reserved4 - Reserved

22 U1[2] - reserved5 - Reserved

24 U1[2] - reserved6 - Reserved

26 U1 - usePPP - 1 = use Precise Point Positioning (only

available with the PPP product variant)

27 U1 - aopCfg - AssistNow Autonomous configuration

(see graphic below)

28 U1[2] - reserved7 - Reserved

30 U2 - aopOrbMaxErr m Maximum acceptable (modeled)

AssistNow Autonomous orbit error (valid

range = 5..1000, or 0 = reset to firmware

default)

32 U1[4] - reserved8 - Reserved

36 U1[3] - reserved9 - Reserved

39 U1 - useAdr - Only supported on certain products

Enable/disable ADR/UDR sensor fusion (if

0: sensor fusion is disabled - if 1: sensor

fusion is enabled).

Bitfield mask1

This graphic explains the bits of mask1

UBX-13003221 - R28 Early production information Page 222 of 460

## Page 238

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

minMax 1 = apply min/max SVs settings

minCno 1 = apply minimum C/N0 setting

initial3dfix 1 = apply initial 3D fix settings

wknRoll 1 = apply GPS weeknumber rollover settings

ackAid 1 = apply assistance acknowledgement settings

ppp 1 = apply usePPP flag

aop 1 = apply aopCfg (useAOP flag) and aopOrbMaxErr settings (AssistNow Autonomous)

Bitfield mask2

This graphic explains the bits of mask2

Name Description

adr Apply ADR/UDR sensor fusion on/off setting (useAdr flag)

sigAttenComp Only supported on certain products

Apply signal attenuation compensation feature settings

Bitfield aopCfg

This graphic explains the bits of aopCfg

Name Description

useAOP 1 = enable AssistNow Autonomous

UBX-13003221 - R28 Early production information Page 223 of 460

## Page 239

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.20.3 Navigation engine expert settings

Message UBX-CFG-NAVX5

Description Navigation engine expert settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19.1 and 19.2

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x23 44 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - version - Message version (0x0003 for this version)

2 X2 - mask1 - First parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

4 X4 - mask2 - Second parameters bitmask. Only the

flagged parameters will be applied,

unused bits must be set to 0. (see graphic

below)

8 U1[2] - reserved1 - Reserved

10 U1 - minSVs #SVs Minimum number of satellites for

navigation

11 U1 - maxSVs #SVs Maximum number of satellites for

navigation

12 U1 - minCNO dBHz Minimum satellite signal level for

navigation

13 U1 - reserved2 - Reserved

14 U1 - iniFix3D - 1 = initial fix must be 3D

15 U1[2] - reserved3 - Reserved

17 U1 - ackAiding - 1 = issue acknowledgements for

assistance message input

18 U2 - wknRollover - GPS week rollover number; GPS week

numbers will be set correctly from this

week up to 1024 weeks after this week.

Setting this to 0 reverts to firmware

default.

20 U1 - sigAttenCompM dBHz Only supported on certain products

ode Permanently attenuated signal

compensation (0 = disabled, 255 =

automatic, 1..63 = maximum expected

C/N0 value)

21 U1 - reserved4 - Reserved

22 U1[2] - reserved5 - Reserved

24 U1[2] - reserved6 - Reserved

UBX-13003221 - R28 Early production information Page 224 of 460

## Page 240

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NAVX5 continued

Byte Offset Number Scaling Name Unit Description

Format

26 U1 - usePPP - 1 = use Precise Point Positioning (only

available with the PPP product variant)

27 U1 - aopCfg - AssistNow Autonomous configuration

(see graphic below)

28 U1[2] - reserved7 - Reserved

30 U2 - aopOrbMaxErr m Maximum acceptable (modeled)

AssistNow Autonomous orbit error (valid

range = 5..1000, or 0 = reset to firmware

default)

32 U1[4] - reserved8 - Reserved

36 U1[3] - reserved9 - Reserved

39 U1 - useAdr - Only supported on certain products

Enable/disable ADR/UDR sensor fusion (if

0: sensor fusion is disabled - if 1: sensor

fusion is enabled).

40 U1[2] - reserved10 - Reserved

42 U1[2] - reserved11 - Reserved

Bitfield mask1

This graphic explains the bits of mask1

Name Description

minMax 1 = apply min/max SVs settings

minCno 1 = apply minimum C/N0 setting

initial3dfix 1 = apply initial 3D fix settings

wknRoll 1 = apply GPS weeknumber rollover settings

ackAid 1 = apply assistance acknowledgement settings

ppp 1 = apply usePPP flag

aop 1 = apply aopCfg (useAOP flag) and aopOrbMaxErr settings (AssistNow Autonomous)

UBX-13003221 - R28 Early production information Page 225 of 460

## Page 241

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield mask2

This graphic explains the bits of mask2

Name Description

adr Apply ADR/UDR sensor fusion on/off setting (useAdr flag)

sigAttenComp Only supported on certain products

Apply signal attenuation compensation feature settings

Bitfield aopCfg

This graphic explains the bits of aopCfg

Name Description

useAOP 1 = enable AssistNow Autonomous

32.10.21 UBX-CFG-NMEA (0x06 0x17)

32.10.21.1 NMEA protocol configuration (deprecated)

Message UBX-CFG-NMEA

Description NMEA protocol configuration (deprecated)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message version is provided for backwards compatibility only. Use the

last version listed below instead (its fields are backwards compatible with this

version, it just has extra fields defined).

Get/set the NMEA protocol configuration. See section NMEA Protocol

Configuration for a detailed description of the configuration effects on NMEA

output.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x17 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - filter - filter flags (see graphic below)

UBX-13003221 - R28 Early production information Page 226 of 460

## Page 242

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NMEA continued

Byte Offset Number Scaling Name Unit Description

Format

1 U1 - nmeaVersion - 0x23: NMEA version 2.3

0x21: NMEA version 2.1

2 U1 - numSV - Maximum number of SVs to report per

TalkerId.

0: unlimited

8: 8 SVs

12: 12 SVs

16: 16 SVs

3 X1 - flags - flags (see graphic below)

Bitfield filter

This graphic explains the bits of filter

Name Description

posFilt Enable position output for failed or invalid fixes

mskPosFilt Enable position output for invalid fixes

timeFilt Enable time output for invalid times

dateFilt Enable date output for invalid dates

gpsOnlyFilter Restrict output to GPS satellites only

trackFilt Enable COG output even if COG is frozen

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 227 of 460

## Page 243

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

compat enable compatibility mode.

This might be needed for certain applications when customer's NMEA parser expects a fixed number

of digits in position coordinates.

consider enable considering mode.

32.10.21.2 NMEA protocol configuration V0 (deprecated)

Message UBX-CFG-NMEA

Description NMEA protocol configuration V0 (deprecated)

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message version is provided for backwards compatibility only. Use the

last version listed below instead (its fields are backwards compatible with this

version, it just has extra fields defined).

Get/set the NMEA protocol configuration. See section NMEA Protocol

Configuration for a detailed description of the configuration effects on NMEA

output.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x17 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - filter - filter flags (see graphic below)

1 U1 - nmeaVersion - 0x23: NMEA version 2.3

0x21: NMEA version 2.1

2 U1 - numSV - Maximum number of SVs to report per

TalkerId.

0: unlimited

8: 8 SVs

12: 12 SVs

16: 16 SVs

3 X1 - flags - flags (see graphic below)

4 X4 - gnssToFilter - Filters out satellites based on their GNSS.

If a bitfield is enabled, the corresponding

satellites will be not output. (see graphic

below)

8 U1 - svNumbering - Configures the display of satellites that do

not have an NMEA-defined value.

Note: this does not apply to satellites with

an unknown ID.

0: Strict - Satellites are not output

1: Extended - Use proprietary numbering

(see Satellite Numbering)

UBX-13003221 - R28 Early production information Page 228 of 460

## Page 244

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NMEA continued

Byte Offset Number Scaling Name Unit Description

Format

9 U1 - mainTalkerId - By default the main Talker ID (i.e. the

Talker ID used for all messages other than

GSV) is determined by the GNSS

assignment of the receiver's channels (see

UBX-CFG-GNSS).

This field enables the main Talker ID to be

overridden.

0: Main Talker ID is not overridden

1: Set main Talker ID to 'GP'

2: Set main Talker ID to 'GL'

3: Set main Talker ID to 'GN'

4: Set main Talker ID to 'GA'

5: Set main Talker ID to 'GB'

6: Set main Talker ID to 'GQ' (available in

NMEA 4.11 and later)

10 U1 - gsvTalkerId - By default the Talker ID for GSV messages

is GNSS-specific (as defined by NMEA).

This field enables the GSV Talker ID to be

overridden.

0: Use GNSS-specific Talker ID (as defined

by NMEA)

1: Use the main Talker ID

11 U1 - version - Message version (0x00 for this version)

Bitfield filter

This graphic explains the bits of filter

UBX-13003221 - R28 Early production information Page 229 of 460

## Page 245

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

posFilt Enable position output for failed or invalid fixes

mskPosFilt Enable position output for invalid fixes

timeFilt Enable time output for invalid times

dateFilt Enable date output for invalid dates

gpsOnlyFilter Restrict output to GPS satellites only

trackFilt Enable COG output even if COG is frozen

Bitfield flags

This graphic explains the bits of flags

Name Description

compat enable compatibility mode.

This might be needed for certain applications when customer's NMEA parser expects a fixed number

of digits in position coordinates.

consider enable considering mode.

Bitfield gnssToFilter

This graphic explains the bits of gnssToFilter

Name Description

gps Disable reporting of GPS satellites

sbas Disable reporting of SBAS satellites

galileo Disable reporting of Galileo satellites

qzss Disable reporting of QZSS satellites

glonass Disable reporting of GLONASS satellites

beidou Disable reporting of BeiDou satellites

UBX-13003221 - R28 Early production information Page 230 of 460

## Page 246

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.21.3 Extended NMEA protocol configuration V1

Message UBX-CFG-NMEA

Description Extended NMEA protocol configuration V1

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Get/set the NMEA protocol configuration. See section NMEA Protocol

Configuration for a detailed description of the configuration effects on NMEA

output.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x17 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - filter - filter flags (see graphic below)

1 U1 - nmeaVersion - 0x4b: NMEA version 4.11 (not available in

all products)

0x41: NMEA version 4.10 (not available in

all products)

0x40: NMEA version 4.0 (not available in

all products)

0x23: NMEA version 2.3

0x21: NMEA version 2.1

2 U1 - numSV - Maximum number of SVs to report per

TalkerId.

0: unlimited

8: 8 SVs

12: 12 SVs

16: 16 SVs

3 X1 - flags - flags (see graphic below)

4 X4 - gnssToFilter - Filters out satellites based on their GNSS.

If a bitfield is enabled, the corresponding

satellites will be not output. (see graphic

below)

8 U1 - svNumbering - Configures the display of satellites that do

not have an NMEA-defined value.

Note: this does not apply to satellites with

an unknown ID.

0: Strict - Satellites are not output

1: Extended - Use proprietary numbering

(see Satellite Numbering)

UBX-13003221 - R28 Early production information Page 231 of 460

## Page 247

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-NMEA continued

Byte Offset Number Scaling Name Unit Description

Format

9 U1 - mainTalkerId - By default the main Talker ID (i.e. the

Talker ID used for all messages other than

GSV) is determined by the GNSS

assignment of the receiver's channels (see

UBX-CFG-GNSS).

This field enables the main Talker ID to be

overridden.

0: Main Talker ID is not overridden

1: Set main Talker ID to 'GP'

2: Set main Talker ID to 'GL'

3: Set main Talker ID to 'GN'

4: Set main Talker ID to 'GA'

5: Set main Talker ID to 'GB'

6: Set main Talker ID to 'GQ' (available in

NMEA 4.11 and later)

10 U1 - gsvTalkerId - By default the Talker ID for GSV messages

is GNSS-specific (as defined by NMEA).

This field enables the GSV Talker ID to be

overridden.

0: Use GNSS-specific Talker ID (as defined

by NMEA)

1: Use the main Talker ID

11 U1 - version - Message version (0x01 for this version)

12 CH[2] - bdsTalkerId - Sets the two characters that should be

used for the BeiDou Talker ID. If these are

set to zero, then the default BeiDou Talker

ID will be used.

14 U1[6] - reserved1 - Reserved

Bitfield filter

This graphic explains the bits of filter

UBX-13003221 - R28 Early production information Page 232 of 460

## Page 248

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

posFilt Enable position output for failed or invalid fixes

mskPosFilt Enable position output for invalid fixes

timeFilt Enable time output for invalid times

dateFilt Enable date output for invalid dates

gpsOnlyFilter Restrict output to GPS satellites only

trackFilt Enable COG output even if COG is frozen

Bitfield flags

This graphic explains the bits of flags

Name Description

compat enable compatibility mode.

This might be needed for certain applications when customer's NMEA parser expects a fixed number

of digits in position coordinates.

consider enable considering mode.

limit82 enable strict limit to 82 characters maximum.

highPrec enable high precision mode.

This flag cannot be set in conjunction with either compatibility mode or Limit82 mode (not

supported in protocol versions less than 20.01).

Bitfield gnssToFilter

This graphic explains the bits of gnssToFilter

Name Description

gps Disable reporting of GPS satellites

sbas Disable reporting of SBAS satellites

galileo Disable reporting of Galileo satellites

qzss Disable reporting of QZSS satellites

glonass Disable reporting of GLONASS satellites

beidou Disable reporting of BeiDou satellites

UBX-13003221 - R28 Early production information Page 233 of 460

## Page 249

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.22 UBX-CFG-ODO (0x06 0x1E)

32.10.22.1 Odometer, low-speed COG engine settings

Message UBX-CFG-ODO

Description Odometer, low-speed COG engine settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This feature is not supported for the FTS product variant.

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x1E 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U1 - flags - Odometer/Low-speed COG filter flags (see

graphic below)

5 X1 - odoCfg - Odometer filter settings (see graphic

below)

6 U1[6] - reserved2 - Reserved

12 U1 1e-1 cogMaxSpeed m/s Speed below which course-over-ground

(COG) is computed with the low-speed

COG filter

13 U1 - cogMaxPosAcc m Maximum acceptable position accuracy

for computing COG with the low-speed

COG filter

14 U1[2] - reserved3 - Reserved

16 U1 - velLpGain - Velocity low-pass filter level, range 0..255

17 U1 - cogLpGain - COG low-pass filter level (at speed < 8

m/s), range 0..255

18 U1[2] - reserved4 - Reserved

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 234 of 460

## Page 250

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

useODO Odometer-enabled flag

useCOG Low-speed COG filter enabled flag

outLPVel Output low-pass filtered velocity flag

outLPCog Output low-pass filtered heading (COG) flag

Bitfield odoCfg

This graphic explains the bits of odoCfg

Name Description

profile Profile type (0=running, 1=cycling, 2=swimming, 3=car, 4=custom)

32.10.23 UBX-CFG-PM2 (0x06 0x3B)

32.10.23.1 Extended power management configuration

Message UBX-CFG-PM2

Description Extended power management configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This feature is not supported for either the ADR, FTS or HPG products.

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x3B 44 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1 - reserved1 - Reserved

2 U1 - maxStartupSta s Maximum time to spend in Acquisition

teDur state. If 0: bound disabled (see

maxStartupStateDur) (not supported in

protocol versions less than 17).

3 U1 - reserved2 - Reserved

4 X4 - flags - PSM configuration flags (see graphic

below)

8 U4 - updatePeriod ms Position update period. If set to 0, the

receiver will never retry a fix and it will wait

for external events

UBX-13003221 - R28 Early production information Page 235 of 460

## Page 251

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PM2 continued

Byte Offset Number Scaling Name Unit Description

Format

12 U4 - searchPeriod ms Acquisition retry period if previously failed.

If set to 0, the receiver will never retry a

startup

16 U4 - gridOffset ms Grid offset relative to GPS start of week

20 U2 - onTime s Time to stay in Tracking state

22 U2 - minAcqTime s minimal search time

24 U1[20] - reserved3 - Reserved

Bitfield flags

This graphic explains the bits of flags

Name Description

extintSel EXTINT pin select

0 EXTINT0

1 EXTINT1

extintWake EXTINT pin control

0 disabled

1 enabled, keep receiver awake as long as selected EXTINT pin is 'high'

extintBackup EXTINT pin control

0 disabled

1 enabled, force receiver into BACKUP mode when selected EXTINT pin is 'low'

limitPeakCurr Limit peak current

00 disabled

01 enabled, peak current is limited

10 reserved

11 reserved

waitTimeFix Wait for Timefix (see waitTimeFix)

0 wait for normal fix OK before starting on time

1 wait for time fix OK before starting on time

updateRTC Update Real Time Clock (see updateRTC)

0 do not wake up to update RTC. RTC is updated during normal on-time.

1 update RTC. The receiver adds extra wake-up cycles to update the RTC.

updateEPH Update Ephemeris (see updateEPH)

0 do not wake up to update Ephemeris data

1 update Ephemeris. The receiver adds extra wake-up cycles to update the Ephemeris data

UBX-13003221 - R28 Early production information Page 236 of 460

## Page 252

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags Description continued

Name Description

doNotEnterOff Behavior of receiver in case of no fix (see doNotEnterOff)

0 receiver enters Inactive) Awaiting next search state

1 receiver does not enter (Inactive) Awaiting next search state but keeps trying to acquire a fix

instead

mode Mode of operation (see mode)

00 ON/OFF operation (PSMOO)

01 cyclic tracking operation (PSMCT)

10 reserved

11 reserved

32.10.23.2 Extended power management configuration

Message UBX-CFG-PM2

Description Extended power management configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3

and 22

Type Get/set

Comment This feature is not supported for either the ADR, FTS or HPG products.

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x3B 48 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x02 for this version)

Note: the message version number is the

same as for protocol version 23.01; select

correct message version based on the

protocol version supported by your

firmware.

1 U1 - reserved1 - Reserved

2 U1 - maxStartupSta s Maximum time to spend in Acquisition

teDur state. If 0: bound disabled (see

maxStartupStateDur) (not supported in

protocol versions less than 17).

3 U1 - reserved2 - Reserved

4 X4 - flags - PSM configuration flags (see graphic

below)

8 U4 - updatePeriod ms Position update period. If set to 0, the

receiver will never retry a fix and it will wait

for external events

12 U4 - searchPeriod ms Acquisition retry period if previously failed.

If set to 0, the receiver will never retry a

startup

UBX-13003221 - R28 Early production information Page 237 of 460

## Page 253

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PM2 continued

Byte Offset Number Scaling Name Unit Description

Format

16 U4 - gridOffset ms Grid offset relative to GPS start of week

20 U2 - onTime s Time to stay in Tracking state

22 U2 - minAcqTime s minimal search time

24 U1[20] - reserved3 - Reserved

44 U4 - extintInactiv ms inactivity time out on EXTINT pin if

ityMs enabled

Bitfield flags

This graphic explains the bits of flags

Name Description

extintSel EXTINT pin select

0 EXTINT0

1 EXTINT1

extintWake EXTINT Pin Control

0 disabled

1 enabled, keep receiver awake as long as selected EXTINT pin is 'high'

extintBackup EXTINT Pin Control

0 disabled

1 enabled, force receiver into BACKUP mode when selected EXTINT pin is 'low'

extintInactiv EXTINT Pin Control

e 0 disabled

1 enabled, force backup in case EXTINT pin is inactive for time longer than extintIncactivityMs

limitPeakCurr Limit Peak Current

00 disabled

01 enabled, peak current is limited

10 reserved

11 reserved

waitTimeFix Wait for Timefix (see waitTimeFix)

0 wait for normal fix OK before starting on time

1 wait for time fix OK before starting on time

updateRTC Update Real Time Clock (see updateRTC)

0 do not wake up to update RTC. RTC is updated during normal on-time.

1 update RTC. The receiver adds extra wake-up cycles to update the RTC.

updateEPH Update Ephemeris (see updateEPH)

0 do not wake up to update Ephemeris data

1 update Ephemeris. The receiver adds extra wake-up cycles to update the Ephemeris data

UBX-13003221 - R28 Early production information Page 238 of 460

## Page 254

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags Description continued

Name Description

doNotEnterOff Behavior of receiver in case of no fix (see doNotEnterOff)

0 receiver enters (Inactive) Awaiting next search state

1 receiver does not enter (Inactive) Awaiting next search state but keeps trying to acquire a fix

instead

mode Mode of operation (see mode)

00 ON/OFF operation (PSMOO)

01 cyclic tracking operation (PSMCT)

10 reserved

11 reserved

32.10.23.3 Extended power management configuration

Message UBX-CFG-PM2

Description Extended power management configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 23.01

Type Get/set

Comment This feature is not supported for either the ADR, FTS or HPG products.

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x3B 48 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x02 for this version)

Note: the message version number is the

same as for protocol versions 18 up to 22;

select correct message version based on

the protocol version supported by your

firmware.

1 U1 - reserved1 - Reserved

2 U1 - maxStartupSta s Maximum time to spend in Acquisition

teDur state. If 0: bound disabled.

(see maxStartupStateDur) (not supported

in protocol versions 23 to 23.01).

3 U1 - reserved2 - Reserved

4 X4 - flags - PSM configuration flags (see graphic

below)

8 U4 - updatePeriod ms Position update period. If set to 0, the

receiver will never retry a fix and it will wait

for external events .

UBX-13003221 - R28 Early production information Page 239 of 460

## Page 255

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PM2 continued

Byte Offset Number Scaling Name Unit Description

Format

12 U4 - searchPeriod ms Acquisition retry period if previously failed.

If set to 0, the receiver will never retry a

startup.

(not supported in protocol versions 23 to

23.01).

16 U4 - gridOffset ms Grid offset relative to GPS start of week

(not supported in protocol versions 23 to

23.01).

20 U2 - onTime s Time to stay in Tracking state

(not supported in protocol versions 23 to

23.01).

22 U2 - minAcqTime s Minimal search time

24 U1[20] - reserved3 - Reserved

44 U4 - extintInactiv ms inactivity time out on EXTINT pin if

ityMs enabled

Bitfield flags

This graphic explains the bits of flags

Name Description

optTarget Optimization target

000 performance (default)

001 power save

010 reserved

011 reserved

100 reserved

101 reserved

110 reserved

111 reserved

extintSel EXTINT pin select

0 EXTINT0

1 EXTINT1

extintWake EXTINT pin control

0 disabled

1 enabled, keep receiver awake as long as selected EXTINT pin is 'high'

UBX-13003221 - R28 Early production information Page 240 of 460

## Page 256

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags Description continued

Name Description

extintBackup EXTINT pin control

0 disabled

1 enabled, force receiver into BACKUP mode when selected EXTINT pin is 'low'

extintInactiv EXTINT pin control

e 0 disabled

1 enabled, force backup in case EXTINT pin is inactive for time longer than extintIncactivityMs

limitPeakCurr Limit peak current

00 disabled

01 enabled, peak current is limited

10 reserved

11 reserved

waitTimeFix Wait for Timefix

(see waitTimeFix)

0 wait for normal fix OK before starting on time

1 wait for time fix OK before starting on time

(not supported in protocol versions 23 to 23.01).

updateRTC Update real time clock

(see updateRTC)

0 do not wake up to update RTC. RTC is updated during normal on-time.

1 update RTC. The receiver adds extra wake-up cycles to update the RTC.

(not supported in protocol versions 23 to 23.01, and 32+).

updateEPH Update ephemeris

(see updateEPH)

0 do not wake up to update Ephemeris data

1 update Ephemeris. The receiver adds extra wake-up cycles to update the Ephemeris data.

doNotEnterOff Behavior of receiver in case of no fix

Behavior of receiver in case of no fix (see doNotEnterOff)

0 receiver enters (Inactive) Awaiting next search state

1 receiver does not enter (Inactive) Awaiting next search state but keeps trying to acquire a fix

instead

(not supported in protocol versions 23 to 23.01).

mode Mode of operation

(see mode)

00 ON/OFF operation (PSMOO) (not supported in protocol versions 23 to 23.01)

01 cyclic tracking operation (PSMCT)

10 reserved

11 reserved

UBX-13003221 - R28 Early production information Page 241 of 460

## Page 257

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.24 UBX-CFG-PMS (0x06 0x86)

32.10.24.1 Power mode setup

Message UBX-CFG-PMS

Description Power mode setup

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 22, 22.01,

23 and 23.01

Type Get/set

Comment Using UBX-CFG-PMS to set Super-E mode to 1, 2 or 4 Hz navigation rates sets

minAcqTime to 180 s instead of the default 300 s in protocol version 23.01.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x86 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - powerSetupVal - Power setup value

ue 0x00 = Full power

0x01 = Balanced

0x02 = Interval

0x03 = Aggressive with 1 Hz

0x04 = Aggressive with 2 Hz

0x05 = Aggressive with 4 Hz

0xFF = Invalid (only when polling)

2 U2 - period s Position update period and search period.

Recommended minimum period is 10 s,

although the receiver accepts any value

bigger than 5 s.

Only valid when powerSetupValue set to

Interval, otherwise must be set to '0'.

4 U2 - onTime s Duration of the ON phase, must be smaller

than the period.

Only valid when powerSetupValue set to

Interval, otherwise must be set to '0'.

6 U1[2] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 242 of 460

## Page 258

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.25 UBX-CFG-PRT (0x06 0x00)

32.10.25.1 Polls the configuration for one I/O port

Message UBX-CFG-PRT

Description Polls the configuration for one I/O port

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Sending this message with a port ID as payload results in having the receiver

return the configuration for the specified port.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x00 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - PortID - Port identifier number (see the other

versions of CFG-PRT for valid values)

32.10.25.2 Port configuration for UART ports

Message UBX-CFG-PRT

Description Port configuration for UART ports

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Several configurations can be concatenated to one input message. In this case

the payload length can be a multiple of the normal length (see the other versions

of CFG-PRT). Output messages from the module contain only one configuration

unit.

Note that this message can affect baud rate and other transmission

parameters. Because there may be messages queued for transmission there

may be uncertainty about which protocol applies to such messages. In addition a

message currently in transmission may be corrupted by a protocol change. Host

data reception parameters may have to be changed to be able to receive future

messages, including the acknowledge message resulting from the CFG-PRT

message.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x00 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - portID - Port identifier number (see the integration

manual for valid UART port IDs)

1 U1 - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 243 of 460

## Page 259

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PRT continued

Byte Offset Number Scaling Name Unit Description

Format

2 X2 - txReady - TX ready PIN configuration (see graphic

below)

4 X4 - mode - A bit mask describing the UART mode

(see graphic below)

8 U4 - baudRate Bits/s Baud rate in bits/second

12 X2 - inProtoMask - A mask describing which input protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port. (see

graphic below)

14 X2 - outProtoMask - A mask describing which output protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port. (see

graphic below)

16 X2 - flags - Flags bit mask (see graphic below)

18 U1[2] - reserved2 - Reserved

Bitfield txReady

This graphic explains the bits of txReady

Name Description

en Enable TX ready feature for this port

pol Polarity

0 High-active

1 Low-active

pin PIO to be used (must not be in use by another function)

thres Threshold

The given threshold is multiplied by 8 bytes.

The TX ready PIN goes active after >= thres*8 bytes are pending for the port and going inactive after

the last pending bytes have been written to hardware (0-4 bytes before end of stream).

0x000 no threshold

0x001 8byte

0x002 16byte

...

0x1FE 4080byte

0x1FF 4088byte

UBX-13003221 - R28 Early production information Page 244 of 460

## Page 260

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield mode

This graphic explains the bits of mode

Name Description

charLen Character length

00 5bit (not supported)

01 6bit (not supported)

10 7bit (supported only with parity)

11 8bit

parity 000 Even parity

001 Odd parity

10X No parity

X1X Reserved

nStopBits Number of Stop bits

00 1 Stop bit

01 1.5 Stop bit

10 2 Stop bit

11 0.5 Stop bit

Bitfield inProtoMask

This graphic explains the bits of inProtoMask

Name Description

inUbx UBX protocol

inNmea NMEA protocol

inRtcm RTCM2 protocol

inRtcm3 RTCM3 protocol (not supported in protocol versions less than 20)

Bitfield outProtoMask

This graphic explains the bits of outProtoMask

UBX-13003221 - R28 Early production information Page 245 of 460

## Page 261

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

outUbx UBX protocol

outNmea NMEA protocol

outRtcm3 RTCM3 protocol (not supported in protocol versions less than 20)

Bitfield flags

This graphic explains the bits of flags

Name Description

extendedTxTim Extended TX timeout: if set, the port will time out if allocated TX memory >=4 kB and no activity for 1.

eout 5 s. If not set the port will time out if no activity for 1.5 s regardless on the amount of allocated TX

memory .

32.10.25.3 Port configuration for USB port

Message UBX-CFG-PRT

Description Port configuration for USB port

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Several configurations can be concatenated to one input message. In this case

the payload length can be a multiple of the normal length (see the other versions

of CFG-PRT). Output messages from the module contain only one configuration

unit.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x00 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - portID - Port identifier number (= 3 for USB port)

1 U1 - reserved1 - Reserved

2 X2 - txReady - TX ready PIN configuration (see graphic

below)

4 U1[8] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 246 of 460

## Page 262

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PRT continued

Byte Offset Number Scaling Name Unit Description

Format

12 X2 - inProtoMask - A mask describing which input protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port. (see

graphic below)

14 X2 - outProtoMask - A mask describing which output protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port. (see

graphic below)

16 U1[2] - reserved3 - Reserved

18 U1[2] - reserved4 - Reserved

Bitfield txReady

This graphic explains the bits of txReady

Name Description

en Enable TX ready feature for this port

pol Polarity

0 High-active

1 Low-active

pin PIO to be used (must not be in use by another function)

thres Threshold

The given threshold is multiplied by 8 bytes.

The TX ready PIN goes active after >= thres*8 bytes are pending for the port and going inactive after

the last pending bytes have been written to hardware (0-4 bytes before end of stream).

0x000 no threshold

0x001 8byte

0x002 16byte

...

0x1FE 4080byte

0x1FF 4088byte

UBX-13003221 - R28 Early production information Page 247 of 460

## Page 263

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield inProtoMask

This graphic explains the bits of inProtoMask

Name Description

inUbx UBX protocol

inNmea NMEA protocol

inRtcm RTCM2 protocol

inRtcm3 RTCM3 protocol (not supported in protocol versions less than 20)

Bitfield outProtoMask

This graphic explains the bits of outProtoMask

Name Description

outUbx UBX protocol

outNmea NMEA protocol

outRtcm3 RTCM3 protocol (not supported in protocol versions less than 20)

32.10.25.4 Port configuration for SPI port

Message UBX-CFG-PRT

Description Port configuration for SPI port

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Several configurations can be concatenated to one input message. In this case

the payload length can be a multiple of the normal length. Output messages

from the module contain only one configuration unit.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x00 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - portID - Port identifier number (= 4 for SPI port)

1 U1 - reserved1 - Reserved

2 X2 - txReady - TX ready PIN configuration (see graphic

below)

UBX-13003221 - R28 Early production information Page 248 of 460

## Page 264

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PRT continued

Byte Offset Number Scaling Name Unit Description

Format

4 X4 - mode - SPI Mode Flags (see graphic below)

8 U1[4] - reserved2 - Reserved

12 X2 - inProtoMask - A mask describing which input protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port.

(The bitfield inRtcm3 is not supported in

protocol versions less than 20) (see

graphic below)

14 X2 - outProtoMask - A mask describing which output protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port.

(The bitfield outRtcm3 is not supported in

protocol versions less than 20) (see

graphic below)

16 X2 - flags - Flags bit mask (see graphic below)

18 U1[2] - reserved3 - Reserved

Bitfield txReady

This graphic explains the bits of txReady

Name Description

en Enable TX ready feature for this port

pol Polarity

0 High-active

1 Low-active

pin PIO to be used (must not be in use by another function)

thres Threshold

The given threshold is multiplied by 8 bytes.

The TX ready PIN goes active after >= thres*8 bytes are pending for the port and going inactive after

the last pending bytes have been written to hardware (0-4 bytes before end of stream).

0x000 no threshold

0x001 8byte

0x002 16byte

...

0x1FE 4080byte

0x1FF 4088byte

UBX-13003221 - R28 Early production information Page 249 of 460

## Page 265

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield mode

This graphic explains the bits of mode

Name Description

spiMode 00 SPI Mode 0: CPOL = 0, CPHA = 0

01 SPI Mode 1: CPOL = 0, CPHA = 1

10 SPI Mode 2: CPOL = 1, CPHA = 0

11 SPI Mode 3: CPOL = 1, CPHA = 1

ffCnt Number of bytes containing 0xFF to receive before switching off reception. Range: 0 (mechanism

off) - 63

Bitfield inProtoMask

This graphic explains the bits of inProtoMask

Bitfield outProtoMask

This graphic explains the bits of outProtoMask

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 250 of 460

## Page 266

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

extendedTxTim Extended TX timeout: if set, the port will time out if allocated TX memory >=4 kB and no activity for 1.

eout 5 s.

32.10.25.5 Port configuration for I2C (DDC) port

Message UBX-CFG-PRT

Description Port configuration for I2C (DDC) port

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment Several configurations can be concatenated to one input message. In this case

the payload length can be a multiple of the normal length (see the other versions

of CFG-PRT). Output messages from the module contain only one configuration

unit.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x00 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - portID - Port identifier number (= 0 for I2C (DDC)

port)

1 U1 - reserved1 - Reserved

2 X2 - txReady - TX ready PIN configuration (see graphic

below)

4 X4 - mode - I2C (DDC) Mode Flags (see graphic below)

8 U1[4] - reserved2 - Reserved

12 X2 - inProtoMask - A mask describing which input protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port.

(The bitfield inRtcm3 is not supported in

protocol versions less than 20) (see

graphic below)

14 X2 - outProtoMask - A mask describing which output protocols

are active.

Each bit of this mask is used for a

protocol. Through that, multiple protocols

can be defined on a single port.

(The bitfield outRtcm3 is not supported in

protocol versions less than 20) (see

graphic below)

16 X2 - flags - Flags bit mask (see graphic below)

18 U1[2] - reserved3 - Reserved

UBX-13003221 - R28 Early production information Page 251 of 460

## Page 267

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield txReady

This graphic explains the bits of txReady

Name Description

en Enable TX ready feature for this port

pol Polarity

0 High-active

1 Low-active

pin PIO to be used (must not be in use by another function)

thres Threshold

The given threshold is multiplied by 8 bytes.

The TX ready PIN goes active after >= thres*8 bytes are pending for the port and going inactive after

the last pending bytes have been written to hardware (0-4 bytes before end of stream).

0x000 no threshold

0x001 8byte

0x002 16byte

...

0x1FE 4080byte

0x1FF 4088byte

Bitfield mode

This graphic explains the bits of mode

Name Description

slaveAddr Slave address

Range: 0x07 < slaveAddr < 0x78. Bit 0 must be 0

Bitfield inProtoMask

This graphic explains the bits of inProtoMask

UBX-13003221 - R28 Early production information Page 252 of 460

## Page 268

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield outProtoMask

This graphic explains the bits of outProtoMask

Bitfield flags

This graphic explains the bits of flags

Name Description

extendedTxTim Extended TX timeout: if set, the port will time out if allocated TX memory >=4 kB and no activity for 1.

eout 5 s .

32.10.26 UBX-CFG-PWR (0x06 0x57)

32.10.26.1 Put receiver in a defined power state

Message UBX-CFG-PWR

Description Put receiver in a defined power state

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Set

Comment This message is deprecated in protocol versions greater than 17. Use UBX-CFG-

RST for GNSS start/stop and UBX-RXM-PMREQ for software backup.

-

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x57 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1[3] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 253 of 460

## Page 269

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-PWR continued

Byte Offset Number Scaling Name Unit Description

Format

4 U4 - state - Enter system state

0x52554E20: GNSS running

0x53544F50: GNSS stopped

0x42434B50: Software backup. USB

interface will be disabled, other wakeup

source is needed.

32.10.27 UBX-CFG-RATE (0x06 0x08)

32.10.27.1 Navigation/measurement rate settings

Message UBX-CFG-RATE

Description Navigation/measurement rate settings

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message allows the user to alter the rate at which navigation solutions (and

the measurements that they depend on) are generated by the receiver. The

calculation of the navigation solution will always be aligned to the top of a

second zero (first second of the week) of the configured reference time system.

(Navigation period is an integer multiple of the measurement period in protocol

versions greater than 17).

• Each measurement triggers the measurements generation and, if available,

raw data output.

• The navRate value defines that every nth measurement triggers a navigation

epoch.

• The update rate has a direct influence on the power consumption. The more

fixes that are required, the more CPU power and communication resources are

required.

• For most applications a 1 Hz update rate would be sufficient.

• When using power save mode, measurement and navigation rate can differ

from the values configured here.

• See Measurement and navigation rate with power save mode for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x08 6 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

UBX-13003221 - R28 Early production information Page 254 of 460

## Page 270

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-RATE continued

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - measRate ms The elapsed time between GNSS

measurements, which defines the rate, e.

g. 100 ms => 10 Hz, 1000 ms => 1 Hz,

10000 ms => 0.1 Hz. Measurement rate

should be greater than or equal to 25 ms.

(Measurement rate should be greater

than or equal to 50 ms in protocol versions

less than 24).

2 U2 - navRate cycles The ratio between the number of

measurements and the number of

navigation solutions, e.g. 5 means five

measurements for every navigation

solution. Maximum value is 127. (This

parameter is ignored and the navRate is

fixed to 1 in protocol versions less than 18).

4 U2 - timeRef - The time system to which measurements

are aligned:

0: UTC time

1: GPS time

2: GLONASS time (not supported in

protocol versions less than 18)

3: BeiDou time (not supported in protocol

versions less than 18)

4: Galileo time (not supported in protocol

versions less than 18)

5: NavIC time (not supported in protocol

versions less than 29)

32.10.28 UBX-CFG-RINV (0x06 0x34)

32.10.28.1 Contents of remote inventory

Message UBX-CFG-RINV

Description Contents of remote inventory

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment If N is greater than 30, the excess bytes are discarded.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x34 1 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - flags - Flags (see graphic below)

UBX-13003221 - R28 Early production information Page 255 of 460

## Page 271

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-RINV continued

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

1 + 1*N U1 - data - Data to store/stored in remote inventory.

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

dump Dump data at startup. Does not work if flag binary is set.

binary Data is binary.

32.10.29 UBX-CFG-RST (0x06 0x04)

32.10.29.1 Reset receiver / Clear backup data structures

Message UBX-CFG-RST

Description Reset receiver / Clear backup data structures

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment Do not expect this message to be acknowledged by the receiver.

• Newer FW version will not acknowledge this message at all.

• Older FW version will acknowledge this message but the acknowledge may not

be sent completely before the receiver is reset.

Notes:

• If Galileo is enabled, UBX-CFG-RST Controlled GNSS start must be followed by

UBX-CFG-CFG to save current configuration to BBR and then by UBX-CFG-RST

with resetMode set to Hardware reset.

• If Galileo is enabled, use resetMode Hardware reset instead of Controlled

software reset or Controlled software reset (GNSS only).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x04 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

UBX-13003221 - R28 Early production information Page 256 of 460

## Page 272

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-RST continued

Byte Offset Number Scaling Name Unit Description

Format

0 X2 - navBbrMask - BBR sections to clear. The following

special sets apply:

0x0000 Hot start

0x0001 Warm start

0xFFFF Cold start (see graphic below)

2 U1 - resetMode - Reset Type

0x00 = Hardware reset (watchdog)

immediately

0x01 = Controlled software reset

0x02 = Controlled software reset (GNSS

only)

0x04 = Hardware reset (watchdog) after

shutdown

0x08 = Controlled GNSS stop

0x09 = Controlled GNSS start

3 U1 - reserved1 - Reserved

Bitfield navBbrMask

This graphic explains the bits of navBbrMask

Name Description

eph Ephemeris

alm Almanac

health Health

klob Klobuchar parameters

pos Position

clkd Clock drift

osc Oscillator parameter

utc UTC correction + GPS leap seconds parameters

rtc RTC

sfdr SFDR Parameters (only available on the ADR/UDR/HPS product variant) and weak signal

compensation estimates

vmon SFDR Vehicle Monitoring Parameter (only available on the ADR/UDR/HPS product variant)

tct TCT Parameters (only available on the ADR/UDR/HPS product variant)

aop Autonomous orbit parameters

UBX-13003221 - R28 Early production information Page 257 of 460

## Page 273

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.30 UBX-CFG-RXM (0x06 0x11)

32.10.30.1 RXM configuration

Message UBX-CFG-RXM

Description RXM configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16 and 17

Type Get/set

Comment For a detailed description see section Power management in the integration

manual

Note that Power save mode cannot be selected when the receiver is configured

to process GLONASS signals (using UBX-CFG-GNSS).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x11 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - reserved1 - Reserved

1 U1 - lpMode - Low power mode

0: Continuous mode

1: Power save mode

4: Continuous mode

Note that for receivers with protocol

versions larger or equal to 14, both Low

power mode settings 0 and 4 configure

the receiver to Continuous mode.

32.10.30.2 RXM configuration

Message UBX-CFG-RXM

Description RXM configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Get/set

Comment For a detailed description see section Power Management.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x11 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - reserved1 - Reserved

1 U1 - lpMode - Low power mode

0: Continuous mode

1: Power save mode

4: Continuous mode

UBX-13003221 - R28 Early production information Page 258 of 460

## Page 274

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.31 UBX-CFG-SBAS (0x06 0x16)

32.10.31.1 SBAS configuration

Message UBX-CFG-SBAS

Description SBAS configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment This message configures the SBAS receiver subsystem (i.e. WAAS, EGNOS,

MSAS).

See SBAS configuration settings description for a detailed description of how

these settings affect receiver operation.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x16 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - mode - SBAS mode (see graphic below)

1 X1 - usage - SBAS usage (see graphic below)

2 U1 - maxSBAS - Maximum number of SBAS prioritized

tracking channels (valid range: 0 - 3) to

use (obsolete and superseded by UBX-

CFG-GNSS in protocol versions 14+).

3 X1 - scanmode2 - Continuation of scanmode bitmask below

(see graphic below)

4 X4 - scanmode1 - Which SBAS PRN numbers to search for

(bitmask).

If all bits are set to zero, auto-scan (i.e. all

valid PRNs) are searched.

Every bit corresponds to a PRN number.

(see graphic below)

Bitfield mode

This graphic explains the bits of mode

UBX-13003221 - R28 Early production information Page 259 of 460

## Page 275

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

enabled SBAS enabled (1) / disabled (0) - This field is deprecated; use UBX-CFG-GNSS to enable/disable SBAS

operation

test SBAS testbed: Use data anyhow (1) / Ignore data when in test mode (SBAS msg 0)

Bitfield usage

This graphic explains the bits of usage

Name Description

range Use SBAS GEOs as a ranging source (for navigation)

diffCorr Use SBAS differential corrections

integrity Use SBAS integrity information. If enabled, the receiver will only use GPS satellites for which

integrity information is available.

Bitfield scanmode2

This graphic explains the bits of scanmode2

Bitfield scanmode1

This graphic explains the bits of scanmode1

UBX-13003221 - R28 Early production information Page 260 of 460

## Page 276

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.32 UBX-CFG-SENIF (0x06 0x88)

32.10.32.1 I2C sensor interface configuration

Message UBX-CFG-SENIF

Description I2C sensor interface configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x88 6 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Type of interface, 0 for I2C

1 U1 - version - Message version, 0 for this message

2 X2 - flags - feature configuration flags (see graphic

below)

4 X2 - pioConf - PIO configuration flags (see graphic below

)

Bitfield flags

This graphic explains the bits of flags

Name Description

senConn Sensor is connected to I2C interface

Bitfield pioConf

This graphic explains the bits of pioConf

UBX-13003221 - R28 Early production information Page 261 of 460

## Page 277

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

i2cSdaPio PIO of the I2C SDA line

Supported options:

i2cSclPio PIO of the I2C SCL line

Supported options:

32.10.33 UBX-CFG-SLAS (0x06 0x8D)

32.10.33.1 SLAS configuration

Message UBX-CFG-SLAS

Description SLAS configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 19.2 (only with ADR or UDR products

)

Type Get/set

Comment This message configures the QZSS SLAS (Sub-meter Level Augmentation

System). See the SLAS Configuration Settings Description for a detailed

description of how these settings affect receiver operation.

To apply SLAS corrections, QZSS operation and L1S signal tracking must be

enabled see UBX-CFG-GNSS

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x8D4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - mode - SLAS Mode (see graphic below)

1 U1[3] - reserved1 - Reserved

Bitfield mode

This graphic explains the bits of mode

Name Description

enabled Apply QZSS SLAS DGNSS corrections: Enabled (1) / Disabled (0)

test Use QZSS SLAS data when in test mode (SLAS msg 0): Use data anyhow (1) / Ignore data when in

Test Mode (0)

raim Raim out measurements that are not corrected by QZSS SLAS, if at least 5 measurements are

corrected: Enabled (1) / Disabled (0)

UBX-13003221 - R28 Early production information Page 262 of 460

## Page 278

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.34 UBX-CFG-SMGR (0x06 0x62)

32.10.34.1 Synchronization manager configuration

Message UBX-CFG-SMGR

Description Synchronization manager configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x62 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - minGNSSFix - Minimum number of GNSS fixes before we

commit to use it as a source

2 U2 - maxFreqChange ppb/s Maximum frequency change rate during

Rate disciplining. Must not exceed 30ppb/s

4 U2 - maxPhaseCorrR ns/s Maximum phase correction rate in

ate coherent time pulse mode.

For maximum phase correction rate in

corrective time pulse mode see

maxSlewRate.

Note that in coherent time pulse mode

phase correction is achieved by intentional

frequency offset. Allowing for a high

phase correction rate can result in large

intentional frequency offset. Must not

exceed 100ns/s

6 U1[2] - reserved1 - Reserved

8 U2 - freqTolerance ppb Limit of possible deviation from nominal

before UBX-TIM-TOS indicates that

frequency is out of tolerance

10 U2 - timeTolerance ns Limit of possible deviation from nominal

before UBX-TIM-TOS indicates that time

pulse is out of tolerance

12 X2 - messageCfg - Sync manager message configuration

(see graphic below)

UBX-13003221 - R28 Early production information Page 263 of 460

## Page 279

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-SMGR continued

Byte Offset Number Scaling Name Unit Description

Format

14 U2 - maxSlewRate us/s Maximum slew rate, the maximum time

correction that shall be applied between

locked pulses in corrective time pulse

mode.

To have no limit on the slew rate, set the

flag disableMaxSlewRate to 1

For maximum phase correction rate in

coherent time pulse mode see

maxPhaseCorrRate.

16 X4 - flags - Flags (see graphic below)

Bitfield messageCfg

This graphic explains the bits of messageCfg

Name Description

measInternal 1 = report the estimated offset of the internal oscillator based on the oscillator model

measGNSS 1 = report the internal oscillator's offset relative to GNSS

measEXTINT0 1 = report the internal oscillator's offset relative to the source on EXTINT0

measEXTINT1 1 = report the internal oscillator's offset relative to the source on EXTINT1

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 264 of 460

## Page 280

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

disableIntern 1 = disable disciplining of the internal oscillator

al

disableExtern 1 = disable disciplining of the external oscillator

al

preferenceMod Reference selection preference

e 0 - best frequency accuracy

1 - best phase accuracy

enableGNSS 1 = enable use of GNSS as synchronization source

enableEXTINT0 1 = enable use of EXTINT0 as synchronization source

enableEXTINT1 1 = enable use of EXTINT1 as synchronization source

enableHostMea 1 = enable use of host measurements on the internal oscillator as synchronization source

sInt Measurements made by the host must be sent to the receiver using a UBX-TIM-SMEAS-DATA0

message.

enableHostMea 1 = enable use of host measurements on the external oscillator as synchronization source

sExt Measurements made by the host must be sent to the receiver using a UBX-TIM-SMEAS-DATA0

message.

useAnyFix 0 - use over-determined navigation solutions only

1 - use any fix

disableMaxSle 0 - use the value in the field maxSlewRate for maximum time correction in corrective time pulse

wRate mode

1 - don't use the value in the field maxSlewRate

issueFreqWarn 1 = issue a warning (via UBX-TIM-TOS flag) when frequency uncertainty exceeds freqTolerance

ing

issueTimeWarn 1 = issue a warning (via UBX-TIM-TOS flag) when time uncertainty exceeds timeTolerance

ing

TPCoherent Control time pulse coherency

0 - Coherent pulses. Time phase offsets will be corrected gradually by varying the GNSS oscillator

rate within frequency tolerance limits. There will always be the correct number of GNSS oscillator

cycles between time pulses. Given tight limits this may take a long time

1 - Non-coherent pulses. In this mode the receiver will correct time phase offsets as quickly as

allowed by the specified maximum slew rate, in which case there may not be the expected number of

GNSS oscillator cycles between time pulses.

2 - Post-initialization coherent pulses. The receiver will run in non-coherent mode as described above

until the pulse timing has been corrected and PLL is active on the internal oscillator, but will then

switch to coherent pulse mode.

disableOffset 1 = disable automatic storage of oscillator offset

UBX-13003221 - R28 Early production information Page 265 of 460

## Page 281

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.35 UBX-CFG-SPT (0x06 0x64)

32.10.35.1 Configure and start a sensor production test

Message UBX-CFG-SPT

Description Configure and start a sensor production test

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with UDR products)

Type Get/set

Comment The production test uses the built-in self-test capabilities of an attached sensor.

This message is only supported if a sensor is directly connected to the u-blox

receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x64 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - reserved1 - Reserved

2 U2 - sensorId - ID of the sensor to be tested; see UBX-

MON-SPT for defined IDs

4 U1[8] - reserved2 - Reserved

32.10.36 UBX-CFG-TMODE2 (0x06 0x3D)

32.10.36.1 Time mode settings 2

Message UBX-CFG-TMODE2

Description Time mode settings 2

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync or

Time Sync products)

Type Get/set

Comment This message is available only for timing receivers

See the Time Mode Description for details. This message replaces the

deprecated UBX-CFG-TMODE message.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x3D28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

UBX-13003221 - R28 Early production information Page 266 of 460

## Page 282

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-TMODE2 continued

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - timeMode - Time Transfer Mode:

0 Disabled

1 Survey In

2 Fixed Mode (true position information

required)

3-255 Reserved

1 U1 - reserved1 - Reserved

2 X2 - flags - Time mode flags (see graphic below)

4 I4 - ecefXOrLat cm_ WGS84 ECEF X coordinate or latitude,

or_ depending on flags above

deg*1e-

7

8 I4 - ecefYOrLon cm_ WGS84 ECEF Y coordinate or longitude,

or_ depending on flags above

deg*1e-

7

12 I4 - ecefZOrAlt cm WGS84 ECEF Z coordinate or altitude,

depending on flags above

16 U4 - fixedPosAcc mm Fixed position 3D accuracy

20 U4 - svinMinDur s Survey-in minimum duration

24 U4 - svinAccLimit mm Survey-in position accuracy limit

Bitfield flags

This graphic explains the bits of flags

Name Description

lla Position is given in LAT/LON/ALT (default is ECEF)

altInv Altitude is not valid, in case lla was set

UBX-13003221 - R28 Early production information Page 267 of 460

## Page 283

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.37 UBX-CFG-TMODE3 (0x06 0x71)

32.10.37.1 Time mode settings 3

Message UBX-CFG-TMODE3

Description Time mode settings 3

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20, 20.01, 20.1, 20.2 and 20.3 (only with

High Precision GNSS products)

Type Get/set

Comment Configures the receiver to be in Time Mode. The position referred to in this

message is that of the Antenna Reference Point (ARP).

See the Time Mode Description for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x71 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - reserved1 - Reserved

2 X2 - flags - Receiver mode flags (see graphic below)

4 I4 - ecefXOrLat cm_ WGS84 ECEF X coordinate (or latitude) of

or_ the ARP position, depending on flags

deg*1e- above

7

8 I4 - ecefYOrLon cm_ WGS84 ECEF Y coordinate (or longitude)

or_ of the ARP position, depending on flags

deg*1e- above

7

12 I4 - ecefZOrAlt cm WGS84 ECEF Z coordinate (or altitude) of

the ARP position, depending on flags

above

16 I1 - ecefXOrLatHP 0.1_ High-precision WGS84 ECEF X coordinate

mm_ (or latitude) of the ARP position,

or_ depending on flags above. Must be in the

deg*1e- range -99..+99.

9 The precise WGS84 ECEF X coordinate in

units of cm, or the precise WGS84 ECEF

latitude in units of 1e-7 degrees, is given by

ecefXOrLat + (ecefXOrLatHP * 1e-2)

UBX-13003221 - R28 Early production information Page 268 of 460

## Page 284

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-TMODE3 continued

Byte Offset Number Scaling Name Unit Description

Format

17 I1 - ecefYOrLonHP 0.1_ High-precision WGS84 ECEF Y coordinate

mm_ (or longitude) of the ARP position,

or_ depending on flags above. Must be in the

deg*1e- range -99..+99.

9 The precise WGS84 ECEF Y coordinate in

units of cm, or the precise WGS84 ECEF

longitude in units of 1e-7 degrees, is given

by

ecefYOrLon + (ecefYOrLonHP * 1e-2)

18 I1 - ecefZOrAltHP 0.1_ High-precision WGS84 ECEF Z coordinate

mm (or altitude) of the ARP position,

depending on flags above. Must be in the

range -99..+99.

The precise WGS84 ECEF Z coordinate, or

altitude coordinate, in units of cm is given

by

ecefZOrAlt + (ecefZOrAltHP * 1e-2)

19 U1 - reserved2 - Reserved

20 U4 - fixedPosAcc 0.1_ Fixed position 3D accuracy

mm

24 U4 - svinMinDur s Survey-in minimum duration

28 U4 - svinAccLimit 0.1_ Survey-in position accuracy limit

mm

32 U1[8] - reserved3 - Reserved

Bitfield flags

This graphic explains the bits of flags

Name Description

mode Receiver Mode:

0 Disabled

1 Survey In

2 Fixed Mode (true ARP position information required)

3-255 Reserved

lla Position is given in LAT/LON/ALT (default is ECEF)

UBX-13003221 - R28 Early production information Page 269 of 460

## Page 285

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.38 UBX-CFG-TP5 (0x06 0x31)

32.10.38.1 Poll time pulse parameters for time pulse 0

Message UBX-CFG-TP5

Description Poll time pulse parameters for time pulse 0

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3 and 22

Type Poll Request

Comment Sending this (empty / no-payload) message to the receiver results in the receiver

returning a message of type UBX-CFG-TP5 with a payload as defined below for

timepulse 0.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x31 0 see below CK_A CK_B

No payload

32.10.38.2 Poll time pulse parameters

Message UBX-CFG-TP5

Description Poll time pulse parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3 and 22

Type Poll Request

Comment Sending this message to the receiver results in the receiver returning a message

of type UBX-CFG-TP5 with a payload as defined below for the specified time

pulse.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x31 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - tpIdx - Time pulse selection (0 = TIMEPULSE, 1 =

TIMEPULSE2)

UBX-13003221 - R28 Early production information Page 270 of 460

## Page 286

u-blox 8 / u-blox M8 Receiver description - Manual

32.10.38.3 Time pulse parameters

Message UBX-CFG-TP5

Description Time pulse parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 15

Type Get/set

Comment This message is used to get/set time pulse parameters. For more information

see section Time pulse.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x31 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - tpIdx - Time pulse selection (0 = TIMEPULSE, 1 =

TIMEPULSE2)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I2 - antCableDelay ns Antenna cable delay

6 I2 - rfGroupDelay ns RF group delay

8 U4 - freqPeriod Hz_or_ Frequency or period time, depending on

us setting of bit 'isFreq'

12 U4 - freqPeriodLoc Hz_or_ Frequency or period time when locked to

k us GPS time, only used if 'lockedOtherSet' is

set

16 U4 - pulseLenRatio us_or_ Pulse length or duty cycle, depending on

2^-32 'isLength'

20 U4 - pulseLenRatio us_or_ Pulse length or duty cycle when locked to

Lock 2^-32 GPS time, only used if 'lockedOtherSet' is

set

24 I4 - userConfigDel ns User-configurable time pulse delay

ay

28 X4 - flags - Configuration flags (see graphic below)

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 271 of 460

## Page 287

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

active if set enable time pulse; if pin assigned to another function, other function takes precedence

lockGpsFreq if set synchronize time pulse to GPS as soon as GPS time is valid, otherwise use local clock

lockedOtherSe if set use 'freqPeriodLock' and 'pulseLenRatioLock' as soon as GPS time is valid and 'freqPeriod' and

t 'pulseLenRatio' if GPS time is invalid,

if flag is cleared 'freqPeriod' and 'pulseLenRatio' used regardless of GPS time

isFreq if set 'freqPeriodLock' and 'freqPeriod' interpreted as frequency, otherwise interpreted as period

isLength if set 'pulseLenRatioLock' and 'pulseLenRatio' interpreted as pulse length, otherwise interpreted as

duty cycle

alignToTow align pulse to top of second (period time must be integer fraction of 1s)

polarity pulse polarity:

0 = falling edge at top of second

1 = rising edge at top of second

gridUtcGps timegrid to use:

0 = UTC

1 = GPS

32.10.38.4 Time pulse parameters

Message UBX-CFG-TP5

Description Time pulse parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x31 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - tpIdx - Time pulse selection (0 = TIMEPULSE, 1 =

TIMEPULSE2)

1 U1 - version - Message version (0x01 for this version)

2 U1[2] - reserved1 - Reserved

4 I2 - antCableDelay ns Antenna cable delay

6 I2 - rfGroupDelay ns RF group delay

8 U4 - freqPeriod Hz_or_ Frequency or period time, depending on

us setting of bit 'isFreq'

12 U4 - freqPeriodLoc Hz_or_ Frequency or period time when locked to

k us GNSS time, only used if 'lockedOtherSet'

is set

16 U4 - pulseLenRatio us_or_ Pulse length or duty cycle, depending on

2^-32 'isLength'

20 U4 - pulseLenRatio us_or_ Pulse length or duty cycle when locked to

Lock 2^-32 GNSS time, only used if 'lockedOtherSet'

is set

UBX-13003221 - R28 Early production information Page 272 of 460

## Page 288

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-TP5 continued

Byte Offset Number Scaling Name Unit Description

Format

24 I4 - userConfigDel ns User-configurable time pulse delay

ay

28 X4 - flags - Configuration flags (see graphic below)

Bitfield flags

This graphic explains the bits of flags

Name Description

active If set enable time pulse; if pin assigned to another function, other function takes precedence.

Must be set for FTS variant.

lockGnssFreq If set, synchronize time pulse to GNSS as soon as GNSS time is valid. If not set, or before GNSS time

is valid, use local clock.

This flag is ignored by the FTS product variant; in this case the receiver always locks to the best

available time/frequency reference (which is not necessarily GNSS).

This flag can be unset only in Timing product variants.

lockedOtherSe If set the receiver switches between the timepulse settings given by 'freqPeriodLocked' &

t 'pulseLenLocked' and those given by 'freqPeriod' & 'pulseLen'. The 'Locked' settings are used where

the receiver has an accurate sense of time. For non-FTS products, this occurs when GNSS solution

with a reliable time is available, but for FTS products the setting syncMode field governs behavior. In

all cases, the receiver only uses 'freqPeriod' & 'pulseLen' when the flag is unset.

isFreq If set 'freqPeriodLock' and 'freqPeriod' are interpreted as frequency, otherwise interpreted as period.

isLength If set 'pulseLenRatioLock' and 'pulseLenRatio' interpreted as pulse length, otherwise interpreted as

duty cycle.

alignToTow Align pulse to top of second (period time must be integer fraction of 1s).

Also set 'lockGnssFreq' to use this feature.

This flag is ignored by the FTS product variant; it is assumed to be always set (as is lockGnssFreq).

Set maxSlewRate and maxPhaseCorrRate fields of UBX-CFG-SMGR to 0 to disable alignment.

polarity Pulse polarity:

0: falling edge at top of second

1: rising edge at top of second

UBX-13003221 - R28 Early production information Page 273 of 460

## Page 289

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags Description continued

Name Description

gridUtcGnss Timegrid to use:

0: UTC

1: GPS

2: GLONASS

3: BeiDou

4: Galileo (not supported in protocol versions less than 18)

This flag is only relevant if 'lockGnssFreq' and 'alignToTow' are set.

Note that configured GNSS time is estimated by the receiver if locked to any GNSS system. If the

receiver has a valid GNSS fix it will attempt to steer the TP to the specified time grid even if the

specified time is not based on information from the constellation's satellites. To ensure timing based

purely on a given GNSS, restrict the supported constellations in UBX-CFG-GNSS.

syncMode Sync Manager lock mode to use:

0: switch to 'freqPeriodLock' and 'pulseLenRatioLock' as soon as Sync Manager has an accurate

time, never switch back to 'freqPeriod' and 'pulseLenRatio'

1: switch to 'freqPeriodLock' and 'pulseLenRatioLock' as soon as Sync Manager has an accurate

time, and switch back to 'freqPeriod' and 'pulseLenRatio' as soon as time gets inaccurate

This field is only relevant for the FTS product variant.

This field is only relevant if the flag 'lockedOtherSet' is set.

32.10.39 UBX-CFG-TXSLOT (0x06 0x53)

32.10.39.1 TX buffer time slots configuration

Message UBX-CFG-TXSLOT

Description TX buffer time slots configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Set

Comment This message configures how transmit time slots are defined for the receiver

interfaces. These time slots are relative to the chosen time pulse. A receiver that

supports this message offers 3 time slots: nr. 0, 1 and 2. These time pulses

follow each other and their associated priorities decrease in this order. The end

of each can be specified in this message, the beginning is when the circularly

previous slot ends (i.e. slot 0 starts when slot 2 finishes).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x53 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - enable - Bitfield of ports for which the slots are

enabled. (see graphic below)

2 U1 - refTp - Reference timepulse source

0 - Timepulse

1 - Timepulse 2

UBX-13003221 - R28 Early production information Page 274 of 460

## Page 290

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-TXSLOT continued

Byte Offset Number Scaling Name Unit Description

Format

3 U1 - reserved1 - Reserved

Start of repeated block (3 times)

4 + 4*N U4 - end - End of timeslot in milliseconds after time

pulse

End of repeated block

Bitfield enable

This graphic explains the bits of enable

Name Description

DDC DDC/I2C

UART1 UART 1

UART2 UART 2

USB USB

SPI SPI

32.10.40 UBX-CFG-USB (0x06 0x1B)

32.10.40.1 USB configuration

Message UBX-CFG-USB

Description USB configuration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Get/set

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x06 0x1B 108 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - vendorID - Vendor ID. This field shall only be set to

registered Vendor IDs. Changing this field

requires special Host drivers.

2 U2 - productID - Product ID. Changing this field requires

special Host drivers.

4 U1[2] - reserved1 - Reserved

6 U1[2] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 275 of 460

## Page 291

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-CFG-USB continued

Byte Offset Number Scaling Name Unit Description

Format

8 U2 - powerConsumpt mA Power consumed by the device

ion

10 X2 - flags - various configuration flags (see graphic

below)

12 CH[32 - vendorString - String containing the vendor name. 32

] ASCII bytes including 0-termination.

44 CH[32 - productString - String containing the product name. 32

] ASCII bytes including 0-termination.

76 CH[32 - serialNumber - String containing the serial number. 32

] ASCII bytes including 0-termination.

Changing the String fields requires special

Host drivers.

Bitfield flags

This graphic explains the bits of flags

Name Description

reEnum force re-enumeration

powerMode self-powered (1), bus-powered (0)

UBX-13003221 - R28 Early production information Page 276 of 460

## Page 292

u-blox 8 / u-blox M8 Receiver description - Manual

32.11 UBX-ESF (0x10)

External Sensor Fusion Messages: i.e. External Sensor Measurements and Status Information.

Messages in the ESF class are used to output external sensor fusion information from the

receiver.

32.11.1 UBX-ESF-ALG (0x10 0x14)

32.11.1.1 IMU alignment information

Message UBX-ESF-ALG

Description IMU alignment information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment This message outputs the IMU alignment angles which define the rotation from

the installation-frame to the IMU-frame (see the IMU-mount Misalignment

section for more details). In addition, it outputs information about the automatic

IMU-mount alignment (if enabled).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x10 0x14 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x01 for this version)

5 U1 - flags - Flags (see graphic below)

6 U1 - error - Flags (see graphic below)

7 U1 - reserved1 - Reserved

8 U4 1e-2 yaw deg IMU-mount yaw angle [0, 360]

12 I2 1e-2 pitch deg IMU-mount pitch angle [-90, 90]

14 I2 1e-2 roll deg IMU-mount roll angle [-180, 180]

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 277 of 460

## Page 293

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

autoMntAlgOn Automatic IMU-mount alignment on/off bit (0: automatic alignment is not running, 1: automatic

alignment is running)

status Status of the IMU-mount alignment (0: user-defined/fixed angles are used, 1: IMU-mount roll/pitch

angles alignment is ongoing, 2: IMU-mount roll/pitch/yaw angles alignment is ongoing, 3: coarse

IMU-mount alignment are used, 4: fine IMU-mount alignment are used)

Bitfield error

This graphic explains the bits of error

Name Description

tiltAlgError IMU-mount tilt (roll and/or pitch) alignment error (0: no error, 1: error)

yawAlgError IMU-mount yaw alignment error (0: no error, 1: error)

angleError IMU-mount misalignment Euler angle singularity error (0: no error, 1: error). If this error bit is set, the

IMU-mount roll and IMU-mount yaw angles cannot uniquely be defined due to the singularity issue

happening with installations mounted with a +/- 90 degrees misalignment around pitch axis. This is

also known as the 'gimbal-lock' problem affecting rotations described by Euler angles.

32.11.2 UBX-ESF-INS (0x10 0x15)

32.11.2.1 Vehicle dynamics information

Message UBX-ESF-INS

Description Vehicle dynamics information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment This message outputs information about the vehicle dynamics.

For ADR products (in protocol versions less than 19.2), the output dynamics

information (angular rates and accelerations) is expressed with respect to the

vehicle-frame. More information can be found in the ADR Navigation Output

section.

For ADR products, the output dynamics information (angular rates and

accelerations) is expressed with respect to the vehicle-frame. More information

can be found in the ADR Navigation Output section.

For UDR products, the output dynamics information (angular rates and

accelerations) are expressed with respect to the body-frame. More information

can be found in the UDR Navigation Output section.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x10 0x15 36 see below CK_A CK_B

UBX-13003221 - R28 Early production information Page 278 of 460

## Page 294

u-blox 8 / u-blox M8 Receiver description - Manual

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - bitfield0 - Bitfield (see graphic below)

4 U1[4] - reserved1 - Reserved

8 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

12 I4 1e-3 xAngRate deg/s Compensated x-axis angular rate.

16 I4 1e-3 yAngRate deg/s Compensated y-axis angular rate.

20 I4 1e-3 zAngRate deg/s Compensated z-axis angular rate.

24 I4 1e-2 xAccel m/s^2 Compensated x-axis acceleration (gravity-

free).

28 I4 1e-2 yAccel m/s^2 Compensated y-axis acceleration (gravity-

free).

32 I4 1e-2 zAccel m/s^2 Compensated z-axis acceleration (gravity-

free).

Bitfield bitfield0

This graphic explains the bits of bitfield0

Name Description

version Message version (0x01 for this version)

xAngRateValid Compensated x-axis angular rate data validity flag (0: not valid, 1: valid).

yAngRateValid Compensated y-axis angular rate data validity flag (0: not valid, 1: valid).

zAngRateValid Compensated z-axis angular rate data validity flag (0: not valid, 1: valid).

xAccelValid Compensated x-axis acceleration data validity flag (0: not valid, 1: valid).

yAccelValid Compensated y-axis acceleration data validity flag (0: not valid, 1: valid).

zAccelValid Compensated z-axis acceleration data validity flag (0: not valid, 1: valid).

UBX-13003221 - R28 Early production information Page 279 of 460

## Page 295

u-blox 8 / u-blox M8 Receiver description - Manual

32.11.3 UBX-ESF-MEAS (0x10 0x02)

32.11.3.1 External sensor fusion measurements

Message UBX-ESF-MEAS

Description External sensor fusion measurements

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16 and 17 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Input/Output

Comment Possible data types for the data field are described in the ESF Measurement

Data section.

Header Class ID Length (Bytes) Payload Checksum

(8 + 4*numMeas) or (12 +

Message Structure 0xB5 0x62 0x10 0x02 see below CK_A CK_B

4*numMeas)

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - timeTag - Time tag of measurement generated by

external sensor

4 X2 - flags - Flags. Set all unused bits to zero. (see

graphic below)

6 U2 - id - Identification number of data provider

Start of repeated block (numMeas times)

8 + 4*N X4 - data - data (see graphic below)

End of repeated block

Start of optional block

8 + U4 - calibTtag ms Receiver local time calibrated.

4*numMea This field must not be supplied when

s calibTtagValid is set to 0.

End of optional block

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 280 of 460

## Page 296

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

timeMarkSent Time mark signal was supplied just prior to sending this message: 0 = none, 1 = on Ext0, 2 = on Ext1

timeMarkEdge Trigger on rising (0) or falling (1) edge of time mark signal

calibTtagVali Calibration time tag available. Always set to zero.

d

numMeas Number of measurements contained in this message (optional, can be obtained from message size)

Bitfield data

This graphic explains the bits of data

Name Description

dataField Data

dataType Type of data (0 = no data; 1..63 = data type)

32.11.4 UBX-ESF-RAW (0x10 0x03)

32.11.4.1 Raw sensor measurements

Message UBX-ESF-RAW

Description Raw sensor measurements

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16 and 17 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Output

Comment The message contains measurements from the active inertial sensors

connected to the GNSS chip. Possible data types for the data field are

accelerometer, gyroscope and temperature readings as described in the ESF

Measurement Data section.

Note that the rate selected in UBX-CFG-MSG is not respected. If a positive rate is

selected then all raw measurements will be output.

See also Raw Sensor Measurement Data.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x10 0x03 4 + 8*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1[4] - reserved1 - Reserved

Start of repeated block (N times)

4 + 8*N X4 - data - data

Same as in UBX-ESF-MEAS (see graphic

below)

UBX-13003221 - R28 Early production information Page 281 of 460

## Page 297

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-ESF-RAW continued

Byte Offset Number Scaling Name Unit Description

Format

8 + 8*N U4 - sTtag - sensor time tag

End of repeated block

Bitfield data

This graphic explains the bits of data

Name Description

dataField data

dataType type of data (0 = no data; 1..255 = data type)

32.11.5 UBX-ESF-STATUS (0x10 0x10)

32.11.5.1 External sensor fusion status

Message UBX-ESF-STATUS

Description External sensor fusion status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15.01, 16 and 17 (only with ADR products)

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x10 0x10 16 + 4*numSens see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x02 for this version)

5 X1 - initStatus1 - Initialization status bitfield, part 1 (see

graphic below)

6 X1 - initStatus2 - Initialization status bitfield, part 2 (see

graphic below)

7 U1[5] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 282 of 460

## Page 298

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-ESF-STATUS continued

Byte Offset Number Scaling Name Unit Description

Format

12 U1 - fusionMode - Fusion mode:

0: Initialization mode: receiver is

initializing some unknown values required

for doing sensor fusion

1: Fusion mode: GNSS and sensor data are

used for navigation solution computation

2: Suspended fusion mode: sensor fusion

is temporarily disabled due to e.g. invalid

sensor data or detected ferry

3: Disabled fusion mode: sensor fusion is

permanently disabled until receiver reset

due e.g. to sensor error

More details can be found in the Fusion

Modes section.

13 U1[2] - reserved2 - Reserved

15 U1 - numSens - Number of sensors

Start of repeated block (numSens times)

16 + 4*N X1 - sensStatus1 - Sensor status, part 1 (see graphic below)

17 + 4*N X1 - sensStatus2 - Sensor status, part 2 (see graphic below)

18 + 4*N U1 - freq Hz Observation frequency

19 + 4*N X1 - faults - Sensor faults (see graphic below)

End of repeated block

Bitfield initStatus1

This graphic explains the bits of initStatus1

UBX-13003221 - R28 Early production information Page 283 of 460

## Page 299

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

wtInitStatus Wheel tick factor initialization status (0: off, 1: initializing, 2: initialized).

mntAlgStatus Automatic IMU-mount alignment status (0: off, 1: initializing, 2: initialized, 3: initialized).

insInitStatus INS initialization status (0: off, 1: initializing, 2: initialized).

Bitfield initStatus2

This graphic explains the bits of initStatus2

Name Description

imuInitStatus IMU initialization status (0: off, 1: initializing, 2: initialized).

Bitfield sensStatus1

This graphic explains the bits of sensStatus1

Name Description

type Sensor data type. See section Sensor data types in the integration manual for details.

used If set, sensor data is used for the current sensor fusion solution.

ready If set, sensor is set up (configuration is available or not required) but not used for computing the

current sensor fusion solution.

Bitfield sensStatus2

This graphic explains the bits of sensStatus2

UBX-13003221 - R28 Early production information Page 284 of 460

## Page 300

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

calibStatus 00: Sensor is not calibrated

01: Sensor is calibrating

10/11: Sensor is calibrated

Good dead reckoning performance is only possible when all used sensors are calibrated. Depending

on the quality of the GNSS signals and the sensor data, the sensors may take a longer time to get

calibrated.

timeStatus 00: No data

01: Reception of the first byte used to tag the measurement

10: Event input used to tag the measurement

11: Time tag provided with the data

Bitfield faults

This graphic explains the bits of faults

Name Description

badMeas Bad measurements detected

badTTag Bad measurement time-tags detected

missingMeas Missing or time-misaligned measurements detected

noisyMeas High measurement noise-level detected

UBX-13003221 - R28 Early production information Page 285 of 460

## Page 301

u-blox 8 / u-blox M8 Receiver description - Manual

32.12 UBX-HNR (0x28)

High Rate Navigation Results Messages: i.e. High rate time, position, speed, heading.

Messages in the HNR class are used to output high rate navigation data for position, altitude,

velocity and their accuracies.

32.12.1 UBX-HNR-ATT (0x28 0x01)

32.12.1.1 Attitude solution

Message UBX-HNR-ATT

Description Attitude solution

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 19.2 (only with ADR or UDR products

)

Type Periodic/Polled

Comment This message outputs the attitude solution as roll, pitch and heading angles.

More details about vehicle attitude can be found in the Vehicle Attitude Output

(ADR) section for ADR products.

More details about vehicle attitude can be found in the Vehicle Attitude Output

(UDR) section for UDR products.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x28 0x01 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the HNR epoch.

4 U1 - version - Message version (0x01 for this version)

5 U1[3] - reserved1 - Reserved

8 I4 1e-5 roll deg Vehicle roll.

12 I4 1e-5 pitch deg Vehicle pitch.

16 I4 1e-5 heading deg Vehicle heading.

20 U4 1e-5 accRoll deg Vehicle roll accuracy (if null, roll angle is

not available).

24 U4 1e-5 accPitch deg Vehicle pitch accuracy (if null, pitch angle

is not available).

28 U4 1e-5 accHeading deg Vehicle heading accuracy (if null, heading

angle is not available).

UBX-13003221 - R28 Early production information Page 286 of 460

## Page 302

u-blox 8 / u-blox M8 Receiver description - Manual

32.12.2 UBX-HNR-INS (0x28 0x02)

32.12.2.1 Vehicle dynamics information

Message UBX-HNR-INS

Description Vehicle dynamics information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3, 22,

22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment This message outputs high rate information about vehicle dynamics computed

by the Inertial Navigation System (INS) during ESF-based navigation.

For ADR products (in protocol versions less than 19.2), the output dynamics

information (angular rates and accelerations) is expressed with respect to the

vehicle-frame. More information can be found in the ADR Navigation Output

section.

For UDR products, the output dynamics information (angular rates and

accelerations) is expressed with respect to the body-frame. More information

can be found in the UDR Navigation Output section.

For ADR products, the output dynamics information (angular rates and

accelerations) is expressed with respect to the vehicle-frame. More information

can be found in the ADR Navigation Output section.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x28 0x02 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X4 - bitfield0 - Bitfield (see graphic below)

4 U1[4] - reserved1 - Reserved

8 U4 - iTOW ms GPS time of week of the HNR epoch.

12 I4 1e-3 xAngRate deg/s Compensated x-axis angular rate.

16 I4 1e-3 yAngRate deg/s Compensated y-axis angular rate.

20 I4 1e-3 zAngRate deg/s Compensated z-axis angular rate.

24 I4 1e-2 xAccel m/s^2 Compensated x-axis acceleration (with

gravity).

28 I4 1e-2 yAccel m/s^2 Compensated y-axis acceleration (with

gravity).

32 I4 1e-2 zAccel m/s^2 Compensated z-axis acceleration (with

gravity).

UBX-13003221 - R28 Early production information Page 287 of 460

## Page 303

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield bitfield0

This graphic explains the bits of bitfield0

Name Description

version Message version (0x00 for this version)

xAngRateValid Compensated x-axis angular rate data validity flag (0: not valid, 1: valid).

yAngRateValid Compensated y-axis angular rate data validity flag (0: not valid, 1: valid).

zAngRateValid Compensated z-axis angular rate data validity flag (0: not valid, 1: valid).

xAccelValid Compensated x-axis acceleration data validity flag (0: not valid, 1: valid).

yAccelValid Compensated y-axis acceleration data validity flag (0: not valid, 1: valid).

zAccelValid Compensated z-axis acceleration data validity flag (0: not valid, 1: valid).

32.12.3 UBX-HNR-PVT (0x28 0x00)

32.12.3.1 High rate output of PVT solution

Message UBX-HNR-PVT

Description High rate output of PVT solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment This message provides the position, velocity and time solution with high output

rate.

Note that during a leap second there may be more or less than 60 seconds in a

minute.

See the description of leap seconds for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x28 0x00 72 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U2 - year y Year (UTC)

6 U1 - month month Month, range 1..12 (UTC)

7 U1 - day d Day of month, range 1..31 (UTC)

8 U1 - hour h Hour of day, range 0..23 (UTC)

9 U1 - min min Minute of hour, range 0..59 (UTC)

10 U1 - sec s Seconds of minute, range 0..60 (UTC)

11 X1 - valid - Validity Flags (see graphic below)

12 I4 - nano ns Fraction of second, range -1e9 .. 1e9 (UTC)

UBX-13003221 - R28 Early production information Page 288 of 460

## Page 304

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-HNR-PVT continued

Byte Offset Number Scaling Name Unit Description

Format

16 U1 - gpsFix - GPSfix Type, range 0..5

0x00 = No Fix

0x01 = Dead Reckoning only

0x02 = 2D-Fix

0x03 = 3D-Fix

0x04 = GPS + dead reckoning combined

0x05 = Time only fix

0x06..0xff: reserved

17 X1 - flags - Fix Status Flags (see graphic below)

18 U1[2] - reserved1 - Reserved

20 I4 1e-7 lon deg Longitude

24 I4 1e-7 lat deg Latitude

28 I4 - height mm Height above Ellipsoid

32 I4 - hMSL mm Height above mean sea level

36 I4 - gSpeed mm/s Ground Speed (2-D)

40 I4 - speed mm/s Speed (3-D)

44 I4 1e-5 headMot deg Heading of motion (2-D)

48 I4 1e-5 headVeh deg Heading of vehicle (2-D)

52 U4 - hAcc mm Horizontal accuracy

56 U4 - vAcc mm Vertical accuracy

60 U4 - sAcc mm/s Speed accuracy

64 U4 1e-5 headAcc deg Heading accuracy

68 U1[4] - reserved2 - Reserved

Bitfield valid

This graphic explains the bits of valid

UBX-13003221 - R28 Early production information Page 289 of 460

## Page 305

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

validDate 1 = Valid UTC Date (see Time Validity section in the integration manual for details)

validTime 1 = Valid UTC Time of Day (see Time Validity section in the integration manual for details)

fullyResolved 1 = UTC Time of Day has been fully resolved (no seconds uncertainty)

Bitfield flags

This graphic explains the bits of flags

Name Description

GPSfixOK >1 = Fix within limits (e.g. DOP & accuracy)

DiffSoln 1 = DGPS used

WKNSET 1 = Valid GPS week number

TOWSET 1 = Valid GPS time of week (iTOW & fTOW)

headVehValid 1= Heading of vehicle is valid

UBX-13003221 - R28 Early production information Page 290 of 460

## Page 306

u-blox 8 / u-blox M8 Receiver description - Manual

32.13 UBX-INF (0x04)

Information Messages: i.e. Printf-Style Messages, with IDs such as Error, Warning, Notice.

Messages in the INF class are used to output strings in a printf style from the firmware or

application code. All INF messages have an associated type to indicate the kind of message.

32.13.1 UBX-INF-DEBUG (0x04 0x04)

32.13.1.1 ASCII output with debug contents

Message UBX-INF-DEBUG

Description ASCII output with debug contents

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message has a variable length payload, representing an ASCII string.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x04 0x04 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 CH - str - ASCII Character

End of repeated block

32.13.2 UBX-INF-ERROR (0x04 0x00)

32.13.2.1 ASCII output with error contents

Message UBX-INF-ERROR

Description ASCII output with error contents

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message has a variable length payload, representing an ASCII string.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x04 0x00 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 CH - str - ASCII Character

End of repeated block

UBX-13003221 - R28 Early production information Page 291 of 460

## Page 307

u-blox 8 / u-blox M8 Receiver description - Manual

32.13.3 UBX-INF-NOTICE (0x04 0x02)

32.13.3.1 ASCII output with informational contents

Message UBX-INF-NOTICE

Description ASCII output with informational contents

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message has a variable length payload, representing an ASCII string.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x04 0x02 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 CH - str - ASCII Character

End of repeated block

32.13.4 UBX-INF-TEST (0x04 0x03)

32.13.4.1 ASCII output with test contents

Message UBX-INF-TEST

Description ASCII output with test contents

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message has a variable length payload, representing an ASCII string.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x04 0x03 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 CH - str - ASCII Character

End of repeated block

UBX-13003221 - R28 Early production information Page 292 of 460

## Page 308

u-blox 8 / u-blox M8 Receiver description - Manual

32.13.5 UBX-INF-WARNING (0x04 0x01)

32.13.5.1 ASCII output with warning contents

Message UBX-INF-WARNING

Description ASCII output with warning contents

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message has a variable length payload, representing an ASCII string.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x04 0x01 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 CH - str - ASCII Character

End of repeated block

UBX-13003221 - R28 Early production information Page 293 of 460

## Page 309

u-blox 8 / u-blox M8 Receiver description - Manual

32.14 UBX-LOG (0x21)

Logging Messages: i.e. Log creation, deletion, info and retrieval.

Messages in the LOG class are used to configure and report status information of the logging and

batching features.

32.14.1 UBX-LOG-BATCH (0x21 0x11)

32.14.1.1 Batched data

Message UBX-LOG-BATCH

Description Batched data

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 23.01

Type Polled

Comment This message combines position, velocity and time solution, including accuracy

figures.

The output of this message can be requested via UBX-LOG-RETRIEVEBATCH.

The content of this message is influenced by UBX-CFG-BATCH. Depending on the

flags extraPvt and extraOdo some of the fields in this message may not be

valid. This validity information is also indicated in this message via flags of the

same name.

See Data Batching for more information.

Note that during a leap second there may be more or less than 60 seconds in a

minute.

See the description of leap seconds for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x11 100 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - contentValid - Content validity flags (see graphic below)

2 U2 - msgCnt - Message counter; increments for each

sent UBX-LOG-BATCH message.

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

Only valid if extraPvt is set.

8 U2 - year y Year (UTC)

10 U1 - month month Month, range 1..12 (UTC)

11 U1 - day d Day of month, range 1..31 (UTC)

12 U1 - hour h Hour of day, range 0..23 (UTC)

13 U1 - min min Minute of hour, range 0..59 (UTC)

14 U1 - sec s Seconds of minute, range 0..60 (UTC)

15 X1 - valid - Validity flags (see graphic below)

16 U4 - tAcc ns Time accuracy estimate (UTC)

Only valid if extraPvt is set.

20 I4 - fracSec ns Fraction of second, range -1e9 .. 1e9 (UTC)

UBX-13003221 - R28 Early production information Page 294 of 460

## Page 310

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-LOG-BATCH continued

Byte Offset Number Scaling Name Unit Description

Format

24 U1 - fixType - GNSSfix Type:

0: no fix

2: 2D-fix

3: 3D-fix

25 X1 - flags - Fix status flags (see graphic below)

26 X1 - flags2 - Additional flags

27 U1 - numSV - Number of satellites used in Nav Solution

Only valid if extraPvt is set.

28 I4 1e-7 lon deg Longitude

32 I4 1e-7 lat deg Latitude

36 I4 - height mm Height above ellipsoid

40 I4 - hMSL mm Height above mean sea level

Only valid if extraPvt is set.

44 U4 - hAcc mm Horizontal accuracy estimate

48 U4 - vAcc mm Vertical accuracy estimate

Only valid if extraPvt is set.

52 I4 - velN mm/s NED north velocity

Only valid if extraPvt is set.

56 I4 - velE mm/s NED east velocity

Only valid if extraPvt is set.

60 I4 - velD mm/s NED down velocity

Only valid if extraPvt is set.

64 I4 - gSpeed mm/s Ground Speed (2-D)

68 I4 1e-5 headMot deg Heading of motion (2-D)

72 U4 - sAcc mm/s Speed accuracy estimate

Only valid if extraPvt is set.

76 U4 1e-5 headAcc deg Heading accuracy estimate

Only valid if extraPvt is set.

80 U2 0.01 pDOP - Position DOP

Only valid if extraPvt is set.

82 U1[2] - reserved1 - Reserved

84 U4 - distance m Ground distance since last reset

Only valid if extraOdo is set.

88 U4 - totalDistance m Total cumulative ground distance

Only valid if extraOdo is set.

92 U4 - distanceStd m Ground distance accuracy (1-sigma)

Only valid if extraOdo is set.

96 U1[4] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 295 of 460

## Page 311

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield contentValid

This graphic explains the bits of contentValid

Name Description

extraPvt Extra PVT information is valid

The fields iTOW, tAcc, numSV, hMSL, vAcc, velN, velE, velD, sAcc, headAcc and pDOP are only valid if

this flag is set.

extraOdo Odometer data is valid

The fields distance, totalDistance and distanceStd are only valid if this flag is set.

Note: the odometer feature itself must also be enabled.

Bitfield valid

This graphic explains the bits of valid

Name Description

validDate 1 = valid UTC Date

(see Time Validity section for details)

validTime 1 = valid UTC Time of Day

(see Time Validity section for details)

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 296 of 460

## Page 312

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

gnssFixOK 1 = valid fix (i.e within DOP & accuracy masks)

diffSoln 1 = differential corrections were applied

psmState Power save mode state

(see Power Management)

0: PSM is not active

1: Enabled (an intermediate state before Acquisition state)

2: Acquisition

3: Tracking

4: Power optimized tracking

5: Inactive

32.14.2 UBX-LOG-CREATE (0x21 0x07)

32.14.2.1 Create log file

Message UBX-LOG-CREATE

Description Create log file

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message is used to create an initial logging file and activate the logging

subsystem.

UBX-ACK-ACK or UBX-ACK-NAK are returned to indicate success or failure.

This message does not handle activation of recording or filtering of log entries

(see UBX-CFG-LOGFILTER).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x07 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - logCfg - Config flags (see graphic below)

2 U1 - reserved1 - Reserved

3 U1 - logSize - Indicates the size of the log:

0 (maximum safe size): Ensures that

logging will not be interrupted and enough

space will be left available for all other

uses of the filestore

1 (minimum size):

2 (user-defined): See 'userDefinedSize'

below

4 U4 - userDefinedSi bytes Sets the maximum amount of space in the

ze filestore that can be used by the logging

task.

This field is only applicable if logSize is set

to user-defined.

UBX-13003221 - R28 Early production information Page 297 of 460

## Page 313

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield logCfg

This graphic explains the bits of logCfg

Name Description

circular Log is circular (new entries overwrite old ones in a full log) if this bit set

32.14.3 UBX-LOG-ERASE (0x21 0x03)

32.14.3.1 Erase logged data

Message UBX-LOG-ERASE

Description Erase logged data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message deactivates the logging system and erases all logged data.

UBX-ACK-ACK or UBX-ACK-NAK are returned to indicate success or failure.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x03 0 see below CK_A CK_B

No payload

32.14.4 UBX-LOG-FINDTIME (0x21 0x0E)

32.14.4.1 Find index of a log entry based on a given time

Message UBX-LOG-FINDTIME

Description Find index of a log entry based on a given time

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message can be used for a time-based search of a log. It can find the index

of the first log entry with time equal to the given time, otherwise the index of the

most recent entry with time less than the given time. This index can then be

used with the UBX-LOG-RETRIEVE message to provide time-based retrieval of

log entries.

Searching a log is effective for a given time later than the base date (January

1st, 2004). Searching a log for a given time earlier than the base date will result

in an 'entry not found' response. (Searching a log for a given time earlier than the

base date will result in a UBX-ACK-NAK message in protocol versions less than

18).

Searching a log for a given time greater than the last recorded entry's time will

return the index of the last recorded entry. (If the logging has stopped due to

lack of file space, such a search will result in a UBX-ACK-NAK message in

UBX-13003221 - R28 Early production information Page 298 of 460

## Page 314

u-blox 8 / u-blox M8 Receiver description - Manual

protocol versions less than 18).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x0E 10 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - type - Message type, 0 for request

2 U2 - year - Year (1-65635) of UTC time

4 U1 - month - Month (1-12) of UTC time

5 U1 - day - Day (1-31) of UTC time

6 U1 - hour - Hour (0-23) of UTC time

7 U1 - minute - Minute (0-59) of UTC time

8 U1 - second - Second (0-60) of UTC time

9 U1 - reserved1 - Reserved

32.14.4.2 Response to FINDTIME request

Message UBX-LOG-FINDTIME

Description Response to FINDTIME request

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x0E 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1 - type - Message type, 1 for response

2 U1[2] - reserved1 - Reserved

4 U4 - entryNumber - Index of the first log entry with time =

given time, otherwise index of the most

recent entry with time < given time. If

0xFFFFFFFF, no log entry found with time

<= given time. The indexing of log entries

is zero-based.

UBX-13003221 - R28 Early production information Page 299 of 460

## Page 315

u-blox 8 / u-blox M8 Receiver description - Manual

32.14.5 UBX-LOG-INFO (0x21 0x08)

32.14.5.1 Poll for log information

Message UBX-LOG-INFO

Description Poll for log information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Upon sending of this message, the receiver returns UBX-LOG-INFO as defined

below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x08 0 see below CK_A CK_B

No payload

32.14.5.2 Log information

Message UBX-LOG-INFO

Description Log information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is used to report information about the logging subsystem.

Note:

• The reported maximum log size will be smaller than that originally specified in

LOG-CREATE due to logging and filestore implementation overheads.

• Log entries are compressed in a variable length fashion, so it may be difficult

to predict log space usage with any precision.

• There may be times when the receiver does not have an accurate time (e.g. if

the week number is not yet known), in which case some entries will not have a

timestamp. This may result in the oldest/newest entry time values not taking

account of these entries.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x08 48 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - filestoreCapa bytes The capacity of the filestore

city

8 U1[8] - reserved2 - Reserved

16 U4 - currentMaxLog bytes The maximum size the current log is

Size allowed to grow to

20 U4 - currentLogSiz bytes Approximate amount of space in log

e currently occupied

UBX-13003221 - R28 Early production information Page 300 of 460

## Page 316

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-LOG-INFO continued

Byte Offset Number Scaling Name Unit Description

Format

24 U4 - entryCount - Number of entries in the log.

Note: for circular logs this value will

decrease when a group of entries is

deleted to make space for new ones.

28 U2 - oldestYear - Oldest entry UTC year (1-65635) or zero if

there are no entries with known time

30 U1 - oldestMonth - Oldest month (1-12)

31 U1 - oldestDay - Oldest day (1-31)

32 U1 - oldestHour - Oldest hour (0-23)

33 U1 - oldestMinute - Oldest minute (0-59)

34 U1 - oldestSecond - Oldest second (0-60)

35 U1 - reserved3 - Reserved

36 U2 - newestYear - Newest year (1-65635) or zero if there are

no entries with known time

38 U1 - newestMonth - Newest month (1-12)

39 U1 - newestDay - Newest day (1-31)

40 U1 - newestHour - Newest hour (0-23)

41 U1 - newestMinute - Newest minute (0-59)

42 U1 - newestSecond - Newest second (0-60)

43 U1 - reserved4 - Reserved

44 X1 - status - Log status flags (see graphic below)

45 U1[3] - reserved5 - Reserved

Bitfield status

This graphic explains the bits of status

Name Description

recording Log entry recording is currently turned on

inactive Logging system not active - no log present

circular The current log is circular

UBX-13003221 - R28 Early production information Page 301 of 460

## Page 317

u-blox 8 / u-blox M8 Receiver description - Manual

32.14.6 UBX-LOG-RETRIEVEBATCH (0x21 0x10)

32.14.6.1 Request batch data

Message UBX-LOG-RETRIEVEBATCH

Description Request batch data

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 23.01

Type Command

Comment This message is used to request batched data.

Batch entries are returned in chronological order, using one UBX-LOG-BATCH per

navigation epoch.

The speed of transfer can be maximized by using a high data rate.

See Data Batching for more information.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x10 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - flags - Flags (see graphic below)

2 U1[2] - reserved1 - Reserved

Bitfield flags

This graphic explains the bits of flags

Name Description

sendMonFirst Send UBX-MON-BATCH message before sending the UBX-LOG-BATCH message(s).

UBX-13003221 - R28 Early production information Page 302 of 460

## Page 318

u-blox 8 / u-blox M8 Receiver description - Manual

32.14.7 UBX-LOG-RETRIEVEPOSEXTRA (0x21 0x0f)

32.14.7.1 Odometer log entry

Message UBX-LOG-RETRIEVEPOSEXTRA

Description Odometer log entry

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is used to report an odometer log entry

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x0f 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - entryIndex - The index of this log entry

4 U1 - version - Message version (0x00 for this version)

5 U1 - reserved1 - Reserved

6 U2 - year - Year (1-65635) of UTC time. Will be zero if

time not known

8 U1 - month - Month (1-12) of UTC time

9 U1 - day - Day (1-31) of UTC time

10 U1 - hour - Hour (0-23) of UTC time

11 U1 - minute - Minute (0-59) of UTC time

12 U1 - second - Second (0-60) of UTC time

13 U1[3] - reserved2 - Reserved

16 U4 - distance - Odometer distance traveled since the last

time the odometer was reset by a UBX-

NAV-RESETODO

20 U1[12] - reserved3 - Reserved

32.14.8 UBX-LOG-RETRIEVEPOS (0x21 0x0b)

32.14.8.1 Position fix log entry

Message UBX-LOG-RETRIEVEPOS

Description Position fix log entry

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is used to report a position fix log entry

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x0b 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - entryIndex - The index of this log entry

UBX-13003221 - R28 Early production information Page 303 of 460

## Page 319

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-LOG-RETRIEVEPOS continued

Byte Offset Number Scaling Name Unit Description

Format

4 I4 1e-7 lon deg Longitude

8 I4 1e-7 lat deg Latitude

12 I4 - hMSL mm Height above mean sea level

16 U4 - hAcc mm Horizontal accuracy estimate

20 U4 - gSpeed mm/s Ground speed (2-D)

24 U4 1e-5 heading deg Heading

28 U1 - version - Message version (0x00 for this version)

29 U1 - fixType - Fix type:

0x01: Dead Reckoning only

0x02: 2D-Fix

0x03: 3D-Fix

0x04: GNSS + Dead Reckoning combined

30 U2 - year - Year (1-65635) of UTC time

32 U1 - month - Month (1-12) of UTC time

33 U1 - day - Day (1-31) of UTC time

34 U1 - hour - Hour (0-23) of UTC time

35 U1 - minute - Minute (0-59) of UTC time

36 U1 - second - Second (0-60) of UTC time

37 U1 - reserved1 - Reserved

38 U1 - numSV - Number of satellites used in the position

fix

39 U1 - reserved2 - Reserved

32.14.9 UBX-LOG-RETRIEVESTRING (0x21 0x0d)

32.14.9.1 Byte string log entry

Message UBX-LOG-RETRIEVESTRING

Description Byte string log entry

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is used to report a byte string log entry

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x0d 16 + 1*byteCount see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - entryIndex - The index of this log entry

4 U1 - version - Message version (0x00 for this version)

5 U1 - reserved1 - Reserved

6 U2 - year - Year (1-65635) of UTC time. Will be zero if

time not known

8 U1 - month - Month (1-12) of UTC time

UBX-13003221 - R28 Early production information Page 304 of 460

## Page 320

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-LOG-RETRIEVESTRING continued

Byte Offset Number Scaling Name Unit Description

Format

9 U1 - day - Day (1-31) of UTC time

10 U1 - hour - Hour (0-23) of UTC time

11 U1 - minute - Minute (0-59) of UTC time

12 U1 - second - Second (0-60) of UTC time

13 U1 - reserved2 - Reserved

14 U2 - byteCount - Size of string in bytes

Start of repeated block (byteCount times)

16 + 1*N U1 - bytes - The bytes of the string

End of repeated block

32.14.10 UBX-LOG-RETRIEVE (0x21 0x09)

32.14.10.1 Request log data

Message UBX-LOG-RETRIEVE

Description Request log data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message is used to request logged data (log recording must first be

disabled, see UBX-CFG-LOGFILTER).

Log entries are returned in chronological order, using the messages UBX-LOG-

RETRIEVEPOS and UBX-LOG-RETRIEVESTRING. If the odometer was enabled at

the time a position was logged, then message UBX-LOG-RETRIEVEPOSEXTRA will

also be used. The maximum number of entries that can be returned in response

to a single UBX-LOG-RETRIEVE message is 256. If more entries than this are

required the message will need to be sent multiple times with different

startNumbers. The retrieve will be stopped if any UBX-LOG message is received.

The speed of transfer can be maximized by using a high data rate and

temporarily stopping the GPS processing (see UBX-CFG-RST).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x09 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - startNumber - Index of first log entry to be transferred. If

it is larger than the index of the last

available log entry, then the first log entry

to be transferred is the last available log

entry. The indexing of log entries is zero-

based.

UBX-13003221 - R28 Early production information Page 305 of 460

## Page 321

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-LOG-RETRIEVE continued

Byte Offset Number Scaling Name Unit Description

Format

4 U4 - entryCount - Number of log entries to transfer in total

including the first entry to be transferred.

If it is larger than the log entries available

starting from the first entry to be

transferred, then only the available log

entries are transferred followed by a UBX-

ACK-NAK. The maximum is 256.

8 U1 - version - Message version (0x00 for this version)

9 U1[3] - reserved1 - Reserved

32.14.11 UBX-LOG-STRING (0x21 0x04)

32.14.11.1 Store arbitrary string in on-board flash

Message UBX-LOG-STRING

Description Store arbitrary string in on-board flash

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message can be used to store an arbitrary byte string in the on-board flash

memory. The maximum length that can be stored is 256 bytes.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x21 0x04 0 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*1 U1 - bytes - The string of bytes to be logged

(maximum 256)

End of repeated block

UBX-13003221 - R28 Early production information Page 306 of 460

## Page 322

u-blox 8 / u-blox M8 Receiver description - Manual

32.15 UBX-MGA (0x13)

Multiple GNSS Assistance Messages: i.e. Assistance data for various GNSS.

Messages in the MGA class are used for GNSS aiding information from and to the receiver.

32.15.1 UBX-MGA-ACK (0x13 0x60)

32.15.1.1 UBX-MGA-ACK-DATA0

Message UBX-MGA-ACK-DATA0

Description Multiple GNSS acknowledge message

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is sent by a u-blox receiver to acknowledge the receipt of an

assistance message.

Acknowledgments are enabled by setting the ackAiding parameter in the UBX-

CFG-NAVX5 message.

See the description of flow control for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x60 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Type of acknowledgment:

0: The message was not used by the

receiver (see infoCode field for an

indication of why)

1: The message was accepted for use by

the receiver (the infoCode field will be 0)

1 U1 - version - Message version (0x00 for this version)

2 U1 - infoCode - Provides greater information on what the

receiver chose to do with the message

contents:

0: The receiver accepted the data

1: The receiver does not know the time so

it cannot use the data (To resolve this a

UBX-MGA-INI-TIME_UTC message should

be supplied first)

2: The message version is not supported

by the receiver

3: The message size does not match the

message version

4: The message data could not be stored

to the database

5: The receiver is not ready to use the

message data

6: The message type is unknown

UBX-13003221 - R28 Early production information Page 307 of 460

## Page 323

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-ACK continued

Byte Offset Number Scaling Name Unit Description

Format

3 U1 - msgId - UBX message ID of the acknowledged

message

4 U1[4] - msgPayloadSta - The first 4 bytes of the acknowledged

rt message's payload

32.15.2 UBX-MGA-ANO (0x13 0x20)

32.15.2.1 Multiple GNSS AssistNow Offline assistance

Message UBX-MGA-ANO

Description Multiple GNSS AssistNow Offline assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message is created by the AssistNow Offline service to deliver AssistNow

Offline assistance to the receiver.

See the description of AssistNow Offline for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x20 76 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x00 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - Satellite identifier (see Satellite

Numbering)

3 U1 - gnssId - GNSS identifier (see Satellite Numbering)

4 U1 - year - years since the year 2000

5 U1 - month - month (1..12)

6 U1 - day - day (1..31)

7 U1 - reserved1 - Reserved

8 U1[64] - data - assistance data

72 U1[4] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 308 of 460

## Page 324

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.3 UBX-MGA-BDS (0x13 0x03)

32.15.3.1 UBX-MGA-BDS-EPH

Message UBX-MGA-BDS-EPH

Description BeiDou ephemeris assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of BeiDou D1/D2 ephemeris assistance to a

receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x03 88 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - BeiDou satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U1 - SatH1 - Autonomous satellite Health flag

5 U1 - IODC - Issue of Data, Clock

6 I2 2^-66 a2 s/s^2 Time polynomial coefficient 2

8 I4 2^-50 a1 s/s Time polynomial coefficient 1

12 I4 2^-33 a0 s Time polynomial coefficient 0

16 U4 2^3 toc s Clock data reference time

20 I2 0.1 TGD1 ns Equipment Group Delay Differential

22 U1 - URAI - User Range Accuracy Index

23 U1 - IODE - Issue of Data, Ephemeris

24 U4 2^3 toe s Ephemeris reference time

28 U4 2^-19 sqrtA m^0.5 Square root of semi-major axis

32 U4 2^-33 e - Eccentricity

36 I4 2^-31 omega semi- Argument of perigee

circles

40 I2 2^-43 Deltan semi- Mean motion difference from computed

circles value

/s

42 I2 2^-43 IDOT semi- Rate of inclination angle

circles

/s

44 I4 2^-31 M0 semi- Mean anomaly at reference time

circles

48 I4 2^-31 Omega0 semi- Longitude of ascending node of orbital of

circles plane computed according to reference

time

UBX-13003221 - R28 Early production information Page 309 of 460

## Page 325

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-BDS continued

Byte Offset Number Scaling Name Unit Description

Format

52 I4 2^-43 OmegaDot semi- Rate of right ascension

circles

/s

56 I4 2^-31 i0 semi- Inclination angle at reference time

circles

60 I4 2^-31 Cuc radian Amplitude of cosine harmonic correction

s term to the argument of latitude

64 I4 2^-31 Cus radian Amplitude of sine harmonic correction

s term to the argument of latitude

68 I4 2^-6 Crc m Amplitude of cosine harmonic correction

term to the orbit radius

72 I4 2^-6 Crs m Amplitude of sine harmonic correction

term to the orbit radius

76 I4 2^-31 Cic radian Amplitude of cosine harmonic correction

s term to the angle of inclination

80 I4 2^-31 Cis radian Amplitude of sine harmonic correction

s term to the angle of inclination

84 U1[4] - reserved2 - Reserved

32.15.3.2 UBX-MGA-BDS-ALM

Message UBX-MGA-BDS-ALM

Description BeiDou almanac assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of BeiDou almanac assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x03 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this version)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - BeiDou satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U1 - Wna week Almanac Week Number

5 U1 2^12 toa s Almanac reference time

6 I2 2^-19 deltaI semi- Almanac correction of orbit reference

circles inclination at reference time

8 U4 2^-11 sqrtA m^0.5 Almanac square root of semi-major axis

UBX-13003221 - R28 Early production information Page 310 of 460

## Page 326

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-BDS continued

Byte Offset Number Scaling Name Unit Description

Format

12 U4 2^-21 e - Almanac eccentricity

16 I4 2^-23 omega semi- Almanac argument of perigee

circles

20 I4 2^-23 M0 semi- Almanac mean anomaly at reference time

circles

24 I4 2^-23 Omega0 semi- Almanac longitude of ascending node of

circles orbit plane at computed according to

reference time

28 I4 2^-38 omegaDot semi- Almanac rate of right ascension

circles

/s

32 I2 2^-20 a0 s Almanac satellite clock bias

34 I2 2^-38 a1 s/s Almanac satellite clock rate

36 U1[4] - reserved2 - Reserved

32.15.3.3 UBX-MGA-BDS-HEALTH

Message UBX-MGA-BDS-HEALTH

Description BeiDou health assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of BeiDou health assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x03 68 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x04 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 U2[30] - healthCode - Each two-byte value represents a BeiDou

SV (1-30). The 9 LSBs of each byte contain

the 9 bit health code from subframe 5

pages 7,8 of the D1 message, and from

subframe 5 pages 35,36 of the D2

message.

64 U1[4] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 311 of 460

## Page 327

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.3.4 UBX-MGA-BDS-UTC

Message UBX-MGA-BDS-UTC

Description BeiDou UTC assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of BeiDou UTC assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x03 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x05 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 2^-30 a0UTC s BDT clock bias relative to UTC

8 I4 2^-50 a1UTC s/s BDT clock rate relative to UTC

12 I1 - dtLS s Delta time due to leap seconds before the

new leap second effective

13 U1[1] - reserved2 - Reserved

14 U1 - wnRec week BeiDou week number of reception of this

UTC parameter set (8-bit truncated)

15 U1 - wnLSF week Week number of the new leap second

16 U1 - dN day Day number of the new leap second

17 I1 - dtLSF s Delta time due to leap seconds after the

new leap second effective

18 U1[2] - reserved3 - Reserved

32.15.3.5 UBX-MGA-BDS-IONO

Message UBX-MGA-BDS-IONO

Description BeiDou ionosphere assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of BeiDou ionospheric assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x03 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x06 for this type)

UBX-13003221 - R28 Early production information Page 312 of 460

## Page 328

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-BDS continued

Byte Offset Number Scaling Name Unit Description

Format

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I1 2^-30 alpha0 s Ionospheric parameter alpha0

5 I1 2^-27 alpha1 s/pi Ionospheric parameter alpha1

6 I1 2^-24 alpha2 s/pi^2 Ionospheric parameter alpha2

7 I1 2^-24 alpha3 s/pi^3 Ionospheric parameter alpha3

8 I1 2^11 beta0 s Ionospheric parameter beta0

9 I1 2^14 beta1 s/pi Ionospheric parameter beta1

10 I1 2^16 beta2 s/pi^2 Ionospheric parameter beta2

11 I1 2^16 beta3 s/pi^3 Ionospheric parameter beta3

12 U1[4] - reserved2 - Reserved

32.15.4 UBX-MGA-DBD (0x13 0x80)

32.15.4.1 Poll the navigation database

Message UBX-MGA-DBD

Description Poll the navigation database

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Poll the whole navigation data base. The receiver will send all available data from

its internal database. The receiver will indicate the finish of the transmission

with a UBX-MGA-ACK. The msgPayloadStart field of the UBX-MGA-ACK message

will contain a U4 representing the number of UBX-MGA-DBD-DATA* messages

sent.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x80 0 see below CK_A CK_B

No payload

32.15.4.2 Navigation database dump entry

Message UBX-MGA-DBD

Description Navigation database dump entry

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input/Output

Comment UBX-MGA-DBD messages are only intended to be sent back to the same

receiver that generated them.

Navigation database entry. The data fields are firmware-specific. Transmission

of this type of message will be acknowledged by UBX-MGA-ACK messages, if

acknowledgment has been enabled.

See the description of flow control for details.

The maximum payload size for firmware 2.01 onwards is 164 bytes (which makes

UBX-13003221 - R28 Early production information Page 313 of 460

## Page 329

u-blox 8 / u-blox M8 Receiver description - Manual

the maximum message size 172 bytes).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x80 12 + 1*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1[12] - reserved1 - Reserved

Start of repeated block (N times)

12 + 1*N U1 - data - firmware-specific data

End of repeated block

32.15.5 UBX-MGA-FLASH (0x13 0x21)

32.15.5.1 UBX-MGA-FLASH-DATA

Message UBX-MGA-FLASH-DATA

Description Transfer MGA-ANO data block to flash

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message is used to transfer a block of MGA-ANO data from host to the

receiver. Upon reception of this message, the receiver will write the payload data

to its internal non-volatile memory (flash). Also, on reception of the first MGA-

FLASH-DATA message, the receiver will erase the flash allocated to storing any

existing MGA-ANO data. The payload can be up to 512 bytes. Payloads larger

than this would exceed the receiver's internal buffering capabilities. The receiver

will ACK/NACK this message using the message alternatives given below. The

host shall wait for an acknowledge message before sending the next data block.

See Flash-based AssistNow Offline for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x21 6 + 1*size see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U2 - sequence - Message sequence number, starting at 0

and increamenting by 1 for each MGA-

FLASH-DATA message sent.

4 U2 - size - Payload size in bytes.

Start of repeated block (size times)

6 + 1*N U1 - data - Payload data.

End of repeated block

UBX-13003221 - R28 Early production information Page 314 of 460

## Page 330

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.5.2 UBX-MGA-FLASH-STOP

Message UBX-MGA-FLASH-STOP

Description Finish flashing MGA-ANO data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message is used to tell the receiver that there are no more MGA-FLASH

type 1 messages coming, and that it can do any final internal operations needed

to commit the data to flash as a background activity. A UBX-MGA-ACK message

will be sent at the end of this process. Note that there may be a delay of several

seconds before the UBX-MGA-ACK for this message is sent because of the time

taken for this processing. See Flash-based AssistNow Offline for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x21 2 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this type)

1 U1 - version - Message version (0x00 for this version)

32.15.5.3 UBX-MGA-FLASH-ACK

Message UBX-MGA-FLASH-ACK

Description Acknowledge last FLASH-DATA or -STOP

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment This message reports an ACK/NACK to the host for the last MGA-FLASH type 1

or type 2 message message received. See Flash-based AssistNow Offline for

details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x21 6 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x03 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - ack - Acknowledgment type. 0 - ACK: Message

received and written to flash. 1 - NACK:

Problem with last message, re-

transmission required (this only happens

while acknowledging a UBX-MGA_FLASH_

DATA message). 2 - NACK: problem with

last message, give up.

UBX-13003221 - R28 Early production information Page 315 of 460

## Page 331

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-FLASH continued

Byte Offset Number Scaling Name Unit Description

Format

3 U1 - reserved1 - Reserved

4 U2 - sequence - If acknowledging a UBX-MGA-FLASH-

DATA message this is the Message

sequence number being ack'ed. If

acknowledging a UBX-MGA-FLASH-STOP

message it will be set to 0xffff.

32.15.6 UBX-MGA-GAL (0x13 0x02)

32.15.6.1 UBX-MGA-GAL-EPH

Message UBX-MGA-GAL-EPH

Description Galileo ephemeris assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of Galileo ephemeris assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x02 76 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - Galileo Satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U2 - iodNav - Ephemeris and clock correction Issue of

Data

6 I2 2^-43 deltaN semi- Mean motion difference from computed

circles value

/s

8 I4 2^-31 m0 semi- Mean anomaly at reference time

circles

12 U4 2^-33 e - Eccentricity

16 U4 2^-19 sqrtA m^0.5 Square root of the semi-major axis

20 I4 2^-31 omega0 semi- Longitude of ascending node of orbital

circles plane at weekly epoch

24 I4 2^-31 i0 semi- Inclination angle at reference time

circles

28 I4 2^-31 omega semi- Argument of perigee

circles

UBX-13003221 - R28 Early production information Page 316 of 460

## Page 332

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GAL continued

Byte Offset Number Scaling Name Unit Description

Format

32 I4 2^-43 omegaDot semi- Rate of change of right ascension

circles

/s

36 I2 2^-43 iDot semi- Rate of change of inclination angle

circles

/s

38 I2 2^-29 cuc radian Amplitude of the cosine harmonic

s correction term to the argument of

latitude

40 I2 2^-29 cus radian Amplitude of the sine harmonic correction

s term to the argument of latitude

42 I2 2^-5 crc radian Amplitude of the cosine harmonic

s correction term to the orbit radius

44 I2 2^-5 crs radian Amplitude of the sine harmonic correction

s term to the orbit radius

46 I2 2^-29 cic radian Amplitude of the cosine harmonic

s correction term to the angle of inclination

48 I2 2^-29 cis radian Amplitude of the sine harmonic correction

s term to the angle of inclination

50 U2 60 toe s Ephemeris reference time

52 I4 2^-34 af0 s SV clock bias correction coefficient

56 I4 2^-46 af1 s/s SV clock drift correction coefficient

60 I1 2^-59 af2 s/s SV clock drift rate correction coefficient

square

d

61 U1 - sisaIndexE1E5 - Signal-In-Space Accuracy index for dual

b frequency E1-E5b

62 U2 60 toc s Clock correction data reference Time of

Week

64 I2 2^-32 bgdE1E5b s E1-E5b Broadcast Group Delay

66 U1[2] - reserved2 - Reserved

68 U1 - healthE1B - E1-B Signal Health Status

69 U1 - dataValidityE - E1-B Data Validity Status

1B

70 U1 - healthE5b - E5b Signal Health Status

71 U1 - dataValidityE - E5b Data Validity Status

5b

72 U1[4] - reserved3 - Reserved

UBX-13003221 - R28 Early production information Page 317 of 460

## Page 333

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.6.2 UBX-MGA-GAL-ALM

Message UBX-MGA-GAL-ALM

Description Galileo almanac assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of Galileo almanac assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x02 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - Galileo Satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U1 - ioda - Almanac Issue of Data

5 U1 - almWNa week Almanac reference week number

6 U2 600 toa s Almanac reference time

8 I2 2^-9 deltaSqrtA m^0.5 Difference with respect to the square root

of the nominal semi-major axis (29 600

km)

10 U2 2^-16 e - Eccentricity

12 I2 2^-14 deltaI semi- Inclination at reference time relative to i0

circles = 56 degree

14 I2 2^-15 omega0 semi- Longitude of ascending node of orbital

circles plane at weekly epoch

16 I2 2^-33 omegaDot semi- Rate of change of right ascension

circles

/s

18 I2 2^-15 omega semi- Argument of perigee

circles

20 I2 2^-15 m0 semi- Satellite mean anomaly at reference time

circles

22 I2 2^-19 af0 s Satellite clock correction bias 'truncated'

24 I2 2^-38 af1 s/s Satellite clock correction linear 'truncated'

26 U1 - healthE1B - Satellite E1-B signal health status

27 U1 - healthE5b - Satellite E5b signal health status

28 U1[4] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 318 of 460

## Page 334

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.6.3 UBX-MGA-GAL-TIMEOFFSET

Message UBX-MGA-GAL-TIMEOFFSET

Description Galileo GPS time offset assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of Galileo time to GPS time offset.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x02 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x03 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I2 2^-35 a0G s Constant term of the polynomial

describing the offset

6 I2 2^-51 a1G s/s Rate of change of the offset

8 U1 3600 t0G s Reference time for GGTO data

9 U1 - wn0G weeks Week Number of GGTO reference

10 U1[2] - reserved2 - Reserved

32.15.6.4 UBX-MGA-GAL-UTC

Message UBX-MGA-GAL-UTC

Description Galileo UTC assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of Galileo UTC assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x02 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x05 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 2^-30 a0 s First parameter of UTC polynomial

8 I4 2^-50 a1 s/s Second parameter of UTC polynomial

12 I1 - dtLS s Delta time due to current leap seconds

UBX-13003221 - R28 Early production information Page 319 of 460

## Page 335

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GAL continued

Byte Offset Number Scaling Name Unit Description

Format

13 U1 3600 tot s UTC parameters reference time of week

(Galileo time)

14 U1 - wnt weeks UTC parameters reference week number

(the 8-bit WNt field)

15 U1 - wnLSF weeks Week number at the end of which the

future leap second becomes effective (the

8-bit WNLSF field)

16 U1 - dN days Day number at the end of which the future

leap second becomes effective

17 I1 - dTLSF s Delta time due to future leap seconds

18 U1[2] - reserved2 - Reserved

32.15.7 UBX-MGA-GLO (0x13 0x06)

32.15.7.1 UBX-MGA-GLO-EPH

Message UBX-MGA-GLO-EPH

Description GLONASS ephemeris assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GLONASS ephemeris assistance to a

receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x06 48 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - GLONASS Satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U1 - FT - User range accuracy

5 U1 - B - Health flag from string 2

6 U1 - M - Type of GLONASS satellite (1 indicates

GLONASS-M)

7 I1 - H - Carrier frequency number of navigation RF

signal, Range=(-7 .. 6), -128 for unknown

8 I4 2^-11 x km X component of the SV position in PZ-90.

02 coordinate System

12 I4 2^-11 y km Y component of the SV position in PZ-90.

02 coordinate System

UBX-13003221 - R28 Early production information Page 320 of 460

## Page 336

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GLO continued

Byte Offset Number Scaling Name Unit Description

Format

16 I4 2^-11 z km Z component of the SV position in PZ-90.

02 coordinate System

20 I4 2^-20 dx km/s X component of the SV velocity in PZ-90.

02 coordinate System

24 I4 2^-20 dy km/s Y component of the SV velocity in PZ-90.

02 coordinate System

28 I4 2^-20 dz km/s Z component of the SV velocity in PZ-90.

02 coordinate System

32 I1 2^-30 ddx km/s^ X component of the SV acceleration in PZ-

2 90.02 coordinate System

33 I1 2^-30 ddy km/s^ Y component of the SV acceleration in PZ-

2 90.02 coordinate System

34 I1 2^-30 ddz km/s^ Z component of the SV acceleration in PZ-

2 90.02 coordinate System

35 U1 15 tb minut Index of a time interval within current day

es according to UTC(SU)

36 I2 2^-40 gamma - Relative carrier frequency deviation

38 U1 - E days Ephemeris data age indicator

39 I1 2^-30 deltaTau s Time difference between L2 and L1 band

40 I4 2^-30 tau s SV clock bias

44 U1[4] - reserved2 - Reserved

32.15.7.2 UBX-MGA-GLO-ALM

Message UBX-MGA-GLO-ALM

Description GLONASS almanac assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GLONASS almanac assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x06 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - GLONASS Satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 321 of 460

## Page 337

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GLO continued

Byte Offset Number Scaling Name Unit Description

Format

4 U2 - N days Reference calender day number of

almanac within the four-year period (from

string 5)

6 U1 - M - Type of GLONASS satellite (1 indicates

GLONASS-M)

7 U1 - C - Unhealthy flag at instant of almanac

upload (1 indicates operability of satellite)

8 I2 2^-18 tau s Coarse time correction to GLONASS time

10 U2 2^-20 epsilon - Eccentricity

12 I4 2^-20 lambda semi- Longitude of the first (within the N-day)

circles ascending node of satellite orbit in PC-90.

02 coordinate system

16 I4 2^-20 deltaI semi- Correction to the mean value of inclination

circles

20 U4 2^-5 tLambda s Time of the first ascending node passage

24 I4 2^-9 deltaT s/orbit Correction to the mean value of Draconian

al- period

period

28 I1 2^-14 deltaDT s/orbit Rate of change of Draconian period

al-

period

^2

29 I1 - H - Carrier frequency number of navigation RF

signal, Range=(-7 .. 6)

30 I2 - omega - Argument of perigee

32 U1[4] - reserved2 - Reserved

32.15.7.3 UBX-MGA-GLO-TIMEOFFSET

Message UBX-MGA-GLO-TIMEOFFSET

Description GLONASS auxiliary time offset assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of auxiliary GLONASS assistance (including the

GLONASS time offsets to other GNSS systems) to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x06 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x03 for this type)

UBX-13003221 - R28 Early production information Page 322 of 460

## Page 338

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GLO continued

Byte Offset Number Scaling Name Unit Description

Format

1 U1 - version - Message version (0x00 for this version)

2 U2 - N days Reference calendar day number within the

four-year period of almanac (from string 5)

4 I4 2^-27 tauC s Time scale correction to UTC(SU) time

8 I4 2^-31 tauGps s Correction to GPS time relative to

GLONASS time

12 I2 2^-10 B1 s Coefficient to determine delta UT1

14 I2 2^-16 B2 s/msd Rate of change of delta UT1

16 U1[4] - reserved1 - Reserved

32.15.8 UBX-MGA-GPS (0x13 0x00)

32.15.8.1 UBX-MGA-GPS-EPH

Message UBX-MGA-GPS-EPH

Description GPS ephemeris assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GPS ephemeris assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x00 68 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - GPS Satellite identifier (see Satellite

Numbering)

3 U1 - reserved1 - Reserved

4 U1 - fitInterval - Fit interval flag

5 U1 - uraIndex - URA index

6 U1 - svHealth - SV health

7 I1 2^-31 tgd s Group delay differential

8 U2 - iodc - IODC

10 U2 2^4 toc s Clock data reference time

12 U1 - reserved2 - Reserved

13 I1 2^-55 af2 s/s Time polynomial coefficient 2

square

d

14 I2 2^-43 af1 s/s Time polynomial coefficient 1

16 I4 2^-31 af0 s Time polynomial coefficient 0

20 I2 2^-5 crs m Crs

UBX-13003221 - R28 Early production information Page 323 of 460

## Page 339

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GPS continued

Byte Offset Number Scaling Name Unit Description

Format

22 I2 2^-43 deltaN semi- Mean motion difference from computed

circles value

/s

24 I4 2^-31 m0 semi- Mean anomaly at reference time

circles

28 I2 2^-29 cuc radian Amplitude of cosine harmonic correction

s term to argument of latitude

30 I2 2^-29 cus radian Amplitude of sine harmonic correction

s term to argument of latitude

32 U4 2^-33 e - Eccentricity

36 U4 2^-19 sqrtA m^0.5 Square root of the semi-major axis

40 U2 2^4 toe s Reference time of ephemeris

42 I2 2^-29 cic radian Amplitude of cos harmonic correction

s term to angle of inclination

44 I4 2^-31 omega0 semi- Longitude of ascending node of orbit

circles plane at weekly epoch

48 I2 2^-29 cis radian Amplitude of sine harmonic correction

s term to angle of inclination

50 I2 2^-5 crc m Amplitude of cosine harmonic correction

term to orbit radius

52 I4 2^-31 i0 semi- Inclination angle at reference time

circles

56 I4 2^-31 omega semi- Argument of perigee

circles

60 I4 2^-43 omegaDot semi- Rate of right ascension

circles

/s

64 I2 2^-43 idot semi- Rate of inclination angle

circles

/s

66 U1[2] - reserved3 - Reserved

UBX-13003221 - R28 Early production information Page 324 of 460

## Page 340

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.8.2 UBX-MGA-GPS-ALM

Message UBX-MGA-GPS-ALM

Description GPS almanac assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GPS almanac assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x00 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - GPS Satellite identifier (see Satellite

Numbering)

3 U1 - svHealth - SV health information

4 U2 2^-21 e - Eccentricity

6 U1 - almWNa week Reference week number of almanac (the

8-bit WNa field)

7 U1 2^12 toa s Reference time of almanac

8 I2 2^-19 deltaI semi- Delta inclination angle at reference time

circles

10 I2 2^-38 omegaDot semi- Rate of right ascension

circles

/s

12 U4 2^-11 sqrtA m^0.5 Square root of the semi-major axis

16 I4 2^-23 omega0 semi- Longitude of ascending node of orbit

circles plane

20 I4 2^-23 omega semi- Argument of perigee

circles

24 I4 2^-23 m0 semi- Mean anomaly at reference time

circles

28 I2 2^-20 af0 s Time polynomial coefficient 0 (8 MSBs)

30 I2 2^-38 af1 s/s Time polynomial coefficient 1

32 U1[4] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 325 of 460

## Page 341

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.8.3 UBX-MGA-GPS-HEALTH

Message UBX-MGA-GPS-HEALTH

Description GPS health assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GPS health assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x00 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x04 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 U1[32] - healthCode - Each byte represents a GPS SV (1-32). The

6 LSBs of each byte contains the 6 bit

health code from subframes 4/5 page 25.

36 U1[4] - reserved2 - Reserved

32.15.8.4 UBX-MGA-GPS-UTC

Message UBX-MGA-GPS-UTC

Description GPS UTC assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GPS UTC assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x00 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x05 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 2^-30 utcA0 s First parameter of UTC polynomial

8 I4 2^-50 utcA1 s/s Second parameter of UTC polynomial

12 I1 - utcDtLS s Delta time due to current leap seconds

13 U1 2^12 utcTot s UTC parameters reference time of week

(GPS time)

UBX-13003221 - R28 Early production information Page 326 of 460

## Page 342

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GPS continued

Byte Offset Number Scaling Name Unit Description

Format

14 U1 - utcWNt weeks UTC parameters reference week number

(the 8-bit WNt field)

15 U1 - utcWNlsf weeks Week number at the end of which the

future leap second becomes effective (the

8-bit WNLSF field)

16 U1 - utcDn days Day number at the end of which the future

leap second becomes effective

17 I1 - utcDtLSF s Delta time due to future leap seconds

18 U1[2] - reserved2 - Reserved

32.15.8.5 UBX-MGA-GPS-IONO

Message UBX-MGA-GPS-IONO

Description GPS ionosphere assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of GPS ionospheric assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x00 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x06 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I1 2^-30 ionoAlpha0 s Ionospheric parameter alpha0 [s]

5 I1 2^-27 ionoAlpha1 s/semi- Ionospheric parameter alpha1 [s/semi-

circle circle]

6 I1 2^-24 ionoAlpha2 s/(sem Ionospheric parameter alpha2 [s/semi-

i- circle^2]

circle^

2)

7 I1 2^-24 ionoAlpha3 s/(sem Ionospheric parameter alpha3 [s/semi-

i- circle^3]

circle^

3)

8 I1 2^11 ionoBeta0 s Ionospheric parameter beta0 [s]

9 I1 2^14 ionoBeta1 s/semi- Ionospheric parameter beta1 [s/semi-

circle circle]

UBX-13003221 - R28 Early production information Page 327 of 460

## Page 343

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-GPS continued

Byte Offset Number Scaling Name Unit Description

Format

10 I1 2^16 ionoBeta2 s/(sem Ionospheric parameter beta2 [s/semi-

i- circle^2]

circle^

2)

11 I1 2^16 ionoBeta3 s/(sem Ionospheric parameter beta3 [s/semi-

i- circle^3]

circle^

3)

12 U1[4] - reserved2 - Reserved

32.15.9 UBX-MGA-INI (0x13 0x40)

32.15.9.1 UBX-MGA-INI-POS_XYZ

Message UBX-MGA-INI-POS_XYZ

Description Initial position assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying position assistance that is inaccurate by more than the specified

position accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of initial position assistance to a receiver in

cartesian ECEF coordinates. This message is equivalent to the UBX-MGA-INI-

POS_LLH message, except for the coordinate system.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x00 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 - ecefX cm WGS84 ECEF X coordinate

8 I4 - ecefY cm WGS84 ECEF Y coordinate

12 I4 - ecefZ cm WGS84 ECEF Z coordinate

16 U4 - posAcc cm Position accuracy (stddev)

UBX-13003221 - R28 Early production information Page 328 of 460

## Page 344

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.9.2 UBX-MGA-INI-POS_LLH

Message UBX-MGA-INI-POS_LLH

Description Initial position assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying position assistance that is inaccurate by more than the specified

position accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of initial position assistance to a receiver in

WGS84 lat/long/alt coordinates. This message is equivalent to the UBX-MGA-

INI-POS_XYZ message, except for the coordinate system.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 1e-7 lat deg WGS84 Latitude

8 I4 1e-7 lon deg WGS84 Longitude

12 I4 - alt cm WGS84 Altitude

16 U4 - posAcc cm Position accuracy (stddev)

32.15.9.3 UBX-MGA-INI-TIME_UTC

Message UBX-MGA-INI-TIME_UTC

Description Initial time assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying time assistance that is inaccurate by more than the specified time

accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of UTC time assistance to a receiver. This

message is equivalent to the UBX-MGA-INI-TIME_GNSS message, except for the

time base.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 24 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x10 for this type)

UBX-13003221 - R28 Early production information Page 329 of 460

## Page 345

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-INI continued

Byte Offset Number Scaling Name Unit Description

Format

1 U1 - version - Message version (0x00 for this version)

2 X1 - ref - Reference to be used to set time (see

graphic below)

3 I1 - leapSecs s Number of leap seconds since 1980 (or

0x80 = -128 if unknown)

4 U2 - year - Year

6 U1 - month - Month, starting at 1

7 U1 - day - Day, starting at 1

8 U1 - hour - Hour, from 0 to 23

9 U1 - minute - Minute, from 0 to 59

10 U1 - second s Seconds, from 0 to 59

11 X1 - bitfield0 - bitfield:

12 U4 - ns ns Nanoseconds, from 0 to 999,999,999

16 U2 - tAccS s Seconds part of time accuracy

18 U1[2] - reserved1 - Reserved

20 U4 - tAccNs ns Nanoseconds part of time accuracy, from

0 to 999,999,999

Bitfield ref

This graphic explains the bits of ref

Name Description

source 0: none, i.e. on receipt of message (will be inaccurate!)

1: relative to pulse sent to EXTINT0

2: relative to pulse sent to EXTINT1

3-15: reserved

fall use falling edge of EXTINT pulse (default rising) - only if source is EXTINT

last use last EXTINT pulse (default next pulse) - only if source is EXTINT

UBX-13003221 - R28 Early production information Page 330 of 460

## Page 346

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.9.4 UBX-MGA-INI-TIME_GNSS

Message UBX-MGA-INI-TIME_GNSS

Description Initial time assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying time assistance that is inaccurate by more than the specified time

accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of time assistance to a receiver in a chosen

GNSS timebase. This message is equivalent to the UBX-MGA-INI-TIME_UTC

message, except for the time base.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 24 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x11 for this type)

1 U1 - version - Message version (0x00 for this version)

2 X1 - ref - Reference to be used to set time (see

graphic below)

3 U1 - gnssId - Source of time information. Currently

supported:

0: GPS time

2: Galileo time

3: BeiDou time

6: GLONASS time

7: NavIC time

4 X1 - bitfield0 - bitfield:

5 U1 - reserved1 - Reserved

6 U2 - week - GNSS week number

8 U4 - tow s GNSS time of week

12 U4 - ns ns GNSS time of week, nanosecond part from

0 to 999,999,999

16 U2 - tAccS s Seconds part of time accuracy

18 U1[2] - reserved2 - Reserved

20 U4 - tAccNs ns Nanoseconds part of time accuracy, from

0 to 999,999,999

UBX-13003221 - R28 Early production information Page 331 of 460

## Page 347

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield ref

This graphic explains the bits of ref

Name Description

source 0: none, i.e. on receipt of message (will be inaccurate!)

1: relative to pulse sent to EXTINT0

2: relative to pulse sent to EXTINT1

3-15: reserved

fall use falling edge of EXTINT pulse (default rising) - only if source is EXTINT

last use last EXTINT pulse (default next pulse) - only if source is EXTINT

32.15.9.5 UBX-MGA-INI-CLKD

Message UBX-MGA-INI-CLKD

Description Initial clock drift assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying clock drift assistance that is inaccurate by more than the specified

accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of clock drift assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x20 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 I4 - clkD ns/s Clock drift

8 U4 - clkDAcc ns/s Clock drift accuracy

UBX-13003221 - R28 Early production information Page 332 of 460

## Page 348

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.9.6 UBX-MGA-INI-FREQ

Message UBX-MGA-INI-FREQ

Description Initial frequency assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment Supplying external frequency assistance that is inaccurate by more than the

specified accuracy, may lead to substantially degraded receiver performance.

This message allows the delivery of external frequency assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x21 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - reserved1 - Reserved

3 X1 - flags - Frequency reference (see graphic below)

4 I4 1e-2 freq Hz Frequency

8 U4 - freqAcc ppb Frequency accuracy

Bitfield flags

This graphic explains the bits of flags

Name Description

source 0: frequency available on EXTINT0

1: frequency available on EXTINT1

2-15: reserved

fall use falling edge of EXTINT pulse (default rising)

UBX-13003221 - R28 Early production information Page 333 of 460

## Page 349

u-blox 8 / u-blox M8 Receiver description - Manual

32.15.9.7 UBX-MGA-INI-EOP

Message UBX-MGA-INI-EOP

Description Earth orientation parameters assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of new earth orientation parameters (EOP) to a

receiver to improve AssistNow Autonomous operation.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x40 72 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x30 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 U2 - d2kRef d reference time (days since 1.1.2000 12.00h

UTC)

6 U2 - d2kMax d expiration time (days since 1.1.2000 12.00h

UTC)

8 I4 2^-30 xpP0 arcsec x_p t^0 polynomial term (offset)

12 I4 2^-30 xpP1 arcsec x_p t^1 polynomial term (drift)

/d

16 I4 2^-30 ypP0 arcsec y_p t^0 polynomial term (offset)

20 I4 2^-30 ypP1 arcsec y_p t^1 polynomial term (drift)

/d

24 I4 2^-25 dUT1 s dUT1 t^0 polynomial term (offset)

28 I4 2^-30 ddUT1 s/d dUT1 t^1 polynomial term (drift)

32 U1[40] - reserved2 - Reserved

32.15.10 UBX-MGA-QZSS (0x13 0x05)

32.15.10.1 UBX-MGA-QZSS-EPH

Message UBX-MGA-QZSS-EPH

Description QZSS ephemeris assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of QZSS ephemeris assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x05 68 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 334 of 460

## Page 350

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-QZSS continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x01 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - QZSS Satellite identifier (see Satellite

Numbering), Range 1-5

3 U1 - reserved1 - Reserved

4 U1 - fitInterval - Fit interval flag

5 U1 - uraIndex - URA index

6 U1 - svHealth - SV health

7 I1 2^-31 tgd s Group delay differential

8 U2 - iodc - IODC

10 U2 2^4 toc s Clock data reference time

12 U1 - reserved2 - Reserved

13 I1 2^-55 af2 s/s Time polynomial coefficient 2

square

d

14 I2 2^-43 af1 s/s Time polynomial coefficient 1

16 I4 2^-31 af0 s Time polynomial coefficient 0

20 I2 2^-5 crs m Crs

22 I2 2^-43 deltaN semi- Mean motion difference from computed

circles value

/s

24 I4 2^-31 m0 semi- Mean anomaly at reference time

circles

28 I2 2^-29 cuc radian Amp of cosine harmonic corr term to arg

s of lat

30 I2 2^-29 cus radian Amp of sine harmonic corr term to arg of

s lat

32 U4 2^-33 e - eccentricity

36 U4 2^-19 sqrtA m^0.5 Square root of the semi-major axis A

40 U2 2^4 toe s Reference time of ephemeris

42 I2 2^-29 cic radian Amp of cos harmonic corr term to angle of

s inclination

44 I4 2^-31 omega0 semi- Long of asc node of orbit plane at weekly

circles epoch

48 I2 2^-29 cis radian Amp of sine harmonic corr term to angle

s of inclination

50 I2 2^-5 crc m Amp of cosine harmonic corr term to orbit

radius

52 I4 2^-31 i0 semi- Inclination angle at reference time

circles

56 I4 2^-31 omega semi- Argument of perigee

circles

UBX-13003221 - R28 Early production information Page 335 of 460

## Page 351

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-QZSS continued

Byte Offset Number Scaling Name Unit Description

Format

60 I4 2^-43 omegaDot semi- Rate of right ascension

circles

/s

64 I2 2^-43 idot semi- Rate of inclination angle

circles

/s

66 U1[2] - reserved3 - Reserved

32.15.10.2 UBX-MGA-QZSS-ALM

Message UBX-MGA-QZSS-ALM

Description QZSS almanac assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of QZSS almanac assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x05 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x02 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1 - svId - QZSS Satellite identifier (see Satellite

Numbering), Range 1-5

3 U1 - svHealth - Almanac SV health information

4 U2 2^-21 e - Almanac eccentricity

6 U1 - almWNa week Reference week number of almanac (the

8-bit WNa field)

7 U1 2^12 toa s Reference time of almanac

8 I2 2^-19 deltaI semi- Delta inclination angle at reference time

circles

10 I2 2^-38 omegaDot semi- Almanac rate of right ascension

circles

/s

12 U4 2^-11 sqrtA m^0.5 Almanac square root of the semi-major

axis A

16 I4 2^-23 omega0 semi- Almanac long of asc node of orbit plane at

circles weekly

20 I4 2^-23 omega semi- Almanac argument of perigee

circles

UBX-13003221 - R28 Early production information Page 336 of 460

## Page 352

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MGA-QZSS continued

Byte Offset Number Scaling Name Unit Description

Format

24 I4 2^-23 m0 semi- Almanac mean anomaly at reference time

circles

28 I2 2^-20 af0 s Almanac time polynomial coefficient 0 (8

MSBs)

30 I2 2^-38 af1 s/s Almanac time polynomial coefficient 1

32 U1[4] - reserved1 - Reserved

32.15.10.3 UBX-MGA-QZSS-HEALTH

Message UBX-MGA-QZSS-HEALTH

Description QZSS health assistance

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Input

Comment This message allows the delivery of QZSS health assistance to a receiver.

See the description of AssistNow Online for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x13 0x05 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0x04 for this type)

1 U1 - version - Message version (0x00 for this version)

2 U1[2] - reserved1 - Reserved

4 U1[5] - healthCode - Each byte represents a QZSS SV (1-5). The

6 LSBs of each byte contains the 6 bit

health code from subframes 4/5, data ID =

3, SV ID = 51

9 U1[3] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 337 of 460

## Page 353

u-blox 8 / u-blox M8 Receiver description - Manual

32.16 UBX-MON (0x0A)

Monitoring Messages: i.e. Communication Status, Stack Usage, Task Status.

Messages in the MON class are used to report the receiver status, such as stack usage, I/O

subsystem statistics etc.

32.16.1 UBX-MON-BATCH (0x0A 0x32)

32.16.1.1 Data batching buffer status

Message UBX-MON-BATCH

Description Data batching buffer status

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 23.01

Type Polled

Comment This message contains status information about the batching buffer.

It can be polled and it can also be sent by the receiver as a response to a UBX-

LOG-RETRIEVEBATCH message before the UBX-LOG-BATCH messages.

See Data Batching for more information.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x32 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U2 - fillLevel - Current buffer fill level, i.e. number of

epochs currently stored

6 U2 - dropsAll - Number of dropped epochs since startup

Note: changing the batching configuration

will reset this counter.

8 U2 - dropsSinceMon - Number of dropped epochs since last

MON-BATCH message

10 U2 - nextMsgCnt - The next retrieved UBX-LOG-BATCH will

have this msgCnt value.

UBX-13003221 - R28 Early production information Page 338 of 460

## Page 354

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.2 UBX-MON-GNSS (0x0A 0x28)

32.16.2.1 Information message major GNSS selection

Message UBX-MON-GNSS

Description Information message major GNSS selection

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Polled

Comment This message reports major GNSS selection. It does this by means of bit masks

in U1 fields. Each bit in a bit mask corresponds to one major GNSS.

Augmentation systems are not reported.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x28 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 X1 - supported - A bit mask showing the major GNSS that

can be supported by this receiver (see

graphic below)

2 X1 - defaultGnss - A bit mask showing the default major

GNSS selection. If the default major GNSS

selection is currently configured in the

efuse for this receiver, it takes precedence

over the default major GNSS selection

configured in the executing firmware of

this receiver. (see graphic below)

3 X1 - enabled - A bit mask showing the current major

GNSS selection enabled for this receiver

(see graphic below)

4 U1 - simultaneous - Maximum number of concurrent major

GNSS that can be supported by this

receiver

5 U1[3] - reserved1 - Reserved

Bitfield supported

This graphic explains the bits of supported

UBX-13003221 - R28 Early production information Page 339 of 460

## Page 355

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

GPSSup GPS is supported

GlonassSup GLONASS is supported

BeidouSup BeiDou is supported

GalileoSup Galileo is supported

Bitfield defaultGnss

This graphic explains the bits of defaultGnss

Name Description

GPSDef GPS is default-enabled

GlonassDef GLONASS is default-enabled

BeidouDef BeiDou is default-enabled

GalileoDef Galileo is default-enabled

Bitfield enabled

This graphic explains the bits of enabled

Name Description

GPSEna GPS is enabled

GlonassEna GLONASS is enabled

BeidouEna BeiDou is enabled

GalileoEna Galileo is enabled

UBX-13003221 - R28 Early production information Page 340 of 460

## Page 356

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.3 UBX-MON-HW2 (0x0A 0x0B)

32.16.3.1 Extended hardware status

Message UBX-MON-HW2

Description Extended hardware status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Status of different aspects of the hardware such as Imbalance, Low-Level

Configuration and POST Results.

The first four parameters of this message represent the complex signal from

the RF front end. The following rules of thumb apply:

• The smaller the absolute value of the variable ofsI and ofsQ, the better.

• Ideally, the magnitude of the I-part (magI) and the Q-part (magQ) of the

complex signal should be the same.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x0B 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 I1 - ofsI - Imbalance of I-part of complex signal,

scaled (-128 = max. negative imbalance,

127 = max. positive imbalance)

1 U1 - magI - Magnitude of I-part of complex signal,

scaled (0 = no signal, 255 = max.

magnitude)

2 I1 - ofsQ - Imbalance of Q-part of complex signal,

scaled (-128 = max. negative imbalance,

127 = max. positive imbalance)

3 U1 - magQ - Magnitude of Q-part of complex signal,

scaled (0 = no signal, 255 = max.

magnitude)

4 U1 - cfgSource - Source of low-level configuration

(114 = ROM, 111 = OTP, 112 = config pins,

102 = flash image)

5 U1[3] - reserved1 - Reserved

8 U4 - lowLevCfg - Low-level configuration (obsolete in

protocol versions greater than 15)

12 U1[8] - reserved2 - Reserved

20 U4 - postStatus - POST status word

24 U1[4] - reserved3 - Reserved

UBX-13003221 - R28 Early production information Page 341 of 460

## Page 357

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.4 UBX-MON-HW (0x0A 0x09)

32.16.4.1 Hardware status

Message UBX-MON-HW

Description Hardware status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/polled

Comment Status of different aspects of the hardware, such as antenna, PIO/peripheral

pins, noise level, automatic gain control (AGC)

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x09 60 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X4 - pinSel - Mask of pins set as peripheral/PIO

4 X4 - pinBank - Mask of pins set as bank A/B

8 X4 - pinDir - Mask of pins set as input/output

12 X4 - pinVal - Mask of pins value low/high

16 U2 - noisePerMS - Noise level as measured by the GPS core

18 U2 - agcCnt - AGC Monitor, as percentage of maximum

gain,range 0 to 8191 (100%)

20 U1 - aStatus - Status of the antenna supervisor state

machine (0=INIT, 1=DONTKNOW, 2=OK,

3=SHORT, 4=OPEN)

21 U1 - aPower - Current power status of antenna (0=OFF,

1=ON, 2=DONTKNOW)

22 X1 - flags - Flags (see graphic below)

23 U1 - reserved1 - Reserved

24 X4 - usedMask - Mask of pins that are used by the virtual

pin manager

28 U1[17] - VP - Array of pin mappings for each of the 17

physical pins

45 U1 - cwSuppression - CW interference suppression level, scaled

(0 = no CW jamming, 255 = strong CW

jamming)

46 U1[2] - reserved2 - Reserved

48 X4 - pinIrq - Mask of pins value using the PIO Irq

52 X4 - pullH - Mask of pins value using the PIO pull high

resistor

56 X4 - pullL - Mask of pins value using the PIO pull low

resistor

UBX-13003221 - R28 Early production information Page 342 of 460

## Page 358

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

rtcCalib RTC is calibrated

safeBoot Safeboot mode (0 = inactive, 1 = active)

jammingState Output from jamming/interference monitor (0 = unknown or feature disabled or flag unavailable, 1 =

ok - no significant jamming, 2 = warning - interference visible but fix OK, 3 = critical - interference

visible and no fix). This flag is deprecated in protocol versions that support UBX-SEC-SIG (version

0x02) and always reported as 0; instead jammingState in UBX-SEC-SIG should be monitored.

xtalAbsent RTC xtal has been determined to be absent (not supported in protocol versions less than 18)

32.16.5 UBX-MON-IO (0x0A 0x02)

32.16.5.1 I/O system status

Message UBX-MON-IO

Description I/O system status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment The size of the message is determined by the number of ports 'N' the receiver

supports, i.e. on u-blox 5 the number of ports is 6.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x02 0 + 20*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (N times)

N*20 U4 - rxBytes bytes Number of bytes ever received

4 + 20*N U4 - txBytes bytes Number of bytes ever sent

8 + 20*N U2 - parityErrs - Number of 100 ms timeslots with parity

errors

10 + 20*N U2 - framingErrs - Number of 100 ms timeslots with framing

errors

12 + 20*N U2 - overrunErrs - Number of 100 ms timeslots with overrun

errors

14 + 20*N U2 - breakCond - Number of 100 ms timeslots with break

conditions

16 + 20*N U1[4] - reserved1 - Reserved

End of repeated block

UBX-13003221 - R28 Early production information Page 343 of 460

## Page 359

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.6 UBX-MON-MSGPP (0x0A 0x06)

32.16.6.1 Message parse and process status

Message UBX-MON-MSGPP

Description Message parse and process status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x06 120 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2[8] - msg1 msgs Number of successfully parsed messages

for each protocol on port0

16 U2[8] - msg2 msgs Number of successfully parsed messages

for each protocol on port1

32 U2[8] - msg3 msgs Number of successfully parsed messages

for each protocol on port2

48 U2[8] - msg4 msgs Number of successfully parsed messages

for each protocol on port3

64 U2[8] - msg5 msgs Number of successfully parsed messages

for each protocol on port4

80 U2[8] - msg6 msgs Number of successfully parsed messages

for each protocol on port5

96 U4[6] - skipped bytes Number skipped bytes for each port

32.16.7 UBX-MON-PATCH (0x0A 0x27)

32.16.7.1 Poll request for installed patches

Message UBX-MON-PATCH

Description Poll request for installed patches

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x27 0 see below CK_A CK_B

No payload

UBX-13003221 - R28 Early production information Page 344 of 460

## Page 360

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.7.2 Installed patches

Message UBX-MON-PATCH

Description Installed patches

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Polled

Comment This message reports information about patches installed and currently enabled

on the receiver. It does not report on patches installed and then disabled. An

enabled patch is considered active when the receiver executes from the code

space where the patch resides on. For example, a ROM patch is reported active

only when the system runs from ROM.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x27 4 + 16*nEntries see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2 - version - Message version (0x0001 for this version)

2 U2 - nEntries - Total number of reported patches

Start of repeated block (nEntries times)

4 + 16*N X4 - patchInfo - Status information about the reported

patch (see graphic below)

8 + 16*N U4 - comparatorNum - The number of the comparator

ber

12 + 16*N U4 - patchAddress - The address that is targeted by the patch

16 + 16*N U4 - patchData - The data that is inserted at the

patchAddress

End of repeated block

Bitfield patchInfo

This graphic explains the bits of patchInfo

UBX-13003221 - R28 Early production information Page 345 of 460

## Page 361

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

activated 1: the patch is active, 0: otherwise

location Indicates where the patch is stored. 0: eFuse, 1: ROM, 2: BBR, 3: file system

32.16.8 UBX-MON-RXBUF (0x0A 0x07)

32.16.8.1 Receiver buffer status

Message UBX-MON-RXBUF

Description Receiver buffer status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x07 24 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2[6] - pending bytes Number of bytes pending in receiver

buffer for each target

12 U1[6] - usage % Maximum usage receiver buffer during the

last sysmon period for each target

18 U1[6] - peakUsage % Maximum usage receiver buffer for each

target

32.16.9 UBX-MON-RXR (0x0A 0x21)

32.16.9.1 Receiver status information

Message UBX-MON-RXR

Description Receiver status information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The receiver ready message is sent when the receiver changes from or to backup

mode.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x21 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 X1 - flags - Receiver status flags (see graphic below)

UBX-13003221 - R28 Early production information Page 346 of 460

## Page 362

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

awake not in backup mode

32.16.10 UBX-MON-SMGR (0x0A 0x2E)

32.16.10.1 Synchronization manager status

Message UBX-MON-SMGR

Description Synchronization manager status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Periodic/Polled

Comment This message reports the status of internal and external oscillators and sources

as well as whether GNSS is used for disciplining.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x2E 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - iTOW ms Time of the week

8 X2 - intOsc - A bit mask, indicating the status of the

local oscillator (see graphic below)

10 X2 - extOsc - A bit mask, indicating the status of the

external oscillator (see graphic below)

12 U1 - discSrc - Disciplining source identifier:

0: internal oscillator

1: GNSS

2: EXTINT0

3: EXTINT1

4: internal oscillator measured by the host

5: external oscillator measured by the host

13 X1 - gnss - A bit mask, indicating the status of the

GNSS (see graphic below)

14 X1 - extInt0 - A bit mask, indicating the status of the

external input 0 (see graphic below)

15 X1 - extInt1 - A bit mask, indicating the status of the

external input 1 (see graphic below)

UBX-13003221 - R28 Early production information Page 347 of 460

## Page 363

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield intOsc

This graphic explains the bits of intOsc

Name Description

intOscState State of the oscillator:

0: autonomous operation

1: calibration ongoing

2: oscillator is steered by the host

3: idle state

intOscCalib 1 = oscillator gain is calibrated

intOscDisc 1 = signal is disciplined

Bitfield extOsc

This graphic explains the bits of extOsc

Name Description

extOscState State of the oscillator:

0: autonomous operation

1: calibration ongoing

2: oscillator is steered by the host

3: idle state

extOscCalib 1 = oscillator gain is calibrated

extOscDisc 1 = signal is disciplined

Bitfield gnss

This graphic explains the bits of gnss

UBX-13003221 - R28 Early production information Page 348 of 460

## Page 364

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

gnssAvail 1 = GNSS is present

Bitfield extInt0

This graphic explains the bits of extInt0

Name Description

extInt0Avail 1 = signal present at this input

extInt0Type Source type:

0: frequency

1: time

extInt0FeedBa This source is used as feedback of the external oscillator

ck

Bitfield extInt1

This graphic explains the bits of extInt1

Name Description

extInt1Avail 1 = signal present at this input

extInt1Type Source type:

0: frequency

1: time

extInt1FeedBa This source is used as feedback of the external oscillator

ck

UBX-13003221 - R28 Early production information Page 349 of 460

## Page 365

u-blox 8 / u-blox M8 Receiver description - Manual

32.16.11 UBX-MON-SPT (0x0A 0x2F)

32.16.11.1 Sensor production test

Message UBX-MON-SPT

Description Sensor production test

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Polled

Comment This message reports the state of, and measurements made during, sensor self-

tests.

This message can also be used to retrieve information about detected sensor(s)

and driver(s) used.

This message is only supported if a sensor is directly connected to the u-blox

chip. This includes modules that contain IMUs.

Note that this message shows the status of the last self-test since sensor

startup. The self-test results are not stored in non-volatile memory.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x2F 4 + 12*numRes + 4*numSensor see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1 - numSensor - number of sensors reported in this

message

2 U1 - numRes - number of result items reported in this

message

3 U1 - reserved1 - Reserved

Start of repeated block (numSensor times)

UBX-13003221 - R28 Early production information Page 350 of 460

## Page 366

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MON-SPT continued

Byte Offset Number Scaling Name Unit Description

Format

4 + 4*N U1 - sensorId - Sensor ID

The following IDs are defined, others are

reserved:

1: ST LSM6DS0 6-axis IMU with

temperature sensor

2: Invensense MPU6500 6-axis IMU with

temperature sensor

3: Bosch BMI160 6-axis IMU with

temperature sensor

7: ST LSM6DS3 6-axis IMU with

temperature sensor

9: Bosch SMI130 6-axis IMU with

temperature sensor

12: MPU6515, 6-axis inertial sensor from

Invensense

13: ST LSM6DSL 6-axis IMU with

temperature sensor

14: SMG130, 3-axis gyroscope with

temperature sensor from Bosch

15: SMI230, 6-axis IMU with temperature

sensor from Bosch

16: BMI260, 6-axis IMU with temperature

sensor from Bosch

17: ICM330DLC, 6-axis IMU with

temperature sensor from ST

18: LSM6DSR, 6-axis IMU with 85 deg

temperature sensor from ST

19: ICM42605, 6-axis IMU with 85 deg

temperature sensor from InvenSense TDK

20: IIM42652, 6-axis IMU with 105 deg

temperature sensor from InvenSense TDK

21: BMI320, 6-axis IMU with 85 deg

temperature sensor from Bosch

22: IAM20680HT, 6-axis IMU with 105 deg

temperature sensor from InvenSense TDK

23: LSM6DSOW, 6-axis IMU with 85 deg

temperature sensor from ST

Not all sensors are supported in any

released firmware. Refer to the release

notes to find out which sensor is

supported by a certain firmware.

5 + 4*N X1 - drvVer - Version information (see graphic below)

UBX-13003221 - R28 Early production information Page 351 of 460

## Page 367

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MON-SPT continued

Byte Offset Number Scaling Name Unit Description

Format

6 + 4*N U1 - testState - State of one sensor's test, it can be

0: test not yet started

1: test started but not yet finished

2: test did not finish due to error during

execution

3: test finished normally, test data is

available

7 + 4*N U1 - drvFileName - 0 if the active driver is loaded from image,

last character of the file name if it is

loaded from separate file.

End of repeated block

Start of repeated block (numRes times)

4 + 12*N + U2 - sensorIdRes - Sensor ID; eligible values are the same as

4*numSen in sensorIdState field

sor

6 + 12*N + U2 - sensorType - Sensor type and axis (if applicable) to

4*numSen which the result refers

sor The following values are defined, others

are reserved:

5: Gyroscope z axis

12: Gyroscope temperature

13: Gyroscope y axis

14: Gyroscope x axis

16: Accelerometer x axis

17: Accelerometer y axis

18: Accelerometer z axis

19: Barometer

22: Magnetometer x axis

23: Magnetometer y axis

24: Magnetometer z axis

25: Barometer temperature

UBX-13003221 - R28 Early production information Page 352 of 460

## Page 368

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MON-SPT continued

Byte Offset Number Scaling Name Unit Description

Format

8 + 12*N + U2 - resType - The type of result stored in the value field

4*numSen 1: Measurement without self-test offset

sor (raw and unscaled digital value)

2: Measurement with positive self-test

offset (raw and unscaled digital value)

3: Measurement with negative self-test

offset (raw and unscaled digital value)

4: Minimum off-to-positive to pass self-

test, as deduced from on-chip trimming

information

5: Maximum off-to-positive to pass self-

test, as deduced from on-chip trimming

information

6: Minimum negative-to-positive to pass

self-test, as deduced from on-chip

trimming information

7: Maximum negative-to-positive to pass

self-test, as deduced from on-chip

trimming information

8: Self-test passed; test passed if value = 1

and failed if 0. Used if the decision is read

out from the sensor itself.

10 + 12*N + U1[2] - reserved2 - Reserved

4*numSen

sor

12 + 12*N + I4 - value - value of the specific test result

4*numSen

sor

End of repeated block

Bitfield drvVer

This graphic explains the bits of drvVer

UBX-13003221 - R28 Early production information Page 353 of 460

## Page 369

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

drvVerMaj Driver major version

drvVerMin Driver minor version

32.16.12 UBX-MON-TXBUF (0x0A 0x08)

32.16.12.1 Transmitter buffer status

Message UBX-MON-TXBUF

Description Transmitter buffer status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x08 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U2[6] - pending bytes Number of bytes pending in transmitter

buffer for each target

12 U1[6] - usage % Maximum usage transmitter buffer during

the last sysmon period for each target

18 U1[6] - peakUsage % Maximum usage transmitter buffer for

each target

24 U1 - tUsage % Maximum usage of transmitter buffer

during the last sysmon period for all

targets

25 U1 - tPeakusage % Maximum usage of transmitter buffer for

all targets

26 X1 - errors - Error bitmask (see graphic below)

27 U1 - reserved1 - Reserved

Bitfield errors

This graphic explains the bits of errors

UBX-13003221 - R28 Early production information Page 354 of 460

## Page 370

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

limit Buffer limit of corresponding target reached

mem Memory Allocation error

alloc Allocation error (TX buffer full)

32.16.13 UBX-MON-VER (0x0A 0x04)

32.16.13.1 Poll receiver and software version

Message UBX-MON-VER

Description Poll receiver and software version

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x04 0 see below CK_A CK_B

No payload

32.16.13.2 Receiver and software version

Message UBX-MON-VER

Description Receiver and software version

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0A0x04 40 + 30*N see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 CH[30 - swVersion - Nul-terminated software version string.

]

30 CH[10] - hwVersion - Nul-terminated hardware version string

Start of repeated block (N times)

UBX-13003221 - R28 Early production information Page 355 of 460

## Page 371

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-MON-VER continued

Byte Offset Number Scaling Name Unit Description

Format

40 + 30*N CH[30 - extension - Extended software information strings.

] A series of nul-terminated strings. Each

extension field is 30 characters long and

contains varying software information.

Not all extension fields may appear.

Examples of reported information: the

software version string of the underlying

ROM (when the receiver's firmware is

running from flash), the firmware version,

the supported protocol version, the

module identifier, the flash information

structure (FIS) file information, the

supported major GNSS, the supported

augmentation systems.

See Firmware and protocol versions for

details.

End of repeated block

UBX-13003221 - R28 Early production information Page 356 of 460

## Page 372

u-blox 8 / u-blox M8 Receiver description - Manual

32.17 UBX-NAV (0x01)

Navigation Results Messages: i.e. Position, Speed, Time, Acceleration, Heading, DOP, SVs used.

Messages in the NAV class are used to output navigation data such as position, altitude and

velocity in a number of formats. Additionally, status flags and accuracy figures are output. The

messages are generated with the configured navigation/measurement rate.

32.17.1 UBX-NAV-AOPSTATUS (0x01 0x60)

32.17.1.1 AssistNow Autonomous status

Message UBX-NAV-AOPSTATUS

Description AssistNow Autonomous status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message provides information on the status of the AssistNow Autonomous

subsystem on the receiver. For example, a host application can determine the

optimal time to shut down the receiver by monitoring the status field for a

steady 0. See the chapter AssistNow Autonomous in the receiver description for

details on this feature.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x60 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - aopCfg - AssistNow Autonomous configuration

(see graphic below)

5 U1 - status - AssistNow Autonomous subsystem is idle

(0) or running (not 0)

6 U1[10] - reserved1 - Reserved

Bitfield aopCfg

This graphic explains the bits of aopCfg

UBX-13003221 - R28 Early production information Page 357 of 460

## Page 373

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

useAOP AOP enabled flag

32.17.2 UBX-NAV-ATT (0x01 0x05)

32.17.2.1 Attitude solution

Message UBX-NAV-ATT

Description Attitude solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3,

22, 22.01, 23 and 23.01 (only with ADR or UDR products)

Type Periodic/Polled

Comment This message outputs the attitude solution as roll, pitch and heading angles.

More details about vehicle attitude can be found in the Vehicle Attitude Output

(ADR) section for ADR products.

More details about vehicle attitude can be found in the Vehicle Attitude Output

(UDR) section for UDR products.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x05 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1[3] - reserved1 - Reserved

8 I4 1e-5 roll deg Vehicle roll.

12 I4 1e-5 pitch deg Vehicle pitch.

16 I4 1e-5 heading deg Vehicle heading.

20 U4 1e-5 accRoll deg Vehicle roll accuracy (if null, roll angle is

not available).

24 U4 1e-5 accPitch deg Vehicle pitch accuracy (if null, pitch angle

is not available).

28 U4 1e-5 accHeading deg Vehicle heading accuracy (if null, heading

angle is not available).

UBX-13003221 - R28 Early production information Page 358 of 460

## Page 374

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.3 UBX-NAV-CLOCK (0x01 0x22)

32.17.3.1 Clock solution

Message UBX-NAV-CLOCK

Description Clock solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment -

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x22 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - clkB ns Clock bias

8 I4 - clkD ns/s Clock drift

12 U4 - tAcc ns Time accuracy estimate

16 U4 - fAcc ps/s Frequency accuracy estimate

32.17.4 UBX-NAV-COV (0x01 0x36)

32.17.4.1 Covariance matrices

Message UBX-NAV-COV

Description Covariance matrices

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message outputs the covariance matrices for the position and velocity

solutions in the topocentric coordinate system defined as the local-level North

(N), East (E), Down (D) frame. As the covariance matrices are symmetric, only

the upper triangular part is output.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x36 64 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1 - posCovValid - Position covariance matrix validity flag

6 U1 - velCovValid - Velocity covariance matrix validity flag

7 U1[9] - reserved1 - Reserved

16 R4 - posCovNN m^2 Position covariance matrix value p_NN

UBX-13003221 - R28 Early production information Page 359 of 460

## Page 375

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-COV continued

Byte Offset Number Scaling Name Unit Description

Format

20 R4 - posCovNE m^2 Position covariance matrix value p_NE

24 R4 - posCovND m^2 Position covariance matrix value p_ND

28 R4 - posCovEE m^2 Position covariance matrix value p_EE

32 R4 - posCovED m^2 Position covariance matrix value p_ED

36 R4 - posCovDD m^2 Position covariance matrix value p_DD

40 R4 - velCovNN m^2/s Velocity covariance matrix value v_NN

^2

44 R4 - velCovNE m^2/s Velocity covariance matrix value v_NE

^2

48 R4 - velCovND m^2/s Velocity covariance matrix value v_ND

^2

52 R4 - velCovEE m^2/s Velocity covariance matrix value v_EE

^2

56 R4 - velCovED m^2/s Velocity covariance matrix value v_ED

^2

60 R4 - velCovDD m^2/s Velocity covariance matrix value v_DD

^2

32.17.5 UBX-NAV-DGPS (0x01 0x31)

32.17.5.1 DGPS data used for NAV

Message UBX-NAV-DGPS

Description DGPS data used for NAV

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message outputs the DGPS correction data that has been applied to the

current NAV Solution. See also the notes on the RTCM protocol.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x31 16 + 12*numCh see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - age ms Age of newest correction data

8 I2 - baseId - DGPS base station identifier

10 I2 - baseHealth - DGPS base station health status

12 U1 - numCh - Number of channels for which correction

data is following

13 U1 - status - DGPS correction type status:

0x00: none

0x01: PR+PRR correction

UBX-13003221 - R28 Early production information Page 360 of 460

## Page 376

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-DGPS continued

Byte Offset Number Scaling Name Unit Description

Format

14 U1[2] - reserved1 - Reserved

Start of repeated block (numCh times)

16 + 12*N U1 - svid - Satellite ID

17 + 12*N X1 - flags - Channel number and usage (see graphic

below)

18 + 12*N U2 - ageC ms Age of latest correction data

20 + 12*N R4 - prc m Pseudorange correction

24 + 12*N R4 - prrc m/s Pseudorange rate correction

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

channel GPS channel number this SV is on. Channel numbers in the firmware greater than 15 are displayed as

having channel number 15

dgpsUsed 1 = DGPS used for this SV

32.17.6 UBX-NAV-DOP (0x01 0x04)

32.17.6.1 Dilution of precision

Message UBX-NAV-DOP

Description Dilution of precision

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment • DOP values are dimensionless.

• All DOP values are scaled by a factor of 100. If the unit transmits a value of e.g.

156, the DOP value is 1.56.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x04 18 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U2 0.01 gDOP - Geometric DOP

UBX-13003221 - R28 Early production information Page 361 of 460

## Page 377

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-DOP continued

Byte Offset Number Scaling Name Unit Description

Format

6 U2 0.01 pDOP - Position DOP

8 U2 0.01 tDOP - Time DOP

10 U2 0.01 vDOP - Vertical DOP

12 U2 0.01 hDOP - Horizontal DOP

14 U2 0.01 nDOP - Northing DOP

16 U2 0.01 eDOP - Easting DOP

32.17.7 UBX-NAV-EELL (0x01 0x3d)

32.17.7.1 Position error ellipse parameters

Message UBX-NAV-EELL

Description Position error ellipse parameters

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.3, 22,

22.01, 23 and 23.01 (only with ADR products)

Type Periodic/Polled

Comment This message outputs the error ellipse parameters for the position solutions.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x3d 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1 - reserved1 - Reserved

6 U2 1e-2 errEllipseOri deg Orientation of semi-major axis of error

ent ellipse (degrees from true north)

8 U4 - errEllipseMaj mm Semi-major axis of error ellipse

or

12 U4 - errEllipseMin mm Semi-minor axis of error ellipse

or

UBX-13003221 - R28 Early production information Page 362 of 460

## Page 378

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.8 UBX-NAV-EOE (0x01 0x61)

32.17.8.1 End of epoch

Message UBX-NAV-EOE

Description End of epoch

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic

Comment This message is intended to be used as a marker to collect all navigation

messages of an epoch. It is output after all enabled NAV class messages (except

UBX-NAV-HNR) and after all enabled NMEA messages.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x61 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

32.17.9 UBX-NAV-GEOFENCE (0x01 0x39)

32.17.9.1 Geofencing status

Message UBX-NAV-GEOFENCE

Description Geofencing status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message outputs the evaluated states of all configured geofences for the

current epoch's position.

See the Geofencing description for feature details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x39 8 + 2*numFences see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1 - status - Geofencing status

0 - Geofencing not available or not reliable

1 - Geofencing active

6 U1 - numFences - Number of geofences

UBX-13003221 - R28 Early production information Page 363 of 460

## Page 379

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-GEOFENCE continued

Byte Offset Number Scaling Name Unit Description

Format

7 U1 - combState - Combined (logical OR) state of all

geofences

0 - Unknown

1 - Inside

2 - Outside

Start of repeated block (numFences times)

8 + 2*N U1 - state - Geofence state

0 - Unknown

1 - Inside

2 - Outside

9 + 2*N U1 - id - Geofence ID (0 = not available)

End of repeated block

32.17.10 UBX-NAV-HPPOSECEF (0x01 0x13)

32.17.10.1 High precision position solution in ECEF

Message UBX-NAV-HPPOSECEF

Description High precision position solution in ECEF

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20.01, 20.1, 20.2 and 20.3

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x13 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

8 I4 - ecefX cm ECEF X coordinate

12 I4 - ecefY cm ECEF Y coordinate

16 I4 - ecefZ cm ECEF Z coordinate

20 I1 0.1 ecefXHp mm High precision component of ECEF X

coordinate. Must be in the range of -99..

+99. Precise coordinate in cm = ecefX +

(ecefXHp * 1e-2).

21 I1 0.1 ecefYHp mm High precision component of ECEF Y

coordinate. Must be in the range of -99..

+99. Precise coordinate in cm = ecefY +

(ecefYHp * 1e-2).

UBX-13003221 - R28 Early production information Page 364 of 460

## Page 380

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-HPPOSECEF continued

Byte Offset Number Scaling Name Unit Description

Format

22 I1 0.1 ecefZHp mm High precision component of ECEF Z

coordinate. Must be in the range of -99..

+99. Precise coordinate in cm = ecefZ +

(ecefZHp * 1e-2).

23 X1 - flags - Additional flags (see graphic below)

24 U4 0.1 pAcc mm Position Accuracy Estimate

Bitfield flags

This graphic explains the bits of flags

Name Description

invalidEcef 1 = Invalid ecefX, ecefY, ecefZ, ecefXHp, ecefYHp and ecefZHp

32.17.11 UBX-NAV-HPPOSLLH (0x01 0x14)

32.17.11.1 High precision geodetic position solution

Message UBX-NAV-HPPOSLLH

Description High precision geodetic position solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20.01, 20.1, 20.2 and 20.3

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

This message outputs the Geodetic position with high precision in the currently

selected ellipsoid. The default is the WGS84 Ellipsoid, but can be changed with

the message UBX-CFG-DAT.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x14 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[2] - reserved1 - Reserved

3 X1 - flags - Additional flags (see graphic below)

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

8 I4 1e-7 lon deg Longitude

12 I4 1e-7 lat deg Latitude

UBX-13003221 - R28 Early production information Page 365 of 460

## Page 381

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-HPPOSLLH continued

Byte Offset Number Scaling Name Unit Description

Format

16 I4 - height mm Height above ellipsoid.

20 I4 - hMSL mm Height above mean sea level

24 I1 1e-9 lonHp deg High precision component of longitude.

Must be in the range -99..+99. Precise

longitude in deg * 1e-7 = lon + (lonHp * 1e-

2).

25 I1 1e-9 latHp deg High precision component of latitude.

Must be in the range -99..+99. Precise

latitude in deg * 1e-7 = lat + (latHp * 1e-2).

26 I1 0.1 heightHp mm High precision component of height above

ellipsoid. Must be in the range -9..+9.

Precise height in mm = height + (heightHp

* 0.1).

27 I1 0.1 hMSLHp mm High precision component of height above

mean sea level. Must be in range -9..+9.

Precise height in mm = hMSL + (hMSLHp *

0.1)

28 U4 0.1 hAcc mm Horizontal accuracy estimate

32 U4 0.1 vAcc mm Vertical accuracy estimate

Bitfield flags

This graphic explains the bits of flags

Name Description

invalidLlh 1 = Invalid lon, lat, height, hMSL, lonHp, latHp, heightHp and hMSLHp

UBX-13003221 - R28 Early production information Page 366 of 460

## Page 382

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.12 UBX-NAV-NMI (0x01 0x28)

32.17.12.1 Navigation message cross-check information

Message UBX-NAV-NMI

Description Navigation message cross-check information

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 22.01

Type Periodic/Polled

Comment Information about the validity of received satellite navigation payload.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x28 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x01 for this version)

5 U1[4] - reserved1 - Reserved

9 X1 - gpsNmiFlags - GPS navigation message cross-check

information flags. (see graphic below)

10 X1 - gpsLsFlags - GPS leap second cross-check information

flags. (see graphic below)

11 X1 - galNmiFlags - Galileo navigation message cross-check

information flags. (see graphic below)

12 X1 - galLsFlags - Galileo leap second cross-check

information flags. (see graphic below)

13 X1 - bdsNmiFlags - BeiDou navigation message cross-check

information flags. (see graphic below)

14 X1 - bdsLsFlags - BeiDou leap second cross-check

information flags. (see graphic below)

15 X1 - gloNmiFlags - GLONASS navigation message cross-

check information flags. (see graphic

below)

Bitfield gpsNmiFlags

This graphic explains the bits of gpsNmiFlags

UBX-13003221 - R28 Early production information Page 367 of 460

## Page 383

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

wnoCheckedGPS 1 = week number check performed.

wnoInvalidGPS 1 = week number invalid.

UTCORefChecke 1 = GPS UTCO reference time check performed.

dGPS

UTCORefInvali 1 = GPS UTCO reference time invalid.

dGPS

Bitfield gpsLsFlags

This graphic explains the bits of gpsLsFlags

Name Description

lsValGPS 1 = Leap second value out of range.

dnRangeGPS 1 = Day number value out of range.

totRangeGPS 1 = Data reference TOW out of range.

lsEventGPS 1 = Unexpected leap second event.

recNowGPS 1 = Data received this epoch.

Bitfield galNmiFlags

This graphic explains the bits of galNmiFlags

Name Description

wnoCheckedGAL 1 = week number check performed.

wnoInvalidGAL 1 = week number invalid.

UBX-13003221 - R28 Early production information Page 368 of 460

## Page 384

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield galLsFlags

This graphic explains the bits of galLsFlags

Name Description

lsValGAL 1 = Leap second value out of range.

dnRangeGAL 1 = Day number value out of range.

totRangeGAL 1 = Data reference TOW out of range.

lsEventGAL 1 = Unexpected leap second event.

recNowGAL 1 = Data received this epoch.

Bitfield bdsNmiFlags

This graphic explains the bits of bdsNmiFlags

Name Description

wnoCheckedBDS 1 = week number check performed.

wnoInvalidBDS 1 = week number invalid.

Bitfield bdsLsFlags

This graphic explains the bits of bdsLsFlags

UBX-13003221 - R28 Early production information Page 369 of 460

## Page 385

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

lsValBDS 1 = Leap second value out of range.

dnRangeBDS 1 = Day number value out of range.

totRangeBDS 1 = Data reference TOW out of range.

lsEventBDS 1 = Unexpected leap second event.

recNowBDS 1 = Data received this epoch.

Bitfield gloNmiFlags

This graphic explains the bits of gloNmiFlags

Name Description

wnoCheckedGLO 1 = week number check performed.

wnoInvalidGLO 1 = week number invalid.

32.17.13 UBX-NAV-ODO (0x01 0x09)

32.17.13.1 Odometer solution

Message UBX-NAV-ODO

Description Odometer solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message outputs the traveled distance since last reset (see UBX-NAV-

RESETODO) together with an associated estimated accuracy and the total

cumulated ground distance (can only be reset by a cold start of the receiver).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x09 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

8 U4 - distance m Ground distance since last reset

12 U4 - totalDistance m Total cumulative ground distance

16 U4 - distanceStd m Ground distance accuracy (1-sigma)

UBX-13003221 - R28 Early production information Page 370 of 460

## Page 386

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.14 UBX-NAV-ORB (0x01 0x34)

32.17.14.1 GNSS orbit database info

Message UBX-NAV-ORB

Description GNSS orbit database info

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Status of the GNSS orbit database knowledge.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x34 8 + 6*numSv see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x01 for this version)

5 U1 - numSv - Number of SVs in the database

6 U1[2] - reserved1 - Reserved

Start of repeated block (numSv times)

8 + 6*N U1 - gnssId - GNSS ID

9 + 6*N U1 - svId - Satellite ID

10 + 6*N X1 - svFlag - Information Flags (see graphic below)

11 + 6*N X1 - eph - Ephemeris data (see graphic below)

12 + 6*N X1 - alm - Almanac data (see graphic below)

13 + 6*N X1 - otherOrb - Other orbit data available (see graphic

below)

End of repeated block

Bitfield svFlag

This graphic explains the bits of svFlag

UBX-13003221 - R28 Early production information Page 371 of 460

## Page 387

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

health SV health:

0: unknown

1: healthy

2: not healty

visibility SV health:

0: unknown

1: below horizon

2: above horizon

3: above elevation mask

Bitfield eph

This graphic explains the bits of eph

Name Description

ephUsability How long the receiver will be able to use the stored ephemeris data from now on:

31: The usability period is unknown

30: The usability period is more than 450 minutes

30 > n > 0: The usability period is between (n-1)*15 and n*15 minutes

0: Ephemeris can no longer be used

ephSource 0: not available

1: GNSS transmission

2: external aiding

3-7: other

Bitfield alm

This graphic explains the bits of alm

UBX-13003221 - R28 Early production information Page 372 of 460

## Page 388

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

almUsability How long the receiver will be able to use the stored almanac data from now on:

31: The usability period is unknown

30: The usability period is more than 30 days

30 > n > 0: The usability period is between n-1 and n days

0: Almanac can no longer be used

almSource 0: not available

1: GNSS transmission

2: external aiding

3-7: other

Bitfield otherOrb

This graphic explains the bits of otherOrb

Name Description

anoAopUsabili How long the receiver will be able to use the orbit data from now on:

ty 31: The usability period is unknown

30: The usability period is more than 30 days

30 > n > 0: The usability period is between n-1 and n days

0: Data can no longer be used

type Type of orbit data:

0: No orbit data available

1: AssistNow Offline data

2: AssistNow Autonomous data

3-7: Other orbit data

UBX-13003221 - R28 Early production information Page 373 of 460

## Page 389

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.15 UBX-NAV-POSECEF (0x01 0x01)

32.17.15.1 Position solution in ECEF

Message UBX-NAV-POSECEF

Description Position solution in ECEF

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x01 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - ecefX cm ECEF X coordinate

8 I4 - ecefY cm ECEF Y coordinate

12 I4 - ecefZ cm ECEF Z coordinate

16 U4 - pAcc cm Position Accuracy Estimate

32.17.16 UBX-NAV-POSLLH (0x01 0x02)

32.17.16.1 Geodetic position solution

Message UBX-NAV-POSLLH

Description Geodetic position solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

This message outputs the Geodetic position in the currently selected ellipsoid.

The default is the WGS84 Ellipsoid, but can be changed with the message UBX-

CFG-DAT.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x02 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 1e-7 lon deg Longitude

8 I4 1e-7 lat deg Latitude

12 I4 - height mm Height above ellipsoid

UBX-13003221 - R28 Early production information Page 374 of 460

## Page 390

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-POSLLH continued

Byte Offset Number Scaling Name Unit Description

Format

16 I4 - hMSL mm Height above mean sea level

20 U4 - hAcc mm Horizontal accuracy estimate

24 U4 - vAcc mm Vertical accuracy estimate

32.17.17 UBX-NAV-PVT (0x01 0x07)

32.17.17.1 Navigation position velocity time solution

Message UBX-NAV-PVT

Description Navigation position velocity time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message combines position, velocity and time solution, including accuracy

figures.

Note that during a leap second there may be more or less than 60 seconds in a

minute.

See the description of leap seconds for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x07 92 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U2 - year y Year (UTC)

6 U1 - month month Month, range 1..12 (UTC)

7 U1 - day d Day of month, range 1..31 (UTC)

8 U1 - hour h Hour of day, range 0..23 (UTC)

9 U1 - min min Minute of hour, range 0..59 (UTC)

10 U1 - sec s Seconds of minute, range 0..60 (UTC)

11 X1 - valid - Validity flags (see graphic below)

12 U4 - tAcc ns Time accuracy estimate (UTC)

16 I4 - nano ns Fraction of second, range -1e9 .. 1e9 (UTC)

20 U1 - fixType - GNSSfix Type:

0: no fix

1: dead reckoning only

2: 2D-fix

3: 3D-fix

4: GNSS + dead reckoning combined

5: time only fix

21 X1 - flags - Fix status flags (see graphic below)

22 X1 - flags2 - Additional flags (see graphic below)

23 U1 - numSV - Number of satellites used in Nav Solution

UBX-13003221 - R28 Early production information Page 375 of 460

## Page 391

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-PVT continued

Byte Offset Number Scaling Name Unit Description

Format

24 I4 1e-7 lon deg Longitude

28 I4 1e-7 lat deg Latitude

32 I4 - height mm Height above ellipsoid

36 I4 - hMSL mm Height above mean sea level

40 U4 - hAcc mm Horizontal accuracy estimate

44 U4 - vAcc mm Vertical accuracy estimate

48 I4 - velN mm/s NED north velocity

52 I4 - velE mm/s NED east velocity

56 I4 - velD mm/s NED down velocity

60 I4 - gSpeed mm/s Ground Speed (2-D)

64 I4 1e-5 headMot deg Heading of motion (2-D)

68 U4 - sAcc mm/s Speed accuracy estimate

72 U4 1e-5 headAcc deg Heading accuracy estimate (both motion

and vehicle)

76 U2 0.01 pDOP - Position DOP

78 X2 - flags3 - Additional flags (see graphic below)

80 U1[4] - reserved1 - Reserved

84 I4 1e-5 headVeh deg Heading of vehicle (2-D), this is only valid

when headVehValid is set, otherwise the

output is set to the heading of motion

88 I2 1e-2 magDec deg Magnetic declination. Only supported in

ADR 4.10 and later.

90 U2 1e-2 magAcc deg Magnetic declination accuracy. Only

supported in ADR 4.10 and later.

Bitfield valid

This graphic explains the bits of valid

UBX-13003221 - R28 Early production information Page 376 of 460

## Page 392

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

validDate 1 = valid UTC Date (see Time Validity section for details)

validTime 1 = valid UTC time of day (see Time Validity section for details)

fullyResolved 1 = UTC time of day has been fully resolved (no seconds uncertainty). Cannot be used to check if time

is completely solved.

validMag 1 = valid magnetic declination

Bitfield flags

This graphic explains the bits of flags

Name Description

gnssFixOK 1 = valid fix (i.e within DOP & accuracy masks)

diffSoln 1 = differential corrections were applied

headVehValid 1 = heading of vehicle is valid, only set if the receiver is in sensor fusion mode

carrSoln Carrier phase range solution status:

0: no carrier phase range solution

1: carrier phase range solution with floating ambiguities

2: carrier phase range solution with fixed ambiguities

(not supported in protocol versions less than 20)

Bitfield flags2

This graphic explains the bits of flags2

UBX-13003221 - R28 Early production information Page 377 of 460

## Page 393

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

confirmedAvai 1 = information about UTC Date and Time of Day validity confirmation is available (see Time Validity

section for details)

This flag is only supported in Protocol Versions 19.00, 19.10, 20.10, 20.20, 20.30, 22.00, 23.00, 23.01,

27 and 28.

confirmedDate 1 = UTC Date validity could be confirmed (see Time Validity section for details)

confirmedTime 1 = UTC Time of Day could be confirmed (see Time Validity section for details)

Bitfield flags3

This graphic explains the bits of flags3

Name Description

invalidLlh 1 = Invalid lon, lat, height and hMSL

lastCorrectio Age of the most recently received differential correction:

nAge 0: Not available

1: Age between 0 and 1 second

2: Age between 1 (inclusive) and 2 seconds

3: Age between 2 (inclusive) and 5 seconds

4: Age between 5 (inclusive) and 10 seconds

5: Age between 10 (inclusive) and 15 seconds

6: Age between 15 (inclusive) and 20 seconds

7: Age between 20 (inclusive) and 30 seconds

8: Age between 30 (inclusive) and 45 seconds

9: Age between 45 (inclusive) and 60 seconds

10: Age between 60 (inclusive) and 90 seconds

11: Age between 90 (inclusive) and 120 seconds

>=12: Age greater or equal than 120 seconds

UBX-13003221 - R28 Early production information Page 378 of 460

## Page 394

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.18 UBX-NAV-RELPOSNED (0x01 0x3C)

32.17.18.1 Relative positioning information in NED frame

Message UBX-NAV-RELPOSNED

Description Relative positioning information in NED frame

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20, 20.01, 20.1, 20.2, 20.3, 22, 22.01, 23

and 23.01 (only with High Precision GNSS products)

Type Periodic/Polled

Comment The NED frame is defined as the local topological system at the reference

station. The relative position vector components in this message, along with

their associated accuracies, are given in that local topological system.

This message contains the relative position vector from the Reference Station

to the Rover, including accuracy figures, in the local topological system defined

at the reference station

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x3C 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - reserved1 - Reserved

2 U2 - refStationId - Reference Station ID. Must be in the range

0..4095

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

8 I4 - relPosN cm North component of relative position

vector

12 I4 - relPosE cm East component of relative position vector

16 I4 - relPosD cm Down component of relative position

vector

20 I1 0.1 relPosHPN mm High-precision North component of

relative position vector.

Must be in the range -99 to +99.

The full North component of the relative

position vector, in units of cm, is given by

relPosN + (relPosHPN * 1e-2)

21 I1 0.1 relPosHPE mm High-precision East component of relative

position vector.

Must be in the range -99 to +99.

The full East component of the relative

position vector, in units of cm, is given by

relPosE + (relPosHPE * 1e-2)

UBX-13003221 - R28 Early production information Page 379 of 460

## Page 395

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-RELPOSNED continued

Byte Offset Number Scaling Name Unit Description

Format

22 I1 0.1 relPosHPD mm High-precision Down component of

relative position vector.

Must be in the range -99 to +99.

The full Down component of the relative

position vector, in units of cm, is given by

relPosD + (relPosHPD * 1e-2)

23 U1 - reserved2 - Reserved

24 U4 0.1 accN mm Accuracy of relative position North

component

28 U4 0.1 accE mm Accuracy of relative position East

component

32 U4 0.1 accD mm Accuracy of relative position Down

component

36 X4 - flags - Flags (see graphic below)

Bitfield flags

This graphic explains the bits of flags

Name Description

gnssFixOK A valid fix (i.e within DOP & accuracy masks)

diffSoln 1 if differential corrections were applied

relPosValid 1 if relative position components and accuracies are valid

carrSoln Carrier phase range solution status:

0 = no carrier phase range solution

1 = carrier phase range solution with floating ambiguities

2 = carrier phase range solution with fixed ambiguities

isMoving 1 if the receiver is operating in moving baseline mode (not supported in protocol versions less than

20.3)

refPosMiss 1 if extrapolated reference position was used to compute moving baseline solution this epoch (not

supported in protocol versions less than 20.3)

refObsMiss 1 if extrapolated reference observations were used to compute moving baseline solution this epoch

(not supported in protocol versions less than 20.3)

UBX-13003221 - R28 Early production information Page 380 of 460

## Page 396

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.19 UBX-NAV-RESETODO (0x01 0x10)

32.17.19.1 Reset odometer

Message UBX-NAV-RESETODO

Description Reset odometer

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message resets the traveled distance computed by the odometer (see UBX-

NAV-ODO).

UBX-ACK-ACK or UBX-ACK-NAK are returned to indicate success or failure.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x10 0 see below CK_A CK_B

No payload

32.17.20 UBX-NAV-SAT (0x01 0x35)

32.17.20.1 Satellite information

Message UBX-NAV-SAT

Description Satellite information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message displays information about SVs that are either known to be visible

or currently tracked by the receiver. All signal related information corresponds to

the subset of signals specified in Signal Identifiers.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x35 8 + 12*numSvs see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x01 for this version)

5 U1 - numSvs - Number of satellites

6 U1[2] - reserved1 - Reserved

Start of repeated block (numSvs times)

8 + 12*N U1 - gnssId - GNSS identifier (see Satellite Numbering)

for assignment

9 + 12*N U1 - svId - Satellite identifier (see Satellite

Numbering) for assignment

10 + 12*N U1 - cno dBHz Carrier to noise ratio (signal strength)

11 + 12*N I1 - elev deg Elevation (range: +/-90), unknown if out of

range

UBX-13003221 - R28 Early production information Page 381 of 460

## Page 397

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-SAT continued

Byte Offset Number Scaling Name Unit Description

Format

12 + 12*N I2 - azim deg Azimuth (range 0-360), unknown if

elevation is out of range

14 + 12*N I2 0.1 prRes m Pseudorange residual

16 + 12*N X4 - flags - Bitmask (see graphic below)

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

qualityInd Signal quality indicator:

0: no signal

1: searching signal

2: signal acquired

3: signal detected but unusable

4: code locked and time synchronized

5, 6, 7: code and carrier locked and time synchronized

Note: Since IMES signals are not time synchronized, a channel tracking an IMES signal can never

reach a quality indicator value of higher than 3.

svUsed 1 = Signal in the subset specified in Signal Identifiers is currently being used for navigation

health Signal health flag:

0: unknown

1: healthy

2: unhealthy

diffCorr 1 = differential correction data is available for this SV

smoothed 1 = carrier smoothed pseudorange used

orbitSource Orbit source:

0: no orbit information is available for this SV

1: ephemeris is used

2: almanac is used

3: AssistNow Offline orbit is used

4: AssistNow Autonomous orbit is used

5, 6, 7: other orbit information is used

ephAvail 1 = ephemeris is available for this SV

almAvail 1 = almanac is available for this SV

anoAvail 1 = AssistNow Offline data is available for this SV

UBX-13003221 - R28 Early production information Page 382 of 460

## Page 398

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags Description continued

Name Description

aopAvail 1 = AssistNow Autonomous data is available for this SV

sbasCorrUsed 1 = SBAS corrections have been used for a signal in the subset specified in Signal Identifiers

rtcmCorrUsed 1 = RTCM corrections have been used for a signal in the subset specified in Signal Identifiers

slasCorrUsed 1 = QZSS SLAS corrections have been used for a signal in the subset specified in Signal Identifiers

spartnCorrUse 1 = SPARTN corrections have been used for a signal in the subset specified in Signal Identifiers

d

prCorrUsed 1 = Pseudorange corrections have been used for a signal in the subset specified in Signal Identifiers

crCorrUsed 1 = Carrier range corrections have been used for a signal in the subset specified in Signal Identifiers

doCorrUsed 1 = Range rate (Doppler) corrections have been used for a signal in the subset specified in Signal

Identifiers

clasCorrUsed 1 = CLAS corrections have been used for a signal in the subset specified in Signal Identifiers

32.17.21 UBX-NAV-SBAS (0x01 0x32)

32.17.21.1 SBAS status data

Message UBX-NAV-SBAS

Description SBAS status data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message outputs the status of the SBAS sub system

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x32 12 + 12*cnt see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - geo - PRN Number of the GEO where correction

and integrity data is used from

5 U1 - mode - SBAS Mode

0 Disabled

1 Enabled integrity

3 Enabled test mode

6 I1 - sys - SBAS System (WAAS/EGNOS/...)

-1 Unknown

0 WAAS

1 EGNOS

2 MSAS

3 GAGAN

16 GPS

7 X1 - service - SBAS Services available (see graphic

below)

8 U1 - cnt - Number of SV data following

UBX-13003221 - R28 Early production information Page 383 of 460

## Page 399

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-SBAS continued

Byte Offset Number Scaling Name Unit Description

Format

9 X1 - statusFlags - SBAS status flags (see graphic below)

10 U1[2] - reserved1 - Reserved

Start of repeated block (cnt times)

12 + 12*N U1 - svid - SV ID

13 + 12*N U1 - reserved2 - Reserved

14 + 12*N U1 - udre - Monitoring status

15 + 12*N U1 - svSys - System (WAAS/EGNOS/...)

same as SYS

16 + 12*N U1 - svService - Services available

same as SERVICE

17 + 12*N U1 - reserved3 - Reserved

18 + 12*N I2 - prc cm Pseudo Range correction in [cm]

20 + 12*N U1[2] - reserved4 - Reserved

22 + 12*N I2 - ic cm Ionosphere correction in [cm]

End of repeated block

Bitfield service

This graphic explains the bits of service

Name Description

Ranging GEO may be used as ranging source

Corrections GEO is providing correction data

Integrity GEO is providing integrity

Testmode GEO is in test mode

Bad Problem with signal or broadcast data indicated

Bitfield statusFlags

This graphic explains the bits of statusFlags

UBX-13003221 - R28 Early production information Page 384 of 460

## Page 400

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

integrityUsed SBAS integrity used

0 = Unknown

1 = Integrity information is not available or SBAS integrity is not enabled

2 = Receiver uses only GPS satellites for which integrity information is available

32.17.22 UBX-NAV-SLAS (0x01 0x42)

32.17.22.1 QZSS L1S SLAS status data

Message UBX-NAV-SLAS

Description QZSS L1S SLAS status data

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 19.2

Type Periodic/Polled

Comment This message outputs the status of the QZSS L1S SLAS sub system

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x42 20 + 8*cnt see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1[3] - reserved1 - Reserved

8 I4 1e-3 gmsLon deg Longitude of the used ground monitoring

station

12 I4 1e-3 gmsLat deg Latitude of the used ground monitoring

station

16 U1 - gmsCode - Code of the used ground monitoring

station according to the QZSS SLAS

Interface Specification, available from

qzss.go.jp/en/

17 U1 - qzssSvId - Satellite identifier of the QZS/GEO whose

correction data is used (see Satellite

Numbering)

18 X1 - serviceFlags - Flags regarding SLAS service (see graphic

below)

19 U1 - cnt - Number of pseudorange corrections

following

Start of repeated block (cnt times)

20 + 8*N U1 - gnssId - GNSS identifier (see Satellite Numbering)

21 + 8*N U1 - svId - Satellite identifier (see Satellite

Numbering)

22 + 8*N U1 - reserved2 - Reserved

23 + 8*N U1[3] - reserved3 - Reserved

26 + 8*N I2 - prc cm Pseudorange correction

End of repeated block

UBX-13003221 - R28 Early production information Page 385 of 460

## Page 401

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield serviceFlags

This graphic explains the bits of serviceFlags

Name Description

gmsAvailable 1 = Ground monitoring station available

qzssSvAvailab 1 = Correction providing QZSS SV available

le

testMode 1 = Currently used QZSS SV in test mode

32.17.23 UBX-NAV-SOL (0x01 0x06)

32.17.23.1 Navigation solution information

Message UBX-NAV-SOL

Description Navigation solution information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message combines position, velocity and time solution in ECEF, including

accuracy figures.

This message has only been retained for backwards compatibility; users are

recommended to use the UBX-NAV-PVT message in preference.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x06 52 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - fTOW ns Fractional part of iTOW (range: +/-

500000).

The precise GPS time of week in seconds

is:

(iTOW * 1e-3) + (fTOW * 1e-9)

8 I2 - week weeks GPS week number of the navigation epoch

UBX-13003221 - R28 Early production information Page 386 of 460

## Page 402

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-SOL continued

Byte Offset Number Scaling Name Unit Description

Format

10 U1 - gpsFix - GPSfix Type, range 0..5

0x00 = No Fix

0x01 = Dead Reckoning only

0x02 = 2D-Fix

0x03 = 3D-Fix

0x04 = GPS + dead reckoning combined

0x05 = Time only fix

0x06..0xff: reserved

11 X1 - flags - Fix Status Flags (see graphic below)

12 I4 - ecefX cm ECEF X coordinate

16 I4 - ecefY cm ECEF Y coordinate

20 I4 - ecefZ cm ECEF Z coordinate

24 U4 - pAcc cm 3D Position Accuracy Estimate

28 I4 - ecefVX cm/s ECEF X velocity

32 I4 - ecefVY cm/s ECEF Y velocity

36 I4 - ecefVZ cm/s ECEF Z velocity

40 U4 - sAcc cm/s Speed Accuracy Estimate

44 U2 0.01 pDOP - Position DOP

46 U1 - reserved1 - Reserved

47 U1 - numSV - Number of SVs used in Nav Solution

48 U1[4] - reserved2 - Reserved

Bitfield flags

This graphic explains the bits of flags

Name Description

GPSfixOK 1 = Fix within limits (e.g. DOP & accuracy)

DiffSoln 1 = DGPS used

WKNSET 1 = Valid GPS week number (see Time Validity section for details)

TOWSET 1 = Valid GPS time of week (iTOW & fTOW, see Time Validity section for details)

UBX-13003221 - R28 Early production information Page 387 of 460

## Page 403

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.24 UBX-NAV-STATUS (0x01 0x03)

32.17.24.1 Receiver navigation status

Message UBX-NAV-STATUS

Description Receiver navigation status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x03 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - gpsFix - GPSfix Type, this value does not qualify a

fix as valid and within the limits. See note

on flag gpsFixOk below.

0x00 = no fix

0x01 = dead reckoning only

0x02 = 2D-fix

0x03 = 3D-fix

0x04 = GPS + dead reckoning combined

0x05 = Time only fix

0x06..0xff = reserved

5 X1 - flags - Navigation Status Flags (see graphic

below)

6 X1 - fixStat - Fix Status Information (see graphic below)

7 X1 - flags2 - further information about navigation

output (see graphic below)

8 U4 - ttff ms Time to first fix (millisecond time tag)

12 U4 - msss ms Milliseconds since Startup / Reset

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 388 of 460

## Page 404

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

gpsFixOk 1 = position and velocity valid and within DOP and ACC Masks.

diffSoln 1 = differential corrections were applied

wknSet 1 = Week Number valid (see Time Validity section for details)

towSet 1 = Time of Week valid (see Time Validity section for details)

Bitfield fixStat

This graphic explains the bits of fixStat

Name Description

diffCorr 1 = differential corrections available

carrSolnValid 1 = valid carrSoln

mapMatching map matching status:

00: none

01: valid but not used, i.e. map matching data was received, but was too old

10: valid and used, map matching data was applied

11: valid and used, map matching data was applied. In case of sensor unavailability map matching

data enables dead reckoning. This requires map matched latitude/longitude or heading data.

Bitfield flags2

This graphic explains the bits of flags2

UBX-13003221 - R28 Early production information Page 389 of 460

## Page 405

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

psmState power save mode state

0: ACQUISITION [or when psm disabled]

1: TRACKING

2: POWER OPTIMIZED TRACKING

3: INACTIVE

spoofDetState Spoofing detection state (not supported in protocol versions less than 18)

0: Unknown or deactivated

1: No spoofing indicated

2: Spoofing indicated

3: Multiple spoofing indications

Note that the spoofing state value only reflects the detector state for the current navigation epoch.

As spoofing can be detected most easily at the transition from real signal to spoofing signal, this is

also where the detector is triggered the most. I.e. a value of 1 - No spoofing indicated does not mean

that the receiver is not spoofed, it simply states that the detector was not triggered in this epoch.

carrSoln Carrier phase range solution status:

0: no carrier phase range solution

1: carrier phase range solution with floating ambiguities

2: carrier phase range solution with fixed ambiguities

32.17.25 UBX-NAV-SVINFO (0x01 0x30)

32.17.25.1 Space vehicle information

Message UBX-NAV-SVINFO

Description Space vehicle information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Information about satellites used or visible

This message has only been retained for backwards compatibility; users are

recommended to use the UBX-NAV-SAT message in preference.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x30 8 + 12*numCh see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - numCh - Number of channels

5 X1 - globalFlags - Bitmask (see graphic below)

6 U1[2] - reserved1 - Reserved

Start of repeated block (numCh times)

8 + 12*N U1 - chn - Channel number, 255 for SVs not

assigned to a channel

9 + 12*N U1 - svid - Satellite ID, see Satellite Numbering for

assignment

UBX-13003221 - R28 Early production information Page 390 of 460

## Page 406

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-SVINFO continued

Byte Offset Number Scaling Name Unit Description

Format

10 + 12*N X1 - flags - Bitmask (see graphic below)

11 + 12*N X1 - quality - Bitfield (see graphic below)

12 + 12*N U1 - cno dBHz Carrier to Noise Ratio (Signal Strength)

13 + 12*N I1 - elev deg Elevation in integer degrees

14 + 12*N I2 - azim deg Azimuth in integer degrees

16 + 12*N I4 - prRes cm Pseudo range residual in centimeters

End of repeated block

Bitfield globalFlags

This graphic explains the bits of globalFlags

Name Description

chipGen Chip hardware generation

0: Antaris, Antaris 4

1: u-blox 5

2: u-blox 6

3: u-blox 7

4: u-blox 8 / u-blox M8

Bitfield flags

This graphic explains the bits of flags

Name Description

svUsed SV is used for navigation

diffCorr Differential correction data is available for this SV

orbitAvail Orbit information is available for this SV (Ephemeris or Almanac)

orbitEph Orbit information is Ephemeris

unhealthy SV is unhealthy / shall not be used

orbitAlm Orbit information is Almanac Plus

orbitAop Orbit information is AssistNow Autonomous

smoothed Carrier smoothed pseudorange used

UBX-13003221 - R28 Early production information Page 391 of 460

## Page 407

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield quality

This graphic explains the bits of quality

Name Description

qualityInd Signal Quality indicator (range 0..7). The following list shows the meaning of the different QI values:

0: no signal

1: searching signal

2: signal acquired

3: signal detected but unusable

4: code locked and time synchronized

5, 6, 7: code and carrier locked and time synchronized

Note: Since IMES signals are not time synchronized, a channel tracking an IMES signal can never

reach a quality indicator value of higher than 3.

32.17.26 UBX-NAV-SVIN (0x01 0x3B)

32.17.26.1 Survey-in data

Message UBX-NAV-SVIN

Description Survey-in data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20, 20.01, 20.1, 20.2 and 20.3 (only with

High Precision GNSS products)

Type Periodic/Polled

Comment This message contains information about survey-in parameters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x3B 40 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

8 U4 - dur s Passed survey-in observation time

12 I4 - meanX cm Current survey-in mean position ECEF X

coordinate

16 I4 - meanY cm Current survey-in mean position ECEF Y

coordinate

20 I4 - meanZ cm Current survey-in mean position ECEF Z

coordinate

UBX-13003221 - R28 Early production information Page 392 of 460

## Page 408

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-SVIN continued

Byte Offset Number Scaling Name Unit Description

Format

24 I1 - meanXHP 0.1_ Current high-precision survey-in mean

mm position ECEF X coordinate. Must be in the

range -99..+99.

The current survey-in mean position ECEF

X coordinate, in units of cm, is given by

meanX + (0.01 * meanXHP)

25 I1 - meanYHP 0.1_ Current high-precision survey-in mean

mm position ECEF Y coordinate. Must be in

the range -99..+99.

The current survey-in mean position ECEF

Y coordinate, in units of cm, is given by

meanY + (0.01 * meanYHP)

26 I1 - meanZHP 0.1_ Current high-precision survey-in mean

mm position ECEF Z coordinate. Must be in

the range -99..+99.

The current survey-in mean position ECEF

Z coordinate, in units of cm, is given by

meanZ + (0.01 * meanZHP)

27 U1 - reserved2 - Reserved

28 U4 - meanAcc 0.1_ Current survey-in mean position accuracy

mm

32 U4 - obs - Number of position observations used

during survey-in

36 U1 - valid - Survey-in position validity flag, 1 = valid,

otherwise 0

37 U1 - active - Survey-in in progress flag, 1 = in-progress,

otherwise 0

38 U1[2] - reserved3 - Reserved

32.17.27 UBX-NAV-TIMEBDS (0x01 0x24)

32.17.27.1 BeiDou time solution

Message UBX-NAV-TIMEBDS

Description BeiDou time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2,

20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message reports the precise BDS time of the most recent navigation

solution including validity flags and an accuracy estimate.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x24 20 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 393 of 460

## Page 409

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-TIMEBDS continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U4 - SOW s BDS time of week (rounded to seconds)

8 I4 - fSOW ns Fractional part of SOW (range: +/-

500000000).

The precise BDS time of week in seconds

is:

SOW + fSOW * 1e-9

12 I2 - week - BDS week number of the navigation epoch

14 I1 - leapS s BDS leap seconds (BDS-UTC)

15 X1 - valid - Validity Flags (see graphic below)

16 U4 - tAcc ns Time Accuracy Estimate

Bitfield valid

This graphic explains the bits of valid

Name Description

sowValid 1 = Valid SOW and fSOW (see Time Validity section for details)

weekValid 1 = Valid week (see Time Validity section for details)

leapSValid 1 = Valid leap second

32.17.28 UBX-NAV-TIMEGAL (0x01 0x25)

32.17.28.1 Galileo time solution

Message UBX-NAV-TIMEGAL

Description Galileo time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message reports the precise Galileo time of the most recent navigation

solution including validity flags and an accuracy estimate.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x25 20 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 394 of 460

## Page 410

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-TIMEGAL continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U4 - galTow s Galileo time of week (rounded to seconds)

8 I4 - fGalTow ns Fractional part of the Galileo time of week

(range: +/-500000000).

The precise Galileo time of week in

seconds is:

galTow + fGalTow * 1e-9

12 I2 - galWno - Galileo week number

14 I1 - leapS s Galileo leap seconds (Galileo-UTC)

15 X1 - valid - Validity Flags (see graphic below)

16 U4 - tAcc ns Time Accuracy Estimate

Bitfield valid

This graphic explains the bits of valid

Name Description

galTowValid 1 = Valid galTow and fGalTow (see section Time validity in the integration manual for details)

galWnoValid 1 = Valid galWno (see section Time validity in the integration manual for details)

leapSValid 1 = Valid leapS

32.17.29 UBX-NAV-TIMEGLO (0x01 0x23)

32.17.29.1 GLONASS time solution

Message UBX-NAV-TIMEGLO

Description GLONASS time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2,

20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message reports the precise GLO time of the most recent navigation

solution including validity flags and an accuracy estimate.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x23 20 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 395 of 460

## Page 411

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-TIMEGLO continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U4 - TOD s GLONASS time of day (rounded to integer

seconds)

8 I4 - fTOD ns Fractional part of TOD (range: +/-

500000000).

The precise GLONASS time of day in

seconds is:

TOD + fTOD * 1e-9

12 U2 - Nt days Current date (range: 1-1461), starting at 1

from the 1st Jan of the year indicated by

N4 and ending at 1461 at the 31st Dec of

the third year after that indicated by N4

14 U1 - N4 - Four-year interval number starting from

1996 (1=1996, 2=2000, 3=2004...)

15 X1 - valid - Validity flags (see graphic below)

16 U4 - tAcc ns Time Accuracy Estimate

Bitfield valid

This graphic explains the bits of valid

Name Description

todValid 1 = Valid TOD and fTOD (see Time Validity section for details)

dateValid 1 = Valid N4 and Nt (see Time Validity section for details)

UBX-13003221 - R28 Early production information Page 396 of 460

## Page 412

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.30 UBX-NAV-TIMEGPS (0x01 0x20)

32.17.30.1 GPS time solution

Message UBX-NAV-TIMEGPS

Description GPS time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message reports the precise GPS time of the most recent navigation

solution including validity flags and an accuracy estimate.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x20 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - fTOW ns Fractional part of iTOW (range: +/-

500000).

The precise GPS time of week in seconds

is:

(iTOW * 1e-3) + (fTOW * 1e-9)

8 I2 - week - GPS week number of the navigation epoch

10 I1 - leapS s GPS leap seconds (GPS-UTC)

11 X1 - valid - Validity Flags (see graphic below)

12 U4 - tAcc ns Time Accuracy Estimate

Bitfield valid

This graphic explains the bits of valid

UBX-13003221 - R28 Early production information Page 397 of 460

## Page 413

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

towValid 1 = Valid GPS time of week (iTOW & fTOW, (see Time Validity section for details)

weekValid 1 = Valid GPS week number (see Time Validity section for details)

leapSValid 1 = Valid GPS leap seconds

32.17.31 UBX-NAV-TIMELS (0x01 0x26)

32.17.31.1 Leap second event information

Message UBX-NAV-TIMELS

Description Leap second event information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Information about the upcoming leap second event if one is scheduled.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x26 24 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U1 - version - Message version (0x00 for this version)

5 U1[3] - reserved1 - Reserved

8 U1 - srcOfCurrLs - Information source for the current number

of leap seconds.

0: Default (hardcoded in the firmware, can

be outdated)

1: Derived from time difference between

GPS and GLONASS time

2: GPS

3: SBAS

4: BeiDou

5: Galileo

6: Aided data

7: Configured

8: NavIC

255: Unknown

9 I1 - currLs s Current number of leap seconds since

start of GPS time (Jan 6, 1980). It reflects

how much GPS time is ahead of UTC time.

Galileo number of leap seconds is the

same as GPS. BeiDou number of leap

seconds is 14 less than GPS. GLONASS

follows UTC time, so no leap seconds.

UBX-13003221 - R28 Early production information Page 398 of 460

## Page 414

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-NAV-TIMELS continued

Byte Offset Number Scaling Name Unit Description

Format

10 U1 - srcOfLsChange - Information source for the future leap

second event.

0: No source

2: GPS

3: SBAS

4: BeiDou

5: Galileo

6: GLONASS

7: NavIC

11 I1 - lsChange s Future leap second change if one is

scheduled. +1 = positive leap second, -1 =

negative leap second, 0 = no future leap

second event scheduled or no information

available. If the value is 0, then the

amount of leap seconds did not change

and the event should be ignored.

12 I4 - timeToLsEvent s Number of seconds until the next leap

second event, or from the last leap second

event if no future event scheduled. If > 0

event is in the future, = 0 event is now, < 0

event is in the past. Valid only if

validTimeToLsEvent = 1.

16 U2 - dateOfLsGpsWn - GPS week number (WN) of the next leap

second event or the last one if no future

event scheduled. Valid only if

validTimeToLsEvent = 1.

18 U2 - dateOfLsGpsDn - GPS day of week number (DN) for the next

leap second event or the last one if no

future event scheduled. Valid only if

validTimeToLsEvent = 1. (GPS and Galileo

DN: from 1 = Sun to 7 = Sat. BeiDou DN:

from 0 = Sun to 6 = Sat.)

20 U1[3] - reserved2 - Reserved

23 X1 - valid - Validity flags (see graphic below)

UBX-13003221 - R28 Early production information Page 399 of 460

## Page 415

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield valid

This graphic explains the bits of valid

Name Description

validCurrLs 1 = Valid current number of leap seconds value.

validTimeToLs 1 = Valid time to next leap second event or from the last leap second event if no future event

Event scheduled.

32.17.32 UBX-NAV-TIMEUTC (0x01 0x21)

32.17.32.1 UTC time solution

Message UBX-NAV-TIMEUTC

Description UTC time solution

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Note that during a leap second there may be more or less than 60 seconds in a

minute.

See the description of leap seconds for details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x21 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 U4 - tAcc ns Time accuracy estimate (UTC)

8 I4 - nano ns Fraction of second, range -1e9 .. 1e9 (UTC)

12 U2 - year y Year, range 1999..2099 (UTC)

14 U1 - month month Month, range 1..12 (UTC)

15 U1 - day d Day of month, range 1..31 (UTC)

16 U1 - hour h Hour of day, range 0..23 (UTC)

17 U1 - min min Minute of hour, range 0..59 (UTC)

18 U1 - sec s Seconds of minute, range 0..60 (UTC)

19 X1 - valid - Validity Flags (see graphic below)

UBX-13003221 - R28 Early production information Page 400 of 460

## Page 416

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield valid

This graphic explains the bits of valid

Name Description

validTOW 1 = Valid Time of Week (see Time Validity section for details)

validWKN 1 = Valid Week Number (see Time Validity section for details)

validUTC 1 = Valid UTC Time

utcStandard UTC standard identifier.

0: Information not available

1: Communications Research Labratory (CRL), Tokyo, Japan

2: National Institute of Standards and Technology (NIST)

3: U.S. Naval Observatory (USNO)

4: International Bureau of Weights and Measures (BIPM)

5: European laboratories

6: Former Soviet Union (SU)

7: National Time Service Center (NTSC), China

8: National Physics Laboratory India (NPLI)

15: Unknown

32.17.33 UBX-NAV-VELECEF (0x01 0x11)

32.17.33.1 Velocity solution in ECEF

Message UBX-NAV-VELECEF

Description Velocity solution in ECEF

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x11 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - ecefVX cm/s ECEF X velocity

8 I4 - ecefVY cm/s ECEF Y velocity

12 I4 - ecefVZ cm/s ECEF Z velocity

16 U4 - sAcc cm/s Speed accuracy estimate

UBX-13003221 - R28 Early production information Page 401 of 460

## Page 417

u-blox 8 / u-blox M8 Receiver description - Manual

32.17.34 UBX-NAV-VELNED (0x01 0x12)

32.17.34.1 Velocity solution in NED frame

Message UBX-NAV-VELNED

Description Velocity solution in NED frame

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment See important comments concerning validity of position given in section

Navigation Output Filters.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x01 0x12 36 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I4 - velN cm/s North velocity component

8 I4 - velE cm/s East velocity component

12 I4 - velD cm/s Down velocity component

16 U4 - speed cm/s Speed (3-D)

20 U4 - gSpeed cm/s Ground speed (2-D)

24 I4 1e-5 heading deg Heading of motion 2-D

28 U4 - sAcc cm/s Speed accuracy Estimate

32 U4 1e-5 cAcc deg Course / Heading accuracy estimate

UBX-13003221 - R28 Early production information Page 402 of 460

## Page 418

u-blox 8 / u-blox M8 Receiver description - Manual

32.18 UBX-RXM (0x02)

Receiver Manager Messages: i.e. Satellite Status, RTC Status.

Messages in the RXM class are used to output status and result data from the Receiver Manager.

32.18.1 UBX-RXM-IMES (0x02 0x61)

32.18.1.1 Indoor Messaging System information

Message UBX-RXM-IMES

Description Indoor Messaging System information

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message shows the IMES stations the receiver is currently tracking, their

data rate, the signal level, the Doppler (with respect to 1575.4282MHz) and what

data (without protocol specific overhead) it has received from these stations so

far.

This message is sent out at the navigation rate the receiver is currently set to.

Therefore it allows users to get an overview on the receiver's current state from

the IMES perspective.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x61 4 + 44*numTx see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - numTx - Number of transmitters contained in the

message

1 U1 - version - Message version (0x01 for this version)

2 U1[2] - reserved1 - Reserved

Start of repeated block (numTx times)

4 + 44*N U1 - reserved2 - Reserved

5 + 44*N U1 - txId - Transmitter identifier

6 + 44*N U1[3] - reserved3 - Reserved

9 + 44*N U1 - cno dBHz Carrier to Noise Ratio (Signal Strength)

10 + 44*N U1[2] - reserved4 - Reserved

12 + 44*N I4 2^-12 doppler Hz Doppler frequency with respect to 1575.

4282MHz [IIIII.FFF Hz]

16 + 44*N X4 - position1_1 - Position 1 Frame (part 1/2) (see graphic

below)

20 + 44*N X4 - position1_2 - Position 1 Frame (part 2/2) (see graphic

below)

24 + 44*N X4 - position2_1 - Position 2 Frame (part 1/3) (see graphic

below)

28 + 44*N I4 180*2^- lat deg Latitude, Position 2 Frame (part 2/3)

24

32 + 44*N I4 360*2^- lon deg Longitude, Position 2 Frame (part 3/3)

25

UBX-13003221 - R28 Early production information Page 403 of 460

## Page 419

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-IMES continued

Byte Offset Number Scaling Name Unit Description

Format

36 + 44*N X4 - shortIdFrame - Short ID Frame (see graphic below)

40 + 44*N U4 - mediumIdLSB - Medium ID LSB, Medium ID Frame (part

1/2)

44 + 44*N X4 - mediumId_2 - Medium ID Frame (part 2/2) (see graphic

below)

End of repeated block

Bitfield position1_1

This graphic explains the bits of position1_1

Name Description

pos1Floor Floor number [1.0 floor resolution] (Offset: -50 floor)

pos1Lat Latitude [deg * (180 / 2^23)]

Bitfield position1_2

This graphic explains the bits of position1_2

Name Description

pos1Lon Longitude [deg * (360 / 2^24)]

pos1Valid Position 1 Frame valid

Bitfield position2_1

This graphic explains the bits of position2_1

UBX-13003221 - R28 Early production information Page 404 of 460

## Page 420

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

pos2Floor Floor number [0.5 floor resolution] (Offset: -50 floor)

pos2Alt Altitude [m] (Offset: -95m)

pos2Acc Accuracy Index (0:undef, 1:<7m, 2:<15m, 3:>15m)

pos2Valid Position 2 Frame valid

Bitfield shortIdFrame

This graphic explains the bits of shortIdFrame

Name Description

shortId Short ID

shortValid Short ID Frame valid

shortBoundary Boundary Bit

Bitfield mediumId_2

This graphic explains the bits of mediumId_2

Name Description

mediumIdMSB Medium ID MSB

mediumValid Medium ID Frame valid

mediumboundar Boundary Bit

y

UBX-13003221 - R28 Early production information Page 405 of 460

## Page 421

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.2 UBX-RXM-MEASX (0x02 0x14)

32.18.2.1 Satellite measurements for RRLP

Message UBX-RXM-MEASX

Description Satellite measurements for RRLP

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment The message payload data is, where possible and appropriate, according to the

Radio Resource LCS (Location Services) Protocol (RRLP) [1]. One exception is the

satellite and GNSS IDs, which here are given according to the Satellite

Numbering scheme. The correct satellites have to be selected and their satellite

ID translated accordingly [1, tab. A.10.14] for use in a RRLP Measure Position

Response Component. Similarly, the measurement reference time of week has

to be forwarded correctly (modulo 14400000 for the 24 LSB GPS measurements

variant, modulo 3600000 for the 22 LSB Galileo and Additional Navigation

Satelllite Systems (GANSS) measurements variant) of the RRLP measure

position response to the SMLC.

Reference: [1] ETSI TS 144 031 V11.0.0 (2012-10), Digital cellular

telecommunications system (Phase 2+), Location Services (LCS), Mobile Station

(MS) - Serving Mobile Location Centre (SMLC), Radio Resource LCS Protocol

(RRLP), (3GPP TS 44.031 version 11.0.0 Release 11).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x14 44 + 24*numSV see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version, currently 0x01

1 U1[3] - reserved1 - Reserved

4 U4 - gpsTOW ms GPS measurement reference time

8 U4 - gloTOW ms GLONASS measurement reference time

12 U4 - bdsTOW ms BeiDou measurement reference time

16 U1[4] - reserved2 - Reserved

20 U4 - qzssTOW ms QZSS measurement reference time

24 U2 2^-4 gpsTOWacc ms GPS measurement reference time

accuracy (0xffff = > 4s)

26 U2 2^-4 gloTOWacc ms GLONASS measurement reference time

accuracy (0xffff = > 4s)

28 U2 2^-4 bdsTOWacc ms BeiDou measurement reference time

accuracy (0xffff = > 4s)

30 U1[2] - reserved3 - Reserved

32 U2 2^-4 qzssTOWacc ms QZSS measurement reference time

accuracy (0xffff = > 4s)

34 U1 - numSV - Number of satellites in repeated block

35 U1 - flags - Flags (see graphic below)

36 U1[8] - reserved4 - Reserved

UBX-13003221 - R28 Early production information Page 406 of 460

## Page 422

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-MEASX continued

Byte Offset Number Scaling Name Unit Description

Format

Start of repeated block (numSV times)

44 + 24*N U1 - gnssId - GNSS ID (see Satellite Numbering)

45 + 24*N U1 - svId - Satellite ID (see Satellite Numbering)

46 + 24*N U1 - cNo - carrier noise ratio (0..63)

47 + 24*N U1 - mpathIndic - multipath index (according to [1]) (0 = not

measured, 1 = low, 2 = medium, 3 = high)

48 + 24*N I4 0.04 dopplerMS m/s Doppler measurement

52 + 24*N I4 0.2 dopplerHz Hz Doppler measurement

56 + 24*N U2 - wholeChips - whole value of the code phase

measurement (0..1022 for GPS)

58 + 24*N U2 - fracChips - fractional value of the code phase

measurement (0..1023)

60 + 24*N U4 2^-21 codePhase ms Code phase

64 + 24*N U1 - intCodePhase ms Integer (part of the) code phase

65 + 24*N U1 - pseuRangeRMSE - pseudorange RMS error index (according

rr to [1]) (0..63)

66 + 24*N U1[2] - reserved5 - Reserved

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

towSet TOW set (0 = no, 1 or 2 = yes)

32.18.3 UBX-RXM-PMREQ (0x02 0x41)

32.18.3.1 Power management request

Message UBX-RXM-PMREQ

Description Power management request

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment This message requests a power management related task of the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x41 8 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 407 of 460

## Page 423

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-PMREQ continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - duration ms Duration of the requested task. The

maximum supported value is 12 days. Set

to 0 to wait for a wakeup signal on a pin

4 X4 - flags - task flags (see graphic below)

Bitfield flags

This graphic explains the bits of flags

Name Description

backup The receiver goes into backup mode for a time period defined by duration, provided that it is not

connected to USB

32.18.3.2 Power management request

Message UBX-RXM-PMREQ

Description Power management request

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Command

Comment This message requests a power management related task of the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x41 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - duration ms Duration of the requested task. The

maximum supported value is 12 days. Set

to 0 to wait for a wakeup signal on a pin

8 X4 - flags - task flags (see graphic below)

12 X4 - wakeupSources - Configure pins to wake up the receiver.

The receiver wakes up if there is either a

falling or a rising edge on one of the

configured pins. (see graphic below)

UBX-13003221 - R28 Early production information Page 408 of 460

## Page 424

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

backup The receiver goes into backup mode for a time period defined by duration, provided that it is not

connected to USB

force Force receiver backup while USB is connected. USB interface will be disabled.

Bitfield wakeupSources

This graphic explains the bits of wakeupSources

Name Description

uartrx Wake up the receiver if there is an edge on the UART RX pin

extint0 Wake up the receiver if there is an edge on the EXTINT0 pin

extint1 Wake up the receiver if there is an edge on the EXTINT1 pin

spics Wake up the receiver if there is an edge on the SPI CS pin

32.18.4 UBX-RXM-RAWX (0x02 0x15)

32.18.4.1 Multi-GNSS raw measurement data

Message UBX-RXM-RAWX

Description Multi-GNSS raw measurement data

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 17 (only with Time Sync products)

Type Periodic/Polled

Comment This message contains the information needed to be able to generate a RINEX 3

multi-GNSS observation file (see ftp://ftp.igs.org/pub/data/format/).

This message contains pseudorange, Doppler, carrier phase, phase lock and

signal quality information for GNSS satellites once signals have been

synchronized. This message supports all active GNSS.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x15 16 + 32*numMeas see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

UBX-13003221 - R28 Early production information Page 409 of 460

## Page 425

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-RAWX continued

Byte Offset Number Scaling Name Unit Description

Format

0 R8 - rcvTow s Measurement time of week in receiver

local time approximately aligned to the

GPS time system. The receiver local time

of week, week number and leap second

information can be used to translate the

time to other time systems. More

information about the difference in time

systems can be found in the RINEX 3

format documentation. For a receiver

operating in GLONASS only mode, UTC

time can be determined by subtracting

the leapS field from GPS time regardless

of whether the GPS leap seconds are valid.

8 U2 - week weeks GPS week number in receiver local time.

10 I1 - leapS s GPS leap seconds (GPS-UTC). This field

represents the receiver's best knowledge

of the leap seconds offset. A flag is given

in the recStat bitfield to indicate if the

leap seconds are known.

11 U1 - numMeas - Number of measurements to follow

12 X1 - recStat - Receiver tracking status bitfield (see

graphic below)

13 U1[3] - reserved1 - Reserved

Start of repeated block (numMeas times)

16 + 32*N R8 - prMes m Pseudorange measurement [m].

GLONASS inter frequency channel delays

are compensated with an internal

calibration table.

24 + 32*N R8 - cpMes cycles Carrier phase measurement [cycles]. The

carrier phase initial ambiguity is initialized

using an approximate value to make the

magnitude of the phase close to the

pseudorange measurement. Clock resets

are applied to both phase and code

measurements in accordance with the

RINEX specification.

32 + 32*N R4 - doMes Hz Doppler measurement (positive sign for

approaching satellites) [Hz]

36 + 32*N U1 - gnssId - GNSS identifier (see Satellite Numbering

for a list of identifiers)

37 + 32*N U1 - svId - Satellite identifier (see Satellite

Numbering)

38 + 32*N U1 - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 410 of 460

## Page 426

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-RAWX continued

Byte Offset Number Scaling Name Unit Description

Format

39 + 32*N U1 - freqId - Only used for GLONASS: This is the

frequency slot + 7 (range from 0 to 13)

40 + 32*N U2 - locktime ms Carrier phase locktime counter (maximum

64500ms)

42 + 32*N U1 - cno dBHz Carrier-to-noise density ratio (signal

strength) [dB-Hz]

43 + 32*N X1 0. prStdev m Estimated pseudorange measurement

01*2^n standard deviation (see graphic below)

44 + 32*N X1 0.004 cpStdev cycles Estimated carrier phase measurement

standard deviation (note a raw value of

0x0F indicates the value is invalid) (see

graphic below)

45 + 32*N X1 0. doStdev Hz Estimated Doppler measurement

002*2^ standard deviation. (see graphic below)

n

46 + 32*N X1 - trkStat - Tracking status bitfield (see graphic below

)

47 + 32*N U1 - reserved3 - Reserved

End of repeated block

Bitfield recStat

This graphic explains the bits of recStat

Name Description

leapSec Leap seconds have been determined

clkReset Clock reset applied. Typically the receiver clock is changed in increments of integer milliseconds.

Bitfield prStdev

This graphic explains the bits of prStdev

UBX-13003221 - R28 Early production information Page 411 of 460

## Page 427

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

prStd Estimated pseudorange standard deviation

Bitfield cpStdev

This graphic explains the bits of cpStdev

Name Description

cpStd Estimated carrier phase standard deviation

Bitfield doStdev

This graphic explains the bits of doStdev

Name Description

doStd Estimated Doppler standard deviation

Bitfield trkStat

This graphic explains the bits of trkStat

Name Description

prValid Pseudorange valid

cpValid Carrier phase valid

halfCyc Half cycle valid

subHalfCyc Half cycle subtracted from phase

UBX-13003221 - R28 Early production information Page 412 of 460

## Page 428

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.4.2 Multi-GNSS raw measurements

Message UBX-RXM-RAWX

Description Multi-GNSS raw measurements

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01 (only with ADR or High Precision GNSS or Time Sync

products)

Type Periodic/Polled

Comment This message contains the information needed to be able to generate a RINEX 3

multi-GNSS observation file (see ftp://ftp.igs.org/pub/data/format/).

This message contains pseudorange, Doppler, carrier phase, phase lock and

signal quality information for GNSS satellites once signals have been

synchronized. This message supports all active GNSS.

The only difference between this version of the message and the previous

version (UBX-RXM-RAWX-DATA0) is the addition of the version field.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x15 16 + 32*numMeas see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 R8 - rcvTow s Measurement time of week in receiver

local time approximately aligned to the

GPS time system.

The receiver local time of week, week

number and leap second information can

be used to translate the time to other time

systems. More information about the

difference in time systems can be found in

the RINEX 3 format documentation. For a

receiver operating in GLONASS only mode,

UTC time can be determined by

subtracting the leapS field from GPS time

regardless of whether the GPS leap

seconds are valid.

8 U2 - week weeks GPS week number in receiver local time.

10 I1 - leapS s GPS leap seconds (GPS-UTC). This field

represents the receiver's best knowledge

of the leap seconds offset. A flag is given

in the recStat bitfield to indicate if the

leap seconds are known.

11 U1 - numMeas - Number of measurements to follow

12 X1 - recStat - Receiver tracking status bitfield (see

graphic below)

13 U1 - version - Message version (0x01 for this version)

14 U1[2] - reserved1 - Reserved

Start of repeated block (numMeas times)

UBX-13003221 - R28 Early production information Page 413 of 460

## Page 429

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-RAWX continued

Byte Offset Number Scaling Name Unit Description

Format

16 + 32*N R8 - prMes m Pseudorange measurement [m].

GLONASS inter frequency channel delays

are compensated with an internal

calibration table.

24 + 32*N R8 - cpMes cycles Carrier phase measurement [cycles]. The

carrier phase initial ambiguity is initialized

using an approximate value to make the

magnitude of the phase close to the

pseudorange measurement. Clock resets

are applied to both phase and code

measurements in accordance with the

RINEX specification.

32 + 32*N R4 - doMes Hz Doppler measurement (positive sign for

approaching satellites) [Hz]

36 + 32*N U1 - gnssId - GNSS identifier (see Satellite Numbering

for a list of identifiers)

37 + 32*N U1 - svId - Satellite identifier (see Satellite

Numbering)

38 + 32*N U1 - sigId - New style signal identifier (see Signal

Identifiers).(not supported in protocol

versions less than 27)

39 + 32*N U1 - freqId - Only used for GLONASS: This is the

frequency slot + 7 (range from 0 to 13)

40 + 32*N U2 - locktime ms Carrier phase locktime counter (maximum

64500ms)

42 + 32*N U1 - cno dBHz Carrier-to-noise density ratio (signal

strength) [dB-Hz]

43 + 32*N X1 0. prStdev m Estimated pseudorange measurement

01*2^n standard deviation (see graphic below)

44 + 32*N X1 0.004 cpStdev cycles Estimated carrier phase measurement

standard deviation (note a raw value of

0x0F indicates the value is invalid) (see

graphic below)

45 + 32*N X1 0. doStdev Hz Estimated Doppler measurement

002*2^ standard deviation. (see graphic below)

n

46 + 32*N X1 - trkStat - Tracking status bitfield (see graphic below

)

47 + 32*N U1 - reserved2 - Reserved

End of repeated block

UBX-13003221 - R28 Early production information Page 414 of 460

## Page 430

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield recStat

This graphic explains the bits of recStat

Name Description

leapSec Leap seconds have been determined

clkReset Clock reset applied. Typically the receiver clock is changed in increments of integer milliseconds.

Bitfield prStdev

This graphic explains the bits of prStdev

Name Description

prStd Estimated pseudorange standard deviation

Bitfield cpStdev

This graphic explains the bits of cpStdev

Name Description

cpStd Estimated carrier phase standard deviation

Bitfield doStdev

This graphic explains the bits of doStdev

UBX-13003221 - R28 Early production information Page 415 of 460

## Page 431

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

doStd Estimated Doppler standard deviation

Bitfield trkStat

This graphic explains the bits of trkStat

Name Description

prValid Pseudorange valid

cpValid Carrier phase valid

halfCyc Half cycle valid

subHalfCyc Half cycle subtracted from phase

32.18.5 UBX-RXM-RLM (0x02 0x59)

32.18.5.1 Galileo SAR short-RLM report

Message UBX-RXM-RLM

Description Galileo SAR short-RLM report

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Output

Comment This message contains the contents of any Galileo Search and Rescue (SAR)

Short Return Link Message detected by the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x59 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - type - Message type (0x01 for Short-RLM)

2 U1 - svId - Identifier of transmitting satellite (see

Satellite Numbering)

3 U1 - reserved1 - Reserved

4 U1[8] - beacon - Beacon identifier (60 bits), with bytes

ordered by earliest transmitted (most

significant) first. Top four bits of first byte

are zero.

12 U1 - message - Message code (4 bits)

13 U1[2] - params - Parameters (16 bits), with bytes ordered

by earliest transmitted (most significant)

first.

UBX-13003221 - R28 Early production information Page 416 of 460

## Page 432

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-RXM-RLM continued

Byte Offset Number Scaling Name Unit Description

Format

15 U1 - reserved2 - Reserved

32.18.5.2 Galileo SAR long-RLM report

Message UBX-RXM-RLM

Description Galileo SAR long-RLM report

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Output

Comment This message contains the contents of any Galileo Search and Rescue (SAR)

Long Return Link Message detected by the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x59 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - type - Message type (0x02 for Long-RLM)

2 U1 - svId - Identifier of transmitting satellite (see

Satellite Numbering)

3 U1 - reserved1 - Reserved

4 U1[8] - beacon - Beacon identifier (60 bits), with bytes

ordered by earliest transmitted (most

significant) first. Top four bits of first byte

are zero.

12 U1 - message - Message code (4 bits)

13 U1[12] - params - Parameters (96 bits), with bytes ordered

by earliest transmitted (most significant)

first.

25 U1[3] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 417 of 460

## Page 433

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.6 UBX-RXM-RTCM (0x02 0x32)

32.18.6.1 RTCM input status

Message UBX-RXM-RTCM

Description RTCM input status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 20.01, 20.1, 20.2 and 20.3

Type Output

Comment This message shows info on a received RTCM input message. It is output upon

successful parsing of an RTCM input message, irrespective of whether the

RTCM message is supported or not by the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x32 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x02 for this version)

1 X1 - flags - RTCM input status flags (see graphic

below)

2 U2 - subType - Message subtype, only applicable to u-

blox proprietary RTCM message 4072 (not

available on all products)

4 U2 - refStation - Reference station ID:

For RTCM 2.3: Reference station ID of the

received RTCM 2 input message. Valid

range 0-1023.

For RTCM 3.3: Reference station ID

(DF003) of the received RTCM input

message. Valid range 0-4095. Reported

only for the standard RTCM messages

that include the DF003 field and for the u-

blox proprietary RTCM messages 4072.x.

For all other messages, reports 0xFFFF.

6 U2 - msgType - Message type

Bitfield flags

This graphic explains the bits of flags

UBX-13003221 - R28 Early production information Page 418 of 460

## Page 434

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

crcFailed 0 when RTCM message received and passed CRC check, 1 when failed, in which case refStation and

msgType might be corrupted and misleading

msgUsed 2 = RTCM message used successfully by the receiver, 1 = not used, 0 = do not know

32.18.7 UBX-RXM-SFRBX (0x02 0x13)

32.18.7.1 Broadcast navigation data subframe

Message UBX-RXM-SFRBX

Description Broadcast navigation data subframe

Firmware Supported on:

• u-blox 8 / u-blox M8 with protocol version 17 (only with Time Sync products)

Type Output

Comment This message reports a complete subframe of broadcast navigation data

decoded from a single signal. The number of data words reported in each

message depends on the nature of the signal. See section Broadcast Navigation

Data for further details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x13 8 + 4*numWords see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - gnssId - GNSS identifier (see Satellite Numbering)

1 U1 - svId - Satellite identifier (see Satellite

Numbering)

2 U1 - reserved1 - Reserved

3 U1 - freqId - Only used for GLONASS: This is the

frequency slot + 7 (range from 0 to 13)

4 U1 - numWords - The number of data words contained in

this message (0..16)

5 U1 - reserved2 - Reserved

6 U1 - version - Message version (0x01 for this version)

7 U1 - reserved3 - Reserved

Start of repeated block (numWords times)

8 + 4*N U4 - dwrd - The data words

End of repeated block

UBX-13003221 - R28 Early production information Page 419 of 460

## Page 435

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.7.2 Broadcast navigation data subframe

Message UBX-RXM-SFRBX

Description Broadcast navigation data subframe

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Output

Comment This message reports a complete subframe of broadcast navigation data

decoded from a single signal. The number of data words reported in each

message depends on the nature of the signal.

See section Broadcast Navigation Data for further details.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x13 8 + 4*numWords see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - gnssId - GNSS identifier (see Satellite Numbering)

1 U1 - svId - Satellite identifier (see Satellite

Numbering)

2 U1 - sigId - Signal identifier (see Signal Identifiers)

3 U1 - freqId - Only used for GLONASS: This is the

frequency slot + 7 (range from 0 to 13)

4 U1 - numWords - The number of data words contained in

this message (up to 10, for currently

supported signals)

5 U1 - chn - The tracking channel number the

message was received on

6 U1 - version - Message version, (0x02 for this version)

7 U1 - reserved1 - Reserved

Start of repeated block (numWords times)

8 + 4*N U4 - dwrd - The data words

End of repeated block

UBX-13003221 - R28 Early production information Page 420 of 460

## Page 436

u-blox 8 / u-blox M8 Receiver description - Manual

32.18.8 UBX-RXM-SVSI (0x02 0x20)

32.18.8.1 SV status info

Message UBX-RXM-SVSI

Description SV status info

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment Status of the receiver manager knowledge about GPS Orbit Validity

This message has only been retained for backwards compatibility; users are

recommended to use the UBX-NAV-ORB message in preference.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x02 0x20 8 + 6*numSV see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - iTOW ms GPS time of week of the navigation epoch.

See the description of iTOW for details.

4 I2 - week weeks GPS week number of the navigation epoch

6 U1 - numVis - Number of visible satellites

7 U1 - numSV - Number of per-SV data blocks following

Start of repeated block (numSV times)

8 + 6*N U1 - svid - Satellite ID

9 + 6*N X1 - svFlag - Information Flags (see graphic below)

10 + 6*N I2 - azim - Azimuth

12 + 6*N I1 - elev - Elevation

13 + 6*N X1 - age - Age of Almanac and Ephemeris: (see

graphic below)

End of repeated block

Bitfield svFlag

This graphic explains the bits of svFlag

UBX-13003221 - R28 Early production information Page 421 of 460

## Page 437

u-blox 8 / u-blox M8 Receiver description - Manual

Name Description

ura Figure of Merit (URA) range 0..15

healthy SV healthy flag

ephVal Ephemeris valid

almVal Almanac valid

notAvail SV not available

Bitfield age

This graphic explains the bits of age

Name Description

almAge Age of ALM in days offset by 4

i.e. the reference time may be in the future:

ageOfAlm = (age & 0x0f) - 4

ephAge Age of EPH in hours offset by 4.

i.e. the reference time may be in the future:

ageOfEph = ((age & 0xf0) >> 4) - 4

UBX-13003221 - R28 Early production information Page 422 of 460

## Page 438

u-blox 8 / u-blox M8 Receiver description - Manual

32.19 UBX-SEC (0x27)

Security Feature Messages

Messages in the SEC class are used for security features of the receiver.

32.19.1 UBX-SEC-UNIQID (0x27 0x03)

32.19.1.1 Unique chip ID

Message UBX-SEC-UNIQID

Description Unique chip ID

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.2, 20.

3, 22, 22.01, 23 and 23.01

Type Output

Comment This message is used to retrieve a unique chip identifier (40 bits, 5 bytes).

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x27 0x03 9 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x01 for this version)

1 U1[3] - reserved1 - Reserved

4 U1[5] - uniqueId - Unique chip ID

UBX-13003221 - R28 Early production information Page 423 of 460

## Page 439

u-blox 8 / u-blox M8 Receiver description - Manual

32.20 UBX-TIM (0x0D)

Timing Messages: i.e. Time Pulse Output, Time Mark Results.

Messages in the TIM class are used to output timing information from the receiver, like Time

Pulse and Time Mark measurements.

32.20.1 UBX-TIM-DOSC (0x0D 0x11)

32.20.1.1 Disciplined oscillator control

Message UBX-TIM-DOSC

Description Disciplined oscillator control

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Output

Comment The receiver sends this message when it is disciplining an external oscillator and

the external oscillator is set up to be controlled via the host.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x11 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1[3] - reserved1 - Reserved

4 U4 - value - The raw value to be applied to the DAC

controlling the external oscillator. The

least significant bits should be written to

the DAC, with the higher bits being

ignored.

32.20.2 UBX-TIM-FCHG (0x0D 0x16)

32.20.2.1 Oscillator frequency changed notification

Message UBX-TIM-FCHG

Description Oscillator frequency changed notification

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Periodic/Polled

Comment This message reports frequency changes commanded by the sync manager for

the internal and external oscillator. It is output at the configured rate even if the

sync manager decides not to command a frequency change.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x16 32 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

UBX-13003221 - R28 Early production information Page 424 of 460

## Page 440

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-FCHG continued

Byte Offset Number Scaling Name Unit Description

Format

1 U1[3] - reserved1 - Reserved

4 U4 - iTOW ms GPS time of week of the navigation epoch

from which the sync manager obtains the

GNSS specific data.

Like for the NAV message, the iTOW can

be used to group messages of a single

sync manager run together (See the

description of iTOW for details)

8 I4 2^-8 intDeltaFreq ppb Frequency increment of the internal

oscillator

12 U4 2^-8 intDeltaFreqU ppb Uncertainty of the internal oscillator

nc frequency increment

16 U4 - intRaw - Current raw DAC setting commanded to

the internal oscillator

20 I4 2^-8 extDeltaFreq ppb Frequency increment of the external

oscillator

24 U4 2^-8 extDeltaFreqU ppb Uncertainty of the external oscillator

nc frequency increment

28 U4 - extRaw - Current raw DAC setting commanded to

the external oscillator

32.20.3 UBX-TIM-HOC (0x0D 0x17)

32.20.3.1 Host oscillator control

Message UBX-TIM-HOC

Description Host oscillator control

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Input

Comment This message can be sent by the host to force the receiver to bypass the

disciplining algorithms in the SMGR and carry out the instructed changes to

internal or external oscillator frequency. No checks are carried out on the size of

the frequency change requested, so normal limits imposed by the SMGR are

ignored.

It is recommended that the disciplining of that oscillator is disabled before this

message is sent (i.e. by clearing the enableInternal or enableExternal flag in the

UBX-CFG-SMGR message), otherwise the autonomous disciplining processes may

cancel the effect of the direct command.

Note that the GNSS subsystem may temporarily lose track of some/all satellite

signals if a large change of the internal oscillator is made.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x17 8 see below CK_A CK_B

Payload Contents:

UBX-13003221 - R28 Early production information Page 425 of 460

## Page 441

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-HOC continued

Byte Offset Number Scaling Name Unit Description

Format

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - oscId - Id of oscillator:

0: internal oscillator

1: external oscillator

2 U1 - flags - Flags (see graphic below)

3 U1 - reserved1 - Reserved

4 I4 2^-8 value ppb/- Required frequency offset or raw output,

depending on the flags

Bitfield flags

This graphic explains the bits of flags

Name Description

raw Type of value:

0: frequency offset

1: raw digital output

difference Nature of value:

0: absolute (i.e. relative to 0)

1: relative to current setting

32.20.4 UBX-TIM-SMEAS (0x0D 0x13)

32.20.4.1 Source measurement

Message UBX-TIM-SMEAS

Description Source measurement

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Input/Output

Comment Frequency and/or phase measurement of synchronization sources. The

measurements are relative to the nominal frequency and nominal phase.

The receiver reports the measurements on its sync sources using this message.

Which measurements are reported can be configured using UBX-CFG-SMGR.

The host may report offset of the receiver's outputs with this message as well.

The receiver has to be configured using UBX-CFG-SMGR to enable the use of the

external measurement messages. Otherwise the receiver will ignore them.

UBX-13003221 - R28 Early production information Page 426 of 460

## Page 442

u-blox 8 / u-blox M8 Receiver description - Manual

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x13 12 + 24*numMeas see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - numMeas - Number of measurements in repeated

block

2 U1[2] - reserved1 - Reserved

4 U4 - iTOW ms Time of the week

8 U1[4] - reserved2 - Reserved

Start of repeated block (numMeas times)

12 + 24*N U1 - sourceId - Index of source. SMEAS can provide six

measurement sources. The first four

sourceId values represent measurements

made by the receiver and sent to the host.

The first of these with a sourceId value of

0 is a measurement of the internal

oscillator against the current receiver

time-and-frequency estimate. The

internal oscillator is being disciplined

against that estimate and this result

represents the current offset between the

actual and desired internal oscillator

states. The next three sourceId values

represent frequency and time

measurements made by the receiver

against the internal oscillator. sourceId 1

represents the GNSS-derived frequency

and time compared with the internal

oscillator frequency and time. sourceId2

give measurements of a signal coming in

on EXTINT0. sourceId 3 corresponds to a

similar measurement on EXTINT1. The

remaining two of these measurements

(sourceId 4 and 5) are made by the host

and sent to the receiver. A measurement

with sourceId 4 is a measurement by the

host of the internal oscillator and sourceId

5 indicates a host measurement of the

external oscillator.

13 + 24*N X1 - flags - Flags (see graphic below)

14 + 24*N I1 2^-8 phaseOffsetFr ns Sub-nanosecond phase offset; the total

ac offset is the sum of phaseOffset and

phaseOffsetFrac

15 + 24*N U1 2^-8 phaseUncFrac ns Sub-nanosecond phase uncertainty

UBX-13003221 - R28 Early production information Page 427 of 460

## Page 443

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-SMEAS continued

Byte Offset Number Scaling Name Unit Description

Format

16 + 24*N I4 - phaseOffset ns Phase offset, positive if the source lags

accurate phase and negative if the source

is early

20 + 24*N U4 - phaseUnc ns Phase uncertainty (one standard

deviation)

24 + 24*N U1[4] - reserved3 - Reserved

28 + 24*N I4 2^-8 freqOffset ppb Frequency offset, positive if the source

frequency is too high, negative if the

frequency is too low.

32 + 24*N U4 2^-8 freqUnc ppb Frequency uncertainty (one standard

deviation)

End of repeated block

Bitfield flags

This graphic explains the bits of flags

Name Description

freqValid 1 = frequency measurement is valid

phaseValid 1 = phase measurement is valid

32.20.5 UBX-TIM-SVIN (0x0D 0x04)

32.20.5.1 Survey-in data

Message UBX-TIM-SVIN

Description Survey-in data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync or

Time Sync products)

Type Periodic/Polled

Comment This message contains information about survey-in parameters. For details

about the Time mode see section Time mode configuration.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x04 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - dur s Passed survey-in observation time

UBX-13003221 - R28 Early production information Page 428 of 460

## Page 444

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-SVIN continued

Byte Offset Number Scaling Name Unit Description

Format

4 I4 - meanX cm Current survey-in mean position ECEF X

coordinate

8 I4 - meanY cm Current survey-in mean position ECEF Y

coordinate

12 I4 - meanZ cm Current survey-in mean position ECEF Z

coordinate

16 U4 - meanV mm^2 Current survey-in mean position 3D

variance

20 U4 - obs - Number of position observations used

during survey-in

24 U1 - valid - Survey-in position validity flag, 1 = valid,

otherwise 0

25 U1 - active - Survey-in in progress flag, 1 = in-progress,

otherwise 0

26 U1[2] - reserved1 - Reserved

32.20.6 UBX-TIM-TM2 (0x0D 0x03)

32.20.6.1 Time mark data

Message UBX-TIM-TM2

Description Time mark data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message contains information for high precision time stamping / pulse

counting.

The delay figures and timebase given in UBX-CFG-TP5 are also applied to the

time results output in this message.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x03 28 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - ch - Channel (i.e. EXTINT) upon which the

pulse was measured

1 X1 - flags - Bitmask (see graphic below)

2 U2 - count - Rising edge counter

4 U2 - wnR - Week number of last rising edge

6 U2 - wnF - Week number of last falling edge

8 U4 - towMsR ms Tow of rising edge

12 U4 - towSubMsR ns Millisecond fraction of tow of rising edge

in nanoseconds

16 U4 - towMsF ms Tow of falling edge

UBX-13003221 - R28 Early production information Page 429 of 460

## Page 445

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-TM2 continued

Byte Offset Number Scaling Name Unit Description

Format

20 U4 - towSubMsF ns Millisecond fraction of tow of falling edge

in nanoseconds

24 U4 - accEst ns Accuracy estimate

Bitfield flags

This graphic explains the bits of flags

Name Description

mode 0=single

1=running

run 0=armed

1=stopped

newFallingEdg New falling edge detected

e

timeBase 0=Time base is Receiver time

1=Time base is GNSS time (the system according to the configuration in UBX-CFG-TP5 for tpIdx=0)

2=Time base is UTC (the variant according to the configuration in UBX-CFG-NAV5)

utc 0=UTC not available

1=UTC available

time 0=Time is not valid

1=Time is valid (Valid GNSS fix)

newRisingEdge New rising edge detected

32.20.7 UBX-TIM-TOS (0x0D 0x12)

32.20.7.1 Time pulse time and frequency data

Message UBX-TIM-TOS

Description Time pulse time and frequency data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Periodic

Comment This message contains information about the time pulse that has just happened

and the state of the disciplined oscillators(s) at the time of the pulse. It gives the

UTC and GNSS times and time uncertainty of the pulse together with frequency

and frequency uncertainty of the disciplined oscillators. It also supplies leap

second information.

UBX-13003221 - R28 Early production information Page 430 of 460

## Page 446

u-blox 8 / u-blox M8 Receiver description - Manual

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x12 56 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - version - Message version (0x00 for this version)

1 U1 - gnssId - GNSS system used for reporting GNSS

time (see Satellite Numbering)

2 U1[2] - reserved1 - Reserved

4 X4 - flags - Flags (see graphic below)

8 U2 - year y Year of UTC time

10 U1 - month month Month of UTC time

11 U1 - day d Day of UTC time

12 U1 - hour h Hour of UTC time

13 U1 - minute min Minute of UTC time

14 U1 - second s Second of UTC time

15 U1 - utcStandard - UTC standard identifier:

0: unknown

3: UTC as operated by the U.S. Naval

Observatory (USNO)

6: UTC as operated by the former Soviet

Union

7: UTC as operated by the National Time

Service Center (NTSC), China

16 I4 - utcOffset ns Time offset between the preceding pulse

and UTC top of second

20 U4 - utcUncertaint ns Uncertainty of utcOffset

y

24 U4 - week - GNSS week number

28 U4 - TOW s GNSS time of week

32 I4 - gnssOffset ns Time offset between the preceding pulse

and GNSS top of second

36 U4 - gnssUncertain ns Uncertainty of gnssOffset

ty

40 I4 2^-8 intOscOffset ppb Internal oscillator frequency offset

44 U4 2^-8 intOscUncerta ppb Internal oscillator frequency uncertainty

inty

48 I4 2^-8 extOscOffset ppb External oscillator frequency offset

52 U4 2^-8 extOscUncerta ppb External oscillator frequency uncertainty

inty

UBX-13003221 - R28 Early production information Page 431 of 460

## Page 447

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield flags

This graphic explains the bits of flags

Name Description

leapNow 1 = currently in a leap second

leapSoon 1 = leap second scheduled in current minute

leapPositive 1 = positive leap second

timeInLimit 1 = time pulse is within tolerance limit (UBX-CFG-SMGR timeTolerance field)

intOscInLimit 1 = internal oscillator is within tolerance limit (UBX-CFG-SMGR freqTolerance field)

extOscInLimit 1 = external oscillator is within tolerance limit (UBX-CFG-SMGR freqTolerance field)

gnssTimeValid 1 = GNSS time is valid

UTCTimeValid 1 = UTC time is valid

DiscSrc Disciplining source identifier:

0: internal oscillator

1: GNSS

2: EXTINT0

3: EXTINT1

4: internal oscillator measured by the host

5: external oscillator measured by the host

raim 1 = (T)RAIM system is currently active. Note this flag only reports the current state of the GNSS

solution; it is not affected by whether or not the GNSS solution is being used to discipline the

oscillator.

cohPulse 1 = coherent pulse generation is currently in operation

lockedPulse 1 = time pulse is locked

32.20.8 UBX-TIM-TP (0x0D 0x01)

32.20.8.1 Time pulse time data

Message UBX-TIM-TP

Description Time pulse time data

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3 and 22

Type Periodic/Polled

Comment This message contains information on the timing of the next pulse at the

TIMEPULSE0 output. The recommended configuration when using this

message is to set both the measurement rate (UBX-CFG-RATE) and the

timepulse frequency (UBX-CFG-TP5) to 1 Hz.

For more information see section Time pulse.

TIMEPULSE0 and this message are not available from DR products using the

dedicated I2C sensor interface, including NEO-M8L and NEO-M8U modules

UBX-13003221 - R28 Early production information Page 432 of 460

## Page 448

u-blox 8 / u-blox M8 Receiver description - Manual

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x01 16 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U4 - towMS ms Time pulse time of week according to time

base

4 U4 2^-32 towSubMS ms Submillisecond part of towMS

8 I4 - qErr ps Quantization error of time pulse

12 U2 - week weeks Time pulse week number according to

time base

14 X1 - flags - Flags (see graphic below)

15 X1 - refInfo - Time reference information (see graphic

below)

Bitfield flags

This graphic explains the bits of flags

Name Description

timeBase 0 = Time base is GNSS

1 = Time base is UTC

utc 0 = UTC not available

1 = UTC available

raim (T)RAIM information

0 = Information not available

1 = Not active

2 = Active

qErrInvalid 0 = Quantization error valid

1 = Quantization error invalid

TpNotLocked 0 = Next TP is locked to GNSS

1 = Next TP is based on local time and not locked to GNSS - week/tow may be invalid

UBX-13003221 - R28 Early production information Page 433 of 460

## Page 449

u-blox 8 / u-blox M8 Receiver description - Manual

Bitfield refInfo

This graphic explains the bits of refInfo

Name Description

timeRefGnss GNSS reference information. Only valid if time base is GNSS (timeBase=0).

0 = GPS

1 = GLONASS

2 = BeiDou

3 = Galileo

4 = NavIC

15 = Unknown

utcStandard UTC standard identifier. Only valid if time base is UTC (timeBase=1).

0 = Information not available

1 = Communications Research Laboratory (CRL), Tokyo, Japan

2 = National Institute of Standards and Technology (NIST)

3 = U.S. Naval Observatory (USNO)

4 = International Bureau of Weights and Measures (BIPM)

5 = European laboratories

6 = Former Soviet Union (SU)

7 = National Time Service Center (NTSC), China

8 = National Physics Laboratory India (NPLI)

15 = Unknown

32.20.9 UBX-TIM-VCOCAL (0x0D 0x15)

32.20.9.1 Stop calibration

Message UBX-TIM-VCOCAL

Description Stop calibration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Command

Comment Stop all ongoing calibration (both oscillators are affected)

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x15 1 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (0 for this message)

UBX-13003221 - R28 Early production information Page 434 of 460

## Page 450

u-blox 8 / u-blox M8 Receiver description - Manual

32.20.9.2 VCO calibration extended command

Message UBX-TIM-VCOCAL

Description VCO calibration extended command

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Command

Comment Calibrate (measure) gain of the voltage controlled oscillator. The calibration is

performed by varying the raw oscillator control values between the limits

specified in raw0 and raw1. maxStepSize is the largest step change that can be

used during the calibration process. The "raw values" are either PWM duty cycle

values or DAC values depending on how the VCTCXO is connected to the

system. The measured gain is the transfer function

dRelativeFrequencyChange/dRaw (not dFrequency/dVoltage). The calibration

process works as follows:

Starting from the current raw output the control value is changed in the

direction of raw0 in steps of size at most maxStepSize. Then the frequency is

measured and the control value is changed towards raw1, again in steps of

maxStepSize. When raw1 is reached, the frequency is again measured and the

message version DATA0 is output containing the measured result. Normal

operation then resumes. If the control value movement is less than maxStepSize

then the transition will happen in one step - this will give fast calibration.

Care must be taken when calibrating the internal oscillator against the GNSS

source. In that case the changes applied to the oscillator frequency could be

severe enough to lose satellite signal tracking, especially when signals are weak.

If too many signals are lost, the GNSS system will lose its fix and be unable to

measure the oscillator frequency - the calibration will then fail. In this case

maxStepSize must be reasonably small.

It is also important that only the chosen frequency source is enabled during the

calibration process and that it remains stable throughout the calibration period;

otherwise incorrect oscillator measurements will be made and this will lead to

miscalibration and poor subsequent operation of the receiver.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x15 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (2 for this message)

1 U1 - version - Message version (0x00 for this version)

2 U1 - oscId - Oscillator to be calibrated:

0: internal oscillator

1: external oscillator

UBX-13003221 - R28 Early production information Page 435 of 460

## Page 451

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-VCOCAL continued

Byte Offset Number Scaling Name Unit Description

Format

3 U1 - srcId - Reference source:

0: internal oscillator

1: GNSS

2: EXTINT0

3: EXTINT1

Option 0 should be used when calibrating

the external oscillator. Options 1-3 should

be used when calibrating the internal

oscillator.

4 U1[2] - reserved1 - Reserved

6 U2 - raw0 - First value used for calibration

8 U2 - raw1 - Second value used for calibration

10 U2 - maxStepSize raw Maximum step size to be used

value/

s

32.20.9.3 Results of the calibration

Message UBX-TIM-VCOCAL

Description Results of the calibration

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 16, 17, 18, 19, 19.1, 19.2, 20, 20.01, 20.1, 20.

2, 20.3, 22, 22.01, 23 and 23.01 (only with Time & Frequency Sync products)

Type Periodic/Polled

Comment This message is sent when the oscillator gain calibration process is finished

(successful or unsuccessful). It notifies the user of the calibrated oscillator gain.

If the oscillator gain calibration process was successful, this message will

contain the measured gain (field gainVco) and its uncertainty (field

gainUncertainty). The calibration process can however fail. In that case the two

fields gainVco and gainUncertainty are set to zero.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x15 12 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - type - Message type (3 for this message)

1 U1 - version - Message version (0x00 for this version)

2 U1 - oscId - Id of oscillator:

0: internal oscillator

1: external oscillator

3 U1[3] - reserved1 - Reserved

6 U2 2^-16 gainUncertain 1/1 Relative gain uncertainty after calibration,

ty 0 if calibration failed

UBX-13003221 - R28 Early production information Page 436 of 460

## Page 452

u-blox 8 / u-blox M8 Receiver description - Manual

UBX-TIM-VCOCAL continued

Byte Offset Number Scaling Name Unit Description

Format

8 I4 2^-16 gainVco ppb/ra Calibrated gain or 0 if calibration failed

w LSB

32.20.10 UBX-TIM-VRFY (0x0D 0x06)

32.20.10.1 Sourced time verification

Message UBX-TIM-VRFY

Description Sourced time verification

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Periodic/Polled

Comment This message contains verification information about previous time received via

assistance data or from RTC.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x0D0x06 20 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 I4 - itow ms integer millisecond tow received by source

4 I4 - frac ns sub-millisecond part of tow

8 I4 - deltaMs ms integer milliseconds of delta time (current

time minus sourced time)

12 I4 - deltaNs ns Sub-millisecond part of delta time

16 U2 - wno week Week number

18 X1 - flags - Flags (see graphic below)

19 U1 - reserved1 - Reserved

Bitfield flags

This graphic explains the bits of flags

Name Description

src Aiding time source

0 = no time aiding done

2 = source was RTC

3 = source was assistance data

UBX-13003221 - R28 Early production information Page 437 of 460

## Page 453

u-blox 8 / u-blox M8 Receiver description - Manual

32.21 UBX-UPD (0x09)

Firmware Update Messages: i.e. Memory/Flash erase/write, Reboot, Flash identification, etc.

Messages in the UPD class are used to update the firmware and identify any attached flash

device.

32.21.1 UBX-UPD-SOS (0x09 0x14)

32.21.1.1 Poll backup restore status

Message UBX-UPD-SOS

Description Poll backup restore status

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Poll Request

Comment Sending this (empty) message to the receiver results in the receiver returning a

System restored from backup message as defined below.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x09 0x14 0 see below CK_A CK_B

No payload

32.21.1.2 Create backup in flash

Message UBX-UPD-SOS

Description Create backup in flash

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment The host can send this message in order to save part of the battery-backed

memory (BBR) in a file in the flash file system. The feature is designed in order to

emulate the presence of the backup battery even if it is not present; the host

can issue the save on shutdown command before switching off the device

supply. It is recommended to issue a GNSS stop command using UBX-CFG-RST

before in order to keep the BBR memory content consistent.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x09 0x14 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - cmd - Command (must be 0)

1 U1[3] - reserved1 - Reserved

UBX-13003221 - R28 Early production information Page 438 of 460

## Page 454

u-blox 8 / u-blox M8 Receiver description - Manual

32.21.1.3 Clear backup in flash

Message UBX-UPD-SOS

Description Clear backup in flash

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Command

Comment The host can send this message in order to erase the backup file present in

flash. It is recommended that the clear operation is issued after the host has

received the notification that the memory has been restored after a reset.

Alternatively the host can parse the startup string Restored data saved on

shutdown or poll the UBX-UPD-SOS message for obtaining the status.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x09 0x14 4 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - cmd - Command (must be 1)

1 U1[3] - reserved1 - Reserved

32.21.1.4 Backup creation acknowledge

Message UBX-UPD-SOS

Description Backup creation acknowledge

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The message is sent from the device as confirmation of creation of a backup file

in flash. The host can safely shut down the device after having received this

message.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x09 0x14 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - cmd - Command (must be 2)

1 U1[3] - reserved1 - Reserved

4 U1 - response - 0 = Not acknowledged

1 = Acknowledged

5 U1[3] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 439 of 460

## Page 455

u-blox 8 / u-blox M8 Receiver description - Manual

32.21.1.5 System restored from backup

Message UBX-UPD-SOS

Description System restored from backup

Firmware Supported on:

• u-blox 8 / u-blox M8 protocol versions 15, 15.01, 16, 17, 18, 19, 19.1, 19.2, 20, 20.01,

20.1, 20.2, 20.3, 22, 22.01, 23 and 23.01

Type Output

Comment The message is sent from the device to notify the host the BBR has been

restored from a backup file in the flash file sysetem. The host should clear the

backup file after receiving this message. If the UBX-UPD-SOS message is polled,

this message will be resent.

Header Class ID Length (Bytes) Payload Checksum

Message Structure 0xB5 0x62 0x09 0x14 8 see below CK_A CK_B

Payload Contents:

Byte Offset Number Scaling Name Unit Description

Format

0 U1 - cmd - Command (must be 3)

1 U1[3] - reserved1 - Reserved

4 U1 - response - 0 = Unknown

1 = Failed restoring from backup

2 = Restored from backup

3 = Not restored (no backup)

5 U1[3] - reserved2 - Reserved

UBX-13003221 - R28 Early production information Page 440 of 460

## Page 456

u-blox 8 / u-blox M8 Receiver description - Manual

33 RTCM Protocol

The RTCM (Radio Technical Commission for Maritime Services) protocol is a protocol that is used

to supply the GNSS receiver with real-time differential correction data. The RTCM protocol

specification is available from http://www.rtcm.org.

33.1 RTCM2

33.1.1 Introduction

This feature is only applicable to GPS operation.

This feature only supports code differential positioning.

For effective differential positioning accuracy, it is necessary that the reference station

antenna is situated in a low multipath environment with an unobstructed view of the

sky. It is recommended that reference receiver applies phase smoothing to the

broadcast corrections.

This feature is not available with the High Precision GNSS products.

33.1.2 Supported Messages

The following RTCM 2.3 messages are supported:

Supported RTCM 2.3 Message Types

Message Description

Type

1 Differential GPS Corrections

2 Delta Differential GPS

Corrections

3 GPS Reference Station

Parameters

9 GPS Partial Correction Set

33.1.3 Configuration

The DGPS feature does not need any configuration to work properly. When an RTCM stream is

input on any of the communication interfaces, the data will be parsed and applied if possible,

which will put the receiver into DGPS mode.

The only configurable parameter of DGPS mode is the timeout that can be specified using UBX-

CFG-NAV5. This value defines the time after which old RTCM data will be discarded.

The RTCM protocol can be disabled/enabled on communication interfaces by means of the UBX-

CFG-PRT message. By default, RTCM is enabled.

33.1.4 Output

DGPS mode will result in following modified output:

• NMEA-GGA: The quality field will be 2 (see NMEA Positon Fix Flags). The age of DGPS corrections

and Reference station ID will be set.

UBX-13003221 - R28 Early production information Page 441 of 460

## Page 457

u-blox 8 / u-blox M8 Receiver description - Manual

• NMEA-GLL, NMEA-RMC, NMEA-VTG, NMEA-GNS: The posMode indicator will be D (see NMEA Positon

Fix Flags).

• NMEA-PUBX-POSITION: The status will be D2/D3; The age of DGPS corrections will be set.

• UBX-NAV-SOL: The DGPS flag will be set.

• UBX-NAV-PVT: The diffSoln flag will be set.

• UBX-NAV-STATUS: The diffSoln flag will be set; the diffCorr flag will be set.

• UBX-NAV-SVINFO: The DGPS flag will be set for channels with valid DGPS correction data.

• UBX-NAV-DGPS: This message will contain all valid DGPS data

• If the base line exceeds 100 km and a message type 3 is received, a UBX-INF-WARNING will be

output, e.g. "WARNING: DGNSS baseline big: 330.3km"

33.1.5 Restrictions

The following restrictions apply to DGPS mode:

• The DGPS solution will only include measurements from satellites for which DGPS corrections

were provided. This is because the navigation algorithms cannot mix corrected with uncorrected

measurements.

• SBAS corrections will not be applied when using RTCM correction data.

• Precise Point Positioning will be deactivated when using RTCM correction data.

• RTCM correction data cannot be applied when using AssistNow Offline or AssistNow

Autonomous.

33.1.6 Reference

The RTCM2 support is implemented according to RTCM 10402.3 ("RECOMMENDED STANDARDS

FOR DIFFERENTIAL GNSS").

33.2 RTCM version 3

(Note: the RTCM3 protocol is not supported in protocol versions less than 20).

33.2.1 Introduction

This feature is only available with High Precision GNSS products.

This feature is only applicable to GPS, GLONASS or BeiDou operation.

This feature supports carrier phase differential positioning.

RTCM3 messages can also be transmitted through NTRIP (Networked Transport of

RTCM via Internet Protocol). u-center incorporates an NTRIP client and an NTRIP

server/caster.

For effective differential positioning accuracy, it is necessary that the reference station

antenna is situated in a low multipath environment with an unobstructed view of the sky

and continuous phase lock on all visible satellites.

UBX-13003221 - R28 Early production information Page 442 of 460

## Page 458

u-blox 8 / u-blox M8 Receiver description - Manual

33.2.2 Supported Messages

The following RTCM 3.3 input messages are supported:

Supported RTCM 3.3 Input Messages

Message Type Description

1001 L1-only GPS RTK observations

1002 Extended L1-only GPS RTK observations

1003 L1/L2 GPS RTK observations

1004 Extended L1/L2 GPS RTK observations

1005 Stationary RTK reference station ARP

1006 Stationary RTK reference station ARP with antenna height

1007 Antenna descriptor

1009 L1-only GLONASS RTK observations

1010 Extended L1-only GLONASS RTK observations

1011 L1/L2 GLONASS RTK observations

1012 Extended L1/L2 GLONASS RTK observations

1074 GPS MSM4

1075 GPS MSM5

1077 GPS MSM7

1084 GLONASS MSM4

1085 GLONASS MSM5

1087 GLONASS MSM7

1124 BeiDou MSM4

1125 BeiDou MSM5

1127 BeiDou MSM7

1230 GLONASS code-phase biases

4072, sub-type Reference station PVT (u-blox proprietary RTCM Message)

0

The following RTCM 3.3 output messages are supported:

When configuring RTCM output messages using the UBX protocol message UBX-CFG-MSG, the

Class/IDs shown in the table shall be used.

Supported RTCM 3.3 Output Messages

Message Type Cls/ID Description

1005 0xF5 0x05 Stationary RTK reference station ARP

1074 0xF5 0x4A GPS MSM4

1077 0xF5 0x4D GPS MSM7

1084 0xF5 0x54 GLONASS MSM4

1087 0xF5 0x57 GLONASS MSM7

1124 0xF5 0x7C BeiDou MSM4

1127 0xF5 0x7F BeiDou MSM7

1230 0xF5 0xE6 GLONASS code-phase biases

4072, sub-type 0xF5 0xFE Reference station PVT (u-blox proprietary RTCM Message)

0

UBX-13003221 - R28 Early production information Page 443 of 460

## Page 459

u-blox 8 / u-blox M8 Receiver description - Manual

33.2.3 u-blox Proprietary RTCM Messages

The RTCM message type 4072 is the u-blox proprietary RTCM message. It is supported by the

RTCM standard version 3.2 and above.

33.2.3.1 Sub-Types

There are different available sub-types of the RTCM message type 4072. The table below shows

the available RTCM 4072 sub-types.

RTCM 4072 Sub-Types

Sub- Message Type Sub-Type Description Message Data (Payload) Length (bits)

Type Number Number

1 0xFE8 0x001 Additional reference 112+48*(2*N)

station information (N = the number of enabled GNSS

constellations)

33.2.4 Configuration

The configuration of the RTK rover and reference station is explained in the RTK Mode

Configuration section.

The RTCM3 protocol can be disabled/enabled on communication interfaces by means of the UBX-

CFG-PRT message. By default, RTCM3 is enabled.

The configuration of the RTCM3 correction stream must be done according to the following rules:

• The RTCM3 stream must contain a reference station message (type 1005 or type 1006) in

addition to the GNSS observation messages.

• The RTCM3 stream must contain a reference station message (type 1005, type 1006, or type

4072, sub-type 0) in addition to the GNSS observation messages.

• All observation messages must be broadcast at the same rate.

• The reference station ID field in the GNSS observation messages must be consistent with the

reference station ID field in the reference station message otherwise the rover will not be able to

compute its position.

• The RTCM3 stream must contain the GLONASS code-phase biases message (type 1230)

otherwise the GLONASS ambiguities can only be estimated as float unless the receiver is able

to identify the code-phase bias from receiver descriptor message (RTCM 1033), even in RTK

fixed mode.

• The static reference station message (type 1005 or type 1006) does not need to be broadcast at

the same rate as the observation messages but the rover will not be able to compute its

position until it has received a valid reference station message.

• The moving baseline reference message (type 4072, sub-type 0) must be broadcast at the same

rate as the observation messages.

• The RTCM3 stream should only contain one type of observation messages per constellation.

When using a multi-constellation configuration, all constellations should use the same type of

observation messages. Mixing RTK and MSM messages will result in undefined rover behavior.

• The moving baseline reference message (type 4072, sub-type 0) should only be used in

combination with MSM7 observation messages.

• If the receiver is configured to output RTCM messages on several ports, they must all have the

same RTCM configuration otherwise the MSM multiple message bit might not be set properly.

UBX-13003221 - R28 Early production information Page 444 of 460

## Page 460

u-blox 8 / u-blox M8 Receiver description - Manual

33.2.5 Output

RTK Rover and MB Rover Modes will result in following modified output:

• NMEA-GGA: The quality field will be 4 for RTK fixed and 5 for RTK float (see NMEA Positon Fix

Flags). The age of differential corrections and reference station ID will be set.

• NMEA-GLL, NMEA-VTG: The posMode indicator will be D for RTK float and RTK fixed (see NMEA

Positon Fix Flags).

• NMEA-RMC, NMEA-GNS: The posMode indicator will be F for RTK float and R for RTK fixed (see

NMEA Positon Fix Flags).

• UBX-NAV-PVT: The carrSoln flag will be set to 1 for RTK float and 2 for RTK fixed.

• UBX-NAV-RELPOSNED: The diffSoln and refPosValid flags will be set. The carrSoln flag will be set

to 1 for RTK float and 2 for RTK fixed. In moving baseline rover mode, the isMoving flag will be

set, and the refPosMiss and refObsMiss flags will be set for epochs during which extrapolated

reference position or observations have been used.

• UBX-NAV-SAT: The diffCorr flag will be set for satellites with valid RTCM data. The

rtcmCorrUsed, prCorrUsed, and crCorrUsed flags will be set for satellites for which the RTCM

corrections have been applied. In moving baseline rover mode, the doCorrUsed flag will also be

set.

• UBX-NAV-STATUS: The diffSoln flag will be set; the diffCorr flag will be set.

• If the baseline exceeds 10 km and a message type 1005, type 1006 or type 4072, sub-type 0 is

received, a UBX-INF-WARNING will be output, e.g. "WARNING: DGNSS baseline big: 12.7km"

33.2.6 Reference

The RTCM3 support is implemented according to RTCM STANDARD 10403.3 DIFFERENTIAL

GNSS (GLOBAL NAVIGATION SATELLITE SYSTEMS) SERVICES - VERSION 3.

UBX-13003221 - R28 Early production information Page 445 of 460

## Page 461

u-blox 8 / u-blox M8 Receiver description - Manual

Appendix

A Satellite Numbering

A summary of all the SV numbering schemes is provided in the following table.

Satellite numbering

GNSS Type SV range UBX gnssId: UBX svId NMEA 2.X- NMEA 2.X-4.0 NMEA 4.10+ NMEA 4.10+

svId 4.0 (strict) (extended) (strict) (extended)

GPS G1-G32 0:1-32 1-32 1-32 1-32 1-32 1-32

SBAS S120- 1:120-158 120-158 33-64 33-64,152- 33-64 33-64,152-

S158 158 158

Galileo E1-E36 2:1-36 211-246 - 301-336 1-36 1-36

BeiDou B1-B37 3:1-37 159-163,33- - 401-437 1-37 1-37

64

IMES I1-I10 4:1-10 173-182 - 173-182 - 173-182

QZSS Q1-Q10 5:1-10 193-202 - 193-202 - 193-202

GLONAS R1-R32, 6:1-32, 6: 65-96, 255 65-96, 65-96, null 65-96, 65-96, null

S R? 255 null null

B UBX and NMEA Signal Identifiers

UBX and NMEA protocols use signal identifiers (commonly abbreviated as "sigId") to distinguish

between different signals from GNSS.

Signal identifiers are only valid when combined with a GNSS identifier (see above). The table below

shows the range of identifiers currently supported in the firmware.

C u-blox 8 / u-blox M8 Default Settings

The default settings listed in this section apply to u-blox 8 / u-blox M8 receivers. These values

assume that the default levels of the configuration pins have been left unchanged and no setting

that affects the default configuration was written to the eFuse. Default settings are dependent

on the configuration pin and eFuse settings. For information regarding these settings, consult the

applicable data sheet.

If nothing else is mentioned, the default settings apply to u-blox 8 and u-blox M8

receivers.

C.1 Antenna Supervisor Settings (UBX-CFG-ANT)

For parameter and protocol description see section UBX-CFG-ANT.

Antenna Supervisor Default Settings

Parameter SPG 2.xx SPG 3.xx, ADR 3.xx ADR 4.xx, FTS 1.xx TIM 1.0x TIM 1.1x

HPG 1.xx UDR 1.xx

flags-svcs 1 1 1 1 0 1 1

flags-scd 1 1 0 0 0 1 0

flags-pdwnOnSCD 1 1 0 0 0 0 0

flags-recovery 1 1 0 0 0 1 0

flags-ocd 0 0 0 0 0 0 0

UBX-13003221 - R28 Early production information Page 446 of 460

## Page 462

u-blox 8 / u-blox M8 Receiver description - Manual

Antenna Supervisor Default Settings continued

Parameter SPG 2.xx SPG 3.xx, ADR 3.xx ADR 4.xx, FTS 1.xx TIM 1.0x TIM 1.1x

HPG 1.xx UDR 1.xx

pins-pinSwitch 16 16 16 16 31 16 16

pins-pinSCD 15 15 31 15 31 15 15

pins-pinOCD 31 14 31 14 31 31 14

C.2 Data Batching Settings (UBX-CFG-BATCH)

For parameter and protocol description see section UBX-CFG-BATCH.

Data Batching Default Settings

Parameter SPG 3.51

flags-enable 0

flags-extraPvt 1

flags-extraOdo 1

flags-pioEnable 0

flags-pioActiveLow 0

bufSize 0

notifThrs 0

pioId 0

C.3 Datum Settings (UBX-CFG-DAT)

For parameter and protocol description see section UBX-CFG-DAT.

Datum Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

datumNum 0

datumName WGS84

majA 6378137

flat 298.257223563

dX 0

dY 0

dZ 0

rotX 0

rotY 0

rotZ 0

scale 0

C.4 Geofencing Settings (UBX-CFG-GEOFENCE)

For parameter and protocol description see section UBX-CFG-GEOFENCE.

Geofencing Default Settings

Parameter SPG 2.xx, SPG 3.xx, HPG 1.xx, ADR 3.xx, ADR 4.xx, UDR 1.xx

numFences 0

confLvl 0

pioEnabled 0

pinPolarity 0

UBX-13003221 - R28 Early production information Page 447 of 460

## Page 463

u-blox 8 / u-blox M8 Receiver description - Manual

Geofencing Default Settings continued

Parameter SPG 2.xx, SPG 3.xx, HPG 1.xx, ADR 3.xx, ADR 4.xx, UDR 1.xx

pin 0

C.5 High Navigation Rate Settings (UBX-CFG-HNR)

For parameter and protocol description see section UBX-CFG-HNR.

High Navigation Rate Default Settings

Parameter ADR 3.xx, UDR 1.xx ADR 4.xx

highNavRate 0 10

C.6 GNSS System Settings (UBX-CFG-GNSS)

For parameter and protocol description see section UBX-CFG-GNSS.

GNSS System Default Settings

Parameter SPG 2.xx, SPG 3.0x ADR 4.xx, FTS 1.xx TIM 1.0x TIM 1.1x, HPG 1.xx

ADR 3.xx UDR 1.xx SPG 3.5x

numTrkChHw 32 32 28 32 32 32 32

numTrkChUse 32 32 28 32 32 32 28

numConfigBlocks 5 7 7 5 6 7 4

gnssId 0, 1, 3, 5, 0, 1, 2, 3, 0, 1, 2, 3, 0, 1, 3, 5, 0, 1, 3, 4, 0, 1, 2, 3, 0, 3, 5, 6

6 4, 5, 6 4, 5, 6 6 5, 6 4, 5, 6

flags-enable 1, 1, 0, 1, 1, 1, 0, 0, 1, 1, 0, 0, 1, 0, 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1, 0, 1, 1

1 0, 1, 1 0, 1, 1 1 1, 1 0, 1, 1

resTrkCh 8, 1, 8, 0, 8, 1, 4, 8, 8, 1, 4, 8, 8, 1, 8, 0, 8, 1, 8, 0, 8, 1, 4, 8, 8, 8, 0, 8

8 0, 0, 8 0, 0, 8 8 0, 8 0, 0, 8

maxTrkCh 16, 3, 16, 16, 3, 8, 16, 3, 8, 16, 3, 16, 16, 3, 16, 16, 3, 8, 16, 16, 3,

3, 14 16, 8, 3, 16, 8, 3, 3, 14 8, 3, 14 16, 8, 3, 14

14 14 14

C.7 INF Messages Settings (UBX-CFG-INF)

For parameter and protocol description see section UBX-CFG-INF.

C.7.1 UBX Protocol

INF Messages Default Settings for UBX protocol

Parameter SPG 2.xx, SPG 3.xx, FTS 1.xx, TIM 1.xx, HPG 1.xx, ADR 3.xx, ADR 4.xx, UDR 1.xx

protocolID 0

infMsgMask-ERROR 0,0,0,0,0,0

infMsgMask-WARNING 0,0,0,0,0,0

infMsgMask-NOTICE 0,0,0,0,0,0

infMsgMask-TEST 0,0,0,0,0,0

infMsgMask-DEBUG 0,0,0,0,0,0

UBX-13003221 - R28 Early production information Page 448 of 460

## Page 464

u-blox 8 / u-blox M8 Receiver description - Manual

C.7.2 NMEA Protocol

INF Messages Default Settings for NMEA protocol

Parameter SPG 2.xx, TIM 1.0x, FTS 1.xx, SPG 3.xx, TIM 1.1x, HPG 1.xx ADR 4.xx, UDR 1.xx

ADR 3.xx

protocolID 1 1 1

infMsgMask-ERROR 1,1,1,1,1,1 1,1,0,1,1,0 1,1,0,1,1,0

infMsgMask-WARNING 1,1,1,1,1,1 1,1,0,1,1,0 1,1,0,1,1,0

infMsgMask-NOTICE 1,1,1,1,1,1 1,1,0,1,1,0 1,1,0,1,1,0

infMsgMask-TEST 0,0,0,0,0,0 0,0,0,0,0,0 0,0,0,0,0,0

infMsgMask-DEBUG 0,0,0,0,0,0 0,0,0,0,0,0 0,0,0,0,0,0

C.8 Jammer/Interference Monitor Settings (UBX-CFG-ITFM)

For parameter and protocol description see section UBX-CFG-ITFM.

Jamming/Interference Monitor Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

config-bbThreshold 3

config-cwThreshold 15

config-enable 0

config2-antSetting 0

config2-enable2 0

C.9 Logging Settings (UBX-CFG-LOGFILTER)

For parameter and protocol description see section UBX-CFG-LOGFILTER.

Logging Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

flags-recordEnabled 0

flags- 0

psmOncePerWakupEnable

d

flags-applyAllFilterSettings 0

minInterval 0

timeThreshold 0

speedThreshold 0

positionThreshold 0

C.10 Navigation Settings (UBX-CFG-NAV5)

For parameter and protocol description see section UBX-CFG-NAV5.

Navigation Default Settings

Parameter SPG 2.xx, SPG 3.xx ADR 4.xx, FTS 1.xx TIM 1.0x TIM 1.1x HPG 1.xx

ADR 3.xx UDR 1.xx

mask-dyn 1 1 1 1 1 1 1

mask-minEl 1 1 1 1 1 1 1

mask-posFixMode 1 1 1 1 1 1 1

mask-drLim 1 1 1 1 1 1 1

UBX-13003221 - R28 Early production information Page 449 of 460

## Page 465

u-blox 8 / u-blox M8 Receiver description - Manual

Navigation Default Settings continued

Parameter SPG 2.xx, SPG 3.xx ADR 4.xx, FTS 1.xx TIM 1.0x TIM 1.1x HPG 1.xx

ADR 3.xx UDR 1.xx

mask-posMask 1 1 1 1 1 1 1

mask-timeMask 1 1 1 1 1 1 1

mask-staticHoldMask 1 1 1 1 1 1 1

mask-dgpsMask 1 1 1 1 1 1 1

mask-cnoThreshold 1 1 1 1 1 1 1

mask-utc 1 1 1 1 1 1 1

dynModel 0 0 4 2 2 2 0

fixMode 3 3 3 3 3 3 3

fixedAlt 0 0 0 0 0 0 0

fixedAltVar 1 1 1 1 1 1 1

minElev 5 5 10 5 5 5 10

drLimit 0 0 0 0 0 0 0

pDop 25 25 25 25 25 25 25

tDop 25 25 25 25 25 25 25

pAcc 100 100 100 100 100 100 100

tAcc 300 350 350 300 350 350 350

staticHoldThresh 0 0 0 0 0 0 0

dgpsTimeOut 60 60 60 60 60 60 60

cnoThreshNumSVs 0 0 0 0 0 0 0

cnoThresh 0 0 0 0 0 0 0

staticHoldMaxDist 200 0 0 200 200 0 0

utcStandard 0 0 0 3 3 3 0

C.11 Navigation Settings (UBX-CFG-NAVX5)

For parameter and protocol description see section UBX-CFG-NAVX5.

Navigation Default Settings (SPG/FTS/TIM)

Parameter SPG 2.xx SPG 3.0x SPG 3.5x FTS 1.xx, TIM 1. TIM 1.1x

0x

mask1-minMax 1 1 1 1 1

mask1-minCno 1 1 1 1 1

mask1-initial3dfix 1 1 1 1 1

mask1-wknRoll 1 1 1 1 1

mask1-ackAid 1 1 1 1 1

mask1-ppp 1 1 1 1 1

mask1-aop 1 1 1 1 1

mask2-adr 0 0 0 0 0

minSVs 3 3 3 1 1

maxSVs 20 32 32 20 32

minCNO 6 6 6 9 9

iniFix3D 0 0 0 0 0

ackAiding 0 0 0 0 0

UBX-13003221 - R28 Early production information Page 450 of 460

## Page 466

u-blox 8 / u-blox M8 Receiver description - Manual

Navigation Default Settings (SPG/FTS/TIM) continued

Parameter SPG 2.xx SPG 3.0x SPG 3.5x FTS 1.xx, TIM 1. TIM 1.1x

0x

wknRollover 1756 1867 (<3.05) 1936 1756 1867 (1.10,

2152 (3.05) 1.11)

2243 (1.12)

usePPP 0 0 0 0 0

aopCfg-useAOP 0 0 0 0 0

aopOrbMaxErr 100 100 100 100 100

gnssTofsCfg-tolerance 0 0 0 0 0

gnssTofsCfg- 0 0 0 0 0

useMeasVarTest

gnssTofsCfg- 0 0 0 0 0

aopPreCalEnabled

gnssTofsCfg-aopPreCalDt 0 0 0 0 0

gnssTofsCfg- 0 0 0 0 0

aopPreCalInhInt

useAdr 0 0 0 0 0

Navigation Default Settings (ADR/UDR/HPG)

Parameter ADR 3.xx ADR 4.0x, ADR 4.2x, UDR 1.00 HPG 1.30 HPG 1.40

ADR 4.1x ADR 4.3x,

UDR 1.2x,

UDR 1.3x

mask1-minMax 1 1 1 1 1 1

mask1-minCno 1 1 1 1 1 1

mask1-initial3dfix 1 1 1 1 1 1

mask1-wknRoll 1 1 1 1 1 1

mask1-ackAid 1 1 1 1 1 1

mask1-ppp 1 1 1 1 1 1

mask1-aop 1 1 1 1 1 1

mask2-adr 0 0 0 0 0 0

mask2-sigAttenComp n/a 0 0 0 0 0

minSVs 2 5 5 5 3 3

maxSVs 20 24 24 24 20 20

minCNO 6 12 20 12 6 6

iniFix3D 0 0 0 0 0 0

ackAiding 0 0 0 0 0 0

wknRollover 1756 1867 - 1867 1867 1867

sigAttenCompMode n/a 0 0 0 0 0

usePPP 0 0 0 0 1 1

aopCfg-useAOP 0 0 0 0 0 0

aopOrbMaxErr 100 100 100 100 100 100

useAdr 1 1 1 1 0 0

wknRollover default value depends on the firmware build date.

UBX-13003221 - R28 Early production information Page 451 of 460

## Page 467

u-blox 8 / u-blox M8 Receiver description - Manual

C.12 NMEA Protocol Settings (UBX-CFG-NMEA)

For parameter and protocol description see section UBX-CFG-NMEA.

NMEA Protocol Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

filter-posFilt 0

filter-mskPosFilt 0

filter-timeFilt 0

filter-dateFilt 0

filter-gpsOnlyFilter 0

filter-trackFilt 0

nmeaVersion 0x40

numSV 0

flags-compat 0

flags-consider 1

flags-limit82 0

flags-highPrec 0

gnssToFilter-gps 0

gnssToFilter-sbas 0

gnssToFilter-qzss 0

gnssToFilter-glonass 0

gnssToFilter-beidou 0

svNumbering 0

mainTalkerId 0

gsvTalkerId 0

bdsTalkerId not set

C.13 Odometer Settings (UBX-CFG-ODO)

For parameter and protocol description see section UBX-CFG-ODO.

ODO Default Settings

Parameter SPG 2.xx, SPG 3.0x, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG SPG 3.5x

1.xx

flags-useODO 0 1

flags-useCOG 0 1

flags-outLPVel 0 1

flags-outLPCog 0 1

odoCfg-profile 0 0

cogMaxSpeed 1 1

cogMaxPosAcc 50 50

velLpGain 153 153

cogLpGain 76 76

C.14 Power Management 2 Configuration (UBX-CFG-PM2)

For parameter and protocol description see section UBX-CFG-PM2.

UBX-13003221 - R28 Early production information Page 452 of 460

## Page 468

u-blox 8 / u-blox M8 Receiver description - Manual

Power Management 2 Configuration Default Settings

Parameter SPG 2.xx, ADR SPG 3.0x SPG 3.51 TIM 1.0x TIM 1.1x

3.xx, FTS 1.xx,

ADR 4.xx, UDR

1.xx

maxStartupStateDur 0 0 0 0 0

flags-extintSel 0 0 0 0 0

flags-extintWake 0 0 0 0 0

flags-extintBackup 0 0 0 0 0

flags-extintInactive n/a 0 0 n/a 0

flags-limitPeakCurr 0 0 0 0 0

flags-waitTimeFix 0 0 0 1 1

flags-updateRTC 0 0 0 0 0

flags-updateEPH 1 1 0 1 1

flags-doNotEnterOff 0 0 1 0 0

flags-mode 1 1 1 1 1

updatePeriod 1000 1000 1000 1000 1000

searchPeriod 10000 10000 10000 10000 10000

gridOffset 0 0 0 0 0

onTime 0 0 0 0 0

minAcqTime 0 0 300 0 0

extintInactivityMs n/a 0 0 n/a 0

C.15 Port Configuration (UBX-CFG-PRT)

For parameter and protocol description see section UBX-CFG-PRT.

C.15.1 UART Port Configuration

For parameter and protocol description see section UBX-CFG-PRT-UART.

UART 1 Default Settings

Parameter SPG 2.xx, SPG 3.xx, FTS 1. ADR 3.xx, ADR 4.xx, UDR 1. HPG 1.xx

xx, TIM 1.xx xx

txReady-en 0 0 0

txReady-pol 0 0 0

txReady-pin 0 0 0

txReady-thres 0 0 0

baudRate 9600 9600 9600

inProtoMask inUbx,inNmea,inRtcm inUbx,inNmea,inRtcm inUbx,inNmea,

inRtcm3

outProtoMask outUbx,outNmea outUbx,outNmea outUbx,outNmea,

outRtcm3

flags-extendedTxTimeout 0 0 0

C.15.2 USB Port Configuration

For parameter and protocol description see section UBX-CFG-PRT-USB.

UBX-13003221 - R28 Early production information Page 453 of 460

## Page 469

u-blox 8 / u-blox M8 Receiver description - Manual

USB Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM HPG 1.xx

1.xx, ADR 4.xx, UDR 1.xx

txReady-en 0 0

txReady-pol 0 0

txReady-pin 0 0

txReady-thres 0 0

inProtoMask inUbx,inNmea,inRtcm inUbx,inNmea,inRtcm3

outProtoMask outUbx,outNmea outUbx,outNmea,outRtcm3

flags-extendedTxTimeout 0 0

C.15.3 SPI Port Configuration

For parameter and protocol description see section UBX-CFG-PRT-SPI.

SPI Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

txReady-en 0

txReady-pol 0

txReady-pin 0

txReady-thres 0

mode-spiMode 0

mode-flowControl 0

mode-ffCnt 0

inProtoMask None

outProtoMask None

flags-extendedTxTimeout 0

C.15.4 DDC Port Configuration

For parameter and protocol description see section UBX-CFG-PRT-DDC.

DDC Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM HPG 1.xx

1.xx, ADR 4.xx, UDR 1.xx

txReady-en 0 0

txReady-pol 0 0

txReady-pin 0 0

txReady-thres 0 0

mode-slaveAddr 0x42 0x42

inProtoMask inUbx,inNmea,inRtcm inUbx,inNmea,inRtcm3

outProtoMask outUbx,outNmea outUbx,outNmea,outRtcm3

flags-extendedTxTimeout 0 0

C.16 Output Rate Settings (UBX-CFG-RATE)

For parameter and protocol description see section UBX-CFG-RATE.

UBX-13003221 - R28 Early production information Page 454 of 460

## Page 470

u-blox 8 / u-blox M8 Receiver description - Manual

Output Rate Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, ADR 4.xx, UDR 1.xx, HPG 1.xx

measRate 1000

navRate 1

timeRef 1

C.17 Remote Inventory Settings (UBX-CFG-RINV)

For parameter and protocol description see section UBX-CFG-RINV.

Remote Inventory Default Settings

Parameter SPG 2.xx, SPG 3.xx, ADR 3.xx, FTS 1.xx, TIM 1.xx, HPG 1.xx

flags-dump 0

flags-binary 0

C.18 Receiver Manager Configuration Settings (UBX-CFG-RXM)

For parameter and protocol description see section UBX-CFG-RXM.

Power Management Default Settings

Parameter SPG 2.xx, FTS 1. SPG 3.0x, TIM 1. ADR 3.xx ADR 4.xx, UDR SPG 3.5x

xx, TIM 1.0x 1x, HPG 1.xx 1.xx

lpMode 0 0 0 0 1

C.19 SBAS Configuration Settings (UBX-CFG-SBAS)

For parameter and protocol description see section UBX-CFG-SBAS.

SBAS Configuration Default Settings

Parameter SPG 2.xx, SPG 3.0x SPG 3.5x ADR 3.xx ADR 4.xx, TIM 1.1x

FTS 1.xx, TIM UDR 1.xx

1.0x

mode-enabled * 1 1 1 1 1 0

mode-test 0 0 0 0 0 0

usage-range 1 1 1 1 1 1

usage-diffCorr 1 1 1 1 1 1

usage-integrity 0 0 0 0 0 0

maxSBAS * 3 3 3 3 3 3

scanmode2 None None None None None None

scanmode1 120,124, 120,123, 120,123, 120,124, 120,123, 120,123,

126,129, 127-129, 127-129, 126,127- 127-129, 127-129,

133,135, 133,135- 133,135- 129,133, 133,135- 133,135-

137,138 138 138 135,137, 138 138

138

* These parameters are deprecated; use UBX-CFG-GNSS instead.

UBX-13003221 - R28 Early production information Page 455 of 460

## Page 471

u-blox 8 / u-blox M8 Receiver description - Manual

C.20 Timepulse Settings (UBX-CFG-TP5)

For parameter and protocol description see section UBX-CFG-TP5.

TIMEPULSE1 Default Settings

Parameter SPG 2.xx SPG 3.xx, HPG 1. ADR 3.xx, ADR FTS 1.xx TIM 1.xx

xx 4.xx, UDR 1.xx

antCableDelay 50 50 50 50 50

rfGroupDelay 0 0 0 0 0

freqPeriod 1000000 1000000 0 0 1000000

freqPeriodLock 1000000 1000000 0 0 1000000

pulseLenRatio 0 0 0 0 0

pulseLenRatioLock 100000 100000 0 0 100000

userConfigDelay 0 0 0 0 0

flags-active 1 1 0 1 1

flags-lockGpsFreq 1 n/a n/a n/a n/a

flags-lockGnssFreq n/a 1 1 1 1

flags-lockedOtherSet 1 1 1 1 1

flags-isFreq 0 0 0 0 0

flags-isLength 1 1 1 1 1

flags-alignToTow 1 1 1 1 1

flags-polarity 1 1 0 0 1

flags-gridUtcGps 0 n/a n/a n/a n/a

flags-gridUtcGnss n/a 0 0 1 1

flags-syncMode n/a 0 0 0 0

C.21 USB Settings (UBX-CFG-USB)

For parameter and protocol description see section UBX-CFG-USB.

USB Default Settings

Parameter SPG 2.xx, ADR 3.xx, FTS 1.xx, TIM 1.0x, ADR SPG 3.xx, TIM 1.1x, HPG 1.xx

4.xx, UDR 1.xx

vendorID 0x1546 0x1546

productID 0x01A8 0x01A8

powerConsumption 100 100

flags-reEnum 0 0

flags-powerMode 1 1

vendorString u-blox AG - www.u-blox.com u-blox AG - www.u-blox.com

productString u-blox GNSS receiver u-blox GNSS receiver

serialNumber not set not set

UBX-13003221 - R28 Early production information Page 456 of 460

## Page 472

u-blox 8 / u-blox M8 Receiver description - Manual

Related Documents

Overview

As part of our commitment to customer support, u-blox maintains an extensive volume of

technical documentation for our products. In addition to product-specific data sheets and

integration manuals, general documents are also available. These include:

• GPS Compendium, GPS-X-02007

• GPS Antennas - RF Design Considerations for u-blox GPS Receivers, GPS-X-08014

Our website www.u-blox.com is a valuable resource for general and product-specific

documentation.

For design and integration projects the Receiver description including interface description should

be used together with the Data sheet and Hardware integration manual of the GNSS receiver.

UBX-13003221 - R28 Early production information Page 457 of 460

## Page 473

u-blox 8 / u-blox M8 Receiver description - Manual

Revision History

Revision Date Name Status / Comments

R01 30-Sep-2013 efav Added u-blox M8 firmware 2.00

R02 01-Nov-2013 efav Added u-blox M8 firmware 2.01

R03 15-Dec-2013 efav Added u-blox M8 ADR product variant

R04 10-Feb-2014 efav Added u-blox M8 Time & Frequency Sync product variant

R05 27-Jun-2014 efav Added u-blox M8 Timing product variant

R06 09-Sep-2014 mfre Minor corrections

R07 09-Sep-2014 mfre Added u-blox M8 firmware 2.30

R08 19-Nov-2014 mfre Added u-blox M8 L-type modules product variant

R09 30-Nov-2015 mfre Added u-blox 8 / u-blox M8 SPG 3.01 firmware

R10 15-Feb-2016 mfre Added u-blox 8 / u-blox M8 TIM 1.10 firmware

R11 04-May-2016 mfre Added u-blox 8 / u-blox M8 ADR 4.00 and UDR 1.00 firmware

R12 28-Apr-2017 jhak Added u-blox 8 / u-blox M8 ADR 4.10, HPG 1.40 and SPG 3.51

firmware

R13 06-Jul-2017 jhak Added HPG 1.40 firmware information

R14 24-Oct-2017 jhak Added ADR 4.11 firmware information

R15 06-Mar-2018 jhak Updated Super-E messages

R16 05-Nov-2018 jhak Added ADR 4.21 and UDR 1.21 firmware information

R17 17-May-2019 ssid Minor corrections

R18 24-Mar-2020 ssid Added ADR 4.31 and UDR 1.31 firmware information

R19 14-May-2020 dama Added TIM 1.11 firmware information

R20 26-Jun-2020 ssid Type numbers updated

NEO-M8N-0-11, NEO-M8Q-0-11, NEO-8Q-0-11, NEO-M8P-0-12,

NEO-M8P-2-12,NEO-M8T-0-11

R21 25-Sep-2020 ssid ADR/UDR scope changed to public, NEO-M8L added to the

product list

New messages added: UBX-CFG-ESFALG, UBX-CFG-ESFG,

UBX-CFG-ESFA, UBX-CFG-ESFWT, UBX-CFG-SENIF, UBX-

CFG-SPT, UBX-ESF-ALG, UBX-HNR-ATT, UBX-MON-SPT, UBX-

NAV-COV, UBX-NAV-EELL, NMEA-GxTHS

Automotive Dead Reckoning: Solution types, installation

configuration, sensor configuration, ADR system

configuration, operation

Untethered Dead Reckoning: Installation configuration, sensor

configuration, UDR system configuration, operation

R22 05-Feb-2021 jesk Galileo-specific information added to UBX-CFG-GNSS and

UBX-CFG-RST

R23 23-Feb-2021 jesk/ss Clarified UBX-CFG-GNSS

id Added ADR 4.50 and UDR 1.50 firmware information

R24 22-Jun-2021 jesk Added NEO-M8J and firmware 3.05

NEO-M8M, NEO-M8N, and NEO-M8Q type numbers updated

R25 19-Aug-2021 dama Update for M8P FW 3.05 HPG 1.43 maintenance release

R26 23-Nov-2021 jesk ZOE-M8B and ZOE-M8G type numbers updated

R27 25-Aug-2022 ssid AID-MAPM update - Temperature compensation topic update

UBX-13003221 - R28 Early production information Page 458 of 460

## Page 474

u-blox 8 / u-blox M8 Receiver description - Manual

continued

Revision Date Name Status / Comments

R28 30-Jan-2023 viha Added TIM 1.12 firmware information

UBX-13003221 - R28 Early production information Page 459 of 460

## Page 475

u-blox 8 / u-blox M8 Receiver description - Manual

Contact

For complete contact information visit us at www.u-blox.com

u-blox Offices

North, Central and South America Headquarters Asia, Australia, Pacific

Europe, Middle East, Africa

u-blox America, Inc. u-blox Singapore Pte. Ltd.

Phone: +1 703 483 3180 u-blox AG Phone: +65 6734 3811

E-mail: info_us@u-blox.com Phone: +41 44 722 74 44 E-mail: info_ap@u-blox.com

E-mail: info@u-blox.com Support: support_ap@u-blox.com

Regional Office West Coast: Support: support@u-blox.com

Phone: +1 408 573 3640 Regional Office Australia:

E-mail: info_us@u-blox.com Phone: +61 3 9566 7255

E-mail: info_anz@u-blox.com

Technical Support: Support: support_ap@u-blox.com

Phone: +1 703 483 3185

E-mail: support_us@u-blox.com Regional Office China (Beijing):

Phone: +86 10 68 133 545

E-mail: info_cn@u-blox.com

Support: support_cn@u-blox.com

Regional Office China (Chongqing):

Phone: +86 23 6815 1588

E-mail: info_cn@u-blox.com

Support: support_cn@u-blox.com

Regional Office China (Shanghai):

Phone: +86 21 6090 4832

E-mail: info_cn@u-blox.com

Support: support_cn@u-blox.com

Regional Office China (Shenzhen):

Phone: +86 755 8627 1083

E-mail: info_cn@u-blox.com

Support: support_cn@u-blox.com

Regional Office India:

Phone: +91 80 4050 9200

E-mail: info_in@u-blox.com

Support: support_in@u-blox.com

Regional Office Japan (Osaka):

Phone: +81 6 6941 3660

E-mail: info_jp@u-blox.com

Support: support_jp@u-blox.com

Regional Office Japan (Tokyo):

Phone: +81 3 5775 3850

E-mail: info_jp@u-blox.com

Support: support_jp@u-blox.com

Regional Office Korea:

Phone: +82 2 542 0861

E-mail: info_kr@u-blox.com

Support: support_kr@u-blox.com

Regional Office Taiwan:

Phone: +886 2 2657 1090

E-mail: info_tw@u-blox.com

Support: support_tw@u-blox.com

UBX-13003221 - R28 Early production information Page 460 of 460