# PDF to Word Output

## Page 1

RINEX

The Receiver Independent Exchange Format

Version 3.03

International GNSS Service (IGS), RINEX Working

Group and Radio Technical Commission for Maritime

Services Special Committee 104 (RTCM-SC104),

July 14th, 2015

Acknowledgment: RINEX Version 3.02 and 3.03 is based on RINEX Version 3.01 which was

developed by: Werner Gurtner, Astronomical Institute of the University of Bern, Switzerland and

Lou Estey, UNAVCO, Boulder Colorado, USA.

## Page 2

RINEX Version 3.03 i

Table of Contents

0. REVISION HISTORY .............................................................................................................................. 1

1. THE PHILOSOPHY AND HISTORY OF RINEX .................................................................................. 8

2. GENERAL FORMAT DESCRIPTION ................................................................................................. 10

3. BASIC DEFINITIONS ........................................................................................................................... 11

3.1 Time .................................................................................................................................................. 11

3.2 Pseudo-Range: .................................................................................................................................. 11

3.3 Phase ................................................................................................................................................. 11

Table 1: Observation Corrections for Receiver Clock Offset ............................................................. 12

3.4 Doppler ............................................................................................................................................. 12

3.5 Satellite numbers ............................................................................................................................... 12

Figure 1: Satellite numbers and Constellation Identifiers ................................................................... 12

4. THE EXCHANGE OF RINEX FILES ................................................................................................... 13

Figure 2: Recommended filename parameters. ................................................................................... 13

Table 2: Description of Filename Parameters ..................................................................................... 14

5. RINEX VERSION 3 FEATURES .......................................................................................................... 15

5.1 Observation codes ............................................................................................................................. 15

Table 3: Observation Code Components ............................................................................................ 15

Table 4 : RINEX Version 3.03 GPS Observation Codes .................................................................... 16

Table 5 : RINEX Version 3.03 GLONASS Observation Codes ......................................................... 17

Table 6 : RINEX Version 3.03 Galileo Observation Codes ............................................................... 17

Table 7 : RINEX Version 3.03 SBAS Observation Codes ................................................................. 18

Table 8 : RINEX Version 3.03 QZSS Observation Codes ................................................................. 18

Table 9 : RINEX Version 3.03 BDS Observation Codes ................................................................... 19

Table 10 : RINEX Version 3.03 IRNSS Observation Codes .............................................................. 19

5.2 Satellite system-dependent list of observables .................................................................................. 20

5.3 Marker type ....................................................................................................................................... 20

Table 11: Proposed Marker Type Keywords ...................................................................................... 20

5.4 Half-wavelength observations, half-cycle ambiguities ..................................................................... 21

5.5 Scale factor........................................................................................................................................ 21

5.6 Information about receivers on a vehicle .......................................................................................... 21

## Page 3

RINEX Version 3.03 ii

5.7 Signal strength .................................................................................................................................. 22

Table 12: Standardized S/N Indicators ............................................................................................... 22

5.8 Date/time format in the PGM / RUN BY / DATE header record ..................................................... 22

5.9 Antenna phase center header record ................................................................................................. 23

5.10 Antenna orientation ......................................................................................................................... 23

5.11 Observation data records ................................................................................................................. 23

Table 13: Example Observation Type Records .................................................................................. 23

Table 14: Example Observation Data Records ................................................................................... 23

5.12 Ionosphere delay as pseudo-observables ........................................................................................ 24

Table 15: Ionosphere Pseudo-Observable Coding .............................................................................. 24

Table 16: Ionosphere Pseudo-Observable Corrections to Observations ............................................. 24

5.13 Channel numbers as pseudo-observables ........................................................................................ 24

5.14 Corrections of differential code biases (DCBs) .............................................................................. 25

5.15 Corrections of antenna phase center variations (PCVs) .................................................................. 25

5.16 Navigation message files ................................................................................................................ 25

Table 17: Example of Navigation File Satellite System and Number Definition Record .................. 25

Table 18: Example of Navigation File Header IONOSPHERIC CORR Record ................................ 25

6. ADDITIONAL HINTS AND TIPS ........................................................................................................ 26

6.1 Versions ............................................................................................................................................ 26

6.2 Leading blanks in CHARACTER fields ........................................................................................... 26

6.3 Variable-length records ..................................................................................................................... 26

6.4 Blank fields ....................................................................................................................................... 26

6.5 Order of the header records, order of data records............................................................................ 26

6.6 Missing items, duration of the validity of values .............................................................................. 27

6.7 Unknown / Undefined observation types and header records ........................................................... 27

6.8 Event flag records ............................................................................................................................. 27

6.9 Receiver clock offset ......................................................................................................................... 27

6.10 Two-digit years ............................................................................................................................... 27

6.11 Fit interval (GPS navigation message file) ..................................................................................... 28

6.12 Satellite health (GPS navigation message file) ............................................................................... 28

Table 19: Description of GPS Satellite Health Field .......................................................................... 28

6.13 Transmission time of message (GPS navigation message file)....................................................... 28

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 4

RINEX Version 3.03 iii

6.14 Antenna references, phase centers .................................................................................................. 28

7. RINEX UNDER ANTISPOOFING (AS) ............................................................................................... 29

8. DEALING WITH DIFFERENT SATELLITE SYSTEMS .................................................................... 30

8.1 Time system identifier ...................................................................................................................... 30

Table 20: Relationship between GPS, QZSS, IRN, GST, GAL, BDS and RINEX Week Numbers .. 31

Table 21: Constellation Time Relationships ....................................................................................... 32

Table 22: GPS and BeiDou UTC Leap Second Relationship ............................................................. 32

8.2 Pseudorange definition ...................................................................................................................... 33

Table 23: Constellation Pseudorange Corrections .............................................................................. 33

8.3 RINEX navigation message files ...................................................................................................... 34

8.3.1 RINEX navigation message files for GLONASS ............................................................................ 34

Table 24: GLONASS Navigation File Data, Sign Convention ........................................................... 34

8.3.2 RINEX navigation message files for Galileo ............................................................................. 35

8.3.3 RINEX navigation message files for GEO satellites ................................................................. 35

8.3.4 RINEX navigation message files for QZSS L1-SAIF ............................................................... 36

8.3.5 RINEX navigation message files for BDS ................................................................................. 37

8.3.6 RINEX navigation message files for IRNSS ............................................................................. 37

8.4 RINEX observation files for GEO satellites ..................................................................................... 37

9. MODIFICATIONS FOR VERSION 3.01, 3.02 and 3.03 ...................................................................... 38

9.1 Phase Cycle Shifts ............................................................................................................................. 38

Table 25: RINEX Phase Alignment Correction Convention .............................................................. 39

Table 26: Example SYS / PHASE SHIFT Record............................................................................. 39

9.2 Galileo: BOC-Tracking of an MBOC-Modulated Signal ................................................................. 40

Table 27: Example of RINEX Coding of Galileo BOC Tracking of an MBOC Signal Record ......... 40

9.3 BDS Satellite System Code............................................................................................................... 40

9.4 New Observation Codes for GPS L1C and BDS .............................................................................. 40

9.5 Header Records for GLONASS Slot and Frequency Numbers ........................................................ 40

Table 28: Example of a GLONASS Slot- Frequency Records ........................................................... 41

9.6 GNSS Navigation Message File: Leap Seconds Record ................................................................... 41

9.7 Clarifications in the Galileo Navigation Message File: .................................................................... 41

9.8 Quasi-Zenith Satellite System (QZSS) Version 3.02 ........................................................................ 41

9.9 GLONASS Mandatory Code-Phase Alignment Header Record....................................................... 41

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 5

RINEX Version 3.03 iv

Table 29: Example of GLONASS Code Phase Bias Correction Record ............................................ 42

Table 30: Example of Unknown GLONASS Code Phase Bias Record ............................................. 42

9.10 BDS system (Replaces Compass) ................................................................................................... 42

9.11 Indian Regional Navigation Satellite System (IRNSS) Version 3.03 ............................................. 42

10 References .......................................................................................................................................... 43

APPENDIX: RINEX FORMAT DEFINITIONS AND EXAMPLES ......................................................... 1

A 1 RINEX File name description ............................................................................................................ 1

A 2 GNSS Observation Data File -Header Section Description ............................................................... 5

A 3 GNSS Observation Data File -Data Record Description ................................................................. 13

A 4 GNSS Observation Data File – Example #1 .................................................................................... 15

A 4 GNSS Observation Data File – Example #2 .................................................................................... 17

A 4 GNSS Observation Data File – Example #3 .................................................................................... 18

A 5 GNSS Navigation Message File – Header Section Description ...................................................... 19

A 6 GNSS Navigation Message File – GPS Data Record Description ................................................... 23

A 7 GPS Navigation Message File – Example ....................................................................................... 24

A 8 GNSS Navigation Message File – GALILEO Data Record Description ......................................... 25

A 9 GALILEO Navigation Message File – Examples ............................................................................ 27

A 10 GNSS Navigation Message File – GLONASS Data Record Description ...................................... 29

A 11 GNSS Navigation Message File – Example: Mixed GPS / GLONASS ........................................ 30

A 12 GNSS Navigation Message File – QZSS Data Record Description .............................................. 31

A 13 QZSS Navigation Message File – Example ................................................................................... 32

A 14 GNSS Navigation Message File – BDS Data Record Description ................................................ 33

A 15 BeiDou Navigation Message File – Example ................................................................................ 34

A 16 GNSS Navigation Message File – SBAS Data Record Description .............................................. 35

A 17 SBAS Navigation Message File -Example .................................................................................... 36

A 18 GNSS Navigation Message File – IRNSS Data Record Description ............................................. 37

A 19 IRNSS Navigation Message File – Example ................................................................................. 39

A 20 Meteorological Data File -Header Section Description ................................................................. 40

A 21 Meteorological Data File -Data Record Description...................................................................... 42

A 22 Meteorological Data File – Example ............................................................................................. 42

A 23 Reference Code and Phase Alignment by Constellation and Frequency Band .............................. 43

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 6

RINEX Version 3.03 1

0. REVISION HISTORY

Version 3.00

02 Feb 2006 A few typos and obsolete paragraphs removed.

08 Mar 2006 Epochs of met data of met files version 2.11 are in GPS time only

(Table A20).

31 Mar 2006 DCB header record label corrected in Table A6: SYS / DCBS

APPLIED.

June 2006 Filenames for mixed GNSS nav mess files.

10 Aug 2006 Table A3: Error in format of EPOCH record: One 6X removed.

Trailing 3X removed.

12 Sep 2006 GNSS navigation message files version 3.00 included (including

Galileo).

Table A4: Example of the kinematic event was wrong (kinematic

event record).

SYS / DCBS APPLIED header record simplified.

Tables A6 and A8: Clarification for adjustment of “Transmission time

of message“.

03 Oct 2006 Table A11: Mixed GPS/GLONASS navigation message file

26 Oct 2006 Table A4: Removed obsolete antispoofing flag

Tables A6/8/10: Format error in SV / EPOCH / SV CLK: Space

between svn and year was missing

Half-cycle ambiguity flag (re-)introduced (5.4 and Table A4).

Clarification of reported GLONASS time (8.1).

New header record SYS / PCVS APPLIED

New Table 10: Relations between GPS, GST, and GAL weeks

Recommendation to avoid storing redundant navigation messages

(8.3)

14 Nov 2006 Tables A6/10/12: Format error in BROADCAST ORBIT – n: 3X

→ 4X. Examples were OK.

21 Nov 2006 Marker type NON_PHYSICAL added

19-Dec-2006 Table A4: Example of SYS / DCBS APPLIED was wrong.

13-Mar-2007 Paragraph 3.3: Leftover from RINEX version 2 regarding wavelength

factor for squaring- type receiver removed and clarified.

14-Jun-2007 Paragraph 5.11: Clarification regarding the observation record length

28-Nov-2007 Frequency numbers for GLONASS –7..+12 (BROADCAST ORBIT

– 2)

Version 3.01

22-Jun-2009 Phase cycle shifts

## Page 7

RINEX Version 3.03 2

Galileo: BOC-tracking of an MBOC-modulated signal

Compass satellite system: Identifier and observation codes

Code for GPS L1C

Header records for GLONASS slot and frequency numbers

Order of data records

Galileo nav. mess record BROADCAST ORBIT – 5: Bits ¾

reserved for Galileo internal use

Version 3.02 – IGS and RTCM-SC104

19-Nov-2011 Added Quasi Zenith Satellite System (QZSS) Constellation

Updated text, tables and graphics

Added Appendix Table 19 - phase alignment table

21-Jan-2012 Split the Constellation table into a table for each GNSS

Added QZSS to the documentation

Edited text to improve clarity

Corrected sign in the phase alignment table,

Removed QZSS P signals

9-May-2012 Edited text to improve clarity,

Updated phase alignment table,

Changed Met PGM / RUN BY / DATE to support 4digit year as in all

other records also changed format to support 4 digit year for met.

Observation record, Changed SYS / PHASE SHIFTS to SHIFT

29-Nov-2012 Changed Table1 and 2 to Figure 1 and 2.

Updated all Table numbers.

Changed file naming convention, Section 4. Added Appendix Table

A1 and increased all, updated all Appendix numbers

Removed the option of supporting unknown tracking mode from

Section 5.1.

Harmonized L1C(new) signal identifiers for QZSS and GPS See :

Table 2 and 6.

Updated BeiDou System (BDS) (was Compass) information

throughout the document added new BDS ephemeris definition to

Appendix. (Based on input from the BDS Office)

Corrected GLONASS SLOT/FRQ format in section 9.5, changed

message status from optional to mandatory (See: Appendix Table

A2).

Added new mandatory GLONASS Code Phase Bias header record

See section 9.9

11-Mar-2013 Updated Sections: 4.x, made .rnx the file name extension and updated

Figure 2; 9.1 to clarify the use of the phase alignment header; A1

Edited to reflect file extension of *.rnx; A14 - BDS ephemeris

changed AODC to IODC and AODE to IODE (as indicated by

BDS Authority and new ICD); Appendix Table A19

(Changed GLONASS Reference Signals to C1-C2) and explicitly

identified reference signal for all constellations and frequencies.

26-Mar-2013 Changed BeiDou to BDS for conform to ICD.

In table 7 changed BDS signals from: C2x to C1x to more closely

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 8

RINEX Version 3.03 3

reflect existing bands in tables 2-6 and Appendix Tables A2 and A21.

Updated Section 8.1: First paragraph updated to indicate current

number of leap seconds; added a row to Table 12 to show the

relationship between GPS week and BDT week. Added a table to

show the approximate relationship of BDT to GPS time.

Changed order of file type: from OG to GO etc in Appendix Table

A1.

Updated Appendix table A21 to show X signals and indicate that the

X phase is to be aligned to the frequencies reference signal.

Fixed a few small typos in A21 for GPS: L1C-D/P and D+P.

RINEX 3.02 Released

03-Dec-2013  Corrected Sections 3.1 to read: TIME OF FIRST OBS rather than

start time record.

 Added text to Section 5.4 and A3 to indicate that the Loss of Lock

Bit is the least significant bit.

 In section 9.5 GLONASS Slot and Frequency Numbers, changed

optional to mandatory (as it was changed from optional to

mandatory in version 3.02).

 In Table A2 record: SYS / # / OBS TYPES changed Satellite

system code (G/R/E/J/C/S/M to G/R/E/J/C/S).

 In Section 5.7 added descriptive text to Table 12 (header-

changed Signal to Carrier and in the body) .

 In Table A3 record OBSERVATION changed 5: from average

to good.

 In note 4 after A8 …Galileo System Time added (GST) to make

the following description more explicit.

 Appendix A14 BeiDou Nav. removed the – sign in front of Cis

24-Jan-2014  Appendix A10 Section SV / EPOCH / SV CLK changed TauN to

–TauN to agree with section 8.3.1

4-Apr-2014  Galileo Table A8 , BROADCAST ORBIT-5 - Bits 0-2 : changed

from non-exclusive to exclusive (only one bit can be set). In

****) section added (GST)

 Corrected Table A23 - BeiDou B1 phase correction column signal

indicator to agree with BeiDou Table 9.

 Corrected Table A2 - Band 1 = E1 (Was E2-L1-E1) to agree with

Galileo Table 6.

 In Table A5 - optional message TIME SYSTEM CORR added text

to clarify the parameters T and W for BeiDou.

 Section 8.3.1 Corrected typo in last line of first paragraph.

6-May-2014  Updated Section 10 Document References

 Changed A6 from GPS/QZSS to GPS only as A12 contains a

description of the QZSS ephemeris.

 Corrected typo in Appendix 23 note 1: L2E changed to L2W

21-May-2014  Appendix A4 added two observation file header examples

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 9

RINEX Version 3.03 4

 Appendix A6 GPS Navigation, Broadcast Orbit-7 Fit Interval,

clarified in accordance with IS-GPS-200H section 20.3.3.4.3.1

 Appendix A9 added Galileo navigation file example

 Appendix A12 QZSS Navigation, Broadcast Orbit-5 field 4 allow

define L2P flag to be set to one section 5.2.2.2.3(6)); Broadcast

Orbit-7 Fit Interval clarified in accordance with section

5.2.2.2.4(4), IS-QZSS 1.5.

 Appendix A13 added QZSS navigation file example

 Appendix A15 added BeiDou navigation file example

26-May-2014  Removed “Added” from section 9.8, 9.9 and 9.10 titles

 Added a note to section 9.9 (GLONASS COD/PHS/BIS) to allow

unknown GLONASS code/phase, observation alignment in

exceptional cases. Added a note to Appendix A2: GLONASS

COD/PHS/BIS record definition.

9-June-2014  Edited Section 6.11 and Table A6:BO7 (GPS) to indicate that the

GPS fit interval field should contain a period in hours. Edited

Table A12:BO7 (QZSS) to make it clear that the fit interval is a

flag and not a time period. Added support for unknown fit interval

(specified as an empty field).

10-June-2014  Removed the reference to QZSS in Appendix 6 SC/EPOCH/SV

CLK record as there is now a QZSS navigation file in Appendix

12.

 Corrected A12 QZSS ICD reference from 5.1.2.3.2 to 5.1.2.1.3.2

12-June-2014  Added text to section 9.9 to indicate that when the GLONASS

COD/PHS/BIS measurements are unknown then all fields in the

record should be left blank (added an example). Updated the

descriptive text in Table A2 GLONASS COD/PHS/BIS.

10-July-2014  Corrected Appendix numbers in body of the text

 Added Note after BeiDou Table 9 to indicate that some RINEX

3.02 files may still use the 3.01 B1 coding convention

16-Jul-2014 - Section 9.1 Replaced: Phase observations must be shifted by the

respective fraction of a cycle, either directly by the receiver or by

a correction program or the RINEX conversion program, prior to

RINEX file generation, to align them to each other with: All phase

observations must be aligned to the designated constellation and

frequency reference signal as specified in Appendix Table A23,

either directly by the receiver or by a correction program or the

RINEX conversion program, prior to RINEX file generation.

Additionally, all data must be aligned with the appropriate

reference signal indicated in Appendix Table A23 even when the

receiver or reporting device is not tracking and/or providing data

from that reference signal e.g. Galileo L5X phase data must be

aligned to L5I.

29-Jul-2014 - Minor edits

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 10

RINEX Version 3.03 5

- Updated last paragraph of section 8.4 re TIME SYSTEM CORR

- Corrected QZSS Appendix Table A12 PRN/EPOCH/SV CLK

record format specification

- Reformatted Appendix Table A12 and A14

31-Oct-2014 - BeiDou updates: changed B1 signal identifiers to C2x;

observation and navigation Header “LEAP SECONDS messages

changed to support both GPS and BDS leap seconds; updated the

navigation header message “IONOSPHERIC CORR” to support

different ionospheric correction parameters from each satellite.

Updated BDS navigation message Table A14. Updated Sections

8.1 and 8.2.

- Added Description of the Indian Regional Navigation Satellite

System (IRNSS) to the document, Updated RINEX release

number to 3.03 Draft 1

19-Jan-2015 - Table 2 grammatical error corrections

- Updated broken http: link on page 17

- Updated Leap Second definition in section 8.1 and Appendix A2

and A5

- Update Galileo Appendix A8 navigation line 5 to indicate the

exclusive and non-exclusive bits

- Added text to further clarify BeiDou Appendix A14 AODE and

AODC definition

- Updated IRNSS Appendix 18 line 5, week number to indicate

that the Week Number is aligned with the GPS week number

- Added IRNSS phase alignment information to Appendix 23

15-April-2015 - Editorial changes/corrections: in section 6.6 specified a new

acronym Blank Not Known (BNK), 8.1 added text to indicate the

relationship between BDT and GPS Time at start of BDT,

corrected typo in Section 8.2 last paragraph changed GLO to

UTC, clarified Section 9.2 Galileo Tracking, clarified Section 9.6

re BDS, removed Section 9.8 RINEX Meteorological section

- Re-formatted Appendix Table A2 and A5

- Clarified Observation (A2) and Navigation (A5) “LEAP

SECONDS” record

- Clarified Navigation (A5) file “IONOSPHERIC CORR” record

- QZSS A12-BO-6, TGD blank if not know

- Corrected typo in A22,

- Clarified filename start time

- Minor punctuation and grammatical corrections throughout the

document.

- Removed reference to unknown tracking mode in Appendix Table

A2 message SYS / # / OBS TYPE.

- Updated all table numbers (some tables were not identified),

improved table descriptions.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 11

RINEX Version 3.03 6

- Minor format changes

15-May-2015 - Added paragraph to section 8.3.2 to specify that RINEX parsers

should expect to encounter F/NAV and I/NAV messages in the

same file…

- Removed “The attribute can be left blank if not known. See text!”

text from the end of A2, SYS/#/OBS TYPES as it was decided in

3.02 not to allow unknown signals therefore this no longer applies

- Updated A8 (Galileo Nav. Message), Record 5 Field 2-

Description to specify only I/NAV or F/NAV can be specified

- Corrected A9 (Galileo Nav. Example), Record 5 Field 2 from 519

to 517 to indicate I/NAV in accordance with the field

specification

- Corrected A9, Record 6 Field 1 from 107(broadcast raw value) to

3.12m

25-May-2015 - Corrected Table of contents to show Section 10.0, References

- Section 2 second last paragraph added IRNSS to list of supported

constellations

- Section 5.3 last paragraph concerning event flags added reference

to Appendix A3

- Section 7, last paragraph, edited second sentence to make it more

clear

1-June-2015 - Minor punctuation corrections

- Added C2X signal tracking example to Section 5.1 example list

- Added paragraph 3 to section 5.1, to indicate only know tracking

modes are supported in RINEX 3.02 and 3.03

- Added note to Appendix A2, SYS/#/OBS TYPES to indicate only

know tracking modes are allowed in RINEX 3.02 and 3.03

- Table 4 in L1 and L2 frequency bands, changed P to P (AS off) to

improve clarity

- Added :”(e.g. units employing a Selective Availability Anti-

Spoofing Module (SAASM))“ to last paragraph on page 18 to

improve clarity.

24-June-2015 - Updated the last paragraph of Section 1 (RINEX 3.03)

- Clarified Section 4: Changed Obs. Freq. To Data Frequency and

update Appendix Table A1 to match

- Added text to Section 8.3.2 (Galileo Navigation) to describe Issue

of Data and related parameters

- Added reference to Galileo ICD in Appendix A8 BROADCAST

ORBIT-6

- Added Galileo Examples to Appendix Table A9

29-June-2015 - Updated BeiDou RINEX 3.02 C1x-C2x Note below Table 9 for

clarity

- Section 8.3.2 added Galileo ICD publication year to reference

- Corrected Beidou C1 to C2 encoding in Appendix A2 SYS/#/OBS

TYPES and in Appendix A4 example 2 and 3

- Appendix A2 and A5 Clarified LEAP SECONDS Day number to

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 12

RINEX Version 3.03 7

be 0-6 for BeiDou and 1-7 for GPS and other constellations.

- Updated all Appendix table references to contain Axx, to

differentiate between body and Appendix tables.

14-July-2015 - Updated LEAP SECONDS record description in Appendix A2

and A5

- Converted Galileo SISA values in Appendix A9 from broadcast

value into metres in accordance with RINEX specification and

Galileo ICD Section 5.1.11 Table 76

- RINEX 3.03 Released

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 13

RINEX Version 3.03 8

1. THE PHILOSOPHY AND HISTORY OF RINEX

The first proposal for the Receiver Independent Exchange Format RINEX was developed by

the Astronomical Institute of the University of Bern for the easy exchange of the Global

Positioning System (GPS) data to be collected during the first large European GPS campaign

EUREF 89, which involved more than 60 GPS receivers of 4 different manufacturers. The

governing aspect during the development was the following fact:

Most geodetic processing software for GPS data use a well-defined set of observables:

 the carrier-phase measurement at one or both carriers (actually being a measurement on

the beat frequency between the received carrier of the satellite signal and a receiver-

generated reference frequency)

 the pseudorange (code) measurement, equivalent to the difference of the time of

reception (expressed in the time frame of the receiver) and the time of transmission

(expressed in the time frame of the satellite) of a distinct satellite signal

 the observation time being the reading of the receiver clock at the instant of validity of

the carrier-phase and/or the code measurements

Usually the software assumes that the observation time is valid for both the phase and the code

measurements, and for all satellites observed.

Consequently all these programs do not need most of the information that is usually stored by the

receivers: they need phase, code, and time in the above mentioned definitions, and some station-

related information like station name, antenna height, etc.

Until now two major format versions have been developed and published:

 The original RINEX Version 1 presented at and accepted by the 5th International

Geodetic Symposium on Satellite Positioning in Las Cruces, 1989. [Gurtner et al. 1989],

[Evans 1989]

 RINEX Version 2 presented at and accepted by the Second International Symposium of

Precise Positioning with the Global Positioning system in Ottawa, 1990, mainly adding

the possibility to include tracking data from different satellite systems (GLONASS,

SBAS). [Gurtner and Mader 1990a, 1990b], [Gurtner 1994].

Several subversions of RINEX Version 2 have been defined:

 Version 2.10: Among other minor changes allowing for sampling rates other

than integer seconds and including raw signal strengths as new observables.

[Gurtner 2002]

 Version 2.11: Includes the definition of a two-character observation code for

L2C pseudoranges and some modifications in the GEO NAV MESS files

[Gurtner and Estey 2005]

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 14

RINEX Version 3.03 9

 Version 2.20: Unofficial version used for the exchange of tracking data from

spaceborne receivers within the IGS LEO pilot project [Gurtner and Estey

2002]

As spin-offs of this idea of a receiver-independent GPS exchange format, other RINEX-like

exchange file formats have been defined, mainly used by the International GNSS Service IGS:

 Exchange format for satellite and receiver clock offsets

determined by processing data of a GNSS tracking network [Ray

and Gurtner 2010]

 Exchange format for the complete broadcast data of space-

based augmentation systems SBAS. [Suard et al. 2004]

 IONEX: Exchange format for ionosphere models determined by

processing data of a GNSS tracking network [Schaer et al. 1998]

 ANTEX: Exchange format for phase center variations of

geodetic GNSS antennae [Rothacher and Schmid 2010]

The upcoming European Navigation Satellite System Galileo and the enhanced GPS with new

frequencies and observation types, especially the possibility to track frequencies on different

channels, requires a more flexible and more detailed definition of the observation codes.

To improve the handling of the data files in case of “mixed” files, i.e. files containing tracking

data of more than one satellite system, each one with different observation types, the record

structure of the data record has been modified significantly and following several requests, the

limitation to 80 characters length has been removed.

As the changes are quite significant, they lead to a new RINEX Version 3. The new version also

includes the unofficial Version 2.20 definitions for space-borne receivers.

The major change leading to the release of version 3.01 was the requirement to generate

consistent phase observations across different tracking modes or channels, i.e. to apply ¼-cycle

shifts prior to RINEX file generation, if necessary, to facilitate the processing of such data.

RINEX 3.02 added support for the Japanese, Quasi Zenith Satellite System (QZSS), additional

information concerning Beidou (based on the released ICD) and a new message to enumerate

GLONASS code phase biases.

RINEX 3.03 adds support for the Indian Regional Satellite System (IRNSS) and clarifies several

implementation issues in RINEX 3.02. RINEX 3.03 also changes the BeiDou B1 signal

convention back to the 3.01 convention where all B1 signals are identified as C2x (not C1 as in

RINEX 3.02). Another issue with the implementation of 3.02 was the GPS navigation message

fit interval field. Some implementations wrote the flag and others wrote a time interval. This

release specifies that the fit interval should be a time period for GPS and a flag for QZSS.

Updated Galileo Navigation section 8.3.2 to clarify the issues related to Issue of Data (IOD).

Updated document to clarify that only known tracking modes can be encoded in RINEX 3.03.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 15

RINEX Version 3.03 10

2. GENERAL FORMAT DESCRIPTION

The RINEX version 3.XX format consists of three ASCII file types:

1. Observation data file

2. Navigation message file

3. Meteorological data file

Each file type consists of a header section and a data section. The header section contains global

information for the entire file and is placed at the beginning of the file. The header section

contains header labels in columns 61-80 for each line contained in the header section. These

labels are mandatory and must appear exactly as given in these descriptions and examples.

The format has been optimized for minimum space requirements independent from the number

of different observation types of a specific receiver or satellite system by indicating in the header

the types of observations to be stored for this receiver and the satellite systems having been

observed. In computer systems allowing variable record lengths, the observation records may be

kept as short as possible. Trailing blanks can be removed from the records. There is no maximum

record length limitation for the observation records.

Each Observation file and each Meteorological Data file basically contain the data from one site

and one session. Starting with Version 2 RINEX also allows including observation data from

more than one site subsequently occupied by a roving receiver in rapid static or kinematic

applications. Although Version 2 and higher allow insertion of certain header records into the

data section, it is not recommended to concatenate data from more than one receiver (or antenna)

into the same file, even if the data do not overlap in time.

If data from more than one receiver have to be exchanged, it would not be economical to include

the identical satellite navigation messages collected by the different receivers several times.

Therefore, the navigation message file from one receiver may be exchanged or a composite

navigation message file created, containing non-redundant information from several receivers in

order to make the most complete file.

The format of the data records of the RINEX Version 1 navigation message file was identical to

the former NGS exchange format. RINEX version 3 navigation message files may contain

navigation messages of more than one satellite system (GPS, GLONASS, Galileo, Quasi Zenith

Satellite System (QZSS), BeiDou System (BDS), Indian Regional Navigation Satellite System

(IRNSS) and SBAS).

The actual format descriptions as well as examples are given in the Appendix Tables at the end

of the document.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 16

RINEX Version 3.03 11

3. BASIC DEFINITIONS

GNSS observables include three fundamental quantities that need to be defined: Time, Phase,

and Range.

3.1 Time

The time of the measurement is the receiver time of the received signals. It is identical for the

phase and range measurements and is identical for all satellites observed at that epoch. For

single-system data files, it is by default expressed in the time system of the respective satellite

system. For mixed files, the actual time system used must be indicated in the TIME OF FIRST

OBS header record.

3.2 Pseudo-Range:

The pseudo-range (PR) is the distance from the receiver antenna to the satellite antenna including

receiver and satellite clock offsets (and other biases, such as atmospheric delays):

PR = distance + c * (receiver clock offset –satellite clock offset + other biases)

so that the pseudo-range reflects the actual behaviour of the receiver and satellite clocks. The

pseudo-range is stored in units of meters.

See also clarifications for pseudoranges in mixed GPS/GLONASS/Galileo/QZSS/BDS files in

chapter 8.2.

3.3 Phase

The phase is the carrier-phase measured in whole cycles. The half-cycles measured by squaring-

type receivers must be converted to whole cycles and flagged by the respective observation code

(see Table 4 and Section 5.4, GPS only).

The phase changes in the same sense as the range (negative doppler). The phase observations

between epochs must be connected by including the integer number of cycles.

The observables are not corrected for external effects such as: atmospheric refraction, satellite

clock offsets, etc.

If necessary, phase observations are corrected for phase shifts needed to guarantee consistency

between phases of the same frequency and satellite system based on different signal channels

(See Section 9.1 and Appendix A23).

If the receiver or the converter software adjusts the measurements using the real-time-derived

receiver clock offsets dT(r), the consistency of the 3 quantities phase / pseudo-range / epoch

must be maintained, i.e. the receiver clock correction should be applied to all 3 observables:

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 17

RINEX Version 3.03 12

Time (corr) = Time(r) - dT(r)

PR (corr) = PR (r) - dT(r)*c

phase (corr) = phase (r) - dT(r)*freq

Table 1: Observation Corrections for Receiver Clock Offset

3.4 Doppler

The sign of the doppler shift as additional observable is defined as usual: Positive for

approaching satellites.

3.5 Satellite numbers

Starting with RINEX Version 2 the former two-digit satellite numbers nn are preceded by a

one-character system identifier s as per Figure 1.

Figure 1: Satellite numbers and Constellation Identifiers

The same satellite system identifiers are also used in all header records when appropriate.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 18

RINEX Version 3.03 13

4. THE EXCHANGE OF RINEX FILES

The original RINEX file naming convention was implemented in the MS-DOS era when file

names were restricted to 8.3 characters. Modern operating systems typically support 255

character file names. The goal of the new file naming convention is to be more descriptive,

flexible and extensible than the RINEX 2.11 file naming convention. Figure 2 below lists the

elements of the RINEX 3.02 (and subsequent versions) file naming convention.

Figure 2: Recommended filename parameters.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 19

RINEX Version 3.03 14

All elements are fixed length and are separated by an underscore “_” except for the: file type and

compression fields that use a period “.” as a separator. Fields must be padded with zeros to fill

the field width. The file compression field is optional. See Appendix A1 for a detailed

description of the RINEX 3.02 (and subsequent versions) file naming convention. Table 2 below

lists sample file names for GNSS observation and navigation files.

File Name Comments

ALGO00CAN_R_20121601000_01H_01S_MO.rnx Mixed RINEX GNSS observation file

containing 1 hour of data, with an

observation every second.

ALGO00CAN_R_20121601000_15M_01S_GO.rnx GPS RINEX observation file containing

15 minutes of data, with an observation

every second.

ALGO00CAN_R_20121601000_01H_05Z_MO.rnx Mixed RINEX GNSS observation file

containing 1 hour of data, with 5

observations per second.

ALGO00CAN_R_20121601000_01D_30S_GO.rnx GPS RINEX observation file containing 1

day of data, with an observation every 30

seconds.

ALGO00CAN_R_20121601000_01D_30S_MO.rnx Mixed RINEX GNSS observation file

containing 1 day of data, with an

observation every 30 seconds.

ALGO00CAN_R_20121600000_01D_GN.rnx RINEX GPS navigation file, containing

one day’s data.

ALGO00CAN_R_20121600000_01D_RN.rnx RINEX GLONASS navigation file,

containing one day’s data

ALGO00CAN_R_20121600000_01D_MN.rnx RINEX mixed navigation file, containing

one day’s data

Table 2: Description of Filename Parameters

In order to further reduce the size of observation files Yuki Hatanaka developed a compression

scheme that takes advantage of the structure of the RINEX observation data by forming higher-

order differences in time between observations of the same type and satellite. This compressed

file is also an ASCII file that is subsequently compressed again using standard compression

programs.

More information on the Hatanaka compression scheme can be found in:

 http://terras.gsi.go.jp/ja/crx2rnx.html

 IGSMails 1525,1686,1726,1763,1785,4967,4969,4975

The file naming and compression recommendations are strictly speaking not part of the RINEX

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 20

RINEX Version 3.03 15

format definition. However, they significantly facilitate the exchange of RINEX data in large

user communities like IGS.

5. RINEX VERSION 3 FEATURES

This chapter contains features that have been introduced for RINEX Version 3.

5.1 Observation codes

The new signal structures for GPS, Galileo and BDS make it possible to generate code and phase

observations based on one or a combination of several channels: Two-channel signals are

composed of I and Q components, three-channel signals of A, B, and C components. Moreover a

wideband tracking of a combined E5a + E5b Galileo frequency is possible. In order to keep the

observation codes short but still allow for a detailed characterization of the actual signal

generation, the length of the codes is increased from two (Version 1 and 2) to three by adding a

signal generation attribute. The observation code tna consists of three parts:

t :observation type C = pseudorange, L = carrier phase, D = doppler, S = signal

strength)

n :band / frequency 1, 2,...,8

a : attribute tracking mode or channel, e.g., I, Q, etc

Table 3: Observation Code Components

Examples:

 L1C: C/A code-derived L1 carrier phase (GPS, GLONASS) Carrier phase on E2-L1-

E1 derived from C channel (Galileo)

 C2L: L2C pseudorange derived from the L channel (GPS)

 C2X: L2C pseudorange derived from the mixed (M+L) codes (GPS)

Tables 4 to 10 describe each GNSS constellation and the frequencies and signal encoding

methods used.

Unknown tracking modes are not supported in RINEX 3.02 and 3.03. Only the complete

specification of all signals is allowed i.e. all three fields must be defined as specified in Tables 4-

10.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 21

RINEX Version 3.03 16

Observation Codes

GNSS Freq. Band

Channel or Code Pseudo Carrier Signal

System /Frequency Doppler

Range Phase Strength

GPS C/A C1C L1C D1C S1C

L1C (D) C1S L1S D1S S1S

L1C (P) C1L L1L D1L S1L

L1C (D+P) C1X L1X D1X S1X

P (AS off) C1P L1P D1P S1P

L1/1575.42

Z-tracking and similar

C1W L1W D1W S1W

(AS on)

Y C1Y L1Y D1Y S1Y

M C1M L1M D1M S1M

codeless L1N D1N S1N

C/A C2C L2C D2C S2C

L1(C/A)+(P2-P1)

C2D L2D D2D S2D

(semi-codeless)

L2C (M) C2S L2S D2S S2S

L2C (L) C2L L2L D2L S2L

L2C (M+L) C2X L2X D2X S2X

L2/1227.60

P (AS off) C2P L2P D2P S2P

Z-tracking and similar

C2W L2W D2W S2W

(AS on)

Y C2Y L2Y D2Y S2Y

M C2M L2M D2M S2M

codeless L2N D2N S2N

I C5I L5I D5I S5I

L5/1176.45 Q C5Q L5Q D5Q S5Q

I+Q C5X L5X D5X S5X

Table 4 : RINEX Version 3.03 GPS Observation Codes

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 22

RINEX Version 3.03 17

Observation Codes

GNSS Freq. Band Channel or

Pseudo Carrier Signal

System /Frequency Code Doppler

Range Phase Strength

GLONASS G1/ C/A C1C L1C D1C S1C

1602+k*9/16

P C1P L1P D1P S1P

k= -7….+12

C/A

C2C L2C D2C S2C

G2/ (GLONASS M)

1246+k*716 P C2P L2P D2P S2P

I C3I L3I D3I S3I

G3 / 1202.025 Q C3Q L3Q D3Q S3Q

I+Q C3X L3X D3X S3X

Table 5 : RINEX Version 3.03 GLONASS Observation Codes

Observation Codes

GNSS Freq. Band

Channel or Code Pseudo Carrier Signal

System /Frequency Doppler

Range Phase Strength

Galileo A PRS C1A L1A D1A S1A

B I/NAV OS/CS/SoL C1B L1B D1B S1B

E1 / 1575.42 C no data C1C L1C D1C S1C

B+C C1X L1X D1X S1X

A+B+C C1Z L1Z D1Z S1Z

I F/NAV OS C5I L5I D5I S5I

E5a / 1176.45 Q no data C5Q L5Q D5Q S5Q

I+Q C5X L5X D5X S5X

I I/NAV OS/CS/SoL C7I L7I D7I S7I

E5b / 1207.140 Q no data C7Q L7Q D7Q S7Q

I+Q C7X L7X D7X S7X

I C8I L8I D8I S8I

E5(E5a+E5b) /

Q C8Q L8Q D8Q S8Q

1191.795

I+Q C8X L8X D8X S8X

A PRS C6A L6A D6A S6A

B C/NAV CS C6B L6B D6B S6B

E6 / 1278.75 C no data C6C L6C D6C S6C

B+C C6X L6X D6X S6X

A+B+C C6Z L6Z D6Z S6Z

Table 6 : RINEX Version 3.03 Galileo Observation Codes

For Galileo the band/frequency number n does not necessarily agree with the official frequency

numbers: n = 7 for E5b, n = 8 for E5a+b.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 23

RINEX Version 3.03 18

Observation Codes

GNSS Freq. Band/ Channel or

Pseudo Carrier Doppler Signal

System Frequency Code

Range Phase Strength

L1 / 1575.42 C/A C1C L1C D1C S1C

I C5I L5I D5I S5I

SBAS

L5 / 1176.45 Q C5Q L5Q D5Q S5Q

I+Q C5X L5X D5X S5X

Table 7 : RINEX Version 3.03 SBAS Observation Codes

Observation Codes

GNSS Freq. Band / Channel or

Pseudo Carrier Signal

System Frequency Code Doppler

Range Phase Strength

QZSS C/A C1C L1C D1C S1C

L1C (D) C1S L1S D1S S1S

L1 / 1575.42 L1C (P) C1L L1L D1L S1L

L1C (D+P) C1X L1X D1X S1X

L1-SAIF C1Z L1Z D1Z S1Z

L2C (M) C2S L2S D2S S2S

L2 / 1227.60 L2C (L) C2L L2L D2L S2L

L2C (M+L) C2X L2X D2X S2X

I C5I L5I D5I S5I

L5 / 1176.45 Q C5Q L5Q D5Q S5Q

I+Q C5X L5X D5X S5X

S C6S L6S D6S S6S

LEX(6) / 1278.75 L C6L L6L D6L S6L

S+L C6X L6X D6X S6X

Table 8 : RINEX Version 3.03 QZSS Observation Codes

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 24

RINEX Version 3.03 19

Observation Codes

GNSS

Freq. Band / Frequency Channel or Code

System Pseudo Carrier Signal

Doppler

Range Phase Strength

BDS I C2I L2I D2I S2I

B1 / 1561.098 Q C2Q L2Q D2Q S2Q

I+Q C2X L2X D2X S2X

I C7I L7I D7I S7I

B2 / 1207.14 Q C7Q L7Q D7Q S7Q

I+Q C7X L7X D7X S7X

I C6I L6I D6I S6I

B3 / 1268.52 Q C6Q L6Q D6Q S6Q

I+Q C6X L6X D6X S6X

Table 9 : RINEX Version 3.03 BDS Observation Codes

Note: When reading a RINEX 3.02 file, both C1x and C2x coding should be accepted and treated

as C2x in RINEX 3.03.

Observation Codes

GNSS

Freq. Band / Frequency Channel or Code

System Pseudo Carrier Signal

Doppler

Range Phase Strength

IRNSS A SPS C5A L5A D5A S5A

B RS (D) C5B L5B D5B S5B

L5 / 1176.45

C RS (P) C5C L5C D5C S5C

B+C C5X L5X D5X S5X

A SPS C9A L9A D9A S9A

S / 2492.028 B RS (D) C9B L9B D9B S9B

C RS (P) C9C L9C D9C S9C

B+C C9X L9X D9X S9X

Table 10 : RINEX Version 3.03 IRNSS Observation Codes

GPS-SBAS and –pseudorandom noise (PRN) code assignments:

See e.g., http://www.losangeles.af.mil/library/factsheets/factsheet.asp?id=8618

Antispoofing (AS) of GPS: True codeless GPS receivers (squaring-type receivers) use the attribute

N. Semi-codeless receivers tracking the first frequency using C/A code and the second frequency

using some codeless options use attribute D. Z-tracking under AS or similar techniques to recover

pseudorange and phase on the “P-code” band use attribute W. Y-code tracking receivers (e.g. units

employing a Selective Availability Anti-Spoofing Module (SAASM)) use attribute Y.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 25

RINEX Version 3.03 20

Appendix Table A23 enumerates the fractional phase corrections required to align each signal to

the frequencies reference signal.

As all observations affected by “AS on” now get their own attribute (codeless, semi-codeless, Z-

tracking and similar) the Antispoofing flag introduced into the observation data records of RINEX

Version 2 has become obsolete.

5.2 Satellite system-dependent list of observables

The order of the observations stored per epoch and satellite in the observation records is given by a

list of observation codes in a header record. As the types of the observations actually generated by a

receiver may heavily depend on the satellite system, RINEX Version 3 requests system-dependent

observation code lists (header record type SYS / # / OBS TYPES).

5.3 Marker type

In order to indicate the nature of the marker, a MARKER TYPE header record has been defined.

Proposed keywords are given in Table 11.

Marker Type Description

Geodetic Earth-fixed high-precision monument

Non Geodetic Earth-fixed low-precision monument

Non_Physical Generated from network processing

Space borne Orbiting space vehicle

Air borne Aircraft, balloon, etc.

Water Craft Mobile water craft

Ground Craft Mobile terrestrial vehicle

Fixed Buoy “Fixed” on water surface

Floating Buoy Floating on water surface

Floating Ice Floating ice sheet, etc

Glacier “Fixed” on a glacier

Ballistic Rockets, shells, etc

Animal Animal carrying a receiver

Human Human being

Table 11: Proposed Marker Type Keywords

The record is required except for GEODETIC and NON_GEODETIC marker types.

Attributes other than GEODETIC and NON_GEODETIC will tell the user program that the data

were collected by a moving receiver. The inclusion of a “start moving antenna” record (event

flag 2) into the data body of the RINEX file is therefore not necessary. Event flags 2 and 3 (See

Appendix A3) are still necessary to flag alternating kinematic and static phases of a receiver

visiting multiple earth-fixed monuments, however. Users may define other project-dependent

keywords.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 26

RINEX Version 3.03 21

5.4 Half-wavelength observations, half-cycle ambiguities

Half-wavelength observations (collected by codeless squaring techniques) get their own

observation codes. A special wavelength factor header line and bit 1 of the LLI flag in the

observation records are no longer necessary. If a receiver changed between squaring and full

cycle tracking within the time period of a RINEX file, observation codes for both types of

observations have to be inserted into the respective SYS / # / OBS TYPES header record.

Half-wavelength phase observations are stored in full cycles. Ambiguity resolution however has

to account for half wavelengths!

Full-cycle observations collected by receivers with possible half cycle ambiguity (e.g., during

acquisition or after loss of lock) are to be flagged with Loss of Lock Indicator bit 1 set (see

Appendix Table A3). Note: The loss of lock bit is the least significant bit.

5.5 Scale factor

The optional SYS / SCALE FACTOR record allows the storage of phase data with 0.0001 of

a cycle resolution, if the data was multiplied by a scale factor of 10 before being stored into the

RINEX file. This feature is used to increase resolution by 10, 100, etc only. It is a modification

of the Version 2.20 OBS SCALE FACTOR record.

5.6 Information about receivers on a vehicle

For the processing of data collected by receivers on a vehicle, the following additional

information can be provided by special header records:

 Antenna position (position of the antenna reference point) in a body-fixed coordinate

system: ANTENNA: DELTA X/Y/Z

 Boresight of antenna: The unit vector of the direction of the antenna axis towards the

GNSS satellites. It corresponds to the vertical axis on earth-bound antenna:

ANTENNA: B.SIGHT XYZ

 Antenna orientation: Zero-direction of the antenna. Used for the application of

“azimuth”-dependent phase center variation models (see 6.14 below):

ANTENNA: ZERODIR XYZ

 Current center of mass of the vehicle (for space borne receivers):

CENTER OF MASS: XYZ

 Average phase center position: ANTENNA: PHASECENTER (see below)

All three quantities have to be given in the same body-fixed coordinate system. The attitude of

the vehicle has to be provided by separate attitude files in the same body-fixed coordinate

system.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 27

RINEX Version 3.03 22

5.7 Signal strength

The generation of the RINEX signal strength indicators sn_rnx in the data records (1 = very

weak,…,9 = very strong) are standardized in case the raw signal strength1 sn_raw is given in

dbHz:

sn_rnx = MIN(MAX(INT(sn_raw/6),1),9)

Carrier to Noise ratio(dbHz) Carrier to Noise ratio(RINEX)

< 12 1 (minimum possible signal strength)

12-17 2

18-23 3

24-29 4

30-35 5 (threshold for good tracking)

36-41 6

42-47 7

48-53 8

≥ 54 9 (maximum possible signal strength)

Table 12: Standardized S/N Indicators

The raw carrier to noise ratio can be optionally (preferred) stored as Sna observations in the

data records and should be given in dbHz if possible. The new SIGNAL STRENGTH UNIT

header record can be used to indicate the units of these observations.

5.8 Date/time format in the PGM / RUN BY / DATE header record

The format of the generation time of the RINEX files stored in the second header record PGM /

RUN BY / DATE is now defined to be

yyyymmdd hhmmss zone

zone: 3 – 4 character code for the time zone

It is recommended to use UTC as the time zone. Set zone to LCL if local time was used with

unknown local time system code.

1S/N is the raw S/N at the output of the correlators, without attempting to recover any correlation losses

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 28

RINEX Version 3.03 23

5.9 Antenna phase center header record

An optional header record for antenna phase center positions ANTENNA: PHASECENTER is

defined to allow for higher precision positioning without need of additional external antenna

information. It can be useful in well-defined networks or applications. It contains the position of

an average phase center relative to the antenna reference point (ARP) for a specific frequency

and satellite system. On vehicles the phase center position can be reported in the body-fixed

coordinate system (ANTENNA: DELTA X/Y/Z). See 6.14 below. See section 5.15 regarding

the use of phase center variation corrections.

5.10 Antenna orientation

Header records have been defined to report the orientation of the antenna zero-direction as well

as the direction of its vertical axis (bore-sight) if mounted tilted on a fixed station. The header

records can also be used for antennas on vehicles. See 6.14 below.

5.11 Observation data records

Aside from the new observation code definitions, the most conspicuous modification of the

RINEX format concerns the observation records. As the types of the observations and their order

within a data record depend on the satellite system, the new format should make it easier for

programs as well as human beings to read the data records. Each observation record begins with

the satellite number snn, the epoch record starts with special character >. It is now also much

easier to synchronize the reading program with the next epoch record in case of a corrupted data

file or when streaming observation data in a RINEX-like format. The record length limitation to

80 characters of RINEX Versions 1 and 2 has been removed.

For the following list of observation types for the four satellite systems G,S,E,R :

G 5 C1P L1P L2X C2X S2X SYS / # / OBS TYPES

R 2 C1C L1C SYS / # / OBS TYPES

E 2 L1B L5I SYS / # / OBS TYPES

S 2 C1C L1C SYS / # / OBS TYPES

Table 13: Example Observation Type Records

The epoch and observation records are as follows:

> 2006 03 24 13 10 54.0000000 0 7 -0.123456789210

G06 23619095.450 -53875.632 8 -41981.375 5 23619112.008 24.158

G09 20886075.667 -28688.027 9 -22354.535 6 20886082.101 38.543

G12 20611072.689 18247.789 9 14219.770 8 20611078.410 32.326

R21 21345678.576 12345.567 5

R22 22123456.789 23456.789 5

E11 65432.123 5 48861.586 7

S20 38137559.506 335849.135 9

Table 14: Example Observation Data Records

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 29

RINEX Version 3.03 24

The receiver clock correction in the epoch record has been placed such that it could be preceded

by an identifier to make it system-dependent in a later format revision, if necessary. The clock

correction is optional and is given in units of seconds.

5.12 Ionosphere delay as pseudo-observables

RINEX files could also be used to make available additional information linked to the actual

observations. One such element is the ionosphere delay having been determined or derived from

an ionosphere model. We add the ionosphere phase delay expressed in full cycles of the

respective satellite system-dependent wavelength as pseudo-observable to the list of the RINEX

observables.

T: observation type I = Ionosphere phase delay

n: band/frequency 1, 2, ...,8

a: attribute blank

Table 15: Ionosphere Pseudo-Observable Coding

The ionosphere pseudo-observable has to be included into the list of observables of the

respective satellite system. Only one ionosphere observable per satellite is allowed.

The user adds the ionosphere delay to the raw phase observation of the same wavelength and

converts it to other wavelengths and to pseudorange corrections in meters:

corr_phase(fi) = raw_phase(fi) + d_ion(fi)

corr_prange(fi) = raw_prange(fi) - d_ion(fi)  c/fi

d_ion(fk) = d_ion(fi)  (fi/fk)**2 (accounting for 1st order effects only)

Table 16: Ionosphere Pseudo-Observable Corrections to Observations

d_ion(fi): Given ionospheric phase correction for frequency fi

5.13 Channel numbers as pseudo-observables

For special applications it might be necessary to know the receiver channel numbers having been

assigned by the receiver to the individual satellites. We may include this information as another

pseudo-observable:

- t : observation type: X = Receiver channel number

- n : band / frequency : 1

- a : attribute: blank

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 30

RINEX Version 3.03 25

The lowest channel number allowed is 1 (re-number channels beforehand, if necessary). In case

of a receiver using multiple channels for one satellite, the channels could be packed with two

digits each right-justified into the same data field, order corresponding to the order of the

observables concerned. Format F14.3 according to (<5-nc>(2X),<nc>I2.2,’.000’), nc being the

number of channels.

Restriction: Not more than 5 channels and channel numbers <100.

Examples:

 0910.000 for channels 9 and 10

 010203.000 for channels 1, 2, and 3

-----F14.3----

5.14 Corrections of differential code biases (DCBs)

For special high-precision applications it might be useful to generate RINEX files with

corrections of the differential code biases (DCBs) already applied. There are programs available

to correct the observations in RINEX files for differential code biases (e.g., cc2noncc, J. Ray

2005). This can be reported by special header records SYS / DCBS APPLIED pointing to the

file containing the applied corrections.

5.15 Corrections of antenna phase center variations (PCVs)

For more precise applications, an elevation-dependent or elevation and azimuth-dependent phase

center variation (pcv) model for the antenna (referring to the agreed-upon ARP) should be used.

For special applications it might be useful to generate RINEX files with these variations already

applied. This can be reported by special header records SYS / PCVS APPLIED pointing to

the file containing the PCV correction models.

5.16 Navigation message files

The header portion has been unified (with respect to the format definitions) for all satellite

systems. The first record of each data block now contains the code for the satellite system and

the satellite number.

G06 1999 09 02 17 51 44 -.839701388031D-03 -.165982783074D-10 .000000000000D+00

Table 17: Example of Navigation File Satellite System and Number Definition Record

Header records with system-dependent contents also contain the system identifier. They are

repeated for each system, if applicable.

GPSA .1676D-07 .2235D-07 .1192D-06 .1192D-06 IONOSPHERIC CORR

GPSB .1208D+06 .1310D+06 -.1310D+06 -.1966D+06 IONOSPHERIC CORR

GAL .1234D+05 .2345D+04 -.3456D+03 IONOSPHERIC CORR

Table 18: Example of Navigation File Header IONOSPHERIC CORR Record

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 31

RINEX Version 3.03 26

6. ADDITIONAL HINTS AND TIPS

6.1 Versions

Programs developed to read RINEX files have to verify the version number. Files of newer

versions may look different even if they do not use any of the newer features

6.2 Leading blanks in CHARACTER fields

We propose that routines to read files automatically delete leading blanks in any CHARACTER

input field. Routines creating RINEX files should also left-justify all variables in the

CHARACTER fields.

6.3 Variable-length records

ASCII files usually have variable record lengths, so we recommend to first read each observation

record into a blank string long enough to accommodate the largest possible observation record2

and decode the data afterwards. In variable length records, empty data fields at the end of a

record may be missing, especially in the case of the optional receiver clock offset.

6.4 Blank fields

In view of future modifications we recommend to carefully skip any fields currently defined to

be blank (format fields nX), because they may be assigned to new contents in future versions.

6.5 Order of the header records, order of data records

As the header record descriptors in columns 61-80 are mandatory, the programs reading a

RINEX Version 3 header must decode the header records with formats according to the record

descriptor, provided the records have been first read into an internal buffer.

We therefore propose to allow free ordering of the header records, with the following exceptions:

 The RINEX VERSION / TYPE record must be the first record in a file

 The SYS / # / OBS TYPES record(s) should precede any SYS / DCBS

APPLIED and SYS / SCALE FACTOR records

 The # OF SATELLITES record (if present) should be immediately followed by the

corresponding number of PRN / # OF OBS records. These records may be handy for

documentary purposes. However, since they may only be created after having read whole

raw data file we define them to be optional.

 The END OF HEADER of course is the last record in the header

2

Record is defined by the satellite system with the largest number of possible observables plus any

“pseudo-observables” such as ionosphere etc. The length limitation to 80 characters of RINEX

Versions 1 and 2 has been removed.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 32

RINEX Version 3.03 27

Data records: Multiple epoch observation data records with identical time tags are not

allowed (exception: Event records). Epochs have to appear ordered in time.

6.6 Missing items, duration of the validity of values

Items that are not known at the file creation time can be set to zero or blank (Blank if Not Known

- BNK) or the respective record may be completely omitted. Consequently items of missing

header records will be set to zero or blank by the program reading RINEX files. Trailing blanks

may be truncated from the record.

Each value remains valid until changed by an additional header record.

6.7 Unknown / Undefined observation types and header records

It is a good practice for a program reading RINEX files to make sure that it properly deals with

unknown observation types, header records or event flags by skipping them and/or reporting

them to the user. The program should also check the RINEX version number in the header record

and take proper action if it cannot deal with it.

6.8 Event flag records

The “number of satellites” also corresponds to the number of records of the same epoch

following the EPOCH record. Therefore, it may be used to skip the appropriate number of data

records if certain event flags are not to be evaluated in detail.

6.9 Receiver clock offset

A receiver-derived clock offset can optionally be reported in the RINEX observation files. In

order to remove uncertainties about whether the data (epoch, pseudorange, phase) have been

corrected or not by the reported clock offset, RINEX Versions 2.10 onward requests a clarifying

header record: RCV CLOCK OFFS APPL. It would then be possible to reconstruct the original

observations, if necessary.

6.10 Two-digit years

RINEX version 2 stores the years of data records with two digits only. The header of observation

files contains a TIME OF FIRST OBS record with the full four-digit year; the GPS nav.

messages contain the GPS week numbers. From these two data items the unambiguous year can

easily be reconstructed.

A hundred-year ambiguity occurs in the met data and GLONASS and GEO nav. messages:

instead of introducing a new TIME OF FIRST OBS header line it is safe to stipulate that any

two-digit years in RINEX Version 1 and Version 2.xx files are understood to represent

80-99: 1980-1999

00-79: 2000-2079

Full 4-digit year fields are defined in RINEX version 3 files.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 33

RINEX Version 3.03 28

6.11 Fit interval (GPS navigation message file)

Bit 17 in word 10 of subframe 2 is a “fit interval” flag which indicates the curve-fit interval used

by the GPS Control Segment in determining the ephemeris parameters, as follows (see IS-GPS-

200H, 20.3.3.4.3.1):

0 = 4 hours

1 = greater than 4 hours.

Together with the IODC values and Table 20-XII (of the ICD) the actual fit interval can be

determined. The second value in the last record of each message shall contain the fit interval in

hours determined using IODC, fit flag, and Table 20-XII, according to the Interface Document

IS-GPS-200H. Note: The QZSS fit interval is not defined the same way as it is in GPS, See

Appendix 12.

6.12 Satellite health (GPS navigation message file)

The health of the signal components (bits 18 to 22 of word three in subframe one) are included

from version 2.10 on using the health value reported in the second field of the sixth navigation

message record.

A program reading RINEX files could easily decide if bit 17 only or all bits (17-22) have been

written:

RINEX Value: 0 Health OK

RINEX Value: 1 Health not OK (bits 18-22 not stored)

RINEX Value: >32 Health not OK (bits 18-22 stored)

Table 19: Description of GPS Satellite Health Field

6.13 Transmission time of message (GPS navigation message file)

The transmission time of a message can be shortly before midnight Saturday/Sunday, with the

ToE and ToC of the message already in the next week.

As the reported week in the RINEX nav message (BROADCAST ORBIT -5 record) goes with

ToE (this is different from the GPS week in the original satellite message!), the transmission

time of message should be reduced by 604800 (i.e., will become negative) to also refer to the

same week.

6.14 Antenna references, phase centers

We distinguish between

 The marker, i.e. the geodetic reference monument, on which an antenna is mounted

directly with forced centering or on a tripod

 The antenna reference point (ARP), i.e., a well-defined point on the antenna, e.g., the

center of the bottom surface of the preamplifier. The antenna height is measured from the

marker to the ARP and reported in the ANTENNA: DELTA H/E/N header record.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 34

RINEX Version 3.03 29

Small horizontal eccentricities of the ARP w/r to the marker can be reported in the same

record. On vehicles the position of the ARP is reported in the body-fixed coordinate

system in an ANTENNA: DELTA X/Y/Z header record.

 The average phase center: A frequency-dependent and minimum elevation-angle-

dependent position of the average phase center above the antenna reference point. Its

position is important to know in mixed-antenna networks. It can be given in an absolute

sense or relative to a reference antenna using the optional header record: ANTENNA:

PHASECENTER. For fixed stations the components are in north/east/up direction, on

vehicles the position is reported in the body-fixed system X,Y,Z. For more precise

applications an elevation-dependent or elevation and azimuth-dependent phase center

variation (PCV) model for the antenna (referring to the agreed-upon ARP) should be

used. For special applications it might be useful to generate RINEX files with these

corrections already applied. This can be reported by special header records SYS /

PCVS APPLIED pointing to the file containing the PCV correction models.

 The orientation of the antenna: The “zero direction” is usually oriented towards north on

fixed stations. Deviations from the north direction can be reported with the azimuth of the

zero-direction in an ANTENNA: ZERODIR AZI header record. On vehicles the zero-

direction is reported as a unit vector in the body-fixed coordinate system in an

ANTENNA: ZERODIR XYZ header record. The zero direction of a tilted antenna on a

fixed station can be reported as unit vector in the left-handed north/east/up local

coordinate system in an ANTENNA: ZERODIR XYZ header record.

 The boresight direction of an antenna on a vehicle: The “vertical” symmetry axis of the

antenna pointing towards the GNSS satellites. It can be reported as unit vector in the

body-fixed coordinate system in the ANTENNA: B.SIGHT XYZ record. A tilted

antenna on a fixed station could be reported as unit vector in the left-handed north/east/up

local coordinate system in the same type of header record.

To be able to interpret the various positions correctly it is important that the MARKER TYPE

record is included in the RINEX header.

7. RINEX UNDER ANTISPOOFING (AS)

Some receivers generate code (pseudorange) delay differences between the first and second

frequency using cross-correlation techniques when AS is on and may recover the phase

observations on L2 in full cycles. Using the C/A code delay on L1 and the observed difference it

is possible to generate a code delay observation for the second frequency. Other receivers

recover P code observations by breaking down the Y code into P and W code.

Most of these observations may suffer from an increased noise level. To enable post-processing

programs to take special actions, AS-infected observations have been flagged in RINEX Version

2 using bit number 2 of the Loss of Lock Indicators (i.e. their current values are increased by 4).

In Version 3 there are special attributes for the observation type to more precisely characterize

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 35

RINEX Version 3.03 30

the observable (codeless, semi-codeless, Z-tracking or similar techniques when AS on, L2C, P-

code when AS off, Y-code tracking), making the AS flag obsolete.

8. DEALING WITH DIFFERENT SATELLITE SYSTEMS

8.1 Time system identifier

GPS time runs, apart from small differences (<< 1 microsecond), parallel to UTC. But it is a

continuous time scale, i.e. it does not insert any leap seconds. GPS time is usually expressed in

GPS weeks and GPS seconds past 00:00:00 (midnight) Saturday/Sunday. GPS time started with

week zero at 00:00:00 UT (midnight) on January 6, 1980. Between 1980 and 2012 16 leap

seconds have been introduced into UTC.

The GPS week is transmitted by the satellites as a 10 bit number. It has a roll-over after week

1023. The first roll-over happened on August 22, 1999, 00:00:00 GPS time.

In order to avoid ambiguities, the GPS week reported in the RINEX navigation message files is a

continuous number without roll-over, i.e. …1023, 1024, 1025, …

We use GPS as time system identifier for the reported GPS time.

QZSS runs on QZSS time, which conforms to UTC Japan Standard Time Group (JSTG) time

and the offset with respect to GPS time is controlled. The following properties apply to the

QZSS time definition: the length of one second is defined with respect to International Atomic

Time (TAI); QZSS time is aligned with GPS time (offset from TAI by integer seconds); the

QZSS week number is defined with respect to the GPS week.

We use QZS as a time system identifier for the reported QZSS time

GLONASS is basically running on UTC (or, more precisely, GLONASS system time linked to

UTC(SU)), i.e. the time tags are given in UTC and not GPS time. It is not a continuous time, i.e.

it introduces the same leap seconds as UTC. The reported GLONASS time has the same hours as

UTC and not UTC+3 h as the original GLONASS System Time!

We use GLO as time system identifier for the reported GLONASS time.

Galileo runs on Galileo System Time (GST), which is, apart from small differences (tens of

nanoseconds), nearly identical to GPS time:

 The Galileo week starts at midnight Saturday/Sunday at the same second as the GPS

week

 The GST week as transmitted by the satellites is a 12 bit value with a roll-over after week

4095. The GST week started at zero at the first roll-over of the broadcast GPS week after

1023, i.e. at Sun, 22-Aug-1999 00:00:00 GPS time.

In order to remove possible misunderstandings and ambiguities, the Galileo week reported in the

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 36

RINEX Version 3.03 31

RINEX navigation message files is a continuous number without roll-over,

i.e., …4095,4096,4097,… and it is aligned to the GPS week.

We use GAL as time system identifier for this reported Galileo time.

The BDS Time (BDT) System is a continuous timekeeping system, with its length of second

being an SI second. BDT zero time started at 00:00:00 UTC on January 1st, 2006 (GPS week

1356) therefore BDT is 14 seconds behind GPS time. BDT is synchronized with UTC within 100

nanoseconds (modulo 1 second).

 The BDT week starts at midnight Saturday/Sunday

 The BDT week is transmitted by the satellites as a 13 bit number. It has a roll-over after

week 8191. In order to avoid ambiguities, the BDT week reported in the RINEX

navigation message files is a continuous number without roll-over, i.e. …8191, 8192,

8193, …

We use BDT as time system identifier for the reported BDS time.

IRNSS runs on Indian Regional Navigation Satellite System Time (IRNSST). The IRNSST

start epoch is 00:00:00 on Sunday August 22nd, 1999, which corresponds to August 21st, 1999,

23:59:47 UTC (same time as the first GPS week roll over). Seconds of week are counted from

00:00:00 IRNSST hours Saturday/Sunday midnight which also corresponds to the start of the

GPS week. Week numbers are consecutive from the start time and will roll over after week 1023

(at the same time as GPS and QZSS roll over).

We use IRN as the time system identifier for the reported IRNSS time.

Constellation GPS GPS GPS GPS GPS GPS

/Archival Time Ephemeris Ephemeris Ephemeris Ephemeris Ephemeris Ephemeris

Representation Week Week Week Week Week Week

Period #1 Period #2 Period #3 Period #4 Period #5 Period #6

GPS Broadcast 0 – 1023 0 – 1023 0 – 1023 0 – 1023 0 – 1023 0 – 1023

QZSS Broadcast 0 – 1023 0 – 1023 0 – 1023 0 – 1023 0 – 1023

IRNSS 0 – 1023 0 – 1023 0 – 1023 0 – 1023 0 – 1023

Broadcast

GST Broadcast 0 – 1023 1024 – 2047 2048 – 3071 3072 – 4095 0 – 1023

BDS Broadcast 0(RINEX 692 – 1715 1716 – 2739 2740 – 3763 3764 – 4787

and RINEX Week 1356)

– 691

GPS/QZS/IRN/ 0 – 1023 1024 – 2047 2048 – 3071 3072 – 4095 4096 – 5119 5120 -6143

GAL RINEX

Table 20: Relationship between GPS, QZSS, IRN, GST, GAL, BDS and RINEX Week Numbers

The header records TIME OF FIRST OBS and (if present) TIME OF LAST OBS in pure

GPS, GLONASS, Galileo, QZSS or BDS observation files can (in mixed

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 37

RINEX Version 3.03 32

GPS/GLONASS/Galileo/QZSS/BDS/IRNSS observation files must) contain the time system

identifier defining the system that all time tags in the file are referring to:

• GPS to identify GPS time

• GLO to identify the GLONASS UTC time system

• GAL to identify Galileo time

• QZS to identify QZSS time

• BDT to identify BDS time

• IRN to identify IRNSS time

Pure GPS observation files default to GPS, pure GLONASS files default to GLO, pure Galileo

files default to GAL and similarly pure BDS observation files default to BDT (same for QZSS

and IRNSS).

Apart from the small errors in the realizations of the different time systems, the relations between

the systems are:

GLO = UTC = GPS - ΔtLS

GPS = GAL = UTC + ΔtLS

GPS = QZS = UTC + ΔtLS

GPS = IRN = UTC + ΔtLS

BDT = UTC + ΔtLS

BDS

Table 21: Constellation Time Relationships

Where:

Delta time between GPS and UTC due to leap seconds,

as transmitted by the GPS satellites in the almanac

ΔtLS =

(2005: ΔtLS = 13, 2006: ΔtLS = 14, 2008: ΔtLS = 15

and 2012: ΔtLS = 16).

Delta time between BDT and UTC due to leap seconds,

as transmitted by the BDS satellites in the almanac

ΔtLS =

BDS

(2006: ΔtLS = 0, 2008: ΔtLS = 1 and 2012:

BDS BDS

ΔtLS = 2). See BDS-SIS-ICD-2.0 Section 5.2.4.17

BDS

Table 22: GPS and BeiDou UTC Leap Second Relationship

In order to have the current number of leap seconds available, we recommend including ΔtLS by

adding a LEAP SECOND line into the RINEX file header.

If there are known non-integer biases between “GPS receiver clock”, “GLONASS receiver

clock”, “BDS receiver clock” or “Galileo receiver clock” in the same receiver, they should be

applied in the process of RINEX conversion. In this case, the respective code and phase

observations have to be corrected too (c * bias if expressed in meters).

Unknown biases will have to be solved for during the post processing.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 38

RINEX Version 3.03 33

The small differences (modulo 1 second) between: BDS system time, Galileo system time,

GLONASS system time, UTC(SU), UTC(USNO) and GPS system time have to be dealt with

during the post-processing and not before the RINEX conversion. It may also be necessary to

solve for remaining differences during the post-processing.

8.2 Pseudorange definition

The pseudorange (code) measurement is defined to be equivalent to the difference of the time of

reception (expressed in the time frame of the receiver) and the time of transmission (expressed in

the time frame of the satellite) of a distinct satellite signal.

In a mixed-mode GPS/GLONASS/Galileo/QZSS/BDS receiver referring all pseudorange

observations to one receiver clock only,

 the raw GLONASS pseudoranges will show the current number of leap seconds between

GPS/GAL/BDT time and GLONASS time if the receiver clock is running in the GPS,

GAL or BDT time frame

 the raw GPS, Galileo and BDS pseudoranges will show the negative number of leap

seconds between GPS/GAL/BDT time and GLONASS time if the receiver clock is

running in the GLONASS time frame

In order to avoid misunderstandings and to keep the code observations within the format fields,

the pseudo-ranges must be corrected in this case as follows:

PR_mod(GPS) = PR(GPS) + C* ΔtLS if generated with a receiver clock

running in the GLONASS time frame

PR_mod(GAL) = PR(GAL) + C* ΔtLS if generated with a receiver clock

running in the GLONASS time frame

PR_mod(BDT) = PR(BDT) + C* ΔtLS if generated with a receiver clock

BDS

running in the GLONASS time frame

PR_mod(GLO) = PR(GLO) - C* ΔtLS if generated with a receiver clock

running in the GPS or GAL time frame

PR_mod(GLO) = PR(GLO) - C*ΔtLS if generated with a receiver clock

BDS

running in the BDT time frame

PR_mod(GPS) = PR(GPS) + C*(ΔtLS- if generated with a receiver clock

ΔtLS ) running in the BDT time frame

BDS

Table 23: Constellation Pseudorange Corrections

to remove the contributions of the leap seconds from the pseudoranges.

ΔtLS is the actual number of leap seconds between GPS/GAL and GLO time, as broadcast in the

GPS almanac and distributed in Circular T of BIPM.

ΔtLS is the actual number of leap seconds between BDT and UTC time, as broadcast in the

BDS

BDT almanac.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 39

RINEX Version 3.03 34

8.3 RINEX navigation message files

The header section of the RINEX version 3.XX navigation message files have been slightly

changed compared to the previous version 2. The format of the header section is identical for all

satellite systems: GPS, GLONASS, Galileo, SBAS, QZSS and IRNSS. One exception is that

BDS “IONOSPHERIC CORR” message has a few extra fields (See: Appendix Table A5 ).

The data portion of the navigation message files contains records with floating point numbers.

The format is identical for all satellite systems; the number of records per message and the

contents, however, are satellite system-dependent. The format of the version 3 data records has

been changed slightly; the satellite codes now also contain the satellite system identifier.

It is possible to generate mixed navigation message files, i.e. files containing navigation

messages of more than one satellite system. Header records with system-dependent contents

have to be repeated for each satellite system, if applicable. Using the satellite system identifier

of the satellite code the reading program can determine the number of data records to be read

for each message block.

The time tags of the navigation messages (e.g., time of ephemeris, time of clock) are given in

the respective satellite time systems!

It is recommended to avoid storing redundant navigation messages (e.g., the same message

broadcast at different times) in the RINEX file.

8.3.1 RINEX navigation message files for GLONASS

The header section and the first data record (epoch, satellite clock information) are equal to the

GPS navigation file. The following three records contain the satellite position, velocity and

acceleration, the clock and frequency biases, as well as auxiliary information such as health,

satellite frequency (channel) and age of the information.

The corrections of the satellite time to UTC are as follows:

GPS: Tutc = Tsv –af0 –af1 *(Tsv-Toc) -... –A0 -... –ΔtLS

GLONASS: Tutc = Tsv + TauN –GammaN*(Tsv-Tb) + TauC

In order to use the same sign conventions for the GLONASS cor-

rections as in the GPS navigation files, the broadcast GLONASS

values are stored as:

-TauN, +GammaN, -TauC.

Table 24: GLONASS Navigation File Data, Sign Convention

The time tags in the GLONASS navigation files are given in UTC (i.e. not Moscow time or

GPS time).

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 40

RINEX Version 3.03 35

8.3.2 RINEX navigation message files for Galileo

The Galileo Open Service allows access to two navigation message types: F/NAV (Freely

Accessible Navigation) and I/NAV (Integrity Navigation). The content of the two messages

differs in various items, however, in general it is very similar to the content of the GPS

navigation message, e.g. the orbit parameterization is the same.

There are items in the navigation message that depend on the origin of the message (F/NAV or

I/NAV): The SV clock parameters actually define the satellite clock for the dual-frequency

ionosphere-free linear combination. F/NAV reports the clock parameters valid for the E5a-E1

combination, the I/NAV reports the parameters for the E5b-E1 combination. The second

parameter in the Broadcast Orbit 5 record (bits 8 and 9) indicate the frequency pair the

stored clock corrections are valid for.

Some parameters contain the information stored bitwise. The interpretation is as follows:

 Convert the floating point number read from the RINEX file into the nearest integer

 Extract the values of the requested bits from the integer

Example:

0.170000000000D+02 -> 17 = 24+20 -> Bits 4 and 0 are set, all others are zero

RINEX file encoders should encode one RINEX Galileo navigation message for each I/NAV and

F/NAV signal decoded. Therefore if both: I/Nav and F/Nav messages are decode, then the

relevant bit fields must be set in the RINEX message and both should be written in separate

messages. The Galileo ICD (2010) Section 5.1.9.2 indicates that some of the contents of the

broadcast navigation message may change, yet the issue of data (IOD) may not change. So that

all relevant information is available message encoders should monitor the contents of the file and

write new navigation messages when the contents have changed.

RINEX file parsers should expect to encounter F/NAV and I/NAV messages with the same IOD

in the same file. Additionally, parsers should also expect to encounter more than one F/NAV or

I/NAV ephemeris message with the same IOD, as the navigation message Data Validity Status

(DVS) and other parameters may change independently of the IOD, yet some other data may be

the same, however, the transmission time will be updated (See Note in Galileo ICD (2010)

Section 5.1.9.2 Issue of Data).

As mentioned above, the GAL week in the RINEX navigation message files is a continuous

number; it has been aligned to the GPS week by the program creating the RINEX file.

8.3.3 RINEX navigation message files for GEO satellites

As the GEO broadcast orbit format differs from the GPS message, a special GEO navigation

message file format has been defined which is nearly identical with the GLONASS navigation

message file format.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 41

RINEX Version 3.03 36

The header section contains information about the generating program, comments, and the

difference between the GEO system time and UTC.

The first data record contains the epoch and satellite clock information; the following records

contain the satellite position, velocity and acceleration and auxiliary information (health, URA

and IODN).

The time tags in the GEO navigation files are given in the GPS time frame, i.e. not UTC.

The corrections of the satellite time to UTC are as follows:

GEO: Tutc = Tsv –aGf0 –aGf1 *(Tsv-Toe) –W0 –ΔtLS

W0 being the correction to transform the GEO system time to UTC. See Toe, aGf0, aGf1 in the

Appendix A16 format definition table.

The Transmission Time of Message (PRN / EPOCH / SV CLK header record) is expressed

in GPS seconds of the week. It marks the beginning of the message transmission. It has to refer

to the same GPS week as the Epoch of Ephemerides. It has to be adjusted by –or + 604800

seconds, if necessary (which would make it lower than zero or larger than 604800, respectively).

It is a redefinition of the Version 2.10 Message frame time.

Health shall be defined as follows:

 bits 0 to 3 equal to health in Message Type 17 (MT17)

 bit 4 is set to 1 if MT17 health is unavailable

 bit 5 is set to 1 if the URA index is equal to 15

8.3.4 RINEX navigation message files for QZSS L1-SAIF

As the QZSS L1-SAIF broadcast orbit format differs from the GPS message, a special L1-SAIF

navigation message file format has been defined which is nearly identical with the GEO

navigation message file format (See A16).

The header section contains information about the generating program, comments, and the

difference between the L1-SAIF system time and UTC.

The first data record contains the epoch and satellite clock information, the following records

contain the satellite position, velocity and acceleration and auxiliary information such as health,

age of the data, etc. To compute L1-SAIF satellite position, note that acceleration in navigation

message represents only perturbation term and it is necessary to add :

The time tags in the L1-SAIF navigation files are given in the GPS time frame, i.e. not UTC.

The corrections of the satellite time to UTC are as follows:

SAIF: Tutc = Tsv –aGf0 –aGf1 *(Tsv-Toe) –W0 –ΔtLS

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 42

RINEX Version 3.03 37

W0 being the correction to transform the L1-SAIF system time to UTC. Toe, aGf0, aGf1 see

below in the format definition tables.

The Transmission Time of Message (PRN / EPOCH / SV CLK header record) is expressed

in GPS seconds of the week. It marks the beginning of the message transmission. It has to refer

to the same GPS week as the Epoch of Ephemerides. It has to be adjusted by –or + 604800

seconds, if necessary (which would make it lower than zero or larger than 604800, respectively).

Health shall be defined as follows:

 bits 0 to 3 equal to health in Message Type 17 (MT17)

 bit 4 is set to 1 if MT17 health is unavailable

 bit 5 is set to 1 if the URA index is equal to 15

Note that accelerations represent only lunar and solar perturbation terms and satellite position

can be computed based on equations in Section A.3.1.2 of GLONASS ICD version 5.0. See

Appendix A16

8.3.5 RINEX navigation message files for BDS

The BDS Open Service broadcast navigation message is similar in content to the GPS navigation

message.

The header section and the first data record (epoch, satellite clock information) are equal to the

GPS navigation file. The following six records are similar to GPS.

The BDT week number is a continuous number. The broadcast 13-bit BDS System Time week

has a roll-over after 8191. It starts at zero on: 1-Jan-2006, hence BDT week = BDT week_BRD

+ (n*8192) (Where n: number of BDT roll-overs). See Appendix Table A14 for details.

8.3.6 RINEX navigation message files for IRNSS

The IRNSS Open Service broadcast navigation message is similar in content to the GPS

navigation message.

The header section and the first data record (epoch, satellite clock information) are equal to the

GPS navigation file. See Appendix Tables A18 and A19 for a description and example of each

field.

8.4 RINEX observation files for GEO satellites

A separate satellite system identifier has been defined for the Satellite-Based Augmentation

System (SBAS) payloads. S, is to be used in the RINEX VERSION / TYPE header line and

in the satellite identifier snn, nn being the GEO PRN number minus 100.

e.g.: PRN = 120 ⇒snn = S20

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 43

RINEX Version 3.03 38

In mixed dual frequency GPS satellite / single frequency GEO payload observation files, the

fields for the second frequency observations of SBAS satellites remain blank, are set to zero

values or (if last in the record) can be truncated.

The time system identifier of GEO satellites generating GPS signals defaults to GPS time. In the

SBAS message definitions, bit 3 of the health word is currently marked as reserved. In case of

bit 4 set to 1, it is recommended to set bits 0,1,2,3 to 1, as well.

User Range Accuracy (URA):

The same convention for converting the URA index to meters is used as with GPS. Set URA =

32767 meters if URA index = 15.

Issue Of Data Navigation (IODN)

The IODN is defined as the 8 first bits after the message type 9, called IODN in RTCA DO229,

Annex A and Annex B and called spare in Annex C.

The D-UTC A0, A1, T, W, S, U record in Version 2.11 has been renamed the TIME SYSTEM

CORR record in RINEX 3.x.

9. MODIFICATIONS FOR VERSION 3.01, 3.02 and 3.03

9.1 Phase Cycle Shifts

Carrier phases tracked on different signal channels or modulation bands of the same frequency

may differ in phase by 1/4 (e.g., GPS: P/Y-code-derived L2 phase vs. L2C-based phase) or, in

some exceptional cases, by other fractional parts of a cycle. Appendix Table23 specifies the

reference signal and the phase shifts that are specified by the Interface Control Documentation

(ICD) for each constellation.

All phase observations must be aligned in RINEX 3.01 and later files and the new SYS /

PHASE SHIFT header is mandatory. See Appendix Table A2 for the messages definition. If

the phase alignment is not known, then the observation data should not be published in a

RINEX 3.0x file. In order to facilitate data processing, phase observations stored in RINEX files

must be consistent across all satellites of a satellite system and across each frequency band.

Within a RINEX 3.0x file:

 All phase observations must be aligned to the designated constellation and frequency

reference signal as specified in Appendix Table A23, either directly by the receiver or by

a correction program or the RINEX conversion program, prior to RINEX file generation.

Additionally, all data must be aligned with the appropriate reference signal indicated in

Appendix Table A23 even when the receiver or reporting device is not tracking and/or

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 44

RINEX Version 3.03 39

providing data from that reference signal e.g. Galileo L5X phase data must be aligned to

L5I.

 Phase corrections must be reported in a new mandatory SYS / PHASE SHIFT header

record to allow the reconstruction of the original values, if needed. The uncorrected

reference signal group of observations are left blank in the SYS / PHASE SHIFT

records. Appendix Table A23 specifies the reference signal that should be used by each

constellation and frequency band. Additionally, Appendix Table A23 indicates the

relationship between the phase observations for each frequency’s signals.

Concerning the mandatory SYS / PHASE SHIFT header records:

 If the SYS / PHASE SHIFT record values are set to zero in the RINEX file, then

either the raw data provided by the receiver or the data format (RTCM-Multiple Signal

Messages format for example) have already been aligned and the RINEX conversion

program did not apply any phase corrections since they had already been applied. In this

case Appendix Table A23 can be used to determine the fractional cycles that had been

added to each signal’s phase observation to align the phase observations to the reference

signal.

 If the file does not contain any observation pairs affected by phase shifts (i.e. only

reference signals reported), the observation code field is defined and the rest of the SYS

/ PHASE SHIFT header record field of the respective satellite system(s) are left

blank.

 If the reported phase correction of an observation type does not affect all satellites of the

same system, the header record allows for the affected satellites to be indicated.

 If the applied phase corrections or the phase alignment is unknown, the observation code

field and the rest of the SYS / PHASE SHIFT header record field of the respective

satellite system(s) are left blank. This use case is intended for exceptional situations

where the data is intended for special projects and analysis.

Sign of the correction Δφ:

φRINEX = φ original + Δφ

φ original : Uncorrected or corrected, i.e. as issued by the GNSS receiver or

in a standardized data stream such as RTCM-MSM

Δφ : Phase correction to align the phase to other phases of

the same frequency but different channel / modulation

band

Table 25: RINEX Phase Alignment Correction Convention

Example (Definition see Appendix Table A2):

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

G L2S -0.25000 03 G15 G16 G17 SYS / PHASE SHIFT

Table 26: Example SYS / PHASE SHIFT Record

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 45

RINEX Version 3.03 40

9.2 Galileo: BOC-Tracking of an MBOC-Modulated Signal

Galileo E1 will be modulated by the so-called MBOC modulation. Obviously it is possible for a

receiver to track the signal also in a BOC mode, though leading to different noise characteristics.

In order to keep this non-standard tracking mode of a MBOC signal apart, bit 2 of the loss-of-

lock indicator LLI (the antispoofing flag not used for Galileo) in the observation data records is

used.

Non-standard BOC tracking of an MBOC-modulated signal: Increase the LLI by 4.

Note: This flag is intended for experimental applications and is optional. In future releases of

RINEX this non-standard tracking mode flag may be removed.

Example: Satellite E11, BOC tracking on L1C, LLI = 4:

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

G 5 C1C L1W L2W C1W S2W SYS / # / OBS TYPES

R 2 C1C L1C SYS / # / OBS TYPES

E 2 L1C L5I SYS / # / OBS TYPES

S 2 C1C L1C SYS / # / OBS TYPES

18.000 INTERVAL

END OF HEADER

> 2006 03 24 13 10 36.0000000 0 5 -0.123456789012

G06 23629347.915 .300 8 -.353 4 23629347.158 24.158

G09 20891534.648 -.120 9 -.358 6 20891545.292 38.123

G12 20607600.189 -.430 9 .394 5 20607600.848 35.234

E11 .32448 .178 7

S20 38137559.506 335849.135 9

Table 27: Example of RINEX Coding of Galileo BOC Tracking of an MBOC Signal Record

9.3 BDS Satellite System Code

The satellite system code for BeiDou navigation satellite System (BDS) has been defined as “C”,

see Figure 1.

9.4 New Observation Codes for GPS L1C and BDS

New observation codes for GPS L1C and BDS observables have been defined: See Tables 4 and

9.

9.5 Header Records for GLONASS Slot and Frequency Numbers

In order to make available a cross-reference list between the GLONASS slot numbers used in the

RINEX files to designate the GLONASS satellites and the allotted frequency numbers, a

mandatory observation file header record is assigned. This allows processing of GLONASS files

without having to get this information from GLONASS navigation message files or other

sources.

Example (Definition See Appendix Table A2):

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

18 R01 1 R02 2 R03 3 R04 4 R05 5 R06 -6 R07 -5 R08 -4 GLONASS SLOT / FRQ #

R09 -3 R10 -2 R11 -1 R12 0 R13 1 R14 2 R15 3 R16 4 GLONASS SLOT / FRQ #

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 46

RINEX Version 3.03 41

R17 5 R18 -5 GLONASS SLOT / FRQ #

Table 28: Example of a GLONASS Slot- Frequency Records

9.6 GNSS Navigation Message File: Leap Seconds Record

The optional LEAP SECONDS record was modified to also include ΔtLS (or ΔtLS for

BDS

BDS), WN (adjusted to continuous week number) and DN.

LSF

9.7 Clarifications in the Galileo Navigation Message File:

Some clarifications in the Galileo BROADCAST ORBIT – 5 and BROADCAST ORBIT – 6

records were added (see Table A8).

9.8 Quasi-Zenith Satellite System (QZSS) Version 3.02

The version number is adjusted to 3.02. Version 3.02 added QZSS: specifications, parameters

and definitions to the document. Each QZSS satellite broadcasts signals using two PRN codes.

The GPS compatible signals are broadcast using PRN codes in the range of 193-197. In a

RINEX observation file the PRN code is: broadcast prn - 192, yielding: J01, J02 etc.. QZSS

satellites also broadcast a SBAS signal (QZSS-SAIF) using PRN codes in the range of 183-187.

In a RINEX SBAS file the PRN code is: broadcast prn - 100, yielding: S83, S84 etc..

See Appendix Table A23 to convert each signal’s aligned phase observations back to raw

satellite phase.

9.9 GLONASS Mandatory Code-Phase Alignment Header Record

Recent analysis has revealed that some GNSS receivers produce biased GLONASS observations.

The code-phase bias results in the code and phase observations not being measured at the same

time. To remedy this problem, a mandatory GLONASS Code-Phase header bias record is

required. Although this header message is mandatory, it can contain zeros if the GLONASS data

issued by the receiver is aligned. See the GLONASS CODE/PHASE BIAS (GLONASS

COD/PHS/BIS) definition in Appendix Table A2. The GLONASS code-phase alignment

message contains: L1C, L1P, L2C and L2P corrections. Phase data from GNSS receivers that

issue biased data has to be corrected by the amount specified in the GLONASS COD/PHS/BIS

record before it is written in RINEX format.

To align the non-aligned L1C phase to the pseudo range observation, the following correction is

required:

AlignedL1Cphase = ObservedL1Cphase + (GLONASSC1C_CodePhaseBias_M / Lambda)

where:

 AlignedL1C phase in cycles (written to RINEX file)

 ObservedL1C phase in cycles

 GLONASSC1C_CodePhaseBias_M is in metres

 Lamba is the wavelength for the particular GLONASS frequency

GLONASS L1P, L2C and L2P are handled in the same manner.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 47

RINEX Version 3.03 42

Example (See Appendix Table A2 for details) :

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

C1C -10.000 C1P -10.123 C2C -10.432 C2P -10.634 GLONASS COD/PHS/BIS#

Table 29: Example of GLONASS Code Phase Bias Correction Record

Note: If the GLONASS code phase alignment is unknown, then all fields within GLONASS

COD/PHS/BIS header record are left blank (see example below). This use case is intended for

exceptional situations where the data is intended for special projects and analysis.

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

GLONASS COD/PHS/BIS#

Table 30: Example of Unknown GLONASS Code Phase Bias Record

9.10 BDS system (Replaces Compass)

Added BDS: naming convention, time system definition, header section description, and

parameters throughout the document. Updated: Sections: 8.1, 8.2, 8.3.5, 9.11 and Appendix

Table A2, added ephemeris Table A14 and updated Table A23.

9.11 Indian Regional Navigation Satellite System (IRNSS) Version 3.03

The RINEX version number was changed to 3.03. Version 3.03 adds IRNSS, specifications,

parameters and definitions to this document.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 48

RINEX Version 3.03 43

10 References

Evans, A. (1989): “Summary of the Workshop on GPS Exchange Formats.” Proceedings of the

Fifth International Geodetic Symposium on Satellite Systems, pp. 917ff, Las Cruces.

Gurtner, W., G. Mader, D. Arthur (1989): “A Common Exchange Format for GPS Data.” CSTG

GPS Bulletin Vol.2 No.3, May/June 1989, National Geodetic Survey, Rockville.

Gurtner, W., G. Mader (1990a): “The RINEX Format: Current Status, Future Developments.”

Proceedings of the Second International Symposium of Precise Positioning with the Global

Positioning system, pp. 977ff, Ottawa.

Gurtner, W., G. Mader (1990b): “Receiver Independent Exchange Format Version 2.” CSTG

GPS Bulletin Vol.3 No.3, Sept/Oct 1990, National Geodetic Survey, Rockville.

Gurtner, W. (1994): “RINEX: The Receiver-Independent Exchange Format.” GPS World,

Volume 5, Number 7, July 1994.

Gurtner, W. (2002): “RINEX: The Receiver Independent Exchange Format Version 2.10”.

ftp://igscb.jpl.nasa.gov/igscb/data/format/rinex210.txt

Gurtner, W., L. Estey (2002),: “RINEX Version 2.20 Modifications to Accommodate Low Earth

Orbiter Data”. ftp://ftp.unibe.ch/aiub/rinex/rnx_leo.txt

Gurtner, W., L. Estey (2005): “RINEX: The Receiver Independent Exchange Format Version

2.11”. ftp://igscb.jpl.nasa.gov/igscb/data/format/rinex211.txt

Gurtner, W., L. Estey (2007): “RINEX: The Receiver Independent Exchange Format Version

3.00”. ftp://igscb.jpl.nasa.gov/igscb/data/format/rinex300.pdf

Hatanaka, Y (2008): “ A Compression Format and Tools for GNSS Observation Data”. Bulletin

of the Geographical Survey Institute, Vol. 55, pp 21-30, Tsukuba, March 2008.

http://www.gsi.go.jp/ENGLISH/Bulletin55.html

Ray, J., W. Gurtner (2010): “RINEX Extensions to Handle Clock Information”.

ftp://igscb.jpl.nasa.gov/igscb/data/format/rinex_clock302.txt.

Ray, J. (2005): “Final update for P1-C1 bias values & cc2noncc”. IGSMail 5260

Rothacher, M., R. Schmid (2010): “ANTEX: The Antenna Exchange Format Version 1.4”.

ftp://igscb.jpl.nasa.gov/pub/station/general/antex14.txt.

Schaer, S., W. Gurtner, J. Feltens (1998): “IONEX: The Ionosphere Map Exchange Format

Version 1“. ftp://igscb.jpl.nasa.gov/igscb/data/format/ionex1.pdf

Suard, N., W. Gurtner, L. Estey (2004): “Proposal for a new RINEX-type Exchange File for

GEO SBAS Broadcast Data”. ftp://igscb.jpl.nasa.gov/igscb/data/format/geo_sbas.txt

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 49

RINEX Version 3.03 44

Document: RTCA DO 229, Appendix A

Document: Global Positioning Systems Directorate, Systems Engineering and Integration

Interface Specification IS-GPS-200H, Navstar GPS Space Segment/Navigation User Interfaces,

Sept. 24, 2013

Document: GLObal NAvigation Satellite System (GLONASS), Interface Control Document,

(Edition 5.1), 2008.

Document: European GNSS (Galileo) Open Service, Signal In Space, Interface Control

Document, Issue 1.1, September, 2010.

Document: Quasi-Zenith Satellite System, Navigation Service, Interface Specification for QZSS

(IS-QZSS), V1.6, Japan Aerospace Exploration Agency, November 28, 2014

Document: BeiDou Navigation Satellite, System, Signal In Space, Interface Control Document,

Open Service Signal, (Version2.0), China Satellite Navigation Office December 2013

Document: RTCM Standard 10403.2, Differential GNSS (Global Navigation Satellite Systems)

Services – Version 3, November 7, 2013.

Document: Indian Regional Navigation Satellite System Signal in Space ICD for Standard

Positioning Service, Version 1.0, June 2014 (Indian Space Research Organization, Bangalore,

2014)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 50

RINEX Version 3.03 Appendix A1

APPENDIX: RINEX FORMAT DEFINITIONS AND EXAMPLES

A 1 RINEX File name description

Table A1

RINEX File name description

Field Field Description Example Required Comment/Example

<SITE/ XXXXMRCCC ALGO00CAN Yes File name supports a maximum of

STATION- Where: 10 monuments at the same station

MONUMENT/ XXXX - existing and a maximum of 10 receivers

RECEIVER/ IGS station name per monument.

COUNTRY/ M – monument or

marker number (0-9) Country codes follow : ISO 3166-

R – receiver number 1 alpha-3

(0-9)

CCC – ISO Country

code

(Total 9 characters)

<DATA SOURCE> Data Source R Yes This field is used to indicate how

R – From Receiver the data was collected either from

data using vendor or the receiver at the station or from

other software a data stream

S – From data Stream

(RTCM or other)

U – Unknown

(1 character)

<START TIME> YYYYDDDHHMM 2012150 Yes For GPS files use : GPS Year,

YYYY – Gregorian 1200 day of year, hour of day, minute

year 4 digits, of day (see text below for details)

DDD – day of Year, Start time should be the nominal

HHMM – hours and start time of the first observation.

minutes of day GLONASS, Galileo, BeiDou etc

use respective time system.

(11 characters)

<FILE PERIOD> DDU 15M Yes File Period

DD – file period 15M–15 Minutes

U – units of file 01H–1 Hour

period. 01D–1 Day

File period is used to 01Y–1 Year

specify intended 00U-Unspecified

collection period of

the file.

(3 characters)

<DATA FREQ> DDU 05Z Mandator XXC – 100 Hertz

y for XXZ – HertZ,

RINEX3.03.IGS.RTCM.doc 2015-04-14

## Page 51

RINEX Version 3.03 Appendix A2

Table A1

RINEX File name description

Field Field Description Example Required Comment/Example

DD – data frequency RINEX XXS – Seconds,

U – units of data rate Obs. Data. XXM – Minutes,

(3 characters) NOT XXH – Hours,

required XXD – Days

for XXU – Unspecified

Navigatio

n Files.

<DATA TYPE > DD MO Yes Two characters represent the data

DD – Data type type:

GO - GPS Obs.,

(2 characters) RO - GLONASS Obs.,

EO - Galileo Obs.

JO - QZSS Obs.

CO - BDS Obs.

IO – IRNSS Obs.

SO - SBAS Obs.

MO Mixed Obs.

GN - Nav. GPS,

RN- Glonass Nav.,

EN- Galileo Nav.,

JN- QZSS Nav.,

CN- BDS Nav.

IN – IRNSS Nav.

SN- SBAS Nav.

MN- Nav. All GNSS

Constellations)

MM-Meteorological Observation

Etc

<FORMAT> FFF rnx Yes Three character indicating the

FFF – File format data format :

RINEX - rnx,

(3 characters) Hatanaka Compressed RINEX –

crx, ETC

<COMPRESSION> (2-3 Characters) gz No gz

Sub Total 34 or 35 Fields

Separators (7 characters –Obs. _ under score between all fields

File) and “.” Between data type and file

(6 characters –Eph. format and the compression

File) method

Total 41-42(Obs. File) Mandatory IGS RINEX obs.

37-38 (Eph. File) Characters

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 52

RINEX Version 3.03 Appendix A3

Filename Details and Examples:

<STATION/PROJECT NAME>: IGS users should follow XXXXMRCCC (9 char) site nd

station naming convention described above.

GNSS industry users could use the 9 characters to indicate the project name and/or number.

<DATA SOURCE>: With real-time data streaming RINEX files for the same station can be

created at many locations. If the RINEX file is derived from data collected at the receiver

(official file) then the source is specified as R. On the other hand if the RINEX file is derived

from a real-time data stream then the data source is marked as S to indicate Streamed data

source. If the data source is unknown the source is marked as U.

<START TIME>: The start time is the file start time which should coincide with the first

observation in the file. GPS file start time is specified in GPS Time. Mixed observation file start

times are defined in the same time system as the file observation time system specified in the

header. Files containing only: GLONASS, Galileo, QZSS, BDS or SBAS observations are all

based on their respective time system.

<FILE PERIOD>: Is used to specify the data collection period of the file.

GNSS observation file name - file period examples:

ALGO00CAN_R_20121601000_15M_01S_GO.rnx.gz //15 min, GPS Obs. 1 sec.

ALGO00CAN_R_20121601000_01H_05Z_MO.rnx.gz //1 hour, Obs Mixed and 5Hz

ALGO00CAN_R_20121601000_01D_30S_GO.rnx.gz //1 day, Obs GPS and 30 sec

ALGO00CAN_R_20121601000_01D_30S_MO.rnx.gz //1 day, Obs. Mixed, 30 sec

GNSS navigation file name - file period examples:

ALGO00CAN_R_20121600000_15M_GN.rnx.gz // 15 minute GPS only

ALGO00CAN_R_20121600000_01H_GN.rnx.gz // 1 hour GPS only

ALGO00CAN_R_20121600000_01D_MN.rnx.gz // 1 day mixed

<DATA FREQ>: Used to distinguish between observation files that cover the same period but

contain data at a different sampling rate. GNSS data file - observation frequency examples:

ALGO00CAN_R_20121601000_01D_01C_GO.rnx.gz //100 Hz data rate

ALGO00CAN_R_20121601000_01D_05Z_RO.rnx.gz //5 Hz data rate

ALGO00CAN_R_20121601000_01D_01S_EO.rnx.gz //1 second data rate

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 53

RINEX Version 3.03 Appendix A4

ALGO00CAN_R_20121601000_01D_05M_JO.rnx.gz //5 minute data rate

ALGO00CAN_R_20121601000_01D_01H_CO.rnx.gz //1 hour data rate

ALGO00CAN_R_20121601000_01D_01D_SO.rnx.gz //1 day data rate

ALGO00CAN_R_20121601000_01D_00U_MO.rnx.gz //Unspecified

Note : Data frequency field not required for RINEX Navigation files.

< DATA TYPE/ FORMAT/>: The data type describes the content of the file. The first character

indicates constellation and the second indicates whether the files contains observations or

navigation data. The next three characters indicate the data file format. GNSS observation

filename - format/data type examples:

ALGO00CAN_R_20121601000_15M_01S_GO.rnx.gz //RINEX obs. GPS

ALGO00CAN_R_20121601000_15M_01S_RO.rnx.gz //RINEX obs. GLONASS

ALGO00CAN_R_20121601000_15M_01S_EO.rnx.gz //RINEX obs. Galileo

ALGO00CAN_R_20121601000_15M_01S_JO.rnx.gz //RINEX obs. QZSS

ALGO00CAN_R_20121601000_15M_01S_CO.rnx.gz //RINEX obs. BDS

ALGO00CAN_R_20121601000_15M_01S_SO.rnx.gz //RINEX obs. SBAS

ALGO00CAN_R_20121601000_15M_01S_MO.rnx.gz //RINEX obs. mixed

GNSS navigation filename examples:

ALGO00CAN_R_20121600000_01H_GN.rnx.gz //RINEX nav. GPS

ALGO00CAN_R_20121600000_01H_RN.rnx.gz //RINEX nav. GLONASS

ALGO00CAN_R_20121600000_01H_EN.rnx.gz //RINEX nav. Galileo

ALGO00CAN_R_20121600000_01H_JN.rnx.gz //RINEX nav. QZSS

ALGO00CAN_R_20121600000_01H_CN.rnx.gz //RINEX nav. BDS

ALGO00CAN_R_20121600000_01H_SN.rnx.gz //RINEX nav. SBAS

ALGO00CAN_R_20121600000_01H_MN.rnx.gz //RINEX nav. mixed

<COMPRESSION>:

Valid compression methods include: gzip - “.gz”, bzip2 - “.bz2” and “.zip”.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 54

RINEX Version 3.03 Appendix A5

A 2 GNSS Observation Data File -Header Section Description

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

RINEX VERSION / TYPE  Format version : 3.03 F9.2, 11X,

 File type: O for Observation Data A1,19X,

 Satellite System: A1,19X

G: GPS

R: GLONASS

E: Galileo

J: QZSS

C: BDS

I: IRNSS

S: SBAS payload

M: Mixed

PGM / RUN BY / DATE  Name of program creating current file A20,

 Name of agency creating current file A20,

 Date and time of file creation A20

Format: yyyymmdd hhmmss zone

zone: 3-4 char. code for time zone.

'UTC ' recommended!

'LCL ' if local time with unknown local time

system code

* COMMENT  Comment line(s) A60

MARKER NAME  Name of antenna marker A60

* MARKER NUMBER  Number of antenna marker A20

MARKER TYPE  Type of the marker: A20,40X

GEODETIC : Earth-fixed, high- precision

monument

NON_GEODETIC : Earth-fixed, low-

precision monument

NON_PHYSICAL : Generated from network

processing

SPACEBORNE : Orbiting space vehicle

GROUND_CRAFT : Mobile terrestrial vehicle

WATER_CRAFT : Mobile water craft

AIRBORNE: Aircraft, balloon, etc.

FIXED_BUOY : "Fixed" on water surface

FLOATING_BUOY: Floating on water surface

FLOATING_ICE : Floating ice sheet, etc.

GLACIER : "Fixed" on a glacier

BALLISTIC : Rockets, shells, etc

ANIMAL : Animal carrying a receiver

HUMAN : Human being

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 55

RINEX Version 3.03 Appendix A6

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

Record required except for GEODETIC and

NON_GEODETIC marker types. Users may

define other project-dependent keywords.

OBSERVER / AGENCY  Name of observer / agency A20,A40

REC # / TYPE / VERS  Receiver number, type, and version (Version: 3A20

e.g. Internal Software Version)

ANT # / TYPE  Antenna number and type 2A20

APPROX POSITION XYZ  Geocentric approximate marker position 3F14.4

(Units: Meters, System: ITRS recommended)

Optional for moving platforms

ANTENNA: DELTA H/E/N  Antenna height: Height of the antenna F14.4,

reference point (ARP) above the marker

 Horizontal eccentricity of ARP relative to the 2F14.4

marker (east/north)

All units in meters

* ANTENNA: DELTA X/Y/Z - Position of antenna reference point for antenna 3F14.4

on vehicle (m): XYZ vector in body-fixed

coord. system

*ANTENNA:PHASECENTER Average phase center position w/r to antenna

reference point (m)

 Satellite system (G/R/E/J/C/I/S) A1,

 Observation code 1X,A3,

 North/East/Up (fixed station) or F9.4,

2F14.4

 X/Y/Z in body-fixed system (vehicle)

* ANTENNA: B.SIGHT XYZ  Direction of the “vertical” antenna axis 3F14.4

towards the GNSS satellites.

Antenna on vehicle: Unit vector in body-fixed

coordinate system.

Tilted antenna on fixed station: Unit vector in

N/E/Up left-handed system.

* ANTENNA: ZERODIR AZI  Azimuth of the zero-direction of a fixed F14.4

antenna (degrees, from north)

* ANTENNA: ZERODIR XYZ  Zero-direction of antenna 3F14.4

Antenna on vehicle: Unit vector in body-fixed

coordinate system

Tilted antenna on fixed station: Unit vector in

N/E/Up left-handed system

* CENTER OF MASS: XYZ  Current center of mass (X,Y,Z, meters) of 3F14.4

vehicle in body-fixed coordinate system. Same

system as used for attitude.

SYS / # / OBS TYPES  Satellite system code (G/R/E/J/C/I/S) A1,

 Number of different observation types for the 2X,I3,

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 56

RINEX Version 3.03 Appendix A7

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

specified satellite system

 Observation descriptors: 13(1X,A3)

Type

Band

Attribute

 Use continuation line(s) for more than 13 6X,

observation descriptors. 13(1X,A3)

In mixed files: Repeat for each satellite system.

These records should precede any SYS / SCALE

FACTOR records (see below).

The following observation descriptors are defined

in RINEX Version 3.XX:

Type:

C = Code / Pseudorange

L = Phase

D = Doppler

S = Raw signal strength(carrier to noise ratio)

I = Ionosphere phase delay

X = Receiver channel numbers

Band:

1 = L1 (GPS, QZSS, SBAS)

G1 (GLO)

E1 (GAL)

2 = L2 (GPS, QZSS)

G2 (GLO)

B1 (BDS)

5 = L5 (GPS, QZSS, SBAS)

E5a (GAL)

L5 (IRNSS)

6 = E6 (GAL)

LEX (QZSS)

B3 (BDS)

7 = E5b (GAL)

B2 (BDS)

8 = E5a+b (GAL)

9 = S (IRNSS)

0 for type X (all)

Attribute:

P = P code-based (GPS,GLO)

C = C code-based (SBAS,GPS,GLO,

QZSS)

D = semi-codeless (GPS)

Y = Y code-based (GPS)

M = M code-based (GPS)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 57

RINEX Version 3.03 Appendix A8

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

N = codeless (GPS)

A = A channel (GAL, IRNSS)

B = B channel (GAL, IRNSS)

C = C channel (GAL, IRNSS)

I = I channel (GPS,GAL, QZSS,

BDS)

Q = Q channel (GPS,GAL, QZSS,

BDS)

S = M channel (L2C GPS, QZSS)

L = L channel (L2C GPS, QZSS)

S = D channel (GPS, QZSS)

L = P channel (GPS, QZSS)

X = B+C channels (GAL, IRNSS)

I+Q channels (GPS,GAL, QZSS,

BDS)

M+L channels (GPS, QZSS)

D+P channels (GPS, QZSS)

W = based on Z-tracking (GPS)

(see text)

Z = A+B+C channels (GAL)

All characters in uppercase only!

Units :

Phase : full cycles

Pseudorange : meters

Doppler : Hz

SNR etc : receiver-dependent

Ionosphere : full cycles

Channel # : See text

Sign definition: See text.

The sequence of the observations in the observation

records has to correspond to the sequence of the

types in this record of the respective satellite

system.

Note: In RINEX 3.02 and 3.03 all fields (Type,

Band and Attribute) must be defined. Only

known tracking modes are allowed.

* SIGNAL STRENGTH UNIT  Unit of the carrier to noise ratio observables A20,40X

Snn (if present) DBHZ : S/N given in dbHz

* INTERVAL  Observation interval in seconds F10.3

TIME OF FIRST OBS  Time of first observation record (4-digit-year, 5I6,F13.7,

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 58

RINEX Version 3.03 Appendix A9

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

month, day, hour, min, sec)

 Time system:

5X,A3

GPS (=GPS time system)

GLO (=UTC time system)

GAL (=Galileo time system)

QZS (= QZSS time system)

BDT (= BDS time system)

IRN (= IRNSS time system)

Compulsory in mixed GNSS files

Defaults:

GPS for pure GPS files

GLO for pure GLONASS files

GAL for pure Galileo files

QZS for pure QZSS files

BDT for pure BDS files

IRN for pure IRNSS files

* TIME OF LAST OBS  Time of last observation record (4-digit-year, 5I6,F13.7,

month,day,hour,min,sec)

 Time system: Same value as in TIME OF 5X,A3

FIRST OBS record

* RCV CLOCK OFFS APPL  Epoch, code, and phase are corrected by I6

applying the realtime-derived receiver clock

offset: 1=yes, 0=no; default: 0=no Record

required if clock offsets are reported in the

EPOCH/SAT records

* SYS / DCBS APPLIED  Satellite system (G/R/E/J/C/I/S) A1,

 Program name used to apply differential code 1X,A17,

bias corrections

 Source of corrections (URL) 1X,A40

Repeat for each satellite system.

No corrections applied: Blank fields or record not

present.

* SYS / PCVS APPLIED  Satellite system (G/R/E/J/C/I/S) A1,

 Program name used to apply phase center 1X,A17,

variation corrections

 Source of corrections (URL) 1X,A40

Repeat for each satellite system.

No corrections applied: Blank fields or record not

present.

* SYS / SCALE FACTOR  Satellite system (G/R/E/J/C/I/S) A1,

 Factor to divide stored observations with 1X,I4,

before use (1,10,100,1000)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 59

RINEX Version 3.03 Appendix A10

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

 Number of observation types involved. 0 or 2X,I2,

blank: All observation types

 List of observation types 12(1X,A3)

 Use continuation line(s) for more than 12 10X,

observation types. 12(1X,A3)

Repeat record if different factors are applied to

different observation types.

A value of 1 is assumed if record is missing.

SYS / PHASE SHIFT Phase shift correction used to generate phases

consistent w/r to cycle shifts

 Satellite system (G/R/E/J/C/I/S) A1,1X,

 Carrier phase observation code: A3,1X,

Type

Band

Attribute

 Correction applied (cycles) F8.5

 Number of satellites involved 0 or blank: All 2X,I2.2,

satellites of system

 List of satellites

10(1X,A3)

 Use continuation line(s) for more than 10

18X,

satellites.

10(1X,A3)

Repeat the record for all affected codes.

See chapter 9.1 for more details!

GLONASS SLOT / FRQ # GLONASS slot and frequency numbers

 Number of satellites in list I3,1X,

List of :

 Satellite numbers (system code, slot) 8(A1,I2.2,

 Frequency numbers (-7...+6) 1X,I2,1X)

 Use continuation lines for more than 8 4X,8(A1,

Satellites I2.2,1X,I2,1

X)

GLONASS COD/PHS/BIS  GLONASS Phase bias correction used to align 4(X1,A3,X1

code and phase observations. ,F8.3)

 GLONASS signal identifier : C1C and Code

Phase bias correction (metres)

 GLONASS signal identifier : C1P and Code

Phase bias correction (metres)

 GLONASS signal identifier : C2C and Code

Phase bias correction (metres)

 GLONASS signal identifier : C2P and Code

Phase bias correction (metres)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 60

RINEX Version 3.03 Appendix A11

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

Note: If the GLONASS code phase bias values are

unknown then all fields in the record are left blank

(see example in Section 9.9) and only the record

header is defined.

* LEAP SECONDS  Current Number of leap seconds I6,

 Future or past leap seconds ΔtLSF(BNK) , i.e. I6,

future leap second if the week and day number I6,

are in the future.

 Respective week number WN_LSF

(continuous number) (BNK). For GPS, GAL,

QZS and IRN, weeks since 6-Jan-1980. When

BDS only file leap seconds specified, weeks I6

since 1-Jan-2006.

 Respective day number DN (0-6) BeiDou and

(1-7) for GPS and others constellations,

(BNK). The day number is the GPS or BeiDou

day before the leap second (See Note 1 below).

In the case of the Tuesday, June 30/2015 (GPS

Week 1851, DN 3) the UTC leap second A3

actually occurred 16 seconds into the next GPS

day.

 Time system identifier: only GPS and BDS are

valid identifiers. Blank defaults to GPS see

Notes section below.

Notes:

1. GPS, GAL, QZS and IRN time systems are

aligned and are equivalent with respect to leap

seconds (Leap seconds since 6-Jan-1980).See

the GPS almanac, and DN reference IS-GPS-

200H 20.3.3.5.2.4.

2. For BDS only observation files, the Number of

leap seconds since 1-Jan-2006 as transmitted by

the BDS almanac ΔtLS(see BDS-SIS-ICD-2.0

5.2.4.17)

* # OF SATELLITES  Number of satellites, for which observations I6

are stored in the file

* PRN / # OF OBS  Satellite numbers, number of observations for 3X,

each observation type indicated in the SYS / # / A1,I2.2,

OBS TYPES record. 9I6

 If more than 9 observation types: 6X,9I6

Use continuation line(s)

In order to avoid format overflows, 99999 indicates

>= 99999 observations in the RINEX file.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 61

RINEX Version 3.03 Appendix A12

TABLE A2

GNSS OBSERVATION DATA FILE - HEADER SECTION DESCRIPTION

This record is (these records are) repeated for each

satellite present in the data file.

END OF HEADER Last record in the header section. 60X

Records marked with * are optional

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 62

RINEX Version 3.03 Appendix A13

A 3 GNSS Observation Data File -Data Record Description

TABLE A3

GNSS OBSERVATION DATA FILE – DATA RECORD DESCRIPTION

DESCRIPTION FORMAT

EPOCH record

 Record identifier : > A1,

Epoch

 year (4 digits): 1X,I4,

 month, day, hour, min (two digits) 4(1X,I2.2),

 sec F11.7,

 Epoch flag, 2X,I1,

0: OK

1: power failure between previous and current epoch

>1: Special event

 Number of satellites observed in current epoch

I3,

 (reserved)

6X,

 Receiver clock offset (seconds, optional) F15.12

Epoch flag = 0 or 1: OBSERVATION records follow

 Satellite number A1,I2.2,

 Observation - repeat within record for each observation m(F14.3,

 LLI - type (same sequence as given in the respective SYS / # / OBS I1,

TYPES record)

I1)

 Signal strength

This record is repeated for each satellite having been observed in the current

epoch. The record length is given by the number of observation types for this

satellite.

Observations: For definition see text.

Missing observations are written as 0.0 or blanks. Phase values overflowing the

fixed format F14.3 have to be clipped into the valid interval (e.g add or subtract

10**9), set bit 0 of LLI indicator.

Loss of lock indicator (LLI).

0 or blank: OK or not known

Bit 0 set: Lost lock between previous and current observation: Cycle slip

possible. For phase observations only. Note: Bit 0 is the least significant

bit.

Bit 1 set: Half-cycle ambiguity/slip possible. Software not capable of

handling half cycles should skip this observation. Valid for the current

epoch only.

Bit 2 set: Galileo BOC-tracking of an MBOC-modulated signal (may

suffer from increased noise).

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 63

RINEX Version 3.03 Appendix A14

TABLE A3

GNSS OBSERVATION DATA FILE – DATA RECORD DESCRIPTION

Signal strength projected into interval 1-9:

1: minimum possible signal strength

5: average/good S/N ratio

9: maximum possible signal strength

0 or blank: not known, don't care

Standardization for S/N values given in dbHz: See text.

Epoch flag 2-5: EVENT: Special records may follow

 Epoch flag

[2X,I1]

 2: start moving antenna

 3: new site occupation (end of kinematic data) (at least MARKER

NAME record follows)

 4: header information follows

 5: external event (epoch is significant, same time frame as

observation time tags)

 "Number of satellites" contains number of special records to follow. 0 if no [I3]

special records follow.

Maximum number of records: 999

For events without significant epoch the epoch fields in the EPOCH

RECORD can be left blank

Epoch flag = 6: EVENT: Cycle slip records follow

 Epoch flag [2X,I1]

 6: cycle slip records follow to optionally report detected and repaired

cycle slips (same format as OBSERVATIONS records;

 slip instead of observation;

 LLI and signal strength blank or zero)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 64

RINEX Version 3.03 Appendix A15

A 4 GNSS Observation Data File – Example #1

+------------------------------------------------------------------------------+

| TABLE A4 |

| GNSS OBSERVATION DATA FILE - EXAMPLE #1 |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 OBSERVATION DATA M RINEX VERSION / TYPE

G = GPS R = GLONASS E = GALILEO S = GEO M = MIXED COMMENT

XXRINEXO V9.9 AIUB 20060324 144333 UTC PGM / RUN BY / DATE

EXAMPLE OF A MIXED RINEX FILE VERSION 3.03 COMMENT

The file contains L1 pseudorange and phase data of the COMMENT

geostationary AOR-E satellite (PRN 120 = S20) COMMENT

A 9080 MARKER NAME

9080.1.34 MARKER NUMBER

BILL SMITH ABC INSTITUTE OBSERVER / AGENCY

X1234A123 GEODETIC 1.3.1 REC # / TYPE / VERS

G1234 ROVER ANT # / TYPE

4375274. 587466. 4589095. APPROX POSITION XYZ

.9030 .0000 .0000 ANTENNA: DELTA H/E/N

0 RCV CLOCK OFFS APPL

G 5 C1C L1W L2W C1W S2W SYS / # / OBS TYPES

R 2 C1C L1C SYS / # / OBS TYPES

E 2 L1B L5I SYS / # / OBS TYPES

S 2 C1C L1C SYS / # / OBS TYPES

18.000 INTERVAL

G APPL_DCB xyz.uvw.abc//pub/dcb_gps.dat SYS / DCBS APPLIED

DBHZ SIGNAL STRENGTH UNIT

2006 03 24 13 10 36.0000000 GPS TIME OF FIRST OBS

18 R01 1 R02 2 R03 3 R04 4 R05 5 R06 -6 R07 -5 R08 -4 GLONASS SLOT / FRQ #

R09 -3 R10 -2 R11 -1 R12 0 R13 1 R14 2 R15 3 R16 4 GLONASS SLOT / FRQ #

R17 5 R18 -5 GLONASS SLOT / FRQ #

G L1C SYS / PHASE SHIFT

G L1W 0.00000 SYS / PHASE SHIFT

G L2W SYS / PHASE SHIFT

R L1C SYS / PHASE SHIFT

E L1B SYS / PHASE SHIFT

E L5I SYS / PHASE SHIFT

S L1C SYS / PHASE SHIFT

C1C -10.000 C1P -10.123 C2C -10.432 C2P -10.634 GLONASS COD/PHS/BIS

END OF HEADER

> 2006 03 24 13 10 36.0000000 0 5 -0.123456789012

G06 23629347.915 .300 8 -.353 4 23629347.158 24.158

G09 20891534.648 -.120 9 -.358 6 20891545.292 38.123

G12 20607600.189 -.430 9 .394 5 20607600.848 35.234

E11 .324 8 .178 7

S20 38137559.506 335849.135 9

> 2006 03 24 13 10 54.0000000 0 7 -0.123456789210

G06 23619095.450 -53875.632 8 -41981.375 4 23619095.008 25.234

G09 20886075.667 -28688.027 9 -22354.535 7 20886076.101 42.231

G12 20611072.689 18247.789 9 14219.770 6 20611072.410 36.765

R21 21345678.576 12345.567 5

R22 22123456.789 23456.789 5

E11 65432.123 5 48861.586 7

S20 38137559.506 335849.135 9

> 2006 03 24 13 11 12.0000000 2 2

*** FROM NOW ON KINEMATIC DATA! *** COMMENT

TWO COMMENT LINES FOLLOW DIRECTLY THE EVENT RECORD COMMENT

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 65

RINEX Version 3.03 Appendix A16

> 2006 3 24 13 11 12.0000000 0 4 -0.123456789876

G06 21110991.756 16119.980 7 12560.510 4 21110991.441 25.543

G09 23588424.398 -215050.557 6 -167571.734 6 23588424.570 41.824

G12 20869878.790 -113803.187 8 -88677.926 6 20869878.938 36.961

G16 20621643.727 73797.462 7 57505.177 2 20621644.276 15.368

> 3 4

A 9081 MARKER NAME

9081.1.34 MARKER NUMBER

.9050 .0000 .0000 ANTENNA: DELTA H/E/N

--> THIS IS THE START OF A NEW SITE <-- COMMENT

> 2006 03 24 13 12 6.0000000 0 4 -0.123456987654

G06 21112589.384 24515.877 6 19102.763 4 21112589.187 25.478

G09 23578228.338 -268624.234 7 -209317.284 6 23578228.398 41.725

G12 20625218.088 92581.207 7 72141.846 5 20625218.795 35.143

G16 20864539.693 -141858.836 8 -110539.435 2 20864539.943 16.345

> 2006 03 24 13 13 1.2345678 5 0

> 4 2

AN EVENT FLAG 5 WITH A SIGNIFICANT EPOCH COMMENT

AND AN EVENT FLAG 4 TO ESCAPE FOR THE TWO COMMENT LINES COMMENT

> 2006 03 24 13 14 12.0000000 0 4 -0.123456012345

G06 21124965.133 0.30213 -0.62614 21124965.275 27.528

G09 23507272.372 -212616.150 7 -165674.789 7 23507272.421 42.124

G12 20828010.354 -333820.093 6 -260119.395 6 20828010.129 37.002

G16 20650944.902 227775.130 7 177487.651 3 20650944.363 18.040

> 4 1

*** LOST LOCK ON G 06 COMMENT

.

.

.

> 4 1

END OF FILE COMMENT

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 66

RINEX Version 3.03 Appendix A17

A 4 GNSS Observation Data File – Example #2

+------------------------------------------------------------------------------+

| TABLE A4 |

| GNSS OBSERVATION DATA FILE - EXAMPLE #2 |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 OBSERVATION DATA M RINEX VERSION / TYPE

sbf2rin-9.3.3 20140511 000610 LCL PGM / RUN BY / DATE

faa1 MARKER NAME

92201M012 MARKER NUMBER

Unknown Unknown OBSERVER / AGENCY

3001320 SEPT POLARX4 2.5.1p1 REC # / TYPE / VERS

725235 LEIAR25.R4 NONE ANT # / TYPE

-5246415.0000 -3077260.0000 -1913842.0000 APPROX POSITION XYZ

0.1262 0.0000 0.0000 ANTENNA: DELTA H/E/N

G 18 C1C L1C D1C S1C C1W S1W C2W L2W D2W S2W C2L L2L D2L SYS / # / OBS TYPES

S2L C5Q L5Q D5Q S5Q SYS / # / OBS TYPES

E 16 C1C L1C D1C S1C C5Q L5Q D5Q S5Q C7Q L7Q D7Q S7Q C8Q SYS / # / OBS TYPES

L8Q D8Q S8Q SYS / # / OBS TYPES

S 4 C1C L1C D1C S1C SYS / # / OBS TYPES

R 12 C1C L1C D1C S1C C2P L2P D2P S2P C2C L2C D2C S2C SYS / # / OBS TYPES

C 8 C2I L2I D2I S2I C7I L7I D7I S7I SYS / # / OBS TYPES

J 12 C1C L1C D1C S1C C2L L2L D2L S2L C5Q L5Q D5Q S5Q SYS / # / OBS TYPES

G L1C SYS / PHASE SHIFT

G L2W SYS / PHASE SHIFT

G L2L 0.00000 SYS / PHASE SHIFT

G L5Q 0.00000 SYS / PHASE SHIFT

E L1C 0.00000 SYS / PHASE SHIFT

E L5Q 0.00000 SYS / PHASE SHIFT

E L7Q 0.00000 SYS / PHASE SHIFT

E L8Q 0.00000 SYS / PHASE SHIFT

S L1C SYS / PHASE SHIFT

R L1C SYS / PHASE SHIFT

R L2P 0.00000 SYS / PHASE SHIFT

R L2C SYS / PHASE SHIFT

C L2I SYS / PHASE SHIFT

C L7I SYS / PHASE SHIFT

J L1C SYS / PHASE SHIFT

J L2L 0.00000 SYS / PHASE SHIFT

J L5Q 0.00000 SYS / PHASE SHIFT

30.000 INTERVAL

2014 5 10 0 0 0.0000000 GPS TIME OF FIRST OBS

2014 5 10 23 59 30.0000000 GPS TIME OF LAST OBS

72 # OF SATELLITES

C1C 0.000 C2C 0.000 C2P 0.000 GLONASS COD/PHS/BIS

DBHZ SIGNAL STRENGTH UNIT

24 R01 1 R02 -4 R03 5 R04 6 R05 1 R06 -4 R07 5 R08 6 GLONASS SLOT / FRQ #

R09 -2 R10 -7 R11 0 R12 -1 R13 -2 R14 -7 R15 0 R16 -1 GLONASS SLOT / FRQ #

R17 4 R18 -3 R19 3 R20 2 R21 4 R22 -3 R23 3 R24 2 GLONASS SLOT / FRQ #

END OF HEADER

> 2014 05 10 00 00 0.0000000 0 28

END OF FILE COMMENT

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 67

RINEX Version 3.03 Appendix A18

A 4 GNSS Observation Data File – Example #3

+------------------------------------------------------------------------------+

| TABLE A4 |

| GNSS OBSERVATION DATA FILE - EXAMPLE #3 |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 OBSERVATION DATA M: MIXED RINEX VERSION / TYPE

GR25 V3.08 20140513 072944 UTC PGM / RUN BY / DATE

SNR is mapped to RINEX snr flag value [1-9] COMMENT

LX: < 12dBHz -> 1; 12-17dBHz -> 2; 18-23dBHz -> 3 COMMENT

24-29dBHz -> 4; 30-35dBHz -> 5; 36-41dBHz -> 6 COMMENT

42-47dBHz -> 7; 48-53dBHz -> 8; >= 54dBHz -> 9 COMMENT

Tokio MARKER NAME

TOKI MARKER NUMBER

SU Japan - Leica Geosystems OBSERVER / AGENCY

1870023 LEICA GR25 3.08/6.401 REC # / TYPE / VERS

LEIAS10 NONE ANT # / TYPE

-3956196.8609 3349495.1794 3703988.8347 APPROX POSITION XYZ

0.0000 0.0000 0.0000 ANTENNA: DELTA H/E/N

G 16 C1C L1C D1C S1C C2S L2S D2S S2S C2W L2W D2W S2W C5Q SYS / # / OBS TYPES

L5Q D5Q S5Q SYS / # / OBS TYPES

R 12 C1C L1C D1C S1C C2P L2P D2P S2P C2C L2C D2C S2C SYS / # / OBS TYPES

E 16 C1C L1C D1C S1C C5Q L5Q D5Q S5Q C7Q L7Q D7Q S7Q C8Q SYS / # / OBS TYPES

L8Q D8Q S8Q SYS / # / OBS TYPES

C 8 C2I L2I D2I S2I C7I L7I D7I S7I SYS / # / OBS TYPES

J 12 C1C L1C D1C S1C C2S L2S D2S S2S C5Q L5Q D5Q S5Q SYS / # / OBS TYPES

S 4 C1C L1C D1C S1C SYS / # / OBS TYPES

DBHZ SIGNAL STRENGTH UNIT

1.000 INTERVAL

2014 05 13 07 30 0.0000000 GPS TIME OF FIRST OBS

2014 05 13 07 34 59.0000000 GPS TIME OF LAST OBS

0 RCV CLOCK OFFS APPL

G L1C SYS / PHASE SHIFT

G L2S -0.25000 SYS / PHASE SHIFT

G L2W SYS / PHASE SHIFT

G L2Q -0.25000 SYS / PHASE SHIFT

R L1C SYS / PHASE SHIFT

R L2P 0.25000 SYS / PHASE SHIFT

R L2C SYS / PHASE SHIFT

E L1C +0.50000 SYS / PHASE SHIFT

E L5Q -0.25000 SYS / PHASE SHIFT

E L7Q -0.25000 SYS / PHASE SHIFT

E L8Q -0.25000 SYS / PHASE SHIFT

C L2I SYS / PHASE SHIFT

C L7I SYS / PHASE SHIFT

J L1C SYS / PHASE SHIFT

J L2S SYS / PHASE SHIFT

J L5Q -0.25000 SYS / PHASE SHIFT

S L1C SYS / PHASE SHIFT

24 R01 1 R02 -4 R03 5 R04 6 R05 1 R06 -4 R07 5 R08 6 GLONASS SLOT / FRQ #

R09 -2 R10 -7 R11 0 R12 -1 R13 -2 R14 -7 R15 0 R16 -1 GLONASS SLOT / FRQ #

R17 4 R18 -3 R19 3 R20 2 R21 4 R22 -3 R23 3 R24 2 GLONASS SLOT / FRQ #

C1C 0.000 C1P 0.000 C2C 0.000 C2P 0.000 GLONASS COD/PHS/BIS

16 1694 7 LEAP SECONDS

END OF HEADER

> 2014 05 13 07 30 0.0000000 0 25

END OF FILE COMMENT

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 68

RINEX Version 3.03 Appendix A19

A 5 GNSS Navigation Message File – Header Section Description

TABLE A5

GNSS NAVIGATION MESSAGE FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

RINEX VERSION / TYPE  Format version : 3.03 F9.2,11X,

 File type ('N' for navigation data) A1,19X,

 Satellite System: A1,19X

G: GPS

R: GLONASS

E: Galileo

J: QZSS

C: BDS

I: IRNSS

S: SBAS Payload

M: Mixed

PGM / RUN BY / DATE  Name of program creating current file A20,

 Name of agency creating current file A20,

 Date and time of file creation A20

Format: yyyymmdd hhmmss zone

zone: 3-4 char. code for time zone.

'UTC ' recommended!

'LCL ' if local time with unknown local time

system code

* COMMENT Comment line(s) A60

* IONOSPHERIC CORR Ionospheric correction parameters

 Correction type: A4,1X,

GAL= Galileo ai0 – ai2

GPSA= GPS alpha0 - alpha3

GPSB= GPS beta0 - beta3

QZSA = QZS alpha0 - alpha3

QZSB = QZS beta0 - beta3

BDSA = BDS alpha0 - alpha3

BDSB = BDS beta0 - beta3

IRNA = IRNSS alpha0 - alpha3

IRNB = IRNSS beta0 - beta3

 Parameters: 4D12.4

GPS: alpha0-alpha3 or beta0-beta3

GAL: ai0, ai1, ai2, Blank

QZS: alpha0-alpha3 or beta0-beta3

BDS: alpha0-alpha3 or beta0-beta3

IRN: alpha0-alpha3 or beta0-beta3

 Time mark, Transmission Time (seconds of

1X,A1

week) converted to hours of day and then to A-

X. See BDS example below:

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 69

RINEX Version 3.03 Appendix A20

TABLE A5

GNSS NAVIGATION MESSAGE FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

A=BDT 00h-01h;

B=BDT 01h-02h;

...

X= BDT 23h-24h.

This field is mandatory for BDS and optional for the

other constellations, (BNK).

 SV ID, identify which satellite provided the

ionospheric parameters. This field is 1X,I2

mandatory for BDS and optional for the other

constellations (BNK).

Note 1: Multiple IONOSPHERIC CORR message

can be written in the header.

Note 2: It is recommended that BDS ionospheric

broadcast model parameters from BDS GEO

satellites, be given the most priority. Then the

parameters from BDS IGSO satellites should be

given secondary priority and then tertiary priority is

given to BDS MEO satellite ionospheric correction

parameters.

* TIME SYSTEM CORR Corrections to transform the system time to UTC or

other time systems

 Correction type: A4,1X,

GAUT = GAL to UTC a0, a1

GPUT = GPS to UTC a0, a1

SBUT = SBAS to UTC a0, a1

GLUT = GLO to UTC a0= -TauC, a1=zero

GPGA = GPS to GAL a0=A0G, a1=A1G

GLGP = GLO to GPS a0=TauGPS, a1=zero

QZGP = QZS to GPS a0, a1

QZUT = QZS to UTC a0, a1

BDUT =BDS to UTC a0=A , a1=A

0UTC 1UTC

IRUT =IRN to UTC a0=A , a1=A

0UTC 1UTC

IRGP =IRN to GPS a0=A , a1=A

0 1

 a0,a1 Coefficients of 1-deg polynomial (a0 sec, D17.10,

a1 sec/sec) CORR(s) = a0 + a1*DELTAT D16.9,

 T Reference time for polynomial (Seconds into I7,

GPS/GAL/ BDS week)

I5,

 W Reference week number

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 70

RINEX Version 3.03 Appendix A21

TABLE A5

GNSS NAVIGATION MESSAGE FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

(GPS/GAL/BDS/IRN/SBAS week, continuous

number from 6-Jan-1980), T and W zero for

GLONASS. BDS week, continuous from: 1-

Jan-2006

 S EGNOS, WAAS, or MSAS ... (left-justified) 1X,A5,1X

Derived from MT17 service provider. If not

known: Use Snn with: nn = PRN-100 of

satellite broadcasting the MT12

 U UTC Identifier (0 if unknown) I2,1X

1=UTC(NIST), 2=UTC(USNO), 3=UTC(SU),

4=UTC(BIPM), 5=UTC(Europe Lab),

6=UTC(CRL), 7=UTC(NTSC) (BDS), >7 =

not assigned yet S and U for SBAS only.

* LEAP SECONDS  Current Number of leap seconds I6,

 Future or past leap seconds ΔtLSF (BNK), i.e. I6,

future leap second if the week and day number

are in the future.

 Respective week number WN_LSF I6,

(continuous number) (BNK). For GPS, GAL,

QZS and IRN, weeks since 6-Jan-1980. When

BDS only file leap seconds specified, weeks

since 1-Jan-2006.

I6

 Respective day number DN (0-6) BeiDou and

(1-7) for GPS and others constellations,

(BNK). The day number is the GPS or BeiDou

day before the leap second (See Note 1 below).

In the case of the Tuesday, June 30/2015 (GPS

Week 1851, DN 3) the UTC leap second

actually occurred 16 seconds into the next GPS

day.

A3

 Time system identifier: only GPS and BDS are

valid identifiers. Blank defaults to GPS, see

Notes section below.

Notes:

1. GPS, GAL, QZS and IRN time systems are

aligned and are equivalent with respect to leap

seconds (Leap seconds since 6-Jan-1980).See

the GPS almanac and DN reference IS-GPS-

200H 20.3.3.5.2.4.

2. For BDS only navigation files, the Number of

leap seconds since 1-Jan-2006 as transmitted by

the BDS almanac ΔtLS(see BDS-SIS-ICD-2.0

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 71

RINEX Version 3.03 Appendix A22

TABLE A5

GNSS NAVIGATION MESSAGE FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

5.2.4.17)

END OF HEADER

Records marked with * are optional, BNK- Blank if Not Know/Defined

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 72

RINEX Version 3.03 Appendix A23

A 6 GNSS Navigation Message File – GPS Data Record Description

TABLE A6

GNSS NAVIGATION MESSAGE FILE – GPS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

SV / EPOCH / SV CLK - Satellite system (G), sat number (PRN) A1,I2.2,

- Epoch: Toc - Time of Clock (GPS) year (4 1X,I4,

digits)

- month, day, hour, minute, second 5(1X,I2.2),

- SV clock bias (seconds) 3D19.12

- SV clock drift (sec/sec)

- SV clock drift rate (sec/sec2) *)

BROADCAST ORBIT - 1 - IODE Issue of Data, Ephemeris 4X,4D19.12

- Crs (meters)

- Delta n (radians/sec) ***)

- M0 (radians)

BROADCAST ORBIT - 2 - Cuc (radians) 4X,4D19.12

- e Eccentricity

- Cus (radians)

- sqrt(A) (sqrt(m))

BROADCAST ORBIT - 3 - Toe Time of Ephemeris (sec of GPS week) 4X,4D19.12

- Cic (radians)

- OMEGA0 (radians)

- Cis (radians)

BROADCAST ORBIT - 4 - i0 (radians) 4X,4D19.12

- Crc (meters)

- omega (radians)

- OMEGA DOT (radians/sec)

BROADCAST ORBIT - 5 - IDOT (radians/sec) 4X,4D19.12

- Codes on L2 channel

- GPS Week # (to go with TOE) Continuous

number, not mod(1024)!

- L2 P data flag

BROADCAST ORBIT - 6 - SV accuracy (meters) See GPS ICD 200H 4X,4D19.12

Section 20.3.3.3.1.3 use specified

equations to define nominal values, N = 0-

6: use 2(1+N/2) (round to one decimal place

i.e. 2.8, 5.7 and 11.3) , N= 7-15:use 2 (N-2),

8192 specifies use at own risk

- SV health (bits 17-22 w 3 sf 1)

- TGD (seconds)

- IODC Issue of Data, Clock

BROADCAST ORBIT - 7 - Transmission time of message **) 4X,4D19.12

(sec of GPS week, derived e.g.from Z-

count in Hand Over Word (HOW))

- Fit Interval in hours see section 6.11.

(BNK).

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 73

RINEX Version 3.03 Appendix A24

TABLE A6

GNSS NAVIGATION MESSAGE FILE – GPS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

- Spare

- Spare

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

**) Adjust the Transmission time of message by + or -604800 to refer to the reported week in

BROADCAST ORBIT 5, if necessary. Set value to 0.9999E9 if not known.

A 7 GPS Navigation Message File – Example

+------------------------------------------------------------------------------+

| TABLE A7 |

| GPS NAVIGATION MESSAGE FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 N: GNSS NAV DATA G: GPS RINEX VERSION / TYPE

XXRINEXN V3 AIUB 19990903 152236 UTC PGM / RUN BY / DATE

EXAMPLE OF VERSION 3.03 FORMAT COMMENT

GPSA .1676D-07 .2235D-07 .1192D-06 .1192D-06 IONOSPHERIC CORR

GPSB .1208D+06 .1310D+06 -.1310D+06 -.1966D+06 IONOSPHERIC CORR

GPUT .1331791282D-06 .107469589D-12 552960 1025 TIME SYSTEM CORR

13 LEAP SECONDS

END OF HEADER

G06 1999 09 02 17 51 44 -.839701388031D-03 -.165982783074D-10 .000000000000D+00

.910000000000D+02 .934062500000D+02 .116040547840D-08 .162092304801D+00

.484101474285D-05 .626740418375D-02 .652112066746D-05 .515365489006D+04

.409904000000D+06 -.242143869400D-07 .329237003460D+00 -.596046447754D-07

.111541663136D+01 .326593750000D+03 .206958726335D+01 -.638312302555D-08

.307155651409D-09 .000000000000D+00 .102500000000D+04 .000000000000D+00

.000000000000D+00 .000000000000D+00 .000000000000D+00 .910000000000D+02

.406800000000D+06 .400000000000E+01

G13 1999 09 02 19 00 00 .490025617182D-03 .204636307899D-11 .000000000000D+00

.133000000000D+03 -.963125000000D+02 .146970407622D-08 .292961152146D+01

-.498816370964D-05 .200239347760D-02 .928156077862D-05 .515328476143D+04

.414000000000D+06 -.279396772385D-07 .243031939942D+01 -.558793544769D-07

.110192796930D+01 .271187500000D+03 -.232757915425D+01 -.619632953057D-08

-.785747015231D-11 .000000000000D+00 .102500000000D+04 .000000000000D+00

.000000000000D+00 .000000000000D+00 .000000000000D+00 .389000000000D+03

.410400000000D+06 .400000000000E+01

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 74

RINEX Version 3.03 Appendix A25

A 8 GNSS Navigation Message File – GALILEO Data Record Description

TABLE A8

GNSS NAVIGATION MESSAGE FILE - GALILEO DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

SV / EPOCH / SV CLK - Satellite system (E), satellite number A1,I2.2,

- Epoch: Toc - Time of Clock GALyear (4 1X,I4,

digits)

- month, day, hour, minute, second 5(1X,I2.2),

- SV clock bias (seconds) af0 3D19.12

- SV clock drift (sec/sec) af1

- SV clock drift rate (sec/sec2) af2 (see *)

Br.Orbit-5, data source, bits 8+9)

BROADCAST ORBIT - 1 - IODnav Issue of Data of the nav batch 4X,4D19.12

- Crs (meters)

- Delta n (radians/sec) ***)

- M0 (radians)

BROADCAST ORBIT - 2 - Cuc (radians) 4X,4D19.12

- e Eccentricity

- Cus (radians)

- sqrt(a) (sqrt(m))

BROADCAST ORBIT - 3 - Toe Time of Ephemeris (sec of GAL week) 4X,4D19.12

- Cic (radians)

- OMEGA0 (radians)

- Cis (radians)

BROADCAST ORBIT - 4 - i0 (radians) 4X,4D19.12

- Crc (meters)

- omega (radians)

- OMEGA DOT (radians/sec)

BROADCAST ORBIT - 5 - IDOT (radians/sec) 4X,4D19.12

- Data sources (FLOAT --> INTEGER)

Bit 0 set: I/NAV E1-B

Bit 1 set: F/NAV E5a-I

Bit 2 set: I/NAV E5b-I

Bits 0 and 2 : Both can be set if the

navigation messages were merged, however,

bits 0-2 cannot all be set, as the I/NAV and

F/NAV messages contain different

information

Bit 3 reserved for Galileo internal use

Bit 4 reserved for Galileo internal use

Bit 8 set: af0-af2, Toc, SISA are for E5a,E1 ****)

Bit 9 set: af0-af2, Toc, SISA are for E5b,E1

Bits 8-9 : exclusive (only one bit can be set)

- GAL Week # (to go with Toe)

- spare

BROADCAST ORBIT - 6 - SISA Signal in space accuracy (meters) 4X,4D19.12

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 75

RINEX Version 3.03 Appendix A26

TABLE A8

GNSS NAVIGATION MESSAGE FILE - GALILEO DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

Undefined/unknown: -1.0

- SV health (FLOAT converted to INTEGER) *****)

See Galileo ICD Section 5.1.9.3

Bit 0: E1B DVS

Bits 1-2: E1B HS

Bit 3: E5a DVS

Bits 4-5: E5a HS

Bit 6: E5b DVS

Bits 7-8: E5b HS

- BGD E5a/E1 (seconds)

- BGD E5b/E1 (seconds)

BROADCAST ORBIT - 7 - Transmission time of message **) 4 X ,4D19.12

(sec of GAL week, derived from WN and

TOW of page type 1)

- spare

- spare

- spare

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

**) Adjust the Transmission time of message by + or -604800 to refer to the reported week in

BROADCAST ORBIT 5, if necessary. Set value to 0.9999E9 if not known.

***) Angles and their derivatives transmitted in units of semi-circles and semi-circles/sec have to

be converted to radians by the RINEX generator.

****) The GAL week number is a continuous number, aligned to (and hence identical to) the

continuous GPS week number used in the RINEX navigation message files. The broadcast 12-bit

Galileo System Time (GST) week has a roll-over after 4095. It started at zero at the first GPS

roll-over (continuous GPS week 1024). Hence GAL week = GST week + 1024 + n*4096 (n:

number of GST roll-overs).

*****)

-If bit 0 or bit 2 of Data sources (BROADCAST ORBIT – 5) is set, E1B DVS & HS, E5b

DVS & HS and both BGDs are valid. -If bit 1 of Data sources is set, E5a DVS & HS and BGD

E5a/E1 are valid. -Non valid parameters are set to 0 and to be ignored

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 76

RINEX Version 3.03 Appendix A27

A 9 GALILEO Navigation Message File – Examples

+------------------------------------------------------------------------------+

| TABLE A9 |

| GALILEO NAVIGATION MESSAGE FILE - EXAMPLES |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 N: GNSS NAV DATA E: GALILEO NAV DATA RINEX VERSION / TYPE

NetR9 5.01 Receiver Operator 20150619 000000 UTC PGM / RUN BY / DATE

GAL .1248D+03 .5039D+00 .2377D-01 .0000D+00 IONOSPHERIC CORR

GAUT .3725290298D-08 .532907052D-14 345600 1849 TIME SYSTEM CORR

16 17 1851 3 LEAP SECONDS

END OF HEADER

E12 2015 06 19 02 10 00 -.138392508961D-02 -.131464616970D-09 .000000000000D+00

.930000000000D+02 -.165531250000D+03 .285797618904D-08 .138275888459D+01

-.782497227192D-05 .346679124050D-03 .114385038614D-04 .544062509727D+04

.439800000000D+06 .298023223877D-07 -.296185101312D+01 -.111758708954D-07

.965683294025D+00 .993750000000D+02 -.629360976005D+00 -.541593988135D-08

-.571452374714D-11 .516000000000D+03 .184900000000D+04

.312000000000D+01 .000000000000D+00 -.651925802231D-08 -.605359673500D-08

.440734000000D+06

E12 2015 06 19 02 10 00 -.138392508961D-02 -.131464616970D-09 .000000000000D+00

.930000000000D+02 -.165531250000D+03 .285797618904D-08 .138275888459D+01

-.782497227192D-05 .346679124050D-03 .114385038614D-04 .544062509727D+04

.439800000000D+06 .298023223877D-07 -.296185101312D+01 -.111758708954D-07

.965683294025D+00 .993750000000D+02 -.629360976005D+00 -.541593988135D-08

-.571452374714D-11 .513000000000D+03 .184900000000D+04

.312000000000D+01 .000000000000D+00 -.651925802231D-08 -.605359673500D-08

.440725000000D+06

E12 2015 06 19 02 10 00 -.138392532244D-02 -.131450406116D-09 .000000000000D+00

.930000000000D+02 -.165531250000D+03 .285797618904D-08 .138275888459D+01

-.782497227192D-05 .346679124050D-03 .114385038614D-04 .544062509727D+04

.439800000000D+06 .298023223877D-07 -.296185101312D+01 -.111758708954D-07

.965683294025D+00 .993750000000D+02 -.629360976005D+00 -.541593988135D-08

-.571452374714D-11 .258000000000D+03 .184900000000D+04

.312000000000D+01 .000000000000D+00 -.651925802231D-08 .000000000000D+00

.440730000000D+06

3.03 NAVIGATION DATA M (Mixed) RINEX VERSION / TYPE

BCEmerge congo 20150620 012902 GMT PGM / RUN BY / DATE

Merged GPS/GLO/GAL/BDS/QZS/SBAS navigation file COMMENT

based on CONGO and MGEX tracking data COMMENT

DLR: O. Montenbruck; TUM: P. Steigenberger COMMENT

BDUT 5.5879354477e-09-2.042810365e-14 14 1849 TIME SYSTEM CORR

GAUT 3.7252902985e-09 5.329070518e-15 345600 1849 TIME SYSTEM CORR

GLGP -3.7252902985e-09 0.000000000e+00 345600 1849 TIME SYSTEM CORR

GLUT 1.0710209608e-08 0.000000000e+00 345600 1849 TIME SYSTEM CORR

GPGA -2.0081643015e-09-9.769962617e-15 432000 1849 TIME SYSTEM CORR

GPUT 4.5110937208e-09 7.105427358e-15 372608 1849 TIME SYSTEM CORR

QZUT 1.9557774067e-08 1.598721155e-14 61440 1850 TIME SYSTEM CORR

16 LEAP SECONDS

END OF HEADER

E12 2015 06 19 02 10 00-1.383925089613e-03-1.314646169703e-10 0.000000000000e+00

9.300000000000e+01-1.655312500000e+02 2.857976189037e-09 1.382758884589e+00

-7.824972271919e-06 3.466791240498e-04 1.143850386143e-05 5.440625097275e+03

4.398000000000e+05 2.980232238770e-08-2.961851013120e+00-1.117587089539e-08

9.656832940254e-01 9.937500000000e+01-6.293609760051e-01-5.415939881349e-09

-5.714523747137e-12 5.130000000000e+02 1.849000000000e+03

3.120000000000e+00 0.000000000000e+00-6.519258022308e-09-6.053596735001e-09

4.404850000000e+05

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 77

RINEX Version 3.03 Appendix A28

E12 2015 06 19 02 10 00-1.383925322443e-03-1.314504061156e-10 0.000000000000e+00

9.300000000000e+01-1.655312500000e+02 2.857976189037e-09 1.382758884589e+00

-7.824972271919e-06 3.466791240498e-04 1.143850386143e-05 5.440625097275e+03

4.398000000000e+05 2.980232238770e-08-2.961851013120e+00-1.117587089539e-08

9.656832940254e-01 9.937500000000e+01-6.293609760051e-01-5.415939881349e-09

-5.714523747137e-12 2.580000000000e+02 1.849000000000e+03

3.120000000000e+00 0.000000000000e+00-6.519258022308e-09 0.000000000000e+00

4.405300000000e+05

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 78

RINEX Version 3.03 Appendix A29

A 10 GNSS Navigation Message File – GLONASS Data Record Description

TABLE A10

GNSS NAVIGATION MESSAGE FILE – GLONASS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

SV / EPOCH / SV CLK - Satellite system (R), satellite number A1,I2.2,

(slot number in sat. constellation)

- Epoch: Toc - Time of Clock (UTC) 1X,I4,

year (4 digits)

- month, day, hour, minute, second 5(1X,I2.2),

- SV clock bias (sec) (-TauN) 3D19.12

- SV relative frequency bias

(+GammaN)

- Message frame time (tk+nd*86400) in

seconds of the UTC week *)

BROADCAST ORBIT - 1 - Satellite position X (km) 4X,4D19.12

- velocity X dot (km/sec)

- X acceleration (km/sec2)

- health (0=OK) (Bn)

BROADCAST ORBIT - 2 - Satellite position Y (km) 4X,4D19.12

- velocity Y dot (km/sec)

- Y acceleration (km/sec2)

- frequency number(-7...+13) (-7...+6

ICD 5.1)

BROADCAST ORBIT - 3 - Satellite position Z (km) 4X,4D19.12

- velocity Z dot (km/sec)

- Z acceleration (km/sec2)

- Age of oper. information (days) (E)

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded two-

digit exponents are required, however.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 79

RINEX Version 3.03 Appendix A30

A 11 GNSS Navigation Message File – Example: Mixed GPS / GLONASS

+------------------------------------------------------------------------------+

| TABLE A11 |

| GNSS NAVIGATION MESSAGE FILE – EXAMPLE MIXED GPS/GLONASS |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 N: GNSS NAV DATA M: MIXED RINEX VERSION / TYPE

XXRINEXN V3 AIUB 20061002 000123 UTC PGM / RUN BY / DATE

EXAMPLE OF VERSION 3.03 FORMAT COMMENT

GPSA 0.1025E-07 0.7451E-08 -0.5960E-07 -0.5960E-07 IONOSPHERIC CORR

GPSB 0.8806E+05 0.0000E+00 -0.1966E+06 -0.6554E+05 IONOSPHERIC CORR

GPUT 0.2793967723E-08 0.000000000E+00 147456 1395 TIME SYSTEM CORR

GLUT 0.7823109626E-06 0.000000000E+00 0 1395 TIME SYSTEM CORR

14 LEAP SECONDS

END OF HEADER

G01 2006 10 01 00 00 00 0.798045657575E-04 0.227373675443E-11 0.000000000000E+00

0.560000000000E+02-0.787500000000E+01 0.375658504827E-08 0.265129935612E+01

-0.411644577980E-06 0.640150101390E-02 0.381097197533E-05 0.515371852875E+04

0.000000000000E+00 0.782310962677E-07 0.188667086536E+00-0.391155481338E-07

0.989010441512E+00 0.320093750000E+03-0.178449589759E+01-0.775925177541E-08

0.828605943335E-10 0.000000000000E+00 0.139500000000E+04 0.000000000000E+00

0.200000000000E+01 0.000000000000E+00-0.325962901115E-08 0.560000000000E+02

-0.600000000000E+02 0.400000000000E+01

G02 2006 10 01 00 00 00 0.402340665460E-04 0.386535248253E-11 0.000000000000E+00

0.135000000000E+03 0.467500000000E+02 0.478269921862E-08-0.238713891022E+01

0.250712037086E-05 0.876975362189E-02 0.819191336632E-05 0.515372778320E+04

0.000000000000E+00-0.260770320892E-07-0.195156738598E+01 0.128522515297E-06

0.948630520258E+00 0.214312500000E+03 0.215165003775E+01-0.794140221985E-08

-0.437875382124E-09 0.000000000000E+00 0.139500000000E+04 0.000000000000E+00

0.200000000000E+01 0.000000000000E+00-0.172294676304E-07 0.391000000000E+03

-0.600000000000E+02 0.400000000000E+01

R01 2006 10 01 00 15 00-0.137668102980E-04-0.454747350886E-11 0.900000000000E+02

0.157594921875E+05-0.145566368103E+01 0.000000000000E+00 0.000000000000E+00

-0.813711474609E+04 0.205006790161E+01 0.931322574615E-09 0.700000000000E+01

0.183413398438E+05 0.215388488770E+01-0.186264514923E-08 0.100000000000E+01

R02 2006 10 01 00 15 0-0.506537035108E-04 0.181898940355E-11 0.300000000000E+02

0.155536342773E+05-0.419384956360E+00 0.000000000000E+00 0.000000000000E+00

-0.199011298828E+05 0.324192047119E+00-0.931322574615E-09 0.100000000000E+01

0.355333544922E+04 0.352666091919E+01-0.186264514923E-08 0.100000000000E+01

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 80

RINEX Version 3.03 Appendix A31

A 12 GNSS Navigation Message File – QZSS Data Record Description

TABLE A12

QZSS NAVIGATION MESSAGE FILE – QZSS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

(Columns 61-80)

PRN / EPOCH / SV CLK - Satellite system (J), Satellite PRN-192 A1,I2,

- Epoch: Toc - Time of Clock year (4 1X,I4,

digits)

- month, day, hour, minutes, seconds 5(1X,I2),

- SV clock bias (seconds) 3D19.12

- SV clock drift (sec/sec)

- SV clock drift rate (sec/sec2) *)

BROADCAST ORBIT - 1 - IODE Issue of Data, Ephemeris 4X,4D19.12

- Crs (meters)

- Delta n (radians/sec)

- M0 (radians)

BROADCAST ORBIT - 2 - Cuc (radians) 4X,4D19.12

- e Eccentricity

- Cus (radians)

- sqrt(A) (sqrt(m))

BROADCAST ORBIT - 3 - Toe Time of Ephemeris (sec of GPS 4X,4D19.12

week)

- Cic (radians)

- OMEGA (radians)

- CIS (radians)

BROADCAST ORBIT - 4 - i0 (radians) 4X,4D19.12

- Crc (meters)

- omega (radians)

- OMEGA DOT (radians/sec)

BROADCAST ORBIT – 5 - IDOT (radians/sec) 4X,4D19.12

- Codes on L2 channel (see IS-QZSS

5.2.2.2.3(2))

- GPS Week # (to go with TOE)

Continuous number, not mod(1024)!

- L2P data flag set to 1 since QZSS does

not track L2P

BROADCAST ORBIT – 6 - SV accuracy (meters) (IS -QZSS, 4X,4D19.12

Section 5.1.2.1.3.2) which refers to: IS

GPS 200H Section 20.3.3.3.1.3 use

specified equations to define nominal

values, N = 0-6: use 2(1+N/2) (round to

one decimal place i.e. 2.8, 5.7 and

11.3) , N= 7-15:use 2 (N-2), 8192

specifies use at own risk

- SV health (bits 17-22 w 3 sf 1) (see IS-

QZSS 5.2.2.2.3(4))

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 81

RINEX Version 3.03 Appendix A32

TABLE A12

QZSS NAVIGATION MESSAGE FILE – QZSS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

(Columns 61-80)

- TGD (seconds) The QZSS ICD

specifies a do not use bit pattern

"10000000" this condition is

represented by a blank field.

- IODC Issue of Data, Clock

BROADCAST ORBIT – 7 - Transmission time of message **) (sec 4X,4D19.12

of GPS week, derived e.g. from Z-count

in Hand Over Word (HOW)

- Fit interval flag (0 / 1) (see IS-QZSS,

5.2.2.2.4(4) 0 – two hours), 1 – more

than 2 hours. Blank if not known.

- Spare

- Spare

Records marked with * are optional

**) Adjust the Transmission time of message by -604800 to refer to the reported week, if

necessary.

*) In order to account for the various compilers, letters E,e,D, and d are allowed between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

A 13 QZSS Navigation Message File – Example

+------------------------------------------------------------------------------+

| TABLE A13 |

| QZSS NAVIGATION MESSAGE FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 N: GNSS NAV DATA J: QZSS RINEX VERSION / TYPE

GR25 V3.08 20140513 072944 UTC PGM / RUN BY / DATE

16 1694 7 LEAP SECONDS

END OF HEADER

J01 2014 05 13 08 15 12 3.323303535581D-04-1.818989403546D-11 0.000000000000D+00

6.900000000000D+01-4.927812500000D+02 2.222949737636D-09 7.641996743610D-01

-1.654587686062D-05 7.542252133135D-02 1.197867095470D-05 6.492895933151D+03

2.025120000000D+05-8.381903171539D-07-9.211997910060D-01-2.041459083557D-06

7.082252892260D-01-1.558437500000D+02-1.575843337115D+00-2.349740733276D-09

-6.793140104410D-10 2.000000000000D+00 1.792000000000D+03 1.000000000000D+00

2.000000000000D+00 1.000000000000D+00-4.656612873077D-09 6.900000000000D+01

1.989000000000D+05 0.000000000000D+00

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 82

RINEX Version 3.03 Appendix A33

A 14 GNSS Navigation Message File – BDS Data Record Description

Table A14

GNSS NAVIGATION MESSAGE FILE – BDS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

- Satellite system (C), sat number (PRN) A1,I2.2,

- Epoch: Toc - Time of Clock (BDT) year 1X,I4

(4 digits)

SV /EPOCH / SV CLK

- month, day, hour, minute, second 5,1X,I2.2,

- SV clock bias (seconds) 3D19.12

- SV clock drift (sec/sec)

- SV clock drift rate (sec/sec2) *)

- AODE Age of Data, Ephemeris (as 4X,4D19.12

specified in BeiDou ICD Table Section

5.2.4.11 Table 5-8) and field range is: 0-

BROADCAST ORBIT – 1

31.

- Crs (meters)

- Delta n (radians/sec)

**)

- M0 (radians)

- Cuc (radians) 4X,4D19.12

BROADCAST ORBIT – 2 - e Eccentricity

- Cus (radians)

- sqrt(A) (sqrt(m))

- Toe Time of Ephemeris (sec of BDT 4X,4D19.12

week)

BROADCAST ORBIT – 3

- Cic (radians)

- OMEGA0 (radians)

- Cis (radians)

- i0 (radians) 4X,4D19.12

BROADCAST ORBIT – 4 - Crc (meters)

- omega (radians)

- OMEGA DOT (radians/sec)

- IDOT (radians/sec) 4X,4D19.12

BROADCAST ORBIT – 5 - Spare

- BDT Week # ***)

- Spare

- SV accuracy (meters See: BDS 4X,4D19.12

ICD Section 5.2.4.: to define nominal

values, N = 0-6: use 2(1+N/2) (round to one

decimal place i.e. 2.8, 5.7 and 11.3) , N=

BROADCAST ORBIT – 6

7-15:use 2 (N-2), 8192 specifies use at own

risk)

- SatH1

- TGD1 B1/B3 (seconds)

- TGD2 B2/B3 (seconds)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 83

RINEX Version 3.03 Appendix A34

- Transmission time of message ****) 4X,4D19.12

(sec of BDT week,)

- AODC Age of Data Clock (as specified

BROADCAST ORBIT – 7

in BeiDou ICD Table Section 5.2.4.9

Table 5-6) and field range is: 0-31.

- Spare

- Spare

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

**) Angles and their derivatives transmitted in units of semi-circles and semi-circles/sec have to

be converted to radians by the RINEX generator.

***) The BDT week number is a continuous number. The broadcast 13-bit BDS System Time

week has a roll-over after 8191. It started at zero at 1-Jan-2006, Hence BDT week = BDT

week_BRD + (n*8192) where (n: number of BDT roll-overs).

****) Adjust the Transmission time of message by + or -604800 to refer to the reported week in

BROADCAST ORBIT -5, if necessary. Set value to 0.9999E9 if not known.

A 15 BeiDou Navigation Message File – Example

+------------------------------------------------------------------------------+

| TABLE A15 |

| BeiDou NAVIGATION MESSAGE FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 NAVIGATION DATA M (Mixed) RINEX VERSION / TYPE

BCEmerge montenbruck 20140517 072316 GMT PGM / RUN BY / DATE

DLR: O. Montenbruck; TUM: P. Steigenberger COMMENT

BDUT -9.3132257462e-10 9.769962617e-15 14 435 TIME SYSTEM CORR

END OF HEADER

C01 2014 05 10 00 00 00 2.969256602228e-04 2.196998138970e-11 0.000000000000e+00

1.000000000000e+00 4.365468750000e+02 1.318269196918e-09-3.118148933476e+00

1.447647809982e-05 2.822051756084e-04 8.092261850834e-06 6.493480609894e+03

5.184000000000e+05-2.654269337654e-08 3.076630958509e+00-3.864988684654e-08

1.103024081152e-01-2.506406250000e+02 2.587808789012e+00-3.039412318009e-10

2.389385241772e-10 0.000000000000e+00 4.350000000000e+02 0.000000000000e+00

2.000000000000e+00 0.000000000000e+00 1.420000000000e-08-1.040000000000e-08

5.184000000000e+05 0.000000000000e+00

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 84

RINEX Version 3.03 Appendix A35

A 16 GNSS Navigation Message File – SBAS Data Record Description

TABLE A16

GNSS NAVIGATION MESSAGE FILE – SBAS/QZSS L1 SAIF DATA RECORD

DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

SV / EPOCH / SV CLK - Satellite system (S), satellite number (slot A1,I2.2,

number in sat. constellation)

- Epoch: Toc - Time of Clock (GPS) year (4 1X,I4,

digits)

- month, day, hour, minute, second 5(1X,I2.2),

- SV clock bias (sec) (aGf0) 3D19.12,

- SV relative frequency bias (aGf1)

- Transmission time of message (start of the *)

message) in GPS seconds of the week

BROADCAST ORBIT - 1 - Satellite position X (km) 4X,4D19.12

- velocity X dot (km/sec)

- X acceleration (km/sec2)

- health (0=OK)

BROADCAST ORBIT - 2 - Satellite position Y (km) 4X,4D19.12

- velocity Y dot (km/sec)

- Y acceleration (km/sec2)

- Accuracy code (URA, meters)

BROADCAST ORBIT - 3 - Satellite position Z (km) 4X,4D19.12

- velocity Z dot (km/sec)

- Z acceleration (km/sec2)

- IODN (Issue of Data Navigation, DO229, 8

first bits after Message Type if MT9)

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

For QZSS L1-SAIF, note that accelerations represent only lunar and solar perturbation terms and

satellite position can be computed based on equations in Section A.3.1.2 of GLONASS ICD

version 5.0.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 85

RINEX Version 3.03 Appendix A36

A 17 SBAS Navigation Message File -Example

+------------------------------------------------------------------------------+

| TABLE A17 |

| SBAS NAVIGATION MESSAGE FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 N: GNSS NAV DATA S: SBAS RINEX VERSION / TYPE

SBAS2RINEX 3.0 CNES 20031018 140100 PGM / RUN BY / DATE

EXAMPLE OF VERSION 3.03 FORMAT COMMENT

SBUT -.1331791282D-06 -.107469589D-12 552960 1025 EGNOS 5 TIME SYSTEM CORR

13 LEAP SECONDS

This file contains navigation message data from a SBAS COMMENT

(geostationary) satellite, here AOR-W (PRN 122 = # S22) COMMENT

END OF HEADER

S22 2003 10 18 0 1 4-1.005828380585D-07 6.366462912410D-12 5.184420000000D+05

2.482832392000D+04-3.593750000000D-04-1.375000000000D-07 0.000000000000D+00

-3.408920872000D+04-1.480625000000D-03-5.000000000000D-08 4.000000000000D+00

-1.650560000000D+01 8.360000000000D-04 6.250000000000D-08 2.300000000000D+01

S22 2003 10 18 0 5 20-9.872019290924D-08 5.456968210638D-12 5.186940000000D+05

2.482822744000D+04-3.962500000000D-04-1.375000000000D-07 0.000000000000D+00

-3.408958936000D+04-1.492500000000D-03-5.000000000000D-08 4.000000000000D+00

-1.628960000000D+01 8.520000000000D-04 6.250000000000D-08 2.400000000000D+01

S22 2003 10 18 0 9 36-9.732320904732D-08 4.547473508865D-12 5.189510000000D+05

2.482812152000D+04-4.325000000000D-04-1.375000000000D-07 0.000000000000D+00

-3.408997304000D+04-1.505000000000D-03-5.000000000000D-08 4.000000000000D+00

-1.606960000000D+01 8.800000000000D-04 6.250000000000D-08 2.500000000000D+01

S22 2003 10 18 0 13 52-9.592622518539D-08 4.547473508865D-12 5.192110000000D+05

2.482800632000D+04-4.681250000000D-04-1.375000000000D-07 0.000000000000D+00

-3.409035992000D+04-1.518125000000D-03-3.750000000000D-08 4.000000000000D+00

-1.584240000000D+01 8.960000000000D-04 6.250000000000D-08 2.600000000000D+01

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 86

RINEX Version 3.03 Appendix A37

A 18 GNSS Navigation Message File – IRNSS Data Record Description

TABLE A18

GNSS NAVIGATION MESSAGE FILE – IRNSS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

SV / EPOCH / SV CLK - Satellite system (I), sat number (PRN) A1,I2.2,

- Epoch: Toc - Time of Clock (IRNSS) year 1X,I4,

(4 digits)

- month, day, hour, minute, second 5(1X,I2.2),

- SV clock bias (seconds) 3D19.12

- SV clock drift (sec/sec)

- SV clock drift rate (sec/sec2) *)

BROADCAST ORBIT - 1 - IODEC Issue of Data, Ephemeris and 4X,4D19.12

Clock

- Crs (meters) ***)

- Delta n (radians/sec)

- M0 (radians)

BROADCAST ORBIT - 2 - Cuc (radians) 4X,4D19.12

- e Eccentricity

- Cus (radians)

- sqrt(A) (sqrt(m))

BROADCAST ORBIT - 3 - Toe Time of Ephemeris (sec of IRNSS 4X,4D19.12

week)

- Cic (radians)

- OMEGA0 (radians)

- Cis (radians)

BROADCAST ORBIT - 4 - i0 (radians) 4X,4D19.12

- Crc (meters)

- omega (radians)

- OMEGA DOT (radians/sec)

BROADCAST ORBIT - 5 - IDOT (radians/sec) 4X,4D19.12

- Blank

- IRN Week # (to go with TOE) Continuous

number, not mod (1024), counted from

1980 (same as GPS).

- Blank

BROADCAST ORBIT - 6 - User Range Accuracy(m), See IRNSS 4X,4D19.12

ICD Section 6.2.1.4 , use specified

equations to define nominal values, N = 0-

6: use 2(1+N/2) (round to one decimal place

i.e. 2.8, 5.7 and 11.3) , N= 7-15:use 2 (N-2),

8192 specifies use at own risk

- Health (Sub frame 1,bits 155(most

significant) and 156(least significant)),

where 0 = L5 and S healthy, 1 = L5

healthy and S unhealthy, 2= L5 unhealthy

and S healthy, 3= both L5 and S unhealthy

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 87

RINEX Version 3.03 Appendix A38

TABLE A18

GNSS NAVIGATION MESSAGE FILE – IRNSS DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

- TGD (seconds)

- Blank

BROADCAST ORBIT - 7 - Transmission time of message **) 4X,4D19.12

(sec of IRNSS week)

- Blank

- Blank

- Blank

*) In order to account for the various compilers, E,e,D, and d are allowed letters between the

fraction and exponent of all floating point numbers in the navigation message files. Zero-padded

two-digit exponents are required, however.

**) Adjust the Transmission time of message by + or -604800 to refer to the reported week in

BROADCAST ORBIT 5, if necessary. Set value to 0.9999E9 if not known.

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 88

RINEX Version 3.03 Appendix A39

A 19 IRNSS Navigation Message File – Example

+------------------------------------------------------------------------------+

| TABLE A19 |

| IRNSS NAVIGATION MESSAGE FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 NAVIGATION DATA I (IRNSS) RINEX VERSION / TYPE

DecodIRNSS montenbruck 20141004 164512 GMT PGM / RUN BY / DATE

Source: IRNSS-1A Navbits COMMENT

END OF HEADER

I01 2014 04 01 00 00 00-9.473115205765e-04 1.250555214938e-12 0.000000000000e+00

0.000000000000e+00-5.820625000000e+02 4.720196615135e-09-1.396094758025e+00

-1.898035407066e-05 2.257102518342e-03-1.068413257599e-05 6.493487739563e+03

1.728000000000e+05 6.705522537231e-08-8.912102146884e-01-5.215406417847e-08

4.758105460020e-01 4.009375000000e+02-2.999907424014e+00-4.414469594664e-09

-4.839487298357e-10 1.786000000000e+03

1.130000000000e+01 0.000000000000e+00-4.190951585770e-09

1.728000000000e+05

I01 2014 04 01 02 00 00-9.473022073507e-04 1.250555214938e-12 0.000000000000e+00

1.000000000000e+00-5.101875000000e+02 4.945920303147e-09-8.741766987741e-01

-1.684948801994e-05 2.254169434309e-03-1.182407140732e-05 6.493469217300e+03

1.800000000000e+05 2.346932888031e-07-8.912408598963e-01-1.117587089539e-08

4.758065024964e-01 4.403750000000e+02-2.996779607145e+00-4.508759236491e-09

-5.464513333200e-10 1.786000000000e+03

1.130000000000e+01 0.000000000000e+00-4.190951585770e-09

1.800000000000e+05

I01 2014 04 01 04 00 00-9.472924284637e-04 1.250555214938e-12 0.000000000000e+00

2.000000000000e+00-5.100000000000e+02 5.217360181136e-09-3.491339518362e-01

-1.697987318039e-05 2.254509832710e-03-1.212581992149e-05 6.493469842911e+03

1.872000000000e+05 1.378357410431e-07-8.912725364615e-01 2.942979335785e-07

4.758010370344e-01 4.460625000000e+02-2.996772972812e+00-4.790199531038e-09

-6.039537285256e-10 1.786000000000e+03

1.130000000000e+01 0.000000000000e+00-4.190951585770e-09

1.872000000000e+05

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 89

RINEX Version 3.03 Appendix A40

A 20 Meteorological Data File -Header Section Description

TABLE A20

METEOROLOGICAL DATA FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

RINEX VERSION / - Format version : 3.03 F9.2,11X,

TYPE - File type: M for Meteorological Data A1,39X

PGM / RUN BY / DATE - Name of program creating current file A20,

- Name of agency creating current file A20,

- Date of file creation (See section 5.8) A20

* COMMENT - Comment line(s) A60

MARKER NAME - Station Name (preferably identical to A60

MARKER NAME in the associated

Observation File)

* MARKER NUMBER - Station Number (preferably identical to A20

MARKER NUMBER in the associated

Observation File)

# / TYPES OF OBSERV - Number of different observation types stored I6,

in the file

- Observation types 9(4X,A2)

The following meteorological observation

types are defined in RINEX Version 3:

PR : Pressure (mbar)

TD : Dry temperature (deg Celsius)

HR : Relative humidity (percent)

ZW : Wet zenith path delay (mm), (for WVR

data)

ZD : Dry component of zen.path delay (mm)

ZT : Total zenith path delay (mm)

WD : Wind azimuth (deg) from where the

wind blows

WS : Wind speed (m/s)

RI : "Rain increment" (1/10 mm): Rain

accumulation since last measurement

HI : Hail indicator non-zero: Hail detected

since last measurement

The sequence of the types in this record must

correspond to the sequence of the

measurements in the data records.

- If more than 9 observation types are being (6X,9(4X,A

used, use continuation lines with format 2))

SENSOR Description of the met sensor

MOD/TYPE/ACC - Model (manufacturer) A20,

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 90

RINEX Version 3.03 Appendix A41

TABLE A20

METEOROLOGICAL DATA FILE - HEADER SECTION DESCRIPTION

HEADER LABEL DESCRIPTION FORMAT

(Columns 61-80)

- Type A20,6X,

- Accuracy (same units as obs values) F7.1,4X,

- Observation type A2,1X

Record is repeated for each observation type

found in # / TYPES OF OBSERV record

SENSOR POS XYZ/H - Approximate position of the met sensor -

Geocentric coordinates X,Y,Z (ITRF 3F14.4,

- Ellipsoidal height H or WGS-84) 1F14.4,

- Observation type 1X,A2,1X

Set X, Y, Z to (BNK).

Make sure H refers to ITRF or WGS-84!

Record required for barometer, recommended

for other sensors.

END OF HEADER Last record in the header section. 60X

Records marked with * are optional

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 91

RINEX Version 3.03 Appendix A42

A 21 Meteorological Data File -Data Record Description

TABLE A21

METEOROLOGICAL DATA FILE - DATA RECORD DESCRIPTION

OBS. RECORD DESCRIPTION FORMAT

EPOCH / MET - Epoch in GPS time (not local time!) year (4 1X,I4.4,

digits, padded with 0 if necessary)

- month, day, hour, min, sec 5(1X,I2),

- Met data in the same sequence as given in the mF7.1

header

- More than 8 met data types: Use continuation 4X,10F7.1

lines

A 22 Meteorological Data File – Example

+------------------------------------------------------------------------------+

| TABLE A22 |

| METEOROLOGICAL DATA FILE - EXAMPLE |

+------------------------------------------------------------------------------+

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

3.03 METEOROLOGICAL DATA RINEX VERSION / TYPE

XXRINEXM V9.9 AIUB 19960401 144333 UTC PGM / RUN BY / DATE

EXAMPLE OF A MET DATA FILE COMMENT

A 9080 MARKER NAME

3 PR TD HR # / TYPES OF OBSERV

PAROSCIENTIFIC 740-16B 0.2 PR SENSOR MOD/TYPE/ACC

HAENNI 0.1 TD SENSOR MOD/TYPE/ACC

ROTRONIC I-240W 5.0 HR SENSOR MOD/TYPE/ACC

0.0000 0.0000 0.0000 1234.5678 PR SENSOR POS XYZ/H

END OF HEADER

1996 4 1 0 0 15 987.1 10.6 89.5

1996 4 1 0 0 30 987.2 10.9 90.0

1996 4 1 0 0 45 987.1 11.6 89.0

----|---1|0---|---2|0---|---3|0---|---4|0---|---5|0---|---6|0---|---7|0---|---8|

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 92

RINEX Version 3.03 Appendix A43

A 23 Reference Code and Phase Alignment by Constellation and Frequency Band

TABLE A23

Reference Code and Phase Alignment by Frequency Band

System Frequency Frequency Signal RINEX Phase Correction

Band [MHz] Observation applied to each

Code observed phase to

obtain aligned phase.

(φRINEX = φ

original(as issued by

the SV) + Δφ)

GPS L1 1575.42 C/A L1C None (Reference

Signal)

L1C-D L1S +¼ cycle

L1C-P L1L +¼ cycle

L1C-(D+P) L1X +¼ cycle

P L1P +¼ cycle

Z-tracking L1W +¼ cycle

Codeless L1N +¼ cycle

L2 1227.60 C/A L2C For Block II/IIA/IIR –

None;

See Note 1

For Block IIR-M/IIF/III

-¼ cycle

See Note 2

Semi- L2D None

codeless

L2C(M) L2S -¼ cycle

L2C(L) L2L -¼ cycle

L2C(M+L) L2X -¼ cycle

P L2P None (Reference

Signal)

Z-tracking L2W None

Codeless L2N None

L5 1176.45 I L5I None (Reference

Signal)

Q L5Q -¼ cycle

I+Q L5X Must be aligned to L5I

GLONASS G1 1602+k*9/1 C/A L1C None (Reference

6 Signal)

P L1P +¼ cycle

G2 1246+k*7/1 C/A L2C None (Reference

6 Signal)

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 93

RINEX Version 3.03 Appendix A44

TABLE A23

Reference Code and Phase Alignment by Frequency Band

System Frequency Frequency Signal RINEX Phase Correction

Band [MHz] Observation applied to each

Code observed phase to

obtain aligned phase.

(φRINEX = φ

original(as issued by

the SV) + Δφ)

P L2P +¼ cycle

G3 1202.025 I L3I None (Reference

Signal)

Q L3Q -¼ cycle

I+Q L3X Must be aligned to L3I

Galileo E1 1575.42 B I/NAV L1B None (Reference

OS/CS/SoL Signal)

C no data L1C +½ cycle

B+C L1X Must be aligned to L1B

E5A 1176.45 I L5I None(Reference Signal)

Q L5Q -¼ cycle

I+Q L5X Must be aligned to L5I

E5B 1207.140 I L7I None (Reference

Signal)

Q L7Q -¼ cycle

I+Q L7X Must be aligned to L7I

E5(A+B) 1191.795 I L8I None (Reference

Signal)

Q L8Q -¼ cycle

I+Q L8X Must be aligned to L8I

E6 1278.75 B L6B None (Reference

Signal)

C L6C -½ cycle

B+C L6X Must be aligned to L6B

QZSS L1 1575.42 C/A L1C None (Reference

Signal)

L1C (D) L1S None

L1C (P) L1L +¼ cycle

L1C-(D+P) L1X +¼ cycle

L1-SAIF L1Z N/A

L2 1227.60 L2C (M) L2S None (Reference

Signal)

L2C (L) L2L None

L2C (M+L) L2X None

L5 1176.45 I L5I None (Reference

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 94

RINEX Version 3.03 Appendix A45

TABLE A23

Reference Code and Phase Alignment by Frequency Band

System Frequency Frequency Signal RINEX Phase Correction

Band [MHz] Observation applied to each

Code observed phase to

obtain aligned phase.

(φRINEX = φ

original(as issued by

the SV) + Δφ)

Signal)

Q L5Q -¼ cycle

I+Q L5X Must be aligned to L5I

1278.75 S L6S None (Reference

LEX(6) Signal)

L L6L None

S+L L6X None

BDS B1 1561.098 I L2I None (Reference

Signal) (See Note 4

Below)

Q L2Q -¼ cycle

I+Q L2X Must be aligned to L2I

B2 1207.140 I L7I None (Reference

Signal)

Q L7Q -¼ cycle

I+Q L7X Must be aligned to L7I

B3 1268.52 I L6I None (Reference

Signal)

Q L6Q -¼ cycle

I+Q L6X Must be aligned to L6I

IRNSS L5 1176.45 A SPS L5A None (Reference

Signal)

B RS(D) L5B Restricted(See Note 3)

C RS(P) L5C None

B+C L5X Must be aligned to L5A

S 2492.028 A SPS L9A None (Reference

Signal)

B RS(D) L9B Restricted(See Note 3)

C RS(P) L9C None

B+C L9X Must be aligned to L9A

RINEX 3.03.IGS.RTCM.doc 2015-07-14

## Page 95

RINEX Version 3.03 Appendix A46

NOTES:

1) The GPS L2 phase shift values ignore FlexPower when the phases of the L2W and L2C can be

changed on the satellite.

2) The phase of the L2 C/A signal is dependent on the GPS satellite generation.

3) There is no public information available concerning the restricted service signals.

4) Note: Both C1x and C2x (RINEX 3.01 definition) have been used to identify the B1 frequency signals

in RINEX 3.02 files. If C2x coding is read in a RINEX 3.02 file treat it as equivalent to C1x.

RINEX 3.03.IGS.RTCM.doc 2015-07-14